3GPP TSG RAN WG1 #104b-e	R1-2102502

e-Meeting, April 12th – 20th, 2021

Source: 	ZTE

Title: 	Discussion on mechanisms to Improve Reliability for RRC_CONNECTED UEs

Agenda item:	8.12.2

Document for:	Discussion/Decision

Introduction

During RAN#88-e meeting, the revised WI on NR MBS has been approved [1]. One of its objectives is to specify required changes to improve reliability of Broadcast/Multicast service for RRC_CONNECTED UEs.

In RAN1#104-e meeting, some main issues about reliability improvement mechanism for MBS transmission were discussed and some agreements were made [2]. In this contribution, we focus on details of mechanisms to improve reliability of Broadcast/Multicast service for RRC_CONNECTED UE.

HARQ-ACK for Multicast

In Rel-16 NR, the existing HARQ-ACK feedback mainly includes: ACK/NACK feedback and NACK-only feedback. ACK/NACK feedback is divided into semi-static HARQ-ACK codebook and dynamic HARQ-ACK codebook, which are widely used. While NACK-only based solution is only supported in NR V2X for now.

For ACK/NACK for Multicast, ACK/NACK feedback can reuse most of the existing methods in multiplexing due to overlapping, while NACK-only feedback will face more new problems, and new multiplexing methods need to be studied. In the following sub-sections, analysis on ACK/NACK feedback and NACK-only feedback are provided.

ACK/NACK feedback

In order to support ACK/NACK feedback for NR MBS, some potential issues are analyzed as below.

PUCCH resource set configuration for ACK/NACK feedback and NACK-only feedback

In the RAN1#104-e meeting, the following agreement was reached. Based on the agreement below, flexible PUCCH resource configuration can be supported for multicast. However, there are still some remaining issues, such as the configuration of the PUCCH resource set.

Agreement: For ACK/NACK based feedback if supported for RRC_CONNECTED UEs receiving multicast, UE can be optionally configured a separate PUCCH-Config for multicast. Otherwise, PUCCH-Config for unicast applies. Agreement: For RRC_CONNECTED UEs receiving multicast, support the following: - ACK/NACK based HARQ-ACK feedback for multicast, - It is up to network to configure orthogonal PUCCH resources among UEs within the same group. - FFS: NACK-only based HARQ-ACK feedback for multicast, - It is up to network to configure the PUCCH resources and the PUCCH resources can be shared among UEs within the same group. - FFS details.

In unicast, the maximum number of PUCCH resource set per BWP is 4, each PUCCH resource set contains multiple PUCCH resources, and their corresponding UCI size is configured, except for the PUCCH resource set 0, which only supports no more than 2 UCI bits. The PUCCH resource set allows the gNB to dynamically select appropriate PUCCH resources for the UE in each scheduling.

Similar to unicast mechanism, for example, a maximum of 4 PUCCH resource sets are configured for multicast, and their corresponding UCI size is configured, except for the PUCCH resource set 0, which only supports no more than 2 UCI bits. The UCI size for each PUCCH resource set corresponding to the unicast PUCCH resource sets and the UCI size for each PUCCH resource set corresponding to the multicast PUCCH resource sets can be independently configured.

If NACK-only feedback is supported, NACK-only PUCCH resources and ACK/NACK PUCCH resources should be configured via the same PUCCH-config. For example, if a separate PUCCH-config is used for multicast, then ACK/NACK PUCCH resources and NACK-only PUCCH resources are configured via the PUCCH-config and can be configured in the same PUCCH resource set for multicast, for example, at least in PUCCH resource set 0, NACK-only PUCCH resources and ACK/NACK PUCCH resources can be configured together.

Proposal 1: Regarding configuration of PUCCH resource sets for multicast (reuse unicast mechanism):

- A maximum of 4 PUCCH resource sets are configured for multicast for the UE.

- NACK-only and ACK/NACK feedback share the same PUCCH resource set.

- UCI size limit for each PUCCH resource sets is configured, except for the PUCCH resource set 0, which only supports no more than 2 UCI bits.

Construction of HARQ-ACK codebook containing HARQ-ACK of MBS service

During RAN1#104-e meeting, the following agreements were made. It has been agreed that both Type-1 and Type-2 HARQ-ACK codebook are supported for MBS. However, there are still some remaining issues.

Agreement: For ACK/NACK based feedback if supported for multicast, construction of Type-1 HARQ-ACK codebook based on the union of the PDSCH TDRA sets of the unicast service and the multicast service (if they are separately configured), at least of the same priority, is supported - FFS details of Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast. - FFS details of Type-1 HARQ-ACK codebook construction for FDM-ed multicast and multicast if supported. - FFS: whether/how to optimize the Type-1 codebook construction to reduce the HARQ-ACK feedback payload size. Agreement: For ACK/NACK based feedback if supported for multicast, for Type-2 HARQ-ACK feedback construction for PTM scheme 1, - DAI for unicast and DAI for multicast are separately counted. - Concatenation of Type-2 HARQ-ACK codebook for unicast and multicast is supported. - FFS details on concatenating the codebooks. - FFS whether to support concatenating more than one Type-2 HARQ-ACK codebook for multicast.

- Type-1 HARQ-ACK codebook

For a UE that only receives MBS services, the following mechanisms on constructing a HARQ-ACK codebook for only MBS services can be considered.

For the MBS service with PDCCH scheduling, it is completely similar to the unicast service based on PDCCH scheduling, so the existing Type-1 HARQ-ACK codebook mechanism can be reused.

For the MBS service without PDCCH scheduling, the Type-1 HARQ-ACK codebook can be constructed according to the period of the MBS service, and the UE generates an ACK/NACK at each period of MBS service, which is similar to constructing a HARQ-ACK codebook for only SPS PDSCH.

For a UE that receives unicast services and MBS services at the same time, construction mechanism for the Type-1 HARQ-ACK codebook in Rel-16 can be reused. The UE always constructs a Type-1 HARQ-ACK codebook according to the union of the PDSCH sets of the unicast service and the MBS service.

For the case where multiple MBS services are FDM-ed or MBS services and unicast services are FDM-ed, we believe that it is necessary to determine the number of HARQ-ACK bits for a PDSCH SLIV group in the type-1 HARQ-ACK codebook based on the UE capability to receive FDM PDSCHs. For example, 3 MBS services are FDM-ed, but the UE capability is to receive 2 FDM PDSCHs at the same time, then for a PDSCH SLIV group, min (2, 3) HARQ-ACK bits should be generated. That is, for a PDSCH SLIV group, the number of HARQ-ACK bits generated by the UE is min (N, M), where N is the number of FDM-ed PDSCHs in the SLIV group, and M is the UE receiving capability for FDM PDSCHs.

Proposal 2: Type-1 HARQ-ACK codebook construction for FDM-ed multicast and multicast or FDM-ed unicast and multicast, for a PDSCH SLIV group, the number of HARQ-ACK bits generated by the UE is min (N, M), where N is the number of FDM-ed PDSCHs in the PDSCH SLIV group, and M is the UE receiving capability for FDM PDSCHs.

- Type-2 HARQ-ACK codebook

For the type-2 HARQ-ACK codebook, the DAI of the PDCCH for unicast and the DAI of the PDCCH for multicast are separately counted. So, sub-codebook 1 for unicast can be constructed based on the DAI of the PDCCH scheduling unicast PDSCH. Sub-codebook2 for MBS services can be constructed based on the DAI of the PDCCH scheduling MBS PDSCH. Then two sub-codebooks are concatenated, for example, sub-codebook 1 is before sub-codebook 2.

The DAI count is performed per MBS service. If the UE receives multiple MBS services at the same time, then the UE should generate HARQ-ACK sub-codebook for each MBS service in order to get the sub-codebook 2. The HARQ-ACK sub-codebook for each MBS service can be concatenated based on the G-RNTI of each MBS service in ascending in order to obtain the sub-codebook 2.

Proposal 3: Type-2 HARQ-ACK codebook construction for MBS:

- The DAI count is performed per MBS service.

- If the UE receives multiple MBS services, multiple sub-codebooks is generated separately for each MBMS service, then UE cconcatenates these sub-codebooks based on the G-RNTI of each MBS service in ascending order to form a type-2 codebook for the multiple MBS services.

- If the UE receives unicast and multicast, the sub-codebooks for unicast and multicast are generated separately, and then the multicast sub-codebook is concatenated after the unicast sub-codebook.

Time domain overlap between MBS PUCCHs or between MBS PUCCH and unicast PUCCH

In the RAN1#104-e meeting, the following agreement on PUCCH time-domain overlap was reached. But there are still some remaining issues, the following analysis is for these issues.

Agreement: For the cases of HARQ-ACK feedback (at least for ACK/NACK based feedback) is available for multicast and unicast for a given UE receiving multicast, for determining the PUCCH resource, - Support multiplexing for the same priority and prioritizing for different priorities at least when the corresponding PUCCH resources overlap in time in a slot. - FFS whether it is subject to UE capability. - FFS the case of non-overlapping PUCCHs resources for HARQ-ACK in the same slot. - FFS whether sub-slot based PUCCH transmission for HARQ-ACK is supported. - FFS the case of HARQ-ACK feedback for multicast and other UCI for unicast.

- The case of non-overlapping PUCCHs resources for HARQ-ACK in the same UL slot/subslot.

In unicast, only one HARQ-ACK PUCCH for the same priority can be transmitted in one UL slot. Note that if the UL subslot is configured for PUCCH, the number of symbols contained in a UL slot is equal to the number of symbols in the UL subslot. This is the overriding mechanism of HARQ-ACK PUCCH in UL slot, that is, in the same UL slot, the following HARQ-ACK PUCCH can replace the previous HARQ-ACK PUCCH. For a UE receiving multicast, this overriding mechanism should be used for the same priority. In one UL slot, only one HARQ-ACK PUCCH for the same priority can be transmitted for multicast.

For a UE receiving unicast and multicast, if HARQ-ACK PUCCH for unicast and HARQ-ACK PUCCH for multicast are indicated in the same UL slot, then these two HARQ-ACK PUCCHs should be determined independently first, and then be multiplexed for the same priority. The multiplexed PUCCH resource is determined based on the PRI in the last DCI corresponding to the unicast HARQ-ACK PUCCH. With this, even for UEs receiving multicast scheduled by a same group-common PDCCH, they can have separate indication of K1 and PRI in their UE specific PDCCH for unicast.

Proposal 4: Regarding the case of non-overlapping PUCCHs resources for HARQ-ACK in the same UL slot:

- HARQ-ACK PUCCH overriding mechanism should be reused for multicast with the same priority.

- if HARQ-ACK PUCCH for unicast and HARQ-ACK PUCCH for multicast are determined in the same UL slot, then they should be multiplexed for the same priority (regardless of whether they overlap in the time domain).

  - The multiplexed PUCCH resource is determined based on the PRI in the last DCI corresponding to the unicast HARQ-ACK PUCCH.

- sub-slot based PUCCH transmission for HARQ-ACK is supported.

- The case of HARQ-ACK feedback for multicast and other UCI for unicast

If HARQ-ACK for multicast overlaps with other UCI (such as SR/CSI) for unicast in a UL slot in the time domain, the existing multiplexing mechanism in unicast can be reused, where the HARQ-ACK for multicast can be treated as HARQ-ACK of unicast.

NACK-only feedback

For NACK-only feedback, different UEs in an MBS group can share the same PUCCH resource. PUCCH resources can be configured per MBS service. The UE receiving the MBS service can feed back in the corresponding PUCCH resource.

PUCCH resources for NACK-only feedback

Since it is impossible for NACK-only feedback to construct HARQ-ACK codebook, it will increase the number of feedbacks (i.e., UE can only transmit one bit information each time), which is not beneficial to save PUCCH resources in the time domain. If NACK-only feedback is supported, then RAN1 should study some ways to save PUCCH resources in the time domain.

For example, consider that PUCCH resources for NACK can only be configured per MBS service. If an MBS service is configured with NACK-only PUCCH resource, all UEs receiving the MBS service can feed back NACK in the NACK-only PUCCH resource. In order to increase flexibility, it should be allowed to configure multiple NACK-only PUCCH resources for one MBS service.

Regarding the PUCCH format applicable to NACK-only, it should be supported at least by PUCCH format 0. In addition, in order to obtain greater coverage for PUCCH format 0, it should be considered to enhance PUCCH format 0 through PUCCH repetition.

Proposal 5: If NACK-only feedback is supported for MBS,

- PUCCH resource configuration per MBS service is supported.

- PUCCH format 0 is supported for NACK-only feedback.

- PUCCH format 0 supports PUCCH repetition.

Time domain overlapping between multiple NACK-only PUCCHs

Furthermore, adopting the aforementioned NACK-only resource determination method means that each MBS service corresponds to a NACK-only resource. Then, a new rule of multiplexing/prioritization should be defined for a new overlapping case, that is, multiple NACK-only PUCCHs corresponding to different MBS services overlap each other in the time domain. For example, if a UE only receives multiple MBS services (no unicast services), the NACK-only PUCCHs corresponding to the multiple MBS services may overlap in the time domain, that is, only multiple NACK-only PUCCHs overlap. Obviously, this is a new case about overlapping.

This overlapping case will easily occur if one UE is allowed to receive multiple MBS at the same time, and when a TDD with a heavier DL slot is configured. Therefore, some necessary solutions and UE behavior need to be considered. Some multiplexing mechanisms should also be discussed in order to better support NACK-only feedback. For example, the following overlapping scenarios should be considered:

- One NACK-only PUCCH overlaps with one NACK-only PUCCH;

- More than 2 NACK-only PUCCH overlap each other.

- One NACK-only PUCCH overlaps with multiple non-overlapped NACK-only PUCCHs.

Proposal 6: If the NACK-only feedback mode is supported, RAN1 further discusses the multiplexing method for scenarios where NACK-only PUCCHs overlap.

Time domain overlapping between NACK-only PUCCH and unicast PUCCH/PUSCH

NACK-only feedback is that multiple UEs share the same PUCCH resource in order to feed back NACK. If a NACK-only PUCCH conflicts with the PUCCH/PUSCH of unicast service in the time domain, how to resolve the time domain overlap should also be studied.

For example, the different overlapping cases here include: unicast ACK/NACK/SR PUCCH F0/F1 overlaps with NACK-only PUCCH; unicast ACK/NACK/CSI PUCCH F2/F3/F4 overlaps with NACK-only PUCCH; unicast PUSCH and NACK-only PUCCH overlap. Necessary solutions need to be considered for these cases, respectively.

Proposal 7: If the NACK-only feedback mode is supported, RAN1 further discusses the multiplexing method for scenarios where NACK-only PUCCH and other PUCCH/PUSCH overlap.

Processing order of NACK-only related multiplexing

If an overlapping case contains both multiple NACK-only PUCCHs and multiple unicast PUCCHs, the processing order of different multiplexing types should be defined.

One potential processing order is to first deal with the overlapping between different NACK-only PUCCHs, and then the overlapping between the unicast PUCCH and the PUCCH obtained in the previous step will be processed. Another processing order is to classify overlapped PUCCHs according to service type corresponding to the UCI, i.e., MBS or unicast, and deal with the overlapping between the same types of UCI PUCCHs first, and then deal with the overlapping between different types of UCI PUCCHs. Obviously, a processing order needs to be defined here.

Support both ACK/NACK feedback and NACK-only feedback

In the RAN1#104-e meeting, ACK/NACK feedback has been supported, and NACK-only feedback is still FFS.

On the other hand, NACK-only feedback has its own applicable scenarios. NACK-only feedback can also be supported if time allowed. If both NACK-only feedback and ACK/NACK feedback are supported in MBS, then a method of configuring the feedback mode is required in order to combine the feedback mode with its applicable scenarios. Possible methods include:

Option 1: Configure the feedback mode semi-statically, for example, through UE-specific RRC signaling or MBS control signaling (MCCH). This method is simple but lacks flexibility. In some cases, the feedback mode cannot be adjusted in time to avoid PUCCH overlap.

Option 2: Dynamically indicate the feedback mode, for example, DCI can be used to determine the feedback mode. For example, a new field is introduced in DCI to indicate the feedback mode.

Option 3: Determine the feedback mode dynamically + semi-statically. For example, PUCCH resource sets is configured for multicast (mentioned in 2.1). ACK/NACK PUCCH resources and NACK-only PUCCH resources are configured in the PUCCH resource sets. Then the PUCCH resource and feedback mechanism can be simultaneously indicated in the PRI in the DCI.

With the Option 2 and Option 3, the feedback mechanism can be flexibly determined. It allows the gNB to flexibly change the NACK-only feedback to ACK/NACK feedback, which facilitates avoiding the complicated intra UE multiplexing caused by the NACK-only PUCCH. But Option 2 will cause DCI overhead.

So, we prefer Option 3.

Proposal 8: If NACK-only feedback is supported, consider the following method to determine the feedback mechanism for the UE between ACK/NACK feedback and NACK-only feedback:

- PUCCH resource sets containing ACK/NACK PUCCH resources and NACK-only PUCCH resources is configured for multicast for the UE.

- PRI in DCI is used to indicate the feedback mechanism and PUCCH resources from the PUCCH resource set.

Enabling/disabling HARQ-ACK feedback for MBS

In the RAN1#104-e meeting, it has been agreed that MBS service supports enabling/disabling feedback. However, there are still some remaining issues. The following analysis is provided as remaining issues.

Agreement: For enabling/disabling HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast, - Option 3: RRC signalling configures the enabling/ disabling function of DCI indicating the enabling /disabling HARQ-ACK feedback. - If RRC signalling configures the function, DCI indicates (explicitly or implicitly) whether HARQ-ACK feedback is enabled/disabled - FFS details on RRC signalling and DCI indicating. - If RRC signalling does not configure the function, DCI does not indicate enabling/disabling the HARQ-ACK feedback. - FFS whether enabling or disabling the feedback is the default mode. - Option 2: RRC indicates enabling/disabling. - FFS: whether down-selection between option 3 and option 2 is needed or support the both options. - FFS: enabling/disabling by MAC-CE.

Option 2: RRC indicates enabling/disabling

It is also a simple method. It is suitable for configuring whether to perform feedback for an MBS service. For example, if an MBS service is disabled for feedback, the feedback will not be performed for each MBS PDSCH scheduling of this MBS service. In this way, it is beneficial to avoid the overhead of DCI. Of course, it is also suitable for requirements, such as MBS services with low reliability requirements.

Option 3: RRC configures the enabling/ disabling function and DCI indicates enabling /disabling

If RRC signalling configures the function, DCI indicates (explicitly or implicitly) whether HARQ-ACK feedback is enabled/disabled. The RRC signaling can be carried in the MCCH per MBS service, and the DCI indicates the enable/disable feedback for the MBMS service. It is also possible that the RRC signaling is UE-specific RRC signaling, and the DCI indicates the enable/disable feedback for the PDSCH of each MBS service.

Option 4: enabling/disabling by MAC-CE

It is a method of avoiding physical layer overhead with moderate flexibility, and we think it can be supported. The MAC CE can be transmitted in unicast or MBS PDSCH. MAC CE is suitable for MBS services based on the SPS mechanism. Obviously, for SPS-based MBS services, the indication of enable/disable feedback in the activated DCI is equivalent to the indication of enable/disable feedback in the RRC signaling, because they are both slowly changing. However, the indication of enable/disable feedback in MAC CE has advantages in terms of flexibility.

In summary, we believe that option 2, option 3, and option 4 can coexist, and the gNB can choose the corresponding method according to the needs.

Proposal 9: Regarding enabling/disabling HARQ-ACK feedback for MBS, option 2, 3 and 4 are supported together.

- Option 2: RRC indicates enabling/disabling

- Option 3: RRC configures the enabling/ disabling function and DCI indicates enabling /disabling

- Option 4: MAC-CE indicates enabling/disabling

Priority for HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast

In RAN1#104-e meeting, the following agreement was achieved, but there is still a lack of details regarding the priority of the HARQ-ACK feedback of the MBS service.

Agreement: The priority for HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast can be, - Lower, higher than or equal to the HARQ-ACK feedback for unicast - FFS: How to reflect the priority in specification, e.g., whether it is configured or indicated to the UE - FFS: The total number of priorities across multicast and unicast - FFS the priority between HARQ-ACK feedback for multicast and other UCI for unicast (SR, CSI) or PUSCH for unicast.

In unicast, a total of 2 levels of physical layer priority are supported for HARQ-ACK. We believe that in multicast, a total of 2 levels of physical layer priority are supported for HARQ-ACK. A small number of levels facilitates the resolution of conflicts between PUCCH/PUSCH with different priorities. If more physical layer priorities are supported, intra UE multiplexing will become more complicated between different physical layer priorities.

For unicast and multicast, the same physical layer priority should be shared. For example, the high priority of multicast is equal to the high priority of unicast, and the low priority of multicast is equal to the low priority of unicast. In this way, for intra UE multiplexing between unicast PUCCH/PUSCH and multicast PUCCH, the existing mechanism is reused to the maximum extent.

For the physical layer priority indication for the multicast PDSCH, the existing unicast priority indication in the DCI can be reused.

In order to minimize the impact of standardization, the unicast priority rules between HARQ-ACK and SR/CSI is reused for the priority between HARQ-ACK feedback for multicast and other UCI for unicast (SR, CSI) or PUSCH for unicast.

From the current observations, we prefer to share a total of 2 levels of physical layer priorities between unicast and multicast in order to reduce the impact of standardization. If in the future, more physical layer priorities are proven to bring obvious benefits, then we are also open here.

Proposal 10: Regarding priority for HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast:

- The unicast priority indication field in the DCI is reused for the multicast PDSCH;

- The high/low priority of multicast is equal to high/low priority of unicast, respectively.

HARQ-ACK feedback for SPS-based MBS transmission

During RAN1#104-e meeting, the following agreement and working assumption on HARQ-ACK feedback for SPS group-common PDSCH for MBS was achieved. And the HARQ-ACK details for SPS PDSCH and activation/deactivation is still FFS.

Agreement: For RRC_CONNECTED UEs, support HARQ-ACK feedback for SPS group-common PDSCH for MBS - FFS: The retransmission scheme(s) - FFS: The HARQ-ACK details for SPS PDSCH and activation/deactivation, which can be discussed in AI 8.12.2 Working assumption: For activation/deactivation of SPS group-common PDSCH for MBS in RRC_CONNECTED state, - At least group-common PDCCH is supported - FFS: Whether and how to address the missed activation and deactivation - FFS: Whether UE-specific PDCCH is supported for activation/deactivation

Some companies concerned that if NACK-only feedback is configured for SPS-based MBS transmission and DTX occurs on an activation PDCCH, the UE will miss multiple SPS group-common PDSCHs. Furthermore, the gNB still thinks that the UE correctly receives all the SPS-based MBS transmission. Introducing ACK/NACK feedback to the activation PDCCH is a potential method. However, a new rule needs to be defined to determine the feedback mechanism, e.g., feedback resource, codebook generation, etc.

For the ACK/NACK feedback of the current SPS-based unicast transmission, PDSCHs can be divided into ‘PDSCH with scheduling PDCCH’ and ‘PDSCH without scheduling PDCCH’. When generating the type2 codebook, the ‘PDSCH with scheduling PDCCH’ will be regarded as a dynamic scheduling PDSCH and is counted together with other dynamic scheduling PDSCHs. However, the feedback of ‘PDSCH without scheduling PDCCH’ will follow the SPS codebook generation rule.

Similarly, PDSCHs in SPS-based MBS transmission can also be divided into two types, i.e., ‘PDSCH with scheduling PDCCH’ and ‘PDSCH without scheduling PDCCH’ as shown in Figure-1. For identifying DTX of the activation PDCCH, the NACK-only feedback mode can be applied to ‘PDSCH without scheduling PDCCH’ only, while the feedback mode to ‘PDSCH with scheduling PDCCH’ can be fixed to ACK/NACK feedback.

Alternatively, the feedback mode to ‘PDSCH with scheduling PDCCH’ can follow the configuration of feedback mode for DG-PDSCH, and it will up to implementation of the gNB.

Regarding feedback for deactivation PDCCH, the NACK-only mode can also not work well. From the perspective of UE reception, there are two possible statuses, that is, 1) the deactivation PDCCH is received correctly; 2) the deactivation PDCCH is DTX. No matter which status appears, the UE will not feedback on the NACK-only resource corresponding to the deactivation PDCCH, which means that NACK-only feedback is invalid for deactivation PDCCH. So the feedback mode for deactivation PDCCH should also be further studied.

Figure-1: HARQ-ACK feedback for SPS-based MBS transmission

According to the analysis above, we have the following proposal.

Proposal 11: Regarding HARQ-ACK feedback for SPS-based MBS transmission, if NACK-only feedback is configured:

- The NACK-only feedback mode applies to ‘PDSCH without scheduling PDCCH’ only

- The feedback mode for ‘PDSCH with scheduling PDCCH’ can be,

- Option 1: fixed to ACK/NACK feedback

- Option 2: follow the configuration of feedback mode for DG-PDSCH

- FFS: the feedback mode for deactivation PDCCH

CSI feedback for Multicast

In NR system, flexible CSI-RS resource configuration and CSI report configuration have already been supported. With the help of CSI measurement and feedback, gNB can perform better link adaptation to dynamically update MCS, precoding, and beam to transmit PDSCH. This is beneficial to the improvement of transmission reliability and system spectrum efficiency for both unicast and multicast transmission.

Considering the tradeoff between CSI feedback overhead and CSI accuracy, in the scenario where channel correspondence can be applied, e.g., gNB with significant large antenna arrays in TDD band, the SRS-assisted CSI acquisition for both unicast and multicast transmission is a candidate approach to simplify the UE complexity on CSI measurement and reporting. However, for the FDD band, channel correspondence cannot be applied, CSI feedback is still indispensable. We have studied the system performance under different CSI feedback modes and provided the simulation results in [3]. According to our preliminary simulation results, for the mode of only CQI feedback but without PMI/RI feedback, there is still much room for the system performance improvement. Therefore, it is necessary for gNB to perform PMI/RI self-adaptive selection through the feedback from a group of UEs.

Based on the agreements reached in RAN1#102e meeting, existing CSI feedback can be used for multicast transmission. However, the current CSI reporting mechanism and reporting quantity are designed for unicast transmission, there may be some enhancement room for multicast transmission, which is targeting a group of UEs instead of single UE. The link adaptation for MBS transmission should take into account the requirements of a group of UEs. Therefore, each UE can be required to feedback multiple candidate {CQI, PMI, RI} sets, such as the optimal {CQI, PMI, RI} set and suboptimal {CQI, PMI, RI} set. So that gNB can learn more comprehensive channel status information of each UE in the group of UEs. Based on such information, gNB can schedule the MBS PDSCH with a more appropriate applicable MCS and precoding.

Proposal 12: UE supports reporting multiple candidate {CQI, PMI, RI} sets in one CSI report for MBS.

Subband size is another problem to be considered in CSI feedback used for MBS transmission. The bandwidth of common frequency resource for MBS may be smaller than a unicast BWP, so the subband size suitable for the common frequency resource may be different from the subband size suitable for the unicast BWP. In order to schedule a group of UEs on the common frequency resource, the gNB should be reported the CSI feedback with aligned subband size from different UEs. Therefore, in order to get CSI feedback that is applied in MBS transmission link adaptation, the subband size should be common for common frequency resource for MBS transmission for all UEs.

If the common frequency resource is defined as an MBS BWP, the existing mechanism can be reused. In this way, network can configure the same subband size for the MBS BWP for all UEs in the same group, without any additional standardization requirement.

If the common frequency resource is defined as an ‘MBS frequency region’ with a number of contiguous PRBs, which is configured within the dedicated unicast BWP, some additional mechanisms should be introduced for the UEs supporting both unicast and MBS transmission to determine the subband size applicable for the MBS frequency region.

Proposal 13: RAN1 further discusses the issues on CSI subband determination for MBS transmission.

- If common frequency resource is defined as an MBS frequency region within the unicast BWP, mechanism to align different UE’s CSI subband size is needed.

- If common frequency resource is defined as an MBS BWP, no additional mechanism is needed (i.e., just reusing the previous MBS mechanism).

Conclusion

In this contribution, we discuss the potential candidate solutions to improve reliability for NR MBS with the following observations and proposals.

ACK/NACK feedback

Proposal 1: Regarding configuration of PUCCH resource sets for multicast (reuse unicast mechanism):

- A maximum of 4 PUCCH resource sets are configured for multicast for the UE.

- NACK-only and ACK/NACK feedback share the same PUCCH resource set.

- UCI size limit for each PUCCH resource sets is configured, except for the PUCCH resource set 0, which only supports no more than 2 UCI bits.

Proposal 2: Type-1 HARQ-ACK codebook construction for FDM-ed multicast and multicast or FDM-ed unicast and multicast, for a SLIV group, the number of HARQ-ACK bits generated by the UE is min (N, M), where N is the number of FDM-ed PDSCHs in the SLIV group, and M is the UE receiving capability for FDM PDSCHs.

Proposal 3: Type-2 HARQ-ACK codebook construction for MBS:

- The DAI count is performed per MBS service.

- If the UE receives multiple MBS services, multiple sub-codebooks is generated separately for each MBMS service, then UE cconcatenates these sub-codebooks based on the G-RNTI of each MBS service in ascending order to form a type-2 codebook for the multiple MBS services.

- If the UE receives unicast and multicast, the sub-codebooks for unicast and multicast are generated separately, and then the multicast sub-codebook is concatenated after the unicast sub-codebook.

Proposal 4: Regarding the case of non-overlapping PUCCHs resources for HARQ-ACK in the same UL slot:

- HARQ-ACK PUCCH overriding mechanism should be reused for multicast with the same priority.

- if HARQ-ACK PUCCH for unicast and HARQ-ACK PUCCH for multicast are determined in the same UL slot, then they should be multiplexed for the same priority (regardless of whether they overlap in the time domain).

  - The multiplexed PUCCH resource is determined based on the PRI in the last DCI corresponding to the unicast HARQ-ACK PUCCH.

- sub-slot based PUCCH transmission for HARQ-ACK is supported.

NACK-only feedback

Proposal 5: If NACK-only feedback is supported for MBS,

- PUCCH resource configuration per MBS service is supported.

- PUCCH format 0 is supported for NACK-only feedback.

- PUCCH format 0 supports PUCCH repetition.

Proposal 6: If the NACK-only feedback mode is supported, RAN1 further discusses the multiplexing method for scenarios where NACK-only PUCCHs overlap.

Proposal 7: If the NACK-only feedback mode is supported, RAN1 further discusses the multiplexing method for scenarios where NACK-only PUCCH and other PUCCH/PUSCH overlap.

Proposal 8: If NACK-only feedback is supported, consider the following method to determine the feedback mechanism for the UE between ACK/NACK feedback and NACK-only feedback:

- PUCCH resource sets containing ACK/NACK PUCCH resources and NACK-only PUCCH resources is configured for multicast for the UE.

- PRI in DCI is used to indicate the feedback mechanism and PUCCH resources from the PUCCH resource set.

Enabling/disabling HARQ-ACK feedback for MBS

Proposal 9: Regarding enabling/disabling HARQ-ACK feedback for MBS, option 2, 3 and 4 are supported together.

- Option 2: RRC indicates enabling/disabling

- Option 3: RRC configures the enabling/ disabling function and DCI indicates enabling /disabling

- Option 4: MAC-CE indicates enabling/disabling

Priority for HARQ-ACK feedback

Proposal 10: Regarding priority for HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast:

- The unicast priority indication field in the DCI is reused for the multicast PDSCH;

- The high/low priority of multicast is equal to high/low priority of unicast, respectively.

HARQ-ACK feedback for SPS-based MBS transmission

Proposal 11: Regarding HARQ-ACK feedback for SPS-based MBS transmission, if NACK-only feedback is configured:

- The NACK-only feedback mode applies to ‘PDSCH without scheduling PDCCH’ only

- The feedback mode for ‘PDSCH with scheduling PDCCH’ can be,

- Option 1: fixed to ACK/NACK feedback

- Option 2: follow the configuration of feedback mode for DG-PDSCH

- FFS: the feedback mode for deactivation PDCCH

CSI feedback for multicast

Proposal 12: UE supports reporting multiple candidate {CQI, PMI, RI} sets in one CSI report for MBS.

Proposal 13: RAN1 further discusses the issues on CSI subband determination for MBS transmission.

- If common frequency resource is defined as an MBS frequency region within the unicast BWP, mechanism to align different UE’s CSI subband size is needed.

- If common frequency resource is defined as an MBS BWP, no additional mechanism is needed (i.e., just reusing the previous MBS mechanism).

Reference

RP-201038, WID revision: NR Multicast and Broadcast Services, RAN#88e.

3GPP RAN1#104-e, Chairman’s notes.

R1-2008829, Preliminary Simulation Results of Rel-17 MBS, RAN1#103-e.