3GPP TSG RAN WG1 #104b-e                                                           R1-2102506

e-Meeting, April 12th – 20th, 2021

Source:	vivo

Title:	Further discussion on multi beam enhancement

Agenda Item:	8.1.1

Document for:	Discussion and Decision

Introduction

The 3GPP Rel-17 NR MIMO WID [1] includes enhancement on multi-beam operation. In RAN1 #102-e ~ #104-e meetings, some issues were discussed, including unified TCI framework and common beam, L1/L2-centric inter-cell mobility, MPUE and MPE, etc. [2,3,4] In this contribution, we share our views on the issues for Rel-17 NR FeMIMO on multi-beam operations.

Common beam and unified TCI framework

Single common beam and multiple common beams

Conclusion On Rel.17 unified TCI framework, based on the agreements in RAN1#102-e and 103-e, the following terms are defined as follows (at least for the purpose of discussion and reaching agreements). For M=1: - DL TCI: The source reference signal(s) (analogous to Rel.15, two, if qcl_Type2 is configured in addition to qcl_Type1) in the DL TCI provides QCL information at least for UE-dedicated reception on PDSCH and all of CORESETs in a CC For N=1: - UL TCI: The source reference signal in the UL TCI provides a reference for determining UL TX spatial filter at least for dynamic-grant/configured-grant based PUSCH and all of dedicated PUCCH resources in a CC For M=N=1: - Joint DL/UL TCI: A TCI refers to at least a common source reference RS used for determining both the DL QCL information and the UL TX spatial filter. - Separate DL/UL TCI: The DL TCI and UL TCI are distinct (therefore, separate). For M>1: - DL TCI: Each of the M source reference signals (or 2M, if qcl_Type2 is configured in addition to qcl_Type1) in the M DL TCIs provides QCL information at least for one of the M beam pair links for UE-dedicated receptions on PDSCH and/or subset of CORESETs in a CC For N>1: - UL TCI: Each of the N source reference signals in the N UL TCIs provide a reference for determining UL TX spatial filter at least for one of the N beam pair links associated with dynamic-grant(s)/configured-grant(s) based PUSCH, and/or subset of dedicated PUCCH resources in a CC For M>1 and/or N>1: - Joint DL/UL TCI: A TCI refers to at least a common source reference RS used for determining both the DL QCL information and the UL TX spatial filter. In this case, M=N. - Separate DL/UL TCI: The M DL TCIs and N UL TCIs are distinct (therefore, separate). Note: Other TCI types/terms such as “common TCI” are not used.

In 3GPP Rel-15/Rel-16, the scheme for beam indication is designed independently for each channel/RS and the default beam is already supported for PDSCH, PUCCH, PUSCH, CSI-RS and SRS, etc. Most of the default TCI state/spatial relation/pathloss RS are determined according to the QCL assumption of CORESET with the lowest controlResourceSetId. In Rel-17 the unified TCI framework and common beam are introduced to unify the signaling design and reduce the signaling overhead. In RAN1 #104-e meeting, the terms, i.e. M>=1, N>=1 TCI state(s), are defined at least for the purpose of discussion and reaching agreements. In general, the typical case is single beam operation because different DL/UL channels/RSs are propagating through almost the same path. So multiple channels can apply a common beam indicated by gNB based on the existing solution of default beam scheme in Rel-15/Rel-16 to unify the beam indication for different channels/RSs, which is an effective way to reduce the signaling overhead for beam indication with minimum standardization efforts.

In the single common beam mode mentioned above, different channels/RSs use the same common beam to reduce the signaling overhead of beam indication. But this single common beam may not be a universal solution. The performance objectives of each channel or RS are different. For example, control channels seek coverage and reliability, while the data channels pursue high data rate. In general, the control channel is transmitted using a wide beam and the data channel is transmitted using a narrow beam, which can better meet the performance objectives of each channel. Therefore, even if the common beam is introduced, the requirement of how to distinguish different performance objectives of each channel or RS still needs to be solved.

The SLS evaluation performance of PDSCH transmission with different beam widths in InH scenario is provided. The two beam widths are formed according to the beamforming parameters, where the main lobe patterns for wide beam and narrow beam in one dimension are depicted in Figure 1. Other simulation parameters can be referred to Table 6. Compared with the narrow beam, wide beam is generated by reducing the size of antenna array and TxRU, while the number of ports remains unchanged.

- Main lobe beam patterns in one dimension of different beam widths.

- Performance of PDSCH with different beam widths

- SE comparison of PDSCH with different beam widths (same λ, RU0.21)

|  | Mean SE | 5% SE | 50% SE | 95% SE |
| --- | --- | --- | --- | --- |
| narrow beam (baseline) | 0.0% | 0.0% | 0.0% | 0.0% |
| wide beam | -7.7% | -14.1% | -17.0% | 0.0% |

It can be seen from the simulation results that the performance of PDSCH degrades with the increase of the beam width. Then the expected data rate and spectrum efficiency cannot be achieved if a common wide beam is used to ensure the coverage and reliability required by the control channels. Although the wide beam helps to obtain better coverage, the SINR and beamforming gain of the wide beam are far less than that of the narrow beam. If the data channel applies the same common beam as the control channel, the performance degradation is obvious, which is not desired.

Narrow beam provides better data transmission efficiency while wide beam provides more robustness.

If the narrow beam is used for the PDCCH transmission, on the other hand, the coverage/robustness will be affected. When the UE moves and rotates, the TCI state of PDCCH needs to be updated frequently, otherwise the UE cannot monitor the scheduling DCI from the gNB in time.

Support M=N>1 joint TCI states, M>1 separate DL TCI states, N>1 separate UL TCI states in Rel-17.

In Rel-17 the joint TCI state and separate TCI state indication are supported. For the separate DL TCI state, the source reference signal(s) provide QCL information at least for UE-dedicated reception on PDSCH and for UE-dedicated reception on all or subset of CORESETs in a CC. Similarly, for the separate UL TCI state, the source reference signal(s) provide a reference for determining common UL TX spatial filter(s) at least for dynamic-grant/configured-grant based PUSCH, all or subset of dedicated PUCCH resources in a CC.

Based on previous discussion, to support multiple common beams, a flexible unified TCI framework could be supported. Multiple channel/RS types can be flexibly grouped to apply a same TCI state. The group index can be added in multiple channel/RS configuration, or the group configuration including multiple channel/RS is provided by gNB. The number of TCI states is equal to the number of the groups. The different groups can include the channel/RS with different performance objectives or different functionality for DL and UL. From the perspective of tradeoff between complexity and flexibility, in Rel-17 the maximum number of groups can be set to 2.

Multiple channel/RS types can be flexibly grouped to apply a same TCI state.

The maximum number of groups is 2 in Rel-17.

In Rel-16 the PUCCH spatial relation activation was enhanced as well. If the indicated PUCCH resource by MAC CE is configured as part of a PUCCH group, no other PUCCH resources within the same PUCCH group are indicated in the MAC CE, and the MAC CE applies to all the PUCCH resources in the PUCCH group. Similer to PUCCH group in Rel-16, the gNB can divide CORESETs and the associated channels into different groups with their own TCI state.

Non-UE specific CORESETs and related scheduling and feedback is more sensitive to robustness of the beam, blockage will cause frequent beam pair link interruption. On the other hand, if a non-UE-specific CORESET, e.g. CORESET#0, uses a wide beam, it can maintain the basic RRC/MAC CE connection and small packet scheduling in case of blockage. A mode should be enabled that the beam indication for joint/separate TCI state update is not applied to non-UE-specific CORESET, and also to the PUSCH/PDSCH scheduled/activated and PUCCH transmission triggered by non-UE-specific CORESETs to increase the robustness of beam link. For example, two groups are configured, where the wide beam is used to the first group including non-UE-specific CORESET and the PUSCH/PDSCH scheduled/activated and PUCCH transmission triggered by non-UE-specific CORESETs, the narrow beam is used to the second group including the other channels/RSs, e.g. UE-dedicated PDSCH and UE-specific CORESET. For non-UE-specific CORESETs, e.g. CORESET#0, the existing beam indication can be reused, i.e. based on MAC CE. Furthermore, the MAC CE indicates a TCI state for the group including non-UE-specific CORESETs and the PUSCH/PDSCH scheduled/activated and PUCCH transmission triggered by non-UE-specific CORESETs.

In order to ensure the robustness of the PDCCH carrying beam indication DCI, a MAC CE can be used to indicate the joint/separate TCI state for a group including the PDCCH carrying beam indication DCI and the associated channels, e.g. the scheduled PDSCH/PUSCH and the PUCCH triggered by the PDCCH.

Support to divide CORESETs and the associated channels into different groups with their own TCI states, e.g. non-UE-specific CORESETs, CORESET with beam indication DCI.

For multi-TRP case, multiple common beams operation also can be considered to reduce the signaling overhead for beam indication. With non-ideal backhaul multi-TRP as the major operation scenario, each TRP may have specific common beam. The channels/RSs in a group correspond to a TRP and apply a same TCI state, i.e. the M/N=2. For RS for channel measurement, e.g. CSI-RS for CSI or SRS for codebook, the RS resource sets/subsets/groups can be associated with different TRPs, where a TCI state is applied to the group including the RS resources corresponding to a TRP.

Support applying single common beam to a TRP, i.e. M/N=2 for multi-TRP case.

TCI state pool and unified TCI design

Agreement On Rel-17 unified TCI framework: - A pool of joint DL/UL TCI state is used for joint DL/UL TCI state update (beam indication). - FFS: The pool for separate DL and UL TCI state update (beam indication) - Note: Here, TCI state pool refers to a pool configured via higher-layer (RRC) signaling - FFS: Whether joint TCI may include UL specific parameter(s) such as UL PC/timing parameters, PL RS, panel-related indication, etc. and if it is included, it is used only for UL transmission of the DL and UL transmissions to which the joint TCI is applied

In Rel-15/Rel-16 separate signaling frameworks for DL TCI state and UL spatial relation are specified. For DL channels/RSs, TCI state list for CORESET is configured in PDCCH-config, but the TCI state list for PDSCH is configured in PDSCH-config. Similarly, the spatial relation list for PUCCH is configured in PUCCH-config. Based on the TCI state list/spatial relation list, MAC CE and/or DCI are used to indicate DL/UL beam. Besides, the TCI state/spatial relation of DL/UL RS is configured by RRC or activated by MAC CE, the spatial relation of PUSCH is determined according to SRI field in DCI. It can be seen from the above that the current solution of TCI state/spatial relation framework of each channel/RS leads to a large amount of signaling overhead.

In RAN1 #103-e meeting, as a solution of unified TCI framework, a common TCI state pool can be configured by RRC and is applied to joint DL/UL TCI state update. For separate DL and UL TCI state update, from the point of view of saving RRC signaling overhead, the single TCI state pool also can be adopted.

A single TCI state pool is applied to joint TCI state update and separate TCI state update.

Agreement On Rel.17 unified TCI framework: - For joint and separate DL/UL TCI, DL large scale QCL properties are inferred from one (qcl-Type1) or two RSs (qcl-Type1 and qcl-Type2) analogous to Rel.15/16 - For joint DL/UL TCI, UL spatial filter is derived from the RS of DL QCL Type D

In RAN1 #104-e meeting, the DL QCL properties are the same as Rel-15/Rel-16, the UL spatial filter is derived from the RS of DL QCL-TypeD in case of joint TCI. The new QCL type is not introduced in Rel-17. In Rel-15/Rel-16 the spatial relation for UL channel is configured to include only one reference signal indicating UL beam.  With shared TCI state pool design for separate TCI case, the TCI state can also be shared between UL TCI and DL TCI.  For such cases, the UL spatial filter can also be designed in the similar way as the joint TCI case, e.g., based on the DL QCL Type D RS in the TCI state. One corner case is when the source RS for spatial filter is an UL RS. Even for such cases, our understanding is that it is still under discussion whether UL RS can be used as source RS for DL. Sharing TCI state design for DL and UL is more future-proof.

For separate DL/UL TCI, UL spatial filter is derived from the RS of QCL-TypeD RS.

Agreement On Rel.17 unified TCI framework, the supported source/target QCL relations in the current TS38.214 V16.4.0 is supported for QCL Type D. - Note: This implies that the following source RS types for DL QCL (Type D, for DL RX spatial filter reference) information for DL UE-dedicated reception on PDSCH and all/subset of CORESETs are supported: - CSI-RS for beam management - CSI-RS for tracking - FFS (to be decided by RAN1#104bis-e): If SSB, CSI-RS for CSI, and/or SRS for BM are also supported as source RS types Agreement On Rel.17 unified TCI framework, the following source RS types for UL TX spatial filter are supported: - CSI-RS for tracking - Note: SRS for BM, SSB, and CSI-RS for BM have been agreed in RAN1#102-e - FFS (to be decided by RAN1#104bis-e): non-BM CSI-RS other than for tracking, non-BM SRS

In RAN1 #102-e meeting, SSB has been agreed as the source RS for UL Tx spatial filter. Thus for TCI framework design, SSB should be possible to be used as source RS for DL QCL-TypeD. In CA case we also see the necessity of informing UE on not tracking beams independently on different cell with signaling design. Putting the same RS in the QCL-D would serve such purpose and this would require SSB as the QCL-D source in the most typical cases.

Existing beam indication scheme does not support SRS as a QCL-TypeD source for DL channel, meaning that when the gNB triggers a SRS resource set to complete the UL beam training, the UL measurement results cannot be directly used for TCI state indication. That is, the UL beam management cannot be fully utilized and thus leads to additional measurement complexity and power consumption at UE.

Due to the size constraints of UE antenna array, the number of beams supported by UE is much less than that of network side. For L1-RSRP computation, the UE may be configured with a CSI-RS resource setting up to 16 CSI-RS resource sets having up to 64 resources within each set. But for each SRS resource set configured by SRS-ResourceSet, a UE may be configured with SRS resources (higher layer parameter SRS-Resource), where the maximum value of K is indicated by UE capability (In TS 38.331 maximum number of SRS resources in an SRS resource set is 16). Compared with UL beam training, DL beam training requires UE to measure quality of more beam links. For example, in case of multi-TRP, when the UE moves or rotates, multiple TRPs would trigger more CSI-RS resource sets or a CSI-RS resource set containing more CSI-RS resources for UE to measure. But if the gNB triggers an SRS resource set with usage=beamManagement for UL beam training, it is enough to configure the SRS resource set that includes a small number of SRS resources which the UE just needs to transmit once with UL beams. And multiple TRPs at the network side can measure the uplink beam link quality of these SRS resources simultaneously. It can reduce the latency of beam training and does not need additional downlink beam management.

As an important solution of unified TCI framework, the source RSs of TCI state and spatial relation are unified into SSB, CSI-RS and SRS, that is, SRS can be introduced as a DL QCL-TypeD source. This solution can fully utilize the UL beam measurement results to reduce the UE’s effort on tracking DL beams, latency on gNB tracking UL beams, overhead for DL beam management, and power consumption of the UE.

For other RS types, e.g. CSI-RS for CSI acquisition and non-BM RS, there is no need to introduce too many QCL chain different from Rel-15/Rel-16 to complicate the TCI framework. So these RS types are not supported as source RS types.

Support SSB, SRS for beam management as source RS types for DL QCL-TypeD.

CSI-RS for CSI, non-BM RS are not supported as source RS types in Rel-17 TCI framework.

Agreement On Rel.17 unified TCI framework, decide by RAN1#104bis-e: - Whether DL or, if applicable, joint TCI also applies to the following signals. If not, FFS any other enhancement over Rel.15/16: - CSI-RS resources for CSI - Some CSI-RS resources for BM, if so, which ones (e.g. aperiodic, repetition ‘ON’) - CSI-RS for tracking - Whether UL or, if applicable, joint TCI also applies to the following signals - Some SRS resources or resource sets for BM

For CSI-RS and SRS for beam management, the joint/separate TCI state cannot be applied since they are used for beam tracking and refinement through beam sweeping. For CSI-RS for CSI acquisition, the QCL assumption can be based on indicated joint TCI state or DL separate TCI state for CSI measurement. Otherwise, the UE needs to maintain CSI measurement for multiple activated TCI states to increase overhead. For CSI-RS for tracking, UE can track the source RSs in the activated TCI states to reduce latency for beam switching.

The joint/separate DL TCI state is applied to CSI-RS for CSI acquisition.

The joint/separate TCI state is not used for beam management RS and for CSI-RS for tracking.

Agreement On Rel.17 unified TCI framework, by RAN1#104bis-e, down select or modify at least one from the following alternatives: - Alt1. A UE can be dynamically indicated with either joint DL/UL TCI or separate DL/UL TCI - Details on dynamic indication are FFS - FFS: UE capability for the support of joint DL/UL TCI and/or separate DL/UL TCI - Alt2A. A UE can be configured with either joint DL/UL TCI or separate DL/UL TCI via RRC signaling - Alt2B. A UE can be configured with either joint DL/UL TCI, separate DL/UL TCI, or both via RRC signaling - Alt3. A UE can be configured with either joint DL/UL TCI or separate DL/UL TCI via MAC CE signaling - Details on how this is signaled in relation to TCI activation are FFS

In RAN1 #103-e meeting, both DCI based and MAC CE based TCI state indication are supported. In order to enable/switch a TCI state indication scheme, RRC parameters can be used. For MAC CE based TCI state indication scheme, a MAC CE can be used for joint or separate TCI state update. For joint TCI state, one TCI state is indicated via a MAC CE. For separate TCI state, two TCI states for DL and UL are indicated via a MAC CE.

Use RRC parameters to enable the DCI based TCI state indication or MAC CE based TCI state indication.

In Rel-17 both joint TCI state indication and separate TCI state indication are supported. In order to reduce the switching latency of two TCI state indication in different cases, MAC CE can be used to switch the joint or separate TCI state indication. For example, a joint TCI state is used for one TRP and a UE panel. When MPE event is declared by UE, if the UL beam needs to be switched to another panel, MPE report should be triggered and transmitted to gNB. RAN2 spec already defines PHR MAC CE to indicate MPE related information. Thus the signaling to switch between joint or separate TCI state indication should also be based on MAC CE.

For DCI based TCI state indication, similarly to Rel-15/Rel-16, in order to increase the flexibility of beam indication and to reduce the signaling overhead of TCI field in DCI, MAC CEs can be used to activate the mapping between TCI codepoint and TCI states for joint/separate TCI state indication including single/multi-TRP cases. And the existing TCI field in DCI for beam indication is reused. For example, one TCI codepoint is associated with a joint TCI state or with two separate TCI states for DL and UL, where the switching between joint and separate TCI is implicitly dynamically indicated by existing 3-bit TCI field in DCI. Furthermore, in case of multi-TRP, the TCI state field in MAC CE can be extended to include the joint or separate TCI states corresponding to different TRP, e.g. for single DCI the MAC CE is used to map up to 8 combinations of one or two TCI states for joint TCI state or two or four TCI states for separate TCI state, for multi-DCI CORESET pool ID is indicated to map the activated TCI states and the codepoint for the specific TRP.

Reuse existing TCI field in DCI for beam indication, support Alt3 or Alt1+Alt3

Use MAC CEs to activate the mapping between TCI codepoint and joint/separate TCI states for beam indication for single/multi-TRP cases.

Use MAC CE to switch joint or separate TCI state indication based on UE capability.

Power control parameters

Agreement On Rel.17 unified TCI framework: - Select at least one of the following alternatives by RAN1#104bis-e for path-loss measurement (PL-RS): - Alt1. PL-RS can be included in UL TCI state or (if applicable) joint TCI state. - FFS: Whether it is always included or not. If not included, PL-RS is the periodic DL-RS used as a source RS for determining spatial TX filter or the PL RS used for the UL RS in UL or (if applicable) joint TCI state. - Alt2. PL-RS can be associated with (but not included in) UL TCI state or (if applicable) joint TCI state - FFS: Exact association mechanism - FFS: Whether it is always associated or not. If not associated, PL-RS is the periodic DL-RS used as a source RS for determining spatial TX filter or the PL RS used for the UL RS in UL or (if applicable) joint TCI state - Alt3. The periodic DL-RS used as a source RS for determining spatial TX filter can be used as PL-RS. In case the periodic DL-RS used as a source RS for determining spatial TX filter is not used as PL-RS, reuse Rel.16 procedure with the same signaling structure (MAC CE+SRI field in UL-related DCI) to indicate PL-RS for UL transmission with minimum enhancement (e.g. pertaining to the use for PUCCH, or using default PL-RS) - PL-RS is not additionally configured in or associated to UL TCI state or (if applicable) joint TCI state - Alt4. UE calculates path-loss based on periodic DL RS configured as the source RS or a periodic QCL-Type-D/spatialRelationInfo source of the source RS in UL TCI state or (if applicable) joint TCI state - FFS: Whether UE can calculate path-loss based on DL periodic RS for path-loss calculation for UL RS in the UL TCI - FFS: Application time of PL-RS - NOTE: As in Rel-16, a UE does not expect to simultaneously maintain more than four path-loss estimates per serving cell for all PUSCH/PUCCH/SRS transmissions - FFS: investigate the condition(s) agreed in Rel-17 and, if needed, study whether a UE can simultaneously maintain more than four path-loss estimates Agreement On the setting of UL PC parameters except for PL-RS (P0, alpha, closed loop index) for Rel.17 unified TCI framework: - The setting of (P0, alpha, closed loop index) is at least associated with UL channel or UL RS - Select or modify from one of the following alternatives by RAN1#104bis-e for PUCCH, PUSCH, and SRS separately: - Alt1. The setting of (P0, alpha, closed loop index) is also associated with UL or (if applicable) joint TCI state - Alt2. The setting of (P0, alpha, closed loop index) is included with UL or (if applicable) joint TCI state - Alt3. The setting of (P0, alpha, closed loop index) is neither associated with nor included in UL or (if applicable) joint TCI state - Alt4. The setting of (P0, alpha, closed loop index) is determined as in Rel-16 without enhancement

In Rel-16 default beam mode, PLRS is associated with updated beams. Such functionality could also be used for joint/separate TCI state update. Considering the matching between the pathloss measurement and the beam indication and reducing overhead of PLRS configuration, for power control in Rel-17 TCI framework, the periodic DL RS used as a source RS for determining spatial Tx filter can be used as PLRS. The existing restriction on the number of PLRS can be reused, i.e. the UE does not expect to simultaneously maintain more than four path-loss estimates per serving cell for all PUSCH/PUCCH/SRS transmissions. So the periodic DL RS from the configured PLRSs can be used as the source RS in TCI state.

In case the source RS in TCI state is not periodic DL RS, e.g. SRS resource, in order to reduce the impact on current spec, the Rel-16 procedure with the same signaling structure can be reused to indicate PLRS for UL transmission with minimum enhancement. For PUCCH, the PLRS in PUCCH-SpatialRelationInfo is reused with the IE referenceSignal changed to optional. Then the beam indication is determined based on the TCI state but not PUCCH-SpatialRelationInfo. For PUSCH and SRS, the Rel-16 signaling structure, e.g. MAC CE + SRI field in UL-related DCI, is used to indicate PLRS.

Other PC parameters can also be determined by using existing scheme. If the other PC parameters are associated with or included in TCI state, this means that the PC parameters are same for multiple channels using the same TCI state, which violates the configuration of PC parameters at least associated with UL channels. SRI in UL-related DCI is associated with TPMI and power control parameters. The major purpose of unified TCI framework is used to update beams. For the functionality for port determination and power control indication, legacy solution based on SRI is flexible enough and thus should not be touched.

Support Alt3 with modified version, the periodic DL-RS used as a source RS for determining spatial TX filter can be used as PL-RS. In case the source RS for determining spatial TX filter is not periodic DL-RS, reuse Rel.16 procedure with the same signaling structure (MAC CE+SRI field in UL-related DCI) to indicate PL-RS for UL transmission with minimum enhancement.

For PUCCH, the referenceSignal in PUCCH-SpatialRelationInfo can be changed to optional.

For PUSCH/SRS, reuse Rel.16 procedure with the same signaling structure (MAC CE+SRI field in UL-related DCI) for minimum impact on SRI functionality.

PL-RS is not additionally configured in or associated to UL TCI state or (if applicable) joint TCI state.

For other PC parameters, support Alt4, the setting of (P0, alpha, closed loop index) is determined as in Rel-16 with similar enhancement as for PLRS.

CA case

Agreement On Rel-17 unified TCI framework, support common TCI state ID update and activation to provide common QCL information and/or common UL TX spatial filter(s) across a set of configured CCs: - The above applies to intra-band CA - The above applies to joint DL/UL and separate DL/UL beam indications - Just as Rel.16, the RS in the TCI state that provides QCL-TypeA [or QCL-TypeB] shall be in the same CC as the target channel or RS - The common TCI state ID implies that the same/single RS determined according to the TCI state(s) indicated by a common TCI state ID is used to provide QCL Type-D indication and to determine UL TX spatial filter across the set of configured CCs - FFS: The above also applies to inter-band CA - FFS: TCI state pool for CA - Opt-1: sharing a single RRC TCI state pool for the set of configured CCs, e.g., cell-group TCI state pool, or reuse TCI state pool for PDSCH in a reference cell; A CC ID for QCL-Type A RS is absent in a TCI state, and the CC ID for QCL-Type A RS is determined according to a target CC of the TCI state. - FFS: Whether it is possible that a single TCI state in the pool includes all source RSs from different CCs - Opt-2: configuring RRC TCI state pool per individual CC - FFS: Whether the Rel-17 common beam update across multiple CCs applies to beam indication for single channel (e.g. PDSCH only, single CORESET), a subset of channels, or all channels

To reduce signaling overhead, Rel-16 introduced the beam update across CCs by a MAC CE including simultaneous TCI state update for CORESETs for a set of CCs/BWPs, simultaneous activation of a set of TCI state IDs for PDSCH for a set of CCs/BWPs, simultaneous spatial relation update for SP-/AP-SRS resources for a set of CCs/BWPs, where the applicable list of CCs is indicated by RRC.

In RAN1 #103-e meeting, CA case is also considered in Rel-17 unified TCI framework. The joint/separate TCI state ID update and activation is supported to provide common QCL information and/or common UL Tx spatial filter(s) across a set of configured CCs. Although better flexibility can be achieved by configuring TCI state pool per individual CC, considering the overhead reduction and the similarity of the beam link propagation characteristics on multi-CC in CA case, a set of configured CCs can share a single TCI state pool.

In TS 38.331, higher layer parameter CellGroupConfig is used to configure an MCG or SCG. In this IE, simultaneousTCI-UpdateList1 and simultaneousTCI-UpdateList2 are included to configure the list of serving cells which can be updated simultaneously for TCI relation. Similarly, for CA case of common beam, the single cell-group TCI state pool and the corresponding CC list can be configured in CellGroupConfig, where it is allowed to include a single CC in the CC list to unify the pool design.

Support Opt-1, sharing a single TCI state pool for the set of configured CCs, e.g., cell-group TCI state pool.

RAN1 #103-e meeting already supports that the common TCI state ID implies that the same/single RS determined according to the TCI state(s) indicated by a common TCI state ID is used to provide QCL Type-D indication and to determine UL TX spatial filter across the set of configured CCs. We prefer not to revert the agreement for single RS as QCL-TypeD in CA case. If separate QCL-TypeD RSs are determined from the common TCI state ID for each of the CCs and the determined QCL-TypeD RSs for the set of CCs are further associated with a same QCL-TypeD RS, the UE should track multiple QCL-TypeD RSs on all CCs which increases the UE overhead and power assumption.  To relax the restriction on the QCL-TypeD RS configuration, e.g. same TRS with QCL-TypeA RS or CSI-RS with repetition, the SSB or TRS different from QCL-TypeA RS can be used as the single QCL-TypeD RS, e.g. the same QCL-TypeD RS is configured on a reference CC and applied to the set of configured CCs to guarantee the same spatial TRx parameters.

Single RS, e.g. SSB/different TRS/CSI-RS with repetition, determined according to the TCI state(s) indicated by a common TCI state ID is used to provide QCL Type-D indication and to determine UL TX spatial filter across the set of configured CCs.

For inter-band CA case, it is necessary to consider whether the single TCI state pool and common TCI state update are applicable to different band, where the frequency characteristics on two band are different. But for some case, such as for adjacent band and co-located sites, the common TCI state ID update can be applied, where the TCI state ID is associated with two RS for different band.

In RAN4, CBM (common beam management) and IBM (independent beam management) are discussed for UE architecture. For CBM operation, the same phase shifter configuration is used at UE and the beam measurement results for PCC are used on SCC to reduce the overhead of beam management. When the optimal Tx beam on PCC is selected based on the reference signal in beam report on PCC, the beam on SCC may squint away from the target direction due to separation in frequency from the reference signal, which degrade the performance on SCC. It can be considered to modify the Tx beam on SCC based on the PCC Tx beam to improve performance on SCC, where two Tx beams on PCC and SCC point to the same Rx beam at UE.

SLS evaluation performance is provided for different SCC Tx beam determination in Figure 3. The simulation setup consists of 7 sites with Euclidean distance wrap-around based on a homogeneous network, each site has 3 sectors. In each sector there are 5 users. The scenario considers a network ISD of 500 m. Three different types of UE antennas are considered, 2x2/1x8/1x16 elements and the gNB antennas consist of 1 panel with 16x8 elements per sector, 3 sectors per site.

The gNB adopts co-located deployment, which means that gNB of different bands are deployed in the same location. The frequency of PCC is 24.25 GHz, SCC is 43.5 GHz, which is widest span of FR2 currently. The simulation result uses RSRP as the metric.

RSRP gain from SCC choose optimal Tx beam

The results show that when SCC uses its own best Tx beam with CBM, the RSRP gain compared to the best Tx beam configured directly with PCC. It can be seen that, regardless of the antenna configuration, the RSRP of about half of the users has been improved, even up to 14dB. This is because when the frequency difference increases, the pattern of SCC with CBM is distorted, and the best Tx beam of PCC is no longer suitable for SCC. The reason why the remaining UE gain is 0 is related to the number of Tx beam of the gNB. If the Tx beam is relatively sparse, even though the UE’s pattern is distorted with CBM, it is difficult for the UE to select a better Tx beam.

For UEs operating in CBM mode, allowing SCC to select the best Tx beam has a significant gain in mitigating CBM performance degradation.

Support the joint/separate TCI state ID update and activation across a set of configured CCs to be applied to inter-band CA.

Consider CBM operation to improve performance for inter-band CA case.

TCI state indication signaling

Agreement On beam indication signaling medium to support joint or separate DL/UL beam indication in Rel.17 unified TCI framework: - Support L1-based beam indication using at least UE-specific (unicast) DCI to indicate joint or separate DL/UL beam indication from the active TCI states - The existing DCI formats 1_1 and 1_2 are reused for beam indication - Support a mechanism for UE to acknowledge successful decoding of beam indication - The ACK/NAK of the PDSCH scheduled by the DCI carrying the beam indication can be used as an ACK also for the DCI - FFS: Whether any additional specification support is needed - Support activation of one or more TCI states via MAC CE analogous to Rel.15/16: - At least for the single activated TCI state, the activated TCI state is applied - The content for the MAC CE is determined based on the outcome of issue 1 - FFS: If supported, default TCI state when more than one TCI states are activated by MAC CE - Note: There is no implications on the support of single TRP or multi-TRP - FFS: Additional enhancement such as L1-based beam indication with group-common DCI - FFS: Whether the Rel.17 beam indication can also apply to beam indication for single channel (e.g. PDSCH only, single CORESET) or a subset of channels - FFS: Additional details on extending the support of L1-based beam indication when separate UL (from DL) common beam indication is configured Agreement On the Rel.17 DCI -based beam indication, in RAN1#104bis-e, down-select at least one of the following alternatives regarding the support of DCI format(s) for beam indication in addition to the agreed DCI formats 1_1/1_2 with DL assignment (in RAN1#103-e): - Alt0: No additional DCI format is supported - Alt1: DCI formats 1_1 and 1_2 without DL assignment, applicable for joint TCI as well as separate DL /UL TCI - Support DCI acknowledgment mechanism, e.g. based on SPS PDSCH release, based on triggered SRS , based on DCI indicating SCell dormancy - FFS : How to identify DCI formats 1_1/1_2 used for beam indication only (not for scheduling a PDSCH reception, not indicating a SPS PDSCH release, or not indicating SCell dormancy), considering impacts on PDCCH coverage and scheduling mechanism - FFS : Whether the UE can/shall assume the gNB configured application time is after ACK transmission - Alt2: Dedicated DCI format other than 1_1/1_2 without DL assignment, applicable for joint TCI as well as separate DL /UL TCI - Support DCI acknowledgment mechanism, e.g. based on SPS PDSCH release, based on triggered SRS , based on DCI indicating SCell dormancy - FFS : If the format is based on an existing DCI format, how to identify the DCI format used for beam indication only - FFS : Whether the UE can/shall assume the gNB configured application time is after ACK transmission - Alt3: UL-related DCI formats 0_1/0_2 with UL grant, applicable only for UL-only TCI of separate DL /UL TCI

For DCI based TCI state indication, the existing DCI format 1_1 and 1_2 with DL assignment are reused for beam indication. The mechanism for UE to acknowledge successful decoding of beam indication is supported, where the ACK/NACK of the PDSCH scheduled by the DCI carrying the beam indication can be used as an ACK also for the DCI. Using NACK of the PDSCH scheduled by the DCI carrying the beam indication is problematic as an ACK. For example, for Type-1 HARQ-ACK codebook determination, if a UE reports NACK as HARQ-ACK information value for a corresponding PDSCH, the gNB cannot determine whether PDCCH is missed or PDSCH is incorrectly decoded. For CBG based ACK/NACK where multiple HARQ-ACK bits are multiplexed, if there is at least one ACK, the at least one ACK can be used as the ACK of beam indication DCI, and new rules may also need to be defined for application of the beam in the DCI.

Using NACK of the PDSCH scheduled by the DCI carrying the beam indication is problematic as an ACK.

Agreement On the beam application time for Rel.17 DCI-based beam indication, the beam application time can be configured by the gNB based on UE capability - Support a UE capability for the minimum value of beam application time - FFS: the exact minimum values of beam application time supported by UE - FFS: whether existing UE capability can be reused as this UE capability. - FFS: whether different beam application time values are supported for uplink and downlink - FFS: whether UE capability needs to be introduced for the maximum value of beam application time - FFS: the reference for defining the UE capability (e.g. from DCI reception or ACK transmission) - FFS: whether a UE is allowed to report more than 1 values in case of MPUE - FFS: the application time when DCI and applied channel(s) are on different CCs with same/different SCS(s)s Agreement On Rel.17 DCI-based beam indication, regarding application time of the beam indication: if beam indication is successfully received and the newly indicated beam in the beam indication is different from the previously indicated beam, down-select (no later than RAN1#105-e) one from the following. No other alternatives will be considered: - Alt1: the first slot that is at least X ms or Y symbols after the [first/last] symbol of the DCI with the joint or separate DL/UL beam indication - Alt2A: the first slot that is at least X ms or Y symbols after the [first/last] symbol of the acknowledgment of the joint or separate DL/UL beam indication - Alt 2B: the first slot that is at least X ms or Y symbols after the [first/last] symbol of the acknowledgment of the joint or separate DL/UL beam indication, except that the (new) TCI state update can be applied to the PDSCH, if it exists, (scheduled by the beam indication DCI) and corresponding ACK transmission (provided that the time offset between the DCI and the scheduled PDSCH exceed the threshold, analogous to Rel.15/16) - Alt2C: Support both Alt1 and Alt2A, and introduce a UE capability that indicates the support of Alt1 or Alt2A - Alt3: the first slot that is at least X1 ms or Y1 symbols after the [first/last] symbol of the DCI with beam indication and X2 ms or Y2 symbols after the [first/last] symbol of the acknowledgment of the beam indication - FFS: whether any existing timing defined for DCI based TCI/spatial relation update can be used for X/Y

For DCI based TCI state indication, the beam application time is considered in RAN1 #103-e and #104-e meetings. We prefer the beam application time is the first slot that is at least X ms or Y symbols after last symbol of the DCI with the joint or separate DL/UL TCI state indication. For PDSCH beam indication, Alt1 is already supported in Rel-15/Rel-16 and the rule of beam application time can be reused in Rel-17 for joint/separate TCI state. The PDCCH performance carrying beam indication DCI is robust enough to avoid the confusion between DCI loss and PDSCH error. The gNB decides whether the beam application time is before or after the HARQ-ACK. All types of channels apply the same beam application time to reduce the impact on spec and UE cost.

Support Alt1: if beam indication is successfully received and the newly indicated beam in the beam indication is different from the previously indicated beam, the beam application time is the first slot that is at least X ms or Y symbols after the last symbol of the DCI with the joint or separate DL/UL beam indication.

In order to further ensure the robustness of the PDCCH carrying beam indication DCI and avoid the confusion between DCI loss and PDSCH error, the joint/separate TCI state cannot be applied to the beam indication DCI and the existing beam indication scheme for PDCCH in Rel-15/Rel-16 is reused. For example, a MAC CE is used to indicate the joint/separate TCI state for a group including PDCCH carrying beam indication DCI and the associated channels, e.g. the scheduled PDSCH/PUSCH and the PUCCH triggered by the PDCCH.

With M>1 and N>1, MAC CE can be used to indicate TCI state for a group including PDCCH carrying beam indication DCI and the PUSCH/PDSCH scheduled/activated and PUCCH transmission triggered by PDCCH.

On the beam application time for Rel-17 DCI-based beam indication, the beam application time can be configured by the gNB based on UE capability, where the minimum value of beam application time is reported by UE. In Rel-15/Rel-16 timeDurationForQCL in UE capability can be reused for Rel-17 joint/separate TCI timing to define minimum number of OFDM symbols required by the UE to perform PDCCH carrying beam indication DCI and applying spatial QCL information for the channels and RSs, where the values per SCS are indicated. For a UE, the beam application time is defined to consider the DCI decoding and Tx/Rx spatial filter adjustment for indicated QCL information. So for downlink and uplink the same beam application time can be supported. If the PDCCH carrying beam indication DCI is used to indicate the joint/separate TCI state for cross carrier case with same or different SCSs, the existing scheme for PDSCH reception preparation time with cross carrier scheduling can be reused, e.g. an additional timing delay is added to the timeDurationForQCL. For MPUE, in general, not all UE panels are activated for power saving, e.g. only single active panel is used for DL and UL transmission. The panel activation delay should be considered to align the panel/beam between gNB and UE.

For beam application time in UE capability,

reuse the values defined in existing UE capability timeDurationForQCL,

in cross carrier case, reuse existing scheme, i.e. additional timing delay is added.

In Rel-15/Rel-16, most of the default TCI state/spatial relation/pathloss RS are determined according to the QCL assumption of a CORESET, e.g. the default TCI state of PDSCH when the scheduling offset is less than the threshold timeDurationForQCL, the default TCI state of PDSCH scheduled by a DCI format not having the TCI field present. In Rel-17 the current scheme of default beam can be reused, where the PDSCH default beam depends on what beam is used for the CORESET, where PDSCH and the corresponding CORESET belong to same group. If a common beam is applied to this group, the default beam of PDSCH is also following the indicated common beam. This principle also applies to the default beam of the other channel/RS, e.g. aperiodic CSI-RS, PUCCH, PUSCH, SRS.

The default beam of PDSCH before indication/activation of new TCI state follows one of the TCI states previously indicated.

Inter-cell mobility

The following agreement was achieved for L1/L2 centric mobility in RAN1 #104-e meeting.

Agreement On Rel.17 multi beam measurement/reporting enhancements for L1/L2-centric inter-cell mobility and inter-cell mTRP: - A quality of up to K beams associated at least with non-serving cell(s) can be reported in a single CSI reporting instance - For each beam, the UE can report at least: (1) a Measured RS Indicator, and (2) a Beam Metric associated with the Measured RS Indicator - FFS: Maximum value of K - FFS: If K is fixed, configured, reported by UE capability, or dynamically selected - FFS: The type of beam metric (e.g. L1-RSRP, L3-RSRP, or hybrid L1/L3-RSRP) and related measurement behavior - FFS: Whether or not beam reporting associated with non-serving cell(s) can be mixed with that with serving-cell in one reporting instance At the end of RAN1#104-e, send an LS to RAN2 with all the RAN1-related inter-cell mobility agreements done so far during Rel17. LS is endorsed in R1- 2102209. Agreement On Rel.17 multi beam measurement/reporting enhancements for L1/L2-centric inter-cell mobility and inter-cell mTRP: - Rel.15 L1-RSRP is used as reporting quantity for measurement and reporting of non-serving-cell(s) - Support SSB as a measurement RS for L1/L2-centric inter-cell mobility and inter-cell mTRP, and Rel.15 SS-RSRP calculated from SSB of non-serving cell(s) - FFS: Whether the measurement for SS-RSRP is limited within SMTC - FFS: Detailed reporting method, e.g. via including existing L1-RSRP report, UE-initiated report etc. - FFS: Whether or not to support CSI-RS (for e.g. mobility and/or tracking) of non-serving cell(s) as a measurement RS for L1/L2-centric inter-cell mobility and inter-cell mTRP. If the support of CSI-RS (for e.g. mobility and/or tracking) of non-serving cell(s) as a measurement RS for L1/L2-centric inter-cell mobility and inter-cell mTRP is confirmed, Rel.15 CSI-RSRP is also supported - Whether the support applies to CSI-RS with or without QCL source, or both - FFS: The number of non-serving cell(s) for measurement/reporting - FFS: time behavior of the reporting, i.e. periodic, semi-persistent, aperiodic, or UE-initiated - FFS: If other reporting quantities are supported, e.g. L3-RSRP, hybrid L1/L3-RSRP - FFS: Dynamic activation/deactivation/selection of the beam measurement on the RS(s) associated with non-serving cell(s) via MAC CE - FFS: Timing assumption (e.g. time of arrival and time of the measurement) for measurement of non-serving cell RS measurement Agreement On Rel.17 enhancements for L1/L2-centric inter-cell mobility, - Discuss whether to support at least the source RS types already agreed for intra-cell mobility for the purpose of referencing to non-serving cell(s). Note: This implies the following source RS(s): - CSI-RS for BM configured for non-serving cell(s) for DL QCL and UL TX spatial references - CSI-RS for tracking (TRS) configured for non-serving cell(s) for DL QCL and UL TX spatial references - SSB configured for non-serving cell(s) for UL TX spatial references - SRS for BM configured for non-serving cell(s) for UL TX spatial references - FFS: whether to support CSI-RS for mobility - FFS: whether to support other source RS(s) potentially agreed later for intra-cell mobility - FFS: whether to support CSI-RS for BM and tracking configured for non-serving cell(s) and without non-serving cell SSB as QCL-TypeD source - Send an LS to RAN2 on TCI state update (beam indication) using source RS configured for non-serving cell(s) for DL reception and UL transmission. The following topics are considered for the LS: - RRC configuration issues - Serving cell issues - C-RNTI issues - Issues related to CU-DU split - Inter-band CA issues - Inter-frequency issues

Inter-cell beam measurement

It has already been agreed to support L1-RSRP based measurement. The simplest L1 inter-cell beam measurement is to reuse the results of current L3 inter-cell beam management. That is, the network connects L3 inter-cell beam management with L1 beam report, by configuring serval cell IDs or RSs of neighboring cells in L1 beam management or other methods, and then the L1 filtered beam qualities in L3 procedure could be reported to the network using current L1 beam report. No new measurement behaviour is defined in this case, and the only difference is that the network could obtain the L1 beam qualities of neighboring cells now.

The above method focuses on the latency reduction of L3 inter-cell beam management, and has very small impact on current spec and is convenient for UE to implement. Current L3 inter-cell beam measurement is restricted to SMTC. SMTC is a window of up to 5ms where the measurements of SSBs are to be performed. For intra-frequency connected mode measurements, up to two measurement window periodicities may be configured. A single measurement window offset and measurement duration are configured per intra-frequency measurement object. By defining SMTC, the inter-cell beam measurement is put together in a short time.

By limiting measurement within SMTC, there are at least the following benefits.

- UE does not assume stringent timing assumption between the measured cell and serving cell with current L3 inter-cell management behavior. UE would freely search for the best cell.

- Large number of cells and SSBs can be measured with current L3 inter-cell beam management behaviour. For mobility purpose, the gNB would be interested in the performance of a few number of SSBs of each neighboring cell, as long as the beam qualities of which are good enough for handover or inter-cell multi-TRP. By limiting within SMTC, UE would also freely search for the best without limitations.

Based on above understanding, L1-RSRP based inter-cell measurement can be supported based on measurement within SMTC window

There are also some disadvantages for L1-RSRP measurement limited within SMTC:

- To further reduce the latency, short period for SMTC needs to be configured. However, this would largely impact the transmission and reception of its serving cell. On FR2, the UE may not be able to receive or transmit other type of channels or signals in SMTC, according to current scheduling availability of UE performing measurements. For example, during SS-RSRP or SS-SINR measurement on an FR2 intra-frequency cell, the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration. If the network configures SMTC with short period, the available time resources for the transmissions of serving cells are largely reduced.

- Thus it is also necessary to support a kind of measurement based on L1-RSRP without any further restrictions within SMTC.

- Support L1-RSRP inter-cell measurement with or without limitation to within SMTC window based on configurations.

  - For cases where the measurement is within the SMTC, timing assumption for L1-RSRP based inter-cell measurement follows current L3 RRM measurement.

For inter-cell measurement, scheduling availabilities of UE performing inter-cell beam measurements have been defined in RAN4 for different scenarios. On FR2, the UE is not expected to received or transmit the signals of its serving cell on the SSB symbols to be measured, as well as 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration. This is because the analog beam of UE is unclear now and the UE is not ready for the transmission for its serving cell. The extra 2 data symbols are left for beam switch and overcoming the timing difference between two cells. These problems still exist in L1 inter-cell beam measurement and the study of scheduling restrictions is needed here.

- Clarify the scheduling restrictions for L1 inter-cell beam measurement.

Inter-cell beam report

For the case of inter-cell beam measurement mentioned above, the SSB/CSI-RS from neighboring cells can be measured in layer1 regardless of the restriction of SMTC to accelerate the beam management procedure. The UE needs to inform the gNB the measurement results including the RS resources from neighboring cells through the beam report. Then the inter-cell mobility can involve applying the RS resource from the neighboring cell as the QCL source for the inter-cell beam management.

In Rel-15/Rel-16, the L1-RSRP/L1-SINR report is used to inform the gNB the best beams indicated by SSBRI/CRI. In CSI report setting for L1-RSRP/L1-SINR report, the associated RS resource setting for beam management is provided. For inter-cell beam report, the PCI can be introduced to indicate the relationship between the RS resource and the source cell or target cell. Since the UE can measure SSB or CSI-RS for mobility during the inter-cell beam management, the type of CSI-RS resource (whether for mobility) can also be known in the configuration of CSI-RS resource setting associated the CSI report setting.

Whether PCI is carried in the inter-cell beam report depends on the number of cells corresponding to RS resources. For example, if one PCI is provided by gNB in CSI report setting, the UE only needs to measure the SSBs corresponding to the PCI and report the selected SSBRI(s) without PCI in beam report. Besides, the UE also can measure the SSBs corresponding to the PCI(s) obtained by blind detection within SMTC.

The PCI in resource setting can be (re-)configured to indicate/update the relationship between the RS resource and the source cell or target cell. There may be a lot of RS resources in case of measurement for neighboring cells. If every possible configurations of report setting are provided to meet the measurement requirement for multiple neighboring cells, the RRC signaling overhead and the resource cost will be heavy. Therefore, in order to speed up the inter-cell management and reduce the overhead, PCI information in CSI report setting/resource setting can be updated dynamically.

Support to enhance CSI reporting/resource setting configuration to include non-serving cell information for inter-cell beam management.

Support to use MAC CE to update the non-serving cell information.

In L1 report, timing offset between source cell and target cell can be carried through positive or negative quantization value. Based on the L1 report, the gNB can determine whether the scheduling restriction applies to the symbol(s) before or after the RS symbols of the neighboring cell. In addition, for inter-cell multi-TRP case, the solution of L1 report carrying timing offset of different cells can assist the gNB to select the appropriate TRPs for simultaneous transmission.

Support to report timing offset between different cells in L1/L2 report.

Enhanced beam indication and power control

For inter-cell mobility, inter-cell multi-TRP is an important approach that could be used to improve the user experience for continuous downlink and uplink transmission. Before the gNB configures UE to handover to the target cell, UE could be connected to the target cell not only for measurement but also for data transmission. Such early transmission from the target cell would enable seamless handover from the source cell to target cell without any interruption.

Based on the inter-cell beam report, the SSBRIs/CRIs corresponding to different PCIs through L1 measurement are known by gNB. Then the gNB can apply the RS resource from the neighboring cell as the QCL source or the source of spatial relation or pathloss RS. It can be noticed that SRS for positioning already supports spatial relation configured/activated targeting another PCI. Similar configuration could also be enhanced for other channels/RSs.

Support to reuse the same way of associating non-serving cell information (at least including PCI information) with TCI for L1/L2 centric mobility and inter-cell MTRP operation.

There are different modes for such operations.

- One mode is with unified TCI framework, the handover procedure is involved. All the channels/RS are switched from the source cell to the target cell

- Another mode is allowing transmission to and reception from the target cell but the RRC anchor is still in the original serving cell.

RAN1 has already sent LS to RAN2 regarding whether changing the serving cell or not changing the serving cell during the L1/L2 mobility procedure. LS feedback from RAN2 would help RAN1 to better understand this issue. Before LS feedback, RAN1 should allow such possibility of transmitting and reception from target cell for some dedicated channels/RS.

Support inter-cell beam indication for UE dedicated transmission and reception to smooth mobility between cells.

The beam indication does not trigger mobility procedure, only involving TRx from a different cell.

Support Rel-17 unified framework to indicate beams to channels/RSs other than e.g. COERSET #0 and TRx initiated through CORESET #0.

Beam selection for multi-panel UE

Conclusion On Rel.17 enhancements to facilitate UL beam selection for MP-UE, the following terms are used at least for the purpose of discussion: - ‘Panel activation’ (at least for DL/UL measurement): activating L out of P available UE panel(s) at least for the purpose of DL and UL beam measurements (e.g. reception of DL measurement RS, transmission of SRS) - ‘Panel selection’ (for UL transmission): selecting 1 out of L activated UE panel(s) for the purpose of UL transmission - Note: UE-initiated panel activation and selection have been agreed in RAN1#103-e Agreement On Rel.17 enhancement for facilitating fast uplink panel selection, - Rel.17 TCI state update (based on MAC CE + DCI along with the necessary TCI state activation, or MAC CE only) can be used for UE UL panel selection: - FFS : Whether specification support for this feature is necessary and if so the details of such spec support, e.g. - Additional spec support in TCI state definition to accommodate UL panel - UE reporting to facilitate UL panel selection - UE reporting, e.g. panel-specific report, including UE -panel state, e.g. inactive, active for DL /UL measurement, active for DL reception only, active for UL transmission, or other combination(s) of UE -panel states - Support for linking or association of UE panels with CSI-RS/SSB resources or resource sets, SRS resource sets, and/or PUCCH resource groups, etc. Agreement On Rel.17 enhancement for facilitating fast uplink panel selection, for discussion purpose, a panel entity corresponds to one or more RS resources: - For CSI/beam reporting, the RS resource is an RS associated with measurement and/or reporting - For beam indication, the RS resource is a source RS for UL TX spatial filter information - Note: For one RS resource, the corresponding panel entity may vary and is controlled by the UE, and whether/how to maintain a common understanding between gNB and UE can be further discussed/decided - Note: The above does not preclude possibility that an RS resource can be mapped to multiple panels - Note: The one or more RS resources may correspond to one or more RS resource set(s) depending on further discussion/decision - Note: Specification should not be designed in such a way that the UE is required to disclose its antenna implementation

In RAN1 #103-e meeting, the use cases are assumed for facilitating fast uplink panel selection which include MPE mitigation, UE power saving, UL interference management, different configurations across panels and UL mTRP. In RAN1 #104-e meeting, the terms ‘Panel activation’, ‘Panel selection’ and panel entity are defined for discussion purpose. In 3GPP Rel-16 the enhancements for enabling panel-specific UL beam selection were investigated, in RAN1 meetings three MPUE categories are proposed as follows.

MPUE-Assumption1: Multiple panels are implemented on a UE and only one panel can be activated at a time, with panel switching/activation delay of [X] ms

MPUE-Assumption2: Multiple panels are implemented on a UE and multiple panels can be activated at a time and one or more panels can be used for transmission

MPUE-Assumption3: Multiple panels are implemented on a UE and multiple panels can be activated at a time but only one panel can be used for transmission

But RAN1 cannot agree on the support of at least one of MPUE-Assumption1, MPUE-Assumption2, MPUE-Assumption3, enhancements on panel-specific beam selection for uplink was not supported in Rel-16. In the discussion of the NR FeMIMO scope of Rel-17, due to the concern on RAN4 impact, hardware implementation limitation and possible cross over with other objectives, the WID discussion narrowed down the scope of the assumptions of UE transmission across multiple panels. STxMP (simultaneous transmission across multiple panels), including simultaneous transmission of different UL channels/RSs across multiple panels was not included in the scope.

For MPUE-Assumption3, the power consumption also needs to be considered whether the multiple panels are always activated simultaneously. To save power consumption, UE panel activation/deactivation scheme can be introduced. For example, if the channel conditions are relatively stable, e.g. no deep fading and blockage, or if the UE status does not change, e.g. no movement and rotation, within a time duration one panel at UE would be enough for transmission/reception. And the other panel at UE can be deactivated by the UE to save power consumption, where the panel status is transparent to the network.

In Rel-17 for MPUE, optimize the UE operation with only one activated panel, considering both MPUE-Assumption1 and MPUE-Assumption3.

Panel information is useful for the agreed enhancement scenarios:

- Multi-panel UE may also facilitate multi-TRP operation with per panel beam management procedure. In multi-TRP case, based on each radio link quality, e.g. RSRP of different TRP, the gNB can schedule DL channel transmission from the selected TRP, and the UE switches to the corresponding panel to receive the DL channel or simultaneously receive from multi-TRP.

- In Rel-17 NR FeMIMO scope, for increasing UL coverage including, e.g. mitigating the UL coverage loss due to meeting the MPE (maximum permissible exposure) regulation, the MPE issue may occur on all transmit beams from one panel, therefore, a solution for MPE mitigation may only be performed per panel basis to meet the regulatory requirement for scenarios of interest. So panel switching caused by MPE mitigation should also be considered. Alignment between gNB and UE on panel information is necessary to make the link performance well adapted to different panels.

- For positioning, UE TEG (timing error group) is associated with the transmissions of one or more UL SRS resources for the positioning purpose or one or more DL measurements, which is similar to the definition of UE panel entity. The UE TEG information can facilitate the alignment between gNB and UE for positioning calculation. The scheme for UE Rx TEG report in positioning can be used for reference in MPUE.

In RAN1 #104-e meeting, for panel entity definition, the corresponding panel entity for one RS resource may vary and is controlled by the UE. One possible way is to indicate UE panel information in beam report to inform gNB of the UE panel corresponding to each SSBRI/CRI. Or the panel information is introduced in report configuration/activation to enable beam management and beam report for the specific UE panel. When the gNB schedules transmission and reception of different channel/RS, the joint/separate TCI state in Rel-17 in beam indication can implicitly be used to indicate which UE panel is for channel/RS transmission and reception, which achieves the purpose of the alignment of panel status between gNB and UE. Based on the correspondence between SSBRI/CRI and UE panel in beam report, the UE can select the panel corresponding to the indicated source RS in TCI state for UL transmission.

Even for group based beam report, the SSBRI/CRI group corresponding to different UE panel ID can be carried in report. The gNB selects two Tx beams for each UE panel to realize the simultaneous transmission to increase throughput for multi-TRP and multi-panel case.

UE panel ID can be introduced to assist network and UE’s alignment.

E.g. enhanced beam report/report configuration/report activation including panel ID.

Rel-17 TCI state update can implicitly be used for UL panel selection based on the enhanced beam report.

MPE issue for multi-panel UE

Agreement On Rel.17 enhancements to facilitate MPE mitigation, - On further enhancing the P-MPR report in Rel.16 (already agreed RAN4 framework, including triggering), down select between beam-level and panel-select reporting - On SSBRI(s)/CRI(s) and/or indication of panel selection, focus study on the following: - Reporting of at least SSBRI(s)/CRI(s) to indicate gNB beam(s) that is feasible for UL transmission: additional reporting quantities are FFS - Reporting of at least an indicator associated with a UE ‘panel’ that is feasible for UL transmission: additional reporting quantities are FFS - Note: Just as agreed in RAN1#103-e, the purpose is to assess whether specification is needed or not Agreement On Rel.17 enhancements to facilitate MPE mitigation, decide in RAN1#104bis-e whether to support at least one the following (not necessarily, but can be, in one reporting instance): - {Rel.16 P-MPR based (beam/panel-level)} + {A}, where A is either Opt1A, Opt1B, Opt1C, or Opt1D: - Option 1A: Virtual PHR or a modified version associated with each activated UL TCI or, if applicable, joint TCI - Option 1B: {SSBRI(s)/CRI(s) and/or panel indication} - Option 1C: {SSBRI(s)/CRI(s) and/or panel indication} + virtual PHR or a modified version associated with each of the reported SSBRI(s)/CRI(s) and/or panel indication (if configured) - Option 1D: No additional reporting quantity - {SSBRI(s)/CRI(s) and/or panel indication} + {A}, where A is either Opt2A, Opt2B, Opt2A+ Opt2B, or Option 2C - Option 2A: L1-RSRP [L1-SINR] or a modified version that accounts for MPE effect associated with each of the reported SSBRI(s)/CRI(s) and/or panel indication (if configured) - FFS: How panel-level L1-RSRP [L1-SINR] is reported if L1-RSRP [L1-SINR] is associated with panel - FFS: Whether/how to account for MPE effect in L1-RSRP [L1-SINR] report, e.g. by using scaled L1-RSRP [L1-SINR] - FFS: Whether/how to enhance existing beam reporting format to support Option 2A - Option 2B: Virtual PHR or a modified version associated with each of the reported SSBRI(s)/CRI(s) and/or panel indication (if configured) - Option 2C: No additional reporting quantity

In Rel-16, RAN4 provides some solution for MPE issue by introducing P-MPR report. P-MPR is the maximum output power reduction. The UE shall apply P-MPR to ensure compliance with applicable electromagnetic power density exposure requirements and addressing unwanted emissions/self-defense requirements. Based on P-MPR report the gNB can obtain the real-time status of the UE accurately, and adjust the scheduling scheme and panel/beam selection accordingly. RAN4 has agreed further details on the Rel-16 FR2 MPE enhancement signaling, which is used to mitigate RLF due to sudden and unpredictable large back off caused UE’s actions to ensure compliance with MPE regulation.

In RAN4 discussion, the reported absolute P-MPR values are defined using 2 bits, and the P-MPR event-triggered threshold and a prohibit timer are introduced for the P-MPR reporting. MPE P-MPR reporting is limited to FR2 and is an optional UE capability. This UE capability is applicable to all FR2 UE power classes.

In current RAN2 spec TS38.321, if mpe-Reporting is configured, the MAC CE entity shall trigger MPE reporting when the triggering condition is met. In single entry PHR MAC CE or multiple entry PHR MAC CE, the applied power backoff to meet MPE requirement is indicated, where the measured values of P-MPR levels in dB are included.

In Rel-17 NR FeMIMO scope, the MPE issue mainly refers to mitigating the UL coverage loss due to meeting the MPE regulation by panel-specific UL beam selection. For MPUE, the propagation links between each UE panel and TRP are different, leading to different channel paths and different beamforming gain. Similarly, MPE event for each panel happens independently, while only the panel close to the human body needs power fallback.

In order to reflect the performance difference of each panel/beam accurately, the panel/beam specific P-MPR report can be introduced in Rel-17. When the UE detects by sensors that the current UL panel is close to the human body, the corresponding mechanism is triggered to meet the emission safety. For example, the UE can reduce the transmission power of the current panel, or deactivate the panel with current UL beam, activate another panel. It is noted that in Rel-17 NR FeMIMO WID the MPE issue may occur on all transmit beams from the panel, therefore, a solution for MPE mitigation may only be performed per panel basis to meet the regulatory requirement for scenarios of interest. Complicated environment sensing and MPE event detection with sufficient accuracy at beam level would not be implemented by UE. Furthermore, in the EVM discussion for Rel-17 NR FeMIMO, the panel blockage modeling for MPE only defines panel specific blockage. When MPE occurs, the maximum TX power for the covered panel is reduced by 10dB P-MPR. Rel-17 MPE related enhancement should focus on the support of panel specific P-MPR report.

In general, the magnitude of MPE detection and beam measurement is different at UE. For example, MPE detection is second level, but the beam measurement is millisecond level. Thus P-MPR values for UE panels carried in layer 1 based beam report are unnecessary and will increase the signaling overhead. Similar to RAN2 PHR MAC CE, the MAC CE format can be enhanced to support flexible number of panels to indicate the panel specific power backoff, i.e. introduce the panel specific P-MPR value in the corresponding MAC CE.

After the P-MPR report reception, the gNB can indicate beam measurement on a target panel at UE, e.g. a MAC CE is used to activate the panel ID in report configuration. Then the gNB calculates the beam quantity based on L1-RSRP and P-MPR.

Support Option 1D: P-MPR per panel report in PHR in Rel-17 without additional reporting quantity.

Introduce panel ID and the corresponding P-MPR value in PHR MAC CE.

When the gNB schedules the UE for UL beam selection based on P-MPR report, different UE capabilities and operation modes need to be considered e.g. for the UE supporting single activated panel or multiple activated panels.

For the UE supporting multiple activated panels, when MPE event is declared by UE, if the UL beam needs to be switched to another panel, the optimal DL beam link can remain unchanged for the original panel. In this mode, multiple panels of UE need to be activated simultaneously and used for downlink reception and uplink transmission respectively, which increases the UE power consumption.

We further analyze the performance of UEs operating with single activated panel even when MPE is declared. We provide the uplink and downlink transmission performance, which includes SE comparison for full buffer and UPT comparison for FTP model 1 with RU for baseline Case1 downlink transmission set to 24% for Dense Urban. Both full buffer and FTP1 traffic model, considerable SE/UPT gain can be observed at 5% and 50% SE/UPT, and mean SE/UPT as well. For FTP1, the same packet arrival rate (λ) for Case1 DL is set for Case1 UL and other cases DL&UL. The simulation results are shown in the tables below. Detailed simulation assumptions and probability statistical results for declaring MPE event can be found in Annex. In simulation, two metrics are modeled for panel/beam switching:

Metric 1: a specific RSRP that considers the impact of blockage.

Metric 2: a specific RSRP that considers the impact of blockage and MPE power back-off.

First, the uplink transmission is evaluated where the cases of beam selection are as follows:

Case1 (baseline): when MPE event is declared by UE, the UE continues to use the blocked panel for uplink transmission.

Case2: when MPE event is declared by UE, the UE may switch to another panel for uplink transmission in case of single TRP.

UL performance with full buffer and FTP1 traffic model using metric 1

|  | Dense Urban |  |  | Indoor Hotspot |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Mean SE/UPT | 5%SE/UPT | 50%SE/UPT | Mean SE/UPT | 5%SE/UPT | 50%SE/UPT |
| Case1 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Case2(UL, full buffer) | 20.10% | 97.95% | 67.93% | 4.18% | 51.00% | 2.33% |
| Case2(UL, FTP1) | 11.73% | 332.69% | 90.60% | 8.56% | 78.26% | 3.74% |

UL performance with full buffer and FTP1 traffic model using metric 2

|  | Dense Urban |  |  | Indoor Hotspot |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Mean SE/UPT | 5%SE/UPT | 50%SE/UPT | Mean SE/UPT | 5%SE/UPT | 50%SE/UPT |
| Case1 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Case2(UL, full buffer) | 23.04% | 193.84% | 98.73% | 5.81% | 56.08% | 2.59% |
| Case2(UL, FTP1) | 15.12% | 345.19% | 95.90% | 12.76% | 211.82% | 4.95% |

- As the average SINR for Dense Urban is low, transmission performance loss is nearly 20dB in case of MPE event. This 20dB loss results in poor performance for the throughput for edge users increase several times after panel switching. From the simulation results, when MPE event is declared by UE, if the panel can be switched for uplink beam selection, a significant performance gain is observed compared to the case that UE continues to use the blocked panel. Furthermore, the panel switching with metric 2 can achieve better performance, which makes it more feasible to report the P-MPR caused by MPE event.

Panel switching based on MPE event performs better than panel switching based on purely DL beam report.

Second, the downlink transmission is evaluated where the cases of beam selection are as follows:

Case3(baseline): when MPE event is declared by UE, the UE continues to use the blocked panel for downlink transmission and may switch to another panel for uplink transmission in case of single TRP.

Case4: when MPE event is declared by UE, the UE may switch to another panel for both downlink and uplink transmission in case of single TRP.

- Note：Since the two cases have the same uplink switching panel, the performance is consistent, so we only give the downlink performance comparison

DL performance with full buffer and FTP1 traffic model using metric 2

|  | Dense Urban |  |  | Indoor Hotspot |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Mean SE/UPT | 5%SE/UPT | 50%SE/UPT | Mean SE/UPT | 5%SE/UPT | 50%SE/UPT |
| Case3 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Case4(DL, full buffer) | -1.35% | -0.48% | 0.00% | -3.77% | -16.0 % | 0.32% |
| Case4(DL, FTP1) | -0.32% | -7.52% | 0.00% | -0.91% | -9.26% | -0.19% |

From the simulation results it can be seen if the same panel is switched for both DL beam selection and UL beam selection, the downlink transmission performance has less loss. From the perspective of power saving, it is a good choice to select the DL beam and UL beam on the same panel, which is also a necessary restriction for the UE supporting single activated panel. After the UE declares MPE event and transmits panel specific P-MPR report, based on the tradeoff between the performance of DL beam link and the performance of UL beam link, the gNB selects an optimal UE panel and schedules data transmission. It is noted that the latency of panel switching should be considered for UE supporting single activated panel, which can be satisfied by the scheduling offset between DCI and the scheduled channel.

For panel switching triggered by MPE, when the panel is switched for both downlink reception and uplink transmission, the downlink performance loss is marginal.

The above simulation scenario is single TRP, and we also provide the corresponding simulation results for the multi-TRP scenario. For the multi-TRP scenario, the cases of beam selection are as follows:

Case5(baseline): when MPE event is declared by UE, the UE continues to use the blocked panel for downlink or uplink transmission.

Case6: when MPE event is declared by UE, the UE may switch to another panel for downlink and uplink transmission in case of single TRP.

Case7: when MPE event is declared by UE, the UE may switch to another panel for downlink and uplink transmission in case of multi-TRP.

DL and UL performance with full buffer and FTP1 traffic model

|  | Dense Urban |  |  | Indoor Hotspot |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Mean SE/UPT | 5%SE/UPT | 50%SE/UPT | Mean SE/UPT | 5%SE/UPT | 50%SE/UPT |
| Case5 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Case6(DL, full buffer) | -1.35% | -0.48% | 0.00% | -3.77% | -16.0 % | -3.77% |
| Case7(DL, full buffer) | -0.77% | 0.26% | 0.00% | -2.71% | -6.48% | -1.52% |
| Case6(UL, full buffer) | 23.04% | 193.84% | 98.73% | 5.81% | 56.08% | 2.59% |
| Case7(UL, full buffer) | 26.89% | 267.81% | 106.52% | 9.12% | 72.48% | 2.61% |
| Case6(DL, FTP1) | -0.32% | -7.52% | 0.00% | -0.91% | -9.26% | -0.19% |
| Case7(DL, FTP1) | -0.14% | -3.67% | 0.00% | -0.50% | -4.05% | 0.0% |
| Case6(UL, FTP1) | 15.12% | 345.19% | 95.90% | 12.76% | 211.82% | 4.95% |
| Case7(UL, FTP1) | 18.60% | 517.79% | 98.16% | 16.94% | 340.19% | 6.68% |

We can see when the panel switching is accompanied by TRP switching in multi-TRP scenario to obtain selection diversity gain, the loss of downlink transmission can be further reduced and the gain of uplink transmission can be further increased.

For multi-TRP scenario, when the panel is switched for both downlink reception and uplink transmission, the performance loss of downlink transmission becomes even smaller compared with single-TRP case.

- Based on above observations, switch of activated panels for both downlink reception and uplink transmission is a reasonable implementation for Rel-17 MPE and MPUE enhancement.

For panel switching triggered by MPE event, support single activated panel operation for both DL reception and UL transmission.

For the UE supporting single activated panel, the gNB needs to align the selected panel based on the beam report and P-MPR report, where the panel information is corresponding to the DL beams and UL beams. For UL beam selection on panel basis, the gNB determines the UL quality for the different panels based on the panel specific P-MPR report including P-MPR values and panel IDs. But for DL and UL beam selection, the gNB cannot ensure that the panel with the selected DL beam is the same as that of the selected UL beam without further specification enhancement. For example, the gNB may select a CSI-RS resource measured by first panel at UE as the source RS in TCI state for downlink channel beam indication, but select another CSI-RS resource measured by second panel at UE as the source RS in spatial relation for uplink channel beam indication. So in order to ensure accurate scheduling and panel switching for this UE capability, mechanisms for alignment of DL reception panel and UL transmission panel should be designed to facilitate single activated panel operation.

Consider mechanisms for alignment of DL reception panel and UL transmission panel due to panel switching triggered by MPE to facilitate single activated panel operation.

Enhancement for beam management

Agreement On Rel.17 enhancements based on the unified TCI framework, perform study and, if needed, specify the following: - Group1: Beam management with reduced DL signaling to reduce latency - Group2: Reducing activation delay of TCI states and PL-RSs (including other WGs, e.g. RAN4) - On RAN4-related matters, assessment/study phase can be done in RAN1. If RAN4-based enhancements are found necessary, a LS to RAN4 will be sent (to prepare RAN4 work) Note: Given its dependence on the maturity compared to other issues (1 to 5), when to start the work and how much work is done on issue 6 should depend on the progress on the other issues. Note: Aim for at most one solution for each of the group in Rel-17 to address issue 6

In RAN1 #104-e meeting, some solutions are provided to enhance the beam management. In RAN4 for TCI state switching, the additional delay of first SSB transmission and processing leads to a large TCI activation latency. To reduce the activation delay of TCI states, the RS other than SSB can be considered to speed up the beam switching time. For example, aperiodic TRS can be triggered for UE tracking the source RSs in activated TCI states. To further reduce the tracking overhead, the UE can measure TRS only in the time interval from beam application time and the first SSB.

For TCI state switching, TRS can be used to speed up the beam switching time.

Another area that could be enhanced to reduce latency is for Rel-16 introduced beam update across CCs.  With simultaneous TCI state update for CORESETs for a set of CCs/BWPs, simultaneous activation of a set of TCI state IDs for PDSCH for a set of CCs/BWPs, simultaneous spatial relation update for SP-/AP-SRS resources for a set of CCs/BWPs, the signaling latency can be largely reduced. In Rel-16 the PUCCH spatial relation activation was enhanced as well. If the indicated PUCCH resource by MAC CE is configured as part of a PUCCH group, no other PUCCH resources within the same PUCCH group are indicated in the MAC CE, and the MAC CE applies to all the PUCCH resources in the PUCCH group.

The enhanced PUCCH spatial relation activation is used in a BWP. Furthermore, we can extend this scheme to the case of multiple CCs/BWPs. When a spatial relation is activated/updated for a PUCCH resource/PUCCH resource group by a MAC CE for a set of CCs/BWPs, the spatial relation can be applied to the PUCCH resource/PUCCH resource group with the same resource ID/group ID for all the BWPs in the indicated CCs.

Similar to SRS resource, for SP-/AP-CSI-RS resource the MAC CE-based TCI state update can be considered. The simultaneous TCI state update for SP-/AP-CSI-RS resource can be applied for a set of CCs/BWPs, i.e. the updated TCI state is applied for the SP-/AP-CSI-RS resources with the same CSI-RS resource ID for all the BWPs in the indicated CCs.

In Rel-16, for PUSCH a mapping between sri-PUSCH-PowerControlId and PUSCH-PathlossReferenceRS-Id can be updated by a MAC CE; for SRS a MAC CE can provide by SRS-PathlossReferenceRS-Id a corresponding RS resource index qd for aperiodic or semi-persistent SRS resource set qs. If the CC list is configured, the updated TCI state/spatial relation is shared across CCs. To perform accurate power control, the pathloss RS and source RS in TCI state/spatial relation need to be aligned. So the updated pathloss RS can be applied for all the BWPs in the indicated CCs, e.g. the updated pathloss RS is applied to the SRS resource set with the same set ID for all the BWPs in the indicated CCs.

Support beam update for all the BWPs in the CC list configured by gNB.

Support MAC CE-based TCI state update for CSI-RS for CSI, for BM and for tracking.

Support simultaneous beam update by a MAC CE for PUCCH resource/PUCCH resource group for all the BWPs in the indicated CCs.

Support pathloss RS update for PUCCH/PUSCH/SRS by a MAC CE for all the BWPs in the indicated CCs.

Conclusions

- In this contribution, we analyze potential enhancements of multi-beam operation in Rel-17. To summarize, we have following observations and proposals.

Narrow beam provides better data transmission efficiency while wide beam provides more robustness.

For UEs operating in CBM mode, allowing SCC to select the best Tx beam has a significant gain in mitigating CBM performance degradation.

Panel switching based on MPE event performs better than panel switching based on purely DL beam report.

For panel switching triggered by MPE, when the panel is switched for both downlink reception and uplink transmission, the downlink performance loss is marginal.

For multi-TRP scenario, when the panel is switched for both downlink reception and uplink transmission, the performance loss of downlink transmission becomes even smaller compared with single-TRP case.

Support M=N>1 joint TCI states, M>1 separate DL TCI states, N>1 separate UL TCI states in Rel-17.

Multiple channel/RS types can be flexibly grouped to apply a same TCI state.

The maximum number of groups is 2 in Rel-17.

Support to divide CORESETs and the associated channels into different groups with their own TCI states, e.g. non-UE-specific CORESETs, CORESET with beam indication DCI.

Support applying single common beam to a TRP, i.e. M/N=2 for multi-TRP case.

A single TCI state pool is applied to joint TCI state update and separate TCI state update.

For separate DL/UL TCI, UL spatial filter is derived from the RS of QCL-TypeD RS.

Support SSB, SRS for beam management as source RS types for DL QCL-TypeD.

CSI-RS for CSI, non-BM RS are not supported as source RS types in Rel-17 TCI framework.

The joint/separate DL TCI state is applied to CSI-RS for CSI acquisition.

The joint/separate TCI state is not used for beam management RS and for CSI-RS for tracking.

Use RRC parameters to enable the DCI based TCI state indication or MAC CE based TCI state indication.

Reuse existing TCI field in DCI for beam indication, support Alt3 or Alt1+Alt3

Use MAC CEs to activate the mapping between TCI codepoint and joint/separate TCI states for beam indication for single/multi-TRP cases.

Use MAC CE to switch joint or separate TCI state indication based on UE capability.

Support Alt3 with modified version, the periodic DL-RS used as a source RS for determining spatial TX filter can be used as PL-RS. In case the source RS for determining spatial TX filter is not periodic DL-RS, reuse Rel.16 procedure with the same signaling structure (MAC CE+SRI field in UL-related DCI) to indicate PL-RS for UL transmission with minimum enhancement.

For PUCCH, the referenceSignal in PUCCH-SpatialRelationInfo can be changed to optional.

For PUSCH/SRS, reuse Rel.16 procedure with the same signaling structure (MAC CE+SRI field in UL-related DCI) for minimum impact on SRI functionality.

PL-RS is not additionally configured in or associated to UL TCI state or (if applicable) joint TCI state.

For other PC parameters, support Alt4, the setting of (P0, alpha, closed loop index) is determined as in Rel-16 with similar enhancement as for PLRS.

Support Opt-1, sharing a single TCI state pool for the set of configured CCs, e.g., cell-group TCI state pool.

Single RS, e.g. SSB/different TRS/CSI-RS with repetition, determined according to the TCI state(s) indicated by a common TCI state ID is used to provide QCL Type-D indication and to determine UL TX spatial filter across the set of configured CCs.

Support the joint/separate TCI state ID update and activation across a set of configured CCs to be applied to inter-band CA.

Consider CBM operation to improve performance for inter-band CA case.

Using NACK of the PDSCH scheduled by the DCI carrying the beam indication is problematic as an ACK.

Support Alt1: if beam indication is successfully received and the newly indicated beam in the beam indication is different from the previously indicated beam, the beam application time is the first slot that is at least X ms or Y symbols after the last symbol of the DCI with the joint or separate DL/UL beam indication.

With M>1 and N>1, MAC CE can be used to indicate TCI state for a group including PDCCH carrying beam indication DCI and the PUSCH/PDSCH scheduled/activated and PUCCH transmission triggered by PDCCH.

For beam application time in UE capability,

reuse the values defined in existing UE capability timeDurationForQCL,

in cross carrier case, reuse existing scheme, i.e. additional timing delay is added.

The default beam of PDSCH before indication/activation of new TCI state follows one of the TCI states previously indicated.

- Support L1-RSRP inter-cell measurement with or without limitation to within SMTC window based on configurations.

  - For cases where the measurement is within the SMTC, timing assumption for L1-RSRP based inter-cell measurement follows current L3 RRM measurement.

- Clarify the scheduling restrictions for L1 inter-cell beam measurement.

Support to enhance CSI reporting/resource setting configuration to include non-serving cell information for inter-cell beam management.

Support to use MAC CE to update the non-serving cell information.

Support to report timing offset between different cells in L1/L2 report.

Support to reuse the same way of associating non-serving cell information (at least including PCI information) with TCI for L1/L2 centric mobility and inter-cell MTRP operation.

Support inter-cell beam indication for UE dedicated transmission and reception to smooth mobility between cells.

The beam indication does not trigger mobility procedure, only involving TRx from a different cell.

Support Rel-17 unified framework to indicate beams to channels/RSs other than e.g. COERSET #0 and TRx initiated through CORESET #0.

In Rel-17 for MPUE, optimize the UE operation with only one activated panel, considering both MPUE-Assumption1 and MPUE-Assumption3.

UE panel ID can be introduced to assist network and UE’s alignment.

E.g. enhanced beam report/report configuration/report activation including panel ID.

Rel-17 TCI state update can implicitly be used for UL panel selection based on the enhanced beam report.

Support Option 1D: P-MPR per panel report in PHR in Rel-17 without additional reporting quantity.

Introduce panel ID and the corresponding P-MPR value in PHR MAC CE.

For panel switching triggered by MPE event, support single activated panel operation for both DL reception and UL transmission.

Consider mechanisms for alignment of DL reception panel and UL transmission panel due to panel switching triggered by MPE to facilitate single activated panel operation.

For TCI state switching, TRS can be used to speed up the beam switching time.

Support beam update for all the BWPs in the CC list configured by gNB.

Support MAC CE-based TCI state update for CSI-RS for CSI, for BM and for tracking.

Support simultaneous beam update by a MAC CE for PUCCH resource/PUCCH resource group for all the BWPs in the indicated CCs.

Support pathloss RS update for PUCCH/PUSCH/SRS by a MAC CE for all the BWPs in the indicated CCs.

References

RP-193133, “New WID: Further enhancements on MIMO for NR”, Samsung, RAN#86.

Chairman’s notes for 3GPP TSG RAN WG1 #102-e Meeting.

Chairman’s notes for 3GPP TSG RAN WG1 #103-e Meeting.

Chairman’s notes for 3GPP TSG RAN WG1 #104-e Meeting.

3GPP TS 38.213 V16.5.0 (2021-03)

3GPP TS 38.214 V16.5.0 (2021-03)

3GPP TS 38.321 V16.4.0 (2021-03)

3GPP TS 38.331 V16.4.1 (2021-03)

- Annex

System-level simulation assumption for common beam

| Parameters | Value |
| --- | --- |
| Duplex, Waveform | FDD, OFDM |
| Multiple access | OFDMA |
| Scenario | Indoor hotspot (InH) |
| Frequency Range | FR2: 30GHz，80MHz BW and 120kHz SCS； |
| Channel model | According to the TR 38.901 |
| Antenna setup and port layouts at TRP for narrow beam | 2 ports: (4,4,2,1,1) for 30GHz (Mp,Np) = (1, 2) |
| Antenna setup and port layouts at TRP for wide beam | 2 ports: (2,4,2,1,1) for 30GHz (Mp,Np) = (1, 2) |
| Analog Codebook setup at TRP for narrow beam | Vertical: [-3/8pi -1/8pi 1/8pi 3/8pi]; horizontal: [-3/8pi -1/8pi 1/8pi 3/8pi] |
| Analog Codebook setup at TRP for wide beam | Vertical: [-1/4pi 1/4pi]; horizontal: [-1/4pi 1/4pi] |
| Antenna setup and port layouts at UE | For 30 GHz: (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV, dH) = (0.5, 0.5)λ. (dV, dH) = (0.5, 0.5)λ. (dg,V, dg,H) = (0, 0)λ. * Θmg,ng=90°; Ω0,1=Ω0,0+180 |
| Analog Codebook setup at UE | Vertical: [1/6pi, 2/6pi]; horizontal: [-3/8pi -1/8pi 1/8pi 3/8pi] |
| Digital Codebook | Type2 Codebook |
| Physical layer abstraction | RBIR |
| Channel estimator | Ideal |
| Receiver | MMSE IRC |
| Rank candidate | Transmission layer 1 or 2 per TRP |
| CSI feedback period | 4 |
| CSI feedback delay | 5 |
| CQI/MCS table | eMBB 256QAM |
| Traffic model | FTP1 |
| Packet Size | 4Mbit |
| UE receiver noise figure | 9dB |
| Number of RBs | 52 |

Simulation assumptions for SLS: CBM under co-located deployment

| Parameters |  | Values |
| --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |
| Inter-site distance |  | 500m |
| BS antenna height |  | 25 m |
| UE location | Outdoor/indoor | Outdoor and indoor |
|  | Indoor UE ratio | 20% |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |
|  | LOS/NLOS | LOS and NLOS |
|  | UE antenna height | Same as 3D-UMa in TR 36.873 |
| Minimum BS - UE distance (2D) |  | 35 m |
| Channel model |  | UMa |
| UE number per cell |  | 5 |
| BS Antenna Configuration |  | (Mg, Ng ,M, N, P) = (1, 1, 8, 16, 2). (dV, dH) = (0.5, 0.5) λ. (dg,V, dg,H) = (2.0, 4.0) λ |
| UE Antenna Model1 |  | (Mg, Ng, M, N, P) = (1, 1, 2, 2, 2) (dV, dH) = (0.5, 0.5) λ. |
| UE Antenna Model2 |  | (Mg, Ng, M, N, P) = (1, 1, 1, 4, 2) (dV, dH) = (0.5, 0.5) λ. |
| UE Antenna Model3 |  | (Mg, Ng, M, N, P) = (1, 1, 1, 8, 2) (dV, dH) = (0.5, 0.5) λ. |

System-level simulation assumption for MPE

| Parameters | Values |
| --- | --- |
| Scenarios | - Dense urban (macro-layer only, TR 38.913) @FR2, 200m ISD, 2-tier model with wrap-around (7 sites, 3 sectors/cells per cell), 100% outdoor - the number of UEs is 105 at each drop - Indoor (TR 38.901/802) - the number of UEs is 120 at each drop |
| UE speed | 3 km/hr for indoor UEs, 30km/hr for outdoor UEs |
| Panel Blockage Modeling for MPE | Only one panel is blocked. The blocked panel is randomly selected at each drop - Blocking entails an additional pathloss of 10dB applied to both DL and UL For simulation with full buffer traffic, a blocking event is determined, started at the beginning of each drop, and sustained throughout the entire drop. |
| MPE Modeling | When MPE occurs, the maximum TX power for the covered panel is reduced by 10dB P-MPR. That is, the actual maximum TX transmit power = maximum EIRP (22dBm) – P-MPR (10dB) |
| UE-side panel switching latency | 0 ms for active panels |
| Performance metrics | CDF of UE throughput, avg. and 5% UE throughput (representing cell-edge coverage) |
| Frequency Range | FR2 @ 30 GHz, - SCS: 120 kHz - BW: 80 MHz |
| Transmission Power | Maximum Power for base station is 40dBm; Maximum Power for UE is 22dBm; |
| BS Antenna Configuration | (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2). (dV, dH) = (0.5, 0.5) λ. (dg,V, dg,H) = (2.0, 4.0) λ 32 beams at BS side: vertical[1/8pi, 3/8pi, 5/8pi, 7/8pi]; horizontal[-7/16pi, -5/16pi, -3/16pi, -1/16pi, 1/16pi, 3/16pi, 5/16pi, 7/16pi]; |
| BS Antenna radiation pattern | TR 38.802 Table A.2.1-6, Table A.2.1-7 |
| UE Antenna Configuration | Number/location of panels: 3 panels (left, right, and back) Panel structure - (M, N, P) = (1, 4, 2) - 8beam at UE side:vertical[0.0]; horizontal[-3/8pi, -1/8pi, 1/8pi, 3/8pi] |
| UE Antenna radiation pattern | TR 38.802 Table A.2.1-8, Table A.2.1-10 |
| Beam correspondence | Perfect beam correspondence，∆EIRPBC =0 |
| Link adaptation | Based on CSI-RS |
| Rank candidate | Only rank1 |
| Traffic Model | Full buffer and FTP model 1 with packet size 0.5Mbytes for DL and 0.1Mbytes for UL |
| Inter-panel calibration for UE | Ideal |
| Control channel decoding | Ideal |
| UE receiver type | MMSE-IRC |
| BF scheme | DFT based |
| Transmission scheme | SU-MIMO |
| Other simulation assumptions | serving TRP selection ：max L1-RSRP scheduling algorithm ：PF |
| Algorithm details (when applicable) | Beam metric: L1-RSRP One panel is active during the data transmission |