3GPP TSG RAN WG1 #104b-e                                                                                 	         R1-2102556

e-Meeting, April 12th – 20th, 2021

Source:	OPPO

Title:	Discussion on the remaining issues of initial access

Agenda Item:	7.2.2

Document for:	Discussion and Decision

Introduction

In this contribution, we point out a potential uncertainty on the determination of shared spectrum that might need further clarification.

Discussion

In RACH procedure, there are some UE behavior or signaling field interpretation dedicated to NRU, including:

- For PDCCH-RAR detection, the UE needs to check the SFN field

- For RAR UL grant scheduling: the UL grant field positions, e.g. PUSCH TDRA, MCS, TPC, CSI request, are changed according to whether or not operation with shared spectrum

What is unclear is that the term ‘operation’ may be interpreted as uplink operation or may also be interpreted as cell operation. One example is that for a PCell operating in unlicensed band, it may have a SUL link in licensed band, e.g. PCell downlink and normal uplink are in unlicensed band, SUL link is in licensed band. For this case, how to determine the term ‘operation’ when a UE selects SUL to transmit PRACH.

We can discuss the following two options:

Option 1: ‘operation’ is determined by whether the PDCCH scheduling RAR is in unlicensed band or licensed band.

Option 2: ‘operation’ is determined by whether the PRACH is transmitted in unlicensed band or licensed band.

Proposal: RAN1 shall clarify the understanding of ‘operation’ with/without shared spectrum channel access.