3GPP TSG RAN WG1 #104b-e			R1-2103012

e-Meeting, April 12th – 20th, 2021

Source:	Mitsubishi Electric

Title:	PT-RS enhancements for NR from 52.6GHz to 71GHz

Agenda Item:	8.2.5 - PDSCH/PUSCH enhancements

Document for: 	Discussion/Decision

# Introduction

Following the conclusions of TR 38.808 [1] issued from the Rel.17 study item on supporting NR from 52.6GHz to 71 GHz [2], a new work item [3] was approved in RAN#90. Among the objectives of the work item, it is required to:

- Evaluate, and if needed, specify the PTRS enhancement for 120kHz SCS, 480kHz SCS and/or 960kHz SCS, as well as DMRS enhancement for 480kHz SCS and/or 960kHz SCS.

[…]

Note 2: UEs supporting a band in the range of 52.6GHz-71GHz are not required to support 480kHz SCS and 960kHz SCS.

In the past RAN1#104-e meeting, the following was agreed:

- At least existing PTRS design for CP-OFDM is supported for NR operation in 52.6 to 71 GHz.

- Companies are encouraged to study the need of potential PTRS enhancement for CP-OFDM with respect to phase noise compensation performance considering at least the following aspects:

  - PTRS density/pattern (e.g. distributed, block-based) and sequence (e.g. cyclic sequence)

  - Frequency domain power boosting and its impact to PDSCH performance and PDSCH to DMRS EPRE

  - Receiver complexity, including possible aspects related to supporting both existing PTRS design and potential PTRS enhancement

  - Possible specification impact of supporting potential PTRS enhancement in addition to existing PTRS design

  - Note: PTRS overhead should be accounted for in the evaluations, e.g. by showing spectral efficiency results and/or reporting effective coding rate

- Note: the decision to support potential enhanced PTRS design in addition to existing PTRS design will be made based on performance benefit, receiver complexity and specification effort aspects of enhanced PTRS design together and not purely on the considerations of the specification effort caused by supporting potential enhanced PTRS design in addition to existing PTRS design.

In this contribution, we give our view on some performance aspects of phase noise compensation for running an NR interface in bands above 52.6GHz.

# PT-RS enhancements for CP-OFDM

In Rel.15 NR, PT-RS was introduced to cope with phase noise effects, and PT-RS pattern design and optimization was performed based on phase noise effects for bands up to 40GHz.

For CP-OFDM waveform, the current PT-RS pattern was devised to compensate for common phase error (CPE), which proved to be effective enough for phase noise effects considered in Rel.15, below 52.6GHz, where inter-carrier interference (ICI) due to phase noise can be neglected after CPE compensation. One single PT-RS tone is inserted every 2 or 4 RBs, with the purpose of ensuring CPE compensation.

Assuming an efficient channel equalization, the received symbol Yk at subcarrier k can be modeled as follows:

Yk=Ψ0Xk+j=1N-1ΨjXk-jN+Zk           (1)

where X is the emitted signal, Ψ is the phase noise, Z is the AWGN, the three of them being in the frequency domain. ∙N is “modulo N”.

## Distributed PT-RS with CPE compensation

For low values of the carrier frequency Fc, the phase noise is mostly represented by its DC component Ψ0 described in (1), that impacts all the received symbols Yk with the Common Phase Error (CPE). In this case the summation term in the model (1) could be merged within the AWGN. Distributed PT-RS symbols on some subcarriers as Rel.15 NR PT-RS allow to estimate Ψ0 and compensate it.

In the past meetings, several companies identified that CPE compensation based on the current PT-RS design for CP-OFDM, does not provide sufficient performance for bands above 52.6GHz. Due to increased CPE/ICI in high bands, low modulation orders can only be supported with degraded performance, while high modulation orders would require unreasonably high subcarrier spacing if the current RS design is reused as is. Results in [6], [7], [8], [9], [10], [11], [12] show that the current PT-RS pattern with CPE compensation is not sufficient to compensate phase errors in higher mmWave bands, and that keeping current design without strongly increasing the subcarrier spacing leads to serious performance degradation. UEs supporting a band in the range of 52.6GHz-71GHz are not required to support subcarrier spacing superior to 120kHz, system performance needs to be guaranteed with the mandatory 120kHz subcarrier spacing.

Simply increasing the PT-RS density beyond the densities specified in Rel.15, while keeping a CPE compensation approach, does not provide enough gain as it is shown in our simulation results.

It is clear that CPE compensation is not sufficient for efficient phase noise compensation in higher mmWave bands, and that enhancements allowing for efficient ICI compensation need to be investigated.

## Distributed PT-RS with ICI compensation

The phase noise effects are more pronounced in the higher parts of the mmWave spectrum. For higher values of Fc, e.g. 60/70GHz in FR2, the components {Ψj}j≠0 become much more significant in such a way that the summation term in (1) should be considered as such. Consequently, the CPE compensation is not relevant anymore. In [13] it is proposed to obtain some ICI compensation based on a distributed PT-RS sequence like the one in Rel.15.

The de-ICI Wiener filter uses the approach described here-below. For a given subcarrier index k, the estimate of the symbol Xk can be written as:

Xk=m=-u+uamRk-m    (2)

Where R are the received symbols after the equalizer and a is the Wiener filter of length 2u+1computed as:

a=RHR-1RHXP    (3)

Where R is a matrix of NPTRS rows and (2u+1) columns that contains some well selected received symbols of R and XP is the vector of the NPTRS emitted PT-RS symbols.

The filter coefficients are computed only once because and the same filter is applied for all the subcarriers (the phase noise is considered to be the same all along the band).

From a complexity point of view, the number of operations per OFDM symbol is:

CWF=L3+2NPTRS-1L2+L+N2L-1          (4)

where L=2u+1 is the number of taps in the filter, NPTRS is the number of PT-RS REs in an OFDM symbol and N is the number of allocated subcarriers.

## Block PT-RS (with any ordinary sequence)

Analyzing (1) shows that each received symbol Yk experiments an interference from the neighborhood of Xk, representing the Inter-Carrier Interference (ICI). Considering the exponential decrease of the phase noise PSD, this neighborhood should be seen as relatively small in such a way that few subcarriers around k should be taken into account into the convolution. This raises the option of selecting a block-based PT-RS pattern instead of the distributed pattern.

At the receiver side, ICI component can be compensated in different manners. One manner is to compensate ICI impact through direct de-ICI filtering with the de-ICI Wiener filter described in equations (2), (3) in section 2.2. Another manner is to use the PT-RS blocks as such to estimate the coefficients Ψj of the ICI filter in (1) and then perform deconvolution to compute the estimates Xk. Both approaches have similar performance.

## Block PT-RS with cyclic sequence

The phase noise in the time domain applies as a point-wise product with the emitted signal. In the frequency domain this corresponds to a circular convolution and results in ICI. More precisely, when inserting PT-RS symbols in the emitted signal X, we obtain data-to-pilot interference on any received PT-RS symbol in Y, which damages the performance.

However, interference can be heavily reduced by selecting a well-suited PT-RS pattern. For a single PT-RS block, similarly to the cyclic prefix insertion in the time domain to obtain a circular channel matrix, using a cyclic PT-RS sequence, as in Figure 1, helps obtain a circular phase noise matrix that only applies on PT-RS symbols without any data-to-pilot interference. This allows estimating the phase noise on an arbitrary bandwidth in the frequency domain.

PT-RS is processed after DMRS-based channel estimation/user de-multiplexing. From this respect, there is no need for PT-RS to observe any orthogonal sequence length optimization or any particular sequence choice criteria. The optimum length of the CP can thus be chosen the highest as possible in order to allow the best data to pilot isolation. For a block of KP PT-RS symbols the optimum size of the CP is thus floor(KP/2).

P1

P2

P3

P4

P5

P2

P3

P4

P5

copy the KΨ-1 last PT-RS symbols at the beginning of the block

KΨ

KΨ-1

KP

data

data

PT-RS

Figure 1. Example of a cyclic PT-RS sequence of length KP=9

The above figure shows that KP is an odd number such that KP=2KΨ-1, which is the optimum sequence length in the sense that it includes the minimum number of coefficients needed to estimate the frequency-domain response {Ψ1, …,ΨN} of the phase noise. However, it is easily possible to extend the cyclic prefix by inserting P1 on the left side to reach an even number KP=2KΨ. Although not necessary from a performance standpoint, such extension allows more flexibility whenever an even size block may be useful from a practical point of view.

P1

P1

P2

P3

P4

P5

P2

P3

P4

P5

copy the KΨ last PT-RS symbols at the beginning of the block

KΨ

KΨ

KP

data

data

PT-RS

Figure 2. Example of a cyclic PT-RS sequence of length KP=10

### An example of receiver description

In the following, we give an example of a receiver performing phase noise compensation based on the proposed pattern. Please note that this is just an example and that any other generic receiver coping with cyclic sequences can be used.

Phase noise compensation based on block PT-RS is done by a simple conventional receiver exploiting the cyclic structure of the PT-RS block. The PT-RS block cyclic structure makes equation (1) a local circular convolution on any PT-RS block (In case Kp=2KΨ, there is one more equation (1) for a single PT-RS block than in the case of Kp=2KΨ -1 that is the exact same equation as another equation for the same block when excepting the AWGN component. This means that averaging them leads to increase the associated SNR). As an example, let’s consider the notations for received symbols as in the next figure:

Then the received symbols can be written in a matrix form as:

Y5Y6Y7Y8Y9=Ψ1Ψ5Ψ4Ψ3Ψ2Ψ2Ψ1Ψ5Ψ4Ψ3Ψ3Ψ2Ψ1Ψ5Ψ4Ψ4Ψ3Ψ2Ψ1Ψ5Ψ5Ψ4Ψ3Ψ2Ψ1P1P2P3P4P5+Z5Z6Z7Z8Z9	(5)

The phase noise matrix is a circulant matrix thanks to the cyclic structure of the PT-RS sequence. A method for obtaining the estimate of the phase noise is to use the diagonalization through Fourier transform property of a circulant matrix, Ψ =FKΨ-1ϕ FKΨ. This way, (5) is equivalent to:

FKΨ-1Y5Y6Y7Y8Y9=ϕ10⋯⋯00ϕ2⋱⋮⋮⋱ϕ3⋱⋮⋮⋱ϕ400⋯⋯0ϕ5FKΨ-1P1P2P3P4P5+FKΨ-1Z5Z6Z7Z8Z9	(6)

Therefore, the estimates of the phase noise components following a Zero-Forcing-like estimation, dividing the result by the invert Fourier transform of the emitted sequence provides an estimate of the phase noise such that:

Ψn=y'n/pn	(7)

Where y'=FKΨ-1Y5Y6Y7Y8Y9 and p=FKΨ-1P1P2P3P4P5. It is also possible to consider an MMSE-like estimation by considering the variance of the additive noise.

Other methods for obtaining the phase noise estimate based the circulant nature of the system can be devised, for example by representing the system by relying on a circulant pilot matrix MP instead of a circulant phase noise matrix as follows:

Y5Y6Y7Y8Y9=P1P5P4P3P2P2P1P5P4P3P3P2P1P5P4P4P3P2P1P5P5P4P3P2P1Ψ1Ψ2Ψ3Ψ4Ψ5+Z5Z6Z7Z8Z9

Then through an equalizer, e.g. Zero-Forcing like or MMSE-like, it is possible to retrieve an estimate Ψ. This implies inverting a matrix of size KΨ×KΨ which could be costly for a high value of KΨ.

For any method, the deconvolution could be then done for example by going to the time domain such that eiφ=FN-1Ψ and operating a simple point-wise product ⊙. This way the estimate Xk of Xk is:

Xk=FNeiφ⊙yk

The interest for using several PT-RS blocks is to refine the phase noise estimate through an averaging process over the blocks. We will denote this conventional receiver by PN compensation filter, to distinguish it with respect to the de-ICI Wiener filter in 2.2 and 2.3.

### Receiver complexity evaluation

From a complexity point of view, the number of operations per OFDM symbol employed by the PN compensation filter described in the previous sub-section is:

CPN=2NPKΨ1+logKΨ+3NlogN+N         (8)

where Np is the number of PT-RS blocks of length Kp inserted in the OFDM symbol and N is the number of allocated subcarriers. A numerical comparison with the complexity of the de-ICI filter used to compensate the phase noise based on the Rel.15 distributed pattern is presented in Annex B. For the simulation results presented in the performance analysis in the next section, the PN compensation filter is shown to be less complex than the de-ICI filter.

### Other considerations

Some concerns have been raised in [13] regarding the collision cases between block PT-RS and other reference symbols such as TRS and CSI-RS. As explained above, the PT-RS sequence itself can be chosen in a very flexible manner (any sequence as long as it is cyclic). Whenever avoiding collision by configuration is not possible, it is possible to reuse the values configured as TRS or CSI-RS for the purpose of phase noise compensation (e.g. build the PT-RS sequence based on the TRS/CSI-RS values) without any performance impact, and remaining cases can be treated by puncturing the PT-RS with minimum performance impact.

Other variants of cyclic sequences were presented for example in [14], where the circular sequence includes the head circular part and the tail circular part, wherein the head circular part is composed of the tail of the base sequence, and the tail circular part is composed of the head of the base sequence, such as the sequence reads as S=[SN-q, …SN, S1, …SP, …, SN-q,….SN, S1, …, SP]. This sequence is overall circulant, and can be seen as a particular sequence choice of the current framework where [P1…PKΨ]=[SP+1, …, SN-q, …SN,S1, …SP ] and the cyclic prefix amounts P+q+1 samples.

## Performance analysis

Performance comparison between the following scenarios was conducted in an extensive manner, including various carrier frequencies /MCS/ bandwidth allocations, and simulation results are presented in the annex A, Figure 3 - Figure 22:

- CPE-based phase noise compensation with scattered PT-RS from Rel.15 design

- De-ICI filtering based on distributed PT-RS from Rel.15 design with the de-ICI Wiener filter

- Phase noise compensation including ICI effects based on block PT-RS (no sequence optimization) with the de-ICI Wiener filter

- Phase noise compensation including ICI effects based on block PT-RS with cyclic sequence as proposed in the current contribution; PN compensation filtering (conventional estimator) is used at the receiver side

In the following results, we optimized performance-wise the length of the de-ICI filter and the presented results correspond to the best-performing version with u=5 (L=11). This filter is shown in the annex to have a somewhat higher complexity than the PN compensation filter used for block PT-RS with cyclic sequence estimations.

Simulation results consist in Frame Error Rates (FER) and Spectral Efficiencies (SE) for various PT-RS patterns. On the figures, the following convention is used:

- Magenta curves represent the performance of CPE compensation (based on scattered PT-RS pattern) with same or more important density than the Rel.15 existing PT-RS pattern.

- Orange curves represent the performance of de-ICI Wiener filtering based on scattered PT-RS pattern with same or more important density than the Rel.15 existing PT-RS pattern; different filter length were tested and we picked to represent the best performing one, which consists of 11 taps.

- Green curves represent the performance of block-based PT-RS pattern/ordinary sequence with different pattern densities and de-ICI Wiener filtering. Each curve is labelled with the block pattern density represented as NpxKp where Np is the number of PT-RS blocks and Kp is the number of PT-RS symbols in each block.

- Blue curves represent the performance of block-based PT-RS pattern with cyclic sequence and different pattern densities. Each curve is labelled with the block pattern density represented as NpxKp where Np is the number of PT-RS blocks and Kp is the number of PT-RS symbols in each block. The PN compensation receiver described in 2.4 was used.

In the SE figures, the colored disks indicate the SE value such as that the corresponding FER is 0.1.

For 16QAM2/3 at 60GHz, CPE needs a higher PT-RS density than the Rel.15 pattern to reach FER=0.1. Increasing the carrier frequency up to 70GHz shows that CPE with Rel.15 PT-RS pattern is not converging to FER=0.1.

For 64QAM1/2, CPE is not able to converge to FER=0.1 whatever the PT-RS density.

Observation 1: In bands above 52.6GHz, the ICI component of the phase noise becomes predominant on CPE.

Observation 2: Distributed PT-RS pattern shows poor performance results with CPE phase noise estimation regardless of the PT-RS pattern density.

When considering more complex receivers, such as the de-ICI Wiener filtering, considering a denser pattern than the Rel.15 PT-RS pattern allows for significantly better performance than CPE in such a way that FER=0.1 can be reached. However, in some cases, e.g. with 64QAM1/2 or at 70GHz with 16QAM2/3 and 64QAM1/2, the FER curves do not go below 0.1. Therefore, there is a need for another kind of pattern than the distributed pattern.

Observation 3:  For a distributed PT-RS pattern, at small carrier allocations, the performance is poor even with de-ICI filtering, due to an insufficient number of PT-RS samples.

Observation 4:  For a distributed PT-RS pattern, de-ICI Wiener filtering outperforms CPE in all cases, but high MCS still not reach FER=0.1.

Observation 5: Distributed PT-RS patterns are not robust enough to ensure system performance in bands above 52.6GHz.

Considering PT-RS blocks with random ordinary sequence, the performance of de-ICI Wiener filtering is dependent on the number of blocks. Performance of block PT-RS can be significantly improved by increasing the number of blocks, but if we impose the constraint of keeping an equivalent overhead, then block PT-RS is outperformed by distributed PT-RS patterns when a same de-ICI Wiener filter is used at the receiver side. Simply using PT-RS blocks doesn’t reap any benefit, the advantage of having blocks needs to be used in a more suited manner.

Observation 6: For a similar overhead, block PT-RS (with any ordinary sequence) is outperformed by distributed PT-RS pattern when a same de-ICI Wiener filter is used at the receiver side.

When considering block PT-RS patterns with cyclic sequence and with phase noise compensation module able of exploiting a circulant phase noise matrix, the performance is significantly improved. First of all, for 16QAM2/3 at 60GHz, for the same overhead (256=16x16), block PT-RS with cyclic sequence gains around 1dB with respect to distributed PT-RS sequence. Overhead wise, block PT-RS with cyclic sequence can reach the same performance as distributed PT-RS/de-ICI with only half of the overhead (8x16=128 PT-RS symbols). This results in improved spectral efficiency performance. For higher MCS and/or higher carrier frequency, block PT-RS with cyclic sequence displays robust behavior even when all other schemes fail to reach 10-1 FER target.

Observation 7: Block PT-RS with cyclic sequence significantly outperforms the distributed PT-RS pattern with ICI compensation. The gain increases with the carrier frequency.

Performance improvement is significant. For medium MCS we observe gains in order of 1-2dB for FER target 10-1, and/or of 10-20% in spectral efficiency in average, and up to 40-50% in some specific scenarios (very large allocations and/or high carrier frequency). For high MCS, in some scenarios only block PT-RS with cyclic sequence reaches target FER of 10-1. Further pattern optimization of block-based PT-RS pattern (block length, number of blocks) can be performed to increase the performance gap. The performance gap increases with the carrier frequency.

Complexity-wise, it is proven in annex B that the PN compensation filter used for block-PTRS with cyclic sequence is significantly less complex than the de-ICI Wiener filter used for ICI compensation of the distributed PT-RS pattern.

Observation 8: Block PT-RS with cyclic sequence requires lower complexity phase noise compensation filtering than the de-ICI filter needed for the distributed PT-RS pattern.

It appears from simulation results that, from a system throughput perspective, support of block PT-RS with cyclic sequence is highly desirable at 60GHz and essential at 70GHz. This pattern can be seen as complementary to the Rel.15 pattern.

Therefore, we make the following proposals:

Proposal 1: Support block PT-RS with cyclic sequence for OFDM waveform.

Proposal 2: A PT-RS sequence for OFDM waveform composed of KP samples includes a cyclic prefix of floor(KP/2) samples.

# On PT-RS enhancements for DFTsOFDM

The Rel.15 pattern for PT-RS compensation for DFTsOFDM is rather flexible and already accounts for within-symbol phase-noise tracking. From this perspective, whenever the performance of current patterns is not enough at high carrier frequencies, increasing the pattern density would be an efficient measure, without the need of a paradigm change. Such density increase is straightforward with minimum spec impact.

Proposal 3: Support density extension of current Rel.15 PT-RS for DFTsOFDM waveform.

# Conclusions

For OFDM waveform, CPE compensation and the de-ICI Wiener filtering are insufficient at carrier frequencies 60GHz and above for reasonable subcarrier spacings, regardless of the PT-RS scattered pattern density. For OFDM waveform, block-based PT-RS with cyclic sequence is clearly outperforming both CPE compensation and de-ICI Wiener filtering based on distributed PT-RS. Further pattern optimization of block-based PT-RS pattern (block length, number of blocks) can be performed to increase the performance gap.

The Rel.15 pattern for PT-RS compensation for DFTsOFDM already accounts for within-symbol phase-noise tracking, simple density increase of the existing pattern is able of coping with more severe PN effects.

Based on simulation results, we drew the following observations:

Observation 1: In bands above 52.6GHz, the ICI component of the phase noise becomes predominant on CPE.

Observation 2: Distributed PT-RS pattern shows poor performance results with CPE phase noise estimation regardless of the PT-RS pattern density.

Observation 3:  For a distributed PT-RS pattern, at small carrier allocations, the performance is poor even with de-ICI filtering, due to an insufficient number of PT-RS samples.

Observation 4:  For a distributed PT-RS pattern, de-ICI Wiener filtering outperforms CPE in all cases, but high MCS still not reach FER=0.1.

Observation 5: Distributed PT-RS patterns are not robust enough to ensure system performance in bands above 52.6GHz.

Observation 6: For a similar overhead, block PT-RS (with any ordinary sequence) is outperformed by distributed PT-RS pattern when a same de-ICI Wiener filter is used at the receiver side.

Observation 7: Block PT-RS with cyclic sequence significantly outperforms the distributed PT-RS pattern with ICI compensation. The gain increases with the carrier frequency.

Observation 8: Block PT-RS with cyclic sequence requires lower complexity phase noise compensation filtering than the de-ICI filter needed for the distributed PT-RS pattern.

It appears from simulation results that, from a system throughput perspective, support of block PT-RS with cyclic sequence is highly desirable at 60GHz and essential at 70GHz. We arrived at the following conclusions and proposals:

Proposal 1: Support block PT-RS with cyclic sequence for OFDM waveform.

Proposal 2: A PT-RS sequence for OFDM waveform composed of KP samples includes a cyclic prefix of floor(KP/2) samples.

Proposal 3: Support density extension of current Rel.15 PT-RS for DFTsOFDM waveform.

# Annex A – Simulation results

Simulations are performed considering the following parameters aligned with the simulation assumptions in TR38.808 Table A.1-1:

| Carrier frequency | 60GHz; 70GHz |
| --- | --- |
| BW | 400MHz |
| SCS | 120kHz |
| Allocation | 16RBs, 64RBs, 160RBs, 256RBs |
| CP | Normal |
| Channel/DS | TDL-A/10ns |
| DMRS configuration | Front-loaded NR-like configuration type 1, PUSCH mapping type A (slot size 12) |
| Speed | 3kmph |
| FEC | NR LDPC with 50 decoding iterations |
| MCS | 16QAM 2/3, 64QAM 1/2 |
| CFO | 0.1ppm at Tx and -0.1ppm at Rx |
| HPA | Polynomial, IBO=-8dB |
| MIMO | 2x2 |
| PN models | Tx: TR 38.803 Example 2 Model 2 (BS) Rx: TR 38.803 Example 2 Model 1 (UE) |
| Channel estimation | Realistic (MMSE) |
| Frame structure | 14 OFDM symbols, 2-symbols PUCCH (front loaded) |
| PT-RS | Distributed PT-RS: every two RBs (Rel-15) or every RB, with CPE compensation (magenta curves), and de-ICI Wiener filtering (orange curves) Block-based/ordinary sequence PT-RS: allocate NP blocks of KP PT-RS symbols with: NP = 8,16 and KP = 16, with de-ICI Wiener filtering (green curves) Block-based/cyclic sequence PT-RS: allocate NP blocks of KP PT-RS symbols with: NP = 8,16 and KP = 16, with PN compensation filtering (blue curves) |

| Figure 3 Various PT-RS patterns, Fc=60GHz, 256RB, 16QAM2/3: FER | Figure 4 Various PT-RS patterns, Fc=60GHz, 256RB, 16QAM2/3: SE |
| --- | --- |
| Figure 5 Various PT-RS patterns, Fc=70GHz, 256RB, 16QAM2/3: FER | Figure 6 Various PT-RS patterns, Fc=70GHz, 256RB, 16QAM2/3: SE |
| Figure 7 Various PT-RS patterns, Fc=60GHz, 256RB, 64QAM1/2: FER | Figure 8 Various PT-RS patterns, Fc=60GHz, 256RB, 64QAM1/2: SE |
| Figure 9 Various PT-RS patterns, Fc=70GHz, 256RB, 64QAM1/2: FER | Figure 10 Various PT-RS patterns, Fc=70GHz, 256RB, 64QAM1/2: SE |
| Figure 11 Various PT-RS patterns, Fc=60GHz, 16RB, 64Q12: FER | Figure 12 Various PT-RS patterns, Fc=60GHz, 16RB, 64Q12: SE |
| Figure 13 Various PT-RS patterns, Fc=60GHz, 64RB, 64Q12: FER | Figure 14 Various PT-RS patterns, Fc=60GHz, 64RB, 64Q12: SE |
| Figure 15 Various PT-RS patterns, Fc=70GHz, 64RB, 64Q12: FER | Figure 16 Various PT-RS patterns, Fc=70GHz, 64RB, 64Q12: SE |
| Figure 17 Various PT-RS patterns, Fc=70GHz, 160RB, 64Q12: FER | Figure 18 Various PT-RS patterns, Fc=70GHz, 160RB, 64Q12: SE |
| Figure 19 Various PT-RS patterns, Fc=70GHz, 64RB, 16Q23: FER | Figure 20 Various PT-RS patterns, Fc=70GHz, 64RB, 16Q23: SE |
| Figure 21 Various PT-RS patterns, Fc=70GHz, 160RB, 16Q23: FER | Figure 22 Various PT-RS patterns, Fc=70GHz, 160RB, 16Q23: SE |

# Annex B – Complexity evaluations

Table 1 Number of operations per OFDM symbol with the PN filter for the simulated scenarios; CPN=2NPKΨ1+logKΨ+3NlogN+N

| N (number of subcarriers) | KP, KΨ | NP | CPN |
| --- | --- | --- | --- |
| 3072 (256RBs) | 16, 8 | 8 | 77472 |
| __ | __ | 16 | 77866 |

Table 2 Number of operations per OFDM symbol with the de-ICI filter for the simulated scenarios; CWF=L3+2NPTRS-1L2+L+N2L-1.

| N (number of subcarriers) | L | KP | CWF |
| --- | --- | --- | --- |
| 3072 (256RBs) | 11 | 256 (every RB) | 133295 |
| __ | __ | 128 (every two RBs) | 99503 |

# References

### TR 38.808 v1.0.0 Study on supporting NR from 52.6 GHz to 71 GHz.

### RP-193259 New SID: Study on supporting NR from 52.6GHz to 71 GHz., RAN#86, December 2019.

### RP-202925, Revised WID: on Extending current NR operation to 71 GHz.

### RAN1#101-e Chairman’s notes, May 2020.

### RAN1#102-e Chairman’s notes, August 2020.

### R1-2005787, On phase noise compensation for NR from 52.6GHz to 71GHz, Mitsubishi Electric, RAN1#102-e, August 2020.

### R1-2003811, Required changes to NR using existing DL/UL NR waveform, Nokia, NSB, RAN1#101-e, May 2020.

### R1-2004247, On Required changes to NR using existing DL/UL NR Waveform, Apple, RAN1#101-e, May 2020.

### R1-2004500, NR using existing DL-UL NR waveform to support operation between 52p6 GHz and 71 GHz, Qualcomm Incorporated, RAN1#101-e, May 2020.

### R1-2003424, Discussion on requried changes to NR using existing DL/UL NR waveform, vivo, RAN1#101-e, May 2020.

### R1-2007046, On NR operations in 52.6 to 71 GHz, Ericsson, RAN1#102-e, August 2020.

### R1-2003764, Discussion on Required Changes to NR in 52.6 – 71 GHz, Intel Corporation, RAN1#101-e, May 2020.

### R1-2005922, Phase noise compensation approaches for OFDM, Ericsson, RAN1#102-e, August 2020.

### R1-2100201, PDSCH/PUSCH enhancements for 52-71GHz band, Huawei, HiSilicon, RAN1#104-e, January-February 2021.