3GPP TSG RAN WG1 Meeting #104bis-e	R1- 2103143

April 12th – 20th, 2021

Agenda item:	7.2.4

Source: 	Qualcomm Incorporated

Title: 	Remaining Issues in Mode 2 Resource Allocation

Document for:	Discussion/Decision

# Introduction

In this contribution, we discuss remaining issues and corrections for NR V2X specifications regarding Mode 2 resource allocation.

# RSRP Threshold Bound

The current wording for describing the seven-step procedure for resource selection in 38.214 Section 8.1.4 [1], specifically the last step, i.e., step 7, can lead to a UE entering never-ending cycle due to lack of condition where a UE can exit this procedure if the number of candidate single-slot resources SA remains smaller than X⋅Mtotal. This could occur when the available-resource threshold is configured to 50% and periodic reservations are enabled. In such a scenario, a UE could exclude multiple slots from the resource selection window due to not monitoring some slots in the sensing window, leading to proportion of available resources that can never be > 50% of the resources regardless of how much the RSRP threshold is increased. This is exacerbated by the excessive resource exclusion caused by excluding all possible periodicities for unmonitored slots.

To address the described scenario an upper bound on the threshold value Th(pi,pj) is required to allow for the UE to exit the resource selection procedure. We propose for the threshold limit for which UE should exit this procedure to be set to 0 dBm, and hence propose the following text update in 38.214 Section 8.1.4:

Observation 1: The current RSRP threshold increase procedure could lead to an unbounded increase in the RSRP threshold, trapping the UE in an infinite loop.

Proposal 1: adopt the following text proposal in 38.214 to introduce a maximum RSRP threshold of 0 dBm to ensure UE does not enter an infinite loop when performing resource selection in Mode 2 operation.

| Reason for change: | Current specifications does not place an upper limit on the RSRP threshold, which could lead to an unbounded increase in some cases. |
| --- | --- |
| Summary of change: | Exit the candidate resource set determination procedure when the RSRP threshold is increased to more than 0 dBm. |
| Consequences if not approved: | The UE could be forced to perform an infinite loop. |
| Clauses affected: | 8.1.4 |

----------------------------------------------------begin text proposal for 38.214----------------------------------------------------

8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<<<<<unchanged text omitted>>>>>

6)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

a)	the UE receives an SCI format 1-A in slot tmSL, and "Resource reservation period" field, if present, and "Priority" field in the received SCI format 1-A indicate the values Prsvp_RX and prioRX, respectively according to Clause 16.4 in [6, TS 38.213];

b)	the RSRP measurement performed, according to clause 8.4.2.1 for the received SCI format 1-A, is higher than ThprioRX,prioTX;

c)	the SCI format received in slot tmSLor the same SCI format which, if and only if the "Resource reservation period" field is present in the received SCI format 1-A,  is assumed to be received in slot(s) tm+q×Prsvp_RX'SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots according to clause 8.1.7, Q=TscalPrsvp_RX  if Prsvp_RX< Tscal and  n'-m≤Prsvp_RX', where tn'SL = n if slot n belongs to the set t0SL,t1SL,...,tTmaxSL, otherwise slot tn'SL is the first slot after slot n belonging to the set t0SL,t1SL,...,tTmaxSL; otherwise Q=1. Tscal is set to selection window size T2 converted to units of ms.

7)	If the number of candidate single-slot resources remaining in the set SA is smaller than X⋅Mtotal, then Th(pi,pj) is increased by 3 dB for each priority value Th(pi,pj) If this increase  results in  Th(pi,pj) <= 0 dBm, the procedure continues with step 4.

The UE shall report set SA to higher layers.

If a resource ri from the set (r0,r1,r2,…) is not a member of SA, then the UE shall report re-evaluation of the resource ri to higher layers.

----------------------------------------------------end text proposal for 38.214----------------------------------------------------

# Conclusion

Observation 1: The current RSRP threshold increase procedure could lead to an unbounded increase in the RSRP threshold, trapping the UE in an infinite loop.

Proposal 1: adopt the following text proposal in 38.214 to introduce a maximum RSRP threshold of 0 dBm to ensure UE does not enter an infinite loop when performing resource selection in Mode 2 operation.

| Reason for change: | Current specifications does not place an upper limit on the RSRP threshold, which could lead to an unbounded increase in some cases. |
| --- | --- |
| Summary of change: | Exit the candidate resource set determination procedure when the RSRP threshold is increased to more than 0 dBm. |
| Consequences if not approved: | The UE could be forced to perform an infinite loop. |
| Clauses affected: | 8.1.4 |

----------------------------------------------------begin text proposal for 38.214----------------------------------------------------

8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<<<<<unchanged text omitted>>>>>

6)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

a)	the UE receives an SCI format 1-A in slot tmSL, and "Resource reservation period" field, if present, and "Priority" field in the received SCI format 1-A indicate the values Prsvp_RX and prioRX, respectively according to Clause 16.4 in [6, TS 38.213];

b)	the RSRP measurement performed, according to clause 8.4.2.1 for the received SCI format 1-A, is higher than ThprioRX,prioTX;

c)	the SCI format received in slot tmSLor the same SCI format which, if and only if the "Resource reservation period" field is present in the received SCI format 1-A,  is assumed to be received in slot(s) tm+q×Prsvp_RX'SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots according to clause 8.1.7, Q=TscalPrsvp_RX  if Prsvp_RX< Tscal and  n'-m≤Prsvp_RX', where tn'SL = n if slot n belongs to the set t0SL,t1SL,...,tTmaxSL, otherwise slot tn'SL is the first slot after slot n belonging to the set t0SL,t1SL,...,tTmaxSL; otherwise Q=1. Tscal is set to selection window size T2 converted to units of ms.

7)	If the number of candidate single-slot resources remaining in the set SA is smaller than X⋅Mtotal, then Th(pi,pj) is increased by 3 dB for each priority value Th(pi,pj) If this increase  results in  Th(pi,pj) <= 0 dBm, the procedure continues with step 4.

The UE shall report set SA to higher layers.

If a resource ri from the set (r0,r1,r2,…) is not a member of SA, then the UE shall report re-evaluation of the resource ri to higher layers.

----------------------------------------------------end text proposal for 38.214----------------------------------------------------

# References

- 3GPP TS 38.214 V16.3.0, NR, Physical Channels and Modulation (Release 16).

- 3GPP TS 38.212 V16.3.0, NR, Multiplexing and Channel Coding (Release 16).

- 3GPP TS 38.213 V16.3.0, NR, Physical Layer Procedures for Control (Release 16).

- 3GPP TS 38.214 V16.3.0, NR, Physical Layer Procedures for Data (Release 16).