3GPP TSG RAN WG1 Meeting #104b-e	R1-2103433

e-Meeting, April 12th – 20th, 2021

Agenda item:		7.2.6

Source:		Nokia, Nokia Shanghai Bell

Title:	Maintenance of Rel-16 Multi-TRP operation

Document for:		Discussion and Decision

# 1	Introduction

In this contribution, we discuss issues related to Rel-16 multi-TRP operation.

# 2	Discussion

We identified one remaining issue related to multi-DCI based multi-TRP and discussed as follows.

## 2.1	Separate HARQ-ACK feedback with multi-DCI multi-TRP

Multi-DCI based multi-TRP and URLLC features of sub-slot PUCCH and codebooks with different priorities were specified separately and there are no FGs for explicitly mentioning their simultaneous support. For the combination of  multi-DCI based multi-TRP and sub-slot PUCCHs capabilities, the present specification could be understood to mean that each sub-slot could carry two PUCCHs for multi-DCI based multi-TRP HARQ feedback. The combination of multi-DCI based multi-TRP and codebooks with different priorities would mean that UE should support four PUCCH transmissions in a slot. In this situation, we see the following alternatives:

- New FGs.

- Specification excludes simultaneous support of multi-DCI based multi-TRP and sub-slot PUCCH or multiple codebooks.

- Specification modifications or interpretations that would mean gNB implementation limiting the number of PUCCH transmissions per (sub-)slot to not go beyond the reported UE capability on the maximum number of PUCCH(s) per (sub-)slot and this way taking care of that the URLLC features may be to some extent utilized with multi-DCI based multi-TRP transmissions.

Alt. 1 may not be feasible due to complexity and the late stage of specifications. Alt. 2 is a straightforward solution but limits the use of URLLC features when M-DCI based M-TRP operation is applied.  In the following, we consider Alt. 3 because expanding multi-DCI based multi-TRP utilization for URLLC use cases may be valuable for improving reliability but at the same time sub-slot PUCCH and two PUCCH configurations (i.e. PHY prioritization) is useful to reduce the HARQ-ACK & PUSCH latency.

When configured with ackNACKFeedbackMode-r16 = separate for multi-DCI based multi-TRP HARQ-ACK feedback, UE may support, according to the UE indication for FG 16-2a-4, transmissions of long-long, long-short, and /or  short-short PUCCH formats in a slot. The two PUCCH’s must not overlap in time but can otherwise be located freely. For instance, they can be transmitted in the first and second symbol of a slot using PUCCH format 0 with the length of one OS. Or two 2-OS PUCCHs could be transmitted starting from the first and third symbol. Both cases would be compatible with 7*2-OS sub-slot PUCCH configurations. In addition, there seems no reason to limit the number of PUCCHs for multi-DCI based multi-TRP feedback to two per slot but rather consider the total number of PUCCH transmissions in slot (which can be up to 7 for 2OS sub-slot PUCCH). If UE indicates the capability of transmitting a certain number of PUCCHs with HARQ-ACK in a slot, it makes no difference if these PUCCHs are directed to the same or different TRPs. gNB implementation should take care of that the number of actual PUCCH transmissions for HARQ-ACK fits to the UE capability, but there is no reason to prevent the operation of Multi-DCI based Multi-TRP with sub-slot PUCCH and/or two PUCCH configurations otherwise.

Proposal 1: Multi-DCI based Multi-TRP operation with sub-slot PUCCH and/or two PUCCH configurations is supported. Adopt the following draft CR to limit the number of PUCCH transmissions for multi-DCI based multi-TRP HARQ-ACK feedback for these cases (changes in green, to be shown as track changes in the final CR).

Draft CR

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | - | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on the number of PUCCHs with separate HARQ-ACK feedback for multi-DCI based multi-TRP transmissions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, NSB |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-04-xx |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In clause 9 of TS 38.213, there is a missing specification for the number of PUCCHs with HARQ-ACK that a UE may be asked to transmit in a slot when the UE is configured with ackNACKFeedbackMode-r16 = separate for multi-DCI based multi-TRP transmissions and with sub-slot and/or multiple PUCCH-configs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The maximum number of PUCCHs with separate HARQ-ACK feedback for multi-DCI based multi-TRP transmissions is limited by the value a UE has indicated for PUCCHs with HARQ-ACK in case that UE is configured with multiple PUCCH configurations and sub-slots. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unspecified UE behaviour. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

9	UE procedure for reporting control information

<omitted text>

If a UE

-	is not provided coresetPoolIndex or is provided coresetPoolIndex with a value of 0 for first CORESETs on active DL BWPs of serving cells, and

-	is provided coresetPoolIndex with a value of 1 for second CORESETs on active DL BWPs of the serving cells, and

-	is provided ackNackFeedbackMode = separate

the UE shall separately apply the procedures described in Clauses 9.1 and 9.2.3 for reporting HARQ-ACK information associated with the first CORESETs on active DL BWP of the serving cells and for reporting HARQ-ACK information associated with the second CORESETs on active DL BWP of the serving cells. HARQ-ACK information reporting is associated with a CORESET through a reception of a PDCCH with a DCI format triggering the reporting of the HARQ-ACK information by the UE. If a UE is provided subslotLengthForPUCCH, the UE expects that the total number of PUCCH transmissions per slot for reporting HARQ-ACK information associated with the first and second CORESETs on active DL BWP of the serving cells does not exceed the maximum number of PUCCH transmissions for HARQ-ACK within a slot the UE has indicated.

<omitted text>

9.2	UCI reporting in physical uplink control channel

UCI types reported in a PUCCH include HARQ-ACK information, SR, LRR, and CSI. UCI bits include HARQ-ACK information bits, if any, SR information bits, if any, LRR information bit, if any, and CSI bits, if any. The HARQ-ACK information bits correspond to a HARQ-ACK codebook as described in Clause 9.1. For the remaining of this clause, any reference to SR is applicable for SR and/or for LRR.

A UE may transmit one or two PUCCHs on a serving cell in different symbols within a slot. When the UE transmits two PUCCHs in a slot and the UE is not provided ackNackFeedbackMode = separate, at least one of the two PUCCHs uses PUCCH format 0 or PUCCH format 2.

If a UE is provided ackNackFeedbackMode = separate, irrespective of whether UE is configured with one or two PUCCH-Config, the UE may transmit only up to two PUCCHs with HARQ-ACK information in different symbols within a slot.

In Clauses 9.2.3, 9.2.5.1 and 9.2.5.2, a UE assumes 11 CRC bits if a number of respective UCI bits is larger than or equal to 360; otherwise, the UE determines a number of CRC bits based on the number of respective UCI bits as described in [5, TS 38.212].

<omitted text>

# References

[1] 		3GPP TS 38.213 V16.5.0