3GPP TSG RAN WG1 #104bis-e	R1-2103487

e-Meeting, 12th - 20th April, 2021

Title:	Discussion on the initial access aspects for 52.6 to 71 GHz

Source: 	ZTE, Sanechips

Agenda Item:	8.2.1

Document for:	Discussion and Decision

# Introduction

According to the outcome of the study item on supporting NR above 52.6 GHz [1], at 3GPP RAN #90 plenary e-meeting in December 2020, an update WID RP-202925 [2] was approved to extend NR operation up to 71 GHz considering both licensed and unlicensed operation. In the WID, some objectives related to initial access were given and are excerpted as follows.

- Physical layer aspects including [RAN1]:

  - In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported.

Note: Except for timing line related aspects, a common design framework shall be adopted for 480kHz to 960kHz

  - Support of up to 64 SSB beams for licensed and unlicensed operation in this frequency range.

  - Supports 120kHz SCS for SSB and 120kHz SCS for initial access related signals/channels in an initial BWP.

    - Study and specify, if needed, additional SCS (240kHz, 480kHz, 960kHz) for SSB, and additional SCS (480kHz, 960kHz) for initial access related signals/channels in initial BWP.

    - Study and specify, if needed, additional SCS (480kHz, 960kHz) for SSB for cases other than initial access.

    - Note: coverage enhancement for SSB is not pursued.

  - Specify support for PRACH sequence lengths (i.e. L=139, L=571 and L=1151) and study, if needed, specify support for RO configuration for non-consecutive RACH occasions (RO) in time domain for operation in shared spectrum

In the RAN1 #104 e-meeting in February 2021, the following agreements related to initial access were reached [3]. In this contribution, we give our views on initial access aspects for NR between 52.6 GHz ~ 71 GHz, including SSB, Type0-PDCCH, PRACH, etc.

Agreement:

Send an LS to RAN4 to get input on gap required for gNBs and UEs for beam switching and for UL/DL and DL/UL switching.

Agreement:

Whether or not to support 240 kHz, 480kHz and 960kHz SCS for SSB and the conditions under which SSB for 240 kHz, 480 kHz and 960 kHz may be supported will be decided no later than RAN1#104bis-e.

Agreement:

For an unlicensed band that requires LBT, further study whether/how to support discovery burst (DB) and discovery burst transmission window (DBTW) at least for 120 kHz SSB SCS

- If DB supported

  - FFS: What signals/channels are included in DB other than SS/PBCH block

- If DBTW is supported

  - Support mechanism to indicate or inform that DBTW is enabled/disabled for both IDLE and CONNECTED mode UEs

    - FFS: how to support UEs performing initial access that do not have any prior information on DBTW.

  - PBCH payload size is no greater than that for FR2

  - Duration of DBTW is no greater than 5 ms

  - Number of PBCH DMRS sequences is the same as for FR2

- The following points are additionally FFS:

  - How to indicate candidate SSB indices and QCL relation without exceeding limit on PBCH payload size

  - Details of the mechanism for enabling/disabling DBTW considering LBT exempt operation and overlapping licensed/unlicensed bands

  - Whether or not to support DBTW for SSB SCS(s) other than 120 kHz if other SSB SCS(s) are supported

Agreement:

For CORESET#0 and Type0-PDCCH search space configured in MIB:

- Support {SS/PBCH Block, CORESET#0 for Type0-PDCCH} SCS equal to {120, 120} kHz

  - Support at least SSB and CORESET#0 multiplexing patterns, number of RBs for CORESET#0, number of symbols (duration of CORESET#0) that are supported in Rel-15/16 for {SS/PBCH Block, CORESET#0 for Type0-PDCCH} SCS = {120, 120} kHz.

    - FFS: Supporting additional values

  - FFS: Supported values for SSB to CORESET#0 offset RBs

- If 480kHz SSB SCS that configures CORESET#0 and Type0-PDCCH CSS in MIB is agreed to be supported,

  - Support {SS/PBCH Block, CORESET#0 for Type0-PDCCH} SCS equal to {480, 480} kHz

- If 960 kHz SSB SCS that configures CORESET#0 and Type0-PDCCH CSS in MIB is agreed to be supported,

  - Support {SS/PBCH Block, CORESET#0 for Type0-PDCCH} SCS equal to {960, 960} kHz

- If 240 kHz SSB SCS is agreed to be supported,

  - Support {SS/PBCH Block, CORESET#0 for Type0-PDCCH} SCS equal to {240, 120} kHz

- FFS: any other combinations between one of SSB SCS (120, 240, 480, 960) and one of CORESET#0 SCS (120, 480, 960)

  - FFS: initial timing resolution based on low SCS (120 kHz) and its impact on the performance of higher SCS (480/960 kHz)

Agreement:

For 480 kHz and 960 kHz SSB SCS (if agreed)

- Study further on reserving symbol gap between SSB positions with different SSB index (and possibly between SSB position and other signal/channels)

  - FFS: whether symbol gap is needed for only 960 kHz or both 480 and 960 kHz.

- Study further on reserving gap for UL/DL switching within the pattern accounting possibility for reserving UL transmission occasions in the SSB pattern

- Study should account for inputs from RAN4

Agreement:

- For initial access and non-initial access use cases, support 120kHz PRACH SCS with sequence length L=571, 1151 (in addition to L=139) for PRACH Formats A1~A3, B1~B4, C0, and C2.

- For non-initial access use cases,

  - if 480kHz and/or 960 kHz SSB SCS is agreed to be supported, support 480 and/or 960 kHz PRACH SCS with sequence length L=139 for PRACH Formats A1~A3, B1~B4, C0, and C2, respectively.

    - FFS: support of sequence length L = 571, 1151

- FFS: Support of 480 and/or 960 kHz PRACH SCS for initial access use cases, if 480 and/or 960 kHz SSB SCS is agreed to be supported for initial access

Agreement:

If 480 and/or 960 kHz PRACH SCS is supported, RAN1 should study whether or not the current RA-RNTI calculation and PRACH identification in RAR correctly provides unique identification of PRACH.

# Discussion on initial access aspects

## 2.1 	SS/PBCH block and other initial access signals/channels

### 2.1.1 Subcarrier spacing

As the WID in [2] has made it clear that NR above 52.6 GHz needs to support SCS 480 kHz and 960 kHz for data/control channels and reference signals, a natural question is whether SSB and other initial access signals/channels in the initial BWP need to support these two SCSs?

First, as mentioned above, in order to support larger bandwidths, the data/control channels need to support SCS 480/960 kHz. If the SSB SCS only supports 120 kHz and the uplink transmission adopts a larger SCS e.g. 960 kHz, it will impact on initial uplink timing accuracy. The reason is that SSB is not wide enough in the frequency domain to provide the required time synchronization accuracy. Secondly, if SSB uses a same SCS as data/control channels, single numerology can help reduce complexity. Therefore, SSB should be configurable and at least it can be configured with a same SCS as the data/control channels. Moreover, the larger the SCS, the lower the synchronization complexity. Absolute frequency offset ∆f of a carrier in 60 GHz band is larger than in FR2. Larger SCS 480/960 kHz has a higher tolerance on frequency offset as its ∆f/SCS is low compared with SCS 120 kHz. In addition, SSB supporting SCS 480/960 kHz may help 5G NR to have a unified design to support even higher frequency band in the future, such as above 71 GHz. Above all, we believe 480/960 kHz for SSB is preferred to be introduced in NR above 52.6 GHz. In addition, regardless of SSB SCSs 480/960 kHz are supported only in non-initial access case or in both initial and non-initial access cases, the design should not impact on supporting ANR function and CGI reporting.

In order to reduce the complexity of multiplexing in standardization and implementation, other initial access signals/channels in the initial BWP also need to support SCS 480/960 kHz if SSB is approved to support them.

For SCS 240 kHz, it is necessary to support it if SCS 480/960 kHz is not introduced for SSB. The reason is that 240 kHz can not only guarantee coverage performance, provide a long CP for beam switching, and reuse FR2 initial access process compared to 480/960 kHz, but also improve the time synchronization accuracy and reduce the synchronization complexity that 120 kHz cannot achieve. However, if SCS 480/960 kHz is introduced for SSB, supporting 240 kHz for SSB is unnecessary. Too many SSB SCSs will complicate the multiplexing of SSB and Type0-PDCCH, and the configuration of Type0-PDCCH SCS, etc.

Proposal 1: The following options can be considered for determining SCSs of SSB and other initial access signals/channels in initial BWP, wherein Option 1 is preferred.

- Option 1: both SSB and other initial access signals/channels support SCS (120kHz, 480kHz, 960kHz)

- Option 2: SSB supports SCS (120kHz, 240kHz); Other initial access signals/channels support SCS (120kHz)

Proposal 2: Regardless of SSB SCSs 480/960 kHz are supported only in non-initial access case or in both initial and non-initial access cases, the design should not impact on supporting ANR function and CGI reporting.

### 2.1.2 SSB pattern design

In Rel-15 NR,  SSB patterns in both Case D (120 kHz) and Case E (240 kHz) only consider the multiplexing with SCS 60/120 kHz control/data channels because FR2 only supports SCS 60/120 kHz control/data channels. In the following, we give our views on designing SSB patterns with different SCSs for NR operation above 52.6 GHz.

- Only support single numerology operation

For SSB pattern with 120 kHz, it can reuse the SSB pattern in Case D in FR2.

For SSB pattern with 480/960 kHz, there are three options can be considered: The first is reusing the SSB patterns in one slot in Case A and Case C. The second is reusing the SSB patterns in two slots in Case B and Case D. In addition, Case E can also be considered for designing SSB pattern with 480/960 kHz if the configuration of CORESET and/or search space set is enhanced.

- Support both single and mixed numerology operation

For SSB pattern with 120 kHz, it can reuse the SSB pattern in Case D in FR2.

For SSB pattern with 480 kHz, it can be designed as follows. Case F1: reusing the SSB patterns in Case A and Case C for multiplexing with same and larger SCS channels (e.g. 480 kHz and 960 kHz). Case E2: reusing the SSB patterns in Case E for multiplexing with smaller SCS channels (e.g. 120 kHz).

For SSB pattern with 960 kHz, it can be designed as follows. Case G1: reusing the SSB patterns in Case B and Case D for multiplexing with same and smaller SCS channels (e.g. 960 kHz and 480 kHz).

- Beam switching for SSB with larger SCSs 480/960 kHz

In Rel-15, RAN4 made impact analysis of the beam switching and agreed that “the worst-case beam switching time is hence based on the analogue implementation and is estimated as < 100 ns.”. That is to say 100 ns is the largest time for beam switching according to RAN4’s understanding.

Rel-17 WID on NR operation in 52.6 GHz to 71 GHz [2] has agreed that supporting new SCSs 480 kHz and 960 kHz for data and control channels and reference signals. For SSB and other initial access channels/signals, 480 kHz and 960 kHz may also be used as discussed in section 2.1.1. Table 2.1.2-1 shows the lengths of CP with four SCSs = 120/240/480/960 kHz respectively. As shown in Table 2.1.2-1, we can see at least for SCSs = 120/240 kHz, NCP lengths can cover delay spread, time errors as well as beam switching time. However, for SCSs = 480/960 kHz, larger SCSs reduces the budget for timing errors and beam switching, and thus it is difficult to ensure that beam switching can be done within the lengths of NCP, if beam switching needs a maximum time of 100 ns.

Table 2.1.2-1: the lengths of CP based on four candidate SCSs

| u | SCS (kHz) | Slot length (us) | Symbol length | NCP length ( and ) | NCP length ( or ) |
| --- | --- | --- | --- | --- | --- |
| 3 | 120 | 125 | 8333 ns | 586 ns | 1107 ns |
| 4 | 240 | 62.5 | 4167 ns | 293 ns | 814 ns |
| 5 | 480 | 31.25 | 2083 ns | 146 ns | 667 ns |
| 6 | 960 | 15.625 | 1042 ns | 73 ns | 594 ns |

Although in the TP R4-2103260 to TR 38.808 approved in the RAN4 #98-e meeting, RAN4 thinks both CPs of SCS 480 kHz and 960 kHz are feasible for beam switching, but their analysis may be only from beam switching point of view. We don't know if they take other factors into account, such as delay spread and time errors. RAN1 can continue to wait for reply LS and clarifications from RAN4.

In the following, we provide some options for supporting beam switching for SSB with new SCS 480 kHz and 960 kHz, if the CPs cannot support beam switching and other functions simultaneously.

Option 1: In a half-frame, any two candidate SSBs are discontinuous in the time domain.

There are several ways to design any two candidate SSBs in a half-frame are discontinuous in the time domain.

- Option 1-1: SSB pattern with SCS 480/960 kHz can adopt the existing pattern of Case A and Case C in one or two slots in a half-frame defined in Rel-15 NR, as shown in Figure 2.1.2-1. In Case A and Case C, there are at least two-symbol intervals between any two candidate SSBs.

Figure 2.1.2-1: SSB patterns in Case A, Case B and Case C

- Option 1-2: SSB pattern with SCS 480/960 kHz should be re-designed to reserve at least one symbol between any two candidate SSBs. One possible way is that only defining one candidate SSB per slot, and placing blank symbols before and after the candidate SSB. Another way is to shift one of the two continuous SSBs in Case B/D forward or backward by one or more symbols.

Option 2: Multiple adjacent candidate SSBs are defined to have a same SSB index or QCL assumption. For example, two (e.g. in Case A/B/C/D) or four (e.g. in Case E) adjacent candidate SSBs have a same SSB index or QCL assumption. The gNB transmit one or more of these candidate SSBs with a same SSB index or QCL assumption.

Proposal 3：For designing SSB patterns with different SCSs for NR operation above 52.6 GHz, it is proposed to reuse the existing design (i.e. Case A/C, Case B/D and Case E) as much as possible, and take different impacts in single/mixed numerology operation into account.

Proposal 4: The following options can be considered for supporting beam switching for SSB with SCS 480 kHz and 960 kHz if the CPs cannot support beam switching and other functions simultaneously.

- Option 1: In a half-frame, any two candidate SSBs are discontinuous in the time domain

- Option 1-1: SSB pattern with SCS 480/960 kHz can adopt the existing pattern of Case A and Case C in one or two slots defined in Rel-15 NR

- Option 1-2: SSB pattern with SCS 480/960 kHz should be re-designed to reserve at least one symbol between any two candidate SSBs, e.g.  only defining one candidate SSB per slot, or shift the existing SSB by one or more symbols

- Option 2: Multiple adjacent candidate SSBs are defined to have a same SSB index or QCL assumption

### 2.1.3 DRS/SSB transmission in discovery burst window(s)

RAN has agreed that supporting up to 64 SSB beams for licensed and unlicensed operation in frequency range 52.6 GHz ~ 71 GHz. Accordingly, it is sufficient to define 64 candidate SSBs corresponding to 64 beams in a half-frame. However, in order to guarantee the SSB coverage, improve the SSB detection performance, and increase the transmission opportunities of SSB in unlicensed carrier, more candidate SSBs can be defined in the half-frame. As shown in Figure 2.1.3-1, at least for SCS 240/480/960 kHz, there are many blank symbols in the half-frame can be used for defining new candidate SSBs.

Figure 2.1.3-1: Candidate SSBs in a half-frame

Take Case E with 240 kHz as an example, a total of 128 candidate SSBs can be defined in the half frame to increase SSB transmission opportunities, as shown in Figure 2.1.3-2. One subsequent problem is how to indicate more candidate SSB indexes to UE for timing synchronization, taking the mechanism of Rel-16 NR-U as the reference.

Figure 2.1.3-2: More candidate SSBs defined in a half frame

Observation 1: More than 64 candidate SSBs with SCS larger than 120 kHz can be defined in a half-frame for Rel-17 NR above 52.6 GHz.

The benefits for inclusion of the CSI-RS, RMSI-CORESET, RMSI-PDSCH associated with SSB in addition to the SSB burst set in one contiguous burst (referred to as the Rel-16 NR-U discovery burst) are still valid for defining Rel-17 discovery burst (DB), as copied below:

- Meeting OCB requirement

- Compacting signals in time domain to limit the required number of channel access and for short channel occupancy

- Support of stand-alone NR-U deployments

- Support of ANR functionality in an NR-U deployment

- Resolution of PCI confusion in an NR-U deployment

Therefore, discovery burst still needs to be supported for Rel-17 NR above 52.6GHz. Discovery burst includes at least an SSB and may also include RMSI-CORESET, RMSI-PDSCH and/or NZP CSI-RS. In order to increase the transmission opportunities of discovery burst, it is also necessary to support configuring a discovery burst transmission window (DBTW).

RAN1 in RAN1 #104 e-meeting has agreed that SSB transmission with LBT is supported at least when the conditions for contention exempt short control signaling based SSB transmission is not met. Therefore, supporting DB is not only the requirement of 120 kHz SSB SCS, but also the requirement of other SSB SCSs if they are agreed to be supported.

Proposal 5: Discovery burst (DB) and discovery burst transmission window (DBTW) should be supported for 120 kHz SSB SCS and other SSB SCSs if they are agreed to be supported.

Proposal 6: A discovery burst (DB) in Rel-17 NR above 52.6 GHz includes at least an SSB and may also include RMSI-CORESET, RMSI-PDSCH and/or NZP CSI-RS.

RAN1 has also agreed that PBCH payload size and the number of PBCH DMRS sequences is no greater than or the same as for FR2, if DBTW is supported. In order to reduce the impact of standardization caused by indicating candidate SSB indices, the number of candidate SSB defined in the half-frame can be limited to 128 or kept unchanged (maintain 64) for 240/480/960 kHz SSB SCS. New candidate SSB indices and QCL relation can be indicated via MIB (and/or SIB1), but whether it is feasible depends on the design of other aspects, e.g. SCS combinations of SSB and CORESET#0, subcarrier level offset.

Proposal 7: In order to reduce the impact of standardization caused by indicating candidate SSB indices, the maximum number of candidate SSB defined in the half-frame can be limited to 128 or kept unchanged (maintain 64) for 240/480/960 kHz SSB SCS.

For LBT exempt operation and overlapping licensed/unlicensed bands, we can eliminate the effects of DBTW through configuration implementation. For example, for 120 kHz SSB SCS, the gNB can indicate that the maximum number of beams Q = 64 and the length of DBTW = 5 ms if LBT exempt operation for SSB is allowed.

Proposal 8: For LBT exempt operation and overlapping licensed/unlicensed bands, it is not necessary to enable/disable the DBTW by explicit signaling. The impacts on LBT exempt operation brought by DBTW can be eliminated by configuration implementation.

### 2.1.4 Multiplexing of SSB and Type0-PDCCH

The association of SSB, Type0-PDCCH and RMSI should be studied for better initial access. Rel-15 FR2 supports SSB with SCS 120/240 kHz and Type0-PDCCH with SCS 60/120 kHz. The multiplexing of SSB and Type0-PDCCH includes three types of patterns: Pattern 1 (TDM), Pattern 2 (FDM with different SCSs) and Pattern 3 (FDM with same SCS), as shown in Figure 2.1.4-1. The frequency range of CORESET#0 of Pattern 1 needs to contain SSB. O can be taken as {0, 2.5, 5, 7.5}, which means that the SFN boundary can be offset by SSB with 0, 2.5, 5 or 7.5 ms. The CORESET#0 and SSB of Pattern 2 and 3 are FDMed in frequency domain, and the detection position of Type0-PDCCH is always located in the same slot or previous slot of the associated SSB.

Figure 2.1.4-1: The multiplexing patterns of SSB and CORESET#0

As discussed in section 2.1.1, we suggest both SSB and other initial access signals/channels can support SCS (120kHz, 480kHz, 960kHz). But if supporting three SCSs for Type0-PDCCH and different combinations of SCSs of SSB and Type0-PDCCH, 1 bit of Subcarrierspacingcommon in MIB is not sufficient to indicate the SCS of Type0-PDCCH. Furthermore, in Rel-16 NR-U, Subcarrierspacingcommon in MIB is used to notify NSSBQCL. Therefore, if Rel-17 NR needs using Subcarrierspacingcommon to indicate the SCS of Type0-PDCCH and still needs to notify NSSBQCL, a new mechanism for notifying NSSBQCL needs to be designed.

In order to avoid above specification impacts, we suggest that the SCS of SSB and CORESET#0 on a carrier is always the same for NR operation above 52.6 GHz. There are three supported SCS pairs for multiplexing between SSB and Type0-PDCCH suggested to be considered as follows. In RAN1#104 e-meeting, RAN1 has already agreed that below three supported SCS pairs for multiplexing between SSB and Type0-PDCCH can be supported if 480/960 kHz SSB SCS that configures CORESET#0 and Type0-PDCCH CSS in MIB is agreed to be supported.

- The multiplexing pattern of (SSB, CORESET#0) with SCS (120 kHz, 120 kHz)

- The multiplexing pattern of (SSB, CORESET#0) with SCS (480 kHz, 480 kHz)

- The multiplexing pattern of (SSB, CORESET#0) with SCS (960 kHz, 960 kHz)

If  the SCS of SSB and CORESET#0 in each SCS pair is always the same, above Pattern 1 and Pattern 3 can be used for multiplexing between SSB and CORESET#0. Figure 2.1.4-2 and Figure 2.1.4-3 give some examples of Pattern 3 with above three SCS pairs.

If the SSB pattern with 480/960 kHz reuses the SSB pattern of Case D, as shown in Figure 2.1.4-2, the first symbol index(s) of the corresponding CORESET(s) of the SSB i (, , , ) can be 4, 8, 2, 6, respectively.  and  . Wherein  and  are the SFN and slot index within a frame of the CORESET based on SCS of the CORESET and  and  are the SFN and slot index based on SCS of the CORESET, respectively, where the SS/PBCH block with index  overlaps in time with system frame  and slot .

Figure 2.1.4-2: Pattern 3 for multiplexing between SSB and CORESET#0 if SSB pattern reuses Case D

If the SSB pattern with 480/960 kHz reuses the SSB pattern of Case E, as shown in Figure 2.1.4-3, the first symbol index(s) of the corresponding CORESET(s) of the SSB i  (SSB i, , , , , , ,  )  can be 8, 12, 2, 6, 8, 12, 2, 6, respectively.  and  .

Figure 2.1.4-3: Pattern 3 for multiplexing between SSB and CORESET#0 if SSB pattern reuses Case E

Proposal 9: The following multiplexing patterns for three approved SCS combinations of SSB and Type0-PDCCH can be considered for Rel-17 NR above 52.6 GHz. Other SCS combinations could be precluded.

- (SSB, Type0-PDCCH): SCS (120 kHz, 120 kHz)

- Multiplexing patterns: 1, 3

- (SSB, Type0-PDCCH): SCS (480 kHz, 480 kHz)

- Multiplexing patterns: 1, 3

- (SSB, Type0-PDCCH): SCS (960 kHz, 960 kHz)

- Multiplexing patterns: 1, 3

## 2.2 	PRACH

### 2.2.1 Subcarrier spacing

According to the PRACH related aspects in [3], 120 kHz is already supported for PRACH in initial and/or non-initial use cases. Further study whether to support additional SCS (480 kHz and/or 960 kHz) for PRACH in initial and/or non-initial use cases.

In SI phase, several companies support the opinion that single subcarrier spacing operation may provide benefit on implementation complexity, additional SCSs should be considered for the enablement of single subcarrier spacing operation. As the agreement in [3] shows that it should be first determined the SCS of SSB and whether single SCS operation is supported or not before introducing additional SCSs for PRACH.

For both initial access and/or non-initial case, the format is configurable, so the configuration of different PRACH format to meet the PRACH requirements of the cell radius. As shown in Table 2.3.1-1, the maximum cell radius can be 312m for 480 kHz.

Table 2.3.1-1: the maximum cell radius for different PRACH SCS

| PRACH SCS | Format | Maximum Cell radius(meter) |
| --- | --- | --- |
| 480 KHz/960KHz | A1 | 44/22 |
| 480 KHz/960KHz | A2 | 88/44 |
| 480 KHz/960KHz | A3 | 132/66 |
| 480 KHz/960KHz | B1(A1/B1) | 10/5 |
| 480 KHz/960KHz | A2/B2 | 32/16 |
| 480 KHz/960KHz | A2/B2 | 57/27 |
| 480 KHz/960KHz | B4 | 120/60 |
| 480 KHz/960KHz | C0 | 167/83 |
| 480 KHz/960KHz | C2 | 312 /156 |
| Note: Path profile is not considered. |  |  |

Proposal 10: Support additional SCSs (480 kHz and/or 960 kHz) for PRACH and SSB if single subcarrier spacing operation is supported.

### 2.2.2 Sequence length

As agreed in 104-e meeting, the sequence length of 139, 571 and 1151 has already been supported for initial access and non-initial use cases when the PRACH SCS is 120 kHz. The sequence length of 139 will be supported for non-initial access use case if the PRACH SCS is 480kHz and/or 960 kHz. If 480kHz and/or 960 kHz for PRACH SCS is agreed to be supported, there is no reason to exclude these two sequence lengths, so the sequence length of 571 and 1151 should be supported.

Proposal 11: Support sequence length 571 and 1151 for PRACH in non-initial use cases.

### 2.2.3 RA-RNTI

The RA-RNTI associated with the PRACH occasion in which the preamble is transmitted, is computed as:

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

- s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14)

- t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80)

- f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8)

- ul_carrier_id is the UL carrier used for preamble transmission (0 for NUL carrier, and 1 for SUL carrier)

The calculation above uses t_id < 80 which assumes a system frame and a maximum SCS of 120 kHz (80 slots in a frame). For higher SCSs (480 and/or 960 kHz), the number of slots in a frame is larger than 80 (320 slots for SCS = 480 kHz and 640 slots for SCS = 960 kHz). Hence, keeping the equation unchanged (with t_id ≥ 80 slots) will make the value of RA-RNTI exceed the range (0, 65535)..

In addition, in NR Rel-16, MsgB RAR window and NR-U RAR window were extended to 40 ms by including the 2-bit LSB of the SFN corresponding to the PRACH occasion which is used to transmit the preamble of MsgA and Msg1.

There could be several solutions to resolve this issue. In this paper, we just propose two options for RA-RNTI enhancement with minimum specification impact.

Option 1: Change the equation of RA-RNTI calculation, without additional signalling overhead

Option 2: Reuse the same RA-RNTI equation in NR Rel-16, divide the system frame into N segments (each segment is 80 slots using the used SCS), and signal the segment index that transmit the preamble in the DCI.

The benefit of Option 1 is that, it can reduce the signalling overhead. Besides, the optimization of equation can be considered for further extension in future when even larger SCS is introduced or the RAR window is extended. The exact equation can be further studied.

As for Option 2, reuse the NR RA-RNTI equation with additional signaling. For example, for 960 kHz PRACH SCS, there would be 640 slots in a system frame. Then it can be divided into 8 segments, with each segment including 80 slots. Therefore, we can signal a 3 bits indication in the DL DCI to indicate which segment the PRACH was sent in. Similarly, for SCS 480 kHz, we need 2 bits (corresponding to 4 segments).

Among the 2 options, they both have pros and cons, so we don’t have strong preference.

Proposal 12: For higher PRACH SCS (480 and/or 960 kHz), consider the following options for RA-RNTI enhancements:

- Option 1: Change the equation of RA-RNTI calculation, without additional signalling overhead

- Option 2: Reuse the same RA-RNTI equation in NR Rel-16, divide the system frame into N segments (each segment is 80 slots using the used SCS), and signal the segment index that transmit the preamble in the DCI.

# Conclusion

In this contribution, we discuss intial access aspects in NR above 52.6 GHz band operation and have the following observations and proposals.

Observation 1: More than 64 candidate SSBs with SCS larger than 120 kHz can be defined in a half-frame for Rel-17 NR above 52.6 GHz.

Proposal 1: The following options can be considered for determining SCSs of SSB and other initial access signals/channels in initial BWP, wherein Option 1 is preferred.

- Option 1: both SSB and other initial access signals/channels support SCS (120kHz, 480kHz, 960kHz)

- Option 2: SSB supports SCS (120kHz, 240kHz); Other initial access signals/channels support SCS (120kHz)

Proposal 2: Regardless of SSB SCSs 480/960 kHz are supported only in non-initial access case or in both initial and non-initial access cases, the SSB design should not impact on supporting ANR function and CGI reporting.

Proposal 3：For designing SSB patterns with different SCSs for NR operation above 52.6 GHz, it is proposed to reuse the existing design (i.e. Case A/C, Case B/D and Case E) as much as possible, and take different impacts in single/mixed numerology operation into account.

Proposal 4: The following options can be considered for supporting beam switching for SSB with SCS 480 kHz and 960 kHz if the CPs can not used to support beam switching and other functions simultaneously.

- Option 1: In a half-frame, any two candidate SSBs are discontinuous in the time domain

- Option 1-1: SSB pattern with SCS 480/960 kHz can adopt the existing pattern of Case A and Case C in one or two slots defined in Rel-15 NR

- Option 1-2: SSB pattern with SCS 480/960 kHz should be re-designed to reserve at least one symbol between any two candidate SSBs, e.g.  only defining one candidate SSB per slot, or shift the existing SSB by one or more symbols

- Option 2: Multiple adjacent candidate SSBs are defined to have a same SSB index or QCL assumption

Proposal 5: Discovery burst (DB) and discovery burst transmission window (DBTW) should be supported for 120 kHz SSB SCS and other SSB SCSs if they are agreed to be supported.

Proposal 6: A discovery burst (DB) in Rel-17 NR above 52.6 GHz includes at least an SSB and may also include RMSI-CORESET, RMSI-PDSCH and/or NZP CSI-RS.

Proposal 7: In order to reduce the impact of standardization caused by indicating candidate SSB indices, the maximum number of candidate SSB defined in the half-frame can be limited to 128 or kept unchanged (maintain 64) for 240/480/960 kHz SSB SCS.

Proposal 8: For LBT exempt operation and overlapping licensed/unlicensed bands, it is not necessary to enable/disable the DBTW by explicit signaling. The impacts on LBT exempt operation brought by DBTW can be eliminated by configuration implementation.

Proposal 9: The following multiplexing patterns for three approved SCS combinations of SSB and Type0-PDCCH can be considered for Rel-17 NR above 52.6 GHz. Other SCS combinations could be precluded.

- (SSB, Type0-PDCCH): SCS (120 kHz, 120 kHz)

- Multiplexing patterns: 1, 3

- (SSB, Type0-PDCCH): SCS (480 kHz, 480 kHz)

- Multiplexing patterns: 1, 3

- (SSB, Type0-PDCCH): SCS (960 kHz, 960 kHz)

- Multiplexing patterns: 1, 3

Proposal 10: Support  additional SCSs (480kHz and/or 960kHz) for PRACH and SSB if single subcarrier spacing is supported.

Proposal 11: Support sequence length  571 and 1151 for PRACH in non-initial use cases.

Proposal 12: For higher PRACH SCS (480 and/or 960 kHz), consider the following options for RA-RNTI enhancements:

- Option 1: Change the equation of RA-RNTI calculation, without additional signalling overhead

- Option 2: Reuse the same RA-RNTI equation in NR Rel-16, divide the system frame into N segments (each segment is 80 slots using the used SCS), and signal the segment index that transmit the preamble in the DCI.

# Reference

- RP-193259, New SID: Study on supporting NR from 52.6GHz to 71GHz. Intel, Sitges, Spain, RAN#86, December 9 – 12, 2019

- RP-202925, Revised WID: Extending current NR operation to 71 GHz, CMCC, 3GPP TSG RAN Meeting #90-e, 7-11 December 2020

- Draft_Minutes_report_RAN1#104-e_v030, 2021-02