3GPP TSG-RAN WG1 Meeting #104-bis	R1-2103543

Online, April 12th-20th 2021

Agenda Item:	8.1.4

Source:	Ericsson

Title:	CSI enhancements for Multi-TRP and FR1 FDD reciprocity

Document for:	Discussion, Decision

# 1	Introduction

The WID [1] objective 4 states a task for evaluating (and, if needed, specifying) enhancements of NR CSI in two parts:

- Enhancement on CSI measurement and reporting:

  - Evaluate and, if needed, specify CSI reporting for DL multi-TRP and/or multi-panel transmission to enable more dynamic channel/interference hypotheses for NCJT, targeting both FR1 and FR2

  - Evaluate and, if needed, specify Type II port selection codebook enhancement (based on Rel.15/16 Type II port selection) where information related to angle(s) and delay(s) are estimated at the gNB based on SRS by utilizing DL/UL reciprocity of angle and delay, and the remaining DL CSI is reported by the UE, mainly targeting FDD FR1 to achieve better trade-off among UE complexity, performance and reporting overhead

# 2	CSI for FR1 FDD reciprocity

The following agreements were made in RAN1#104-e:

Agreement

For PS codebook enhancements utilization DL/UL reciprocity of angle and/or delay, support codebook structure W=W1W2 WfH where

- W1 is a free selection matrix, with identity matrix as special configuration

  - FFS polarization-common/specific selection

- Wf is a DFT based compression matrix in which N3 = NCQISubband*R and Mv>=1

  - At least one value of Mv>1 is supported

    - Decide on the value(s) of Mv, e.g. Mv=2,  in RAN1# 104bis-e

  - Working assumption:  Support of Mv>1 is a UE optional feature if the UE supports Rel-17 PS codebook enhancement, taking into account UE complexity related to codebook parameters

  - FFS candidate value(s)  of R, mechanism for configuring/indicating to the UE and/or mechanism for selecting/reporting by UE for Wf

- Wf can be turned off by gNB. When turned off, Wf  is an all-one vector (FFS; the length of all-one vector)

- FFS other signaling/CSI reporting mechanism for trade-off among signaling overhead, UE complexity and UPT gain

Agreement

For PS codebook enhancements utilization DL/UL reciprocity of angle and/or delay,

- W1 ∈ N^{P×K1} (K1≤P) is a port selection matrix in order to freely select K1 ports out of P CSI-RS ports or K1/2 ports out of P /2 CSI-RS ports

- Note that P is the number of CSI-RS ports for port selection (whose value depends on the outcome of the CSI-RS related study).

Agreement

For PS codebook enhancements utilization DL/UL reciprocity of angle and/or delay, study following options (or combinations) for CSI-RS configurations associated with Rel-17 PS codebook for supporting low CSI-RS overhead and/or CSI-RS processing complexity considering the impact on UPT performance under realistic CSI-RS measurement:

- Option 0: No further CSI-RS enhancement as the baseline

- Option 1: Support configuring a lower CSI-RS density per CSI-RS resource, e.g. 0.25

- Option 2: Support configuring one or multiple CSI-RS patterns per CSI-RS resource associated with Rel-17 PS codebook

- Option 3: Support configuring multiple CSI-RS resources per CSI reporting configuration associated with Rel-17 PS codebook

## 2.1       Deciding on codebook parameters

Based on the RAN1#104e agreements, a few codebook parameters are to be studied, including

- the structure of W1 (whether port selection is polarization-common or polarization-specific),

- the number of FD basis vectors in Wf, denoted Mv

- the number of PMI subbands per CQI subband, R.

### 2.1.1 Simulation setup

Simulations have been conducted based on the agreed evaluation methodology assumptions in [2], see a detailed description of the used procedures in Section 2.3, and also the Appendix for additional parameters.

For comparison, both R16 regular and port-selection eType II with paramCombination-r16 #1-6 are simulated. R16 PS eType II with paramCombination-r16 #1 is used as benchmark. In the simulations, a 32-port antenna is used at the gNB and 2 antennas are used at the UE.

The results below are shown for MU-MIMO with rank-1 per UE at a traffic load corresponding to the very high 70% resource utilization for the benchmark case. For R17 PS, results are calculated based on the procedure described in Section 2.3.

In all simulated cases, CSI-RS is transmitted every 5 ms whenever there is an active user. An average CSI-RS overhead per cell per slot is calculated by normalizing the total number of CSI-RS ports, which is given by the total number of active users times the number of CSI-RS ports per user, by the number of cells and CSI-RS periodicity. The obtained average CSI-RS overhead is taken into account for throughput calculation.

For R17, the UL overhead may consist of bits for indicating the selected ports, the selected FD basis vectors, the NZC bitmap, the strongest coefficient, and for reporting the quantized NZCs. The actual overhead depends on the scheme that is used. In our simulations, we performed layer-common and polarization-specific port selection unless otherwise stated, therefore the number of bits for indicating the selected ports equals log2PCSI-RSK1, where PCSI-RS is the number of precoded CSI-RS ports by gNB, while K1 is the number of selected CSI-RS ports by UE.

For FD basis vectors selection, UE follows gNB indication, thus, no additional overhead is needed. For reporting of W2, a bitmap of size K1M is used for indicating the locations of NZCs, with Mv being the number of FD basis vectors in Wf for laver v. Each NZC is reported with 7 bits, that is 3 bits for amplitude and 4 bits for phase. Polarization reference amplitude is not used herein. The total number of NZCs for layer v is given by βK1Mv, where β=1, 0.5, 0.25 determines of ratio of reported coefficients in W2.

### 2.1.1 On polarization-common or polarization-specific W1

In Figure 1, we compare the performance achieved by polarization-common and polarization-specific port selection matrix, W1. The Rel-16 Type II, both regular and port-selection, are illustrated as reference. Both Rel-16 schemes use 32 CSI-RS ports and parameter combination 1-6. For all the schemes in the figure, the relative gains are normalized with respect to the Rel-16 PS scheme with parameter combination 1. We denote PCSI-RS as the number of CSI-RS ports transmitted at gNB, K1 as the total number of selected CSI-RS ports by the UE, β as the ratio of non-zero coefficients (NZCs). We use M=1, i.e., a single DFT FD basis (DC component) for the Rel-17 schemes. To compare polarization-common and polarization-specific port selection, we consider PCSI-RS=16, K1=8 and PCSI-RS=32, K1=16, i.e., half of the precoded ports are selected.

For both Rel-17 schemes in the figure, polarization-specific port selection gives slightly higher mean UPT gain, around 1-5%, at the expense of slightly higher UL overhead. The performance gain comes from additional flexibility of selecting ports, since polarization-common port selection is a special case of polarization-specific port selection. However, the additional flexibility also introduces slightly increased reporting overhead.

It is also noted that when the number of reported NZCs increases, i.e., more SD-FD pairs are selected by UE, the gain by having port-specific selection is less. For example, there is about 1% gain for PCSI-RS=32, K1=16 and β=1, comparing to, say PCSI-RS=16, K1=8 and β=1, where 4% gain is seen. This could be due to that with larger number of selected ports, K1, most dominant clusters have already been selected, so that the gain by having more flexible selection may diminish. In fact, when PCSI-RS=K1, there is no difference between polarization-specific/-common port selection, then both PCSI-RS= K1=32 and PCSI-RS= K1=16 will give no additional gain by having polarization-specific selection.

Despite of this, for most cases we can still gain a few percent in UPT, and the additional overhead is quite small. In addition, real world channels may sometimes differ from standardized 3GPP channels and such polarization specific port selection provides an additional robustness.   Therefore polarization-specific port selection should be supported.

Figure 1 Performance with polarization-common and polarization-specific port selection.

Polarization-specific port selection gives 1-5% gain in throughput, with slightly increased UL overhead.

Support polarization-specific port selection.

### 2.1.2 On the number of FD basis vectors

Figure 2 investigates the impact of having different number of FD basis vectors, Mv=1, 2 and 4. For the Rel-17 schemes shown below, we assume PCSI-RS= K1 and β=1, i.e., all beamformed CSI-RS ports are selected by the UE and all coefficients are reported. A window-based CSI-RS precoding is used at the gNB, and a window-based UE filtering is used at the UE, more details are described in Section 2.3.

Note that the window-based CSI-RS precoding and UE filtering is able to combat imperfections such as non-ideal delay reciprocity in FDD (in fact what is needed is reciprocal relative delay for different channel taps, not absolute delay), which cannot be captured by the used 3GPP channel model. Such gain is thus not seen from simulations but may happen in real networks.

Robustness of having Mv>1 against non-ideal delay reciprocity is not seen in the simulations due to the used 3GPP channel model

It can be observed that increasing the number of FD basis vectors can improve the performance by a few percent. In particular, for PCSI-RS= K1=8, increasing M from 1 to 2 gives 5% UPT gain while further increasing Mv to 4 gives additional 3% gain. Similar is seen for PCSI-RS= K1=16 and 32, but the UPT gain tends to saturate, especially by having Mv=4. This should be straightforward to understand: since the window-based CSI-RS precoding method as described in Section 2.3 is used, the total number of precoded ports is given by PCSI-RS, which corresponds to PCSI-RSMv precoded SD-FD basis pairs. The actual required number of precoded SD-FD pairs is determined by the number of dominant clusters of the propagation channel, therefore when PCSI-RS is sufficiently large, further increasing Mv will not provide much gain. Improved UPT may come at the cost of higher UL overhead if all coefficients are reported. Based on the above discussion, we think that Mv=2 is a good tradeoff among performance, robustness, complexity and overhead.

Figure 2 Performance with 1, 2, 4 FD basis vectors.

Mv=2 is a good tradeoff among performance, robustness, complexity and overhead.

Support Mv=1 and 2, larger value of Mv is not needed.

### 2.1.3 On the value of R

In Figure 3, we studied the impact by having different number of PMI subbands per CQI subband, R. For the simulations considered herein, the CSI-RS density is 1 and the CQI subband size is 8 PRBs, for R=8, 4, 1, the corresponding PMI granularity are 1, 2, 8 PRBs, respectively. The PMI subband size affects the PMI calculation via the number FD basis vector candidates, i.e., the number of basis vectors in Wf. It is noted that when the number of FD basis vectors is Mv=1, the UE calculates the PMI based on wideband filtering of the channel by applying the DC DFT FD component, whose size is the total number of PMI subbands, to the estimated DL channel, such averaging is not affected by the R value. Therefore, we showed only the case with Mv=2.

In our simulation, since UE follows gNB indication for FD basis selection, the R value will not have any impact on UL overhead. Based on the results shown in Figure 3, there is a marginal loss by decreasing R from 8 to 4, say around 1-2%, while further decreasing R from 4 to 1 will cause a performance loss of approximately 5%. The R value affects the UE complexity through averaging/equalizing the DL channel in FD domain, there is no significant complexity increase by having a R value that allows per PRB PMI granularity, at least for the 20MHz bandwidth case considered herein. If we denote NSBCQI as the CQI subband size in PRBs, RAN1 should consider supporting larger R values, say R=NSBCQI. The reason why large R value for Rel-16 Type II is not feasible is that R will also affect the number of FD basis candidates, which increases UE complexity for selection. Such problem does not exist in Rel-17, since UE can either follow gNB indication, as what we assume here, or UE can choose FD basis from an indicated set of FD basis which contains fewer candidates and does not depend on R. Thus, small R values are unnecessary and should not be considered in Rel-17.

Figure 3 Performance with R=1, 4 and 8 PMI subbands per CQI subband.

The R value does not affect overhead for Rel-17 Type II PS codebook. Having R as large as the CQI subband size gives the best performance without significantly increase UE complexity.

RAN1 should consider supporting large R values, for example R=NSBCQI, where NSBCQI is the CQI subband size.

## 2.2       On CSI-RS extensions

### 2.2.1 CSI-RS density

The impact of CSI-RS density ρ is studied in Figure 4. In this figure, we set PCSI-RS= K1=8, 16, 32, β=1 and Mv=1, while ρ=1, 0.5, 0.25 is the only variable. In the simulation, transmission power for CSI-RS is not boosted even for reduced CSI-RS density ρ=0.5, 0.25, since in our opinion using the saved resources to co-schedule other UEs is the intention of saving CSI-RS overhead. Based on these assumptions, reducing CSI-RS density will cause worse channel estimation, due to a power loss as well as channel interpolation for unsounded PRBs. Therefore, a performance loss is foreseen. On the other hand, however, reducing CSI-RS density can reduce the corresponding overhead and thus may increase the number of resource elements for data, which gives a higher throughput. Figure 4 shows the combined impacts that are discussed above, in which we can see that decreasing the CSI-RS density ρ from 1 to 0.5 introduces about 1-2% performance loss, while further decreasing ρ to 0.25 introduces about additional 2-3% loss. The results indicate that, when CSI-RS density is reduced, the performance loss due to worse channel estimation outweighs the performance gain by having lower CSI-RS overhead. Hence, we don’t see the need for supporting lower CSI-RS density ρ=0.25.

Figure 4 Performance with CSI-RS density 1, 0.5 and 0.25.

When CSI-RS density is reduced, the performance loss due to worse channel estimation outweighs the performance gain by having lower CSI-RS overhead.

Do not support lower CSI-RS density, e.g., ρ=0.25.

### Multiple CSI-RS resources

A main objective of this agenda is to reduce UE complexity. We think that UE complexity can be reduced even further by supporting distribution of the ports over multiple symbols. We make the following observation.

For 16 and 32 port CSI-RS resources, each OFDM symbol contains at least 8 ports

This means that the UE need to process 8 ports in parallel when it comes to channel estimation. In addition, the gNB need to perform precoding and delay compensation for 8 ports in parallel. To reduce the implementation complexity for both UE and gNB, we suggest to support Option 3, where multiple CSI-RS is configured per CSI reporting configuration, and used as CMR.

An example is shown in Figure 5, where a 32 ports CSI-RS resource (left) use 8 ports per OFDM symbol but is in Rel.17 extended to a merge of 8 four port resources (right).

Figure 5 If Option 3 is supported, then the number of CSI-RS ports to be processed per OFDM symbol can be reduced from 8 to 4 for a 32 port resource.

The following table shows how the number of ports to be processed reduces when it is possible to spread the ports across more OFDM symbols by merging existing CSI-RS resources of lower number of ports to obtain a larger number of ports. It is assumed here that up to 8 symbols can be used for CSI-RS in a slot.

Table 1 The maximum number of CSI-RS ports a UE and gNB need to process in an OFDM symbol in Rel.15 and for Rel.17 if Option 3 is agreed (assuming at most 8 symbols can be used for CSI-RS in a slot)

It is noted here that no new CSI-RS resources are introduced, so there is no need to change 38.211 specification on the definition of a CSI-RS resource and existing implementations for CSI-RS channel estimation can be reused. So for the 32 port example, 8 resources with 4 ports using FD-cdm2 is merged.

We believe that Option 2 could also obtain the same benefit, but requires introducing new CSI-RS resource definitions in 38.211, which is a drawback of Option 2. Based on these motivations, we propose to support Option 3:

Support Option 3: Support configuring multiple CSI-RS resources per CSI reporting configuration associated with Rel-17 PS codebook

## 2.3	Description of used gNB and UE procedures for studying port selection codebook enhancement

In this section we describe the gNB and UE procedures that have been used for the evaluations of Type II port selection codebook enhancement that are presented in Section 2.1.

### 2.3.1	CSI-RS precoding at the gNB

Based on an estimated UL channel matrix the gNB determines SD and FD bases for the CSI-RS precoding. The selection of SD and FD bases is up to gNB implementation. In this section we describe a DFT-based CSI-RS precoding scheme that has been used for the evaluations presented in Section 2.1.

For each CSI-RS port, a pair of SD basis vector, bp∈CNt×1, and FD basis vector, fp∈CNRB×1,  are jointly selected to precode CSI-RS port p, for p=1, …, PCSI-RS, where PCSI-RS is the number of CSI-RS ports, while Nt and NRB are the number of gNB antenna ports and resource blocks (RBs), respectively. The joint SD-FD precoder for CSI-RS port p, denoted by Wp∈CNt×NRB, is then given by Wp=bpfpH . The SD and FD precoding vectors are taken from a set of rotated DFT matrices, where the number of rotated DFT matrices are given by the corresponding oversampling factors. For each SD basis matrix Br1,r2 with rotations r1,r2 in azimuth/elevation domain, and FD basis matrix Frf with rotation rf in delay domain, a corresponding angle/delay power spectrum is calculated according to

m=1NrBr1,r2HUL,mFrfH2

where HUL,m is the Nt×NRB estimated UL channel matrix for UE antenna port m, and Nr is the number of UE antenna ports. The best rotations are determined by summing the powers of the PCSI-RS strongest SD-FD pairs in the angle/delay spectrum for each rotation. The SD-FD pairs for CSI-RS precoding are then determined by selecting the PCSI-RS strongest SD-FD pairs from the basis matrix with the best rotation. In case of polarization-common port selection, the same precoder is applied on two orthogonal polarizations for each SD-FD pair. In this case, the angle-delay power spectrum is averaged over polarizations and the PCSI-RS/2 strongest SD-FD pairs are selected to precode each polarization.

### 2.3.2	UE measurement and reporting

The UE measures the DL channel based on the configured CSI-RS ports. First, since delay has been pre-compensated in the precoded channel which reduces frequency selectivity, an FD compression can be performed by the UE by equalizing the estimated DL channel over frequency. Due to delay pre-compensation, it is expected that most of the channel information from different ports will be captured in the same tap, e.g. the tap corresponding to zero delay. In this case, a single DC DFT component can be used for averaging the channel over subbands. In a more general case where multiple dominant taps are observed, Mv>1 DFT components can be used to equalize the channel over subbands.

Denote the FD compression matrix as Wf∈CN3×Mv and the estimated DL channel for the precoded CSI-RS ports as HDL(1), …,HDL(N3), where HDL(n)∈CNr×PCSI-RS for n=1, …, N3, and N3 is the number of subbands. The FD compressed channel can then be written as HDLm=n=1N3HDL(n)Wf*(n,m), for m=1, …, Mv. After FD compression, the UE selects K1 out of the PCSI-RS ports. Let HDL=HDL(1)⋯HDL(Mv)∈CNr×K1Mv , where HDL(m) is the Nr×K1 channel matrix for the K1 selected ports compressed with the m-th FD component. The coefficient matrix Cl∈CK1×Mv for layer l is then given by Cl=cl(1:K1)⋯cl(K1Mv-1+1:K1Mv) where  cl is the K1Mv×1 l-th right singular vector of HDL. The UE then reports a selected number of non-zero coefficients in Cl to the gNB in the PMI report.

The above procedure for CSI-RS precoding and UE PMI calculation is further explained in Figure 6 and Figure 7 for Mv=1 and Mv=2, respectively. In these examples, there are 8 clusters in the original channel, tagged as A-G. Clusters that are located in the same direction are coded with the same color. In Figure 6, the UE is configured with Mv=1 FD basis vector. In this case, gNB pre-compensates the delay for each of the 8 CSI-RS ports such that they are aligned in the delay domain when received by the UE. The UE then filters the corresponding tap of the beamformed DL channel with a window of size 1 (represented by the red box) and calculates PMI based on the filtered channel. In Figure 7, another example is shown, where the gNB precodes 4 CSI-RS ports and the UE is configured with Mv=2 adjacent FD basis vectors. In this case, the gNB pre-compensates delay with 4 windows of size 2, where each window contains 1-2 taps. Delay pre-compensation is applied to the selected windows during CSI-RS precoding such that they are aligned in delay domain. The UE will then filter the corresponding taps of the beamformed DL channel with a window of size 2, based on which PMI is calculated. In this example, we implicit assumes that cluster H is the weakest and can be ignored during CSI-RS precoding; otherwise, gNB can use an additional CSI-RS port to cover that cluster. Note that the example in Figure 6 can be treated as a special case of Figure 7, where a size-1 window is used. The above window-based CSI-RS precoding is transparent to the UE and can be applied to any value of Mv.

Figure 6 An example of gNB CSI-RS precoding and UE PMI calculation with Mv=1.

Figure 7 An example of gNB CSI-RS precoding and UE PMI calculation with Mv=2.

### 2.3.3	PDSCH precoding

Based on the reported PMI from the UE the gNB reconstructs the precoder for that UE by combining the PMI with the applied CSI-RS precoder. The reconstructed Nt×1 precoding vector, Vn, for subbband n and the first layer can be obtained as

Vn=WCSI-RS,nWn

where WCSI-RS,n is the Nt×PCSI-RS CSI-RS precoding matrix for subband n and Wn is the n-th column of the reported W=W1W2WfH.

By combining reports from multiple UEs the gNB can determine a precoder for MU-MIMO PDSCH transmission, e.g., an MMSE or SLNR precoder.

# 3	CSI for DL multi-TRP and/or multi-panel transmission

The following agreement was made in RAN1#103-e:

Agreement

For CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT, [at least for multi-DCI based and single-DCI based schemes (scheme 1a)], NZP CSI-RS resources for channel measurement are associated to different TRPs/TCI states at resource level

- CMRs corresponding to different TRPs respectively shall be configured within the same resource set (i.e. scheme 1-2) and have the same number of ports among CMRs.

- At least ‘typeI-SinglePanel’ codebook is supported

  - FFS: Other codebook types

- Note that RAN1 shall strive to finalize NCJT CSI enhancement with single reporting setting firstly.

- The support of larger than 32 ports across two CMRs is optional for a UE supporting Rel. 17 mTRP CSI

And the following agreements were made in RAN1#104-e:

Agreement

For CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT, the UE can be configured with Ks ≥ 2 NZP CSI-RS resources in a CSI-RS resource set for CMR and N ≥ 1 NZP CSI-RS resource pairs whereas each pair is used for a NCJT measurement hypothesis

- Configure UE with two CMR groups with Ks=K1+K2 CMRs. CMR pairs are determined from two CMR groups by following method(s).

  - K1 and K2 are the number of CMRs in two groups respectively. FFS K1=K2 or different K1/K2.

  - Note that CMRs in each CMR group can be used for both NCJT and Single-TRP measurement hypotheses

  - N CMR pairs are higher-layer configured by selecting from all possible pairs

    - signalling mechanism can be discussed further, e.g. using a bitmap

    - FFS: Whether MAC-CE or RRC+MAC CE indication is needed

    - FFS: how to support NCJT measurement hypotheses in FR2

- Support N=1 and Ks =2, FFS other maximal values of N>1 and Ks>2

- Note: for CPU/resource/port occupation, NCJT hypothesis is considered separately from single TRP hypothesis

Agreement

- Strive to agree at most one of the following options, if needed

  - Option 1: Confirm the Working Assumption from RAN1 103e.

  - Option 2: The UE can be expected to report one RI, one PMI, one LI and one CQI per TRP, up to 2 TRPs, for Multi-DCI based NCJT

- The time of decision is RAN1#105e (May 2021)

Agreement

For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting, support following two options:

- Option 1: the UE can be configured to report X CSIs associated with single-TRP measurement hypotheses and one CSI associated with NCJT measurement hypothesis

  - X = 0, 1, 2

    - If X=2, two CSIs are associated with two different single-TRP measurement hypotheses with CMRs from different CMR groups

    - Support of X=1,2 is UE optional for the UE supporting option 1

  - FFS omission of CSI associated with NCJT measurement hypothesis

- Option 2: the UE can be configured to report one CSI associated with the best one among NCJT and single-TRP measurement hypotheses

  - FFS how to report recommended measurement hypothesis associated with that CSI report

### 3.1  CSI measurement

For NC-JT CSI enhancement with single reporting setting, it was agreed in RAN1#104-e that the UE can be configured with Ks NZP CSI-RS resources in a CSI-RS resource set for CMR and where the Ks NZP CSI-RS resources can be divided in to two CMR groups where each CMR group consists of K1 and K2 CMRs respectively (and where Ks=K1+K2). Furthermore, it was agreed that the UE can be higher layer configured with N CMR pairs, where each CMR pair consist of one CMR from each CMR group, and where each CMR pair is used as a NCJT CSI measurement hypothesis (and possibly two single-TRP measurement hypotheses).

Since computing multiple NC-JT CSI measurement hypotheses would increase UE computation, it is preferred that the number N of CMR pairs higher layer configured for NC-JT CSI measurement hypotheses is kept to a moderate  number, say up to N=4.

For the number of CMR pairs indicated for NC-JT CSI hypotheses,  support  up to N=4.

In Rel-15, up to 8 CSI-RS resources can be configured in a CSI resource set. In FR1, each CSI-RS resource is associated with a TRP while in FR2, each CSI-RS resource can also be associated with a beam. With 8 CSI-RS resources, it can support 4 beams from each TRP for two TRPs in FR2 and a cluster of 3 or 4 TRPs in FR1.   Hence, for the number of NZP CSI-RS resources in the CSI-RS resource set for CMR, agreeing a value of Ks = 8 may be reasonable.  Regarding the number of CMRs in each CMR group (i.e., K1 and K2), we do not see any strong need to restrict K1 to be equal to K2.  In our view, K1 and K2 can be either equal or different.

Support up to Ks=8 CMR resources in the CSI-RS resource set for a multi-TRP CSI report setting.

In NR Rel-17, the number of CMRs in the two CMR groups, K1 and K2, can be either equal or different.

In a serving cell with M TRPs, and where one CMR is configured per TRP, which N CMR pairs that the UE should calculate NC-JT CSI measurement hypotheses for is likely to change time to time depending on UE position, traffic load (availability) of the different TRPs etc. One example of this is illustrated in Figure 8, where a coordination cluster consists of M=4 TRPs. In this example, a UE can be configured with a Multi-TRP CSI report setting wherein the CSI-RS resource set for CMR is configured with Ks = 4 CMR resources.  Each of the Ks = 4 CMR resources correspond to one of the TRPs.  Let us assume that CMR1-CMR2 are in the 1st CMR group, and CMR3-CMR4 are in the 2nd CMR group.  Since each CMR pair should consist of one CMR from each CMR group, there exists the following candidate CMR pairs in this example

- CMR1 & CMR3

- CMR 1 & CMR4

- CMR2 & CMR3

- CMR2 & CMR 4

As mentioned above, which CMR pair among the candidate CMR pairs gets higher layer configured to the UE may change from time to time depending on UE position, etc.  If the N CMR pairs are RRC configured and the CMR pairing needs to be updated, then an RRC reconfiguration would be needed which is cumbersome.  Hence, there is a need to dynamically update the CMR pairs.  We propose to indicate the CMR pairs to the UE via either MAC-CE or RRC+MAC CE.  The detailed signalling for the CMR pairs can be discussed further.

Support dynamic update of CMR pairs for NC-JT CSI measurement hypothesis using MAC-CE and/or RRC signalling

Figure 8.  Coordination cluster of M = 4 TRPs

### 3.2  CSI reporting mechanism

For NC-JT CSI enhancement with single reporting setting, it was agreed to support the following two reporting options in the last meeting, i.e.,

- Option 1: the UE can be configured to report X CSIs associated with single-TRP measurement hypotheses and one CSI associated with NCJT measurement hypothesis

  - X = 0, 1, 2

    - If X=2, two CSIs are associated with two different single-TRP measurement hypotheses with CMRs from different CMR groups

    - Support of X=1,2 is UE optional for the UE supporting option 1

  - FFS omission of CSI associated with NCJT measurement hypothesis

- Option 2: the UE can be configured to report one CSI associated with the best one among NCJT and single-TRP measurement hypotheses

  - FFS how to report recommended measurement hypothesis associated with that CSI report

For enabling flexible scheduling between single TRP and NC-JT, the gNB should preferably, in addition to the reported NCJT CSI, have single TRP CSI for the two associated TRPs, which would correspond to setting X equal to 2 in Option 1. However, one drawback with this solution is the large CSI feedback overhead, since one NCJT and two single-TRP CSIs must be reported, which would require the UE to fed back the following:

● 2 RIs, 2 PMIs, and 1 CQI for NC-JT CSI

● 2 pairs of RI, PMI and CQI for the two single-TRP CSIs

Since in most practical scenarios, the best NC-JT CSI and the two best single-TRP CSIs would correspond to the two strongest TRPs, it may be possible to share RIs and PMIs between NC-JT CSI and the two single TRP CSIs.  One example of how RI and PMI can be shared is illustrated below, where the red colour indicates CSI associated to a first TRP, and the green colour indicates CSI associated to a second TRP:

● NC-JT CSI -> [RI1, RI2], [PMI1, PMI2], [CQI]

● 1st single TRP CSI -> [RI1], [PMI1], [CQI1]

● 2nd single TRP CSI -> [RI2], [PMI2], [CQI2]

In this case [RI1, RI2] and [PMI1, PMI2] can be shared between NC-JT CSI and single-TRP CSIs, while CQIs are reported separately for NC-JT CSIs and the two single-TRP CSIs. With this solution CSI reporting overhead can be reduced significantly.  However, it would be good to study the performance-overhead tradeoff of this scheme compared to the case where the PMIs and RIs of NC-JT CSI and single TRP CSIs are not shared.

For NC-JT CSI reports that include single-TRP CSIs (i.e., Option 1 with X = 2), study the CSI overhead-performance tradeoff among the following schemes:

- -> A scheme where RI/PMI of NC-JT CSI and single-TRP CSIs are reported independently

- -> A scheme where RI/PMI of the NC-JT CSI and single-TRP CSIs are shared

One of the open issues is when NC-JT CSI measurement hypothesis can be omitted.  From our earlier contribution [3], we compared the performance of single-TRP vs NC-JT when the UE is equipped with 4 Rx antennas.   The results from [3]for the case with 4 Rx antennas are reproduced in Table 2.  For single TRP, up to rank 4 transmission is allowed.  For NC-JT, the UE can receive the following layer combinations from the two TRPs [1, 1], [1, 2], [2,1], or [2,2].  As can be seen from the results, when up to four layers is allowed for single TRP scheduling, NC-JT performs poorly when compared to single-TRP scheduling.  However, in the evaluations performed in Rel-16, it was demonstrated that if the single-TRP PDSCH is restricted to 2 layers or fewer, then NC-JT provided gains at low loads.  Hence, from these results, we can derive a reasonable criteria on when to omit the NC-JT CSI measurement hypothesis.  For instance, when the rank of at least one TRP is greater than 2, then NC-JT CSI can be omitted.  We, thus, propose the following:

If the rank associated with at least one TRP is greater than 2, then the UE may omit CSI associated with NCJT measurement hypothesis.

Table 2: NC-JT performance with and without layer restriction with 4Rx ports at UE.

| RU of single TRP | Cell edge UE throughput gain |  | Mean UE throughput gain |  |
| --- | --- | --- | --- | --- |
|  | Single TRP | NC-JT | Single TRP | NC-JT |
| 10% | 0% | -4% | 0% | -3% |
| 20% | 0% | -6% | 0% | -6% |
| 30% | 0% | -9% | 0% | -7% |
| 40% | 0% | -7% | 0% | -8% |

### 3.3 Interference measurement

Another open issue on NC-JT CSI with a single reporting setting is on interference measurement as described in the following agreement:

Agreement

For NCJT CSI measurement configured with single reporting setting, study following measurement resource configuration/association mechanism

- Whether/how to support interference measurement based on NZP CSI-RS given by nzp-CSI-RS-ResourcesForInterference or based on CSI-IM given by csi-IM-ResourcesForInterference

- Whether/how to interpret measurement based on CMRs associated with different TRPs/TCI states respectively for a NCJT measurement hypothesis

- CMR/IMR resource configuration restrictions/associations, e.g. for reference resource/time domain behavior/frequency domain behavior

- Note that RAN1 shall strive for commonality of CSI measurement/reporting mechanisms for NCJT CSI measurement configured by single or two reporting settings

If the same Rx antennas are used to receive a PDSCH from two TRPs, typically in case of FR1, the UE received signal y∈CM×1 can be expressed as

y=H1W1s1+H2W2s2+n

where

- H1∈CM×N1 and H2∈CM×N2  are the channel matrix associated with TRP1 and TRP2, respectively;

- N1 and N2 are the number of transmit antennas at TRP1 and TRP2, respectively;

- M is the number of UE receive antennas;

- W1∈CN1×r1 and W2∈CN2×r2  are the precoding matrices applied in TRP1 and TRP2, respectively;

- r1 and r2 are the number of MIMO layers from TRP1 and TRP2, respectively.

- s1∈Cr1×1 and s2∈Cr2×1 are the data symbols transmitted from TRP1 and TRP2, respectively;

- n∈CM×1 is the receive noise and interference from other cells;

In this case, interference measured on the two CSI-IM resources represents two observations of n, and thus the interference can be averaged.

When same antennas are used to receive from two TRPs, the interference on the two CSI-IM resources represents two observations of a same interference.

For NC-JT CSI with a single CSI reporting setting , if the NZP CSI-RS resources for channel measurement are configured without QCL-type D or with the same QCL-type D, a UE assumes that the interference on the CSI-IM resources represents two observations of a same interference.

On the other hands, if two different groups of Rx antennas, e.g.,  two Rx antenna panels typically in FR2, are used to receive a PDSCH from two TRPs, respectively, the UE received signal y∈CM×1 can be expressed as

y=y1y2=H11H12H21H22W1s1W2s2+n1n2

where

- y1∈CM1×1 and y2∈CM2×1 are the signals received on a first and a second antenna panels, respectively;

- H11∈CM1×N1 and H22∈CM2×N2  are the channel matrices associated with (TRP1, Rx panel #1)  and (TRP2, Rx panel #2), respectively;

- H12∈CM1×N2 and H21∈CM2×N1  are the cross over channel matrices associated with (TRP2, Rx panel #1)  and (TRP1,Rx panel #2), respectively;

- n1∈CM1×1 and n2∈CM2×1 are the received noise and interference on the first and the second groups of antenna ports;

- M1 and M2  (M=M1+M2) are  the number of UE receive antennas in the first and the second antenna group or panel;

In this case, interference measured on each of the two CSI-IM resources represents interference observed with each of the two antenna panels, i.e.,  n1 and n2 are measured on CSI-IM #1 and CSI-IM#2, respectively.  When different antenna panels are used to receive from two different TRPs, the interference on each of the two CSI-IM resources represent different interference.

For NC-JT CSI with a single CSI reporting setting, if the NZP CSI-RS resources for channel measurement are configured with different QCL-type D source RS, a UE assumes that the interferences on different CSI-IM resources correspond to different interference sources.

For an NZP CSI-RS resource configured for interference measurement, the UE would measure interference using the same Rx beam for receiving the associated NZP CSI-RS resource for channel measurement regardless of the QCL configuration of the NZP CSI-RS resource for interference measurement. In other words, the UE should assume the NZP CSI-RS resource is QCLed with the associated NZP CSI-RS for channel measurement with respect to “QCL-type D”,

For NC-JT CSI with a single CSI reporting setting, a UE assumes that an NZP CSI-RS or CSI-IM resource for interference measurement is QCLed with respect to “QCL-type D” with the associated NZP CSI-RS resource for channel measurement.

# Conclusion

Based on the discussion in the previous sections, we make the following observations and proposals:

Proposal 1	Support polarization-specific port selection.

Proposal 2	Support Mv=1 and 2, larger value of Mv is not needed.

Proposal 3	RAN1 should consider supporting large R values, for example R=NSBCQI, where NSBCQI is the CQI subband size.

Proposal 4	Do not support lower CSI-RS density, e.g., ρ=0.25.

Proposal 5	Support Option 3: Support configuring multiple CSI-RS resources per CSI reporting configuration associated with Rel-17 PS codebook

Proposal 6	For the number of CMR pairs indicated for NC-JT CSI hypotheses,  support  up to N=4.

Proposal 7	Support up to Ks=8 CMR resources in the CSI-RS resource set for a multi-TRP CSI report setting.

Proposal 8	In NR Rel-17, the number of CMRs in the two CMR groups, K1 and K2, can be either equal or different.

Proposal 9	Support dynamic update of CMR pairs for NC-JT CSI measurement hypothesis using MAC-CE and/or RRC signalling

Proposal 10	For NC-JT CSI reports that include single-TRP CSIs (i.e., Option 1 with X = 2), study the CSI overhead-performance tradeoff among the following schemes:

-> A scheme where RI/PMI of NC-JT CSI and single-TRP CSIs are reported independently

-> A scheme where RI/PMI of the NC-JT CSI and single-TRP CSIs are shared

Proposal 11	If the rank associated with at least one TRP is greater than 2, then the UE may omit CSI associated with NCJT measurement hypothesis.

Proposal 12	For NC-JT CSI with a single CSI reporting setting , if the NZP CSI-RS resources for channel measurement are configured without QCL-type D or with the same QCL-type D, a UE assumes that the interference on the CSI-IM resources represents two observations of a same interference.

Proposal 13	For NC-JT CSI with a single CSI reporting setting, if the NZP CSI-RS resources for channel measurement are configured with different QCL-type D source RS, a UE assumes that the interferences on different CSI-IM resources correspond to different interference sources.

Proposal 14	For NC-JT CSI with a single CSI reporting setting, a UE assumes that an NZP CSI-RS or CSI-IM resource for interference measurement is QCLed with respect to “QCL-type D” with the associated NZP CSI-RS resource for channel measurement.

# References

RP-193133, New WID: Further enhancements on MIMO for NR, Samsung, RAN#86, Sitges, December 2019.

RP-2006973, Discussion Summary for CSI enhancements MTRP and FR1 FDD reciprocity, RAN#102e, E-meeting, August 2020.

R1-1902857, Additional evaluation results for NC-JT with 4Tx ports per TRP under indoor scenario, Ericsson, RAN1#96, February 2019.

# Appendix. Simulation assumptions.

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense urban macro |
| Frequency Range |  | 2 GHz with duplexing gap of 200 MHz between DL and UL |
| Inter-BS distance |  | 200 m |
| Channel model |  | Based on TR 38.901 with the reciprocity model of DL/UL channel in Section 5.3 of TR 36.897 |
| Antenna setup and port layouts at gNB |  | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ 100 deg subarray downtilt (zenith angle) |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1) |
| BS Tx power |  | 44 dBm |
| BS antenna height |  | 25 m |
| UE antenna height & gain |  | According to TR36.873 |
| UE receiver noise figure |  | 9 dB |
| Modulation |  | Up to 256QAM |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Simulation bandwidth |  | 20 MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | MU-MIMO with rank one per UE |
| CSI feedback |  | CSI feedback periodicity: 5 ms Scheduling delay: 4 ms |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) |  | 70% |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC |
| Baseline for performance evaluation |  | Rel-16 PS eType II |
| SRS modeling for UL channel estimation |  | SRS periodicity = 5 ms SRS error modelling according to Table A.1-2 in TR 36.897 with Δ = 9 dB |
| FDD DL/UL calibration error model at gNB |  | - HUL is the spatial UL channel at gNB side with calibration error - HUL is the ideal spatial UL channel without calibration error - E represents the mismatch of transmission and reception circuits of gNB - ai is the amplitude error - θi is the phase error - N is the number of antennas at gNB side With amplitude error (expressed in decibel of x=20log10a) and phase error are normal distribution with 0.7dB and 5 degrees standard deviation, respectively. Both amplitude/phase errors are assumed to be constant during a simulation drop and constant across whole simulation bandwidth |