3GPP TSG RAN WG1 #104-bis-e                   		                                  R1-2103679

e-Meeting, April 12th – April 20th, 2021

Source: 	Fraunhofer IIS, Fraunhofer HHI

Title:	Enhancements on SRS for coverage and capacity

Agenda item:       8.1.3

Document for:	Decision

# Introduction

In RAN1#104-e [1], the following agreements were made for the enhancements on SRS coverage and capacity.

Agreement For Rel-17 SRS capacity and coverage enhancement, support the following - Increase the maximum number of repetition symbols in one slot and one SRS resource to S - Support at least one S value from {8, 10, 12, 14} - FFS other candidate values - Support to transmit SRS only in contiguous RBs in one OFDM symbol, where indicates the number of RBs configured by BSRS and CSRS - Support at least one PF value from {2, [3], 4, 8} - FFS other candidate values, e.g., non-integer values for PF - Note: SRS sequence shorter than the minimum length supported in the current specification is not pursued. - No new sequence including length is introduced - FFS it is applicable to frequency hopping and non-frequency hopping - FFS detailed signaling mechanism to determine PF and the location of the RBs - Support Comb 8 - Note: SRS sequence shorter than the minimum length supported in the current specification is not pursued. - FFS whether and if needed, how to use harmonized approach to define the three supported schemes - Note: other schemes for SRS capacity and coverage enhancements are not supported in Rel-17.

In this contribution, we present our views on partial frequency sounding and on repetition for SRS coverage and capacity enhancements.

# Coverage and capacity enhancements

## Partial frequency sounding

SRS sounding is performed by the UE on a specific set of frequency domain resources defined by the bandwidth configuration. Each SRS resource is configured with a set of higher layer bandwidth configuration parameters cSRS, bSRS, bhop, nshift, nRRC. These parameters are indicated to a UE via the RRC information element ‘SRS-Config’. The total bandwidth assigned for the SRS resource is determined by the parameter CSRS, while the actual sounding bandwidth is determined by the parameters bhop and bSRS. Regardless of the radio channel, once the UE is configured with the bandwidth parameters, the SRS transmission takes place on the entire configured sounding bandwidth. For cell-edge UEs, due to the limited transmit power, the channel estimation quality obtained at the base station from the SRS is often not sufficient. Moreover, the channel is typically frequency selective and varying in the frequency domain. Therefore, not all of the configured frequency domain resources may be useful for the SRS transmission. Therefore, the SRS resource may be configured to be transmitted only on partial frequency resources of the configured bandwidth, the partial frequency resources are either a set of contiguous subbands or non-contiguous subbands. The advantage of such partial sounding is two-fold, one is the power boosting on the partial frequency resources and the other one is the increase in the multiplexing capacity of the UEs as the un-used frequency resources can be configured to be used by other UEs.

Partial frequency sounding may also be useful in case of strong frequency correlation among subbands of the configured bandwidth. In this case, adjacent subbands need not to be sounded, as the channel remains almost identical for a number of subbands. Therefore, only a few non-contiguous/contiguous subbands may be sounded and the channel on the remaining subbands can be calculated by interpolation/extrapolation.

In RAN1#104-e, it was agreed that only mSRS,BSRS/PF continuous PRBs are sounded in the configured actual sounding bandwidth of mSRS,BSRS PRBs. Based on the proposed values for PF, the number of sounding PRBs can be large for some SRS bandwidth configurations. Therefore, large values for PF shall be considered in order to reduce the sounding bandwidth. Also, for some bandwidth configurations with a small number of PRBs, partial sounding is not required and is not a target of this feature. Per current specification, the size of the subband is 4 PRBs and for simplicity, the sounding can be performed on the subband level.  Consequently, PF can be written as PF=4⋅P, where P∈{14,12,1,2,3,4}. The choice of P depends on whether full sounding for small bandwidth configurations or partial sounding for large bandwidth configurations is considered. For bandwidth configurations with small number of PRBs, a value of P<1 can be used, whereas for bandwidth configurations with large number of PRBs, P≥1 can be used.

Proposal: Support PF=4⋅P, P∈{14,12,1,2,3,4}.

In order to determine the starting index of the partial sounding bandwidth of mSRS,BSRS/PF PRBs, the UE may be indicated by a PF-length bitmap, where a ‘1’ in the n-th position sounds the n-th bandwidth segment comprising mSRS,BSRS/PF PRBs.

Proposal: The location of mSRS,BSRS/PF PRBs can be indicated to the UE by a PF-length bitmap.

## Repetition

SRS coverage can be enhanced by repeating the SRS transmission on adjacent symbols. Especially, for cell-edge UEs, due to limited transmit power, frequency hopping combined with repetition greatly enhances the received SNR; thus increasing the coverage. In the current specifications, repetition values of 2 and 4 are supported. Although, supporting repetition values greater than 4 increases the coverage, it has the disadvantage that the multiplexing capacity of the UEs may be reduced as repetition requires multiple symbols. Therefore, repetition combined with partial frequency sounding may help to expand UE coverage as well as capacity primarily for cell-edge UEs.

Observation: Increasing the value of the repetition factor decreases the multiplexing capacity.

Proposal: Supporting a repetition factor of 8 is sufficient.

Proposal: Repetition combined with partial frequency sounding may expand both coverage and capacity, especially for cell-edge UEs.

# Conclusions

Based on the above discussion, we have the following observations and proposals.

Proposal: Support PF=4⋅P, P∈{14,12,1,2,3,4}.

Proposal: The location of mSRS,BSRS/PF PRBs can be indicated to the UE by a PF-length bitmap.

Observation: Increasing the value of the repetition factor decreases the multiplexing capacity.

Proposal: Supporting a repetition factor of 8 is sufficient.

Proposal: Repetition combined with partial frequency sounding may expand both coverage and capacity, especially for cell-edge UEs.

# References

- Chairman Notes, RAN1#104-e, E-meeting, Jan 25th – Feb 05th, 2020.