3GPP TSG RAN WG1 #104-bis-e						    R1-2103798

e-Meeting, April 12th – 20th, 2021

Source:	Moderator (Intel Corporation)

Title:	Feature lead summary on Mode-2 resource allocation maintenance issues in Rel.16 5G V2X

Agenda item:	7.2.4

Document for:	Discussion and Decision

Introduction

This contribution provides a summary of proposed correction to Mode-2 sidelink resource allocation for NR-V2X communication, based on review of the submitted contributions [1]-[12]. Note, that the other contributions [13]-[39] do not seem to include Mode-2 issues.

Identification of email discussions

It is proposed to organize the following email discussion:

Issue M2-1 – Infinite loop due to excessive resource exclusion in step 5)

+

Issue M2-2 – Resource exclusion/selection for multiple transport blocks

Finally agreed threads:

[104b-e-NR-5G_V2X-03] Email discussion/approval on issue M2-1: Infinite loop due to excessive resource exclusion in step 5) till 4/15, with potential CRs till 4/19 – Sergey (Intel)

Issue list

Issue M2-1 – Infinite loop due to excessive resource exclusion in step 5)

- Discussed partially

- Can be re-discussed due to major interest (4 sources)

Issue M2-2 – Resource exclusion/selection for multiple transport blocks

- Discussed extensively w/o conclusion

- Can be re-discussed due to major interest (4 sources)

Issue M2-5 – HARQ RTT time gap capturing issue in MAC, send LS

- Was not discussed in RAN1, but discussed in RAN2

- In preparation to the last meeting, we decided to gain support in RAN2 directly to fix this issue w/o sending LS

- Can be discussed in RAN1 if there is interest

Issue M2-6 – ‘sl-ThresPSSCH-RSRP-List’ to ‘sl-Thres-RSRP-List’ replacement

- New editorial issue due to RRC spec update

- Can be included into an editorial / alignment CR

Issue M2-10 – Replacing ‘sl-ResourceReservePeriod1’ by ‘sl-ResourceReservePeriodList’ in 213, clause 16.4

- New editorial issue

- Can be included into an editorial / alignment CR

Analysis of Draft Corrections

Issue M2-1 – Infinite loop due to excessive resource exclusion in step 5)

- Discussed partially

- Can be re-discussed due to major interest (4 sources)

Issue M2-2 – Resource exclusion/selection for multiple transport blocks

- Discussed extensively w/o conclusion

- Can be re-discussed due to major interest (4 sources)

Issue M2-3 – Backward indication

- Discussed extensively

- Optimization at this stage

Issue M2-4 – Introduce a dropping condition when HARQ RTT time gap is not met

- Was not discussed

- Optimization at this stage

Issue M2-5 – HARQ RTT time gap capturing issue in MAC, send LS

- Was not discussed in RAN1, but discussed in RAN2

- In preparation to the last meeting, we decided to gain support in RAN2 directly to fix this issue w/o sending LS

- Can be discussed in RAN1 if there is interest

Issue M2-6 – ‘sl-ThresPSSCH-RSRP-List’ to ‘sl-Thres-RSRP-List’ replacement

- New editorial issue due to RRC spec update

- Can be included into an editorial / alignment CR

Issue M2-7 – Exclude the slots with PSFCH when sl-LengthSymbols≤9 in the identification of candidate resources in the sensing procedure

- Was not discussed

- Optimization at this stage

Issue M2-8 – Clarification on timing relation between re-evaluation moment and initial selection moment

- Was not discussed

- Optimization at this stage

Issue M2-9 – Correction to step 6) to include slots within Tproc0

- Was discussed during the preparation last time, but considered as an optimization

- Proponents insist there is still an issue

- Optimization at this stage

Issue M2-10 – Replacing ‘sl-ResourceReservePeriod1’ by ‘sl-ResourceReservePeriodList’ in 213, clause 16.4

- New editorial issue

- Can be included into an editorial / alignment CR

| Tdoc# | Issue within the tdoc | Issue index |
| --- | --- | --- |
| R1-2102369 OPPO | Infinite loop due to excessive resource exclusion in step 5) | 1 |
| R1-2102589 CATT, GOHIGH | Resource exclusion/selection for multiple transport blocks | 2 |
|  | Backward indication | 3 |
| R1-2102941 vivo | Introduce a dropping condition when HARQ RTT time gap is not met | 4 |
|  | HARQ RTT time gap capturing issue in MAC, send LS | 5 |
|  | ‘sl-ThresPSSCH-RSRP-List’ to ‘sl-Thres-RSRP-List’ replacement | 6 |
| R1-2103081 Apple | Resource exclusion/selection for multiple transport blocks | 2 |
|  | Infinite loop due to excessive resource exclusion in step 5) | 1 |
| R1-2103143 Qualcomm | Infinite loop due to excessive resource exclusion in step 5) | 1 |
| R1-2103467 Sharp | Exclude the slots with PSFCH when sl-LengthSymbols≤9 in the identification of candidate resources in the sensing procedure | 7 |
|  | Clarification on timing relation between re-evaluation moment and initial selection moment | 8 |
| R1-2103501 ZTE, Sanechips | ‘sl-ThresPSSCH-RSRP-List’ to ‘sl-Thres-RSRP-List’ replacement | 6 |
| R1-2103516 NEC | Correction to step 6) to include slots within Tproc0 | 9 |
| R1-2103639 ASUSTeK | Resource exclusion/selection for multiple transport blocks | 2 |
| R1-2103750 Huawei, HiSilicon | Resource exclusion/selection for multiple transport blocks | 2 |
| R1-2103751 Huawei, HiSilicon | Infinite loop due to excessive resource exclusion in step 5) | 1 |
| R1-2103765 Nokia, Nokia Shanghai Bell | replacing ‘sl-ResourceReservePeriod1’ by ‘sl-ResourceReservePeriodList’ in 213, clause 16.4 | 10 |

- References

Contributions identified by FL to contain Mode-2 related issues:

- R1-2102369	Remaining open issues and corrections for mode 2 RA	OPPO

- R1-2102589	Discussion and TPs on resource allocation in NR V2X	CATT, GOHIGH

- R1-2102941	Maintenance on NR sidelink mode-2 resource allocation mechanism	vivo

- R1-2103081	On Remaining Issues of Mode 2 Resource Allocation	Apple

- R1-2103143	Remaining Issues in Mode 2 Resource Allocation	Qualcomm Incorporated

- R1-2103467	Remaining issues on resource allocation for NR sidelink	Sharp

- R1-2103501	Draft CR of TS38.214	ZTE, Sanechips

- R1-2103516	Remaining issues on resource allocation mode 2	NEC

- R1-2103639	Remaining issues on sidelink mode 2	ASUSTeK

- R1-2103750	Correction on resource exclusion for other TBs	Huawei, HiSilicon

- R1-2103751	Correction on step 5 of mode 2 resource allcoation	Huawei, HiSilicon

- R1-2103765	Maintenance for Resource allocation for sidelink - Mode 2	Nokia, Nokia Shanghai Bell

Other contributions

- R1-2102368	Remaining open issues and corrections for mode 1 RA	OPPO

- R1-2102590	Discussion and TPs on Tie-break issues for PSFCH Tx and PSFCH Rx	CATT, GOHIGH

- R1-2102591	Discussion on timing offset between eNB and gNB in NR V2X	CATT, GOHIGH

- R1-2102710	A remaining issue on Mode-1 resource allocation for NR sidelink	Fujitsu

- R1-2102711	Remaining issues on physical layer procedures for NR sidelink	Fujitsu

- R1-2102795	Remaining open issues and corrections for procedure	OPPO

- R1-2102940	Maintenance on NR sidelink mode-1 resource allocation mechanism	vivo

- R1-2102942	Maintenance on NR sidelink synchronization and procedure	vivo

- R1-2103080	On Remaining Issue of Sidelink Physical Layer Procedures	Apple

- R1-2103213	Discussion on Issues for NR Sidelink Physical Layer Structure	Samsung

- R1-2103375	Discussion on essential corrections in physical layer structure	LG Electronics

- R1-2103376	Discussion on essential corrections in resource allocation procedure	LG Electronics

- R1-2103377	Discussion on essential corrections in physical layer procedure	LG Electronics

- R1-2103391	Remaining issues for sidelink physical layer procedure	Huawei, HiSilicon

- R1-2103466	Remaining issues on physical layer structure for NR sidelink	Sharp

- R1-2103468	Remaining issues on synchronization mechanism and QoS management for NR sidelink	Sharp

- R1-2103499	Remaining issues on mode 1	ZTE, Sanechips

- R1-2103500	SL PHY procedure on prioritization	ZTE, Sanechips

- R1-2103555	Maintenance for resource allocation mechanism mode 1	NTT DOCOMO, INC.

- R1-2103556	Maintenance for sidelink physical layer procedure	NTT DOCOMO, INC.

- R1-2103638	Remaining issues on DL pathloss for sidelink power control	ASUSTeK

- R1-2103672	Remaining issues on resource allocation mode-1 and sidelink procedure	ASUSTeK

- R1-2103707	Draft_CR_TS38.213	Ericsson

- R1-2103708	Draft_CR_TS38.214	Ericsson

- R1-2103709	Correction to PSCCH reserved bits definition	Ericsson

- R1-2103764	Maintenance for Resource allocation for sidelink - Mode 1	Nokia, Nokia Shanghai Bell

- R1-2103766	Correction to PSFCH reception procedure for NACK-only case to mitigate half-duplex issue	Intel Corporation