3GPP TSG RAN WG1 Meeting #104bis-e	R1-2102316

E-meeting, April 12 – 20, 2021

Agenda Item:	8.7.1.1

Source:	Huawei, Hiilicon

Title:	Paging enhancements for UE power saving in IDLE/inactive mode

Document for:	Discussion and Decision

Introduction

In RAN1#104-e, the criteria for evaluating PEI physical layer candidate designs were discussed. This contribution further provides the evaluation and comparison based on the agreed criteria and assumptions. Based on the evaluations, it is proposed to adopt the DCI based paging early indication as the physical layer design to carry the PEI indication.

Some further considerations on the remaining issues are also discussed in this contribution.

PEI physical layer designs

In RAN1#103-e, it was agreed that paging early indication (PEI) is supported from RAN1 perspective, and the physical layer design of PEI is based on DCI, SSS or TRS/CSI-RS. In RAN1#104, the criteria for the design of physical layer design of PEI were agreed for further narrow down. This section provides evaluation results, analysis for the three candidates including the co-existence problem, resource overhead aspects etc.

Detection performance

The following agreements were agreed to be used for the link level evaluation. The joint MDR performance under behavior A and behavior B are provided and the detailed link level evaluation assumptions are listed in Appendix A, which aligns with the agreement in RAN1#102.

Agreements: - Take Alt 1 as mandatory, and Alt 2 as optional Alt 1: For the performance evaluations of PEI candidate designs based on PDCCH, TRS/CSI-RS and SSS, - The following are assumed, at the SNR where the Miss-Detection Rate (MDR) of paging PDSCH is 1%, - When Behv-A is assumed: - The joint miss-detection rate (MDR) of PEI and paging PDCCH defined below should be no worse than 1%: MDR_Joint_A = MDR_PEI + (1 – MDR_PEI) MDR_PagingPDCCH - The False-Alarm Rate (FAR) of PEI should be no larger than [1%] - When Behv-B is assumed: - The joint miss-detection rate (MDR) of PEI and paging PDCCH defined below should be no worse than 1%: MDR_Joint_B = FAR_PEI + (1 – FAR_PEI) MDR_PagingPDCCH - The MDR of PEI should be no larger than [1%] - Note: The CFO is modeled at the input of PEI detection and based on LLS assumptions agreed in RAN1 #102-e. Companies should justify the applied random range for the CFO. - Companies to provide: - Information on the utilized detection method for each PEI candidate design (e.g., non-coherent detection or coherent detection) - The required #REs to comply with the performance assumptions - The maximum number of subgroups that can be carried in PEI, subject to the performance assumptions

The performance for Behv-A and Behv-B are evaluated respectively and the results for MDR_Joint_A and MDR_Joint_B for the three candidates are illustrated in Figure 1 and Figure 2. Both 4Rx and 2Rx UEs are evaluated to investigate the PEI performance for non-RedCap UE and RedCap UEs. According to the agreements, the SNR where the Miss-Detection Rate (MDR) of paging PDSCH is 1%  is the working point where the PEI is required to have less than 1% joint detection performance. The CFO before PEI is assumed to be uniformly distributed in [-0.5, 0.5] ppm, and the CFO for paging PDCCH and PDSCH is 0ppm.

Figure 1. Link level evaluation results for Behv-A & Behv-B for 4Rx UE, e.g. non-RedCap UE

Figure 2. Link level evaluation results for Behv-A & Behv-B for 2Rx UE, e.g. RedCap UE

The evaluation results showed the 1-symbol SSS sequence cannot meet the requirements of paging PDSCH, therefore, 2-symbol SSS sequence need to be used for SSS-based PEI. 24RB one-slot TRS pattern slightly requires higher SNR than the paging PDSCH. Therefore, 28RB one slot TRS pattern is considered to fulfill the reqirements. On the other hand, the PDCCH-based PEI with AL4 can satisfy the requirements of paging PDSCH. In Behv-B, the PDCCH-based PEI performs much better than the TRS-based and SSS-based PEI, this is because the extremely low FAR of PDCCH.

- To fulfil the requirement of joint detection performance of PEI and paging DCI is no worse than 1% at the SNR where the MDR of paging PDSCH is 1%, the required transmissions or configurations for different physical layer candidates are listed in Table 1.

Table 1 Minimum resource requirement of PEI candidates for BLER1% of paging PDSCH

| Required transmission/configuration of each PEI candidate |  | DCI based PEI (12 bits payload) | SSS based PEI | TRS based PEI |
| --- | --- | --- | --- | --- |
| 2T4R UE (e.g. non-RedCap UE) | Behv-A | AL4 | 127RE*2symbol | TRS pattern within 28RB and 1slot |
|  | Behv-B | AL4 | 127RE*2symbol | TRS pattern within 28RB and 1slot |
| 2T2R UE (e.g. RedCap UE) | Behv-A | AL4 | 127RE*2symbol | TRS pattern within 28RB and 1slot |
|  | Behv-B | AL4 | 127RE*2symbol | TRS pattern within 28RB and 1slot |

Co-existence with legacy channels/signals

This section discusses how to multiplex the PEI channel with physical signals/channels of legacy UEs.

DCI based PEI

For PDCCH-based PEI design, multiplexing of the PDCCH and legacy PDSCH traffic has already been supported by Rel-15 specification and there is no need to introduce any new mechanism to support the coexistence between PEI DCI and legacy channels of legacy UEs. Furthermore, Rel-15 can also support the multiplexing PDCCHs from different UEs in a CORESET. Therefore, DCI based PEI can naturally coexist with other PDCCHs. Rel-15 already specifies how a PDCCH candidate coexists with SSB, which can be found in Appendix C. Therefore, DCI based PEI can fully support coexistence with the legacy downlink physical layer channels of legacy UE.

- DCI based PEI can fully support coexistence with the legacy downlink physical layer channels/signals of legacy UE based on Rel-15 specifications.

SSS based PEI

For SSS-based PEI, the only coexistence way is to use semi-statically configured rate matching pattern to multiplex the SSS-based PEI and legacy PDSCH, considering SSS-based PEI is basically a new downlink physical layer signal/channel that is not visible to legacy UEs. It would require a RB * symbol level rate matching pattern.

Based on UE capability #5-27 in [3], dynamic RB-level rate matching is an optional capability in Rel-15 specification, which cannot be expected to be supported by legacy UEs. Therefore, the SSS-based PEI occasions can only be semi-statically configured to avoid colliding with PDSCH of legacy UE. Unnecessary resource overhead increase is introduced for SSS-based PEI, which will be analyzed in Section 2.4.

For the case of multiplexing SSS-based PEI with SSB and PDCCH transmission of legacy UEs, there is no mechanism defined to handle it in Rel-15/16 specification. The legacy UE cannot handle the collision cases considering SSS-based PEI is not visible to legacy UEs. It can only rely on gNB to carefully handle the overlapping of SSS-based PEI with PDCCH(s) of legacy UE and SSB, which may or may not be feasible. gNB complexity would be increased.

- Semi-statically configured rate matching pattern needs to be used to multiplex SSS-based PEI and PDSCH(s) of legacy UE.

- It can only rely on gNB to carefully handle the overlapping of SSS-based PEI with PDCCH(s) of legacy UE and SSB.

TRS based PEI

For TRS/CSI-RS based PEI coexistence with legacy signals, a way is to configure aperiodic ZP-CSI-RS resource set to cover the TRS/CSI-RS based PEI transmissions. However, the number of aperiodic ZP-CSI-RS resource sets is very limited, which is up to 3 per BWP. Considering the ZP-CSI-RS resource sets are mainly used for multiplexing NZP-CSI-RS for CSI and beam management etc., it is expected that there shall be no available ZP-CSI-RS resource set to handle the coexistence of TRS/CSI-RS based PEI. Therefore, semi-static RB*symbol-level rate matching pattern would be the way by network to multiplex TRS/CSI-RS base PEI with legacy channels of legacy UE.

- Rel-15 only supports up to 3 aperiodic ZP-CSI-RS resource sets per BWP and it is expected that there is no available ZP-CSI-RS resource set for the coexistence of TRS/CSI-RS based PEI

- Semi-static RB-level rate matching pattern is expected to be used for the coexistence of TRS/CSI-RS based PEI.

The analysis is summarized in Table 2 as below:

Table 2. Summary of the co-existence of PEI candidate design with legacy UE

|  | General coexistence method | Coexistence with PDSCH of legacy UE | Coexistence with PDCCH of legacy UE | Coexistence with SSB |
| --- | --- | --- | --- | --- |
| DCI based PEI | A CORESET that can be shared with legacy UE | The Rel-15 supports the multiplexing CORESET with PDSCHs from different UEs. | The Rel-15 PDCCH design supports the multiplexing between PDCCHs from different UEs. | Follow Rel-15 behavior: UE is not required to monitor the PDCCH candidate colliding with SSB. |
| SSS based PEI | Semi-static RB-level rate matching pattern | Based on UE capability #5-27, dynamic RB-level rate matching is an optional capability | Not defined in legacy specification for a new SSS based PEI and legacy UE’s PDCCH, SSB and SIB1. Legacy UE cannot handle it. gNB needs to carefully handle it. |  |
| TRS/CSI-RS based PEI | Semi-static RB-level rate matching pattern | Dynamic rate matching is mandatory feature. However only 3 aperiodic ZP-CSI-RS resource sets for one BWP are supported at most. | Legacy UE assumes reception of PDCCH/SSB regardless of whether they collide with ZP CSI-RS or not. gNB needs to carefully handle it. |  |

Indication of association of POs and sub-group indication

In RAN1#104-e, it was agreed that whether and how indication(s) to multiple POs and/or UE subgroups by one PEI need to be considered in the evaluation of PEI resource overhead.  It was agreed that carrying UE sub-groups information is considered in physical layer design. As described in [5], significant power saving gain is observed when sub-group indication is carried by PEI.

In this section, we discuss how to associate POs with PEI and how to carry the sub-grouping indication over different PEI physical layer design.

For PDCCH-based PEI, it is straight forward to use the DCI payload to convey the sub-group indication and PEI indication bits for multiple POs. For example, one PEI can associate with a total paging frame of four paging occasion and the users in each paging occasion are divided into 3 subgroups. So a total of 12 sub-groups can be indicated by the PEI, which could be carried in a DCI format with 12 bit payload size.

For sequence-based PEI such as SSS and TRS/CSI-RS, since only one sequence can be transmitted at one time. Additional design should be provided for carrying sub-grouping indication and association of multiple POs, and the following alternative solutions can be considered:

- Alternative 1: FDM or TDM.

- Alternative 2: Single sequence CDM.

The alternative 1 is a straight forward solution to configure each sub-group/PO with a dedicated PEI occasion. The resource of PEI occasions of different POs/subgroups can be FDM or TDM multiplexed. However, it would requires more resources. If multiple sub-groups or multiple POs are paged, PEI resources would be increased by multiple times.

The alternative 2 only allocates one occasion for PEI associated with multiple sub-groups/POs, and configures one group specific sequence for each sub-group/PO and one common sequence. If only one sub-group/PO is paged for the associated PO(s), the corresponding group specific sequence is transmitted in the occasion. If more than one sub-group or UEs in more than one POs are paged, the gNB would transmit the common sequence in the PEI occasion to inform all the UEs associated with the PEI. Obviously, alternative 2 saves the resource overhead for PEI compared to alternative 1, however the idle mode UEs’ waked up rate would be increased depending on the number of supported sub-groups or associated POs. The wake up rate needs to be carefully controlled to avoid the reduction power saving gains if single sequence based CDM is utilized.

- For PDCCH-based PEI, the payload can be used for sub-group indication and associated with multiple POs flexibly.

Resource overhead

The evaluation methodology on the resource overhead was discussed and agreed in the following agreements.

Agreements: For the evaluation of resource overhead with PEI candidate designs based on PDCCH, TRS/CSI-RS and SSS, companies to provide estimated overheads for PEI candidate designs based on the following factors: - Assumption of Behv-A/B - Required #REs from performance evaluations - 10% group paging rate per PO as baseline; other group paging rates can be optionally considered and based on the following assumptions with justification (up to each company) - Whether and how coexistence with legacy UEs is considered - Whether and how indication(s) to multiple POs and/or UE subgroups by one PEI is considered - Whether and how multi-beam transmission is considered

The resource overhead is defined by

O=(NRE,PEI∙PPEI)/NPO per PEI , where

- NRE,PEI is the required RE number ensuring no paging detection performance impact, which is obtained by the link level evaluations in section 2.1;

- PPEI resource occupation probability by PEI

- NPO per ,PEI is the number of PO sharing the same PEI signal/channel.

The PEI occasion configuration is based on link level evaluations in section 2.1, and the required REs per PEI transmission are referring to Table 1.

The resource occupied probability depends on the detection behavior (Behv-A/B), the transmission characteristic of the PEI signals/channels and the co-existence mechanism:

- For the utilization of semi-statically configured rate matched pattern for co-existence, the resource occupation probability is 100%;

- For the case dynamic rate matching is supported, the following aspects need to be considered:

  - For Behv-A, the PEI should be transmitted except for the case where no UE is paged for all the POs associated with the PEI.

  - For Behv-B, the PEI should be transmitted except for the case when there are paging in all the POs associated with the PEI.

  - Both the co-existence method of using semi-static configured rate-matching pattern and the way by using one ZP-CSI-RS resource set are evaluated for TRS based PEI.

Regarding the paging configurations for the evaluation, two typical scenarios are evaluated and the corresponding paging configurations are shown in Table 3. The scenario 1 is the typical case where frequent POs are configured in time domain and therefore smaller number of UEs sharing the same PO. Therefore, it is assumed that there is no sub-grouping division for each PO. The scenario 2 corresponds to a scenario where sparse POs are configured in the system, where 8 paging frames are configured for one DRX cycle and 4 paging occasions for each paging frame, i.e. N=8 and Ns=4. Considering sparse POs are configured in time domain, more UEs, e.g. 50, per one paging occasion are considered. 3 sub-groups are considered in each PO for this scenario.

Table 3. Paging related configurations for evaluated scenarios

|  | Scenario 1 (Scenario without sub-group indication) | Scenario 2 (Scenarios with sub-group indication) |
| --- | --- | --- |
| N | 128 | 8 |
| Ns | 4 | 4 |
| Paging cycle length(ms) | 1280 | 1280 |
| PO number per PEI DCI | 12 | 4 |
| PO paging rate | 10.47% | 39.50% |
| UE paging rate | 1.00% | 1.00% |
| UE per PO | 11.00 | 50.00 |
| group per PO | 1 | 3 |
| UE receiver | 2T4R | 2T4R |

Scenario without sub-group indication (Scenario 1)

The resource overhead results for Scenario 1 is shown in Table 4. In order to make a fair comparison, the UE wake-up rate is aligned between different PEI candidates, which corresponds to similar power saving gains.

For PDCCH-based PEI, one PEI occasion would cover 12 POs with each payload bit to indicate one PO. For sequence-based PEI, as shown in Figure 3, if single sequence based CDM is used to multiplex the PEI transmissions of 12 POs, the wake-up rate of a UE shall be increased from 10% to 40% due to the transmission of group common sequence transmission, which will significantly impact the power saving benefit of PEI solution.

Therefore, one-to-one association between PEI and PO is the reasonable consideration for sequence based PEI design, which can control the wake-up rate of a UE to guarantee the attractive power saving gain.

Figure 3 UE wake-up rate if single sequence based CDM applied for SSS-based/TRS-based PEI in scenario 1

It can be observed that, for both Behv-A and Beh-v B, the PDCCH-based PEI can provide the lowest resource overhead.

Table 4. Resource overhead for Scenario 1 (without sub-group indication)

|  |  | PDCCH based PEI | SSS based PEI | TRS based PEI (semi-static rate matching pattern) | TRS based PEI (one A-ZP-CSI-RS resource set for rate matching) |
| --- | --- | --- | --- | --- | --- |
| PEI occasion configuration |  | AL4 | 127RE, 2sym | 28RB, 1slot TRS pattern | 28RB, 1slot TRS pattern |
| Total RE per PEI occasion |  | 288 | 254 | 168 | 168 |
| PO association configuration |  | 12bits payload for 12 POs | One PEI transmission associate with one PO | One PEI transmission associate with one PO | One PEI transmission associate with one PO |
| Behv-A | Resource occupation probability | 73.46% | 100.00%Note2 | 100%Note3 | 73.46%Note4 |
|  | Average resource overhead (RE) per PO | 17.6 | 254.0 | 168.0 | 123.4 |
|  | UE waked up rate | 10.47% | 10.47% | 10.47% | 10.47% |
| Behv-B | Resource occupation probability | 100% | 100.00% Note2 | 100% Note3 | 100% Note4 |
|  | Average resource overhead (RE) per PO | 24.0 | 254.0 | 168.0 | 168.0 |
|  | UE waked up rate | 10.47% | 10.47% | 10.47% | 10.47% |
| NOTE: - For Behv-A, PEI should always be transmitted except no sub-group is paged for all the POs associated with the PEI. For Behv-B, PEI should always be transmitted except the case when all sub-group is paged. - SSS-PEI can only be semi-statically configured, therefore the resource occupation probability is 100%. - Considering the ZP-CSI-RS resource sets are mainly used for multiplexing NZP-CSI-RS for CSI and beam management etc., it is expected that there shall be no available ZP-CSI-RS resource set to handle the coexistence of TRS/CSI-RS based PEI. Therefore, semi-static RB*symbol-level rate matching pattern is assumed for co-existence. - All the 12 PEI occasions for TRS/CSI-RS are configured within one aperiodic ZP-CSI-RS resource set due to the ZP-CSI-RS resource set budget. |  |  |  |  |  |

Scenario with sub-group indication (Scenario 2)

The resource overhead results for Scenario 2 is shown in Table 5. For PDCCH-based PEI, one PEI occasion would cover the 4 POs of one paging frame with each payload bit to indicate one of three sub-groups per PO. For sequence-based PEI, if single sequence based CDM is used to associate one PEI occasion with 4 POs, the wake-up rate will be increased to 57.62%, which will significantly impact the power saving benefit of PEI solution.

Therefore, one-to-one association between PEI and PO is the reasonable consideration for sequence based PEI design, which can control the wake-up rate of a UE to guarantee the attractive power saving gain.

Table 5. Resource overhead for Scenario 2

|  |  | PDCCH based PEI | SSS based PEI | TRS based PEI (semi-static rate matching pattern) | TRS based PEI (one A-ZP-CSI-RS resource set for rate matching) |
| --- | --- | --- | --- | --- | --- |
| PEI occasion configuration |  | AL4 | 127RE, 2sym | 28RB, 1slot TRS pattern | 28RB, 1slot TRS pattern |
| Total RE per PEI occasion |  | 288 | 254 | 168 | 168 |
| PO and Sub-group association configuration |  | 12bit payload associate with 4 POs. Three sub-groups for each PO. | One PEI occasion associate with one PO Three sub-groups for each PO by using single sequence based CDM | One PEI occasion associate with one PO Three sub-groups for each PO by using single sequence based CDM | One PEI occasion associate with one PO Three sub-groups for each PO by using single sequence based CDM |
| # PO with one PEI occasion |  | 4 | 1 | 1 | 1 |
| Behv-A | Resource occupation probability | 85.48% | 100.00% Note2 | 100.00% Note3 | 86.60%Note4 |
|  | Average resource overhead (RE) per PO | 61.5 | 254 | 168 | 145.5 |
|  | UE waked up rate | 14.85% | 16.73% | 16.73% | 16.73% |
| Behv-B | Resource occupation probability | 100% | 100% | 100% | 97.57% |
|  | Average resource overhead (RE) per PO | 72.0 | 254.0 | 168 | 163.9 |
|  | UE waked up rate | 14.85% | 16.73% | 16.73% | 16.73% |
| NOTE: - For Behv-A, PEI should always be transmitted except no sub-group is paged for all the POs associated with the PEI. For Behv-B, PEI should always be transmitted except the case when all sub-group is paged. - SSS-PEI can only be semi-statically configured, therefore the resource occupation probability is 100%. - Considering the ZP-CSI-RS resource sets are mainly used for multiplexing NZP-CSI-RS for CSI and beam management etc., it is expected that there shall be no available ZP-CSI-RS resource set to handle the coexistence of TRS/CSI-RS based PEI. Therefore, semi-static RB*symbol-level rate matching pattern is assumed for co-existence. - All the 4 PEI occasions for TRS/CSI-RS are configured within one aperiodic ZP-CSI-RS resource set due to the ZP-CSI-RS resource set budget of 3. |  |  |  |  |  |

From Table 4 and Table5, it can be observed the PDCCH based PEI can provide the most efficient utilization of resource for both Scenario 1 and Scenario 2.

- PDCCH-based PEI can provide the best efficient resource utilization and the lowest UE wake up rate compared to SSS-based and TRS based PEI for both Behv-A and Behv-B.

Impact to other legacy functionalities

The SSS-based PEI might also impact the legacy cell search procedure. As shown in Figure 4, if the SSS-based PEI is transmitted in the same frequency range as the SSS, a legacy UE which does not support PEI identifies the PEI as a SSS signal, and if two neighbor cells share the same PSS (as the Cell_A and Cell_B in Figure 4) sequence, the legacy UE might identify the combination of PSS from Cell_B and PEI from Cell_A, which will be recognized as a new cell, however does not exist.

Figure 4. SSS-based PEI might impact the legacy cell search procedure

- The SSS-based PEI has impact on the legacy cell search procedure.

When considering cooperative deployment with other functionalities, for DCI-based PEI, besides the paging indication, the availability indication for assistance RS, the short message to inform SI change and/or ETWS information can be also informed in one PEI DCI. However, if SSS-based and TRS-based PEI are deployed, multiple PEIs would be required to convey the short message, otherwise in order to get the SI change and/or ETWS information, the UE shall always receive paging DCI regardless of what is indicated in the PEI. Then there is no obvious power saving gain.

- Sequence-based PEI signals is not suitable to support the indication of short message.

- Sequence-based PEI signals is not suitable to support other power related information, e.g. assistance TRS availability indication

- DCI-based PEI can be flexibly configured to carry PEI indication, sub-group indication, short message and other power saving functionalities, e.g. the availability indication of assistance TRS.

Summary

Based on the above summary and observation, we propose to agree the following proposals.

- Adopt DCI carried by PDCCH as the physical layer channel for PEI indication in Rel-17.

General design of PEI occasion

In this section some of our general consideration of PEI is discussed.

Consideration on different time location of PEI occasion with respect to SS bursts

Generally the PEI occasion should be located before the incoming paging occasion and also near a SS burst, then UE can wake up once to receive both SSB and PEI. However the actually occasion needs to be carefully designed to avoid introduce additional power consumption due to unnecessary wake up and power state transition, and it should also consider the flexibility and compatibility for UEs in different coverage of different capabilities.

Firstly considering that all the UEs need at least one SS burst for intra-frequency measurements before paging reception and the PEI can rely on the SS burst for T/F synchronization for the reception of PEI, it would be better to place the PEI after one SS burst transmission.

Secondly for UEs in good coverage, which might require only one SS burst for time/frequency tracking and AGC adjustment for paging PDSCH reception before paging occasion, it would be better to place the PEI after the nearest SS burst before paging reception. However, if PEI is actually located in that position, for UEs in bad coverage, which requires three SS bursts for time/frequency tracking and AGC adjustment for paging PDSCH reception, the UEs would have receive all three SS bursts before the PEI received, which might lead to very limited power saving gain as in Figure 5a.

On the other sides, the PEI can also be located after the third nearest SS burst before the paging occasion. As evaluated in our previous paper [5], even the UE in bad coverage can also use only one SS burst for synchronization for PEI. If the PEI indicates no paging in corresponding paging occasion, the power consumption for the reception of the remaining two SS bursts can also been saved. Nevertheless, for UEs only need one SS burst for time/frequency tracking and AGC adjustment, it would suffer one additional power state transition as illustrated Figure 5c.

Therefore, to provide larger power saving gain for UEs in both good coverage and bad coverage, the PEI indicating the same PO can be transmitted on multiple occasions. For example, gNB can transmit the PEI in both the three possible occasions in Figure 5d.

- PEI locates behind the nearest SS burst before PO.

- PEI locates behind the third nearest SS burst before PO which can save two SS burst reception for bad coverage UEs

- PEI locates behind the third nearest SS burst before PO which introduces a short time deep/light sleep and power state transition for good coverage UE

- Multiple PEI transmissions for one PO

Figure 5. Idle mode UE procedures with respect to different PEI locations.

- Considering there are different UEs in a cell, there can be multiple PEI occasionss indicating the same PO.

Monitoring occasion for PEI

At least one SS burst needs to be received by UE for serving cell measurement. The SS burst can be also used for T/F tracking and AGC adjustment. To obtain a better power saving gain, the PEI should be transmitted after the SS burst. An offset between the SS burst and the monitoring window can be specified for the PEI design as showed in Figure 6. In our understanding, the offset should be equal to or larger than zero slot considering RF retuning might be required if the bandwidth of SSB and PEI are different. And if the offset is a non-zero value, UE can measurement all the SSBs in the time duration of the offset and determine which SSB has the best receive quality, then UE can only receive the PEI which is associated with the SSB having best quality and no need to receive all the PEI occasion.

Figure 6. Monitoring occasion for PEI.

- A monitoring window and an offset between the SS burst and the monitoring window can be specified for the PEI design.

Conclusions

In this contribution, we discuss the paging procedure enhancement. Based on the analysis, we have the following observations and proposals:

- To fulfil the requirement of joint detection performance of PEI and paging DCI is no worse than 1% at the SNR where the MDR of paging PDSCH is 1%, the required transmissions or configurations for different physical layer candidates are listed in Table 1.

- DCI based PEI can fully support coexistence with the legacy downlink physical layer channels/signals of legacy UE based on Rel-15 specifications.

- Semi-statically configured rate matching pattern needs to be used to multiplex SSS-based PEI and PDSCH(s) of legacy UE.

- It can only rely on gNB to carefully handle the overlapping of SSS-based PEI with PDCCH(s) of legacy UE and SSB.

- Rel-15 only supports up to 3 aperiodic ZP-CSI-RS resource sets per BWP and it is expected that there is no available ZP-CSI-RS resource set for the coexistence of TRS/CSI-RS based PEI

- Semi-static RB-level rate matching pattern is expected to be used for the coexistence of TRS/CSI-RS based PEI.

- For PDCCH-based PEI, the payload can be used for sub-group indication and associated with multiple POs flexibly.

- PDCCH-based PEI can provide the best efficient resource utilization and the lowest UE wake up rate compared to SSS-based and TRS based PEI for both Behv-A and Behv-B.

- The SSS-based PEI has impact on the legacy cell search procedure.

- Sequence-based PEI signals is not suitable to support the indication of short message.

- Sequence-based PEI signals is not suitable to support other power related information, e.g. assistance TRS availability indication

- DCI-based PEI can be flexibly configured to carry PEI indication, sub-group indication, short message and other power saving functionalities, e.g. the availability indication of assistance TRS.

- Adopt DCI carried by PDCCH as the physical layer channel for PEI indication in Rel-17.

- Considering there are different UEs in a cell, there can be multiple PEI occasionss indicating the same PO.

- A monitoring window and an offset between the SS burst and the monitoring window can be specified for the PEI design.

- References

R2-2010884, LS on Paging Enhancement, MediaTek

TR38.213, Physical layer procedures for control, 3GPP

TR38.822, User Equipment(UE) feature list (Release 15), 3GPP

TS 38.211, Physical channels and modulation, 3GPP

R1-2100216, “Paging enhancement(s) for UE power saving in IDLE/inactive mode”, Huawei, Hisilicon

R1-2100217, “Assistance RS occasions for IDLE/inactive mode”, Huawei, Hisilicon

- Appendix A. Paging enhancement agreements in RAN1#104-e

Agreement: - Carrying UE subgroups information is considered in physical layer design for paging enhancement Agreements: For the evaluation and comparison of PEI candidate designs based on PDCCH, TRS/CSI-RS and SSS, the following are assumed: - Behv-A: - PEI indicates UE should monitor a PO if UE’s group/subgroup is paged - UE is not required to monitor a PO if UE does not detect PEI at all PEI occasion(s) for the PO - Behv-B: - PEI indicates whether or not UE should monitor a PO - UE is required to monitor a PO if UE does not detect PEI at all PEI occasion(s) for the PO Agreements: For the evaluation and comparison of PEI candidate designs, companies to report - Description of how PEI design can co-exist with existing channels/signals, and impact to legacy UEs. - Rel-15 designs for multiplexing PEI with legacy channels/signals are assumed as baseline - Other multiplexing method with legacy channels/signals can be additionally reported with justification Agreements: - Take Alt 1 as mandatory, and Alt 2 as optional Alt 1: For the performance evaluations of PEI candidate designs based on PDCCH, TRS/CSI-RS and SSS, - The following are assumed, at the SNR where the Miss-Detection Rate (MDR) of paging PDSCH is 1%, - When Behv-A is assumed: - The joint miss-detection rate (MDR) of PEI and paging PDCCH defined below should be no worse than 1%: MDR_Joint_A = MDR_PEI + (1 – MDR_PEI) MDR_PagingPDCCH - The False-Alarm Rate (FAR) of PEI should be no larger than [1%] - When Behv-B is assumed: - The joint miss-detection rate (MDR) of PEI and paging PDCCH defined below should be no worse than 1%: MDR_Joint_B = FAR_PEI + (1 – FAR_PEI) MDR_PagingPDCCH - The MDR of PEI should be no larger than [1%] - Note: The CFO is modeled at the input of PEI detection and based on LLS assumptions agreed in RAN1 #102-e. Companies should justify the applied random range for the CFO. - Companies to provide: - Information on the utilized detection method for each PEI candidate design (e.g., non-coherent detection or coherent detection) - The required #REs to comply with the performance assumptions - The maximum number of subgroups that can be carried in PEI, subject to the performance assumptions Alt 2: For the performance evaluations of PEI candidate designs based on PDCCH, TRS/CSI-RS and SSS, - The following are assumed, at the SNR where the Miss-Detection Rate (MDR) of paging DCI is 1%, - When Behv-A is assumed: - The MDR of PEI should be no larger than 0.1% - The False-Alarm Rate (FAR) of PEI should be no larger than 1% - When Behv-B is assumed: - The FAR of PEI should be no larger than 0.1% - The MDR of PEI should be no larger than 1% - Note: The CFO is modeled at the input of PEI detection and based on LLS assumptions agreed in RAN1 #102-e. Companies should justify the applied random range for the CFO. - Companies to provide: - Information on the utilized detection method for each PEI candidate design (e.g., non-coherent detection or coherent detection) - The required #REs to comply with the performance assumptions - The maximum number of subgroups that can be carried in PEI, subject to the performance assumptions Agreements: For the evaluation of resource overhead with PEI candidate designs based on PDCCH, TRS/CSI-RS and SSS, companies to provide estimated overheads for PEI candidate designs based on the following factors: - Assumption of Behv-A/B - Required #REs from performance evaluations - 10% group paging rate per PO as baseline; other group paging rates can be optionally considered and based on the following assumptions with justification (up to each company) - Whether and how coexistence with legacy UEs is considered - Whether and how indication(s) to multiple POs and/or UE subgroups by one PEI is considered - Whether and how multi-beam transmission is considered Decision: As per email posted on Feb 5th, Agreements: Further study the design on how to provide the indications for UE subgroups over PEI and/or paging PDCCH, subject to the metrics agreed in RAN1 102e.

- Appendix B. Simulation assumptions

Table 6. Simulation assumptions for PEI

| Carrier Frequency | 4GHz |
| --- | --- |
| SNR group | [-12, 3]dB |
| Channel Model | TDL-C, 300ns delay spread |
| Subcarrier spacing | 30kHz |
| Transmission BW | 20MHz |
| UE speed | 3km/h |
| Antenna configuration | 2TX and [2, 4]RX |
| Frequency error | For PEI: Uniform distribution [-0.5, 0.5]ppm For paging PDCCH and PDSCH: 0ppm |
| Paging PDCCH configuration | 48RB, 2symbols, AL8, and AL16 40 info + 24 CRC bits, REG bundle size 6 |
| Paging PDSCH configuration | Mapping type A, MCS 0, 48 PRB, TB scaling 1, DMRS type 1 with 2 additional DMRS |
| PDCCH-PEI configuration | 24RB, 2symbols, AL4 12 info + 24 CRC bits, REG bundle size 6 |
| SSS configuration | 127 length SSS sequence, 1 symbols |
| TRS configuration | 24 RB, 2 symbols, 3RE/RB/port |

- Appendix C. UE procedure for receiving control information [2]

For monitoring of a PDCCH candidate in a slot - If the UE has received ssb-PositionsInBurst in SIB1 and has not received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell and if the UE does not monitor PDCCH candidates in a Type0-PDCCH CSS set and at least one RE for a PDCCH candidate overlaps with at least one RE corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in SIB1, the UE is not required to monitor the PDCCH candidate. - If a UE has received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell and if the UE does not monitor PDCCH candidates in a Type0-PDCCH CSS set and at least one RE for a PDCCH candidate overlaps with at least one RE corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in ServingCellConfigCommon, the UE is not required to monitor the PDCCH candidate. - If the UE monitors the PDCCH candidate for a Type0-PDCCH CSS set on the serving cell according to the procedure described in Subclause 13, the UE may assume that no SS/PBCH block is transmitted in REs used for monitoring the PDCCH candidate on the serving cell. - If at least one RE of a PDCCH candidate on the serving cell overlaps with at least one RE of lte-CRS-ToMatchAround, the UE is not required to monitor the PDCCH candidate.

- Appendix D. Layer-1 features related to rate matching [3]

| Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5-27 | Dynamic rate-matching resource set configuration for DL | Bitmap 1/2/3 |  | rateMatchingResrcSetDynamic | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
| 5-27a | Dynamic rate-matching control resource set for DL | Dynamic rate-matching control resource set for DL |  | rateMatchingCtrlResrcSetDynamic | Phy-ParametersCommon | No | No |  | Mandatory with capability signalling |