3GPP TSG RAN WG1 Meeting #104bis-e	R1-2102344

E-meeting, April 12th – April 20th, 2021

Agenda Item:	8.15.2

Source:	Huawei, HiSilicon

Title:	Discussion on time and frequency synchronization enhancement for IoT in NTN

Document for:	Discussion and Decision

# Introduction

In RAN1#104-e, several aspects were identified to enhance the timing and frequency synchronization for IoT in NTN [1]

Agreement:

Study potential impact of GNSS Position fix on UE power consumption using battery life methodology in Rel-13 TR 45.820 (Section 5.4)

FFS: Details of the study

Agreement:

Discuss whether GNSS measurement window is needed and beneficial for initial access.

Agreement:

For the study of potential impact of GNSS Position fix on UE power consumption consider at least the following parameters

- GNSS power consumption value

- GNSS position Time To First Fix

Agreement:

Study potential impact of NTN SIB carrying the satellite ephemeris on

- UE power consumption in NB-IoT and eMTC

- Accuracy of satellite location tracking

- PRACH congestion

Agreement:

Study the UE pre-compensation of satellite delay during long UL transmission on (N)PUSCH in NB-IoT and eMTC.

Agreement:

Study the UE pre-compensation of satellite delay and Doppler during long UL transmission on PRACH in NB-IoT and eMTC.

Agreement:

Study the UE pre-compensation of satellite Doppler shift during long UL transmission on (N)PUSCH in NB-IoT and eMTC.

In this contribution, we discuss the key issues of time and frequency synchronization for IoT over NTN.

# Discussion

## UL time and frequency synchronization enhancement

GNSS capability in the UE is taken as the working assumption for the study of IoT over NTN. With this assumption, UE can estimate and pre-compensate the timing and frequency offset of the service link based on its GNSS acquired position and serving satellite ephemeris with sufficient accuracy [2]. From this perspective, some of the agreement and solutions developed in NR NTN can be applied for IoT over NTN

Timing advance based on common TA and UE-specific TA

As discussed in [2], the timing advance applied by UE for UL transmission is given below

TA= NTA+NTA,UE-specific+NTA, offset+NTA, common with margin×Tc

NTA, common with margin=NTA, common-NTA,margin , is a common TA offset that includes the maximum allowed TA estimation error NTA,margin to avoid inter-symbol interference (ISI) at the gNB receiver. NTA,UE-specific is the UE-specific UE TA based on its GNSS acquired position and serving satellite ephemeris. As discussed in [2], it is important to support the case where the UL and DL frame timing are aligned at a network controlled reference point. This not only ensures compatibility with the current gNB implementations but also avoids the specification impact such as MAC-CE action timing, determination of RAR window timing, etc. From this perspective, at least common TA indication should also be supported for IoT over NTN.

Proposal 1: An IoT-NTN UE at least supports TA calculation based on common TA indication and a UE specific TA.

TA drift rate for TA maintenance

Before discussing the enhancements on TA maintenance, the primary features of NB-IoT are identified as follows：

- Both DL and UL may require a large number of repetitions to ensure the coverage. The number of repetitions for UL can be up to 128 and the corresponding transmission duration will be longer than 2 seconds.

- A 40 ms UL gap is applied after 256 ms UL transmission duration to allow UEs to perform time and frequency synchronization

- For preamble transmission, there will an UL gap inserted after 64 times of preamble repetition and the maximum times of repetition is 128.

- Even with UL gap after each segment of 256 ms, an NB-IoT will not receive TAC for TA adjustment during each report.

- An NB-IoT UE will only monitor system information in RRC idle mode, which means that any common timing/frequency offset broadcast in SIB1 cannot be received by UE during UL transmission with long duration.

- An NB-IoT UE works in half duplex mode, which means that UL transmission and DL reception will not happen simultaneously.

Table 1. Preamble formats of NB IoT

| Preamble format | P | TCP | TSEQ | Duration of one repetion | Maximum time-continuous transmission | Time duration with maximum repetions |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 4 |  |  | 5.6ms | 358.4ms | 756.8ms |
| 1 | 4 |  |  | 6.4ms | 409.6ms | 859.2ms |
| 2 | 6 |  | 3∙24576Ts | 19.2ms | 307.2ms | 2737.6ms |

Figure 1.  Maximum allowed time-continuous transmission with different elevation for LEO-600km

There are 3 preamble formats applicable for FDD as shown in Table 1 where P is the number of symbol groups in a preamble repetition. For preamble formats 0 and 1, a gap of 40ms (40‧30730Ts) will be inserted after transmissions of 64 repetitions (64∙4TCP+TSEQ) while for formats 2 a gap with the same length will be inserted after 16 times of repetitions (16∙6TCP+TSEQ). The maximum number of repetitions for preamble is 128. According to Table 1, the maximum duration of time-continuous  transmission is up to 409.6ms and the duration with maximum preamble repetition number is up to 2737.6ms. The timing offset introduced by the movement of satellite during the whole preamble transmission is 4174.2 Ts with 10° elevation. Such a large timing offset will degrade the detection of the preamble sequence and as a result cause the failure of random access. If TA is compensated during the UP gap, the maximum timing drift after the time-continuous transmission is 624.5Ts with 10° elevation which is larger than the timing error limit of 80Ts defined in [4]. Figure 1 illustrates the maximum allowed time-continuous transmission with different elevation for LEO-600km. Therefore, at least TA adjustment should be applied during the long preamble transmission to compensation the large timing drift. It should be noted that with different TA adjustment during preamble transmission the detection window at the gNB side need to be adjusted accordingly.

Observation 1: There will be a large timing drift in case of large number of repetitions for preamble transmission.

Proposal 2: UE autonomous TA adjustment should be applied during the long preamble transmission duration to compensate the large timing drift.

For UL data transmission, an UL gap duration will be inserted after each 256 ms of transmission, the timing offset is around 390 Ts with 10° elevation, which is larger than the timing error requirement for the UL transmission. To compensate the timing drift and reduce the timing error to within the maximum allowed timing error, shorter segmentation for UL transmission should be introduced for NTN IoT. In other words, extra UP gaps should be inserted during each 256 ms transmission.

Observation 2: There will be a large timing drift in case of 256ms time-contiguous transmission for NPUSCH.

Proposal 3: More UL gaps should be inserted according to the maximum allowed time-continuous transmission for IoT over NTN.

If the TA compensation for TA drift caused by the movement of satellite TA needs to be updated during one UL transmission, UE needs to acquire the system information carrying satellite ephemeris and common TA indication for TA drift calculation. However, an NB-IoT device will not receive system information in the RRC connected mode. Moreover, decoding system information during UL repetition or receiving closed loop TAC command will introduce extra power consumption which is not preferable for IoT devices. To solve this issue, TA drift rate can be applied to calculate the timing drift during long UL transmission. The TA drift rate applied by UE side is composed by common TA drift rate part (drift rate of X) and UE specific TA drift rate part (drift rate of NTA). The UE specific part can be derived by UE location and satellite ephemeris and the common TA drift rate part should be indicated by the network. Both of the drift rates have to be applied by the UE side to track the TA variation from service link part and feeder link part, respectively. The detailed TA calculation based on TA drift rate in given in [2].

Observation 3: Decoding system information or receiving closed loop TAC command for TA adjustment during UL repetition will introduce extra power consumption for IoT devices.

Proposal 4: Using TA drift rate to calculate and compensate the TA drift for UL transmission with long duration.

UL frequency alignment

Similar to NR NTN, frequency pre-compensation should be applied to reduce the DL detection complexity at the UE side. It is discussed in our companion paper [3] that the indication of frequency pre-compensation, is normalized to a predefined subcarrier spacing considering the tolerance of frequency offset for UL. For UL frequency alignment, as there is almost no difference of complexity for UE to do frequency pre-compensation with different values, it is straightforward to adopt a scheme with as less indication as possible. To avoid extra signaling overhead, the post-compensated common frequency offset for UL transmission can be equivalent to the pre-compensated common frequency offset applied for DL due to the same geometry of UL and DL.  Another scheme is that the frequency offset from the feeder link part is compensated by the satellite and the service link part is compensated by UE according to the position and ephemeris. In summary, only frequency offset (DL pre-compensation) is needed for IoT NTN.

Proposal 5: The indication of DL frequency pre-compensation is normalized to a predefine subcarrier spacing.

Proposal 6: To reduce the signaling overhead, only DL pre-compensation indication is needed and sufficient for UL frequency alignment.

## RACH failure due to long transmission duration

Figure 2. NB-IoT UE RACH failure due to insufficient coverage time

In NB-IoT, an extensive number of repetitions is supported to effectively exploit the time variation of the wireless channel for coverage enhancement, up to 2048 repetitions in DL and up to 128 repetitions in UL. The large number of repetitions implies that the UL and DL transmission can take a long time, for instance, RACH procedure may take more than 20 seconds for an NB-IoT. Take Figure 2 as an example, an NB-IoT UE performs random access NTN in a moving cell coverage (Cell 1), and the remaining time for the UE to stay in Cell 1 is 18 seconds. If RACH procedure of this UE needs more than 18 seconds, this UE may not be able to complete initial access in Cell 1. The reason for this is that the UE has entered another cell coverage (Cell 2), when the Cell 1 sends Msg4 to it. As a result, RACH failure may occur due to the fact that an NB-IoT UE may stay in a cell for a short time. This leads to increased UE power consumption which needs further study, since low-cost NB-IoT UEs are usually also power-sensitive.

Observation 4: RACH failure may happen for an NB-IoT UE since it may stay in the cell for a short time, which leads to increased power consumption.

## Power consumption for IoT NTN

IoT NTN is featured with GNSS capability and extra SIB information, e.g. ephemeris, Koffset, common TA, common frequency pre-compensation, et al. The battery life of IoT NTN should be evaluated at least with the extra power consumption introduced by this two features. For stationary IoT devices, the GNSS position fix can be done only once and the power consumption for GNSS signal reception can be neglected. For moving IoT devices with high speed, it may have to do the position fix before each UL transmission. Both the GNSS power consumption model and GNSS position Time To First Fix will have an impact on the power consumption. There are three kind of GNSS start mode with different time required by the receiver to perform the positioning after the module was switched on.

- The first is cold start with which no almanac information is stored in the receiver. The UE have to search signals from all the possible satellites and at least 4 satellites are needed for the positioning. Therefore, the duration will be affected by the rate of GNSS signal transmission and quality of reception. The time duration of cold start can range from tens of seconds to more than ten minutes. The typical values of cold start is 30 s if the GNSS signal is received with not much interruption.

- The second start is warm start which is based on the assumption that the some ephemeris information and clock correction data is already obtained. With some available information, the positioning time will be reduced to several seconds.

- The third start is hot start which is based on the assumption that GNSS receiver has valid ephemeris, clock correction and GNSS time reference with time for positioning can be as low as 1~2s.

Generally, cold start is needed for the first time the devices is turned on or the battery is depleted that all the ephemeris information is lost. In summary, cold start will be operated only once for the IoT NTN devices. Hot start will be valid for about 2 hours, which mean that the UE can use hot start with period of report is not larger than 2 hours. For other services with report period longer than 2 hours, warm start can be suggested.

For extra SIB information, as the information changed from time to time, the power consumption depends on how frequent the UE have to read the NTN related SIB information for each UL transmission. As the total power consumption introduced by NTN related SIB information and GNSS will be impacted by GNSS position fix time, GNSS period, time for NTN related SIB reading, we give the battery life evaluation based on the simulation assumption provided in the Appendix.  It can be observed that power consumption of GNSS and NTN related SIB reading have a large impact to battery life and the extent is related to the report period and UE speed, which determines the start mode of GNSS receiver.

Observation 5: Power consumption of GNSS and NTN related SIB reading have a large impact to battery life of IoT devices.

## DL synchronization

In [1], two extra sets of satellite are agreed with larger beam diameter sizes and as a result there will be a lager residual Doppler shift at UE side. The differential Doppler of LEOs for set 3 and set 4 at 2GHz central frequency is shown in Table 2.

Table 2 Differential Doppler of set3 and set4 with 2GHz central frequency

| Satellite | Set 3 | Set 3 | Set 4 |
| --- | --- | --- | --- |
| Satellite orbit | LEO-1200 | LEO-600 | LEO-600 |
| Satellite altitude | 1200 km | 600 km | 600 km |
| Central beam edge elevation | 30 degree | 30 degree | 30 degree |
| Central beam center elevation | 46.05 degree | 43.8 degree | 90 degree |
| Beam diameter size | 1110.09Km | 610.8Km | 1701.8Km |
| Differential Doppler | +/-21.56KHz | +/-21.14KHz | +/-39.9KHz |

From table 2, it can be observed that the differential Doppler shift caused by satellite movement can be up to +/-39.9KHz. Considering the frequency error caused by local oscillator, i.e. +/-20kHz assuming 10ppm, the total differential frequency shift become larger than +/-50KHz assumption for synchronization raster of NB-IoT. This will cause the error in estimation of central frequency. To solve this issue, one simple way is to increase the raster to be larger than the maximum differential frequency shift.

Observation 6: The differential frequency shift in IoT over NTN may become larger than the existing channel raster in some deployment.

# Conclusion

In this contribution, we discuss the time/frequency adjustment, RACH enhancement and power consumption introduced by GNSS and NTN related SIB reading in IoT NTN. The following observations and proposals are presented.

Observation 1: There will be a large timing drift in case of large number of repetitions for preamble transmission.

Observation 2: There will be a large timing drift in case of 256ms time-contiguous transmission for NPUSCH.

Observation 3: Decoding system information or receiving closed loop TAC command for TA adjustment during UL repetition will introduce extra power consumption for IoT devices.

Observation 4: RACH failure may happen for an NB-IoT UE since it may stay in the cell for a short time, which leads to increased power consumption.

Observation 5: Power consumption of GNSS and NTN related SIB reading have a large impact to battery life of IoT devices.

Observation 6: The differential frequency shift in IoT over NTN may become larger than the existing channel raster in some deployment.

Proposal 1: An IoT-NTN UE at least supports TA calculation based on common TA indication and a UE specific TA.

Proposal 2: UE autonomous TA adjustment should be applied during the long preamble transmission duration to compensate the large timing drift.

Proposal 3: More UL gaps should be inserted according to the maximum allowed time-continuous transmission for IoT over NTN.

Proposal 4: Using TA drift rate to calculate and compensate the TA drift for UL transmission with long duration.

Proposal 5: The indication of DL frequency pre-compensation is normalized to a predefine subcarrier spacing.

Proposal 6: To reduce the signaling overhead, only DL pre-compensation indication is needed and sufficient for UL frequency alignment.

# References

RAN1#104-e Chairman's Notes

RP-202689, “Study on NB-IoT/eMTC support for Non-Terrestrial Network”

R1-2102342, “Discussion on UL time and frequency synchronization enhancement for NTN”, Huawei, HiSilicon

TR 36.133, Requirements for support of radio resource management

# Appendix

Table 1. Power consumption with 2 hours report period

| Report, 2hours |  |  |  |
| --- | --- | --- | --- |
| Flow assumptions | Duration(ms)/each report | Power(mW) | Power consumption(mWh) |
| GNSS(DL) | 2000 | 100 | 0.055556 |
| NPSS(DL) | 20 | 80 | 0.000444 |
| NSSS(DL) | 20 | 80 | 0.000444 |
| NTN SIB(DL, 256bits) | 24 | 70 | 0.000467 |
| MIB-NB(DL) | 60 | 70 | 0.001167 |
| Msg1(UL) | 102.4 | 500 | 0.014222 |
| NPDCCH(DL) | 8 | 70 | 0.000156 |
| Msg2(DL, 56bits) | 12 | 70 | 0.000233 |
| Msg3(UL) | 96 | 500 | 0.013333 |
| NPDCCH(DL) | 8 | 70 | 0.000156 |
| Msg4(DL, 256bits) | 24 | 70 | 0.000467 |
| ACK/NACK for Msg4(UL) | 16 | 500 | 0.002222 |
| PDCCH(DL) | 8 | 70 | 0.000156 |
| Msg5(105bytes) | 320 | 500 | 0.044444 |
| Idle | 30000 | 3 | 0.025000 |
| Sleep(NTN) | 7167281.6 | 0.015 | 0.029864 |
| Sleep(TN) | 7169305.6 | 0.015 | 0.029872 |
| Total (TN, mWh) | 0.132317 |  |  |
| Total (NTN,mWH) | 0.188330 |  |  |
| Battery(Wh) | 5.000000 |  |  |
| Battery lifte (TN, year) | 8.627436 |  |  |
| Battery lifte (NTN, year) | 6.061437 |  |  |

Table 2. Power consumption with 6 hours report period

| Report, 6hours |  |  |  |
| --- | --- | --- | --- |
| Flow assumptions | Duration(ms)/each report | Power(mW) | Power consumption(mWh) |
| GNSS(DL) | 5000 | 100 | 0.138889 |
| NPSS(DL) | 20 | 80 | 0.000444 |
| NSSS(DL) | 20 | 80 | 0.000444 |
| NTN SIB(DL, 256bits) | 24 | 70 | 0.000467 |
| MIB-NB(DL) | 60 | 70 | 0.001167 |
| Msg1(UL) | 102.4 | 500 | 0.014222 |
| NPDCCH(DL) | 8 | 70 | 0.000156 |
| Msg2(DL, 56bits) | 12 | 70 | 0.000233 |
| Msg3(UL) | 96 | 500 | 0.013333 |
| NPDCCH(DL) | 8 | 70 | 0.000156 |
| Msg4(DL, 256bits) | 24 | 70 | 0.000467 |
| ACK/NACK for Msg4(UL) | 16 | 500 | 0.002222 |
| PDCCH(DL) | 8 | 70 | 0.000156 |
| Msg5(105bytes) | 320 | 500 | 0.044444 |
| Idle | 30000 | 3 | 0.025000 |
| Sleep(NTN) | 21564281.6 | 0.015 | 0.089851 |
| Sleep(TN) | 21569305.6 | 0.015 | 0.089872 |
| Total (TN, mWh) | 0.192317 |  |  |
| Total (NTN,mWH) | 0.331651 |  |  |
| Battery(Wh) | 5.000000 |  |  |
| Battery lifte (TN, year) | 17.807399 |  |  |
| Battery lifte (NTN, year) | 10.326083 |  |  |

- Table 3. Power consumption with 6 hours report period

| Report, 12hours |  |  |  |
| --- | --- | --- | --- |
| Flow assumptions | Duration(ms)/each report | Power(mW) | Power consumption(mWh) |
| GNSS(DL) | 5000 | 100 | 0.13889 |
| NPSS(DL) | 20 | 80 | 0.00044 |
| NSSS(DL) | 20 | 80 | 0.00044 |
| NTN SIB(DL, 256bits) | 24 | 70 | 0.00047 |
| MIB-NB(DL) | 60 | 70 | 0.00117 |
| Msg1(UL) | 102.4 | 500 | 0.01422 |
| NPDCCH(DL) | 8 | 70 | 0.00016 |
| Msg2(DL, 56bits) | 12 | 70 | 0.00023 |
| Msg3(UL) | 96 | 500 | 0.01333 |
| NPDCCH(DL) | 8 | 70 | 0.00016 |
| Msg4(DL, 256bits) | 24 | 70 | 0.00047 |
| ACK/NACK for Msg4(UL) | 16 | 500 | 0.00222 |
| PDCCH(DL) | 8 | 70 | 0.00016 |
| Msg5(105bytes) | 320 | 500 | 0.04444 |
| Idle | 30000 | 3 | 0.02500 |
| Sleep(NTN) | 43164281.6 | 0.015 | 0.17985 |
| Sleep(TN) | 43169305.6 | 0.015 | 0.17987 |
| Total (TN, mWh) | 0.282317 |  |  |
| Total (NTN,mWH) | 0.421651 |  |  |
| Battery(Wh) | 5.000000 |  |  |
| Battery lifte (TN, year) | 24.261118 |  |  |
| Battery lifte (NTN, year) | 16.244032 |  |  |