3GPP TSG RAN WG1 Meeting #104-bis-e  									R1-2102360

e-Meeting, April 12th – April 20th, 2021

Agenda Item:	8.4.2

Source:	Thales

Title:	Considerations on UL timing and frequency synchronization in NTN

Document for:	Discussion and decision

# Introduction

The agreements related to UL time and frequency synchronization in NTN achieved during RAN1 Meeting #104-e are captured in FL summary for UL synchronization [ R1-2102215].

In this contribution, we make observations and proposals to address issues identified and discussed during RAN1#104e meeting.

# UL timing synchronization in NTN

## NTN UE Timing Advance

The Timing Advance to be applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED was discussed in RAN1 Meeting #104-e, the following formula was proposed but not yet agreed:

TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc

NTA is defined as in Release-16.

NTA,UE-specific is derived from the User specific TA self-estimation corresponding to the service link RTD and autonomously acquired by the UE  based on its GNSS position and the satellite ephemeris.

NTA, offset is specified in TS 38.213 section 4.2 and a set of NTA, offset  values are specified in TS 38.133

is the basic timing unit defined in TS 38.211: where  Hz and .

NTA,common is referring to network-controlled common TA, it may include any timing offset considered necessary by the network: the gNB may indicate it to the UE to deal with the RTD on the feeder link and the GW-gNB RTD. If common TA is not present in the SIB the UE shall apply: NTA,common=0.

Figure 1 TA components in NTN

The Proposal made by Moderator in [1] can be updated as follows:

Proposal 1:

The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by:

When common TA is indicated by the Network:

TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc

When common TA is not indicated by the Network:

TTA=NTA+NTA,UE-specific+NTA,offset×Tc

Where:

  NTA and NTA,offset are defined as in Release-16.

  NTA,UE-specific  is UE self-estimated TA

  NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network.

When the reference point for time synchronization is the satellite, the gNB may not need to indicate the Common TA which is de facto equal to 0. But the UE should be informed that it does not have to perform Common TA acquisition in the SIB.

Proposal 2:

Introduce a flag in the SIB to indicate whether the Common TA is present and need to be applied for TTA calculation

## Common TA characterization

### Common TA equations

When Common TA is indicated by the Network, the UE needs to estimate NTA,common before each uplink transmission (including the PRACH transmission).

To assist the UE to self-estimate the NTA,common  at time t, the gNB shall broadcast the Common TA (NTA,common t0) ) and Common TA drift rate ( TACommondrift , t0 ) regularly (e.g. every 160ms). The UE can then estimate the  NTA,Common (t) based  on a first-order approximation using Equation 1 hereafter, expressed at unit of Tc:

Equation 1:

NTA,Common t= NTA,common t0+   TACommondrift, t0×(t-t0+δt0)

Further, the timing drift rate variation may need also to be indicated by the network so the UE can keep track of quick common delay changes without the need of frequent decoding of system information. Thus, if the gNB broadcast the TACommondrift_variation_rate , The UE can then estimate the  NTA,Common (t) based  on a second-order approximation using Equation 2 below, expressed at unit of Tc:

Equation 2:

NTA,Common t= NTA,common t0+   TACommondrift, t0×t-t0+δt0+12 TACommondrift_variation_rate, t0×t-t0+δt02

NTA,common t0, TACommondrift, t0 and TACommondrift_variation_rate, t0 denote respectively the Common TA, Common TA drift rate and Common TA drift variation rate acquired by the UE at SIB period t0.

δt0 denotes one-way propagation delay from gNB to the UE at time t0 (UE’s timing). This needs to be considered  in both equations to take into account the ageing of NTA,common t0 values (the NTA,common t0 at time t0 is outdated by δt0 at the reception by the UE).

For better accuracy of self-estimated Common TA a second-order approximation may be needed, thereby, both Common TA drift rate and Common TA drift variation rate may need to be indicated.

### Required time period for common TA acquisition

The cyclic prefix must accommodate the round trip propagation delay and the propagation channel delay spread. Therefore, for PRACH transmission the UE shall be able to estimate its TA with an accuracy better than ±(CP-delay spread)/2. Depending on the PRACH format. Similarly, in connected state, the UE shall be able to update its TA with an accuracy better than ±(CP-delay spread)/2.  depending on the numerology in use.

Further, the maximum allowed error of the round trip propagation delay estimation can be equally split between the estimation of  NTA,UE-specific and NTA,common. Therefore the UE shall be able to self-estimate the Common TA within an error range Common TAerror which at least satisfies (RAN4 may define different requirement; e.g based on Time transmission error limit Te) the following requirement:

Common TAerror<CP-Delay spread4  .

And so, for the following most restrictive cases, if the delay spread is neglected, the maximum tolerable error on common TA estimation should be less than:

- ± 24.20 μs, in case of long PRACH Format0 and 3.

- ± 2.34 μs, in case of short PRACH format A1 with 15 kHz subcarrier spacing (SCS)

- ± 0.29 μs, in case of  short PRACH format A1 with 120 kHz SCS

- ± 1.17 μs, in case of connected state and 15 kHz SCS

- ± 0.15 μs, in case of connected state and 120 kHz SCS

To self-estimate the common TA, the UE needs to acquire the periodically broadcasted assistance information; that is the Common TA, Common TA drift rate and possibly the Common TA drift variation rate. Then,  it  shall perform a first-order approximation of Common TA using Equation 1 or a second-order approximation Common TA using Equation 2.

The assistance information acquired in the SIB  will be valid only during a validity time period (equals to CTA_validity_time_max) which  depends on the maximum tolerable error on common TA estimation and the order of the Common TA approximation to be carried out by the UE. At the reception of new assistance information, the UE shall start/restart related timer CTA_validity_time. At expiry of this timer a new acquisition of Common TA estimation assistance information is required.

The following Figures shows that by acquiring necessary assistance information indicated by the network, the UE can self-estimate the common TA during CTA_validity_time_max which is of the order of tens of seconds:

- In Idle or Inactive state, when first-order approximation of Common TA is used the UE needs to acquire via the SIB new assistance information at most 9 seconds before PRACH Format0 and 3 transmission as shown in Figure 2. And at most 28 seconds when second-order approximation of Common TA is used as shown in Figure 3.

- In Connected state, in case of  120 kHz SCS, the UE needs to acquire new assistance information at most 700 ms before each uplink transmission when first-order approximation of Common TA is used as shown in Figure 4. And at most 5 seconds when second-order approximation of Common TA is used as shown in Figure 5.

By considering the most restrictive case (i.e. UE in Connected state and 120kHz SCS) the UE can self-estimate the Common TA with sufficient accuracy provided that an updated related assistance information is available at the UE during the last 0.7 s when Common TA drift rate is indicated or during the last 5s when both Common TA drift rate  and Common drift variation rate are indicated.

A validity timer configured for Common TA defines the maximum time during which the UE can track the Common RTD without having to perform new assistance information acquisition to be used for Common TA self -estimation.

Figure 2 First-order approximation of Common TA for PRACH format 0/3 transmission

Figure 3 Second-order approximation of Common TA for PRACH format 0/3 transmission

Figure 4 First-order approximation of Common TA in Connected state – SCS 120 kHz

Figure 5 Second-order approximation of Common TA in Connected state – SCS 120 kHz

Based on simulation results, the maximum time during which the UE can track the Common RTD without having acquired new assistance information is determined for the most restrictive cases as recapped in Table 1.

The following assumptions were considered:  LEO @600 km scenario, the satellite flies over the GW (worst case) and the gNB knows exactly the satellite trajectory.

The values of CTA_validity_time_max given in Table 1 need to be revisited to take into account the maximum error on the Common RTD estimated by the gNB (before being broadcast) and the error of  quantization of assistance information.

Table 1 maximum time that the UE can track the Common RTD without having acquired new assistance information

| Uplink transmission | minimum required period to acquire assistance information for self-common TA estimation |  |  |
| --- | --- | --- | --- |
|  | Broadcast of Instantaneous Common TA DRIFT W/O Common TA drift variation indication | Broadcast of averaged Common TA DRIFT W/O Common TA drift variation indication | Broadcast of Instantaneous Common TA DRIFT With Common TA drift variation indication |
| PRACH Format0 and 3 | 9 s | 18 s | 28 s |
| PRACH format A1 with 15 kHz SCS | 2.8 s | 5,5 s | 13 s |
| PRACH format A1 with 120 kHz SCS | 0.98 s | 1,9 s | 6.5 s |
| Connected state and 15 kHz SCS | 2 s | 4 s | 10,5 s |
| Connected state and 120 kHz SCS | 0.7 s | 1.4 s | 5 s |

Based on the above results, it can be seen that apart from indicating the instantaneous Common TA Drift rate it would be beneficial to indicate as well the Common TA drift variation rate. This will avoid to some degree frequent decoding of system information. But this comes at the price of signaling overhead increase.

Alternatively, to assist the UE to self-estimate the Common TA, without indicating Common TA drift variation rate, the network may indicate an averaged Common TA Drift rate (instead of  Instantaneous Common TA Drift rate). Common TA Drift rate can be averaged over a sliding window X as follows:

TACommondrift(t)= CommonTAt+X-CommonTA(t)  X

The averaging period X can be determined based on the maximum tolerable error on common TA estimation.

Figure 6 First-order approximation of Common TA using averaged TA Drift rate- PRACH format 0/3 transmission

Figure 7 First-order approximation of Common TA using averaged TA Drift rate - Connected state with SCS 120 kHz

To assist the UE in self-estimating the Common TA, without indicating Common TA drift variation rate, the network may indicate an averaged Common TA Drift rate (instead of  Instantaneous Common TA Drift rate).

### Common TA And Common TA drift signaling overhead

NTA,Common  values shall be indicated in SIB by index values of  TA_common = 0, 1, 2, ..., p. Where an amount of the common TA with subcarrier spacing of kHz is given by:

NTA,Common =

p being the maximum range of TA_common. Thereby, the maximum  common TA for numerology µ is given by :

NTA,Common, max =p*16*64/2μ, where  NTA,Common, max is calculated for 120kHz subcarrier spacing and  TA_common = p.

Thus, when the reference point for uplink time synchronization is at the Radio Unit (RU) collocated with GW:

p= NTA,Common, max =maximum RTD on the feeder (ms) 1000.Tc.

The maximum RTD on the feeder link that shall be supported in NTN is 270.73 ms in case GEO based non-terrestrial access network, 12.89 ms (600km), 20.89 ms (1200km) in case of LEO based non-terrestrial access network

With the above considerations in mind, the maximum range of TA-Common as well as the number of bits to be used for TA-Common in SIB are given in the following table:

Table 2 Payload of common TA

| Scenarios | GEO based NTN | LEO based NTN |
| --- | --- | --- |
| maximum range of TA-Common | 4158413 | 197990 (600km), 320870 (1200km) |
| Payload (bits) | 23 bits | 19 bits (600km), 19 bits (1200Km) |

Moreover, it was observed through the simulations that in case of LEO-600, a field of 13 bits will be needed for Common TA drift rate indication and a field of 6 bits will be needed for Common TA drift variation rate.

Proposal 3:

If Common TA shall be considered in UE autonomous TA calculation, the Network shall periodically broadcast:

- in case of GEO based non-terrestrial access network:

  - Common TA (NTA,common ) in a field of 23 bits

- in case of LEO/MEO based non-terrestrial access network:

  - Common TA (NTA,common ) in a field of 19 bits

  - Common TA drift rate( NCommondrift) in a field of 13 bits

  - FFS: Common TA drift rate variation (NCommondrift_variation_rate ) in a field of 6 bits

## The need and the indication of TA margin

UE can either underestimate or overestimate its TA. In case of overestimation, the PRACH sequence will be received at gNB side in advance w.r.t. the RACH occasion leading to unwanted interference with previous OFDM symbols. To avoid such interference, the TA_margin is used by the UE for pre-adjustment/pre-refinement of the autonomously acquired initial TA as shown in Figure 8:

Figure 8  Pre-adjustment of autonmously acquired initial TA

Further, if initial TA does not include a margin for maximum TA estimation error. A bipolar TA command is needed in msg2. i.e support negative TA adjusting in RAR. It is preferable to avoid such specification impact.

if initial TA does not include a margin for maximum TA estimation error. A bipolar TA command is needed in msg2. i.e support negative TA adjusting in RAR

The value to be considered for TA-margin in the cell depends on maximum allowed TA estimation error. And as such, it is related to the discussions for timing synchronization requirements in RAN4.

Proposal 4: RAN1 to discuss the indication of TA-margin after RAN4 response to the LS on timing synchronization requirements

## TA update in connected mode

The following agreement was made in RAN#104-e

For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops shall be supported for NTN.

FFS: Details of the combination of open and closed loop TA control

### TA update based on closed loop

For TA update in RRC_CONNECTED state, closed loop based on TA Command  field in msg2/msgB and MAC CE TA command will dynamically update the component  NTA. The TA command is transmitted on a per-need basis with a granularity in the step size of 0.52 μs.

When TAC (TA) in msg2/msgB is received,  UE makes the first adjustment and NTA is updated as follows:

NTA= TA× 16×64, where, TA is the TAC field in msg2/msgB.  If initial TA does not include a margin for maximum TA estimation error, a bipolar TA command is needed in msg2. i.e support negative TA adjusting in RAR.

When TACs (TA) provided within the MAC CE is received, NTA is updated as follows:

NTA_new=NTA_old+TA-31.16.642μ, where, TA is the TAC field received in MAC CE  command.

Proposal 5:

Existing NTA update in RRC_CONNECTED based on TA Command  field in msg2/msgB and MAC CE TA command is used for UL timing alignment correction without specification change

- FFS: Whether to support negative TA Command in RAR

### TA update based on open loop

NTN UE needs to keep track of RTD variation on service link and on feeder link if reference point for uplink synchronization is not at satellite antenna input.

To do so, the UE needs to acquire in SIB the assistance information for TA maintenance including the Common TA,  the Common TA drift rate and the ephemeris data.

Proposal 6:

NTN UE in RRC_CONNECTED shall have an active BWP with common search space configured to monitor system information.

In our view, in RRC_Connected state the NTN UE shall update  NTA,UE-specific+NTA,common so that the cyclic prefix could absorb the propagation delay variation due to satellite movement  .

As discussed in section 2.2.2 the UE could autonomously predict and correct the common delay within 0,15 µs provided that an updated Common TA related assistance information is available at the UE during the last 0.7 s when Common TA drift rate is indicated or during the last 5s when both Common TA drift rate  and Common drift variation rate are indicated.

Moreover, as will be discussed in section 4.5 the UE could autonomously predict and correct the delay on service link within a timing error range of 0.15 µs provided that an updated ephemeris data is available at the UE during the last 45 s.

As a result, by updating NTA,UE-specific+NTA,common the UE could autonomously track the RTD variation on both service link and feeder link to keep the residual timing error within a maximum tolerable range that could be absorbed by the cyclic prefix when using 120kHz SCS.

### Time Alignment Timers

The legacy timeAlignmentTimer shall be used for closed loop: when the UE receives a MAC TAC, it updates its existing NTAand restarts the timeAlignmentTimer which defines the maximum time the UE can remain uplink synchronized without having received a TAC from the gNB. If this timer expires the UE assumes that it has lost uplink synchronization.

For NTA,UE-specific update the UE needs to acquire satellite ephemeris and use its own propagator to predict its specific TA during a certain time period without the need of acquiring new ephemeris data. The newly acquired ephemeris data will be valid only  during a period depending on propagator model used by the UE and the maximum tolerable error on the estimation of NTA,UE-specific as will be discussed in 4.5.

Proposal 7:

A validity timer configured for UE specific TA defines the maximum time during which the UE can track the RTD on service link without having acquired new ephemeris data to be used for UE specific TA estimation.

- This timer is restarted each time the UE receives new ephemeris data

- The UE assumes that it has lost uplink synchronization if this timer expires and new ephemeris data is not available.

Similarly, as discussed in section 2.2.2 for NTA,common update, the assistance information (Common TA, Common TA drift rate) acquired in the SIB will be valid only during a validity time period which depends on the maximum tolerable error on common TA estimation and the order of the Common TA approximation to be carried out by the UE. At the reception of new assistance information, the UE shall start/restart related timer. At expiry of this timer a new acquisition of Common TA estimation assistance information is required.

Proposal 8:

A validity timer configured for Common TA defines the maximum time during which the UE can track the Common RTD without having acquired new assistance information to be used for Common TA estimation.

- This timer is restarted each time the UE receives new assistance information.

- The UE assumes that it has lost uplink synchronization if this timer expires and assistance information is not available.

# UL frequency synchronization in NTN

Common Doppler shift pre/post compensation on the feeder links

As working assumption, RAN1 to consider that the GW pre/post compensates common Doppler shift / Doppler shift variation over the feeder link and any transponder frequency error at the satellite in a transparent way to the UE and gNB.

Proposal 9:

RAN1 working assumption is that GW compensates common Doppler shift / Doppler shift variation over the feeder link  and any transponder frequency error  in a transparent way to the UE and gNB.

Common Doppler shift pre compensation on the DL service link.

RAN1 made the following conclusion in RAN#104-e:

If DL frequency compensation for the service link Doppler is applied, indication of the amount of frequency compensation is necessary.

- FFS: support of DL frequency compensation for the service link Doppler.

We believe that common Doppler shift pre compensation on the DL service link is needed to assist DL synchronization at UE side and limit the complexity of SSB detection. As a consequence, a Reference Point (RP) w.r.t this pre-compensation is made shall be defined for each Earth-fixed cell.

Proposal 10:

If DL frequency compensation for the service link Doppler is applied, indication of the amount of frequency compensation is necessary:

- In case of earth-fixed cell, the beam-specific ECEF co-ordinates of a fixed Reference Point w.r.t the common Doppler shift experienced on the DL service link is pre-compensated by the gNB.

- In case of earth-moving beam, the beam-specific common Doppler shift value.

# Satellite orbit determination and prediction

The aim of this section is to provide some insights on how and with which accuracy satellite position and velocity can be determined and predicted.

Satellite orbit determination

The satellite orbit determination (OD) consists in estimating the most likely track or orbit which have been taken by a satellite based on past and noisy measurements of its position and velocity. In general, the satellite orbit determination is performed as follows:

Step 1: Measurements of the satellite position and velocity are made and dated. Most of the time, these measurements are GNSS-based measurements performed on-board. The accuracy of GNSS measurements depends on several factors such as:

- The GNSS receiver characteristics (mono-frequency vs bi-frequency, single vs multi-constellation, quality of the GNSS receiver)

- The satellite orbit, and the number of GNSS satellites in view

Step 2: The measurements are collected in the NTN Control Center (NCC) where the satellite OD will be performed. In general, the satellite GNSS measurements are delivered to the ground via dedicated telemetry channels between the satellite and the NTN GW network. The reporting period of new GNSS measurements is implementation specific but it is directly related to the accuracy target associated to the instantaneous knowledge of the satellite position and velocity in the system.

Step 3: The satellite OD is performed in the NCC. This operation can be more or less complex depending on the models considered, the quantity of measurements available and the algorithms used. Traditionally, two filtering techniques are used on input measurements: Kalman filtering or least square filtering. Least square filtering usually provides more accurate results but requires more computing power.

An overview of a typical satellite system architecture is given on 4 where the locations of each OD step are explicitly represented.

The satellite OD is useful for several reasons. It helps estimating what was the satellite past trajectory so one can identify whether the satellite is diverging from a reference orbit and if needed schedule satellite station keeping manoeuvres to get it back on track.

Figure 9 : System Architecture Overview

Hereafter, we provide several examples of orders of magnitude of precision which can be expected for orbit determination. This performance shall not be confused with orbit prediction performances which concern the future satellite trajectory and are discussed in the next section.

One shall distinguish between orbit determination performance based on past measurements of the satellite trajectory and orbit prediction performance which concern the future satellite trajectory.

Case 1 : “Best” Precision Orbit Determination

For satellite systems designed for navigation purposes delivering very accurate and abundant GNSS measurements, it is possible to achieve the following performance:

- 3D Position RMS Error = 0.05 m

- 3D Velocity RMS Error = 0.05 mm/s

However, in commercial telecommunication satellites such performance cannot be met because the cost would be prohibitive and such OD precision is not required for the system to work.

Case 2 : “Typical” Precision Orbit Determination

For telecommunication satellites, it is more typical to consider the following values:

- 3D Position RMS Error = 0.5 m

- 3D Velocity RMS Error = 0.5 mm/s

Case 3 : “Low Quality” Precision Orbit Determination

Finally, for low cost system the following performance can be considered:

- 3D Position RMS Error = 5 m

- 3D Velocity RMS Error =5 mm/s

Note: As a rule of thumb; it can be assumed that there is a factor of 1000 between the position error (in m) and the velocity error (in m/s). This is important to keep in mind when defining accuracy targets for satellite position and velocity.

As a rule of thumb; it can be assumed that there is a factor of 1000 between the position error (in m) and the velocity error (in m/s). This is important to keep in mind when allocating an error budget for satellite position and velocity estimations.

Satellite orbit prediction

The satellite orbit determination can also be used to derive updated satellite ephemeris. The satellite ephemeris are useful to predict future satellite positions and velocity vectors. This can be useful for various reasons such as:

- Determine when the satellite is going to fly by over a specific zone of interest

- Anticipate collision avoidance operations

- Anticipate future feeder link switch over operations within the NTN gateways network.

- Perform Doppler compensation (for instance on the feeder links between the satellite and the serving NTN GW)

Of course, the accuracy of the orbit prediction required to perform such operations can be significant different from an application to another. Finally, the NCC can share the satellite ephemeris (under various formats) with other entities in the system responsible for performing such operations (e.g. with the NTN GW).

Then, one can wonder with which accuracy one can predict the future satellite position and velocity vectors and over what time horizon.

The orbit prediction accuracy may depend on several factors:

- The accuracy of the orbit determination used to derive the satellite ephemeris: this will drive the initial errors of the propagation model.

- The accuracy of the orbit propagation model: there are many propagation models which takes into account more or less precisely the physical effects on the satellite movement (J2/4 perturbations, aerodynamical drag, anticipated stationkeeping manoeuvres)

- The time horizon over which the prediction is made: The initial errors combined with the propagation model errors tend to increase the prediction errors when the prediction period increases.

The orbit prediction accuracy depends on:

  - -	The accuracy of the orbit determination used to derive the satellite ephemeris

  - -	The accuracy of the orbit propagation model

  - -	The time horizon over which the prediction is made

## Orbit prediction error at NTN Control Center

In this study, we tried to quantify the propagation error evolution depending on the initial PV errors. The main assumptions and results of this study are summarized in Table 3.

A simplified relative position-velocity model for circular orbits (Clohessy-Wiltshire) has been considered. The CW equations are given in Figure 10 where the z-axis is along the radius vector of the target spacecraft, the y-axis is along the angular momentum vector of the target spacecraft, and the x-axis completes the right handed system.	These equations are usually used for rendez-vous or docking relative position evaluations.

However, they are used here to evaluate the propagation error evolution between the reference orbit and the predicted orbit which progressively moves away from the reference orbit due to propagation errors.

Figure 10: Clohessy-Wiltshire equations of motion

This study characterizes only the impact of the initial PV error on the prediction errors. The additional errors related to the orbit propagation model limits are not simulated here. However, when considering short prediction period (i.e. inferior to the orbital period), the propagation errors induced by the propagator models limitations (J2/J4 perturbation, air drag, …) may be considered negligible w.r.t. to the prediction errors induced by the initial PV errors.

Effect of initial errors on tangential (X), normal (Y), radial (Z) axes in position and velocity gives very different results. The worst case effects are observed when the initial error is applied on Z axis for position and X axis for velocity.

For illustrative purpose, the propagation errors over 2 revolution orbits for each worst case initial errors on position (dz0 = 1 m) and on velocity (dvx0 = 1mm/s) have been illustrated on Figure 11 and Figure 12 respectively. It can be observed that the velocity error is not an increasing function of time.

Figure 11 Propagation errors – dZ0 = 1 m

Figure 12 Propagation errors – dvX0 = 1 mm/s

For initial PV errors, the POD assumptions presented in the previous section have been considered.

Finally, one shall define a PV accuracy target to be guaranteed. When the propagation errors exceed the PV accuracy target then it means that a new orbit determination shall be performed and updated ephemeris shall be provided. The PV accuracy target is going to be overshoot after a more or less long prediction period depending on the initial PV error assumption.

In this study, the target PV accuracy has been driven by the accuracy expected to perform Doppler pre/post-compensation on feeder links. The Doppler compensation residual errors experienced on the feeder link shall be almost transparent to the gNB/UE. Then, it shall be an order of magnitude lower than the frequency error budget allocated at UE side (~0.1 ppm). Moreover, the carrier frequencies used on the feeder links may be significantly higher than the one used on the service link so additional margin shall be considered to counter shift this effect. As a consequence, it is proposed to assume that the satellite radial velocity w.r.t the NTN GW should be known with an accuracy better than +/- 0.3 m/s when performing Doppler compensation on the feeder link. It has been demonstrated by simulation (see Figure 13 and Table 4) that when the following assumptions on the PV errors are met the previous requirement is guaranteed :

- Gaussian position error distribution with 3D Position RMS Error = 10 m => 3sigma = 30 m

- Gaussian velocity error distribution with 3D Velocity RMS Error = 10 mm/s => 3sigma = 30 mm/s

Therefore, it is proposed to consider the following PV accuracy target:

- Position error < 30 m

- Velocity error < 30 mm/s

The simulation results are summarized in Table 4. It can be observed that the maximal propagation time before overshooting the PV accuracy target is significantly driven by the initial error. More precisely, depending on the quality of the orbit determination, the maximal propagation time can go from 8 hours to less than 10 minutes. The less accurate is the orbit determination, the more often orbit determination shall be performed based on more recent measurements and updated satellite ephemeris shall be distributed.

It can be also observed that the velocity error degradation is negligible w.r.t the position error. As a consequence, the radial velocity (or Doppler) error is mainly driven by the position error degradation.

In conclusion, it has been demonstrated that even for a satellite system with “low quality” orbit determination Algorithm, challenging operations relying on accurate prediction of satellite trajectories such as Doppler compensation can be performed reliably. The limiting condition is that the system should be designed such that new orbit determination and satellite ephemeris update can be performed frequently enough.

Even for a satellite system with “low quality” orbit determination algorithm, challenging operations relying on accurate prediction of satellite trajectories such as Doppler compensation can be performed reliably.

Figure 13 Monte Carlo simulation to evaluate radial velocity error [m/s] distribution assuming a PV Gaussian errors with (3D Position RMS Error = 10 m / 3D Velocity RMS Error = 10 mm/s)

Table 3 : Monte Carlo simulation parameters

| Satellite orbit | Polar orbit (i = 90°) |
| --- | --- |
| GW position | Equatorial ground station Min elevation = 30° |
| Error on position/velocity | Gaussian distribution on norm - 3D Position RMS Error = 10 m - 3D Velocity RMS Error = 10 mm/s Uniform distribution on direction |

Table 4 : Results of propagation error study

| Model | Kepler Model Clohessy-Wiltshire equations |
| --- | --- |
| Orbit | Circular @ 600 km Period = 5801 s |
| Initial PV Errors | Case 1 : “Best” Precision Orbit Determination - 3D Position RMS Error = 0.05 m - 3D Velocity RMS Error = 0.05 mm/s Case 2 : “Typical” Precision Orbit Determination - 3D Position RMS Error = 0.5 m - 3D Velocity RMS Error = 0.5 mm/s Case 3 : “Low Quality” Precision Orbit Determination - 3D Position RMS Error = 5 m - 3D Velocity RMS Error =5 mm/s |
| PV accuracy target | Position error < 30 m Velocity error < 30 mm/s |
| Prediction period before overshooting PV accuracy target | Case 1 : “Best” Precision Orbit Determination Propagation to 5 orbital periods before overshooting PV accuracy target ~ 8 h - Statistics on max position error - Mean = 4.35 m - Std dev = 4.10 m - Max = 35.9 m - Statistics on max velocity error - Mean = 0.3 mm/s - Std dev = 0.3 mm/s - Max = 2.6 mm/s Case 2 : “Typical” Precision Orbit Determination Propagation to 0.5 period before overshooting PV accuracy target ~ 48 mn - Statistics on max position error - Mean = 4.63 m - Std dev = 4.23 m - Max = 36.7 m - Statistics on max velocity error - Mean = 0.3 mm/s - Std dev = 0.3 mm/s - Max = 2.6 mm/s Case 3 : “Low Quality” Precision Orbit Determination Propagation to 0.1 period before overshooting PV accuracy target ~ 10 mn - Statistics on max position error - Mean = 5.65 m - Std dev = 3.61 m - Max = 28.9 m - Statistics on max velocity error - Mean = 7.5 mm/s - Std dev = 5.1 mm/s - Max = 46 mm/s |

## Satellite orbit format

Serving satellite ephemeris broadcast to UEs is required to enable autonomous adjustments of UL TA and frequency at UE side based on the satellite predicted position and velocity vectors. The satellite ephemeris format to be used was discussed in RAN1#104e. The following agreement was made:

- RAN1 to support satellite ephemeris broadcast based at least on one of the following format options:

  - Option 1: Ephemeris format based on satellite position and velocity state vectors

    - FFS: Details on state vectors formats

    - FFS: Details on time reference provisioning/format

  - Option 2: Ephemeris format based on orbital elements

    - FFS: Details on orbital elements formats

    - FFS: Details on time reference provisioning/format

- FFS: Whether down-selection is needed or both options are supported

The required accuracy for the satellite ephemeris format shall be driven based on the prediction budget error on satellite position and velocity vectors and the foreseen  prediction time horizon. It can be further discussed once the timing and frequency synchronization requirements are agreed.

The signalling overhead introduced by satellite ephemeris broadcast shall be carefully quantified.

There are several ways to provide a time reference or Epoch time to the satellite ephemeris broadcast. The first approach is to transmit this time explicitly. It is the simplest approach but it increases signalling overhead. It can also be done in an implicit way to limit the signalling overhead. For instance, the date can be considered equal to the reception time of the frame including the satellite ephemeris information.

To reduce the signalling overhead, implicit reference time provisioning can be considered for both formats. Further, in case of ephemeris format based on satellite position and velocity state vectors, separate formats for GEO orbits, LEO orbits and HAPS/ATG may be defined. And in case of ephemeris format based on orbital elements, pre-provisioned ephemeris based on orbital elements can be used as reference. Only delta corrections can be broadcast in order to reduce the overhead..

To reduce the signalling overhead in case of ephemeris format based on satellite position and velocity state vectors, separate formats for GEO orbits, LEO orbits and HAPS/ATG may be defined.

To reduce the signalling overhead in case of ephemeris format based on orbital elements, pre-provisioned ephemeris based on orbital elements can be used as reference. Only delta corrections can be broadcast in order to reduce the overhead

Proposal 11:

RAN1 to discuss whether:

- Option 1: Epoch time is implicitly known as a reference time linked to DL slot where satellite ephemeris is broadcast

- Option 2: Epoch time is explicitly indicated within the ephemeris data

The following table shows the payload size in the SIB needed for satellite ephemeris to allow required accuracy for UE pre-compensation of delay and Doppler in case of  a non-Geostationary Satellites:

Table 5 Payload size in the SIB needed for satellite ephemeris

|  | Max | Resolution | Nbits |
| --- | --- | --- | --- |
| PV in cartesian format |  |  |  |
| position X,Y,Z absolute in ECEF (m) | 8.0E+06 | 5.0E-01 | 25 |
| velocity VX,VY,VZ absolute in ECEF (m/s) | 8.0E+03 | 5.0E-04 | 25 |
| Payload (bits) |  |  | 150 |
| Payload (bytes) |  |  | 19 |
| Orbital parameters in adapted format |  |  |  |
| Da relative in J2000 (m) | 1.00E+04 | 5.00E-02 | 19 |
| ex (absolute in J2000) | 1.00E-02 | 5.00E-08 | 19 |
| ey (absolute in J2000) | 1.00E-02 | 5.00E-08 | 19 |
| Di (relative in J2000) (rad) | 8.73E-03 | 5.00E-08 | 19 |
| RAAN (absolute in J2000) (rad) | 3.14E+00 | 5.00E-08 | 27 |
| AOL (absolute in J2000) (rad) | 3.14E+00 | 5.00E-08 | 27 |
| Payload (bits) |  |  | 130 |
| Payload (bytes) |  |  | 17 |

Proposal 12:

RAN1 to support satellite ephemeris format:

based on satellite position and velocity state vectors:

- position X,Y,Z absolute in ECEF (m)  in a field of 75 bits

- velocity VX,VY,VZ absolute in ECEF (m/s) in a field of 75 bits

based on orbital elements:

- Da relative in J2000 (m) in a field of 19 bits

- ex (absolute in J2000) in a field of 19 bits

- ey (absolute in J2000) in a field of 19 bits

- Di (relative in J2000) (rad) in a field of 19 bits

- RAAN (absolute in J2000) (rad) in a field of 27 bits

- AOL (absolute in J2000) (rad) in a field of 27 bits

## Orbit prediction error at UE

Depending on adopted ephemeris format; Option 1 or Option 2, different propagator models can be considered for implementation at UE side. Some have been developed for orbital elements and others for PV state vectors. To enhance the propagator model additional elements may be included in the ephemeris data.

The propagator model to be used at UE side can be left to implementation. Yet, the adopted ephemeris format can strongly orientate the choice of model (e.g. TLE format and TLE propagation models).

It was observed based on simulation that the prediction time period before overshooting the delay and Doppler pre-compensation target is linked to the broadcast satellite format and to the propagation model used in the UE. As shown in the table below  prediction 45 s ahead for UE pre-compensation is accurate within 0.29 µs for round trip delay error and within 45.93 Hz for Doppler error (at FR2) when Kepler model is used. This is sufficient to accommodate UE pre-compensation requirement when using 120 kHz SCS. Prediction 180 s ahead for UE pre-compensation is accurate within 1.74 µs for round trip delay error and within 17.60 Hz (in S band) for Doppler error when Kepler model is in use. This is sufficient to accommodate UE pre-compensation requirement when using 15 kHz SCS.

Using other propagation models in the UE will significantly enhance UE pre-compensation accuracy during a larger prediction time period. For example, by using analytical Eckstein-Hechler model, prediction 5 minutes ahead for UE pre-compensation is accurate within 0.29 µs for round trip delay error and within 6.20 Hz (in FR2) for Doppler error.

It is worth noting that adequate fitting can be performed based on the reference orbit and the prediction time horizon considered to further enhance UE pre-compensation accuracy. Of course, this is possible only if the propagator model to be used at UE side is known by the network. Thus, an agreement on an universal model to be used can be beneficial.

Propagator model at UE side can be left to implementation. But, the adopted ephemeris format can strongly orientate the choice of model. Further, an agreement on an universal model can be beneficial. This is useful to perform suitable fitting at network side to enhance orbit prediction by the NR NTN UEs.

Table 6 Orbit propagation model comparison

|  |  |  | Maximum errors |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Without fitting | Model | Duration of prediction (minute) | DP (m) | UE specific TA Error (µs) | DV (m/s) | Doppler error (Hz) |
|  | Kepler | 3 | 261 | 1.74 | 2.64 | 17.60 |
|  | Eckstein Hechler (J6) | 21 | 303 | 2.02 | 0.48 | 3.20 |
|  | 2x2 num | 22,5 | 301.4 | 2.01 | 0.42 | 2.80 |
|  | 6x6 num | 27 | 302.5 | 2.02 | 0.44 | 2.93 |
|  |  |  | Maximum errors |  |  |  |
|  | Model | Duration of prediction (second) | DP (m) | UE specific TA Error (µs) | DV (m/s) | Doppler error (Hz) |
| Without fitting | Kepler | 45 | 43.4 | 0.29 | 0.689 | 45.93 |
|  | Eckstein Hechler (J6) | 300 | 43.4 | 0.29 | 0.093 | 6.20 |
|  | 2x2 num | 300 | 44.3 | 0.30 | 0.1 | 6.67 |
|  | 6x6 num | 350 | 44.2 | 0.29 | 0.084 | 5.60 |
| With fitting | TLE | 370 | 44,1 | 0.29 | 0.1 | 6.67 |
|  | Eckstein Hechler (J6) | 390 | 44,2 | 0.29 | 0.083 | 5.53 |
|  | 2x2 num | 375 | 44 | 0.29 | 0.092 | 6.13 |
|  | 6x6 num | 395 | 44,2 | 0.29 | 0.079 | 5.27 |

Proposal 13:

RAN1 to support one of the following options regarding the NR NTN UE propagator model:

- Option 1: a specified propagation model is to be implemented at UE side

- Option 2: the propagator model to be used at UE side can be left to implementation

# Conclusion

In this contribution, the following observations and proposal have been made:

Proposal 1:

The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by:

When common TA is indicated by the Network:

TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc

When common TA is not indicated by the Network:

TTA=NTA+NTA,UE-specific+NTA,offset×Tc

Where:

  NTA and NTA,offset are defined as in Release-16.

  NTA,UE-specific  is UE self-estimated TA

  NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network.

Proposal 2:

Introduce a flag in the SIB to indicate whether the Common TA is present and need to be applied for TTA calculation

Proposal 3:

If Common TA shall be considered in UE autonomous TA calculation, the Network shall periodically broadcast:

- in case of GEO based non-terrestrial access network:

  - Common TA (NTA,common ) in a field of 23 bits

- in case of LEO/MEO based non-terrestrial access network:

  - Common TA (NTA,common ) in a field of 19 bits

  - Common TA drift rate( NCommondrift) in a field of 13 bits

  - FFS: Common TA drift rate variation (NCommondrift_variation_rate ) in a field of 6 bits

Proposal 4: RAN1 to discuss the indication of TA-margin after RAN4 response to the LS on timing synchronization requirements

Proposal 5:

Existing NTA update in RRC_CONNECTED based on TA Command  field in msg2/msgB and MAC CE TA command is used for UL timing alignment correction without specification change

- FFS: Whether to support negative TA Command in RAR

Proposal 6:

NTN UE in RRC_CONNECTED shall have an active BWP with common search space configured to monitor system information.

Proposal 7:

A validity timer configured for UE specific TA defines the maximum time during which the UE can track the RTD on service link without having acquired new ephemeris data to be used for UE specific TA estimation.

- This timer is restarted each time the UE receives new ephemeris data

- The UE assumes that it has lost uplink synchronization if this timer expires and new ephemeris data is not available.

Proposal 8:

A validity timer configured for Common TA defines the maximum time during which the UE can track the Common RTD without having acquired new assistance information to be used for Common TA estimation.

- This timer is restarted each time the UE receives new assistance information.

- The UE assumes that it has lost uplink synchronization if this timer expires and assistance information is not available.

Proposal 9:

RAN1 working assumption is that GW compensates common Doppler shift / Doppler shift variation over the feeder link  and any transponder frequency error  in a transparent way to the UE and gNB.

Proposal 10:

If DL frequency compensation for the service link Doppler is applied, indication of the amount of frequency compensation is necessary:

- In case of earth-fixed cell, the beam-specific ECEF co-ordinates of a fixed Reference Point w.r.t the common Doppler shift experienced on the DL service link is pre-compensated by the gNB.

- In case of earth-moving beam, the beam-specific common Doppler shift value.

Proposal 11:

RAN1 to discuss whether:

- Option 1: Epoch time is implicitly known as a reference time linked to DL slot where satellite ephemeris is broadcast

- Option 2: Epoch time is explicitly indicated within the ephemeris data

Proposal 12:

RAN1 to support satellite ephemeris format:

based on satellite position and velocity state vectors:

- position X,Y,Z absolute in ECEF (m)  in a field of 75 bits

- velocity VX,VY,VZ absolute in ECEF (m/s) in a field of 75 bits

based on orbital elements:

- Da relative in J2000 (m) in a field of 19 bits

- ex (absolute in J2000) in a field of 19 bits

- ey (absolute in J2000) in a field of 19 bits

- Di (relative in J2000) (rad) in a field of 19 bits

- RAAN (absolute in J2000) (rad) in a field of 27 bits

- AOL (absolute in J2000) (rad) in a field of 27 bits

Proposal 13:

RAN1 to support one of the following options regarding the NR NTN UE propagator model:

- Option 1: a specified propagation model is to be implemented at UE side

- Option 2: the propagator model to be used at UE side can be left to implementation

Observation 1.	For better accuracy of self-estimated Common TA a second-order approximation may be needed, thereby, both Common TA drift rate and Common TA drift variation rate may need to be indicated.

Observation 2.	By considering the most restrictive case (i.e. UE in Connected state and 120kHz SCS) the UE can self-estimate the Common TA with sufficient accuracy provided that an updated related assistance information is available at the UE during the last 0.7 s when Common TA drift rate is indicated or during the last 5s when both Common TA drift rate  and Common drift variation rate are indicated.

Observation 3.	A validity timer configured for Common TA defines the maximum time during which the UE can track the Common RTD without having to perform new assistance information acquisition to be used for Common TA self -estimation.

Observation 4.	The values of CTA_validity_time_max given in Table 1 need to be revisited to take into account the maximum error on the Common RTD estimated by the gNB (before being broadcast) and the error of  quantization of assistance information.

Observation 5.	To assist the UE in self-estimating the Common TA, without indicating Common TA drift variation rate, the network may indicate an averaged Common TA Drift rate (instead of  Instantaneous Common TA Drift rate).

Observation 6.	if initial TA does not include a margin for maximum TA estimation error. A bipolar TA command is needed in msg2. i.e support negative TA adjusting in RAR

Observation 7.	One shall distinguish between orbit determination performance based on past measurements of the satellite trajectory and orbit prediction performance which concern the future satellite trajectory.

Observation 8.	As a rule of thumb; it can be assumed that there is a factor of 1000 between the position error (in m) and the velocity error (in m/s). This is important to keep in mind when allocating an error budget for satellite position and velocity estimations.

Observation 9.	The orbit prediction accuracy depends on:

- The accuracy of the orbit determination used to derive the satellite ephemeris

- The accuracy of the orbit propagation model

- The time horizon over which the prediction is made

Observation 10.	Even for a satellite system with “low quality” orbit determination algorithm, challenging operations relying on accurate prediction of satellite trajectories such as Doppler compensation can be performed reliably.

Observation 11.	To reduce the signalling overhead in case of ephemeris format based on satellite position and velocity state vectors, separate formats for GEO orbits, LEO orbits and HAPS/ATG may be defined.

Observation 12.	To reduce the signalling overhead in case of ephemeris format based on orbital elements, pre-provisioned ephemeris based on orbital elements can be used as reference. Only delta corrections can be broadcast in order to reduce the overhead

Observation 13.	Propagator model at UE side can be left to implementation. But, the adopted ephemeris format can strongly orientate the choice of model. Further, an agreement on an universal model can be beneficial. This is useful to perform suitable fitting at network side to enhance orbit prediction by the NR NTN UEs.

# Bibliography

| [1] | RAN1#104e, Thales, FL summary for UL synchronization in R1-2102215, February 2021. |
| --- | --- |
| [2] | "TR38.821 Solutions for NR to support non-terrestrial networks". |
| [3] | "TR38.213 Physical layer procedures for control". |
| [4] | Thales, "RP-201256 Solutions for NR to support non-terresttrial networks (NTN)," RAN#88e, June 2020. |
| [5] | Thales, "R1-2005306 NR_NTN_solutions work plan," RAN1#102-e. |