3GPP TSG RAN WG1 #104b-e		R1-2102377

e-Meeting, April 12th – 20th, 2021

Source:	OPPO

Title:	Text Proposals for Tx Switching between Two Uplink Carriers

Agenda Item:	7.2.12

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss an issue for the current specification of UL Tx switching between two uplink carriers and present the corresponding text proposal to address.

Correction of RRC parameter “uplinkTxSwitchRequest”

| Reason for change: | The RRC parameter “uplinkTxSwitchRequest” is used to request filtered UE capabilities. However, the current version of TS 38.214 misused it for UE to report the UE capablity. Thus, it should be chagned to “BandCombination-UplinkTxSwitch”, which is used in Section 6.1.6.1/6.1.6.2/6.1.6.3 |
| --- | --- |
| Summary of change: | Change the RRC parameter “uplinkTxSwitchRequest” to “BandCombination-UplinkTxSwitch” |
| Consequences if not approved: | RRC parameter is incorrect. |
| Clauses affected: | 6.1.6 |

The corresponding part in TS 38.311 is copied as below

- – UE-CapabilityRequestFilterCommon The IE UE-CapabilityRequestFilterCommon is used to request filtered UE capabilities. The filter is common for all capability containers that are requested. UE-CapabilityRequestFilterCommon information element -- ASN1START -- TAG-UE-CAPABILITYREQUESTFILTERCOMMON-START UE-CapabilityRequestFilterCommon ::= SEQUENCE { mrdc-Request SEQUENCE { omitEN-DC ENUMERATED {true} OPTIONAL, -- Need N includeNR-DC ENUMERATED {true} OPTIONAL, -- Need N includeNE-DC ENUMERATED {true} OPTIONAL -- Need N } OPTIONAL, -- Need N ..., [[ codebookTypeRequest-r16 SEQUENCE { type1-SinglePanel-r16 ENUMERATED {true} OPTIONAL, -- Need N type1-MultiPanel-r16 ENUMERATED {true} OPTIONAL, -- Need N type2-r16 ENUMERATED {true} OPTIONAL, -- Need N type2-PortSelection-r16 ENUMERATED {true} OPTIONAL -- Need N } OPTIONAL, -- Need N uplinkTxSwitchRequest-r16 ENUMERATED {true} OPTIONAL -- Need N ]] } -- TAG-UE-CAPABILITYREQUESTFILTERCOMMON-STOP -- ASN1STOP

Moreover, the parameter “uplinkTxSwitchRequest”” is used in Section 6.1.6 whereas the RRC parameter “BandCombination-UplinkTxSwitch” is used in the same sentence in Section 6.1.6.1/6.1.6.2/6.1.6.3. The corresponding part in TS 38.214 is copied as below

- 6.1.6 Uplink switching The UE may omit uplink transmission during the uplink switching gap NTx1-Tx2 if the conditions defined in this clause are met and the UE is configured with uplinkTxSwitching. The switching gap NTx1-Tx2 is indicated by UE capability uplinkTxSwitchingPeriod: - If a UE indicated a capability for uplink switching with uplinkTxSwitchRequest for a band combination, and if it is for that band combination - Configured with a MCG using E-UTRA radio access and with a SCG using NR radio access (EN-DC), or - Configured with uplink carrier aggregation, or - Configured in a serving cell with two uplink carriers with higher layer parameter supplementaryUplink. the conditions under which the switching gap may be present and the location of the switching gap are defined for each of the cases in clauses 6.1.6.1, 6.1.6.2, and 6.1.6.3 respectively. <omitted text> - 6.1.6.1 Uplink switching for EN-DC For a UE indicating a capability for uplink switching with BandCombination-UplinkTxSwitch for a band combination, and if it is for that band combination configured with a MCG using E-UTRA radio access and with a SCG using NR radio access (EN-DC), if the UE is configured with uplink switching with parameter uplinkTxSwitching, - for the UE configured with switchedUL by the parameter uplinkTxSwitchingOption, when the UE is to transmit in the uplink based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s): <omitted text> - 6.1.6.2 Uplink switching for carrier aggregation For a UE indicating a capability for uplink switching with BandCombination-UplinkTxSwitch for a band combination, and if it is for that band combination configured with uplink carrier aggregation: - If the UE is configured with uplink switching with parameter uplinkTxSwitching, when the UE is to transmit in the uplink based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s): <omitted text> - 6.1.6.3 Uplink switching for supplementary uplink For a UE indicating a capability for uplink switching with BandCombination-UplinkTxSwitch for a band combination, and if it is for that band combination configured in a serving cell with two uplink carriers with higher layer parameter supplementaryUplink: - If the UE is configured with uplink switching with parameter uplinkTxSwitching, <omitted text>

Based on the above discussion, we propose the following text proposal to correct the RRC parameter in TS 38.214:

In TS 38.214 Section 6.1.6 - 6.1.6 Uplink switching The UE may omit uplink transmission during the uplink switching gap NTx1-Tx2 if the conditions defined in this clause are met and the UE is configured with uplinkTxSwitching. The switching gap NTx1-Tx2 is indicated by UE capability uplinkTxSwitchingPeriod: - If a UE indicated a capability for uplink switching with uplinkTxSwitchRequest BandCombination-UplinkTxSwitch for a band combination, and if it is for that band combination - Configured with a MCG using E-UTRA radio access and with a SCG using NR radio access (EN-DC), or - Configured with uplink carrier aggregation, or - Configured in a serving cell with two uplink carriers with higher layer parameter supplementaryUplink. the conditions under which the switching gap may be present and the location of the switching gap are defined for each of the cases in clauses 6.1.6.1, 6.1.6.2, and 6.1.6.3 respectively. If an uplink switching is triggered for an uplink transmission starting at T0, after T0-Toffset, the UE is not expected to cancel the uplink switching, or to trigger any other new uplink switching occurring before T0 for any other uplink transmission that is scheduled after T0-Toffset, where Toffset is the UE processing procedure time defined for the uplink transmission triggering the switch given in clause 5.3, clause 5.4, clause 6.2.1, clause 6.4 and in clause 9 of [6, TS 38.213]. The UE does not expect to perform more than one uplink switching in a slot with µUL = max(µUL, 1, µUL, 2), where the µUL, 1 corresponds to the subcarrier spacing of the active UL BWP of one uplink carrier before the switching gap and the µUL, 2 corresponds to the subcarrier spacing of the active UL BWP of the other uplink carrier after the switching gap. - 6.1.6.1 Uplink switching for EN-DC For a UE indicating a capability for uplink switching with BandCombination-UplinkTxSwitch for a band combination, and if it is for that band combination configured with a MCG using E-UTRA radio access and with a SCG using NR radio access (EN-DC), if the UE is configured with uplink switching with parameter uplinkTxSwitching, <omitted text>

Conclusion

In this contribution, we discuss one remaining issue on the current specification of UL Tx switching between two uplink carriers and also propose the corresponding text proposal to fix it.

References

- TS 38.214, NR; Physical layer procedures for data

- TS 36.331, E-UTRA; Radio Resource Control (RRC) protocol specification