3GPP TSG RAN WG1 #104b-e		R1-2102420

e-Meeting, April 12th – 20th, 2021

Source:	OPPO

Title:	Initial performance results for XR evaluation

Agenda Item:	8.14.3

Document for:	Discussion and Decision

- Introduction

A study item of XR and could game (CG) evaluations for NR was approved in RAN#88e with the following objectives [1]:

- Confirm XR and Cloud Gaming applications of interest - Identify the traffic model for each application of interest taking outcome of SA WG4 work as input, including considering different upper layer assumptions, e.g. rendering latency, codec compression capability etc. - Identify evaluation methodology to assess XR and CG performance along with identification of KPIs of interest for relevant deployment scenarios - Once traffic model and evaluation methodologies are agreed, carry out performance evaluations towards characterization of identified KPIs

To be specific, there are four typical applications suggested as the starting points for this study [1]:

- VR1: “Viewport dependent streaming”

- VR2: “Split Rendering: Viewport rendering with Time Warp in device”

- AR1: “XR Distributed Computing”

- AR2: “XR Conversational”

- CG: Cloud Gaming

In last two meeting, there were some discussions on the traffic models and evaluation methodology. Some relevant agreements for traffic model and simulation assumption were made in [2][3].

In this contribution, we present some initial evaluation results of the system capacity based on the agreed traffic models and simulation assumptions.

- System capacity

In this section, we will show some initial evaluation results of the system capacity in dense urban scenarios. Tge details of simulation parameters are summarized in the table below.

| Parameters | Value |
| --- | --- |
| Scenario | Dense Urban (Uma) hexagonal layout with (7, 3) Sectors |
| Inter-BS distance | 200m |
| Carrier frequency | 4GHz |
| Duplex Mode / Simulation bandwidth | 100MHz |
| SCS | 30KHz |
| TDD pattern | DDDSU |
| BS Antenna Configuration | 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1,8,2) (dH, dV) = (0.5λ, 0.85λ) |
| UE Antenna Configuration | 4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ |
| Transmit Power | 51dBm |
| Antenna Height | 25 m for BS and 1.5 m for UE |
| Receiver Noise Figure | 5 dB for BS and 9 dB for UE |
| UE distribution | 80% of users are indoor,20%of users are outdoor; |
| Number of UEs per cell | 10 |
| UE speed | 3km/h |
| Scheduling Algorithm | SU-MIMO+PF |
| Channel estimation | Realist |
| BS receiver | MMSE-IRC |

In the evaluation, a DL video stream with 60Mbps and 60 frame per second (fps) is used. The packet size is generated according to the truncated Gaussian distribution with both upper and lower bounds. The corresponding parameters of DL video stream are shown in the table below.

|  | Mean | STD | Max | Min | Arrival interval |
| --- | --- | --- | --- | --- | --- |
| DL video stream | 125000Bytes | Mean × 15% 18750Bytes | Mean × 1.5 187500Byte | 75Bytes | 16.67ms |

The packet arrival jitter is also generated by the truncated Gaussian distribution. The corresponding parameters are shown in table below:

|  | Mean | STD | Range |
| --- | --- | --- | --- |
| Packet arrival jitter | 0 | 2ms | [-4, 4] ms |

The simulation results of system capacity and resource utilization for NR supporting XR/CG DL video stream are shown in Fig.1 and Fig.2, showing the ratio of satisfied UE and PRB utilization, respectively.

Fig.1: Ratio of the satisfied UE with the PDB of 10ms

Fig.2: DL PRB utilization

As shown in Fig.1 and Fig.2, we can observe that :

- The percentage of satisfied UEs will decrease as the number of UEs per cell increases

- The DL PRB utilization will increase as the number of UEs per cell increases.

According to the simulation results shown in Fig.1, we can make the following observations:

Observation 1: In FR1, for XR/VR DL video stream with 60Mbps @60 fps and PDB=10ms in dense urban scenarios

  - If 90% of UEs are satisfied (baseline), the capacity is about 3 user pers cell

  - If 95% of UEs are satisfied (optional), the capacity is 2 user pers cell

- Conclusions

In this contribution, we provide some initial simulation results for XR/VR service in dense urban scenarios and have the following observation:

Observation 1: In FR1, for XR/VR DL video stream with 60Mbps @60 fps and PDB=10ms in dense urban scenarios

  - If 90% of UEs are satisfied (baseline), the capacity is about 3 user pers cell

  - If 95% of UEs are satisfied (optional), the capacity is 2 user pers cell

- Reference

RP-201145 Study on XR Evaluations for NR

Chairman’s Notes of RAN1#103e

Chairman’s Notes of RAN1#104e