3GPP TSG RAN WG1 #104b-e		R1-2102514

e-Meeting, April 12th – 20th, 2021

Source:	vivo

Title:	Discussions on initial access aspects for NR operation from 52.6GHz to 71GHz

Agenda Item:	8.2.1

Document for:	Discussion and Decision

- Introduction

The following objective is included in this work item [1]:

- In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported.

- Support of up to 64 SSB beams for licensed and unlicensed operation in this frequency range.

- Supports 120kHz SCS for SSB and 120kHz SCS for initial access related signals/channels in an initial BWP.

- Study and specify, if needed, additional SCS (240kHz, 480kHz, 960kHz) for SSB, and additional SCS (480kHz, 960kHz) for initial access related signals/channels in initial BWP.

- Study and specify, if needed, additional SCS (480kHz, 960kHz) for SSB for cases other than initial access.

- Note: coverage enhancement for SSB is not pursued.

- Specify timing associated with beam-based operation to new SCS (i.e., 480kHz and/or 960kHz), study, and specify if needed, potential enhancement for shared spectrum operation

- Study which beam management will be used as a basis: R15/16 or R17 in RAN #91-e

- Specify support for PRACH sequence lengths (i.e., L=139, L=571 and L=1151) and study, if needed, specify support for RO configuration for non-consecutive RACH occasions (RO) in time domain for operation in shared spectrum.

In this contribution, we first discuss the SSB and initial DL BWP related aspects which include SSB numerology selection, SSB design in time design and SSB multiplexing pattern. Then the PRACH and initial UL BWP issues which include PRACH numerology selection, PRACH sequence design, RO configuration and RA-RNTI will be studied. Finally, we discuss the SSB and PRACH design other than initial access.

- SSB and initial DL BWP

SSB and initial DL BWP numerology selection

In RAN1#104-e meeting, the numerology of SSB and initial BWP was discussed and the following options were agreed [2]:

Agreement:

Whether or not to support 240 kHz, 480kHz and 960kHz SCS for SSB and the conditions under which SSB for 240 kHz, 480 kHz and 960 kHz may be supported will be decided no later than RAN1#104bis-e.

Agreement:

For CORESET#0 and Type0-PDCCH search space configured in MIB:

- Support {SS/PBCH Block, CORESET#0 for Type0-PDCCH} SCS equal to {120, 120} kHz

  - Support at least SSB and CORESET#0 multiplexing patterns, number of RBs for CORESET#0, number of symbols (duration of CORESET#0) that are supported in Rel-15/16 for {SS/PBCH Block, CORESET#0 for Type0-PDCCH} SCS = {120, 120} kHz.

    - FFS: Supporting additional values

  - FFS: Supported values for SSB to CORESET#0 offset RBs

- If 480kHz SSB SCS that configures CORESET#0 and Type0-PDCCH CSS in MIB is agreed to be supported,

  - Support {SS/PBCH Block, CORESET#0 for Type0-PDCCH} SCS equal to {480, 480} kHz

- If 960 kHz SSB SCS that configures CORESET#0 and Type0-PDCCH CSS in MIB is agreed to be supported,

  - Support {SS/PBCH Block, CORESET#0 for Type0-PDCCH} SCS equal to {960, 960} kHz

- If 240 kHz SSB SCS is agreed to be supported,

  - Support {SS/PBCH Block, CORESET#0 for Type0-PDCCH} SCS equal to {240, 120} kHz

- FFS: any other combinations between one of SSB SCS (120, 240, 480, 960) and one of CORESET#0 SCS (120, 480, 960)

  - FFS: initial timing resolution based on low SCS (120 kHz) and its impact on the performance of higher SCS (480/960 kHz)

Before discussing the selection of SSB and initial DL BWP numerology in B52.6GHz, it is beneficial to study clear for that in FR2. In FR2, the supported SCS pairs for SSB and initial DL BWP including CORESET 0 are (120K, 60K), (120K, 120K), (240K, 60K) and (240K, 120K) [3]. Meanwhile, the applied numerology and configuration for different channels in FR2 is summarized in the following table:

Table 1	   List of numerologies for different channels in FR2 [4]

| Channels | FR2 Numerology | Configuration |
| --- | --- | --- |
| SSB/PBCH | 120K /240K | Per band definition / By ssbSubcarrierSpacing |
| Msg 1 PRACH | 60K/120K | By RACH-ConfigCommon / BeamFailureRecoveryConfig |
| Type 0/0A/1/2 PDCCH, PDSCH for (SIB1, Msg.2/4 for initial access, paging and broadcast SI-messages) | 60K /120K | By subCarrierSpacingCommon in MIB |
| PDCCH, PDSCH, CSI-RS within a DL BWP | 60K /120K | By BWP-DownlinkCommon |

It is clearly observed that all supported SCS for data/control in FR2 could be used for initial DL BWP.

Observation 1: All supported SCS for data/control SCS in FR2 could be used for initial DL BWP.

From the agreed WID [4], the SCS 120KHz for SSB is supported while one or more out of (240K, 480K, 960K) needs to be studied further. One straight forward method is to reuse the existing FR2 numerology pair for SSB and initial DL BWP:

- Alt. 1: (120K, 120K) + (240K, 120K)

However, this alternative shows some undesirable characteristic. Firstly, to support peak data rate use case, 960KHz BWP is preferred as operation BWP since it could have large bandwidth and good support of high modulation. If following Alt. 1, UE is mandatory to connect the cell via initial BWP first and switch to the preferred operation BWP which introduces more delay. Besides, mixed numerology operation (120KHz in initial BWP and 960KHz in operation BWP) is mandatory to both UE and gNB, which will bring more complexity and overhead to the implementation. To solve the above problem, supporting all SCS for data/control as SCS for initial DL BWP is needed as that in FR2. Therefore, the following proposal is made:

Proposal 1: Support SCS 120KHz, 480KHz and 960KHz for initial DL BWP in NR operation from 52.6-71GHz.

Then, with different supported SSB SCS, the following alternatives can be considered:

- Alt. 2: (120K, 120K) + (120/240K, 480K) + (120/240K, 960K)

- Alt. 3: (120K, 120K) + (480K, 480K) + (480K, 960K)

- Alt. 4: (120K, 120K) + (960K, 480K) + (960K, 960K)

For Alt. 2, we can find that the SCS of SSB is much smaller than initial DL BWP, where it brings extra overload to koffset which is the frequency domain offset between SSB and the overall resource block grid in number of subcarriers referring to the smaller SCS between SSB and initial DL BWP. In details, the koffset is indicated by ‘ssb-SubcarrierOffset’ with 4 bits in FR2, where the SCS of koffset is the same as that of initial DL BWP in FR2 since SCS of initial DL BWP is always smaller or equal to that of SSB. Therefore, if the SCS of SSB is smaller than initial DL BWP, the bit number of koffset needs to be increased. The smaller the SCS of SSB, the more bit number of koffset required. Besides, one of primary functions of SSB is to achieve time synchronization, which is based on the design of PSS and SSS sequence, and the corresponding SCS. If the SCS of SSB is smaller than that of other signals, it is unable to complete the required time synchronization accuracy due to the narrow SSB bandwidth allocation in the frequency domain.

For Alt. 3, the same issue is also existing as Alt. 2, i.e. (480K, 960K) SCS pair where SSB SCS is smaller than SCS of initial DL BWP. As described above, this will result in the problem on number of koffset bits and time synchronization accuracy.

For Alt. 4, we believe it is the outstanding choice because it does not need to increase the number of koffset bits, and it can achieve the required time synchronization accuracy. Moreover, it requires the minimum number of frequency offset estimation searchers when making frequency domain offset estimation during SSB detection.

In summary, Table 2 compares four alternatives in terms of different aspects as shown below:

Table 2  List of problems for these four alternatives

| Alt. | BWP switching problem | koffset problem | Time synchronization accuracy problem |
| --- | --- | --- | --- |
| Alt. 1: (120K, 120K) + (240K, 120K) | YES | NO | NO |
| Alt. 2: (120K, 120K) + (120/240K, 480K) + (120/240K, 960K) | NO | YES | YES |
| Alt. 3: (120K, 120K) + (480K, 480K) + (480K, 960K) | NO | YES | YES |
| Alt. 4: (120K, 120K) + (960K, 480K) + (960K, 960K) | NO | NO | NO |

From the Table 2, it is obvious seen that Alt. 4 is better than others. Therefore, the following proposal is made regarding the SCS selection for SSB and initial DL BWP:

Proposal 2: Support the following SCS pairs for SSB and initial DL BWP in NR operation from 52.6-71GHz：(120K, 120K) + (960K, 480K) + (960K, 960K).

SSB design in time design

In NR FR1&FR2, SSB pattern design is supported for 15KHz and 30KHz (FR1), and 120KHz and 240KHz (FR2). With the introduction of higher SCS for B52.6GHz system, the SSB pattern needs to be re-defined because of the newly introduced problems: 1, frequency domain offset estimation during SSB detection; 2, beam switching time problem.

SSB period for initial access

With increasing of the center frequency, the absolute value for frequency domain offset is increased if assuming the same ratio (e.g. 10ppm). It should be estimated by two steps with coarse estimation first and fine estimation second. The target of coarse estimation is to narrow down the offset into a small range, which is called frequency offset estimation searcher. Then fine estimation will be used for determining the actual offset value. However, the estimable frequency offset range by fine estimation is limited, i.e. where Nd is the time interval of PSS sequence and SSS sequence. Particularly, the range values of frequency offset for different SCSs are given below:

- SCS120: [-28K 28K]

- SCS240: [-56K 56K]

- SCS480: [-112K 112K]

- SCS960: [-224K 224K]

For a given offset value (e.g. -600K~600K Hz for 60GHz center frequency), the needed number of branches are given below:

- SCS120: searcher range 28k, steps = 1200k/56k≈22

- SCS240: searcher range 56k, steps = 1200k/112k ≈11

- SCS480: searcher range 112k, steps = 1200k/224k ≈6

- SCS960: searcher range 224k, steps = 1200k/448k ≈3

If using a smaller number of searchers than the above value, significant performance loss is observed from the figure below (240 KHz as example), i.e. >3dB loss in terms of PBCH BLER in 60GHz. However, more searchers mean more hardware complexity and device cost. One possible way is to perform frequency offset estimation in multiple SSB period using limited number of searchers, i.e. 6 branches monitoring 6 ranges in one SSB period and the other 6 ranges in next SSB period. This will result in larger latency for cell search. Therefore, to maintain the same cell search latency and hardware cost, smaller SSB period than that in FR1 & FR2 is expected for 52.6-71GHz system.

Figure 1  SSB detection performance for 240K SCS (TDL-A 10ns)

Observation 2：For frequency domain offset estimation during SSB detection, using SSB with low SCS such as 120K/240KHz may increase hardware complexity or cell search latency.

The second issue is the number of buffering samples for higher SCS such as 960KHz during the process of SSB detection. Currently, UE assumes 20ms SSB period during initial access, which means UE may need to buffer 20ms samples so that at least one SSB could be detected if it is transmitted on one certain sync raster. Naturally, smaller SSB period could solve this problem.

Observation 3: For number of buffering samples during SSB detection, using SSB with high SCS such as 960KHz will need larger buffer cost compared to that in FR2 if adopting the same SSB period (20ms).

According to the above analysis, assuming smaller SSB period for cell search could solve both the problems for low SCS (120/240K) and high SCS (960K). Another alternative is to relax RAN4 requirement for initial cell search time. Therefore, the following proposal is made:

Proposal 3: For initial cell search in 52.6-71GHz, a UE may assume that half frames with SSB occur with smaller period than FR2 (e.g. 5ms), or lower RAN4 requirement for the cell search time.

SSB pattern in time domain

Considering the possibility of introducing large SCS, the SSB pattern design in Time domain was discussed in RAN1#104-e meeting, and the following options were agreed[2]:

Agreement:

For 480 kHz and 960 kHz SSB SCS (if agreed)

- Study further on reserving symbol gap between SSB positions with different SSB index (and possibly between SSB position and other signal/channels)

  - FFS: whether symbol gap is needed for only 960 kHz or both 480 and 960 kHz.

- Study further on reserving gap for UL/DL switching within the pattern accounting possibility for reserving UL transmission occasions in the SSB pattern

- Study should account for inputs from RAN4

In Rel-16 (FR1), SSB can be configured by 15KHz and 30KHz SCS and only multiplexing pattern 1 was supported. In Rel-16 (FR2), the 120KHz and 240KHz was supported (case D and case E respectively) for SSB with multiplexing pattern 1,2 and3.

The case D and case E definition is shown in [3]:

-	Case D - 120 kHz SCS: the first symbols of the candidate SS/PBCH blocks have indexes . For carrier frequencies within FR2, n=0, 1, 2, 3, 5, 6, 7, 8, 10, 11, 12, 13, 15, 16, 17, 18.

-	Case E - 240 kHz SCS: the first symbols of the candidate SS/PBCH blocks have indexes . For carrier frequencies within FR2, n=0, 1, 2, 3, 5, 6, 7, 8.

According to the configuration, the time domain design for SCS 120KHz and 240KHz can be obtained:

Figure 2 SSB time domain design for SCS 120KHz (case D)

Figure 3 SSB time domain design for SCS 240KHz (case E)

For the design of SSB time domain pattern with SCS of 480KHz and 960KHz, it is preferable to reuse the existing pattern to minimize the specification impacts. Therefore, it is better to choose one of case D and case E as the baseline for designing the SCS 480 kHz and 960 kHz time domain pattern.

From Figure 3, we can see that four SSBs are consecutive with no gap. In such case, the UL date can only be transmitted after four DL SSB transmission, which is unfriendly to latency-sensitive scenario. One the other hand, the time duration of SCS 480KHz and 960KHz is 4 and 8 times greater than that of SCS 120KHz. The beam switching between Type0 PDCCH and RMSI will be more frequent which increase implementation complexity. Thus, it is preferable to reuse case D as the baseline for designing the SCS 480 kHz and 960 kHz time domain pattern.

Proposal 4: Support to reuse case D as the baseline for designing the SCS 480 kHz and 960 kHz time domain pattern.

Beam switching time is a key factor affecting the SSB time domain design. Whether the CP length can cover the beam switching time needs to be considered. For different SCS, the CP coverage is inconsistent which is shown in the following Table 3:

Table 3 List of NCP length for different numerologies

| SCS (kHz) | Symbol length | NCP length（6.57%） |
| --- | --- | --- |
| 15 | 66.67 us | 4.688 us |
| 30 | 33.33 us | 2.344 us |
| 60 | 16.67 us | 1.172 us |
| 120 | 8.33 us | 585.94 ns |
| 240 | 4.17 us | 292.97 ns |
| 480 | 2.08 us | 146.48 ns |
| 960 | 1.04 us | 73.24 ns |

As we can see form Table 3, the NCP length for SCS 480KHz is larger than 100ns (beam switching time), but the NCP length for SCS 960KHz is not long enough to cover the switching time. To solve the above problems, the following alternatives could be considered:

- Alt. 1: New SSB pattern introducing gaps between contiguous candidate SSBs;

- Alt. 2: The same QCL assumptions for contiguous candidate SSBs

For Alt. 1, the key idea is to reserve gap OFDM symbols between neighboring contiguous SSBs for beam switching. One example is given in Figure 4, i.e. move 2 OFDM symbols forward (from symbol 4-7 to 2-4) for the 1nd SSB in the slot. In this case, all of the SSB beams can be guaranteed not to cross the slot which can simplify the specification design. Note that scheduling restriction should be made on the gap symbols for beam switching.

Figure 4  The new SSB pattern with certain gaps

For Alt. 2, the key idea is to assume the same QCL (i.e. the same beam) for contiguous candidate SSBs. One example is given in Figure 5: The number of candidates SSB positions in time domain is extended to 128, where the neighboring continuous SSB are assumed to be QCLed in default. Namely, SSB 0 is QCLed with SSB 1, SSB 2 is QCLed with SSB 3, and etc. In this way, the total number of SSB beams is still 64. Note that the same color of SSBs is assumed to be QCLed in Figure 5.

Figure 5  The neighbouring two SSB beams are QCLed

In such case, the bits in ssb-PositionsInBurst indicator is used to indicate whether the same QCLed SSBs is sent or not. That is, ‘0’ represents the same QCLed SSBs will not be transmitted, and ‘1’ represents at least one of the QCLed SSBs should be transmitted. Then, another 1 bit can be introduced to indicate the meaning of ‘1’, where this bit can indicate all the QCLed SSB are transmitted or one unique of the QCLed SSBs is transmitted.

Proposal 5: The following alternatives could be considered to solve beam switching problem for contiguous candidate SSBs:

- Alt. 1: New SSB pattern introducing gaps between contiguous candidate SSBs;

- Alt. 2: The same QCL assumptions for contiguous candidate SSBs;

SSB transmission window

In RAN1#104-e meeting, the SSB transmission window was discussed and the following options were agreed [2]:

Agreement:

For an unlicensed band that requires LBT, further study whether/how to support discovery burst (DB) and discovery burst transmission window (DBTW) at least for 120 kHz SSB SCS

- If DB supported

  - FFS: What signals/channels are included in DB other than SS/PBCH block

- If DBTW is supported

  - Support mechanism to indicate or inform that DBTW is enabled/disabled for both IDLE and CONNECTED mode UEs

    - FFS: how to support UEs performing initial access that do not have any prior information on DBTW.

  - PBCH payload size is no greater than that for FR2

  - Duration of DBTW is no greater than 5 ms

  - Number of PBCH DMRS sequences is the same as for FR2

- The following points are additionally FFS:

  - How to indicate candidate SSB indices and QCL relation without exceeding limit on PBCH payload size

  - Details of the mechanism for enabling/disabling DBTW considering LBT exempt operation and overlapping licensed/unlicensed bands

  - Whether or not to support DBTW for SSB SCS(s) other than 120 kHz if other SSB SCS(s) are supported

In the design of NR-U, the DB has been introduced which includes Type0-PDCCH, SIB1 and CSI-RS. Besides, DBTW is also introduced to enable more flexible transmission upon LBT. Therefore, for NR 52.6 GHz to 71 GHz, it is better to reuse the design criterion to increase transmission opportunities of these signals and channels.

Proposal 6: Type0-PDCCH, SIB1 and CSI-RS should be included in DB other than SSB for NR operation from 52.6 GHz to 71 GHz.

Proposal 7: Support DBTW in un-licensed band from 52.6 GHz to 71 GHz, no matter which SSB SCS.

In case the DBTW is supported, how the UE knows whether there is DBTW in initial access process needs to be studied.  To solve the above problem, the following alternatives could be considered:

- Alt. 1: Determine whether there is DBTW based on the frequency band

- Alt. 2: Determine whether there is DBTW based on the indicator in PBCH

- Alt. 3: Determine whether there is DBTW based on the design of SSB sequence

For Alt. 1, after receiving SSB, the UE judges whether there is DBTW according to the frequency band information or sync raster where the SSB is located. That is, exist DBTW if operating in un-licensed band, no DBTW if operation in licensed band.  For Alt. 2, one bit in PBCH can be utilized to indicate whether there is DBTW. For Alt. 3, the time domain relationship between PSS sequence and SSS sequence, phase difference between PSS sequence and SSS sequence and the phase of PBCH DM-RS can be used to determine whether there is DBTW.

Proposal 8: The following methods could be considered to determine whether there is DBTW:

- Alt. 1: Frequency band (licensed or un-licensed);

- Alt. 2: The indicator in PBCH;

- Alt. 3: The design of SSB sequence (PSS, SSS and DMRS).

The SSB distributed in different DBTW windows also have a certain QCL relationship. For this purpose, the Q was designed in [3], which is related to the SCS and the LSB of ssb-SubcarrierOffset as shown in the following table.

Table 4 Mapping between the combination of subCarrierSpacingCommon and LSB of ssb-SubcarrierOffset to Q [3]

| subCarrierSpacingCommon | LSB of ssb-SubcarrierOffset | NSSBQCL |
| --- | --- | --- |
| scs15or60 | 0 | 1 |
| scs15or60 | 1 | 2 |
| scs30or120 | 0 | 4 |
| scs30or120 | 1 | 8 |

From Table 4, we can see that only one bit is used to indicate the value of Q for each SCS. However, with the introduce of large SCS (480KHz and 960KHz) and the increase number of transmitted SSB, the value of Q needs to be increased, such as Q=16 or 32. Thus, how to indicate the value of Q needs to be researched. Some solutions can be considered:

- Alt. 1: Specify the value of Q for each SCS;

- Alt. 2: Utilize the bits in PBCH;

For Alt. 1, the Q can be determined by SSB SCS or SSB pattern. For instance, when the SSB SCS is 480KHz or 960KHz, the Q can be defaulted to 16 or 32 respectively. For Alt. 2, some bits in PBCH can be utilized to indicate the value of Q.

Proposal 9: The following methods could be considered to indicate the value of Q:

- Alt. 1: Specify the value of Q for each SCS;

- Alt. 2: Utilize the bits in PBCH;

Not all SSBs in an SS Burst Set must be sent. Actually, the transmitted SSB is indicted by ssb-PositionsInBurst, which shows different types in SIB1 and servingcellconfigcommone. In servingcellconfigcommone, there has 64 bits to indicate the transmission of 64 SSBs. However, there has only 16 bits in SIB1, which contains two indicators: groupPresence and inOneGroup with each has 8 bits. The groupPresence indicator is used to indicate whether to transmit every 8 consecutive SSB (a group); the inOneGroup indicator is used to indicate which SSBs are sent in a group.

With the increase value of Q and the introduction of DBTW, the ssbPositionsInBurst in SIB1 may be ambiguated. We discuss the solutions under the following two scenarios:

- The protocol SSB pattern and new SSB pattern which introduce gaps between contiguous candidate SSBs;

- The SSB pattern where two continuous SSBs are assumed to be QCLed.

For case a), one solution is to ensure how many bits in the groupPresence are valid in terms of the value of Q. For instance, if Q=32, then the valid bits in the groupPresence are 4 (Q/8). Another solution is to re-define the groupPresence and inOneGroup in terms of Q. That is, determine the number of SSBs represented by each bit in groupPresence and inOneGroup according to the value of Q. For instance, if Q=32, Q/8=4, then 1 bit represent 4 SSBs.

For case b), if we assume the number of continuous QCLed SSBs is Y. Then, all candidate SSBs with the same modula (X, Q*Y) value have the same QCL relationship. In such a case, the bit in groupPresence need to be related to the value of Q*Y.

Proposal 10: With the increase value of Q and the introduction of DBTW, the ssbPositionsInBurst in SIB1 should be clarified.

SSB multiplexing pattern

Different (SSB, Coreset 0) SCS pair may involve different SSB-Coreset 0 multiplexing pattern as described in clause 13 of [3]. In FR2, the following multiplexing patterns are supported for each SCS pair as listed below:

- (120K, 60K): Pattern 1, Pattern 2

- (120K, 120K): Pattern 1, Pattern 3

- (240K, 60K): Pattern 1

- (240K, 120K): Pattern 1, Pattern 2

Similarly, the following proposal on multiplexing pattern is made based on Proposal 2 for B52.6GHz:

Proposal 11：The following SSB-Coreset 0 multiplexing patterns are supported for each SCS pair:

- (120K, 120K): Pattern 1, Pattern 3

- (960K, 960K): Pattern 1, Pattern 3

- (960K, 480K): Pattern 1, Pattern 2

Because the (120K, 120K) has been supported in the existing protocol, we mainly discuss the (960K, 960K) and (960K, 480K) here. In general, the design of SSB-Coreset 0 multiplexing pattern needs to consider the SSB time domain pattern as discussed in section 2.2.2. Thus, two aspects can be divided:

- New SSB pattern which introduce gaps between contiguous candidate SSBs;

- Two continuous SSBs are assumed to be QCLed.

For case a), the design of (960K, 960K) pattern 3 can be figured below:

Figure 6: Gaps between SSBs, (960K, 960K) pattern 3

The design of (960K, 480K) pattern 2 can be shown in Figure 7:

Figure 7：Gaps between SSBs, (960K, 480K) pattern 2

In the figures, the same color represents the SSB and corresponding Type0 PDCCH.

For case b), the design of (960K, 960K) pattern 3 can be figured below:

Figure 8: QCLed SSBs, (960K, 960K) pattern 3

In this figure, the same color SSBs means they are QCLed.

The design of (960K, 480K) pattern 2 can be shown in Figure 9:

Figure 9:  QCLed SSBs, (960K, 960K) pattern 2

Type0 PDCCH design

For the unlicensed band operation from 52.6GHz to 71GHz, more bits need to be introduced by considering the following aspects:

- The candidate SSB index may be extended, thus at least one bit should be introduced to indicate the SSB index;

- With the introduce of more SSB SCSs, the 1-bit subCarrierSpacingCommon indicator is insufficient;

- With the extend of the Q (QCL relationship), more bits are required.

Therefore, it is necessary to find some bits in PBCH to use.  We believe the sufficient method is to utilize the bits in search space configuration. In FR1, the search space configuration is different for licensed band and un-licensed, which can be find below:

Table 5: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {15, 15} kHz for frequency bands with minimum channel bandwidth 5 MHz or 10 MHz [3]

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 24 | 2 | 2 |
| 2 | 1 | 24 | 2 | 4 |
| 3 | 1 | 24 | 3 | 0 |
| 4 | 1 | 24 | 3 | 2 |
| 5 | 1 | 24 | 3 | 4 |
| 6 | 1 | 48 | 1 | 12 |
| 7 | 1 | 48 | 1 | 16 |
| 8 | 1 | 48 | 2 | 12 |
| 9 | 1 | 48 | 2 | 16 |
| 10 | 1 | 48 | 3 | 12 |
| 11 | 1 | 48 | 3 | 16 |
| 12 | 1 | 96 | 1 | 38 |
| 13 | 1 | 96 | 2 | 38 |
| 14 | 1 | 96 | 3 | 38 |
| 15 | Reserved |  |  |  |

Table 6: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {15, 15} kHz for frequency bands operated with shared spectrum channel access [3]

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs NRBCORESET | Number of Symbols NsymbCORESET | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 96 | 1 | 10 |
| 1 | 1 | 96 | 1 | 12 |
| 2 | 1 | 96 | 1 | 14 |
| 3 | 1 | 96 | 1 | 16 |
| 4 | 1 | 96 | 2 | 10 |
| 5 | 1 | 96 | 2 | 12 |
| 6 | 1 | 96 | 2 | 14 |
| 7 | 1 | 96 | 2 | 16 |
| 8 | Reserved |  |  |  |
| 9 | Reserved |  |  |  |
| 10 | Reserved |  |  |  |
| 11 | Reserved |  |  |  |
| 12 | Reserved |  |  |  |
| 13 | Reserved |  |  |  |
| 14 | Reserved |  |  |  |
| 15 | Reserved |  |  |  |

As we can see from Table 5 and Table 6, all the 4 bits in licensed are used, and three bits are used in un-licensed band. The reason why 1bit can be saved in un-licensed band is that the design of CORESET only support 96 RBs on the frequency domain. For the CORESET design of un-licensed band operation from 52.6GHz to 71GHz, we can leverage the design criterion that were considered in NR-U, which is occupy as much bandwidth as possible in the frequency domain.  One example is shown in Table 7.

Table 7: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {120, 120} kHz for frequency bands operated with shared spectrum channel access

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 96 | 1 | 14 |
| 1 | 1 | 96 | 2 | 14 |
| 2 | 3 | 96 | 2 | -20 if -21 if |
| 3 | 3 | 96 | 2 | 48 |
| 4 | Reserved |  |  |  |
| 5 | Reserved |  |  |  |
| … | Reserved |  |  |  |
| 11 | Reserved |  |  |  |
| 12 | Reserved |  |  |  |
| 13 | Reserved |  |  |  |
| 14 | Reserved |  |  |  |
| 15 | Reserved |  |  |  |

Proposal 12: To save more bits, the CORESET design of un-licensed band operation from 52.6GHz to 71GHz can re-use the design criterion in NR-U, which occupies as much bandwidth as possible in the frequency domain.

- PRACH and initial UL BWP

In initial access process, in addition to the SSB channel, the PRACH channel is essential which will be discussed in this section.

  - PRACH and initial UL BWP numerology selection

In RAN1#104-e meeting, the numerology of PRACH was discussed and the following options were agreed [2]:

Agreement:

- For initial access and non-initial access use cases, support 120kHz PRACH SCS with sequence length L=571, 1151 (in addition to L=139) for PRACH Formats A1~A3, B1~B4, C0, and C2.

- For non-initial access use cases,

  - if 480kHz and/or 960 kHz SSB SCS is agreed to be supported, support 480 and/or 960 kHz PRACH SCS with sequence length L=139 for PRACH Formats A1~A3, B1~B4, C0, and C2, respectively.

    - FFS: support of sequence length L = 571, 1151

- FFS: Support of 480 and/or 960 kHz PRACH SCS for initial access use cases, if 480 and/or 960 kHz SSB SCS is agreed to be supported for initial access

In NR FR1&FR2, there are different PRACH formats defined in TS 38.211 as shown in Table 8 below. Special SCS is defined for format 0-3 mainly targeting for very large coverage scenario (e.g. >1000m) and FR1. However, in 52.6-71GHz, the target use case is small cell and there is no need to support such scenario. Thus, PRACH format 0-3 need not to be supported in 52.6-71GHz. Therefore, the following proposal is made:

Proposal 13: Format 0-3 with special SCS is not supported.

Table 8	   PRACH formats for NR FR1&FR2

| Format |  |  |
| --- | --- | --- |
| 0 | 839 | 1.25 kHz |
| 1 | 839 | 1.25 kHz |
| 2 | 839 | 1.25 kHz |
| 3 | 839 | 5 kHz |
| A1 | 139 |  |
| A2 | 139 |  |
| A3 | 139 |  |
| B1 | 139 |  |
| B2 | 139 |  |
| B3 | 139 |  |
| B4 | 139 |  |
| C0 | 139 |  |
| C2 | 139 |  |

In  FR2，the applicable numerologies for PRACH format (A, B, C) are 60KHz and 120KHz, which is the same as that for BWP although they are separately configured. Namely, the supported SCS pairs for PRACH format (A, B, C) and initial UL BWP in FR2 are (60K, 60K), (60K, 120K), (120K, 60K) and (120K, 120K). It is clearly to observe that all supported SCS for data/control SCS could be used for initial UL BWP. Besides, to save UE implementation complexity, there are only two applicable SCSs for PRACH format (A, B, C).

Observation 4: All supported SCS for data/control SCS could be used for initial UL BWP in NR FR2 operation.

Observation 5: There are only two applicable SCSs for PRACH format (A, B, C) in NR FR2 operation.

From the agreed WID [5],, the following agreement have been made: (120, 480, 960) KHz SCS will be supported for data/control channel of a DL/UL BWP. Based on this, we will discuss the numerology selection for PRACH and initial UL BWP. There are two use cases for the numerology selection for PRACH: initial access use cases and non-initial access use cases. First, we will discuss the numerology selection for PRACH (for initial access use cases) together with initial UL BWP. The first alternative is to reuse the existing FR2 SCS pair for PRACH and initial UL BWP:

- Alt. 1: (120K, 120K)

For Alt. 1, only single numerology is supported for initial UL BWP. However, considering that 960KHz SCS could have large bandwidth and good support of high modulation, it is preferred to use 960KHz BWP as operation BWP for peak data rate use case. As discussed in Section 2.1, if following Alt.1, the BWP switching between initial BWP and the preferred operation BWP may introduce more delay and bring more complexity to both UE and gNB. Therefore, to solve the above problem, supporting all SCS for data/control as SCS for initial UL BWP is needed as that in FR2, the following proposal is made:

Proposal 14: Support 120KHz, 480KHz and 960KHz as candidate SCS of initial UL BWP.

It has been agreed that 120KHz PRACH SCS is supported and additional PRACH SCS (480KHz, 960KHz) can be candidate for initial access in 52.6-71GHz. Since the UE capability is limited, no more than two candidate numerologies should be supported for PRACH for initial access use cases. Then, the following alternative SCS pairs for PRACH format (A, B, C) and initial UL BWP should be considered:

- Alt. 2: (120K, 120K) + (120K, 480K) + (120K, 960K)

- Alt. 3: (120K, 120K) + (480K, 480K) + (480K, 960K)

- Alt. 4: (120K, 120K) + (960K, 960K) + (960K, 480K)

It clearly observes that the issue of BWP switching can be solved by above three alternatives. For Alt. 2, we can find that only 120KHz SCS is supported for PRACH, where it brings a restriction to UL/DL configuration, because the RO in time domain should be configured in UL and does not support for other data/control transmission. Besides, the operation for the mixed numerology of PRACH and initial UL BWP (e.g., (120K, 480K) and (120K, 960K)) will increase the complexity of implementation.

Despite of the support of 120KHz SCS, the difference between Alt.3 and Alt.4 is the introduction of a new SCS: 480KHz SCS is supported for Alt.3, while 960KHz SCS is supported for Alt.4. Compared to Alt.2, Alt.3 and Alt.4 can support single numerology operation on 480KHz and 960KHz initial BWP respectively. Since 960KHz SCS is preferred to be supported for high peak date rate use cases, a better choice is to support 960KHz SCS for initial UL BWP and PRACH.

Proposal 15: Support 960KHz SCS for PRACH format (A, B, C) in addition to 120KHz SCS for initial access use cases.

  - Sequence length

For PRACH design, there are two important metrics: coverage and capacity. For a given PRACH format, CP length is defined separately with other channels and is still related with configured SCS for the PRACH. Particularly, CP length becomes smaller when SCS increases. Thus, the coverage should be also estimated by considering TA estimation accuracy. Ncs is a factor related with PRACH capacity, which is calculated as Ncs=2rc+τ×LRA（Tseq×Ts)+ng, where r is the cell radius, c is the speed of light and τ is obtained from channel model (e.g. τ= 96*10-9 for TDL-A with 10ns delay spread). Clearly it also depends on different SCS and Ncs is increasing with higher SCS. In this case, the PRACH sequence capacity may be limited.

Ncs, which is used to control the cyclic shift of the root sequence, play an important part in the process of preamble generation. Thus, one issue which should be considered is that with the usage of higher SCS, a larger value of Ncs needs to be used to match the certain coverage area, as shown in Table 9. Therefore, the number of cyclic shifts should be large enough to ensure the orthogonality of the sequence at the receiver, and large number of root sequences is needed to generate 64 preamble sequences. In this case, the same root sequence may be used between adjacent cells, which caused the cell edge users cannot determine which cell the received preamble sequence was sent from, and then access the wrong cell. Especially for 139 length LRA, because only 138 root sequences can be used to generate 64 preamble sequences.

Table 9  List of Ncs value to match the coverage area of 100m radius for different numerologies

| SCS (kHz) | Ncs value |  |  |
| --- | --- | --- | --- |
|  | LRA=139 | LRA=571 | LRA=1151 |
| 120 | 13 | 53 | 106 |
| 240 | 26 | 105 | 211 |
| 480 | 51 | 210 | 422 |
| 960 | 102 | 419 | 843 |

Observation 6: With the usage of higher SCS, the PRACH sequence capacity is very limited when the preamble sequence length is 139.

Based on the above problem, one solution is to send different preamble sequences in adjacent cells, so that there may be less than 64 preamble sequences available in each cell. In this case, in order not to affect the PRACH process, the issue of preamble sequence generation needs to be considered.

Proposal 16: With the usage of higher SCS, the issue of preamble sequence generation needs to be considered to match the certain coverage area.

For sequence lengths 571 and 1151, the number of root sequence is 570 and 1150 respectively, which are sufficient to generate different 64 preambles in different cells. Thus, the issue of preamble sequence generation can be solved by gNB implement.

Proposal 17: The PRACH sequence lengths 571 and 1151 can be supported for 480/960KHz SCS.

  - RO Configuration

Since LBT is necessary for unlicensed band operation in 52.6-71GHz, the RO configuration in time domain should be discussed to avoid the scenario where another UE fail to transmit preamble due to the utilizing of the previous RO. For NR-U, only 15kHz and 30kHz are supported and consecutive RO is fine. However, with the increase of SCS, the probability of LBT failure would be larger since the LBT period would be longer than the RO length. For example, the LBT period will overlap with 2 or more ROs for 960kHz SCS, and the LBT before PRACH transmission may fail due to the previous 2 or more ROs may have been selected by other UE for PRACH transmission.  Therefore, it would be necessary to support non-consecutive RO in time domain for unlicensed band operation in 52.6-71GHz.

Proposal 18: Support non-consecutive RO in time domain to avoid LBT failure.

Random access preambles can only be transmitted in the time domain resources obtained from Tables 6.3.3.2-2 to 6.3.3.2-4 [6]. They are configured by the higher-layer parameter prach-ConfigurationIndex-v1610, or prach-ConfigurationIndex, or msgA-PRACH-ConfigurationIndex, which depends on different frequency band (FR1 or FR2) and the spectrum type as defined in TS38.104 [5]. In FR2, the SCS 60kHz and 120kHz for PRACH are supported, meanwhile, the 60KHz SCS is used to be the reference SCS. As shown in Figure 10and Figure 11 , only one PRACH slot can be configured in each reference slot when PRACH SCS is 60kHz, and one or two PRACH slots can be configured in each reference slot when PRACH SCS is 120kHz. With the introduction of larger SCS in 52.6-71GHz, such as 960kHz SCS, there will be more than two PRACH slots within a reference slot. However, the existing configuration mechanism in specification can only support up to two PRACH slots configuration within a reference slot. In order to make full use of the time domain resource for PRACH transmission, how to configure ROs should be considered for new SCS.

Figure 10  RO configuration of PRACH SCS=60 kHz based on Reference SCS=60 kHz

Figure 11  RO configuration of PRACH SCS=120 kHz based on Reference SCS=60 kHz

Proposal 19: With the introduction of larger SCS in 52.6-71GHz, such as 480/960kHz, how to configure time domain ROs should be considered.

As discussed above, one approach is to reuse FR2 RO slot configuration but to define new reference slot and re-interpret RACH slot index. There are two key configuration parameters to determine the RACH slot index: one is the slot number (reference slot index) and the other is the number of PRACH slots within a reference slot.

By defining different reference slot, different configuration parameters can be obtained to determine the RACH slot index. One potential solution is to define different reference slot for different PRACH SCS, e.g., the SCS of the reference slot is half of the PRACH SCS. In such a case, the number of PRACH slots within a reference slot and the rule of slot numbering are the same as FR2, but could include more reference slots, as shown in Figure 12.

We can also use the same reference slot as FR2, where the number of PRACH slots within a reference slot should be adapted to the PRACH SCS. Since both the PRACH processing load and access efficiency should be considered, the number of PRACH slots within a reference slot should be carefully selected. As an example, the new slot configuration within a reference slot shown in Figure 13 can be obtained by repeating the slot configuration of 120kHz PRACH SCS in every two slots.

Figure 12  RO configuration of PRACH SCS=960 kHz based on Reference SCS=480 kHz

Figure 13  RO configuration of PRACH SCS=960 kHz based on Reference SCS=60 kHz

If SCSs of 120kHz and 960kHz are supported for PRACH in 52.6-71GHz, in order to minimize changes to the existing specifications, a better solution to this problem is reusing FR2 configuration for existing SCS (120kHz) and defining new reference slot for other SCS (960kHz).

Proposal 20: One approach is to reuse FR2 RO slot configuration rule but to define new reference slot and re-interpret RACH slot index for high PRACH SCS in 52.6-71GHz.

  - RA-RNTI calculation

Random Access RNTI（RA-RNTI）is used during Random access response procedure. In response to a PRACH transmission, UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI [3]. RA-RNTI can characterize the time-frequency resource associated with the PRACH where Random Access Preamble is transmitted. RA-RNTI is of 16-bit in length and its value can be computed follow [7]:

RA-RNTI=1+s_id+14×t_id+14×80×f_id +14×80×8×ul_carrier_id

where s_id is the index of the first OFDM symbol of the PRACH occasion(0 ≤ s_id < 14)； t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80), where it is determined by the value of μ(related to SCS) , specified in clause 5.3.2 in TS 38.211; f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8); ul_carrier_id is the Uplink carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier).

In the formula, the fixed value ‘80’ is based on the maximum 120kHz SCS for FR1&FR2. When a larger PRACH SCS is introduced in 52.6-71GHz, if this formula is adopted, the same RA-RNTI may be obtained in different time-frequency resource. In such a case, the UE may be confused with the RA-RNTI which it needs to detect. Therefore, the UE requires a contention resolution mechanism to resolve this conflict, which may reduce the access efficiency. Another issue which should be considered is that, since the RA-RNTI has a specific range of value, some data overflow may be occurred if modifying the fixed value ‘80’. Therefore, the issue of RA-RNTI calculation needs to be investigated. In this paper, we will provide some solutions to this issue for consideration.

- Alt.1:

It is naturally straightforward to extend the range of slot indexes t_id to the maximum number of slots based on the maximum PRACH SCS supported in 52.6-70GHz.

RA-RNTI = (1+s_id+14×t_id+14×X×f_id +14×X×8×ul_carrier_id) mod A

where X is the maximum number of slots in a RAR window based on the maximum supported PRACH SCS. The number of in a RAR window is 320 for 480KHz SCS and 640 for 960KHz SCS. Since the RA-RNTI may be overflow when X is more than 320, modular operation needs to be applied, where A can be the specific range of RA-RNTI or RNTI. However, in this case, some ROs with RA-RNTI may be same or conflict with the pre-allocate RNTI. Some contention resolution mechanism is required to resolve the conflict.

- Alt.2:

Another way to minimize the changes to specification is reuse the current RA-RNTI formula while introducing additional indicator field to indicate the time-frequency resource together with RA-RNTI. In NR Rel-16, 2-bit for the DCI format 1_0 was used to indicate the LSBs of SFN corresponding to the RO used to transmit the preamble if the RAR window is larger than 10ms. Thus, we can also reuse the current RA-RNTI formula while introducing additional indicator field in DCL format 1_0.

For example, if 960KHz PRACH SCS is supported in 52.6-71GHz, there are 640 slots in a 10ms RAR window. The current range of slot indexes t_id is below 80. Therefore, we can divide the 640 slots into 8 segments and each segment contains 80 slots. 3-bit for the DCI format 1_0 is needed to indicate which segment the RO is on. In this case, we can set different bits for indicator according to the supported PRACH SCS.

- Alt.3:

In fact, the calculation of RA-RNTI depends on the RO configuration pattern. For some pattern, there is no need to modify the RA-RNTI formula but express the slot indexes t_id based on a new specific subcarrier spacing. For example, following the Rel15/16 design, if two 480/960 kHz are defined within a 60kHz reference slot, we can reuse the RA-RNTI formula and the slot indexes t_id is numbered in SCS 120kHz.

Proposal 21: For larger PRACH SCS (480KHz/960KHz), the following options can be considered for RA-RNTI calculation:

- Alt.1: Modify the RA-RNTI formula as following and introduce some contention resolution mechanism to resolve the conflict.

RA-RNTI = (1+s_id+14×t_id+14×X×f_id +14×X×8×ul_carrier_id) mod A

- Alt.2: Reuse the current RA-RNTI formula while introducing additional indicator field to indicate the time-frequency resource together with RA-RNTI.

- Alt.3: Depending on the RO configuration pattern, reuse the RA-RNTI formula and express the slot indexes t_id based on a new specific subcarrier spacing.

- SSB and PRACH design other than initial access

In addition to initial access, SSB could also be used for beam management, RSRP measurement, BFR and etc... For PRACH, it can be used for RRC Connection re-establishment procedure, transition from RRC INACTIVE, BFR and etc. In this case, it is preferable to design the SSB and PRACH (BWP and SCS) as follows that of DL/UL channels BWP to ensure accuracy. Meanwhile, to avoid switching between different BWPs can simplify the implementation complexity for both gNB and UE. In order to minimize the changes to specification, the design of SSB and PRACH should be consistent for initial access and non-initial access.

Proposal 22: The design of SSB and PRACH should be consistent for initial access and non-initial access.

- Conclusion

In this contribution, we focus on the required changes to NR using existing NR waveform, and have the following proposals:

Observation 1: All supported SCS for data/control SCS in FR2 could be used for initial DL BWP.

Observation 2：For frequency domain offset estimation during SSB detection, using SSB with low SCS such as 120K/240KHz may increase hardware complexity or cell search latency.

Observation 3: For number of buffering samples during SSB detection, using SSB with high SCS such as 960KHz will need larger buffer cost compared to that in FR2 if adopting the same SSB period (20ms).

Observation 4: All supported SCS for data/control SCS could be used for initial UL BWP in NR FR2 operation.

Observation 5: There are only two applicable SCSs for PRACH format (A, B, C) in NR FR2 operation.

Observation 6: With the usage of higher SCS, the PRACH sequence capacity is very limited when the preamble sequence length is 139.

Proposal 1: Support SCS 120KHz, 480KHz and 960KHz for initial DL BWP in NR operation from 52.6-71GHz.

Proposal 2: Support the following SCS pairs for SSB and initial DL BWP in NR operation from 52.6-71GHz：(120K, 120K) + (960K, 480K) + (960K, 960K).

Proposal 3: For initial cell search in 52.6-71GHz, a UE may assume that half frames with SSB occur with smaller period than FR2 (e.g. 5ms), or lower RAN4 requirement for the cell search time.

Proposal 4: Support to reuse case D as the baseline for designing the SCS 480 kHz and 960 kHz time domain pattern.

Proposal 5: The following alternatives could be considered to solve beam switching problem for contiguous candidate SSBs:

- Alt. 1: New SSB pattern introducing gaps between contiguous candidate SSBs;

- Alt. 2: The same QCL assumptions for contiguous candidate SSBs;

- Alt. 3: Hopping transmission for contiguous candidate SSBs.

Proposal 6: Type0-PDCCH, SIB1 and CSI-RS should be included in DB other than SSB for NR operation from 52.6 GHz to 71 GHz.

Proposal 7: Support DBTW in un-licensed band from 52.6 GHz to 71 GHz, no matter which SSB SCS.

Proposal 8: The following methods could be considered to determine whether there is DBTW:

- Alt. 1: Frequency band (licensed or un-licensed);

- Alt. 2: The indicator in PBCH;

- Alt. 3: The design of SSB sequence (PSS, SSS and DMRS).

Proposal 9: The following methods could be considered to indicate the value of Q:

- Alt. 1: Specify the value of Q for each SCS;

- Alt. 2: Utilize the bits in PBCH;

Proposal 10: With the increase value of Q and the introduction of DBTW, the ssbPositionsInBurst in SIB1 should be clarified.

Proposal 11：The following SSB-Coreset 0 multiplexing patterns are supported for each SCS pair:

- (120K, 120K): Pattern 1, Pattern 3

- (960K, 960K): Pattern 1, Pattern 3

- (960K, 480K): Pattern 1, Pattern 2

Proposal 12: To save more bits, the CORESET design of un-licensed band operation from 52.6GHz to 71GHz can re-use the design criterion in NR-U, which is occupy as much bandwidth as possible in the frequency domain.

Proposal 13: Format 0-3 with special SCS is not supported.

Proposal 14: Support 120KHz, 480KHz and 960KHz as candidate SCS of initial UL BWP.

Proposal 15: Support 960KHz SCS for PRACH format (A, B, C) in addition to 120KHz SCS for initial access use cases.

Proposal 16: With the usage of higher SCS, the issue of preamble sequence generation needs to be considered to match the certain coverage area.

Proposal 17: The PRACH sequence lengths 571 and 1151 can be supported for 480/960KHz SCS.

Proposal 18: Support non-consecutive RO in time domain to avoid LBT failure.

Proposal 19: With the introduction of larger SCS in 52.6-71GHz, such as 480/960kHz, how to configure time domain ROs should be considered.

Proposal 20: One approach is to reuse FR2 RO slot configuration rule but to define new reference slot and re-interpret RACH slot index for high PRACH SCS in 52.6-71GHz.

Proposal 21: For larger PRACH SCS (480KHz/960KHz), the following options can be considered for RA-RNTI calculation:

- Alt.1: Modify the RA-RNTI formula as following and introduce some contention resolution mechanism to resolve the conflict.

RA-RNTI = (1+s_id+14×t_id+14×X×f_id +14×X×8×ul_carrier_id) mod A

- Alt.2: Reuse the current RA-RNTI formula while introducing additional indicator field to indicate the time-frequency resource together with RA-RNTI.

- Alt.3: Depending on the RO configuration pattern, reuse the RA-RNTI formula and express the slot indexes t_id based on a new specific subcarrier spacing.

Proposal 22: The design of SSB and PRACH should be consistent for initial access and non-initial access.

- References

- RP-202925, RAN #90-e, Electronic Meeting, December 7-11, 2020.

- R1-2101971, RAN1 #104-e, Summary #4 of email discussion on initial access aspect of NR extension up to 71 GHz, January 25th – February 5th, 2021

- TS 38.213 v16.4.0, NR Physical layer procedures for control, 2020-12.

- R1-2005920, Discussions on required changes to NR using existing NR waveform, vivo, RAN WG1 #103, e-Meeting, October 26th – November 13th, 2020.

- 3GPP RAN1#103-e, Chairman’s notes.

- TS 38.211 v16.3.0, NR Physical channels and modulation, 2020-09

- TS 38.321 v16.2.1, NR Medium Access Control (MAC) protocol specification, 2020-09