3GPP TSG RAN WG1 Meeting #104b-e	R1-2102680

E-Meeting, April 12th – 20th, 2021

Agenda Item:	8.9.1

Source:	Mediatek

Title:	Switching point between QPSK and 16QAM and DCI design for 16QAM in R17

Document for:	Discussion and Decision

Introduction

In the RAN1 104-e meeting, following agreements have been captured in [1].

Working Assumption

- The following TBS indices are introduced for downlink

| I_TBS | I_SF |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 | 2856 |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 | 3112 |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 | 2600 | 3240 |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 | 2856 | 3624 |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 | 3112 | 4008 |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2600 | 3496 | 4264 |
| 20 | 440 | 904 | 1384 | 1864 | 2344 | 2792 | 3752 | 4584 |
| 21 | 488 | 1000 | 1480 | 1992 | 2472 | 2984 | 4008 | 4968 |

- I_SF>7 is not supported in Rel-17.

Agreement

- Repetition is not used for 16-QAM in uplink.

Agreement

- Repetition of 2 is NOT supported for 16-QAM in downlink

There is company insists the simulation should base on fading channel and accordingly we did further simulation to confirm whether majorities’ view (ITBS 14) is suitable or not. At the same time, repetition is not supported for 16QAM has been confirmed, it’s time to discuss the DCI fields related to supporting 16QAM.

Analysis and Discussion

Further investigation to switching point in Downlink TBS indices

In order to identify what’s the best switching point, since Ran1 103# meeting, many companies provided simulation results based on simulation assumption from RAN1 102# meeting, i.e. AWGN channel. According to the simulation results and other considerations, majorities concluded that the switching point should be ITBS=14 for downlink SB/GB. However, [4] declared that the main objective of introducing 16QAM is aiming at mobile scenarios, consequently simulation should base on fading channel and proposed that switching point should be ITBS=12 for downlink SB/GB. At the end of 104# meeting, this difference was discussed again and providing simulation result based on fading channel was encouraged. Considering the difference is whether switching point should move from ITBS=14 down to ITBS=12 or not, our simulation focus on ITBS=12/13.

For DL 1T1R Standalone operation mode, Figure 1 and Figure 2 show the simulation result for ETU1 and ETU5, respectively. For ITBS=12 and ETU=1Hz, the performance of QPSK is better than 16QAM about 2.0dB. For ITBS=13 and ETU=1Hz, the performance of QPSK is almost the same as 16QAM. For ITBS=12 and ETU=5Hz, the performance of QPSK is better than 16QAM about 1.6dB. For ITBS=13 and ETU=5Hz, the performance of QPSK is almost the same as 16QAM.

For DL 2T1R Standalone operation mode, Figure 3 and Figure 4 show the simulation result for ETU1 and ETU5, respectively. For ITBS=12 and ETU=1Hz, the performance of QPSK is better than 16QAM about 2.9dB. For ITBS=13 and ETU=1Hz, the performance of QPSK is better than 16QAM about 1.8dB. For ITBS=12 and ETU=5Hz, the performance of QPSK is better than 16QAM about 2.8dB. For ITBS=13 and ETU=5Hz, the performance of QPSK is better than 16QAM about 1.5dB.

Figure 1 NPDSCH BLER of standalone mode, ITBS=12/13, Isf = 4, QPSK vs. 16QAM with 1T1R and in ETU1

Figure 2 NPDSCH BLER of standalone mode, ITBS=12/13, Isf = 4, QPSK vs. 16QAM with 1T1R and in ETU5

Figure 3 NPDSCH BLER of standalone mode, ITBS=12/13, Isf = 4, QPSK vs. 16QAM with 2T1R and in ETU1

Figure 4 NPDSCH BLER of standalone mode, ITBS=12/13, Isf = 4, QPSK vs. 16QAM with 2T1R and in ETU5

For convenience, we put the comparison in following Table 1, and we can observe that QPSK is much better than 16QAM in most cases.

Table 1 Summary of simulation results

| Antenna | Channel | ITBS | QPSK >16QAM |
| --- | --- | --- | --- |
| 1T1R | ETU1 | 12 | 2.0dB |
|  |  | 13 | 0.1dB |
|  | ETU5 | 12 | 1.6dB |
|  |  | 13 | 0.1dB |
| 2T1R | ETU1 | 12 | 2.9dB |
|  |  | 13 | 1.8dB |
|  | ETU5 | 12 | 2.8dB |
|  |  | 13 | 1.5dB |

Therefore, we propose:

Proposal1：ITBS=14 and ITBS=11 are switching point for SB/GB and IB, respectively.

Consideration on DCI design

In last RAN1 meeting, progress was achieved with regard to whether repetition is support or not.

Agreement

- Repetition is not used for 16-QAM in uplink.

Agreement

- Repetition of 2 is NOT supported for 16-QAM in downlink

Since repetition is not supported for 16QAM, the repetition number fields in DCI should be invalid, i.e. 3 spare bits for DCI N0 and 4 spare bits for DCI N1. This greatly makes it easy to implement 16QAM related scheduling.

Firstly, in order to achieve better granularity of TB scheduling, adding ITBS11~ ITBS21 has been agreed which requires 5-bits MCS field in DCI. Without increasing DCI size, both MCS fields in DCI N0 and DCI N1 can borrow 1 bit from the spare bits to reach it.

Secondly, as mentioned in [2] and [3], except for the downlink channel quality reporting in legacy NB-IOT, L1 layer reporting method is important and necessary. The straightforward way is reusing the LTE/e-MTC CQI reporting mechanism. Specifically considering NB-IOT’s characteristics, the reporting method is much simpler than LTE/e-MTC. As we know, the most application for NB-IOT is burst data transmission, the periodic reporting will waste limited resource in this situation. Therefore, periodic reporting should not be supported for NB-IOT. For the aperiodic reporting, due to NB-IOT just uses one physical RB and transmission mode is fixed, reporting mode should be very simple and added 1 bit is enough to indicate reporting request. And the added 1 bit also can borrow from the repetition number fields.

Observation1: Aperiodic CQI reporting should be supported in NB-IOT R17.

In legacy NB-IOT, the reported repetition number is derived from the measurement on NRS in downlink sub-frames. NB-IOT R17 also can base on the same measurement framework. There is no need to define extra measurement reference resource.

Observation2: There is no need to define extra measurement reference resource for CQI reporting.

Even UE stays in 16QAM enable state, in view of the time variation of mobile channel, it’s still possible to use QPSK when UE encounters relatively poor channel quality. Therefore, the range of reported CQI value should cover both QPSK and 16QAM, i.e. at least 3-bits of CQI value is expected. This still has no problem because capacity of NPUSCH is enough.

Observation3: The CQI table should cover both QPSK and 16QAM.

Based above discussions, following proposals are proposed:

Proposal2: Reuse 1 bit from the spare bits to reach 5 bits MCS field both in DCI N0 and DCI N1.

Proposal3: Reuse 1 bit from the spare bits to indicate CQI reporting request in DCI N0.

Conclusions

We have following observations and proposals:

Observation1: Aperiodic CQI reporting should be supported in NB-IOT R17.

Observation2: There is no need to define extra measurement reference resource for CQI reporting.

Observation3: The CQI table should cover both QPSK and 16QAM.

Proposal1：ITBS=14 and ITBS=11 are switching point for SB/GB and IB, respectively

Proposal2: Reuse 1 bit from the spare bits to reach 5 bits MCS field both in DCI N0 and DCI N1.

Proposal3: Reuse 1 bit from the spare bits to indicate CQI reporting request in DCI N0.

- References

- “https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_104-e/Report/Draft_Minutes_report_RAN1%23104-e_v030.zip.

- R1-2100581, “Consideration on CQI report and Repetition applicability for 16QAM in R17”, Mediatek

- R1-2100570, “Channel quality report for 16QAM in NB-IOT”,ZTE

- R1-2100567,” Discussion on UL and DL 16QAM for NB-IOT”, ZTE