3GPP TSG RAN WG1 Meeting #104bis-e  	R1-2102700

e-Meeting, April 12th – April 20th, 2021

Agenda Item: 8.6.1.2

Source: MediaTek Inc.

Title: On Reduced Number of RX Branches for RedCap UEs

Document for: Discussion and Decision

# Introduction

The Rel-17 study item on Reduced Capability NR devices was approved in [1] and updated in [2]. The new WI has been agreed in RAN#91 [3], also supporting 1Rx option across FR1:

WI [3]: - Reduced minimum number of Rx branches: - For frequency bands where a legacy NR UE is required to be equipped with a minimum of 2 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands. - For frequency bands where a legacy NR UE (other than 2-Rx vehicular UE) is required to be equipped with a minimum of 4 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands. - A means shall be specified by which the gNB can know the number of Rx branches of the UE.

In the past RAN1 meeting the following agreement was reached:

Agreements: - For reduced minimum number of Rx branches in FR1 and FR2 frequency bands where a legacy NR UE is required to be equipped with a minimum of 2 Rx antenna ports: - FFS: need for solutions to reduced PDCCH blocking - FFS: need for reporting of UE antenna related information to gNB (e.g., # of panels, polarization, etc.) - Information related to the reduction of the number of antenna branches is assumed to be known at the gNB (either implicitly or explicitly, to be FFS)

# UE Capability Reporting

# Irrespective of early detection, the RedCap UE shall report its antennae configurations, such as number of Rx branches and antenna panel configurations, to the gNB as part of UE capability reporting.

# Proposal 1: The RedCap UE reports its antennae configurations, such as number of Rx branches and antenna panel configurations, to the gNB as part of UE capability reporting.

# Conclusions

# Proposal 1: The RedCap UE reports its antennae configurations, such as number of Rx branches and antenna panel configurations, to the gNB as part of UE capability reporting.

# References

- RP-193238 “New SID on support of reduced capability NR devices”, Ericsson, RAN#86, December 2019

- RP-201386 “Revised SID on Study on support of reduced capability NR devices”, Ericsson, RAN#88e, July 2020

- RP-210918 New WID on support of reduced capability NR devices, Ericsson, Nokia, March 2021

- 3GPP TR 38.875, Study on support of reduced capability NR devices, Rel-17, V2.0.0, March 2021