3GPP TSG RAN WG1 #104bis-e                                            R1-2102858

e-Meeting, April 12th – 20th, 2021

Source:               ZTE

Title:                    Remaining issues on 14-HARQ processes in DL for eMTC

Agenda item:      8.9.2

Document for:    Discussion and Decision

Introduction

In RAN1 #103 e-meeting, the following agreements were achieved for 14-HARQ processes in DL for LTE-MTC [1]:

- The design of the 14 HARQ processes feature accounts for the presence of non-BL/CE UL and DL subframes in the PUCCH non-repetition case.

- FFS: PDSCH scheduling delays

- FFS: HARQ-ACK delays

- FFS: Configurable/dynamic set of PDSCH delays/HARQ-ACK delays

- For the support of 14 HARQ processes, the solution to assign PDSCH scheduling delays should be able to minimize unnecessary waste of subframes derived from the presence of non-BL/CE DL subframes and non-BL/CE UL subframes.

- The following solutions will be further investigated:

  - The indication of subframe types for the PDSCH scheduling delay of 7 are:

    - 1 BL/CE DL subframe + 1 subframe + 3 [BL/CE UL subframes] + 1 subframe + 1 BL/CE DL subframe.

    - 1 subframe + 3 [BL/CE UL subframes] + 1 subframe + 2 BL/CE DL subframes.

  - Configurable delays including other values than 2 and 7.

- Other solutions are not precluded.

- For the support of 14 HARQ processes, the solution to assign HARQ-ACK delays should aim to maximize the number of HARQ processes that can be scheduled in presence of non-BL/CE DL subframes and non-BL/CE UL subframes.

- Different percentages of presence of non-BL/CE subframes can be analyzed as to represent typical scenarios and determine which HARQ-ACK delays should be included.

In RAN1 #104 e-meeting, the following agreements were achieved for 14-HARQ processes in DL for LTE-MTC [2]:

- The PDSCH scheduling delay for the PUCCH non-repetition case (i.e., PUCCH repetitions = 1):

- 2 BL/CE DL subframes.

- The PDSCH scheduling delay of 7 is expressed as:

  - 1 BL/CE DL subframe + 1 subframe + [3 subframes] + 1 subframe + 1 BL/CE DL subframe.

  - 1 subframe + [3 subframes] + 1 subframe + 2 BL/CE DL subframes.

- For the 14 HARQ processes feature, when PUCCH is used with 1 repetition and there is presence of non-BL/CE UL subframes (i.e., invalid UL subframes):

- The term surrounded by brackets in Solution 1 is resolved as 3 BL/CE UL subframes.

In this contribution, we discuss the remaining issues on 14-HARQ processes in DL for LTE-MTC.

Discussion

Percentage of presence of non-BL/CE subframes

The intention of introducing 14 HARQ processes is to achieve a target peak data rate. Ideally, the peak data rate of 14 HARQ processes is 705 kbps. However, the peak data rate of 14 HARQ processes (571 kbps) in the case of 20% presence of non-BL/CE subframes would be similar to the ideal peak data of 10 HARQ processes (588 kbps). When the percentage of presence of non-BL/CE subframes is larger than 20%, the peak data rate of 14 HARQ processes is much lower than the target peak data. There is no need to enable 14 HARQ processes for the scenario that percentage of presence of non-BL/CE subframes is larger than 20%.

Proposal 1: 14 HARQ processes is not enabled for the scenario that percentage of presence of non-BL/CE subframes is larger than 20%.

HARQ-ACK feedback delay

HARQ-ACK feedback delay may be different for different percentage of presence of Dl/UL non-BL/CE subframes.

- Percentage of presence of DL/UL non-BL/CE subframes is 0

When PDSCH scheduling delay is 2, the legacy HARQ-ACK delay value, i.e., 4~11, can be reused. When PDSCH scheduling delay is 7, the HARQ-ACK delay value is 12, 13.

- Percentage of presence of DL non-BL/CE subframes is 20%, percentage of presence of UL non-BL/CE subframes is 0

As shown in Figure 1, the distribution of DL non-BL/CE subframes would have impact on the specific HARQ-ACK delay value. The corresponding HARQ-ACK delay value range is 4~15 for PDSCH scheduling delay of 2 while the corresponding HARQ-ACK delay value range is 12~17 for PDSCH scheduling delay of 7.

Figure 1

- Percentage of presence of DL/UL non-BL/CE subframes is 20%

As shown in Figure 2, the HARQ-ACK delay, the maximum HARQ-ACK delay value would be 17 for PDSCH scheduling delay of 2 while the maximum HARQ-ACK delay value would be 19 for PDSCH scheduling delay of 7.

Figure 2

Observation 1: Percentage of presence of DL non-BL/CE subframes would have impact on the value range of HARQ-ACK delay.

According to above analysis, the corresponding HARQ-ACK delay value range is 4~17 for PDSCH scheduling delay of 2. For PDSCH scheduling delay of 7, the corresponding HARQ-ACK delay value range is 12~19.

Proposal 2: For 14 HARQ processes, the set of HARQ-ACK delay values is defined according to the PDSCH scheduling delay.

Details of signaling

Based on above analysis, the PDSCH scheduling delay and HARQ-ACK feedback delay should be indicated by DCI. From the perspective of signaling overhead, the joint indication is preferred. From the perspective of peak data rate, feature of 14-HARQ processes is not enabled in the case of PDSCH repetitions. Therefore, the repetition number field can be repurposed when 14-HARQ processes is enabled. The PDSCH scheduling delay and HARQ-ACK feedback delay can be jointly indicated by repetition number field and HARQ-ACK delay field.

Observation 2: For 14-HARQ processes, the ‘repetition number’ field in DCI can be repurposed since feature of 14-HARQ processes is not enabled in the case of PDSCH repetitions.

As agreed in last meeting, the PDSCH scheduling delay can be 2 BL/CE DL subframes, 7 (Case 1: 1 BL/CE DL subframe + 1 subframe + 3 BL/CE UL subframes + 1 subframe + 1 BL/CE DL subframe) and 7 (Case 2: 1 subframe + 3 BL/CE UL subframes + 1 subframe + 2 BL/CE DL subframes). One signaling example is shown in Table 1.

Table 1 One example of joint indication

| 'Repetition number' field in DCI | 'HARQ-ACK delay' field in DCI | Scheduling delay for PDSCH | HARQ-ACK delay value when ce-pdsch-fourteenProcesses’ is set |
| --- | --- | --- | --- |
| 00 | 000 | 2 | 4 |
|  | 001 | 2 | 5 |
|  | 010 | 2 | 6 |
|  | 011 | 2 | 7 |
|  | 100 | 2 | 8 |
|  | 101 | 2 | 9 |
|  | 110 | 2 | 10 |
|  | 111 | 2 | 11 |
| 01 | 000 | 2 | 12 |
|  | 001 | 2 | 13 |
|  | 010 | 2 | 14 |
|  | 011 | 2 | 15 |
|  | 100 | 2 | 16 |
|  | 101 | 2 | 17 |
|  | 110 | 7 (Case 1) | 12 |
|  | 111 | 7 (Case 2) | 12 |
| 10 | 000 | 7 (Case 1) | 13 |
|  | 001 | 7 (Case 2) | 13 |
|  | 010 | 7 (Case 1) | 14 |
|  | 011 | 7 (Case 2) | 14 |
|  | 100 | 7 (Case 1) | 15 |
|  | 101 | 7 (Case 2) | 15 |
|  | 110 | 7 (Case 1) | 16 |
|  | 111 | 7 (Case 2) | 16 |
| 11 | 000 | 7 (Case 1) | 17 |
|  | 001 | 7 (Case 2) | 17 |
|  | 010 | 7 (Case 1) | 18 |
|  | 011 | 7 (Case 2) | 18 |
|  | 100 | 7 (Case 1) | 19 |
|  | 101 | 7 (Case 2) | 19 |
|  | 110~111 | Reserved |  |

Proposal 3: The PDSCH scheduling delay and HARQ-ACK feedback delay can be jointly indicated by repetition number field and HARQ-ACK delay field.

PUCCH repetition

The main purpose of introducing 14-HARQ processes is to improve peak data rate of UE. That means the UE supporting 14-HARQ processes should be a high SNR UE and the UE does not need coverage and performance enhancement. Therefore, we propose that PUCCH repetition does not need to be considered if feature of 14-HARQ processes is enabled.

Proposal 4: For 14-HARQ processes, there is no need to consider the case of PUCCH repetition.

Conclusions

In this contribution, we have discussed the additional PDSCH scheduling delay and related HARQ-ACK delay to support DL 14-HARQ processes for H-FDD LTE-MTC. We make the following observations and proposals:

Observation 1: Percentage of presence of DL non-BL/CE subframes would have impact on the value range of HARQ-ACK delay.

Observation 2: For 14-HARQ processes, the ‘repetition number’ field in DCI can be repurposed since feature of 14-HARQ processes is not enabled in the case of PDSCH repetitions.

Proposal 1: 14 HARQ processes is not enabled for the scenario that percentage of presence of non-BL/CE subframes is larger than 20%.

Proposal 2: For 14 HARQ processes, the set of HARQ-ACK delay values is defined according to the PDSCH scheduling delay.

Proposal 3: The PDSCH scheduling delay and HARQ-ACK feedback delay can be jointly indicated by repetition number field and HARQ-ACK delay field.

Proposal 4: For 14-HARQ processes, there is no need to consider the case of PUCCH repetition.

- References

- 3GPP, RAN1 Chairman’s Notes, RAN1 #103-e

- 3GPP, RAN1 Chairman’s Notes, RAN1 #104-e