3GPP TSG RAN WG1 #104b-e                                    R1-2102916

e-Meeting, April 12th – 20th, 2021

Source:	ZTE

Title:	Discussion on the scenarios and assumption for IoT-NTN

Agenda Item:	8.15.1

Document for:   Discussion

- Introduction

In RAN1#104e meeting, agreements on detailed assumption for satellite and UE including typical configuration for link budget were made in [1], some of them are highlighted as following:

Agreement:

The following assumptions are agreed for a common set of link budget parameters:

- UE power class (PC5=20 dBm)

- UE Noise Figure (NF=9 dB)

- Other losses

| Other Losses | GEO (35786 km) | LEO (1200 km) | LEO (600 km) |
| --- | --- | --- | --- |
| Scintillation losses | 2.2 | 2.2 | 2.2 |
| Atmospheric losses | 0.2 | 0.1 | 0.1 |
| Polarization loss | 3 | 3 | 3 |
| Shadow margin | 3 | 3 | 3 |

NOTE 1: With PC3 (23 dBm) there is a 3dB gain compared to the PC5 (20 dBm) assumption on UL.

NOTE 2: With NF=7 dB, there is a 2 dB improvement compare to NF=9 dB on DL.

NOTE 3: Link budgets with other link budget parameters are not excluded from being captured in the TR.

NOTE 4: These parameters are only for the purpose of link budget calculations.

NOTE 5: Atmospheric losses are a function of elevation angle.

Agreement:

Link budget analysis assumes 3 dB polarization loss for DL and 3 dB polarization loss on UL for satellite parameters Set 1, Set 2, Set 3, and Set 4

In this contribution, analysis of link budget results and evaluation on the overall service condition for IoT-NTN according to the assumptions above is elaborated.

- Discussion on the coverage/link budget for IoT-NTN

For the link budget analysis, the central beam edge elevation is also used in the calculation for CNR. It should be noted that the signal from the satellite to the UE at the beam edge cannot get the maximum antenna gain of the satellite since the boresight of satellite’s antenna aims at the beam center. In general, it will lead to 3 dB loss comparing to the UE located at beam center.

In addition, for the evaluation in NR-NTN, FRF=1, 3 and FRF=2 with 2 different polarization types are considered to evaluate the impact of interference. Similarly, in IoT-NTN, FRF can be also considered, by which inter-beam interference can be alleviated. In our contribution, FRF=1 and FRF=3 are preferred for simplicity. It should be noticed that for Set-4, beam layout with multiple beams per satellite is not feasible and no CIR is needed to be considered unless multiple satellite based assumption is preferred. Then, it’s prefer to capture the link budget results for all cases listed in Table 1 with corresponding results in Table 5~Table 12.

Table 1 Cases for link budget analysis

| Case | Satellite orbit | Satellite parameter set | Central beam center elevation (deg) | Central beam edge elevation (deg) | Frequency Reuse Factor |
| --- | --- | --- | --- | --- | --- |
| 1 | GEO | Set 1 | 12.5 | 10 | 1 |
| 2 | GEO | Set 1 | 12.5 | 10 | 3 |
| 3 | LEO-600 | Set 1 | 30 | 26.98 | 1 |
| 4 | LEO-600 | Set 1 | 30 | 26.98 | 3 |
| 5 | LEO-1200 | Set 1 | 30 | 26.27 | 1 |
| 6 | LEO-1200 | Set 1 | 30 | 26.27 | 3 |
| 7 | GEO | Set 2 | 20 | 10.95 | 1 |
| 8 | GEO | Set 2 | 20 | 10.95 | 3 |
| 9 | LEO-600 | Set 2 | 30 | 23.80 | 1 |
| 10 | LEO-600 | Set 2 | 30 | 23.80 | 3 |
| 11 | LEO-1200 | Set 2 | 30 | 22.16 | 1 |
| 12 | LEO-1200 | Set 2 | 30 | 22.16 | 3 |
| 13 | GEO | Set 3 | 20.88 | 12.5 | 1 |
| 14 | GEO | Set 3 | 20.88 | 12.5 | 3 |
| 15 | LEO-600 | Set 3 | 43.78 | 30 | 1 |
| 16 | LEO-600 | Set 3 | 43.78 | 30 | 3 |
| 17 | LEO-1200 | Set 3 | 46.05 | 30 | 1 |
| 18 | LEO-1200 | Set 3 | 46.05 | 30 | 3 |
| 19 | LEO-600 | Set 4 | 90 | 30 | 1 |

Proposal 1: Cases listed in Table-1 within consideration on the different FR factor should be considered for link budget evaluation.

Proposal 2: Capturing the link budget results for cases listed in Table-1 into the TR.

- Coupling loss analysis based on agreed assumption

According to the aforementioned assumptions, the coupling loss is provided in Table 2. In all the cases, the coupling loss would be less than 164 dB, but in some cases of Set-3 LEO-1200 and Set-4 LEO-600, the coupling loss would be larger than 159 dB.

Observation 1: For Set-3 and Set-4, coupling loss of LOS UE in some cases will be larger than 159 dB.

Table 2 Coupling loss of edge UE

|  |  | GEO | LEO-600 | LEO-1200 |
| --- | --- | --- | --- | --- |
| Set-1 | Coupling loss (dB) | 151.04 | 140.99 | 146.39 |
| Set-2 | Coupling loss (dB) | 156.50 | 147.71 | 153.15 |
| Set-3 | Coupling loss (dB) | 156.24 | 154.16 | 159.55 |
| Set-4 | Coupling loss (dB) |  | 159.38 |  |

- CNR analysis based on agreed assumption

For performance comparing, the target SINR for NB-IoT/eMTC in terrestrial network are taken as the baseline. For example:

- W.r.t the DL:

- For NB-IoT, -12.6 dB, which corresponds the maximal MCL in standalone mode is selected.

- For eMTC, the target SNRs for PDSCH as -19.3 dB (refer to Table 9.5.6.1-2 in [2]) is selected.

- W.r.t the UL:

- For NB-IoT, for 3.75 kHz, 15 kHz, 45 kHz, 90 kHz, and 180 kHz bandwidth, the target SNRs , i.e., -5.6 dB, -11.8 dB, -16.5 dB, -19.5 dB, -22.6 dB are selected, respectively;

- For eMTC, for 30 kHz, 45 kHz, 90 kHz, 180 kHz, 360 kHz and 1080 kHz bandwidth, the target SNRs for PUSCH as -13.50 dB, -15.27 dB, -18.28 dB, -21.29 dB, -24.3 dB, -29.07 dB (refer to Table 9.5.7.1-3 in [2]) are selected, respectively.

It can be found that, for downlink, in the case of Set-2 GEO and Set-4 LEO-600, the CNR (yellow marked in Table 3) would be worse than -12.6 dB. And for uplink, in the case of Set-2 GEO, the CNR (red marked in Table 4) is worse than the target SNRs for eMTC. Furthermore, in the case of Set-3 LEO-1200 and Set-4 LEO-600, the CNR (grey marked in Table 4) is even worse than the target SNRs for both NB-IoT and eMTC.

Table 3 DL CNR for NB-IoT/eMTC

|  |  | GEO | LEO-600 | LEO-1200 |
| --- | --- | --- | --- | --- |
| Set-1 | Minimum DL CNR (dB) | -8.06 | -2.02 | -1.41 |
| Set-2 | Minimum DL CNR (dB) | -13.52 | -8.73 | -8.17 |
| Set-3 | Minimum DL CNR (dB) | -7.17 | -7.08 | -7.08 |
| Set-4 | Minimum DL CNR (dB) |  | -13.95 |  |

Table 4 UL CNR for NB-IoT/eMTC

|  | Bandwidth | GEO | LEO-600 | LEO-1200 |
| --- | --- | --- | --- | --- |
| Set-1 | 3.75 kHz 15 kHz 30 kHz 45 kHz 90 kHz 180 kHz 360 kHz 1080 kHz | -0.18 -6.20 -9.21 -10.97 -13.98 -16.99 -20.00 -24.77 | 12.97 6.95 3.94 2.17 -0.84 -3.85 -6.86 -11.63 | 7.57 1.55 -1.46 -3.22 -6.23 -9.24 -12.25 -17.03 |
| Set-2 | 3.75 kHz 15 kHz 30 kHz 45 kHz 90 kHz 180 kHz 360 kHz 1080 kHz | -5.14 -11.16 -14.17 -15.93 -18.94 -21.95 -24.96 -29.73 | 6.25 0.23 -2.78 -4.54 -7.55 -10.56 -13.57 -18.34 | 0.81 -5.21 -8.22 -9.98 -12.99 -16.00 -19.01 -23.78 |
| Set-3 | 3.75 kHz 15 kHz 30 kHz 45 kHz 90 kHz 180 kHz 360 kHz 1080 kHz | -2.38 -8.40 -11.41 -13.17 -16.18 -19.20 -22.21 -26.98 | -0.30 -6.32 -9.33 -11.09 -14.10 -17.11 -20.12 -24.89 | -5.69 -11.71 -14.72 -16.48 -19.49 -22.50 -25.52 -30.29 |
| Set-4 | 3.75 kHz 15 kHz 30 kHz 45 kHz 90 kHz 180 kHz 360 kHz 1080 kHz |  | -6.12 -12.14 -15.15 -16.91 -19.92 -22.93 -25.94 -30.71 |  |

Observation 2: In some cases for Set-2, Set-3, and Set-4, even the coupling loss is smaller than 164 dB for NB-IoT and 159 dB for eMTC, the CNR is worse than the target SNR.

- Distribution of CL all UEs within the coverage of serving satellite

In order to have an overall insight on the performance of IoT over NTN, the evaluation on the CL for all UEs within the coverage of serving satellite based on all parameter sets according to the parameters in rural, urban, dense urban scenarios with the LoS probability of the UEs assumed according to the Table 6.6.1-1 in [3].

In terrestrial network, the MCL (Maximum Coupling Loss) requirements is set as 164 dB and 159 dB for NB-IoT and eMTC, respectively. As shown in Figure 1~ Figure 9 below, it can be found that:

- In rural scenario, the CLs of about 5%~10% GEO UEs are larger than 164 dB, and the CLs of about 5% LEO-600 and LEO-1200 UEs are larger than 164 dB;

- In urban scenario, the CLs of about 50% GEO UEs are larger than 164 dB, about 30% LEO-600 and LEO-1200 UEs are larger than 164 dB;

- In dense urban scenario, the CLs of about 50% GEO UEs are larger than 164 dB, the CLs of about 30%~40% LEO-600 and LEO-1200 UEs are larger than 164 dB.

| Figure 1 Illustration of DL CL for GEO in rural | Figure 2 Illustration of DL CL for LEO-600 in rural |
| --- | --- |
| Figure 3 Illustration of DL CL for LEO-1200 in rural |  |
| Figure 4 Illustration of DL CL for GEO in ubran | Figure 5 Illustration of DL CL for LEO-600 in urban |
| Figure 6 Illustration of DL CL for LEO-1200 in urban |  |
| Figure 7 Illustration of DL CL for GEO in Dense urban | Figure 8 Illustration of DL CL for LEO-600 in Dense urban |
| Figure 9 Illustration of DL CL for LEO-1200 in Dense urban |  |

The reason for different performances per scenario is that the probability of LoS/NLoS varies over scenarios. It can be found that even for rural scenario, a number of NLoS UEs will be failed to be supported by existing design.

Observation 3: A large number of UEs would experience a worse coupling loss larger than 164 dB for urban and dense urban scenarios. And even for rural scenario, there are about 5% UEs which experience coupling loss larger than 164 dB.

Proposal 3: Further enhancement on the transmission may be needed to support cases with large coupling loss and/or low CNR.

- Conclusions

In this contribution, detailed link budget results are provided and discussed with the following observations and proposals:

Observation 1: For Set-3 and Set-4, coupling loss of LOS UE in some cases will be larger than 159 dB.

Observation 2: In some cases for Set-2, Set-3, and Set-4, even the coupling loss is smaller than 164 dB for NB-IoT and 159 dB for eMTC, the CNR is worse than the target SNR.

Observation 3: A large number of UEs would experience a worse coupling loss larger than 164 dB for urban and dense urban scenarios. And even for rural scenario, there are about 5% UEs which experience coupling loss larger than 164 dB.

Proposal 1: Cases listed in Table-1 within consideration on the different FR factor should be considered for link budget evaluation.

Proposal 2: Capturing the link budget results for cases listed in Table-1 into the TR.

Proposal 3: Further enhancement on the transmission may be needed to support cases with large coupling loss and/or low CNR.

# References

- Chairman’s notes, RAN1#104e meeting.

- TR36.888-c00, Study on provision of low-cost Machine-Type Communications (MTC) User Equipment (UEs) based on LTE (Release 12).

- TR38.811-f20, Study on New Radio (NR) to support non-terrestrial networks (Release 15).

# Appendix

Table 5 Link budget results for DL with Set-1

|  | GEO |  | LEO-600 |  | LEO-1200 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency reuse factor | 1 | 3 | 1 | 3 | 1 | 3 |
| Central beam center elevation (deg) | 12.5 | 12.5 | 30 | 30 | 30 | 30 |
| Central beam edge elevation (deg) | 10 | 10 | 26.9845 | 26.9845 | 26.2709 | 26.2709 |
| Central beam edge satellite-UE distance(km) | 40581.2 | 40581.2 | 1154.2 | 1154.2 | 2148.2 | 2148.2 |
| EIRP density (dBW/MHz) | 59 | 59 | 34 | 34 | 40 | 40 |
| Max satellite antenna gain (dBi) | 51 | 51 | 30 | 30 | 30 | 30 |
| Antenna loss at edge (dB) | 3 | 3 | 3 | 3 | 3 | 3 |
| Satellite Tx power (dBm) 180 kHz 1080 kHz | 78.55 86.33 | 78.55 86.33 | 53.55 61.33 | 53.55 61.33 | 59.55 67.33 | 59.55 67.33 |
| Max UE antenna gain (dB) | 0 | 0 | 0 | 0 | 0 | 0 |
| UE noise figure (dB) | 9 | 9 | 9 | 9 | 9 | 9 |
| UE antenna G/T (dB/K) | -33.62 | -33.62 | -33.62 | -33.62 | -33.62 | -33.62 |
| Free space path loss (dB) | 190.64 | 190.64 | 159.72 | 159.72 | 165.11 | 165.11 |
| Atmosphere Loss(clear sky) (dB) | 0.20 | 0.20 | 0.08 | 0.08 | 0.08 | 0.08 |
| Ionospheric Scintilation Loss (dB) | 2.20 | 2.20 | 2.20 | 2.20 | 2.20 | 2.20 |
| Troposhperic loss (dB) | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Shadowing Margin (dB) | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 |
| Additional Loss (dB) | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Polarization loss (dB) | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 |
| Total loss (dB) | 199.04 | 199.04 | 167.99 | 167.99 | 173.39 | 173.39 |
| Coupling loss (dB) | 151.04 | 151.04 | 140.99 | 140.99 | 146.39 | 146.39 |
| CNR (dB) | -8.06 | -8.06 | -2.02 | -2.02 | -1.42 | -1.42 |
| CIR (dB) | 4.99 | 13.87 | -0.64 | 10.64 | -0.64 | 10.64 |
| CINR (dB) | -8.27 | -8.09 | -4.39 | -2.25 | -4.06 | -1.68 |
| DL repetition gain (dB) (2048 repetition times) | 26.34 | 26.34 | 26.34 | 26.34 | 26.34 | 26.34 |
| CINR-repetition (dB) | 18.07 | 18.25 | 21.95 | 24.09 | 22.28 | 24.66 |

Table 6 Link budget results for UL with Set-1

|  | GEO |  | LEO-600 |  | LEO-1200 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Frequenct reuse factor | 1 | 3 | 1 | 3 | 1 | 3 |
| Central beam center elevation (deg) | 12.5 | 12.5 | 30 | 30 | 30 | 30 |
| Central beam edge elevation (deg) | 10 | 10 | 26.9845 | 26.9845 | 26.2709 | 26.2709 |
| Central beam edge satellite-UE distance(km) | 40581.2 | 40581.2 | 1154.2 | 1154.2 | 2148.2 | 2148.2 |
| Max satellite antenna gain (dBi) | 51 | 51 | 30 | 30 | 30 | 30 |
| Antenna loss at edge (dB) | 3 | 3 | 3 | 3 | 3 | 3 |
| satellite antenna G/T (dB/K) | 19 | 19 | 1.1 | 1.1 | 1.1 | 1.1 |
| Max UE antenna gain (dB) | 0 | 0 | 0 | 0 | 0 | 0 |
| UE transmission power (dBm) | 20 | 20 | 20 | 20 | 20 | 20 |
| Free space path loss (dB) | 190.64 | 190.64 | 159.72 | 159.72 | 165.11 | 165.11 |
| Atmosphere Loss(clear sky) (dB) | 0.20 | 0.20 | 0.08 | 0.08 | 0.08 | 0.08 |
| Ionospheric Scintilation Loss (dB) | 2.20 | 2.20 | 2.20 | 2.20 | 2.20 | 2.20 |
| Troposhperic loss (dB) | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Shadowing Margin (dB) | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 |
| Additional Loss (dB) | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Polarization loss (dB) | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 |
| Total loss (dB) | 199.04 | 199.04 | 167.99 | 167.99 | 173.39 | 173.39 |
| Coupling loss (dB) | 151.04 | 151.04 | 140.99 | 140.99 | 146.39 | 146.39 |
| CNR (dB) 3.75 kHz (Tx 20 dBm) 15 kHz (Tx 20 dBm) 30 kHz (Tx 20 dBm) 45 kHz (Tx 20 dBm) 90 kHz (Tx 20 dBm) 180 kHz (Tx 20 dBm) 360 kHz (Tx 20 dBm) 1080 kHz (Tx 20 dBm) | -0.18 -6.20 -9.21 -10.97 -13.98 -16.99 -20.00 -24.77 | -0.18 -6.20 -9.21 -10.97 -13.98 -16.99 -20.00 -24.77 | 12.97 6.95 3.94 2.17 -0.84 -3.85 -6.86 -11.63 | 12.97 6.95 3.94 2.17 -0.84 -3.85 -6.86 -11.63 | 7.57 1.55 -1.46 -3.22 -6.23 -9.24 -12.25 -17.03 | 7.57 1.55 -1.46 -3.22 -6.23 -9.24 -12.25 -17.03 |
| CIR (dB) | 4.62 | 14.00 | -0.26 | 11.72 | -0.24 | 11.81 |
| CINR (dB) 3.75 kHz (Tx 20 dBm) 15 kHz (Tx 20 dBm) 30 kHz (Tx 20 dBm) 45 kHz (Tx 20 dBm) 90 kHz (Tx 20 dBm) 180 kHz (Tx 20 dBm) 360 kHz (Tx 20 dBm) 1080 kHz (Tx 20 dBm) | -1.42 -6.55 -9.39 -11.09 -14.04 -17.02 -20.02 -24.78 | -0.34 -6.24 -9.23 -10.99 -13.99 -17.00 -20.00 -24.77 | -0.46 -1.02 -1.66 -2.22 -3.57 -5.42 -7.72 -11.93 | 9.29 5.70 3.27 1.72 -1.07 -3.96 -6.92 -11.65 | -0.91 -2.45 -3.90 -4.99 -7.21 -9.76 -12.52 -17.12 | 6.18 1.16 -1.66 -3.36 -6.30 -9.28 -12.27 -17.03 |
| UL repetition gain (dB) (128 repetition times) | 16.84 | 16.84 | 16.84 | 16.84 | 16.84 | 16.84 |
| CINR-repetition (dB) 3.75 kHz (Tx 20 dBm) 15 kHz (Tx 20 dBm) 30 kHz (Tx 20 dBm) 45 kHz (Tx 20 dBm) 90 kHz (Tx 20 dBm) 180 kHz (Tx 20 dBm) 360 kHz (Tx 20 dBm) 1080 kHz (Tx 20 dBm) | 15.42 10.29 7.45 5.75 2.80 -0.18 -3.18 -7.94 | 16.50 10.60 7.61 5.85 2.85 -0.16 -3.16 -7.93 | 16.38 15.82 15.18 14.62 13.27 11.42 9.12 4.91 | 26.13 22.54 20.11 18.56 15.77 12.88 9.92 5.19 | 15.93 14.39 12.94 11.85 9.63 7.08 4.32 -0.28 | 23.02 18.00 15.18 13.48 10.54 7.56 4.57 -0.19 |

Table 7 Link budget results for DL with Set-2

|  | GEO |  | LEO-600 |  | LEO-1200 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency reuse factor | 1 | 3 | 1 | 3 | 1 | 3 |
| Central beam center elevation (deg) | 20 | 20 | 30 | 30 | 30 | 30 |
| Central beam edge elevation (deg) | 10.9499 | 10.9499 | 23.8019 | 23.8019 | 22.1635 | 22.1635 |
| Central beam edge satellite-UE distance(km) | 40480.2 | 40480.2 | 1251.9 | 1251.9 | 2340.7 | 2340.7 |
| EIRP density (dBW/MHz) | 53.5 | 53.5 | 28 | 28 | 34 | 34 |
| Max satellite antenna gain (dBi) | 45.50 | 45.50 | 24 | 24 | 24 | 24 |
| Antenna loss at edge (dB) | 3 | 3 | 3 | 3 | 3 | 3 |
| Satellite Tx power (dBm) 180 kHz 1080 kHz | 73.05 80.83 | 73.05 80.83 | 47.55 55.33 | 47.55 55.33 | 53.55 61.33 | 53.55 61.33 |
| Max UE antenna gain (dB) | 0 | 0 | 0 | 0 | 0 | 0 |
| UE noise figure (dB) | 9 | 9 | 9 | 9 | 9 | 9 |
| UE antenna G/T (dB/K) | -33.62 | -33.62 | -33.62 | -33.62 | -33.62 | -33.62 |
| Free space path loss (dB) | 190.62 | 190.62 | 160.42 | 160.42 | 165.86 | 165.86 |
| Atmosphere Loss(clear sky) (dB) | 0.19 | 0.19 | 0.09 | 0.09 | 0.09 | 0.09 |
| Ionospheric Scintilation Loss (dB) | 2.20 | 2.20 | 2.20 | 2.20 | 2.20 | 2.20 |
| Troposhperic loss (dB) | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Shadowing Margin (dB) | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 |
| Additional Loss (dB) | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Polarization loss (dB) | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 |
| Total loss (dB) | 199.00 | 199.00 | 168.71 | 168.71 | 174.15 | 174.15 |
| Coupling loss (dB) | 156.50 | 156.50 | 147.71 | 147.71 | 153.15 | 153.15 |
| CNR (dB) | -13.52 | -13.52 | -8.73 | -8.73 | -8.17 | -8.17 |
| CIR (dB) | 1.48 | 13.90 | -0.50 | 11.38 | 1.45 | 13.87 |
| CINR (dB) | -13.66 | -13.53 | -9.34 | -8.77 | -8.62 | -8.20 |
| DL repetition gain (dB) (2048 repetition times) | 26.34 | 26.34 | 26.34 | 26.34 | 26.34 | 26.34 |
| CINR-repetition (dB) | 12.68 | 12.81 | 17.00 | 17.57 | 17.72 | 18.14 |

Table 8 Link budget results for UL with Set-2

|  | GEO |  | LEO-600 |  | LEO-1200 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Frequenct reuse factor | 1 | 3 | 1 | 3 | 1 | 3 |
| Central beam center elevation (deg) | 20 | 20 | 30 | 30 | 30 | 30 |
| Central beam edge elevation (deg) | 10.9499 | 10.9499 | 23.8019 | 23.8019 | 22.1635 | 22.1635 |
| Central beam edge satellite-UE distance(km) | 40480.2 | 40480.2 | 1251.9 | 1251.9 | 2340.7 | 2340.7 |
| Max satellite antenna gain (dBi) | 45.50 | 45.50 | 24 | 24 | 24 | 24 |
| Antenna loss at edge (dB) | 3 | 3 | 3 | 3 | 3 | 3 |
| satellite antenna G/T (dB/K) | 14 | 14 | -4.9 | -4.9 | -4.9 | -4.9 |
| Max UE antenna gain (dB) | 0 | 0 | 0 | 0 | 0 | 0 |
| UE transmission power (dBm) | 20 | 20 | 20 | 20 | 20 | 20 |
| Free space path loss (dB) | 190.62 | 190.62 | 160.42 | 160.42 | 165.86 | 165.86 |
| Atmosphere Loss(clear sky) (dB) | 0.19 | 0.19 | 0.09 | 0.09 | 0.09 | 0.09 |
| Ionospheric Scintilation Loss (dB) | 2.20 | 2.20 | 2.20 | 2.20 | 2.20 | 2.20 |
| Troposhperic loss (dB) | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Shadowing Margin (dB) | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 |
| Additional Loss (dB) | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Polarization loss (dB) | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 |
| Total loss (dB) | 199.00 | 199.00 | 168.71 | 168.71 | 174.15 | 174.15 |
| Coupling loss (dB) | 156.50 | 156.50 | 147.71 | 147.71 | 153.15 | 153.15 |
| CNR (dB) 3.75 kHz (Tx 20 dBm) 15 kHz (Tx 20 dBm) 30 kHz (Tx 20 dBm) 45 kHz (Tx 20 dBm) 90 kHz (Tx 20 dBm) 180 kHz (Tx 20 dBm) 360 kHz (Tx 20 dBm) 1080 kHz (Tx 20 dBm) | -5.14 -11.16 -14.17 -15.93 -18.94 -21.95 -24.96 -29.73 | -5.14 -11.16 -14.17 -15.93 -18.94 -21.95 -24.96 -29.73 | 6.25 0.23 -2.78 -4.54 -7.55 -10.56 -13.57 -18.34 | 6.25 0.23 -2.78 -4.54 -7.55 -10.56 -13.57 -18.34 | 0.81 -5.21 -8.22 -9.98 -12.99 -16.00 -19.01 -23.78 | 0.81 -5.21 -8.22 -9.98 -12.99 -16.00 -19.01 -23.78 |
| CIR (dB) | 2.36 | 14.10 | -0.14 | 11.75 | 1.75 | 12.68 |
| CINR (dB) 3.75 kHz (Tx 20 dBm) 15 kHz (Tx 20 dBm) 30 kHz (Tx 20 dBm) 45 kHz (Tx 20 dBm) 90 kHz (Tx 20 dBm) 180 kHz (Tx 20 dBm) 360 kHz (Tx 20 dBm) 1080 kHz (Tx 20 dBm) | -5.85 -11.35 -14.27 -16.00 -18.98 -21.97 -24.97 -29.74 | -5.19 -11.17 -14.18 -15.94 -18.95 -21.95 -24.96 -29.74 | -1.04 -2.97 -4.67 -5.89 -8.28 -10.94 -13.76 -18.41 | 5.17 -0.07 -2.93 -4.64 -7.60 -10.59 -13.58 -18.35 | -1.76 -6.01 -8.64 -10.26 -13.14 -16.08 -19.05 -23.80 | 0.54 -5.28 -8.26 -10.01 -13.00 -16.01 -19.02 -23.79 |
| UL repetition gain (dB) (128 repetition times) | 16.84 | 16.84 | 16.84 | 16.84 | 16.84 | 16.84 |
| CINR-repetition (dB) 3.75 kHz (Tx 20 dBm) 15 kHz (Tx 20 dBm) 30 kHz (Tx 20 dBm) 45 kHz (Tx 20 dBm) 90 kHz (Tx 20 dBm) 180 kHz (Tx 20 dBm) 360 kHz (Tx 20 dBm) 1080 kHz (Tx 20 dBm) | 10.99 5.49 2.57 0.84 -2.14 -5.13 -8.13 -12.90 | 11.65 5.67 2.66 0.90 -2.11 -5.11 -8.12 -12.90 | 15.80 13.87 12.17 10.95 8.56 5.90 3.08 -1.57 | 22.01 16.77 13.91 12.20 9.24 6.25 3.26 -1.51 | 15.08 10.83 8.20 6.58 3.70 0.76 -2.21 -6.96 | 17.38 11.56 8.58 6.83 3.84 0.83 -2.18 -6.95 |

Table 9 Link budget results for DL with Set-3

|  | GEO |  | LEO-600 |  | LEO-1200 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency reuse factor | 1 | 3 | 1 | 3 | 1 | 3 |
| Central beam center elevation (deg) | 20.9 | 20.9 | 46.05 | 46.05 | 43.8 | 43.8 |
| Central beam edge elevation (deg) | 12.5 | 12.5 | 30.0 | 30.0 | 30.0 | 30.0 |
| Central beam edge satellite-UE distance(km) | 40316.0 | 40316.0 | 1074.0 | 1074.0 | 1998.0 | 1998.0 |
| EIRP density (dBW/MHz) | 59.8 | 59.8 | 28.3 | 28.3 | 33.7 | 33.7 |
| Max satellite antenna gain (dBi) | 45.70 | 45.70 | 16.2 | 16.2 | 16.2 | 16.2 |
| Antenna loss at edge (dB) | 3 | 3 | 3 | 3 | 3 | 3 |
| Satellite Tx power (dBm) 180 kHz 1080 kHz | 79.35 87.13 | 79.35 87.13 | 47.85 55.63 | 47.85 55.63 | 53.25 61.03 | 53.25 61.03 |
| Max UE antenna gain (dB) | 0 | 0 | 0 | 0 | 0 | 0 |
| UE noise figure (dB) | 9 | 9 | 9 | 9 | 9 | 9 |
| UE antenna G/T (dB/K) | -33.62 | -33.62 | -33.62 | -33.62 | -33.62 | -33.62 |
| Free space path loss (dB) | 190.58 | 190.58 | 159.09 | 159.09 | 164.48 | 164.48 |
| Atmosphere Loss(clear sky) (dB) | 0.16 | 0.16 | 0.07 | 0.07 | 0.07 | 0.07 |
| Ionospheric Scintilation Loss (dB) | 2.20 | 2.20 | 2.20 | 2.20 | 2.20 | 2.20 |
| Troposhperic loss (dB) | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Shadowing Margin (dB) | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 |
| Additional Loss (dB) | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Polarization loss (dB) | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 |
| Total loss (dB) | 198.94 | 198.94 | 167.36 | 167.36 | 172.75 | 172.75 |
| Coupling loss (dB) | 156.24 | 156.24 | 154.16 | 154.16 | 159.55 | 159.55 |
| CNR (dB) | -7.17 | -7.17 | -7.08 | -7.08 | -7.08 | -7.08 |
| CIR (dB) | 1.48 | 13.90 | 1.13 | 13.86 | 1.38 | 13.87 |
| CINR (dB) | -7.72 | -7.20 | -7.69 | -7.12 | -7.66 | -7.11 |
| DL repetition gain (dB) (2048 repetition times) | 26.34 | 26.34 | 26.34 | 26.34 | 26.34 | 26.34 |
| CINR-repetition (dB) | 18.62 | 19.14 | 18.65 | 19.22 | 18.68 | 19.23 |

Table 10 Link budget results for UL with Set-3

|  | GEO |  | LEO-600 |  | LEO-1200 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Frequenct reuse factor | 1 | 3 | 1 | 3 | 1 | 3 |
| Central beam center elevation (deg) | 20.9 | 20.9 | 46.05 | 46.05 | 43.8 | 43.8 |
| Central beam edge elevation (deg) | 12.5 | 12.5 | 30.0 | 30.0 | 30.0 | 30.0 |
| Central beam edge satellite-UE distance(km) | 40316.0 | 40316.0 | 1074.0 | 1074.0 | 1998.0 | 1998.0 |
| Max satellite antenna gain (dBi) | 45.70 | 45.70 | 16.2 | 16.2 | 16.2 | 16.2 |
| Antenna loss at edge (dB) | 3 | 3 | 3 | 3 | 3 | 3 |
| satellite antenna G/T (dB/K) | 16.7 | 16.7 | -12.8 | -12.8 | -12.8 | -12.8 |
| Max UE antenna gain (dB) | 0 | 0 | 0 | 0 | 0 | 0 |
| UE transmission power (dBm) | 20 | 20 | 20 | 20 | 20 | 20 |
| Free space path loss (dB) | 190.58 | 190.58 | 159.09 | 159.09 | 164.48 | 164.48 |
| Atmosphere Loss(clear sky) (dB) | 0.16 | 0.16 | 0.07 | 0.07 | 0.07 | 0.07 |
| Ionospheric Scintilation Loss (dB) | 2.20 | 2.20 | 2.20 | 2.20 | 2.20 | 2.20 |
| Troposhperic loss (dB) | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Shadowing Margin (dB) | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 |
| Additional Loss (dB) | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Polarization loss (dB) | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 |
| Total loss (dB) | 198.94 | 198.94 | 167.36 | 167.36 | 172.75 | 172.75 |
| Coupling loss (dB) | 156.24 | 156.24 | 154.16 | 154.16 | 159.55 | 159.55 |
| CNR (dB) 3.75 kHz (Tx 20 dBm) 15 kHz (Tx 20 dBm) 30 kHz (Tx 20 dBm) 45 kHz (Tx 20 dBm) 90 kHz (Tx 20 dBm) 180 kHz (Tx 20 dBm) 360 kHz (Tx 20 dBm) 1080 kHz (Tx 20 dBm) | -2.38 -8.40 -11.41 -13.17 -16.18 -19.20 -22.21 -26.98 | -2.38 -8.40 -11.41 -13.17 -16.18 -19.20 -22.21 -26.98 | -0.30 -6.32 -9.33 -11.09 -14.10 -17.11 -20.12 -24.89 | -0.30 -6.32 -9.33 -11.09 -14.10 -17.11 -20.12 -24.89 | -5.69 -11.71 -14.72 -16.48 -19.49 -22.50 -25.52 -30.29 | -5.69 -11.71 -14.72 -16.48 -19.49 -22.50 -25.52 -30.29 |
| CIR (dB) | 2.34 | 14.44 | 2.39 | 12.44 | 1.33 | 12.62 |
| CINR (dB) 3.75 kHz (Tx 20 dBm) 15 kHz (Tx 20 dBm) 30 kHz (Tx 20 dBm) 45 kHz (Tx 20 dBm) 90 kHz (Tx 20 dBm) 180 kHz (Tx 20 dBm) 360 kHz (Tx 20 dBm) 1080 kHz (Tx 20 dBm) | -3.64 -8.75 -11.59 -13.29 -16.25 -19.23 -22.22 -26.98 | -2.47 -8.43 -11.42 -13.18 -16.19 -19.20 -22.21 -26.98 | -2.17 -6.87 -9.61 -11.28 -14.20 -17.16 -20.15 -24.90 | -0.53 -6.38 -9.36 -11.11 -14.11 -17.12 -20.13 -24.90 | -6.48 -11.92 -14.83 -16.56 -19.53 -22.52 -25.52 -30.29 | -5.76 -11.73 -14.73 -16.49 -19.50 -22.51 -25.52 -30.29 |
| UL repetition gain (dB) (128 repetition times) | 16.84 | 16.84 | 16.84 | 16.84 | 16.84 | 16.84 |
| CINR-repetition (dB) 3.75 kHz (Tx 20 dBm) 15 kHz (Tx 20 dBm) 30 kHz (Tx 20 dBm) 45 kHz (Tx 20 dBm) 90 kHz (Tx 20 dBm) 180 kHz (Tx 20 dBm) 360 kHz (Tx 20 dBm) 1080 kHz (Tx 20 dBm) | 13.20 8.09 5.25 3.55 0.59 -2.39 -5.38 -10.14 | 14.37 8.41 5.42 3.66 0.65 -2.36 -5.37 -10.14 | 14.67 9.97 7.23 5.56 2.64 -0.32 -3.31 -8.06 | 16.31 10.46 7.48 5.73 2.73 -0.28 -3.29 -8.06 | 10.36 4.92 2.01 0.28 -2.69 -5.68 -8.68 -13.45 | 11.08 5.11 2.11 0.35 -2.66 -5.67 -8.68 -13.45 |

Table 11 Link budget results for DL with Set-4

|  | LEO-600 |
| --- | --- |
| Frequency reuse factor | 1 |
| Central beam center elevation (deg) | 90 |
| Central beam edge elevation (deg) | 30 |
| Central beam edge satellite-UE distance(km) | 1076 |
| EIRP density (dBW/MHz) | 21.45 |
| Max satellite antenna gain (dBi) | 11 |
| Antenna loss at edge (dB) | 3 |
| Satellite Tx power (dBm) 180 kHz 1080 kHz | 41 48.78 |
| Max UE antenna gain (dB) | 0 |
| UE noise figure (dB) | 9 |
| UE antenna G/T (dB/K) | -33.62 |
| Free space path loss (dB) | 159.11 |
| Atmosphere Loss(clear sky) (dB) | 0.07 |
| Ionospheric Scintilation Loss (dB) | 2.20 |
| Troposhperic loss (dB) | 0.00 |
| Shadowing Margin (dB) | 3.00 |
| Additional Loss (dB) | 0.00 |
| Polarization loss (dB) | 3 |
| Total loss (dB) | 167.38 |
| Coupling loss (dB) | 159.38 |
| CNR (dB) | -13.95 |
| DL repetition gain (dB) (2048 repetition times) | 26.34 |
| CNR-repetition (dB) | 12.39 |

Table 12 Link budget results for UL with Set-4

|  | LEO-600 |
| --- | --- |
| Frequency reuse factor | 1 |
| Center elevation (deg) | 90 |
| Edge elevation (deg) | 30 |
| Path length | 1076 |
| Max satellite antenna gain (dBi) | 11 |
| Antenna loss at edge (dB) | 3 |
| satellite antenna G/T (dB/K) | -18.6 |
| Max UE antenna gain (dB) | 0 |
| UE transmission power (dBm) | 20 |
| Free space path loss (dB) | 159.11 |
| Atmosphere Loss(clear sky) (dB) | 0.07 |
| Ionospheric Scintilation Loss (dB) | 2.20 |
| Troposhperic loss (dB) | 0.00 |
| Shadowing Margin (dB) | 3.00 |
| Additional Loss (dB) | 0.00 |
| Polarization loss (dB) | 3 |
| Total loss (dB) | 167.38 |
| Coupling loss (dB) | 159.38 |
| CNR (dB) 3.75 kHz (Tx 20 dBm) 15 kHz (Tx 20 dBm) 30 kHz (Tx 20 dBm) 45 kHz (Tx 20 dBm) 90 kHz (Tx 20 dBm) 180 kHz (Tx 20 dBm) 360 kHz (Tx 20 dBm) 1080 kHz (Tx 20 dBm) | -6.12 -12.14 -15.15 -16.91 -19.92 -22.93 -25.94 -30.71 |
| DL repetition gain (dB) (128 repetition times) | 16.84 |
| CNR-repetition (dB) 3.75 kHz (Tx 20 dBm) 15 kHz (Tx 20 dBm) 30 kHz (Tx 20 dBm) 45 kHz (Tx 20 dBm) 90 kHz (Tx 20 dBm) 180 kHz (Tx 20 dBm) 360 kHz (Tx 20 dBm) 1080 kHz (Tx 20 dBm) | 10.72 4.70 1.69 -0.07 -3.08 -6.09 -9.10 -13.87 |