3GPP TSG RAN WG1 104b-e                                                               R1-2103033

e-Meeting, 12th – 20th April, 2021

Source: 	Intel Corporation

Title:	On UL time and frequency synchronization for NTN

Agenda item:	8.4.2

Document for:	Discussion and Decision

# Introduction

At the 3GPP TSG RAN meeting #86 the new work item (WI) on solutions for NR to support non-terrestrial networks (NTN) was approved in [1]. The NR NTN WI aims to specify the enhancements identified for NR NTN especially LEO and GEO with implicit compatibility to support HAPS (high altitude platform station) and ATG (air to ground) scenarios according to the following principles:

- FDD is assumed for core specification work for NR-NTN.

  - NOTE: This does not imply that TDD cannot be used for relevant scenarios e.g. HAPS, ATG

- Earth fixed Tracking area is assumed with Earth fixed and moving cells

- UEs with GNSS capabilities are assumed.

According to the 3GPP TR 38.821 [2] changes to NR specification are required to support NTN due to long propagation delays, Doppler effects, and moving cells. One of the main challenges for NTN to operate is UL time and frequency synchronization. In this contribution aspects related to UL time and frequency synchronization are discussed considering principles identified in [1].

# Discussion

At the previous RAN1 meetings [3, 4] it was agreed that NTN UE is required to support UE specific TA calculation and frequency pre-compensation based on its GNSS-acquired position and the serving satellite ephemeris in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED states. Thus, it is assumed that UE applies UE-specific autonomous TA and frequency offset pre-compensation for UL transmission to compensate delay and Doppler for the link between the satellite and the UE (service link).

For transparent payload case where gNB is located in the gateway, UL signal received at the gNB has time and frequency offset which is common for all UEs corresponding to the delay and Doppler in the link between satellite and the gateway (feeder link). The time and frequency offset introduced in feeder link can be post-compensated by the gNB or pre-compensated by the UE. For the compensation of feeder link delay it was already agreed in [3] to support common TA indication which allows to enable deployment with aligned UL and DL timing at the gNB. In order to decrease the overhead of common TA reconfiguration for non-GEO satellite case indication of common TA drift rate can be supported. Alternately, indication of reference point for pre-compensation can be supported. In this case UE can determine common TA based on the distance between satellite and reference point and update it autonomously.

Proposal 1:

- Support common TA drift rate indication

  - Alternatively, broadcast of reference point for pre-compensation of feeder link delay can be considered to avoid frequent reconfiguration

At the last RAN1 meeting the need of TA margin was discussed. In our view there is no need to introduce separate configuration for TA margin since it can be considered within common TA.

Observation 1:

- Separate configuration for TA margin is not needed since it can be considered within common TA

DL synchronization procedure may be more complex for NTN comparing to terrestrial networks due to larger frequency uncertainty at the UE side introduced by Doppler shift in service link. This frequency uncertainty for DL synchronisation can be reduced by applying pre-compensation of cell-specific frequency offset for DL transmission at the gNB side. Such possibility for cell-specific frequency offset pre-compensation for DL should be considered for UL synchronization to avoid potential misalignment of UL carrier frequency expected at the gNB receiver. As it was shown in our contribution [5], it is not necessary to indicate the value of frequency offset for pre-compensation applied for DL transmission since this value is known at the gNB side and post-compensation for UL transmission can be applied at the gNB. However, the indication of cell-specific frequency offset is needed for handover and inter-cell measurements. Thus, we propose to support DL frequency compensation for the service link Doppler with indication of the applied frequency pre-compensation value.

Proposal 2:

- Support DL frequency pre-compensation for the service link Doppler

  - Indication of the applied frequency pre-compensation value is required

For the frequency shift introduced in feeder link channel for transparent payload case, pre/post-compensation can be applied at the gNB without any impact on the NR specification. Compensation of feeder link Doppler shift by the UE may be problematic due to use of different bands for feeder link and service link.

Proposal 3:

- It is assumed by RAN1 that compensation of feeder link Doppler shift is done at the gNB side for UL/DL

# Conclusion

In this contribution aspects related to UL time and frequency synchronization were discussed. The following proposals and observations are made.

Proposal 1:

- Support common TA drift rate indication

  - Alternatively, broadcast of reference point for pre-compensation of feeder link delay can be considered to avoid frequent reconfiguration

Observation 1:

- Separate configuration for TA margin is not needed since it can be considered within common TA

Proposal 2:

- Support DL frequency pre-compensation for the service link Doppler

  - Indication of the applied frequency pre-compensation value is required

Proposal 3:

- It is assumed by RAN1 that compensation of feeder link Doppler shift is done at the gNB side for UL/DL

# References

- RP-193234 Solutions for NR to support non-terrestrial networks (NTN)

- 3GPP TR 38.821 V16.0.0 (2019-12); 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Solutions for NR to support non-terrestrial networks (NTN) (Release 16)

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #103-e, e-Meeting

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #104-e, e-Meeting

- R1-2008990 On UL time and frequency synchronization for NTN; Intel Corporation