| 3GPP TSG-RAN WG1 Meeting #104b e-Meeting, April 12th – 20th, 2021 | R1-2103139 |
| --- | --- |

Title: 		[Draft] Reply to RAN2 LS on UL Timing Alignment for SDT

Response to:		R2-2102090

Release:           			NR R17

Work Item:       		NR_SmallData_INACTIVE-Core

AI: 		5.2

Source:		Qualcomm [RAN1]

To:		RAN2

Send any reply LS to:	  3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:		  None

# Overall Description

RAN1 would like to thank RAN2 for the LS on UL timing alignment (TA) of small data transmission (SDT). From RAN1 perspective, a RRC inactive UE configured with CG-SDT resources can measure L1-RSRP based on SSB (SS-RSRP) to validate the UL timing alignment of SDT.

Since the SSB transmission in NR is beam-specific, and M (M≥1) SSB beams can be associated with a CG-SDT configuration, UE needs to select a SSB beam as reference for TA validation. The beam selection needs to be completed before the 1st CG-SDT occasion. The criterion for beam selection has been agreed by RAN2, which is based on a SS-RSRP threshold γs determined by the NW. UE selects one of the SSB beams whose SS-RSRP is above the threshold and uses the associated CG resources for SDT. At the end of beam selection, the L1-RSRP measurement of the reference beam,  say γ0 , is kept by UE as a baseline for TA validation.

Once the reference beam is selected, UE will keep measuring the L1-RSRP of the reference beam before each CG-SDT occasion, and comparing the latest measurement γn with baseline γ0 and threshold  γs.

- If γn is above γs and the difference between the baseline and the latest measurement is within a range determined by NW, i.e.

γn>γs   and   ∆RSRP,decrease≤γn-γ0≤ ∆RSRP,increase

where ∆TA, decrease  and ∆TA,  increase are two thresholds configured by the NW,  the TA is validated and the CG-SDT can proceed.

- Otherwise, the TA is considered as invalid and UE needs to skip the CG-SDT and re-acquire UL timing by RACH.

The thresholds used in the reference beam selection and TA validation procedures can be signaled to UE in RRCRelease message.

A special case for TA validation is a stationary UE configured with CG-SDT resources. Based on the UE capability signaling of a stationary UE, NW can relax the TA validation criteria, including reducing the duty cycle of measurements or configuring the TAT-SDT expiration time as infinity.

To 3GPP RAN2

ACTION: RAN1 respectfully asks RAN2 to take RAN1’s feedback on UL timing alignment of small data transmission into account in their specification work.

# Dates of Next TSG-RAN1 Meetings:

# RAN1 #104bis-e			12 April – 20 April 2021			Electronic

# RAN1 #105-e			19 May – 27 May 2021			Electronic

# References

- RP-210870, “Work Item on NR small data transmissions in INACTIVE state, ” 3GPP TSG RAN#91e.

- R2-2102090, “LS on uplink timing alignment for small data transmissions ” 3GPP TSG RAN2#113e, February, 2021.