3GPP TSG RAN WG1 Meeting #104bis-e	R1-2103389

E-meeting, April 12 – 20, 2021

Agenda Item:	8.7.1.3

Source:	Huawei, HiSilicon

Title:	Analysis on power consumption for IDLE mode and RedCap

Document for:	Discussion and Decision

# Introduction

In RAN#86, it is approved to further specify enhancements for UE power saving in Rel-17 on top of Rel-16 schemes. For idle/inactive mode, enhancements for paging procedure and assistance RS will be specified.

- Specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects [RAN2, RAN1] - Study and specify paging enhancement(s) to reduce unnecessary UE paging receptions, subject to no impact to legacy UEs [RAN2, RAN1] - NOTE: RAN1 to check and update, if needed, evaluation methodology in RAN1 #100 meeting - Specify means to provide potential TRS/CSI-RS occasion(s) available in connected mode to idle/inactive-mode UEs, minimizing system overhead impact [RAN1] - NOTE: Always-on TRS/CSI-RS transmission by gNodeB is not required

In this contribution, analysis on power consumption of legacy idle mode UE is provided to facilitate the discussion of power saving mechanism for NR idle mode UE, and the relationship between paging early indication and assistance RS is also discussed.

# Analysis on power consumption of idle mode UE

## Contribution of power consumption in idle mode

According to our investigation, the time and power distribution of typical wearable devices (smart watch) are shown in Figure 1. It is observed that idle mode contributes about 60% power consumption of the daily use. Therefore, it is strongly desired to reduce the RedCap UE power consumption in idle mode. In our view, the paging enhancement and assistance TRS should be both specified to improve the power consumption of eMBB UE and RedCap UE, as claimed in WID.

- The time and power distribution of typical wearable devices

- Idle mode power consumption contributes significant power share of wearable devices, for which the power consumption is also intended to be reduced in the WID.

For eMBB UE (e.g. smart phone), the energy consumption of IDLE mode also occupies significant share of the whole energy consumption. This is also the motivation for the introduction of RRM relaxation for IDLE mode in Rel-16 power saving WI. In Rel-17, paging enhancement for IDLE mode is also important for the power consumption of eMBB UE.

- Paging enhancement for IDLE mode is also important for the power consumption of eMBB UE.

## Analysis on power consumption of LTE and NR legacy idle mode UE

- Illustration for UE behaviors in idle/inactive mode

In idle mode, UE usually stays in deep sleep state in most of the time and Figure 2 illustrates a typical UE processing procedure on one paging occasion to monitor the paging message and do RRM measurements. Before UE receives paging, UE needs to wake up, and perform pre-sync procedures including AGC adjustment and T/F synchronization. In addition to paging, UE needs to perform RRM measurement for serving cell and neighbor cells when measurement criterion is met.

In LTE, UE uses CRS for pre-sync procedures. Since the CRS is an always-transmitted reference signal, an idle mode UE needs to wake up just one or two slots before its paging occasion to perform T/F tracking and AGC adjustment. However, in NR Rel-15/16, the only RS available for idle mode UE is SSB and the periodicity of SSB (typically 20ms) can be much larger than that of CRS (1ms), UE has to pre-wake up by several SSB bursts periods before its paging occasion. Figure 3 illustrates typical paging reception procedures in FR1 for both LTE and NR Rel-15/16 idle mode UE. The power consumption is analyzed based on the power model specified in [2] and agreed in RAN1#102-e.

The time gap from the start of a paging occasion to the closest SSB burst before the PO would be from 0ms to 18ms, considering the SSB periodicity is 20ms and SSB burst duration is 2ms. For simplicity, we only analyze the average case, that is, the paging occasion occurs 10ms after a SSB assuming the periodicity of SSB is 20ms. For paging reception, we assume that UE measures all the SSB before a PO and only receives the paging PDCCH in the monitoring occasion which corresponds to the SSB with best reception quality. Therefore the PO reception occurs 0.5ms in 30kHz SCS when Paging PDSCH is scheduled in the same slot of Paging PDCCH.

In RAN1#102, the power model used for IDLE mode power evaluation was agreed. The analysis results in Figure 4 is based on the agreed power model, which is listed in Table 2 in Appendix. For LTE case, the power consumption to receive CRS, PDCCH and PDSCH is assumed the same as that of NR. Other evaluation assumption can be found in our companion paper and the Appendix.

- Illustration of default association paging monitoring occasions

The power consumption distribution of each processing operation in paging procedure in idle mode for legacy LTE and NR with RRM relaxation is shown in Figure 4.

- The relative power consumption for LTE and NR

The light grey bars in Figure 4 correspond to the deep sleep mode and related state transition energy overhead. Considering the time duration in which an idle mode UE stays in deep sleep mode occupies the most of the time in a paging DRX cycle (1.28s in this evaluation). Therefore, the absolute energy consumption of the grey bar should be almost the same for LTE and NR in the figure. From the detailed power consumption, it can be seen that compared with LTE the main reason of power consumption increment for NR comes from the light sleep and power state transition between light sleep and active state, especially in the low SINR scenario.

The NR idle mode UE needs 1~3 SS bursts to perform T/F tracking and AGC adjustment, and additional SMTC window for neighbor cell measurements. However, the 1~3 SS bursts and the PO are transmitted in non-continuous slots, which is quite different from LTE. Therefore a NR idle mode UE has to pre-wake-up, and then transitions to light sleep to wait for a next signal arrival, which consumes much power of the UE. However, the LTE idle mode UE can finish all the operation without any light sleep and power state transition.

- The light sleep state and power state transition between light sleep and active state in NR consume more power compared to LTE system, which is the main reason for the high power consumption of NR Rel-15/16.

# The relationship between paging early indication and assistance RS

In RAN1#103, paging early indication (PEI) was agreed to be supported. Assistance TRS shall be also supported in Rel-17. This section, the relationship between PEI and assistance RS are discussed.

Firstly, PEI and assistance RS can be used separately and can provide the power saving gain independently. As analyzed in the companion contributions [3][4].

- Either paging early indication or assistance RS can provide power saving gain when the other scheme is not configured.

Furthermore, PEI and assistance TRS can be configured simultaneously and utilized by UE to obtain better power saving gains. Considering the power consumption of NR Rel-16 UE, according to our analysis in section 2.2, the more power consumption mainly comes from unnecessary ‘state transition’ and ‘light sleep’ in the unpaged DRX cycles and long pre-wake time duration needed for time/frequency tracking relying on serval number of SS bursts.

- Illustration of utilization of PEI and assistance TRS simultaneously by UE.

For those DRX cycles that idle/inactive mode UEs are not paged, the power consumption of ‘light sleep’, ‘light sleep state transition’ and the PO reception are actually not needed, which just waste the power consumption. If the network could transmit the paging information carried by the paging DCI earlier, the UE could know whether a PO reception is needed so that unnecessary ‘state transition’ and ‘light sleep’ in the unpaged DRX cycles can be avoided. Therefore, the UE can go to deep sleep immediately after the PEI reception if there is no paging message for the UE, i.e. Procedure A in Figure 5. More analysis for PEI can be found in our companion paper [3].

For those DRX cycles in which the concerned idle/inactive mode UE is paged, the UE shall be indicated to receive the corresponding PO. In this case, the UE needs to perform time/frequency tracking and AGC in order to reliably receive the paging PDSCH. The assistance TRS/CSI-RS can reduce the power consumption for the AGC and time/frequency tracking depending on the assistance TRS/CSI-RS position. As an example in Procedure B1 in Figure 5, if the TRS occasion at certain position is available, the UE can reduce the number of received SS bursts, therefore, additional power saving gain can be obtained.

According to the analysis in Figure 5, if PEI and assistance TRS are utilized by UE simultaneously, further power saving can be obtained.

- UE can utilize both paging early indication and assistance RS for better power saving gain if both features are configured.

The early paging information can be used to inform the availability of TRS/CSI-RS for idle/inactive mode UE, and the detailed mechanism can be found in our companion paper [4].

- Paging early indication can be used to inform the availability of TRS/CSI-RS for idle/inactive mode UE.

# Power saving for REDCAP UEs

Rel-16 has already introduced power saving mechanisms including WUS indication, cross-slot scheduling based power saving, and BWP-based maximum MIMO layer(s) adaptation. As analyzed in Table 1, the following Rel-16 features can be utilized by the REDCAP UE:

Table 1 Analysis for R16 power saving techniques

| Power saving techniques | Can be utilized by RedCap UEs | Detailed analysis |
| --- | --- | --- |
| PDCCH based wake-up indication | YES | - When a RedCap UE is in CONNECTED mode, it can be configured with C-DRX to save power. Wake-up indication bit before the ON duration of C-DRX cycle can be used to skip PDCCH monitoring in the C-DRX cycle to save power. - Dormancy indication is not needed considering RedCap UE are not likely to support CA. |
| Cross-slot scheduling | YES | - Cross-slot scheduling based power saving can provide the power saving gain by avoiding unnecessary DL OFDM symbol buffering and by relaxing PDCCH processing. - Cross-slot scheduling may introduce little latency increment. However, RedCap UEs with power saving requirements are usually not latency tolerant. |
| BWP based maximum MIMO layer adaptation | YES | - For 2Rx UE, gNB can switch UE to a BWP with maximum MIMO layer of 1 for power saving purpose. In this case, UE can utilize one single Rx chain for power saving. |
| SCell dormancy like behaviour | NO | - Dormancy like behaviour is a scheme to reduce PDCCH monitoring on SCell(s). - Due to the relatively low data rate requirement, there is no need to support CA in RedCap, thus dormancy like behaviour is not needed by RedCap UEs. |
| RRM relaxation for neighbour cell | YES | - RRM relaxation is already part of the RedCap SID, thus the RRM relaxation schemes in Rel-16 should be utilized as baseline for RedCap. |
| UE assistance information | YES | In Rel-16, the following assistance information are specified: - RRC Release Request, to avoid waiting for RRC release message from gNB for a long time - Preferred C-DRX configuration, to make C-DRX configuration more suitable - Preferred max aggregated Bandwidth, to make BWP parameter more suitable - Preferred max number of MIMO layers, to make MIMO parameter more suitable for UE’s traffic - Preferred K0min/K2min value, to make cross-slot scheduling parameter more suitable It is useful to let UE report necessary information to gNB, thus the above assistance information can be utilized for RedCap UE. |

- The following Rel-16 power saving mechanism can be utilized by RedCap UEs for power saving with potential modifications, if needed:

- PDCCH based wake-up indication

- Cross-slot scheduling

- maximum MIMO layer adaptation

- RRM relaxation for neighbor cell

- UE assistance information

In RAN#91e, the WID of RedCap was revised and approved in RP-210918. One note was added in the WID regarding the relationship with power saving enhancement WI:

Notes: - Uplink coverage enhancement solutions specified in the NR Coverage Enhancement WI (NR_cov_enh) shall be assumed to be available also to RedCap UEs by default (with small modifications for RedCap UEs if found necessary). - Power saving enhancement solutions specified in the UE Power Saving Enhancements WI (NR_UE_pow_sav_enh) shall be assumed to be available also to RedCap UEs by default.

Therefore, the discussion in power saving enhancement WI needs to take the RedCap UEs into consideration, which is also stated in the justification section of power saving enhancement WID.

In Rel-17, additional enhancements are required to address outstanding issues in Rel-16, namely idle/inactive-mode power consumption in NR SA deployments, considering both eMBB UEs and Reduced Capability NR Devices, connected-mode power consumption with FR2 deployments, and optimizing network utilization of Rel-16 UE assistance information.

- The power saving enhancement mechanisms need to also consider REDCAP UEs.

# Conclusions

In this contribution, we analysis the power consumption of legacy idle/inactive mode UE and discuss the relationship between the paging early indication and assistance RS. Based on the analysis, we have the following observations:

- Idle mode power consumption contributes significant power share of wearable devices, for which the power consumption is also intended to be reduced in the WID.

- Paging enhancement for IDLE mode is also important for the power consumption of eMBB UE.

- The light sleep state and power state transition between light sleep and active state in NR consume more power compared to LTE system, which is the main reason for the high power consumption of NR Rel-15/16.

- Either paging early indication or assistance RS can provide power saving gain when the other scheme is not configured.

- UE can utilize both paging early indication and assistance RS for better power saving gain if both features are configured.

- Paging early indication can be used to inform the availability of TRS/CSI-RS for idle/inactive mode UE.

- The following Rel-16 power saving mechanism can be utilized by RedCap UEs for power saving with potential modifications, if needed:

- PDCCH based wake-up indication

- Cross-slot scheduling

- maximum MIMO layer adaptation

- RRM relaxation for neighbor cell

- UE assistance information

- The power saving enhancement mechanisms need to also consider REDCAP UEs.

# References

RP-193239, “New WID: UE Power Saving Enhancements”, MediaTek

TR 38.840, Study on UE power saving.

R1-2102316, " Paging enhancements for UE power saving in IDLE/inactive mode", Huawei, Hisilicon

R1-2102317, " Assistance RS occasions for IDLE/inactive mode", Huawei, Hisilicon