3GPP TSG RAN WG1 Meeting #104bis-e	R1-2103393

E-meeting, April 12 – 20, 2021

Agenda Item:	8.2.7

Source:	Huawei, HiSilicon

Title:	Link level and system evaluation results for 52-71 GHz

Document for:	Discussion and Decision

Introduction

In RAN1#104-e meeting[1] and revised WID[2], it is agreed to evaluated the enhancements of PUCCH formats 0/1/4 by link level simulation. In addition to the PUCCH enhancements evaluation, we also provided some LLS results about DMRS and PTRS enhancements for 120/480/960kHz SCS. For channel access mechanism, some new SLS results are updated.

Assumptions and results for Link level evaluation

Simulation assumptions

Table 1.1 Simulation assumption of PDSCH/PUSCH evaluation

| Assumptions | Value |
| --- | --- |
| Carrier frequency | 60 GHz |
| Duplexing | TDD |
| Waveform | CP-OFDM for DL/UL, DFT-s-OFDM for UL |
| Numerology | [120 kHz] |
| Antenna Configuration (Mg,Ng,M,N,P) | - (Mg,Ng,M,N,P) = (1,1,4,8,2) BS with (0.5 dv, 0.5 dH) - (Mg,Ng,M,N,P) = (1,1,2,2,2) UE with (0.5 dv, 0.5 dH) |
| PN model | Example 2 phase noise model scaling to 60 GHz in 38.803 |
| Channel coding | LDPC code |
| frame structure | 2(PDCCH)/1(DMRS)/11(data+PTRS) |
| System BW | 400MHz：4096 FFT size |
| Transmission BW | 64 RBs for CP-OFDM 256 RBs for DFT-s-OFDM |
| TTI | 1 slot |
| DMRS | 1/2 in frequency in each port |
| PTRS | For CP-OFDM: K=2 in frequency domain, L=1 in time domain For DFT-s-OFDM: 8*4 or 16*2 |
| Receiver | ICI compensation |
| Channel estimation | Realistic |

Table 1.2 Simulation assumption of PUCCH design evaluation

| Parameter | Value |
| --- | --- |
| Carrier Frequency [GHz] | 60 GHz |
| Subcarrier spacing, SCS [kHz] | 120,480,960kHz |
| Number of usable RBs per carrier | 256 for 120 kHz SCS (corresponds to ~400 MHz carrier) 256 for 480 kHz SCS (corresponds to ~1600 MHz carrier) 160 for 960 kHz SCS (corresponds to ~2000 MHz carrier) |
| PUCCH Frequency Hopping | On |
| PUCCH Frequency Domain Resource Mapping | N_RB contiguous RBs per hop (with all REs allocated per PRB) |
| Waveform | CP-OFDM for PF0/1 DFT-s-OFDM for PF4 |
| CP Type | Normal CP |
| Channel Model | TDL-A, Delay spread (DS) = 10ns |
| BS Antenna Configuration (Mg,Ng,M,N,P) | {1,1,1,1,2} |
| UE Antenna Configuration (Mg,Ng,M,N,P) | {1,1,1,1,1} |
| Mobility | 3 km/hr |
| PA Model | None |
| gNB TRP PN Model | Zero phase noise |
| UE PN Model | Zero phase noise |
| Pre-loaded Tx EVM | 0% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |

Table 1.3 Reporting metrics for PUCCH design

| Parameter | Value |
| --- | --- |
| PUCCH Format | PF0, PF1, PF4 |
| Subcarrier spacing, SCS [kHz] | 120,480,960 kHz |
| Frequency hopping details | Frequency offset 46 RBs |
| Number of RBs used per hop (N_RB) | {1, 2, 4, 8} RBs |
| PUCCH bandwidth per hop, BW [MHz] | N_RB*SCS MHz |
| Number of OFDM symbols used for PUCCH resource | 2 for PF0 14 for PF1/4 |
| Sequence construction details | Short repetition sequence for PF0 Long sequence for PF0/1 Long sequence for DMRS of PF4 |
| OCC configuration details | N.A. |
| Cyclic shift configuration details | Initial cyclic shift 0 for PF0/1/4 |
| Number of multiplexed users, e.g., by code division, if applicable | 1 user |
| PUCCH payload encoder type | Reed Muller or Polar for PF4 |
| PUCCH payload size(s) (bits) | For PF4 Low: 4 bits Moderate:11 bits High: 22 bits |
| PUCCH encoding rate(s) | According to Reed Muller code for 4/11 bits payload According to Polar code and N_RB for 22 bits payload |
| Required SNR (dB) | Based on link level simulation results in Section 2.2.3. Detection criteria follows R1-2102127 |
| Cubic Metric, CM (dB) | Values shown in link budget calculation tables Appendix C. |
| UE Tx Beamforming gain (dBi) | TxBF = 6dBi |
| BS Rx Beamforming gain (dBi) | RxBF = 20dBi |
| UE Power Limitations | Maximum EIRP: UE_EIRP = 25 dBm Maximum conduced power (prior to consideration of backoff): UE_P = 21 dBm |
| Pmax (dBm) | Maximum allowed conducted power considering limit per region |
| Backoff (dB) | Power backoff is equal to the cubic metric |
| Transmit power, P_TX (dBm) | Maximum allowed transmit power including UE power limitation and backoff P_TX = min(Pmax, UE_EIRP – TxBF, UE_P – Backoff) |
| Noise power, P_N (dBm) | BS Noise Figure, NF = 7 dB Noise PSD = -174 dBm/Hz P_N = Noise PSD + 10*log10(BW * 1e6) + NF |
| Maximum Isotropic Loss, MIL (dB) | MIL = P_TX – P_N – Required SNR + TxBF + RxBF |

Simulation results

DMRS

Both different DMRS frequency density and different DMRS pattern in time domain for new SCSs are evaluated, and the results are shown in this section.

Increase DMRS frequency domain density

For new SCSs, increased frequency density of DMRS is proposed to reduce the influence of less RE’s number in coherence bandwidth for channel estimation. And in the simulations, DMRS type 1 are applied with two frequency densities, which are the exist density denoted as “FD=1/2” and the new frequency density denote as “FD=1”, means DMRS is mapped on the all the REs in the scheduled bandwidth. In addition, the TBS size are same for the two frequency densities, and 3 dB power boosting are applied to the DMRS for existed DMRS frequency density, with no data multiplexed with DMRS in the DMRS symbol. From the results shown in Figure 1 for different MCS and different channel scenarios, we can observed that the new frequency density provides no benefit in BLER performance.

Figure 1. BLER performance comparison of different DMRS frequency density for 480 kHz and 960 kHz under different channel scenarios

Observation 1: Increase frequency density for DMRS type 1 provides no benefit in BLER performance, when compared with the existing DMRS pattern in frequency domain.

Bundling DMRS

Put more than one DMRS symbols or all the symbols of the multi-slot PDSCH together in the front symbols of the multi-slot PDSCH, can be used to address the decreased accuracy of channel estimation caused by the shorter symbol duration for new SCSs. Evaluation results shown in Figure 2 are the BLER performance comparison between the existed DMRS pattern and the bundling pattern, and for the existed DMRS pattern, DMRS is mapped per slot, and channel estimation is estimated individually, for the bundling pattern, 4 DMRS symbols are mapped on the four consecutive symbols of the first slot per 4 slots of 480 kHz, and 8 DMRS symbols are mapped on eight consecutive symbols of the first slot per 8 slots of 960 kHz, and channel is estimated jointly with all the DMRS symbols.

As the Figure 2 shown, bundling DMRS with joint channel estimation improves the BLER performance of all the simulation case, with 0.2~0.6dB gain.

Figure 2. BLER performance comparison of different DMRS pattern in time domain for 480 kHz and 960 kHz under different channel scenarios

Observation 2: Bundling DMRS with joint channel estimation provides benefit when compared with per slot channel estimation based on existing DMRS pattern in time domain.

PTRS

In this section, PTRS of CP-OFDM in PDSCH and PTRS of DFT-S OFDM in PUSCH are discussed.

CP-OFDM PTRS

For ICI compensation, as analysis in [4], block PTRS with a new sequence is proposed, especially when power boosting is enabled. Power boosting cannot work effectively for the distributed PTRS as most of the received signal used to estimate ICI coefficients are from unknown data signal. However, it can be avoid by block PTRS, due to most of the received signal used to estimate ICI coefficients are from PTRS with power boosting.

To evaluate the performance benefit of block PTRS for ICI estimation, simulations under different MCS, SCS, and scheduled bandwidth are compared. In the evaluations, every K-th (K=4) RB is used as a baseline for Rel-15 PTRS, (CN,CS) is applied to block PTRS, where CN means block number per OFDM symbol, and CS means block size per PTRS block, the detailed design of (CN,CS) are shown in Table 2.1, and the block is mapped on the middle of each sub-band. Algorithm 1 which is direct algorithm discussed in [3] (denoted as Alg-1) is applied to Rel-15 PTRS, and both the algorithm 1 and algorithm 2 (proposed in [4] with lower complexity, denoted as Alg-2) are applied to block PTRS.

Table 2.1. Length of base sequence and circular sequence for block PTRS

| Block size M | 64 | 41 | 41 | 33 | 33 | 17 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ICI order estimated | 5 | 5 | 3 | 5 | 3 | 5 | 3 |
| Length of base sequence L | 59 | 37 | 37 | 29 | 31 | 13 | 13 |
| Length of head circular sequence q | 2 | 2 | 2 | 2 | 1 | 2 | 2 |
| Length of tail circular sequence p | 3 | 2 | 2 | 2 | 1 | 2 | 2 |

The results are shown from Figure 3 to Figure 11. As can be observed from the results, the demodulation SNR at BLER of 1% for block PTRS are lower than that of Rel-15 PTRS for almost all the cases, especially when scheduled bandwidth is 64RB: (1) with 256RB, block PTRS with both algorithms provide slightly better than distributed PTRS, where the detailed gains are about 0.2dB of CDL-B, and 0.1dB of CDL-D for 120 kHz and 480 kHz of MCS22; (2) with 160RB, block PTRS provides 0.5dB benefit of CDL-B, and 0.2~0.3dB benefit of CDL-D for 960 kHz; (3) with 128RB, the gain provides by block PTRS are 0.4~0.6dB of CDL-B, 0.2~0.4dB of CDL-D respectively; (4) with 64RB, the gain provides by block PTRS are increased further, which are about 0.8~1.5dB for MCS22 and 120 kHz, about 0.8~1.6dB for MCS26 and 480 kHz, about 1~1.8dB for MCS26 and 960 kHz.

Figure 3. BLER performance comparison of different PTRS pattern for 120 kHz with 256RB and power boosting

Figure 4. BLER and SE performance comparison of different PTRS pattern for 120 kHz with 128RB and power boosting

Figure 5. BLER and SE performance comparison of different PTRS pattern for 120 kHz with 64RB and power boosting

Figure 6. BLER performance comparison of different PTRS pattern for 480 kHz with 256RB and power boosting

\

Figure 7. BLER and SE performance comparison of different PTRS pattern for 480 kHz with 128RB and power boosting

Figure 8. BLER and SE performance comparison of different PTRS pattern for 480 kHz with 64RB and power boosting

Figure 9. BLER and SE performance comparison of different PTRS pattern for 960 kHz with 160RB and power boosting

Figure 10. BLER and SE performance comparison of different PTRS pattern for 960 kHz with 128RB and power boosting

Figure 11. BLER and SE performance comparison of different PTRS pattern for 960 kHz with 64RB and power boosting

Observation 3: With power boosting, performance of block PTRS is improved significantly and the required SNR at BLER of 1% are lower than that of Rel-15 PTRS for almost all the cases, especially when scheduled bandwidth is 64RB.

DFT-S OFDM PTRS

- Performance of different PTRS pattern

The evaluation results of PTRS for DFT-s-OFDM are shown in details in this section. As discussed [5] before, the PTRS pattern of DFT-s-OFDM defined in Rel-15 is insufficient when wide scheduled bandwidth and a high scheduled MCS is used due to the long interpolation range between two adjacent PTRS groups. As a solution to this problem, a new pattern with the same PTRS overhead as (CN, CS) = 16*2 is evaluated which has more PTRS groups and fewer PTRS samples per PTRS group where CN means NgroupPT-RS, CS means Nsamplegroup. The performance of more PTRS groups and more PTRS samples per PTRS group as (CN, CS) = 16*4 is also evaluated followed.

Figure 12. BLER performance comparison of different PTRS pattern for 120 kHz

Figure 13. BLER performance comparison of different PTRS pattern for 480 kHz

Figure 14. BLER performance comparison of different PTRS pattern for 960 kHz

As the results from Figure 12 to Figure 14 for different SCS and different Channel show, the new pattern provides significant gains for high MCS: With half of the interpolation range of pattern (CN, CS) = 8*4, the new pattern improves the BLER floor at least one order of magnitude. For medium MCS, its performance is almost the same as that of pattern (CN, CS) = 8*4 due to a worse noise reduction performance with only 2 PTRS samples per PTRS group at medium SNR region.

Observation 4: The required SNR to reach 10-1 and 10-2 BLER for 64QAM (MCS22/26) can be significantly improved for all SCS of 120 kHz, 480 kHz and 960 kHz, if the new PTRS pattern is introduced for DFT-S-OFDM.

- Performance of different PTRS location

As discussed in [3], due to Rx timing shift, (at least part of) a PTRS group placed at the tail of the transmitter’s DFT-s-OFDM symbol, may wrap-around to the head of the symbol from the receiver’s perspective, thus spoiling the original intention of the design and unnecessarily increasing Rx complexity, as well as deteriorating PN compensation performance. The BLER performance of DFT-S OFDM PTRS with different Rx advance shift when the DFT-S OFDM’s tail includes PTRS groups is shown followed.

The location 0 means the old PTRS location defined in Rel.15. Z% presents the Rx advance shift percentage of the CP duration. CN means NgroupPT-RS and CS means Nsamplegroup.

Figure 15. BLER performance comparison of different Rx advance shift (0%, 10%, 20%) for 120 kHz

Figure 16. BLER performance comparison of different Rx advance shift (0%, 10%, 20%) for 480 kHz

Figure 17. BLER performance comparison of different Rx advance shift (0%, 10%, 20%) for 960 kHz

As the results shown from Figure 15 to Figure 17 for different SCS and different Channel, the Rx advance shift will decrease the system performance especially for high MCS. The Rx advance shift leads to PTRS group placed at the tail of the transmitter’s DFT-s-OFDM symbol wrap-around to the head of the symbol from the receiver’s perspective, thus deteriorating Phase Noise compensation performance with PTRS.

Observation 5: Simulation results for DFT-s-OFDM with different Rx advance shift show that, if the PTRS location defined in Rel-15 is used, BLER performance of 120 kHz/480kHz/960kHz will be decreased especially for high MCS due to a worse PN compensation caused by Rx timing shift.

To solve the problem discussed here, a new PTRS location which puts all PTRS sequences in the middle of each interval is presented. In that case, even if there is Rx advance shift, the PTRS group won’t wrap-around to the head of the symbol and may not influence Phase Noise compensation performance. The BLER performance comparison of different PTRS location with different Rx advance shift for 120kHz/480kHz/960kHz are shown followed. The location 0 means the old PTRS location defined in Rel.15 while location 1 means the new PTRS location which is in the middle of each interval.

From the results of 120kHz, [CN,CS]=[4,4] is insufficient for high MCS such as MCS26. The simulation adopts  [CN,CS]=[8,4] for MCS26 in 120kHz, [CN,CS]=[4,4] for others.

Figure 18. BLER performance comparison of different PTRS location with different Rx advance shift (0%, 10%) for 120 kHz

Figure 19. BLER performance comparison of different PTRS location with different Rx advance shift (0%, 10%) for 480 kHz

Figure 20. BLER performance comparison of different PTRS location with different Rx advance shift (0%, 10%) for 960 kHz

Figure 21. BLER performance comparison of different PTRS location with different Rx advance shift (0%, 10%, 20%) in different Channel for 120 kHz @ MCS22

As the results shown from Figure 18 to Figure 21 for different SCS and different Channel with different PTRS location and different Rx advance shift, the BLER performance with new PTRS location is the best especially for high MCS. For example, the BLER performance of 120kHz/480kHz/960kHz with MCS26 reaches a floor above 10-2 due to 10% Rx advance shift.

Observation 6: Simulation results for DFT-s-OFDM with different PTRS location and different Rx advance shift show that, New PTRS location which is in the middle of each interval can solve the influence on BLER performance caused by Rx advance timing shift.

PUCCH

In order to make decisions on the number of supported RBs for enhanced PUCCH format 0/1/4, detection performance is evaluated by link level simulation in Figure 22 to 37. As required SNR obtained, link budget calculation is done by taking regional regulatory and UE limitations into consideration. We summarize the results in terms of maximum isotropic loss (MIL) in Appendix C. Table 15 to 18.

As shown in Figure 22 and 23, the required SNR difference between short repetition sequence and long sequence is ~0.1dB with 4 PRBs and nearly 0 with 8PRB for PUCCH format 0.

Observation 7: The required SNR is almost the same among short repetition sequence and long sequence, as no interference from multiplexed UE is considered.

Observation 8: Based on detection performance, the following can be concluded:

- Required SNR decreases with the increase of number of PRB for PF0/1/4

- Required SNR increases with the increase of PF4 payload size

PUCCH format 0 is evaluted with two OS and two bits payload. The MIL is shown in Appendix C.

- For 120kHz SCS, there is 8.9 dB MIL gain in USA by increasing the number of RBs from 1 to 8. MIL gain is 0.6dB in EU from 1 RB to 2 RBs. There is no further MIL gain achieved in EU by increasing the number of RBs (>2).

- For 480kHz SCS, there is 4.0 dB MIL gain in USA by increasing the number of RBs from 1 to 4. MIL gain is 0.3dB in EU from 1 RB to 2RBs. There is no further MIL gain achieved in EU by increasing the number of RBs (>2).

- For 960kHz SCS, there is 0.2 dB MIL gain in USA by increasing the number of RBs from 1 to 2.  There is no MIL gain achieved in EU by increasing the number of RBs (>1).

PUCCH format 1 is evaluted with fourteen OS and two bits payload. The MIL is shown in Appendix C.

- For 120kHz SCS, there is 8.8 dB MIL gain in USA by increasing the number of RBs from 1 to 8. While there is merely 0.2 dB MIL gain in EU with 1 RB and 8 RBs.

- For 480kHz SCS, there is 4.1 dB MIL gain in USA by increasing the number of RBs from 1 to 4. MIL gain is 0.1dB in EU from 1 RB to 2RBs. There is no further MIL gain achieved in EU by increasing the number of RBs (>2).

- For 960kHz SCS, there is 1.4 dB MIL gain in USA by increasing the number of RBs from 1 to 2.There is no MIL gain achieved in EU by increasing the number of RBs (>1).

PUCCH format 4 is evaluted with fourteen OS and 4/11/22 bits payload. The MIL is shown in Appendix C.

- For 120kHz SCS and 4 bits payload, there is 9.1 dB MIL gain in USA by increasing the number of RBs from 1 to 8. MIL gain is 0.4dB in EU from 1 RB to 4 RBs. There is no further MIL gain achieved in EU by increasing the number of RBs (>4).

- For 120kHz SCS and 11 bits payload, there is 9.0 dB MIL gain in USA and 0.3dB in EU by increasing the number of RBs from 1 to 8.

- For 120kHz SCS and 22 bits payload, there is 9.6 dB MIL gain in USA and 0.9dB in EU by increasing the number of RBs from 1 to 8.

- For 480kHz SCS and 4/11 bits payload, there is 3.6 dB MIL gain in USA by increasing the number of RBs from 1 to 4. MIL gain is 0.1dB in EU from 1 RB to 2RBs. There is no further MIL gain achieved in EU by increasing the number of RBs (>2).

- For 480kHz SCS and 22 bits payload, there is 3.5 dB MIL gain in USA and 0.6 dB MIL gain in EU by increasing the number of RBs from 1 to 2. There is no further MIL gain achieved both in USA and in EU by increasing the number of RBs (>2).

- For 960kHz SCS and 4 bits payload, there is 1.6 dB MIL gain in USA and 0.7 dB by increasing the number of RBs from 1 to 2.

- For 960kHz SCS and 11 bits payload, there is 1.7 dB MIL gain in USA and 0.8 dB by increasing the number of RBs from 1 to 2.

- For 960kHz SCS and 22 bits payload, there is 2.8 dB MIL gain in USA and 1.9 dB by increasing the number of RBs from 1 to 2.

Observation 9: Based on link budget calculation, the following can be concluded:

- For 120 kHz SCS, better MIL performance can be achieved by increasing the number of RBs for PF0/PF1/PF4 according to USA regulation. MIL gain according to EU regulation is limited.

- For 480 kHz SCS, better MIL performance can be achieved by increasing the number of RBs for P0/PF1 and PF4 with 4/11 bits payload. There is around 0.1 MIL gain according to EU regulation with 2 RBs. For PF4 with 22 bits payload, no further MIL gain can be achieved more than 2 RBs.

- For 960 kHz SCS, MIL gain can be achieved with 2 RBs according to USA and EU regulation for PF4 with different payload. For PF0/PF1, there is MIL gain according to USA regulation, but no MIL gain according to EU regulation.

- (a) 4 PRBs                                                    (b) 8 PRBs

- Figure 22. PF0 at 120kHz with different number of PRBs and short repetition sequence

- (a) 1 PRB                                                       (b) 2 PRBs

- (c) 4 PRBs                                                    (d) 8 PRBs

- Figure 23. PF0 at 120kHz with different number of PRBs and long sequence

- (a) 1 PRB                                                       (b) 2 PRBs

- (c) 4 PRBs                                                    (d) 8 PRBs

- Figure 24. PF1 at 120kHz with different number of PRBs and long sequence

(a) 1 PRB                                                       (b) 2 PRBs

(c) 4 PRBs                                                     (d) 8 PRBs

- Figure 25. PF4 at 120kHz with different number of PRBs with 4 bits payload and long sequence

(a) 1 PRB                                                      (b) 2 PRBs

(c) 4 PRBs                                                     (d) 8 PRBs

- Figure 26. PF4 at 120kHz with different number of PRBs with 11 bits payload and long sequence

(a) 1 PRB                                                      (b) 2 PRBs

(c) 4 PRBs                                                     (d) 8 PRBs

- Figure 27 PF4 at 120kHz with different number of PRBs with 22 bits payload and long sequence

- (a) 1 PRB                                                       (b) 2 PRBs

- (c) 4 PRBs

- Figure 28. PF0 at 480kHz with different number of PRBs and long sequence

- (a) 1 PRB                                                      (b) 2 PRBs

- (c) 4 PRBs

- Figure 29. PF1 at 480kHz with different number of PRBs and long sequence

(a) 1 PRB                                                      (b) 2 PRBs

(c) 4 PRBs

- Figure 30. PF4 at 480kHz with different number of PRBs with 4 bits payload and long sequence

(a) 1 PRB                                                      (b) 2 PRBs

(c) 4 PRBs

- Figure 31. PF4 at 480kHz with different number of PRBs with 11 bits payload and long sequence

(a) 1 PRB                                                       (b) 2 PRBs

(c) 4 PRBs

- Figure 32. PF4 at 480kHz with different number of PRBs with 22 bits payload and long sequence

- (a) 1 PRB                                                      (b) 2 PRBs

- Figure 33. PF0 at 960kHz with different number of PRBs and long sequence

- (a) 1 PRB                                                      (b) 2 PRBs

- Figure 34. PF1 at 960kHz with different number of PRBs and long sequence

(a) 1 PRB                                                       (b) 2 PRBs

- Figure 35. PF4 at 960kHz with different number of PRBs with 4 bits payload and long sequence

(a) 1 PRB                                                       (b) 2 PRBs

- Figure 36. PF4 at 960kHz with different number of PRBs with 11 bits payload and long sequence

(a) 1 PRB                                                      (b) 2 PRBs

- Figure 37. PF4 at 960kHz with different number of PRBs with 22 bits payload and long sequence

Assumptions and results for system level evaluation

In RAN1 #103-e and RAN #90-e, it was agreed that both LBT mode and no-LBT mode are recommended to be supported to cover a wide range of use cases and regulatory requirements. For LBT mode, the channel access mechanism defined in EN302 567[6] has been identified as the baseline and enhancements have been identified for further discussion. In the revised WID, objectives were set to specify both LBT and No-LBT related procedures, and to study, and if needed specify, omni-directional LBT, directional LBT and receiver assistance in channel access.  In this section, we mainly study the receiver-assisted/receiver-only LBT and provide some simulation results for indoor scenario A.

Simulation assumptions for indoor scenario A

| Parameters | Assumptions |
| --- | --- |
| Layout | Indoor scenario-A: Office box 120m x 50m，12 BS per operator, 2 operator.BS randomly deployed within 10m x 10m virtual box. BS mounting on the ceiling. |  |
| Inter-BS distance | Random with minimum distance of 2m for scenario-A |  |
| UE distribution | 100% Indoor, randomly distributed within the office box, 5 users per BS |  |
| Carrier bandwidth | 2000MHz |  |
| Numerology | 960kHz for 2000MHz |  |
| Carrier frequency | 60GHz |  |
| Channel model | InH open channel model in 38.901 |  |
| Max. allowed BS Tx power(EIRP) | 40dBm |  |
| Max. allowed UE Tx Power(EIRP) | 25dBm |  |
| BS antenna configurations | See 38.802 Table A.2.1-7 for ceiling mount(antenna gain = 5dB) |  |
| UE antenna configuration | See 38.802 Table A.2.1-8, boresight parallel to ground(antenna gain = 5dB) |  |
| BS antenna height | 3m |  |  |
| UE antenna height | 1.5m |  |  |
| BS receiver noise figure | 7dB |  |  |
| UE receiver noise figure | 10dB |  |
| BS antenna array configuration | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), Dh = Dv = 0.5 λ or (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), Dh = Dv = 0.5 λ |  |
| UE antenna array configuration | (M, N, P, Mg, Ng) = (2, 2, 2, 1, 2), Dh = Dv = 0.5 λ or (M, N, P, Mg, Ng) = (1, 2, 2, 1, 2), Dh = Dv = 0.5 λ |  |
| MCOT | 5ms |  |
| Traffic model | FTP3 with packet size of 27 Mbytes. |  |
| UE receiver | MMSE-IRC as the baseline receiver |  |
| Feedback assumption | Realistic |  |
| Channel estimation | Realistic/ideal |  |
| Traffic type | 50% DL, 50% UL |  |

Observations regarding Receiver-assisted/receiver-only LBT performance

Impact of number of gNB/UE antennas on Receiver-assisted/receiver-only LBT performance

Table 3.1. Antenna configuration for indoor scenario

| gNB/UE antenna configuration | gNB antenna configuration | UE antenna configuration |
| --- | --- | --- |
| Baseline assumption in TR38.808 | M, N, P, Mg, Ng) = (4, 8, 2, 1, 1) | (M, N, P, Mg, Ng) = (2, 2, 2, 1, 2) |
| Assumption of low-cost devices | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1) | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 2) |

In TR38.808, 64 antenna elements and 8 antenna elements are configured for gNB and UE, respectively, as the baseline SLS assumption in the indoor scenario. The probability of experiencing significant interference levels form hidden nodes is low under the current baseline assumption, mainly due to the narrow analog transmission beamwidth. However, in practice, a low-cost device may be equipped with a smaller number of antennas and wider transmission beams would have to be used. The performance of different antenna configuration assumptions as illustrated in Table 3.1 are thus evaluated.

Figure 38 shows the average and fifth percentile DL and UL user perceived throughput (UPT) under the baseline SLS assumption for No-LBT/Omni-directional LBT/Directional LBT/Received-assisted LBT/Receiver-only LBT in indoor scenario A with InH open channel models for BS to UE links. Table 3.2 shows the related results and gains at high traffic load. More detailed results could be found in Appendix B, Tables 09 to 011, respectively. More details of the receiver-assisted and receiver-only LBT mechanisms could be found in our companion contribution [7].We assume that the CW is set to a fixed value of 5 observation slots and MCOT equals 5ms. We also assume that the EDT used for the Rx-side LBT procedure before sending CTS with interference level feedback in receiver-assisted/receiver-only LBT is the same as the baseline EDT (-47dBm). Antenna directional gain is also considered in the EDT calculation.

Figure 39 illustrates the average and fifth percentile DL and UL UPT under the assumption of reduced antenna elements for No LBT/Omni-directional LBT/Directional LBT/received-assisted LBT/receiver-only directional LBT in indoor scenario A with InH open channel models for BS to UE links. Table 3.3 shows the related results and gains at high traffic load. More detailed results could be found in Appendix B, Tables 12 to 14, respectively.

According to the above evaluation results, receiver-only LBT has the best performance compared to other channel access types in fifth percentile DL and UL UPT. The gain of receiver-only LBT compared to No LBT could be 177% in DL and 75% in UL for cell edge at high traffic load under the baseline assumption. In turn, the gain of receiver-only LBT compared to No LBT could be 291% in DL and 103% in UL for cell edge at high traffic load when reduced number of antenna elements are configured.

Table 3.2. Performance of various channel access type at high traffic load (64 antenna elements for gNB, 8 antenna elements for UE)

| Channel access type/User throughput(Mbps) | DL mean UPT | UL mean UPT | DL 5% UPT | UL 5% UPT | Gain of DL mean UPT w.r.t No-LBT | Gain of UL mean UPT w.r.t No-LBT | Gain of 5% DL UPT w.r.t No-LBT | Gain of 5% UL UPT w.r.t No-LBT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No-LBT | 2388.7 | 1521.8 | 85.7 | 86.6 |  |  |  |  |
| Directional LBT | 2446.7 | 1521.1 | 101.3 | 82.3 | 2% | 0% | 18% | -5% |
| Omni-dir LBT | 2452.9 | 1538 | 95.9 | 89.8 | 3% | 1% | 12% | 4% |
| Receiver-assisted LBT | 2494.2 | 1607.9 | 191.7 | 145.6 | 4% | 6% | 123% | 68% |
| Receiver-only LBT | 2715.7 | 1637.6 | 237.6 | 151.5 | 14% | 8% | 177% | 75% |

Table 3.3. Performance of various channel access type at high traffic load (32 antenna elements for gNB, 4 antenna elements for UE)

| Channel access type/User throughput(Mbps) | DL mean UPT | UL mean UPT | DL 5% UPT | UL 5% UPT | Gain of DL mean UPT | Gain of UL mean UPT | Gain of 5% DL UPT | Gain of 5% UL UPT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No-LBT | 1842.4 | 1208.4 | 34.7 | 44.5 |  |  |  |  |
| Directional LBT | 1852.7 | 1193.5 | 39.3 | 16.6 | 1% | -1% | 13% | -37% |
| Omni-dir LBT | 1964.1 | 1218.8 | 52.4 | 40.9 | 7% | 1% | 51% | -8% |
| Receiver-assisted LBT | 2033.6 | 1299.1 | 88.9 | 76.4 | 10% | 8% | 156% | 72% |
| Receiver-only LBT | 2215.7 | 1340 | 135.6 | 90.4 | 20% | 11% | 291% | 103% |

Figure 38. Simulation results with various channel access mechanism for 60GHz band in indoor scenario-A(64 antenna elements for gNB, 8 antenna elements for UE)

Figure 39. Simulation results with various channel access mechanism for 60GHz band in indoor scenario-A (32 antenna elements for gNB, 4 antenna elements for UE)

Observation 10: Reducing the number of antenna elements by 50 percent (32 for gNB and 4 for UE) compared to the baseline assumption in TR38.808 for the indoor scenario increases the probability of hidden nodes. This results in significantly more gains at DL cell-edge in Receiver-assisted/Receiver-only LBT compared to No-LBT.

Impact of EDT of Rx-side LBT on Receiver-assisted LBT performance

For receiver-assisted/receiver-only LBT, EDT_Rx = -71dBm + 10log10 (BW/2GHz) + offset_dB was assumed at the UE side for LBT before assistance information reporting (for DL) or at the gNB for directional sensing in the direction of potential PUSCH transmission from the corresponding UE (for UL).  The intuition is that, for the DL for instance, the maximum tolerable interference at a receiving UE could be defined as offset_dB above the minimum received power level from the serving gNB, i.e., the minimum association threshold agreed for SLS assumptions -71dBm + 10log10 (BW/2GHz).  Figure 40 illustrates the average and fifth percentile DL and UL user throughput for different EDT_Rx threshold at receiver side in indoor scenario A at medium traffic load. The performances of both the mean and the fifth percentile do not degrade when EDT_Rx becomes lower. In this evaluation, we assume that the measured interference level during the Rx-side LBT is reported to the transmitter.

Figure 40. Simulation results with different EDT_Rx threshold in receiver assisted LBT in scenario-A

Observation 11: For Receiver-assisted LBT, if the interference level at the receiver is reported to gNB  (for DL) or assessed by gNB (for UL) when the corresponding LBT procedure is successful, reducing EDT results in more LBT failures and less number of interference assessments/reports. However, this does not considerably change the UPT performance if gNB uses the reported/assessed interference levels to prioritize UE scheduling.

Impact of receiver assistance information on Receiver-assisted LBT performance in DL

Table 3.4 shows the performance of the average and fifth percentile DL UPT if only CTS/idle indication is fed back when the interference level sensed by the UE in certain directional beam is lower than the EDT_Rx, i.e., interference level is not reported. Table 3.4 also compares these results with the corresponding results where the actual interference levels are also fed back to the gNB along with the one-bit CTS indication. As can be observed from the table, sending CTS/idle indication along with actual interference level would be beneficial since gNB can use the reported interference levels to prioritize the scheduling of the UEs. The performance of reporting the actual interference level does significantly change when the EDT_Rx is varied. It is due to the fact that even when a higher EDT_Rx is used and more UEs pass the CCA, thegNB still prioritizes the UEs with the least interference levels for scheduling. However, if only one-bit CTS information is fed back from the UEs, gNB has no means to properly prioritize scheduling the UEs among the UE candidates based on their interference level and, as the number of candidates for scheduling increases, the used scheduling scheme based only on one-bit feedback CTS becomes increasingly sub-optimal.

Table 3.4. Simulation results with different EDT_Rx and feedback information in DL receiver assisted LBT in indoor scenario-A

| EDT_Rx(dBm) | 0 |  | -47 |  | -51 |  | -61 |  | -67 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| User perceived throughput(Mbps) | Mean DL UPT | 5% DL UPT | Mean DL UPT | 5% DL UPT | Mean DL UPT | 5% DL UPT | Mean DL UPT | 5% DL UPT | Mean DL UPT | 5% DL UPT |
| CTS/idle indication with actual interference level reporting | 3873.5 | 559.5 | 3873.5 | 559.5 | 3875.8 | 566.6 | 3832.6 | 563.7 | 3857.0 | 555.1 |
| Only CTS/idle indication reporting |  |  | 3528.2 | 377.4 | 3551.1 | 419.5 | 3771.9 | 550.1 | 3781.0 | 549.7 |
| Relative Loss |  |  | -9% | -33% | -8% | -26% | -2% | -2% | -1.9% | -0.9% |

Observation 12: If the EDT_Rx threshold used at the receiver in Receiver-assisted LBT is increased, the DL cell-edge performance degrades when only CTS/idle indication (and not actual interference level) is fed back to the gNB after CCA assessment.

Conclusions

In this contribution, we have provided link level and system level simulation results for both licensed and unlicensed bands. Based on the discussion, we have the following observations:

Observations from LLS for DMRS:

Observation 1: Increase frequency density for DMRS type 1 provides no benefit in BLER performance, when compared with the existing DMRS pattern in frequency domain.

Observation 2: Bundling DMRS with joint channel estimation provides benefit when compared with per slot channel estimation based on existing DMRS pattern in time domain.

Observations from LLS for PTRS:

Observation 3: With power boosting, performance of block PTRS is improved significantly and the required SNR at BLER of 1% are lower than that of Rel-15 PTRS for almost all the cases, especially when scheduled bandwidth is 64RB.

Observation 4: The required SNR to reach 10-1 and 10-2 BLER for 64QAM (MCS22/26) can be significantly improved for all SCS of 120 kHz, 480 kHz and 960 kHz, if the new PTRS pattern is introduced for DFT-S-OFDM.

Observation 5: Simulation results for DFT-s-OFDM with different Rx advance shift show that, if the PTRS location defined in Rel-15 is used, BLER performance of 120 kHz/480kHz/960kHz will be decreased especially for high MCS due to a worse PN compensation caused by Rx timing shift.

Observation 6: Simulation results for DFT-s-OFDM with different PTRS location and different Rx advance shift show that, New PTRS location which is in the middle of each interval can solve the influence on BLER performance caused by Rx advance timing shift.

Observations from LLS for PUCCH:

Observation 7: The required SNR is almost the same among short repetition sequence and long sequence, as no interference from multiplexed UE is considered.

Observation 8: Based on detection performance, the following can be concluded:

- Required SNR decreases with the increase of number of PRB for PF0/1/4

- Required SNR increases with the increase of PF4 payload size

Observation 9: Based on link budget calculation, the following can be concluded:

- For 120 kHz SCS, better MIL performance can be achieved by increasing the number of RBs for PF0/PF1/PF4 according to USA regulation. MIL gain according to EU regulation is limited.

- For 480 kHz SCS, better MIL performance can be achieved by increasing the number of RBs for P0/PF1 and PF4 with 4/11 bits payload. There is around 0.1 MIL gain according to EU regulation with 2 RBs. For PF4 with 22 bits payload, no further MIL gain can be achieved more than 2 RBs.

- For 960 kHz SCS, MIL gain can be achieved with 2 RBs according to USA and EU regulation for PF4 with different payload. For PF0/PF1, there is MIL gain according to USA regulation, but no MIL gain according to EU regulation.

Observations from SLS for channel access:

Observation 10: Reducing the number of antenna elements by 50 percent (32 for gNB and 4 for UE) compared to the baseline assumption in TR38.808 for the indoor scenario increases the probability of hidden nodes. This results in significantly more gains at DL cell-edge in Receiver-assisted/Receiver-only LBT compared to No-LBT.

Observation 11: For Receiver-assisted LBT, if the interference level at the receiver is reported to gNB  (for DL) or assessed by gNB (for UL) when the corresponding LBT procedure is successful, reducing EDT results in more LBT failures and less number of interference assessments/reports. However, this does not considerably change the UPT performance if gNB uses the reported/assessed interference levels to prioritize UE scheduling.

Observation 12: If the EDT_Rx threshold used at the receiver in Receiver-assisted LBT is increased, the DL cell-edge performance degrades when only CTS/idle indication (and not actual interference level) is fed back to the gNB after CCA assessment.

- References

- Chairman’s Notes RAN1#104-e

- RP-210862. Revised WID: on Extending current NR operation to 71 GHz, RAN#91-e

- P. Mathecken, T. Riihonen, S. Werner, and R. Wichman, “Constrained phase noise estimation in OFDM
using scattered pilots without decision feedback,” IEEE Transactions on Signal Processing, vol. 65, no. 9, pp. 2348–2362, May 2017

- R1-21xxxx. PDSCH/PUSCH enhancements for 52-71GHz band. Huawei, HiSilicon.

- R1-2007604 PHY design in 52.6-71 GHz using NR waveform, Huawei, HiSilicon.

- ETSI EN 302 567 Multiple-Gigabit/s radio equipment operating in the 60 GHz band; V2.1.20

- R1- 2102332 Channel access mechanism for 60 GHz unlicensed operation, Huawei, HiSilicon.

- Appendix A.

Table 01. SINR in dB achieving PDSCH BLER of 10% /1% with different DMRS frequency domain density

| SCS,RB | Channel | DS | MCS | MCS7 |  | MCS16 |  | MCS22 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FD | FD=1/2 | FD=1 | FD=1/2 | FD=1 | FD=1/2 | FD=1 |
| 480kHZ, 256RB | CDL-B | 20ns | 10%th | 2.938 | 2.936 | 10.927 | 11.056 | 16.552 | 16.7 |
|  |  |  | 1%th | 4.733 | 4.815 | 12.323 | 12.549 | 17.854 | 18.239 |
|  | CDL-D | 20ns | 10%th | -0.958 | -0.944 | 6.501 | 6.522 | 11.431 | 11.478 |
|  |  |  | 1%th | 0.139 | 0.215 | 7.208 | 7.245 | 11.879 | 11.955 |
|  |  | 30ns | 10%th | -0.962 | -0.946 | 6.496 | 6.523 | 11.433 | 11.475 |
|  |  |  | 1%th | 0.159 | 0.227 | 7.236 | 7.267 | 11.88 | 11.961 |
| 960kHz, 160RB | CDL-B | 20ns | 10%th | 4.13 | 4.131 | 11.89 | 11.931 | 17.778 | 17.896 |
|  |  |  | 1%th | 6 | 6 | 15 | 15 | 20.818 | 20.766 |
|  | CDL-D | 20ns | 10%th | 0.152 | 0.151 | 7.188 | 7.23 | 12.624 | 12.648 |
|  |  |  | 1%th | 1.153 | 1.163 | 8.02 | 8.051 | 13.454 | 13.469 |
|  |  | 30ns | 10%th | 0.158 | 0.15 | 7.192 | 7.243 | 12.631 | 12.646 |
|  |  |  | 1%th | 1.153 | 1.164 | 8.013 | 8.06 | 13.434 | 13.462 |

Table 02. SINR in dB achieving PDSCH BLER of 10% /1% with different DMRS pattern in time domain

| SCS,RB | Channel | DS | MCS | MCS7 |  | MCS16 |  | MCS22 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | DMRS pattern | Per slot | bund | Per slot | bund | Per slot | bund |
| 480kHz, 256RB | CDL-B | 20ns | 10%th | 3.111 | 2.522 | 11.28 | 10.948 | 16.733 | 16.414 |
|  |  |  | 1%th | 4.645 | 4.071 | 12.654 | 12.44 | 18.063 | 17.775 |
|  | CDL-D | 20ns | 10%th | -0.874 | -1.291 | 6.61 | 6.355 | 11.683 | 11.383 |
|  |  |  | 1%th | -0.012 | -0.797 | 7.281 | 7.057 | 12.133 | 11.864 |
|  |  | 30ns | 10%th | -0.873 | -1.291 | 6.611 | 6.356 | 11.676 | 11.383 |
|  |  |  | 1%th | -0.024 | -0.794 | 7.303 | 7.047 | 12.15 | 11.872 |
| 960kHz, 160RB | CDL-B | 20ns | 10%th | 3.991 | 3.166 | 11.606 | 11.188 | 17.566 | 16.998 |
|  |  |  | 1%th | 6.0 | 7.0 | 15 | 14.262 | 20.472 | 19.911 |
|  | CDL-D | 20ns | 10%th | 0.367 | -0.227 | 7.55 | 7.197 | 12.995 | 12.528 |
|  |  |  | 1%th | 1.336 | 0.59 | 8.184 | 7.83 | 13.667 | 13.16 |
|  |  | 30ns | 10%th | 0.379 | -0.189 | 7.548 | 7.201 | 12.992 | 12.528 |
|  |  |  | 1%th | 1.321 | 0.616 | 8.184 | 7.84 | 13.663 | 13.164 |

Table 03. SINR in dB achieving PDSCH BLER of 10% /1% with different PTRS pattern and 256RB with power boosting

| SCS | channel | BLER | MCS22 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | K=4, ICI1 | [1,64], ICI1 | [1,64], ICI2 |
| 120k | CDL-B, 20ns | 0.1 | 18.018 | 17.802 | 17.822 |
|  |  | 0.01 | 19.695 | 19.317 | 19.338 |
|  | CDL-B, 50ns | 0.1 | 17.272 | 17.009 | 17.026 |
|  |  | 0.01 | 18.823 | 18.39 | 18.472 |
|  | CDL-D, 20ns | 0.1 | 12.244 | 12.123 | 12.139 |
|  |  | 0.01 | 13 | 12.867 | 12.946 |
|  | CDL-D, 30ns | 0.1 | 12.246 | 12.116 | 12.133 |
|  |  | 0.01 | 12.991 | 12.892 | 12.978 |
| 480k | CDL-B, 20ns | 0.1 | 16.343 | 16.086 | 16.094 |
|  |  | 0.01 | 17.634 | 17.345 | 17.372 |
|  | CDL-B, 50ns | 0.1 | 16.567 | 16.264 | 16.293 |
|  |  | 0.01 | 17.809 | 17.498 | 17.531 |
|  | CDL-D, 20ns | 0.1 | 11.894 | 11.797 | 11.803 |
|  |  | 0.01 | 12.404 | 12.282 | 12.282 |
|  | CDL-D, 30ns | 0.1 | 11.897 | 11.792 | 11.808 |
|  |  | 0.01 | 12.398 | 12.269 | 12.294 |

Table 04. SINR in dB achieving PDSCH BLER of 10% /1% with different PTRS pattern and 160RB with power boosting

| SCS | channel | BLER | MCS22 |  |  | MCS26 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | K=4, ICI | [1,41], ICI1 | [1,41], ICI2 | K=4, ICI | [1,41], ICI1 | [1,41], ICI2 |
| 960k | CDL-B,20ns | 0.1 | 18.582 | 17.904 | 17.93 | 23.861 | 23.317 | 23.313 |
|  |  | 0.01 | 22 | 21.453 | 21.449 | 28.8 | 27.431 | 27.365 |
|  | CDL-D,20ns | 0.1 | 13.446 | 13.234 | 13.265 | 17.979 | 17.628 | 17.65 |
|  |  | 0.01 | 14.247 | 14.044 | 14.082 | 19.004 | 18.53 | 18.611 |
|  | CDL-D,30ns | 0.1 | 13.446 | 13.234 | 13.269 | 17.977 | 17.633 | 17.653 |
|  |  | 0.01 | 14.242 | 14.042 | 14.072 | 19.014 | 18.506 | 18.619 |

Table 05. SINR in dB achieving PDSCH BLER of 10% /1% with different PTRS pattern and 128RB with power boosting

| SCS | channel | BLER | MCS22 |  |  | MCS26 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | K=4, ICI | [1,33], ICI1 | [1,33], ICI2 | K=4, ICI | [1,33], ICI1 | [1,33], ICI2 |
| 120k | CDL-B, 20ns | 0.1 | 17.402 | 17.037 | 17.139 | -- | -- | -- |
|  |  | 0.01 | 21.253 | 20.912 | 20.897 | -- | -- | -- |
|  | CDL-B, 50ns | 0.1 | 17.189 | 16.689 | 16.849 | -- | -- | -- |
|  |  | 0.01 | 20.598 | 19.857 | 19.929 | -- | -- | -- |
|  | CDL-D, 20ns | 0.1 | 13.024 | 12.744 | 12.83 | -- | -- | -- |
|  |  | 0.01 | 13.84 | 13.558 | 13.669 | -- | -- | -- |
|  | CDL-D, 30ns | 0.1 | 13.023 | 12.741 | 12.827 | -- | -- | -- |
|  |  | 0.01 | 13.84 | 13.544 | 13.653 | -- | -- | -- |
| 480k | CDL-B, 20ns | 0.1 | 16.485 | 16.083 | 16.258 | -- | -- | -- |
|  |  | 0.01 | 19.668 | 19.143 | 19.318 | -- | -- | -- |
|  | CDL-B, 50ns | 0.1 | 16.6 | 16.286 | 16.459 | -- | -- | -- |
|  |  | 0.01 | 19.118 | 18.527 | 18.794 | -- | -- | -- |
|  | CDL-D, 20ns | 0.1 | 12.78 | 12.566 | 12.665 | -- | -- | -- |
|  |  | 0.01 | 13.507 | 13.316 | 13.453 | -- | -- | -- |
|  | CDL-D, 30ns | 0.1 | 12.778 | 12.56 | 12.67 | -- | -- | -- |
|  |  | 0.01 | 13.507 | 13.31 | 13.454 | -- | -- | -- |
| 960k | CDL-B,20ns | 0.1 | 16.313 | 16.19 | 16.209 | 21.793 | 21.21 | 21.297 |
|  |  | 0.01 | 19.238 | 18.931 | 18.939 | 25.414 | 24.54 | 24.54 |
|  | CDL-D,20ns | 0.1 | 12.778 | 12.722 | 12.743 | 17.663 | 17.207 | 17.26 |
|  |  | 0.01 | 13.517 | 13.495 | 13.531 | 18.582 | 18.069 | 18.122 |
|  | CDL-D,30ns | 0.1 | 12.786 | 12.72 | 12.746 | 17.677 | 17.21 | 17.257 |
|  |  | 0.01 | 13.51 | 13.489 | 13.537 | 18.555 | 18.082 | 18.133 |

Table 06. SINR in dB achieving PDSCH BLER of 10% /1% with different PTRS pattern and 64RB with power boosting

| SCS | channel | BLER | MCS22 |  |  | MCS26 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | K=4, ICI | [1,17], ICI1 | [1,17], ICI2 | K=4, ICI | [1,17], ICI1 | [1,17], ICI2 |
| 120k | CDL-B, 20ns | 0.1 | 17.661 | 16.72 | 16.887 | -- | -- | -- |
|  |  | 0.01 | 23.2 | 20.749 | 21.066 | -- | -- | -- |
|  | CDL-B, 50ns | 0.1 | 18.541 | 16.988 | 17.2 | -- | -- | -- |
|  |  | 0.01 | 22 | 20.113 | 20.676 | -- | -- | -- |
|  | CDL-D, 20ns | 0.1 | 13.896 | 13.144 | 13.312 | -- | -- | -- |
|  |  | 0.01 | 14.837 | 14.067 | 14.286 | -- | -- | -- |
|  | CDL-D, 30ns | 0.1 | 13.903 | 13.146 | 13.313 | -- | -- | -- |
|  |  | 0.01 | 14.843 | 14.076 | 14.256 | -- | -- | -- |
| 480k | CDL-B, 20ns | 0.1 | 16.585 | 16.266 | 16.464 | 23.15 | 21.574 | 21.762 |
|  |  | 0.01 | 19.143 | 18.923 | 19.358 | 28 | 24.685 | 24.982 |
|  | CDL-B, 50ns | 0.1 | 16.555 | 16.382 | 16.629 | 22.993 | 21.65 | 21.866 |
|  |  | 0.01 | 19.386 | 19.296 | 19.829 | 27.071 | 25.06 | 25.155 |
|  | CDL-D, 20ns | 0.1 | 13.16 | 13.001 | 13.139 | 18.282 | 17.476 | 17.651 |
|  |  | 0.01 | 13.927 | 13.814 | 14.054 | 19.419 | 18.595 | 18.898 |
|  | CDL-D, 30ns | 0.1 | 13.156 | 12.999 | 13.134 | 18.278 | 17.482 | 17.653 |
|  |  | 0.01 | 13.938 | 13.831 | 14.045 | 19.438 | 18.594 | 18.916 |
| 960k | CDL-B,20ns | 0.1 | 16.764 | 16.496 | 16.722 | 23.483 | 21.636 | 21.879 |
|  |  | 0.01 | 20.446 | 19.368 | 19.64 | 28.4 | 24.679 | 24.854 |
|  | CDL-D,20ns | 0.1 | 13.216 | 13.041 | 13.197 | 18.527 | 17.51 | 17.678 |
|  |  | 0.01 | 13.986 | 13.857 | 14.212 | 19.676 | 18.506 | 18.74 |
|  | CDL-D,30ns | 0.1 | 13.216 | 13.044 | 13.198 | 18.521 | 17.513 | 17.686 |
|  |  | 0.01 | 13.992 | 13.875 | 14.2 | 19.649 | 18.481 | 18.744 |

Table 07. SINR in dB achieving PUSCH BLER of 10% /1% for DFT-s-OFDM and different PTRS patterns

| MCS | Channel | 120 kHz,256RB |  |  | 480 kHz,256RB |  |  | 960 kHz,160RB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (CN, CS) = 8*4 | (CN, CS) = 16*2 | (CN, CS) = 16*4 | (CN, CS) = 8*4 | (CN, CS) = 16*2 | (CN, CS) = 16*4 | (CN, CS) = 4*4 | (CN, CS) = 8*4 | (CN, CS) = 16*2 |
| 16 | CDL-B, 20ns | 12.03 /13.38 | 12.10 /13.39 | 11.69 /12.99 | 11.95 /13.21 | 12.07 /12.99 | 11.80 /12.92 | 13.60 /16.97 | 13.37 /16.81 | 13.56 /16.76 |
|  | CDL-D, 20ns | 6.54 /6.94 | 6.61 /6.97 | 6.15 /6.62 | 6.42 /6.82 | 6.61 /7.05 | 6.21 /6.55 | 7.93 /8.77 | 7.58 /8.30 | 7.82 /8.58 |
|  | CDL-D, 30ns | 6.54 /6.95 | 6.61 /9.97 | 6.15 /6.62 | 6.42 /6.83 | 6.62 /7.05 | 6.21 /6.52 | 7.94 /8.78 | 7.59 /8.33 | 7.82 /8.59 |
| 22 | CDL-B, 20ns | 20.46 /NAN | 17.64 /19.46 | 17.78 /19.75 | 19.14 /27.12 | 17.72 /19.16 | 17.82 /19.44 | 20.95 /29.10 | 19.60 /23.34 | 19.36 /22.83 |
|  | CDL-D, 20ns | 14.24 /17.89 | 11.73 /12.21 | 11.84 /12.56 | 12.66 /14.83 | 11.64 /12.07 | 11.63 /12.28 | 14.42 /20.40 | 13.20 /14.16 | 12.83 /13.61 |
|  | CDL-D, 30ns | 14.26 /17.87 | 11.73 /12.22 | 11.83 /12.56 | 12.66 /14.78 | 11.64 /12.06 | 11.63 /12.26 | 14.44 /20.40 | 13.19 /14.16 | 12.83 /13.62 |
| 26 | CDL-B, 20ns | NAN /NAN | 22.46 /24.65 | NAN /NAN | NAN /NAN | 22.22 /24.10 | 29.40 /NAN | NAN /NAN | 29.73 /NAN | 23.89 /27.46 |
|  | CDL-D, 20ns | NAN /NAN | 16.54 /17.24 | 21.38 /NAN | NAN /NAN | 15.95 /16.40 | 18.31 /NAN | NAN /NAN | 22.79 /NAN | 17.07 /17.89 |
|  | CDL-D, 30ns | NAN /NAN | 16.54 /17.25 | 21.42 /NAN | NAN /NAN | 15.96 /16.40 | 18.33 /NAN | NAN /NAN | 22.77 /NAN | 17.08 /17.89 |

Table 08. SINR in dB achieving PUSCH BLER of 10% /1% for DFT-s-OFDM with different PTRS location and Rx advance shift

| MCS | Channel | 120 kHz,64RB |  |  | 480 kHz,64RB@1600 MHz |  |  | 960 kHz,64RB@1960 MHz |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | old_location+0%shift | old_location+10%shift | new_location+10%shift | old_location+0%shift | old_location+10%shift | new_location+10%shift | old_location+0%shift | old_location+10%shift | new_location+10%shift |
| 16 | CDL-B, 20ns | 11.17 /14.66 | 11.35 /14.82 | 11.03 /14.61 | 11.28 /13.97 | 11.56 /14.11 | 11.28 /13.88 | 11.48 /14.94 | 11.55 /14.94 | 11.43 /14.93 |
|  | CDL-D, 20ns | 7.32 /8.05 | 7.52 /8.31 | 7.21 /7.88 | 6.93 /7.57 | 7.14 /7.85 | 6.93 /7.48 | 6.93 /7.54 | 7.05 /7.65 | 6.94 /7.55 |
|  | CDL-D, 30ns | 7.31 /8.05 | 7.51 /8.25 | 7.21 /7.88 | 6.93 /7.54 | 7.15 /7.82 | 6.94 /7.46 | 6.92 /7.56 | 7.06 /7.70 | 6.92 /7.55 |
| 22 | CDL-B, 20ns | 18.10 /22.16 | 20.27 /28.22 | 17.32 /21.02 | 17.50 /20.29 | 18.73 /21.96 | 17.36 /20.18 | 17.86 /20.98 | 18.27 /21.45 | 17.69 /20.79 |
|  | CDL-D, 20ns | 14.35 /16.45 | 17.21 /25.05 | 13.37 /14.37 | 12.22 /12.95 | 13.82 /16.58 | 12.16 /12.87 | 12.15 /12.77 | 12.78 /13.72 | 12.10 /12.73 |
|  | CDL-D, 30ns | 14.36 /16.43 | 17.19 /25.05 | 13.37 /14.34 | 12.21 /12.97 | 13.80 /16.50 | 12.16 /12.83 | 12.14 /12.76 | 12.78 /13.74 | 12.11 /12.72 |
| 26 | CDL-B, 20ns | 22.02 /26.25 | NAN /NAN | 21.52 /25.53 | 23.06 /26.85 | NAN /NAN | 22.31 /25.37 | 23.67 /NAN | NAN /NAN | 22.25 /25.32 |
|  | CDL-D, 20ns | 18.12 /20.28 | NAN /NAN | 17.55 /18.53 | 17.38 /18.62 | NAN /NAN | 16.86 /17.65 | 16.65 /17.43 | NAN /NAN | 16.40 /17.09 |
|  | CDL-D, 30ns | 18.12/ 20.11 | NAN /NAN | 17.61 /18.55 | 17.38 /18.62 | NAN /NAN | 16.86 /17.65 | 16.64 /17.43 | NAN /NAN | 16.40 /17.08 |

- Appendix B.

Table 09. System level evaluation results for indoor scenario A (no-LBT and omni-directional LBT)

| Cases |  | no-LBT |  |  | omni-directional LBT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
| DL UPT (Mbps) | 5%ile | 1570.8 | 456.8 | 85.7 | 1506.8 | 490.0 | 95.9 |
|  | 50%ile | 5647.9 | 3130.5 | 1630.7 | 5656.8 | 3153.5 | 1712.7 |
|  | 95%ile | 8913.9 | 8901.2 | 8155.6 | 8929.5 | 8914.1 | 8080.0 |
|  | mean | 5424.9 | 3894.8 | 2388.7 | 5407.0 | 3938.5 | 2452.9 |
| DL delay (s) | 5%ile | 0.0242 | 0.0243 | 0.0264 | 0.0242 | 0.0242 | 0.0258 |
|  | 50%ile | 0.0380 | 0.0682 | 0.1298 | 0.0381 | 0.0676 | 0.1236 |
|  | 95%ile | 0.1415 | 0.4434 | 1.3619 | 0.1433 | 0.4190 | 1.2857 |
|  | mean | 0.0571 | 0.1383 | 0.4710 | 0.0570 | 0.1318 | 0.4465 |
| UL UPT (Mbps) | 5%ile | 915.6 | 310.5 | 86.7 | 899.0 | 329.8 | 89.8 |
|  | 50%ile | 2740.6 | 2070.7 | 1329.6 | 2700.5 | 2060.4 | 1372.9 |
|  | 95%ile | 4500.4 | 4235.8 | 3676.1 | 4429.1 | 4175.5 | 3678.3 |
|  | mean | 2680.1 | 2106.7 | 1521.8 | 2635.4 | 2090.3 | 1538.0 |
| UL delay (s) | 5%ile | 0.0480 | 0.0498 | 0.0565 | 0.0487 | 0.0505 | 0.0572 |
|  | 50%ile | 0.0783 | 0.1034 | 0.1549 | 0.0798 | 0.1038 | 0.1527 |
|  | 95%ile | 0.2326 | 0.6326 | 1.4036 | 0.2360 | 0.6097 | 1.3439 |
|  | mean | 0.1013 | 0.2031 | 0.4990 | 0.1027 | 0.2028 | 0.4878 |
| Arrival rate (files/s) |  | 0.4 | 0.8 | 1.2 | 0.4 | 0.8 | 1.2 |
| 𝜌DL |  | 99.60% | 98.78% | 92.27% | 99.59% | 98.90% | 92.45% |
| 𝜌UL |  | 99.04% | 98.39% | 91.80% | 99.07% | 98.40% | 92.02% |
| BO |  | 22% | 48% | 70% | 22% | 48% | 69% |
| Additional report/notes: BW = 2GHz, FTP3 traffic file size = 27Mbyte, Fixed CW = 5, two operator, MCOT=5ms; BS to UE: InH open office channel. Antenna Configuration: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), Dh = Dv = 0.5 λ for gNB. (M, N, P, Mg, Ng) = (2, 2, 2, 1, 2), Dh = Dv = 0.5 λ for UE. MIMO: single layer |  |  |  |  |  |  |  |

Table 10. System level evaluation results for indoor scenario A (directional LBT and receiver assisted LBT)

| Cases |  | directional LBT schemes |  |  | receiver-assisted LBT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
| DL UPT (Mbps) | 5%ile | 1536.6 | 486.6 | 101.3 | 1572.3 | 559.5 | 191.7 |
|  | 50%ile | 5728.0 | 3148.4 | 1715.4 | 5610.2 | 3186.3 | 1880.3 |
|  | 95%ile | 8929.2 | 8912.7 | 8057.0 | 8359.5 | 8337.7 | 7802.4 |
|  | mean | 5432.0 | 3928.0 | 2446.7 | 5304.3 | 3873.5 | 2494.2 |
| DL delay (s) | 5%ile | 0.0242 | 0.0242 | 0.0257 | 0.0258 | 0.0259 | 0.0269 |
|  | 50%ile | 0.0375 | 0.0676 | 0.1225 | 0.0383 | 0.0669 | 0.1129 |
|  | 95%ile | 0.1403 | 0.4190 | 1.1880 | 0.1367 | 0.3723 | 0.8499 |
|  | mean | 0.0564 | 0.1322 | 0.4309 | 0.0557 | 0.1228 | 0.3813 |
| UL UPT (Mbps) | 5%ile | 896.6 | 318.9 | 82.0 | 961.9 | 376.6 | 145.6 |
|  | 50%ile | 2703.0 | 2053.8 | 1350.8 | 2741.9 | 2125.2 | 1435.6 |
|  | 95%ile | 4427.9 | 4156.9 | 3674.3 | 4429.3 | 4331.7 | 3756.6 |
|  | mean | 2639.0 | 2086.1 | 1521.1 | 2698.9 | 2157.0 | 1607.9 |
| UL delay (s) | 5%ile | 0.0487 | 0.0504 | 0.0570 | 0.0487 | 0.0493 | 0.0565 |
|  | 50%ile | 0.0795 | 0.1042 | 0.1550 | 0.0781 | 0.1008 | 0.1463 |
|  | 95%ile | 0.2359 | 0.6222 | 1.3580 | 0.2160 | 0.5311 | 1.0102 |
|  | mean | 0.1025 | 0.2022 | 0.4940 | 0.0989 | 0.1838 | 0.4269 |
| Arrival rate (files/s) |  | 0.4 | 0.8 | 1.2 | 0.4 | 0.8 | 1.2 |
| 𝜌DL |  | 99.60% | 98.89% | 92.80% | 99.59% | 98.98% | 93.54% |
| 𝜌UL |  | 99.07% | 98.44% | 91.67% | 99.10% | 98.64% | 93.06% |
| BO |  | 22% | 48% | 70% | 23% | 48% | 70% |
| Additional report/notes: BW = 2GHz, FTP3 traffic file size = 27Mbyte, Fixed CW = 127, two operator, MCOT=5ms; BS to UE: InH open office channel. Antenna Configuration: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), Dh = Dv = 0.5 λ for gNB. (M, N, P, Mg, Ng) = (2, 2, 2, 1, 2), Dh = Dv = 0.5 λ for UE. MIMO: single layer |  |  |  |  |  |  |  |

Table 11. System level evaluation results for indoor scenario A (receiver-only directional LBT)

| Cases |  | receiver-only directional LBT |  |  |
| --- | --- | --- | --- | --- |
| Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
| DL UPT (Mbps) | 5%ile | 1694.6 | 624.5 | 237.6 |
|  | 50%ile | 6044.3 | 3499.3 | 2035.2 |
|  | 95%ile | 8957.8 | 8858.4 | 8566.0 |
|  | mean | 5675.1 | 4170.1 | 2715.7 |
| DL delay (s) | 5%ile | 0.0241 | 0.0244 | 0.0248 |
|  | 50%ile | 0.0356 | 0.0609 | 0.1104 |
|  | 95%ile | 0.1273 | 0.3372 | 0.7293 |
|  | mean | 0.0521 | 0.1135 | 0.3369 |
| UL UPT (Mbps) | 5%ile | 973.0 | 383.6 | 151.5 |
|  | 50%ile | 2762.1 | 2179.3 | 1472.7 |
|  | 95%ile | 4431.0 | 4335.5 | 3787.7 |
|  | mean | 2708.9 | 2186.9 | 1637.6 |
| UL delay (s) | 5%ile | 0.0487 | 0.0492 | 0.0559 |
|  | 50%ile | 0.0776 | 0.0986 | 0.1429 |
|  | 95%ile | 0.2122 | 0.5235 | 0.9657 |
|  | mean | 0.0983 | 0.1802 | 0.4157 |
| Arrival rate (files/s) |  | 0.4 | 0.8 | 1.2 |
| 𝜌DL |  | 99.63% | 99.15% | 94.43% |
| 𝜌UL |  | 99.10% | 98.67% | 93.34% |
| BO |  | 22% | 47% | 68% |
| Additional report/notes: BW = 2GHz, FTP3 traffic file size = 27Mbyte, two operator, MCOT=5ms; BS to UE: InH open office channel. Antenna Configuration: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), Dh = Dv = 0.5 λ for gNB. (M, N, P, Mg, Ng) = (2, 2, 2, 1, 2), Dh = Dv = 0.5 λ for UE. MIMO: single layer |  |  |  |  |

Table 12. System level evaluation results for indoor scenario A (no-LBT and omni-directional LBT)

| Cases |  | no-LBT |  |  | omni-directional LBT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
| DL UPT (Mbps) | 5%ile | 1521.7 | 441.2 | 34.7 | 1583.4 | 456.9 | 52.4 |
|  | 50%ile | 5043.8 | 2906.9 | 1145.7 | 5053.4 | 3015.4 | 1223.9 |
|  | 95%ile | 8910.1 | 8626.6 | 6789.3 | 8927.6 | 8851.4 | 6963.3 |
|  | mean | 5083.6 | 3517.4 | 1842.4 | 5149.1 | 3590.2 | 1964.1 |
| DL delay (s) | 5%ile | 0.0242 | 0.0249 | 0.0313 | 0.0242 | 0.0243 | 0.0304 |
|  | 50%ile | 0.0427 | 0.0732 | 0.1813 | 0.0424 | 0.0707 | 0.1684 |
|  | 95%ile | 0.1399 | 0.4582 | 1.8838 | 0.1346 | 0.4381 | 1.7086 |
|  | mean | 0.0581 | 0.1387 | 0.6160 | 0.0569 | 0.1359 | 0.5564 |
| UL UPT (Mbps) | 5%ile | 923.9 | 301.5 | 44.5 | 902.6 | 334.5 | 40.9 |
|  | 50%ile | 2410.6 | 1716.5 | 946.9 | 2379.6 | 1784.9 | 970.7 |
|  | 95%ile | 4447.3 | 3841.6 | 3240 | 4383.4 | 3804.6 | 3236.9 |
|  | mean | 2474.2 | 1882.6 | 1208.4 | 2436.5 | 1884.3 | 1218.8 |
| UL delay (s) | 5%ile | 0.0481 | 0.0557 | 0.0650 | 0.0491 | 0.0557 | 0.0648 |
|  | 50%ile | 0.0890 | 0.1206 | 0.2171 | 0.0898 | 0.1199 | 0.2120 |
|  | 95%ile | 0.2269 | 0.6403 | 2.002 | 0.2325 | 0.5775 | 2.0632 |
|  | mean | 0.1066 | 0.2054 | 0.6635 | 0.1082 | 0.1986 | 0.6692 |
| Arrival rate (files/s) |  | 0.25 | 0.6 | 1.0 | 0.25 | 0.6 | 1.0 |
| 𝜌DL |  | 99.70% | 98.67% | 89.27% | 99.68% | 98.43% | 89.98% |
| 𝜌UL |  | 98.79% | 98.26% | 89.66% | 98.79% | 98.08% | 89.49% |
| BO |  | 16% | 45% | 74% | 16% | 44% | 72% |
| Additional report/notes: BW = 2GHz, FTP3 traffic file size = 27Mbyte, Fixed CW = 5, two operator, MCOT=5ms; BS to UE: InH open office channel. Antenna Configuration: (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), Dh = Dv = 0.5 λ for gNB. (M, N, P, Mg, Ng) = (1, 2, 2, 1, 2), Dh = Dv = 0.5 λ for UE. MIMO: single layer |  |  |  |  |  |  |  |

Table 13. System level evaluation results for indoor scenario A (directional LBT and receiver assisted LBT)

| Cases |  | directional LBT schemes |  |  | receiver-assisted LBT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
| DL UPT (Mbps) | 5%ile | 1581.3 | 328.9 | 39.3 | 1591.0 | 506.1 | 88.9 |
|  | 50%ile | 4862.9 | 2696.7 | 1178.4 | 5149.8 | 3037.9 | 1400 |
|  | 95%ile | 8923.7 | 8841.6 | 6820.2 | 8348.1 | 8284.5 | 6685.9 |
|  | mean | 5070.8 | 3423.3 | 1852.7 | 5044.8 | 3568.9 | 2033.6 |
| DL delay (s) | 5%ile | 0.0242 | 0.0243 | 0.0313 | 0.0259 | 0.0260 | 0.0317 |
|  | 50%ile | 0.0439 | 0.0794 | 0.1754 | 0.0418 | 0.0698 | 0.1489 |
|  | 95%ile | 0.1337 | 0.5506 | 1.7758 | 0.1335 | 0.3909 | 1.2342 |
|  | mean | 0.0569 | 0.1625 | 0.6028 | 0.0563 | 0.1259 | 0.4852 |
| UL UPT (Mbps) | 5%ile | 913.6 | 282.4 | 16.6 | 987.4 | 366.3 | 76.4 |
|  | 50%ile | 2359.1 | 1695.8 | 950.9 | 2427.1 | 1861.2 | 1067.5 |
|  | 95%ile | 4346.9 | 3780.7 | 3142.6 | 4388.0 | 3823.8 | 3347.9 |
|  | mean | 2410.8 | 1829.2 | 1193.5 | 2485.1 | 1942.1 | 1299.1 |
| UL delay (s) | 5%ile | 0.0492 | 0.0568 | 0.0663 | 0.0491 | 0.0548 | 0.0629 |
|  | 50%ile | 0.0913 | 0.1250 | 0.2177 | 0.0870 | 0.1148 | 0.1953 |
|  | 95%ile | 0.2320 | 0.6401 | 2.1766 | 0.2158 | 0.5371 | 1.5271 |
|  | mean | 0.1092 | 0.2203 | 0.6956 | 0.1047 | 0.1871 | 0.5598 |
| Arrival rate (files/s) |  | 0.25 | 0.6 | 1.0 | 0.25 | 0.6 | 1.0 |
| 𝜌DL |  | 99.70% | 98.34% | 89.19% | 99.68% | 98.60% | 91.22% |
| 𝜌UL |  | 99.00% | 97.86% | 88.13% | 98.79% | 98.37% | 91.28% |
| BO |  | 16% | 46% | 73% | 16% | 44% | 72% |
| Additional report/notes: BW = 2GHz, FTP3 traffic file size = 27Mbyte, Fixed CW = 127, two operator, MCOT=5ms; BS to UE: InH open office channel. Antenna Configuration: (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), Dh = Dv = 0.5 λ for gNB. (M, N, P, Mg, Ng) = (1, 2, 2, 1, 2), Dh = Dv = 0.5 λ for UE. MIMO: single layer |  |  |  |  |  |  |  |

Table 14. System level evaluation results for indoor scenario A (receiver-only directional LBT)

| Cases |  | receiver-only directional LBT |  |  |
| --- | --- | --- | --- | --- |
| Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
| DL UPT (Mbps) | 5%ile | 1782.7 | 586.0 | 135.6 |
|  | 50%ile | 5568.2 | 3326.4 | 1514.8 |
|  | 95%ile | 8863.0 | 8813.6 | 7150.0 |
|  | mean | 5402.7 | 3857.1 | 2215.7 |
| DL delay (s) | 5%ile | 0.0244 | 0.0245 | 0.0297 |
|  | 50%ile | 0.0386 | 0.0634 | 0.1372 |
|  | 95%ile | 0.1195 | 0.3514 | 1.0153 |
|  | mean | 0.0517 | 0.1148 | 0.4364 |
| UL UPT (Mbps) | 5%ile | 999.6 | 397.9 | 90.4 |
|  | 50%ile | 2462.5 | 1869.8 | 1105.4 |
|  | 95%ile | 4386.4 | 3822.1 | 3385.8 |
|  | mean | 2494.4 | 1967.4 | 1340.0 |
| UL delay (s) | 5%ile | 0.0491 | 0.0543 | 0.0618 |
|  | 50%ile | 0.0866 | 0.1143 | 0.1875 |
|  | 95%ile | 0.2096 | 0.4987 | 1.3874 |
|  | mean | 0.1038 | 0.1805 | 0.5289 |
| Arrival rate (files/s) |  | 0.25 | 0.6 | 1.0 |
| 𝜌DL |  | 99.70% | 98.84% | 92.39% |
| 𝜌UL |  | 98.79% | 98.43% | 91.78% |
| BO |  | 16% | 43% | 70% |
| Additional report/notes: BW = 2GHz, FTP3 traffic file size = 27Mbyte, two operator, MCOT=5ms; BS to UE: InH open office channel. Antenna Configuration: (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), Dh = Dv = 0.5 λ for gNB. (M, N, P, Mg, Ng) = (1, 2, 2, 1, 2), Dh = Dv = 0.5 λ for UE. MIMO: single layer |  |  |  |  |

- Appendix C.

Table 15. Link budget for PF0 with 120kHz SCS & short repetition sequence

| Format | Regulation | Payload Size [bit] | Number of PRB | SCS [kHz] | Required SNR [dB] | Noise level [dBm] | 95%CM [dB] | Tx power [dBm] | MIL [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PF0 | EU | 2 | 4 | 120 | -4.4 | -99.4 | 3.30 | 19.00 | 147.5 |
| PF0 | EU | 2 | 8 | 120 | -7.4 | -96.4 | 2.37 | 19.00 | 148.4 |
| PF0 | USA | 2 | 4 | 120 | -4.4 | -99.4 | 3.30 | 14.60 | 144.4 |
| PF0 | USA | 2 | 8 | 120 | -7.4 | -96.4 | 2.37 | 17.61 | 147.4 |

Table 16. Link budget for PF0/1/4 with 120kHz SCS & long sequence

| Format | Regulation | Payload Size [bit] | Number of PRB | SCS [kHz] | Required SNR [dB] | Noise level [dBm] | 95%CM [dB] | Tx power [dBm] | MIL [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PF0 | EU | 2 | 1 | 120 | 1.5 | -105.4 | 0.67 | 18.58 | 148.5 |
| PF0 | EU | 2 | 2 | 120 | -1.7 | -102.4 | 0.78 | 19.00 | 149.1 |
| PF0 | EU | 2 | 4 | 120 | -4.5 | -99.4 | 1.78 | 19.00 | 148.9 |
| PF0 | EU | 2 | 8 | 120 | -7.4 | -96.4 | 1.59 | 19.00 | 148.8 |
| PF1 | EU | 2 | 1 | 120 | -6.0 | -105.4 | 0.67 | 18.58 | 156.0 |
| PF1 | EU | 2 | 2 | 120 | -8.0 | -102.4 | 0.78 | 19.00 | 155.4 |
| PF1 | EU | 2 | 4 | 120 | -11.5 | -99.4 | 1.78 | 19.00 | 155.9 |
| PF1 | EU | 2 | 8 | 120 | -14.8 | -96.4 | 1.59 | 19.00 | 156.2 |
| PF4 | EU | 4 | 1 | 120 | -5.0 | -105.4 | 2.53 | 18.47 | 154.9 |
| PF4 | EU | 4 | 2 | 120 | -8.0 | -102.4 | 2.39 | 18.61 | 155.0 |
| PF4 | EU | 4 | 4 | 120 | -11.2 | -99.4 | 2.27 | 18.73 | 155.3 |
| PF4 | EU | 4 | 8 | 120 | -14.1 | -96.4 | 2.23 | 18.77 | 155.3 |
| PF4 | EU | 11 | 1 | 120 | -1.8 | -105.4 | 2.53 | 18.47 | 151.7 |
| PF4 | EU | 11 | 2 | 120 | -4.5 | -102.4 | 2.39 | 18.61 | 151.5 |
| PF4 | EU | 11 | 4 | 120 | -7.8 | -99.4 | 2.27 | 18.73 | 151.9 |
| PF4 | EU | 11 | 8 | 120 | -10.8 | -96.4 | 2.23 | 18.77 | 152.0 |
| PF4 | EU | 22 | 1 | 120 | 1.9 | -105.4 | 2.53 | 18.47 | 148.0 |
| PF4 | EU | 22 | 2 | 120 | -1.4 | -102.4 | 2.39 | 18.61 | 148.4 |
| PF4 | EU | 22 | 4 | 120 | -4.4 | -99.4 | 2.27 | 18.73 | 148.5 |
| PF4 | EU | 22 | 8 | 120 | -7.7 | -96.4 | 2.23 | 18.77 | 148.9 |
| PF0 | USA | 2 | 1 | 120 | 1.5 | -105.4 | 0.67 | 8.58 | 138.5 |
| PF0 | USA | 2 | 2 | 120 | -1.7 | -102.4 | 0.78 | 11.59 | 141.7 |
| PF0 | USA | 2 | 4 | 120 | -4.5 | -99.4 | 1.78 | 14.60 | 144.5 |
| PF0 | USA | 2 | 8 | 120 | -7.4 | -96.4 | 1.59 | 17.61 | 147.4 |
| PF1 | USA | 2 | 1 | 120 | -6.0 | -105.4 | 0.67 | 8.58 | 146.0 |
| PF1 | USA | 2 | 2 | 120 | -8.0 | -102.4 | 0.78 | 11.59 | 148.0 |
| PF1 | USA | 2 | 4 | 120 | -11.5 | -99.4 | 1.78 | 14.60 | 151.5 |
| PF1 | USA | 2 | 8 | 120 | -14.8 | -96.4 | 1.59 | 17.61 | 154.8 |
| PF4 | USA | 4 | 1 | 120 | -5.0 | -105.4 | 2.53 | 8.58 | 145.0 |
| PF4 | USA | 4 | 2 | 120 | -8.0 | -102.4 | 2.39 | 11.59 | 148.0 |
| PF4 | USA | 4 | 4 | 120 | -11.2 | -99.4 | 2.27 | 14.60 | 151.2 |
| PF4 | USA | 4 | 8 | 120 | -14.1 | -96.4 | 2.23 | 17.61 | 154.1 |
| PF4 | USA | 11 | 1 | 120 | -1.8 | -105.4 | 2.53 | 8.58 | 141.8 |
| PF4 | USA | 11 | 2 | 120 | -4.5 | -102.4 | 2.39 | 11.59 | 144.5 |
| PF4 | USA | 11 | 4 | 120 | -7.8 | -99.4 | 2.27 | 14.60 | 147.8 |
| PF4 | USA | 11 | 8 | 120 | -10.8 | -96.4 | 2.23 | 17.61 | 150.8 |
| PF4 | USA | 22 | 1 | 120 | 1.9 | -105.4 | 2.53 | 8.58 | 138.1 |
| PF4 | USA | 22 | 2 | 120 | -1.4 | -102.4 | 2.39 | 11.59 | 141.4 |
| PF4 | USA | 22 | 4 | 120 | -4.4 | -99.4 | 2.27 | 14.60 | 144.4 |
| PF4 | USA | 22 | 8 | 120 | -7.7 | -96.4 | 2.23 | 17.61 | 147.7 |

Table 17. Link budget for PF0/1/4 with 480kHz SCS & long sequence

| Format | Regulation | Payload Size [bit] | Number of PRB | SCS [kHz] | Required SNR [dB] | Noise level [dBm] | 95%CM [dB] | Tx power [dBm] | MIL [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PF0 | EU | 2 | 1 | 480 | 0.3 | -99.4 | 0.67 | 19.00 | 144.1 |
| PF0 | EU | 2 | 2 | 480 | -3.0 | -96.4 | 0.78 | 19.00 | 144.4 |
| PF0 | EU | 2 | 4 | 480 | -5.3 | -93.4 | 1.78 | 19.00 | 143.7 |
| PF1 | EU | 2 | 1 | 480 | -5.9 | -99.4 | 0.67 | 19.00 | 150.3 |
| PF1 | EU | 2 | 2 | 480 | -9.0 | -96.4 | 0.78 | 19.00 | 150.4 |
| PF1 | EU | 2 | 4 | 480 | -11.6 | -93.4 | 1.78 | 19.00 | 150.0 |
| PF4 | EU | 4 | 1 | 480 | -6.5 | -99.4 | 2.53 | 18.47 | 150.4 |
| PF4 | EU | 4 | 2 | 480 | -9.5 | -96.4 | 2.39 | 18.61 | 150.5 |
| PF4 | EU | 4 | 4 | 480 | -12.0 | -93.4 | 2.27 | 18.73 | 150.1 |
| PF4 | EU | 11 | 1 | 480 | -3.0 | -99.4 | 2.53 | 18.47 | 146.9 |
| PF4 | EU | 11 | 2 | 480 | -6.0 | -96.4 | 2.39 | 18.61 | 147.0 |
| PF4 | EU | 11 | 4 | 480 | -8.5 | -93.4 | 2.27 | 18.73 | 146.0 |
| PF4 | EU | 22 | 1 | 480 | 0.0 | -99.4 | 2.53 | 18.47 | 143.9 |
| PF4 | EU | 22 | 2 | 480 | -3.5 | -96.4 | 2.39 | 18.61 | 144.5 |
| PF4 | EU | 22 | 4 | 480 | -5.0 | -93.4 | 2.27 | 18.73 | 143.1 |
| PF0 | USA | 2 | 1 | 480 | 0.3 | -99.4 | 0.67 | 14.60 | 139.7 |
| PF0 | USA | 2 | 2 | 480 | -3.0 | -96.4 | 0.78 | 17.61 | 143.0 |
| PF0 | USA | 2 | 4 | 480 | -5.3 | -93.4 | 1.78 | 19.00 | 143.7 |
| PF1 | USA | 2 | 1 | 480 | -5.9 | -99.4 | 0.67 | 14.60 | 145.9 |
| PF1 | USA | 2 | 2 | 480 | -9.0 | -96.4 | 0.78 | 17.61 | 149.0 |
| PF1 | USA | 2 | 4 | 480 | -11.6 | -93.4 | 1.78 | 19.00 | 150.0 |
| PF4 | USA | 4 | 1 | 480 | -6.5 | -99.4 | 2.53 | 14.60 | 146.5 |
| PF4 | USA | 4 | 2 | 480 | -9.5 | -96.4 | 2.39 | 17.61 | 149.5 |
| PF4 | USA | 4 | 4 | 480 | -12.0 | -93.4 | 2.27 | 18.73 | 150.1 |
| PF4 | USA | 11 | 1 | 480 | -3.0 | -99.4 | 2.53 | 14.60 | 143.0 |
| PF4 | USA | 11 | 2 | 480 | -6.0 | -96.4 | 2.39 | 17.61 | 146.0 |
| PF4 | USA | 11 | 4 | 480 | -8.5 | -93.4 | 2.27 | 18.73 | 146.6 |
| PF4 | USA | 22 | 1 | 480 | 0.0 | -99.4 | 2.53 | 14.60 | 140.0 |
| PF4 | USA | 22 | 2 | 480 | -3.5 | -96.4 | 2.39 | 17.61 | 143.5 |
| PF4 | USA | 22 | 4 | 480 | -5.0 | -93.4 | 2.27 | 18.73 | 143.1 |

Table 18. Link budget for PF0/1/4 with 960kHz SCS & long sequence

| Format | Regulation | Payload Size [bit] | Number of PRB | SCS [kHz] | Required SNR [dB] | Noise level [dBm] | 95%CM [dB] | Tx power [dBm] | MIL [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PF0 | EU | 2 | 1 | 960 | 0.4 | -96.4 | 0.67 | 19.00 | 141.0 |
| PF0 | EU | 2 | 2 | 960 | -1.4 | -93.4 | 0.78 | 19.00 | 140.0 |
| PF1 | EU | 2 | 1 | 960 | -5.5 | -96.4 | 0.67 | 19.00 | 146.9 |
| PF1 | EU | 2 | 2 | 960 | -8.5 | -93.4 | 0.78 | 19.00 | 146.9 |
| PF4 | EU | 4 | 1 | 960 | -5.2 | -96.4 | 2.53 | 18.47 | 146.1 |
| PF4 | EU | 4 | 2 | 960 | -8.8 | -93.4 | 2.39 | 18.61 | 146.8 |
| PF4 | EU | 11 | 1 | 960 | -2.0 | -96.4 | 2.53 | 18.47 | 142.9 |
| PF4 | EU | 11 | 2 | 960 | -5.7 | -93.4 | 2.39 | 18.61 | 143.7 |
| PF4 | EU | 22 | 1 | 960 | 1.9 | -96.4 | 2.53 | 18.47 | 139.0 |
| PF4 | EU | 22 | 2 | 960 | -2.9 | -93.4 | 2.39 | 18.61 | 140.9 |
| PF0 | USA | 2 | 1 | 960 | 0.4 | -96.4 | 0.67 | 17.61 | 139.6 |
| PF0 | USA | 2 | 2 | 960 | -1.4 | -93.4 | 0.78 | 19.00 | 139.8 |
| PF1 | USA | 2 | 1 | 960 | -5.5 | -96.4 | 0.67 | 17.61 | 145.5 |
| PF1 | USA | 2 | 2 | 960 | -8.5 | -93.4 | 0.78 | 19.00 | 146.9 |
| PF4 | USA | 4 | 1 | 960 | -5.2 | -96.4 | 2.53 | 17.61 | 145.2 |
| PF4 | USA | 4 | 2 | 960 | -8.8 | -93.4 | 2.39 | 18.61 | 146.8 |
| PF4 | USA | 11 | 1 | 960 | -2.0 | -96.4 | 2.53 | 17.61 | 142.0 |
| PF4 | USA | 11 | 2 | 960 | -5.7 | -93.4 | 2.39 | 18.61 | 143.7 |
| PF4 | USA | 22 | 1 | 960 | 1.9 | -96.4 | 2.53 | 17.61 | 138.1 |
| PF4 | USA | 22 | 2 | 960 | -2.9 | -93.4 | 2.39 | 18.61 | 140.9 |