3GPP TSG-RAN WG1#104b-e			R1-2103408

e-Meeting, April 12th – April 20th, 2021

Agenda item:	8.1.1

Title:	Enhancements on Multi-beam Operation

Source:	Convida Wireless

Document for:	Discussion

# Introduction

Rel-16 eMIMO introduced various enhancements for multi-beam operation. Rel-17 FeMIMO WI will further enhance the multi-beam operation [1].

- Extend specification support in the following areas [RAN1] - Enhancement on multi-beam operation, mainly targeting FR2 while also applicable to FR1: - Identify and specify features to facilitate more efficient (lower latency and overhead) DL/UL beam management to support higher intra- and L1/L2-centric inter-cell mobility and/or a larger number of configured TCI states: - Common beam for data and control transmission/reception for DL and UL, especially for intra-band CA - Unified TCI framework for DL and UL beam indication - Enhancement on signaling mechanisms for the above features to improve latency and efficiency with more usage of dynamic control signaling (as opposed to RRC) - Identify and specify features to facilitate UL beam selection for UEs equipped with multiple panels, considering UL coverage loss mitigation due to MPE, based on UL beam indication with the unified TCI framework for UL fast panel selection

# Discussion

## 2.1 Unified TCI and Common Beam Operation

The following was agreed in RAN1#102-e:

Agreement Note: the enumeration for issues (such as “issue 1a), 1b), 6) in the proposal below refers to the enumeration within the proposals, not Table 1 in the FL summary. - [Issue 1] For Rel.17 NR FeMIMO, on the unified TCI framework - Support joint TCI for DL and UL based on and analogous to Rel.15/16 DL TCI framework - The term “TCI” at least comprises a TCI state that includes at least one source RS to provide a reference (UE assumption) for determining QCL and/or spatial filter - The source reference signal(s) in M TCIs provide common QCL information at least for UE-dedicated reception on PDSCH and all or subset of CORESETs in a CC - FFS: Optionally this common QCL information can also apply to CSI-RS resource for CSI, CSI-RS resource for BM, and CSI-RS for tracking - FFS: Applicability on PDSCH includes PDSCH default beam - Working Assumption: Select between M=1 and M>=1 - The source reference signal(s) in N TCIs provide a reference for determining common UL TX spatial filter(s) at least for dynamic-grant/configured-grant based PUSCH, all or subset of dedicated PUCCH resources in a CC, - Optionally, this UL TX spatial filter can also apply to all SRS resources in resource set(s) configured for antenna switching/codebook-based/non-codebook-based UL transmissions - FFS: applicability of this UL TX spatial filter to SRS configured for beam management (BM) - FFS: PUSCH port determination based on the TCI, e.g., to be mapped with SRS ports analogous to Rel.15/16 - Working Assumption: Select between N=1 and N>=1 - FFS: extension to common QCL information applied to only some of the CORESETs or PUCCH resources in a CC, e.g. for mTRP - FFS: When used for the purpose of joint beam indication for UL and DL, whether a joint TCI pool for DL and UL dedicated for the purpose is used, or the same TCI pool as that used for the purpose of separate DL/UL beam indication is used - Note: The resulting beam indication directly refers to the associated source RS(s) - FFS (RAN1#103-e): Details on extension to intra- and inter-band CA - FFS (RAN1#103-e): The supported number of active TCI states considering factors such as multi-TRP and issue 6 - FFS (RAN1#103-e): Applicable QCL types, and co-existence with DL TCI and spatial relation indication in Rel.15/16 - In RAN1#103-e, investigate, for the purpose of down selection, the following alternatives for accommodating the case of separate beam indication for UL and DL - Alt1. Utilize the joint TCI to include references for both DL and UL beams - Alt2. Utilize two separate TCI states, one for DL and one for UL. The TCI state for the DL is the same as agreed in 1a. The TCI state for the UL can be newly introduced. - Alt 2-1: The UL TCI state is taken from the same pool of TCI states as the DL TCI state - Alt 2-2: The UL TCI state is taken from another pool of TCI states than the DL TCI state - Note: The resulting beam indication directly refers to the associated source RS(s) - FFS (RAN1#103-e): Details on extension to intra- and inter-band CA - Note: This may be related to issue 5 as well as other reasons for different TCIs such as network flexibility/scheduling - Support the use of SSB/CSI-RS for BM and/or SRS for BM as source RS to determine a UL TX spatial filter in the unified TCI framework - Whether the UL TX spatial filter corresponds to UL TCI (separate from DL TCI) depends on the outcome of 1b) above - FFS: Support the use of non-BM CSI-RS and/or non-BM SRS in addition - In RAN1#103-e, decide if SRS for BM can be configured as a source RS to represent a DL RX spatial filter in the unified TCI framework - In RAN1#103-e, decide/finalize all other parameters included in or concurrent with (but not included in) the TCI, e.g. UL-PC-related parameters (involving P0/alpha, PL RS, and/or closed loop index), UL-timing-related parameters - In RAN1#103-e, identify issues pertaining to alignment between DL and UL default beam assumptions using the unified TCI framework

In RAN1#103-e, the following was agreed.

Agreement On Rel-17 unified TCI framework, to accommodate the case of separate beam indication for UL and DL: - Utilize two separate TCI states, one for DL and one for UL. - FFS: Contents of separate UL TCI state - Note: For FR1, UE does not expect UL TCI to provide a reference for determining common UL TX spatial filter(s), if UL TCI is supported for FR1 - For the separate DL TCI: - The source reference signal(s) in M TCIs provide QCL information at least for UE-dedicated reception on PDSCH and for UE-dedicated reception on all or subset of CORESETs in a CC - For the separate UL TCI: - The source reference signal(s) in N TCIs provide a reference for determining common UL TX spatial filter(s) at least for dynamic-grant/configured-grant based PUSCH, all or subset of dedicated PUCCH resources in a CC - Optionally, this UL TX spatial filter can also apply to all SRS resources in resource set(s) configured for antenna switching/codebook-based/non-codebook-based UL transmissions - FFS: Whether the UL TCI state is taken from a common/same or separate TCI state pool from DL TCI state - Note that TCI state pool for joint DL and UL beam indication is still FFS - FFS: Whether Rel.17 supports TCI configured for single channel (e.g. PDSCH only, single CORESET) - Note: This does not preclude the type of UE supporting only 1 beam tracking loop, i.e. UE reports value of 1 in UE FG 2-62. Agreement On Rel-17 unified TCI framework: - A pool of joint DL/UL TCI state is used for joint DL/UL TCI state update (beam indication). - FFS: The pool for separate DL and UL TCI state update (beam indication) - Note: Here, TCI state pool refers to a pool configured via higher-layer (RRC) signaling - FFS: Whether joint TCI may include UL specific parameter(s) such as UL PC/timing parameters, PL RS, panel-related indication,etc. and if it is included, it is used only for UL transmission of the DL and UL transmissions to which the joint TCI is applied

In RAN1#104-e, the following was agreed.

Agreement On Rel.17 unified TCI framework, the supported source/target QCL relations in the current TS38.214 V16.4.0 is supported for QCL Type D. - Note: This implies that the following source RS types for DL QCL (Type D, for DL RX spatial filter reference) information for DL UE-dedicated reception on PDSCH and all/subset of CORESETs are supported: - CSI-RS for beam management - CSI-RS for tracking - FFS (to be decided by RAN1#104bis-e): If SSB, CSI-RS for CSI, and/or SRS for BM are also supported as source RS types Agreement On Rel.17 unified TCI framework, the following source RS types for UL TX spatial filter are supported: - CSI-RS for tracking - Note: SRS for BM, SSB, and CSI-RS for BM have been agreed in RAN1#102-e - FFS (to be decided by RAN1#104bis-e): non-BM CSI-RS other than for tracking, non-BM SRS Agreement On Rel.17 unified TCI framework: - For joint and separate DL/UL TCI, DL large scale QCL properties are inferred from one (qcl-Type1) or two RSs (qcl-Type1 and qcl-Type2) analogous to Rel.15/16 - For joint DL/UL TCI, UL spatial filter is derived from the RS of DL QCL Type D Agreement On Rel.17 unified TCI framework, decide by RAN1#104bis-e: - Whether DL or, if applicable, joint TCI also applies to the following signals. If not, FFS any other enhancement over Rel.15/16: - CSI-RS resources for CSI - Some CSI-RS resources for BM, if so, which ones (e.g. aperiodic, repetition ‘ON’) - CSI-RS for tracking - Whether UL or, if applicable, joint TCI also applies to the following signals - Some SRS resources or resource sets for BM

The main use case for common beam operation is when the same beam is used for UE-specific control and data channels. Overhead and latency can be reduced if a single TCI state activation command can be used to update the beam for the channels/signals using the common beam, instead of one TCI state activation per CORESET and PDSCH.

In typical operation, the CSI-RS resource for CSI should use the same beam that would subsequently be used for PDSCH, in order to guarantee accurate CSI reports. Therefore, as the TCI state for PDSCH is updated based on common beam, it is natural to also update the CSI-RS resource for CSI. It can be noted that (UL) common beam operation is supported for SRS for antenna switching, which is also used for DL CSI.

CSI-RS resources for BM are typically transmitted on different beams than the beam used for PDCCH/PDSCH, perhaps except the CSI-RS resource for BM that is currently used as a serving beam, i.e. used for PDCCH/PDSCH and as a BFD RS. Therefore, it is in general not suitable to apply common QCL information to CSI-RS resources for BM.

Proposal 1: Common QCL information can also apply to CSI-RS resource for CSI.

Common beam operation is meant to support very efficient operation when all control and data channels as well as their associated reference signals use the same beam, i.e. M=1.

If M>1, multiple TCI states, e.g. multiple common beams, are activated, which is already supported by Rel-15/16 operation. For multi-TRP operation, it might be motivated to support M=2, for example to maintain a TCI state per CORESET pool. However, we propose to support at least M=1 initially, and further discuss the support for M=2 at a later stage in Rel-17 if time permits, or in a later release.

Proposal 2: Support at least M=1. Further discuss M=2.

Similarly as for the downlink, the most important case is single common beam operation, i.e. N=1. We propose to proceed with N=1 and come back to N=2 later in Rel-17 if time permits, or in a later release.

Proposal 3: Support at least N=1. Further discuss N=2.

The most attractive (efficient) mode of common beam operation is the use of the same common beam for both DL and UL. For example, DL control and data channels and their associated reference signals as well as UL control and data channels and their associated reference signals use the same CSI-RS resource for BM as spatial source/reference. A beam switch of all those DL/UL channels/signals could potentially be achieved with a single TCI state update, with low latency and overhead.

On the other hand, it should also be possible to activate different TCI states for UL and DL, which for instance may be useful in heterogeneous networks and during MPE events. However, such operation with different TCI states for UL and DL is the exception, while common beam operation for UL and DL is the more common situation. Enhancements should therefore be optimized for joint UL/DL beam indication, while still supporting separate indications.

Note that separate UL/DL beam indication could be used by the network during a time period, e.g. during an MPE event, and joint UL/DL beam indication could be used during other periods. RRC reconfiguration to switch between those modes of operation should be avoided. From a configuration overhead perspective, it is most efficient to use the same TCI state pool for joint and separate UL/DL beam indication.

It was agreed that at least for joint DL/UL TCI, the UL TX beam could be determined from the source RS with QCL-TypeD in the TCI state. Since both DL and UL TCI can be determined from the same TCI state in the case of joint DL/UL TCI, it seems feasible to also determine a separate UL TCI from a TCI state in the same pool.

Proposal 4: For the case of separate DL and UL TCI state update, the UL TCI state is taken from the same TCI state pool as the DL TCI state.

In massive MIMO and multi-TRP deployments, a large number of CSI-RSs for BM is required to ensure that a reported DL beam is close to the optimal DL beam. To some extent, this issue can be alleviated by the iterative beam refinement procedure, which however has the drawback of increased latency and complexity, and potentially increased reconfiguration overhead.

Using SRS for BM as a spatial source RS for DL could be beneficial in the context of massive MIMO and multi-TRP, since the amount of CSI-RS for BM and corresponding measurements and reporting can be reduced.

The beam management latency using CSI-RS involves CSI-RS transmission and UE RX beam adjustments, L1-RSRP/SINR measurement, CSI report encoding/transmission on PUCCH/PUSCH on the UE side, and decoding of CSI report on the gNB side.

The beam management latency using SRS for BM involves SRS transmission on the UE side and gNB RX beam adjustments and measurements on the gNB side.

Hence, beam management latency could be reduced by using SRS for BM as spatial source RS for DL.

In terms of UE power consumption, more SRS transmission may be introduced, but this may be compensated by less CSI measurement and CSI reporting on PUCCH/PUSCH.

Proposal 5: SRS for BM can be configured as a source RS to represent a DL RX spatial filter in the unified TCI framework.

## 2.2 Dynamic TCI State Signaling

The following was agreed in RAN1#103-e:

Agreement On beam indication signaling medium to support joint or separate DL/UL beam indication in Rel.17 unified TCI framework: - Support L1-based beam indication using at least UE-specific (unicast) DCI to indicate joint or separate DL/UL beam indication from the active TCI states - The existing DCI formats 1_1 and 1_2 are reused for beam indication - Support a mechanism for UE to acknowledge successful decoding of beam indication - The ACK/NAK of the PDSCH scheduled by the DCI carrying the beam indication can be used as an ACK also for the DCI - FFS: Whether any additional specification support is needed - Support activation of one or more TCI states via MAC CE analogous to Rel.15/16: - At least for the single activated TCI state, the activated TCI state is applied - The content for the MAC CE is determined based on the outcome of issue 1 - FFS: If supported, default TCI state when more than one TCI states are activated by MAC CE - Note: There is no implications on the support of single TRP or multi-TRP - FFS: Additional enhancement such as L1-based beam indication with group-common DCI - FFS: Whether the Rel.17 beam indication can also apply to beam indication for single channel (e.g. PDSCH only, single CORESET) or a subset of channels - FFS: Additional details on extending the support of L1-based beam indication when separate UL (from DL) common beam indication is configured Agreement In RAN1#104-e, on the Rel-17 L1-based TCI state update (beam indication) for the unified TCI framework, interested companies are to provide the following: - How to use DCI formats 1_1 and 1_2 for UL-only (in case of separate DL/UL) TCI state update (beam indication) - Note: The agreement implies that DCI formats 1_1 and 1_2 can be used for UL-only TCI state update beam indication). - FFS: Using DCI format 1_1 and 1_2 without DL assignment, and with a new acknowledgment mechanism directly in response to decoding DCI format 1_1 and 1_2, e.g., analogous to SPS PDSCH release - Whether/how to support at least one additional DCI format dedicated for UL-only beam indication (in case of separate DL/UL), including: - Whether the format can also be used for DL-only beam indication (in case of separate DL/UL) and joint DL/UL beam indication - Whether it is a “brand new” format or based on some extension of the existing DCI formats other than 1_1 and 1_2 (e.g. 1_0, 0_0, 0_1, or 0_2) - If UL-related DCI is used, whether it is accompanied with UL grant or not - Acknowledgment mechanism

The following was agreed in RAN1#104-e:

Agreement On Rel.17 unified TCI framework, by RAN1#104bis-e, down select or modify at least one from the following alternatives: - Alt1. A UE can be dynamically indicated with either joint DL/UL TCI or separate DL/UL TCI - Details on dynamic indication are FFS - FFS: UE capability for the support of joint DL/UL TCI and/or separate DL/UL TCI - Alt2A. A UE can be configured with either joint DL/UL TCI or separate DL/UL TCI via RRC signaling - Alt2B. A UE can be configured with either joint DL/UL TCI, separate DL/UL TCI, or both via RRC signaling - Alt3. A UE can be configured with either joint DL/UL TCI or separate DL/UL TCI via MAC CE signaling - Details on how this is signaled in relation to TCI activation are FFS Agreement On the beam application time for Rel.17 DCI-based beam indication, the beam application time can be configured by the gNB based on UE capability - Support a UE capability for the minimum value of beam application time - FFS: the exact minimum values of beam application time supported by UE - FFS: whether existing UE capability can be reused as this UE capability. - FFS: whether different beam application time values are supported for uplink and downlink - FFS: whether UE capability needs to be introduced for the maximum value of beam application time - FFS: the reference for defining the UE capability (e.g. from DCI reception or ACK transmission) - FFS: whether a UE is allowed to report more than 1 values in case of MPUE - FFS: the application time when DCI and applied channel(s) are on different CCs with same/different SCS(s)s Agreement On the Rel.17 DCI -based beam indication, in RAN1#104bis-e, down-select at least one of the following alternatives regarding the support of DCI format(s) for beam indication in addition to the agreed DCI formats 1_1/1_2 with DL assignment (in RAN1#103-e): - Alt0: No additional DCI format is supported - Alt1: DCI formats 1_1 and 1_2 without DL assignment, applicable for joint TCI as well as separate DL /UL TCI - Support DCI acknowledgment mechanism, e.g. based on SPS PDSCH release, based on triggered SRS , based on DCI indicating SCell dormancy - FFS : How to identify DCI formats 1_1/1_2 used for beam indication only (not for scheduling a PDSCH reception, not indicating a SPS PDSCH release, or not indicating SCell dormancy), considering impacts on PDCCH coverage and scheduling mechanism - FFS : Whether the UE can/shall assume the gNB configured application time is after ACK transmission - Alt2: Dedicated DCI format other than 1_1/1_2 without DL assignment, applicable for joint TCI as well as separate DL /UL TCI - Support DCI acknowledgment mechanism, e.g. based on SPS PDSCH release, based on triggered SRS , based on DCI indicating SCell dormancy - FFS : If the format is based on an existing DCI format, how to identify the DCI format used for beam indication only - FFS : Whether the UE can/shall assume the gNB configured application time is after ACK transmission - Alt3: UL-related DCI formats 0_1/0_2 with UL grant, applicable only for UL-only TCI of separate DL /UL TCI Agreement On Rel.17 DCI-based beam indication, regarding application time of the beam indication: if beam indication is successfully received and the newly indicated beam in the beam indication is different from the previously indicated beam, down-select (no later than RAN1#105-e) one from the following. No other alternatives will be considered: - Alt1: the first slot that is at least X ms or Y symbols after the [first/last] symbol of the DCI with the joint or separate DL/UL beam indication - Alt2A: the first slot that is at least X ms or Y symbols after the [first/last] symbol of the acknowledgment of the joint or separate DL/UL beam indication - Alt 2B: the first slot that is at least X ms or Y symbols after the [first/last] symbol of the acknowledgment of the joint or separate DL/UL beam indication, except that the (new) TCI state update can be applied to the PDSCH, if it exists, (scheduled by the beam indication DCI) and corresponding ACK transmission (provided that the time offset between the DCI and the scheduled PDSCH exceed the threshold, analogous to Rel.15/16) - Alt2C: Support both Alt1 and Alt2A, and introduce a UE capability that indicates the support of Alt1 or Alt2A - Alt3: the first slot that is at least X1 ms or Y1 symbols after the [first/last] symbol of the DCI with beam indication and X2 ms or Y2 symbols after the [first/last] symbol of the acknowledgment of the beam indication - FFS: whether any existing timing defined for DCI based TCI/spatial relation update can be used for X/Y

The existing DCI format 1_1 and 1_2 can indicate one out of up to eight TCI codepoints, where the codepoints correspond to TCI states that are activated by MAC CE. There are several ways to solve the separate DL/UL beam indication using only the existing DCI formats 1_1 and 1_2. Hence, there is no need to introduce a new format or enhance another format (e.g. 0_1, 0_2). However, there may be a need to update the TCI state also when there is no DL data to transmit, since the unified TCI is also applicable to CORESETs. Hence, TCI indication in DCI formats 1_1/1_2 without DL assignment may be supported.

Proposal 6: Support unified TCI indication in Alt1 (DCI formats 1_1 and 1_2 without DL assignment). Do not support Alt 2 (Dedicated DCI format) or Alt 3 (UL-related DCI formats 0_1/0_2 with UL grant).

When a DL TCI state is activated (by MAC CE), the UE may need to perform time, frequency and beam tuning before the corresponding DL Rx beam can be used. When an UL TCI state is activated (by MAC CE), the UE may need to perform additional actions, such as powering on a power amplifier, performing Rx/Tx circuit calibration (for beam correspondence), etc. Hence, for the case of separate DL/UL TCI state update, it may be advantageous if the UE knows which of the activated TCI states that are DL TCI states, which that are UL TCI states, and which that joint DL/UL TCI states. This can be done by indicating in the MAC CE that activates a TCI state if it is jointly applicable to DL and UL, if the TCI state is applicable to DL only, or if the TCI state is applicable to UL only. Since events triggering separate DL/UL TCI may be dynamic, the corresponding TCI indication is preferably dynamic. Hence, Alt 1 (A UE can be dynamically indicated with either joint DL/UL TCI or separate DL/UL TCI) can be supported.

Proposal 7: Support Alt 1 (A UE can be dynamically indicated with either joint DL/UL TCI or separate DL/UL TCI).

In RAN1#103-e, it was L1-based beam indication was selected over MAC CE based indication with the motivation that it can provide faster beam update. Hence, RAN1 should strive to enable the fastest possible L1-based beam indication mechanism, which should be to apply the indicated beam a certain time after the DCI was successfully decoded. If the beam isn’t applied until after the scheduled PDSCH is acknowledged, there would be no benefit compared to the MAC CE based mechanism. The potential consequences of a TCI indication that isn’t received by the UE can be handled by the network.

Proposal 8: Support Alt1 (the first slot that is at least X ms or Y symbols after the [first/last] symbol of the DCI with the joint or separate DL/UL beam indication or separate DL/UL beam indication).

In Rel-15/16, if the PDSCH is scheduled by a DCI format having the TCI field present, the indicated TCI should be based on the activated TCIs in the slot with the scheduled PDSCH. The same principle can be applied for Rel-17 unified TCI framework.

Proposal 9: On Rel.17 DCI-based beam indication, the indicated TCI should be based on the activated TCIs in the slot with the scheduled PDSCH (as in Rel-15/16).

## 2.3 MPE Mitigation

In RAN1#104-e, the following was agreed.

Agreement On Rel.17 enhancements to facilitate MPE mitigation, decide in RAN1#104bis-e whether to support at least one the following (not necessarily, but can be, in one reporting instance): - {Rel.16 P-MPR based (beam/panel-level)} + {A}, where A is either Opt1A, Opt1B, Opt1C, or Opt1D: - Option 1A: Virtual PHR or a modified version associated with each activated UL TCI or, if applicable, joint TCI - Option 1B: {SSBRI(s)/CRI(s) and/or panel indication} - Option 1C: {SSBRI(s)/CRI(s) and/or panel indication} + virtual PHR or a modified version associated with each of the reported SSBRI(s)/CRI(s) and/or panel indication (if configured) - Option 1D: No additional reporting quantity - {SSBRI(s)/CRI(s) and/or panel indication} + {A}, where A is either Opt2A, Opt2B, Opt2A+ Opt2B, or Option 2C - Option 2A: L1-RSRP [L1-SINR] or a modified version that accounts for MPE effect associated with each of the reported SSBRI(s)/CRI(s) and/or panel indication (if configured) - FFS: How panel-level L1-RSRP [L1-SINR] is reported if L1-RSRP [L1-SINR] is associated with panel - FFS: Whether/how to account for MPE effect in L1-RSRP [L1-SINR] report, e.g. by using scaled L1-RSRP [L1-SINR] - FFS: Whether/how to enhance existing beam reporting format to support Option 2A - Option 2B: Virtual PHR or a modified version associated with each of the reported SSBRI(s)/CRI(s) and/or panel indication (if configured) - Option 2C: No additional reporting quantity

An MPE event may affect a sub-set of the beams, which means that some beams may be unaffected while other beams require significant power reduction. Hence, it is important for UL beam selection that a UE that experiences MPE can indicate how different beams are affected by the MPE event. This can be achieved by including a beam-level virtual PHR in a reporting instance, where the PHR takes the MPE-related power reduction into account. Note that a reported PHR can be negative. Hence, our preference is Option 2B.

Proposal 10: Support option 2B ({SSBRI(s)/CRI(s) and/or panel indication} + {Virtual PHR or a modified version associated with each of the reported SSBRI(s)/CRI(s) and/or panel indication (if configured)}).

# Conclusion

In this contribution, we considered aspects of multi-beam operation enhancement. We have the following proposals:

Proposal 1: Common QCL information can also apply to CSI-RS resource for CSI.

Proposal 2: Support at least M=1. Further discuss M=2.

Proposal 3: Support at least N=1. Further discuss N=2.

Proposal 4: For the case of separate DL and UL TCI state update, the UL TCI state is taken from the same TCI state pool as the DL TCI state.

Proposal 5: SRS for BM can be configured as a source RS to represent a DL RX spatial filter in the unified TCI framework.

Proposal 6: Support unified TCI indication in Alt1 (DCI formats 1_1 and 1_2 without DL assignment). Do not support Alt 2 (Dedicated DCI format) or Alt 3 (UL-related DCI formats 0_1/0_2 with UL grant).

Proposal 7: Support Alt 1 (A UE can be dynamically indicated with either joint DL/UL TCI or separate DL/UL TCI).

Proposal 8: Support Alt1 (the first slot that is at least X ms or Y symbols after the [first/last] symbol of the DCI with the joint or separate DL/UL beam indication or separate DL/UL beam indication).

Proposal 9: On Rel.17 DCI-based beam indication, the indicated TCI should be based on the activated TCIs in the slot with the scheduled PDSCH (as in Rel-15/16).

Proposal 10: Support option 2B ({SSBRI(s)/CRI(s) and/or panel indication} + {Virtual PHR or a modified version associated with each of the reported SSBRI(s)/CRI(s) and/or panel indication (if configured)}).

# References

- RP-202024, “Revised WID: Further enhancements on MIMO for NR”, Samsung, RAN#89e, Sep. 2020