3GPP TSG RAN WG1 #104b-e			R1-2103454

e-Meeting, April 12th – 20th, 2021

Agenda Item:	8.2.7

Source:	InterDigital, Inc.

Title:	Efficient beam management for NR in 52.6 – 71 GHz

Document for:	Discussion and Decision

# Introduction

In RAN#90-e [1], work scope of extending current NR operation to 71 GHz has been updated with the updated work scopes on SCS, initial access, processing time, PUCCH enhancement, PDCCH monitoring enhancement, multi-PDSCH/PUSCH scheduling and DMRS/PTRS enhancement. In addition, in RAN#91-e [2], an updated WID was approved with the updated beam management basis (i.e., both Rel-15/16 and Rel-17). In this contribution, we provide our views on efficient beam management based on FR2 beam information.

# Discussions

In 52.6 – 71GHz, it is well known fact that pathloss increases as carrier frequency increases [2]. In order to compensate the increased pathloss, utilization of narrower beam than FR2 is expected. If beamwidth in higher frequencies reduces N times, then the total number of beams should be increased to N times to have identical coverages in both horizontal and vertical domains. Based on the increased number of beams, beam management procedure may not be efficient considering following aspects:

- Beam measurement

  - As number of beams increases, latency and signaling overhead for RS transmission and measurement for beam determination may drastically increase. Although higher frequencies provide wider bandwidth in frequency domain, analog beam measurement may not efficiently utilize the wider bandwidth as analog beam measurement does not allow multiplexing in frequency domain, but in time domain.

- Beam indication

  - Due to narrower beamwidth and increased number of beams, it is expected to receive new beam indication more frequently (e.g., via MAC CE based on Rel-15/16 or DCI based on Rel-17 beam indication method). In addition, increased number of activated beams can be needed to reduce frequent activation/deactivation signaling via MAC CE.

- Beam failure recovery

  - As well as beam measurement and beam indication, increased number of beams require increased number of beams for failure monitoring and new beam selection to achieve a reliable operation. Existing BFR operation allows up to 10 monitoring RSs and 16 candidate RSs for beam failure detection and new beam selection. If beamwidth in higher frequencies reduces N times, then the maximum number of monitoring RSs and new candidate RSs should be increased to N times to have identical coverage in both horizontal and vertical domains.

Observation 1: Due to narrower beamwidth and increased number of beams in higher frequencies, utilization of the existing beam management operation in NR 52.6 – 71 GHz is not efficient.

One possible way to efficiently support beam management in higher frequencies would be efficient beam management based on FR2 beam information, when TRPs for FR2 and TRPs for higher frequencies are co-located. Figure 1 shows an exemplary operation of beam management based on FR2 beam information.

Figure 1 Exemplary operation of beam management based on FR2 beam information

As shown in Figure 1, beam measurement, beam indication and beam failure recovery can be done based on a determined FR2 beam. For example, instead of measuring all possible candidate beams, UE can measure candidate beams which are aligned to the determined FR2 beam.

Observation 2: Utilization of FR2 beam information may provide efficient operation for beam management in higher frequencies.

Proposal 1: Study utilization of FR2 beam information for beam management in higher frequencies.

In 52.6-71GHz, it is expected to utilize more antenna elements and panels considering short wave lengths and increased pathloss of the frequencies. In that regard, enhancements on beam associated timing parameters such as timeDurationForQCL, beamSwitchTiming and beamReportTiming can be considered. For example, when UE activates larger number of panels, relatively longer time durations can be supported to allow enough time gap for the beam changes across the activated panels. On the other hand, when the UE activates smaller number of panels, relatively shorter time durations may be enough as the UE need to change limited number of beams.

Observation 3: It is expected to utilize more antenna elements and panels considering short wave lengths and increased pathloss of the frequencies.

Proposal 2: Considering increased number of antenna elements and panels, enhance beam associated timing parameters.

# Summary

In this contribution, we discussed the possible beam management operation for higher frequencies based on FR2 beam information. From the discussions, we made following observations and proposals:

Observation 1: Due to narrower beamwidth and increased number of beams in higher frequencies, utilization of the existing beam management operation in NR 52.6 – 71 GHz is not efficient.

Observation 2: Utilization of FR2 beam information may provide efficient operation for beam management in higher frequencies.

Observation 3: It is expected to utilize more antenna elements and panels considering short wave lengths and increased pathloss of the frequencies.

Proposal 1: Study utilization of FR2 beam information for beam management in higher frequencies.

Proposal 2: Considering increased number of antenna elements and panels, enhanced beam associated timing parameters.

# References

- RP-202925, “Revised WID: Extending current NR operation to 71 GHz”

- RP-218062, “Revised WID: Extending current NR operation to 71 GHz”.