3GPP TSG-RAN WG1 Meeting #104bis-e                      R1-2103468

e-Meeting, April 12th – 20th, 2021

Source:	Sharp

Title:	Remaining issues on synchronization mechanism and QoS management for NR sidelink

Agenda Item:	7.2.4

Document for:	Discussion and Decision

Introduction

In this document, we share our views on remaining issues on synchronization mechanism and QoS management for NR sidelink.

Discussion

Synchronization mechanism

Indication of the non-TDD case in sl-TDD-Config

In the latest version of TS 38.213 indication of non-TDD case in sl-TDD-Config is captured in the following yellow highlighted text.

For transmission of an S-SS/PSBCH block, a UE includes a bit sequence a0, a1, a2, a3, …, a11 in the PSBCH payload to indicate sl-TDD-Config and provide a slot format over a number of slots. For paired spectrum, or if tdd-UL-DL-ConfigurationCommon and sl-TDD-Configuration are not provided for a spectrum indicated with only PC5 interface in Table 5.2E.1-1 in [TS 38.101-1], - a0, a1, a2, a3,a4, a5, a6, a7,a8, a9, a10, a11 are set to '1'; else - a0=0 if pattern1 is provided by sl-TDD-Configuration-r16 or tdd-UL-DL-ConfigurationCommon; a0=1 if both pattern1 and pattern2 are provided by sl-TDD-Configuration-r16 or tdd-UL-DL-ConfigurationCommon as described in Clause 11.1 […]

In case of shared SL carrier and SUL carrier, tdd-UL-DL-ConfigurationCommon is provided in the cell, but it is for the non-SUL carrier, and cannot be used to derive the 12 bits in sl-TDD-Config. Therefore, this should be added as another “non-TDD” case.

A TP (i.e. TP#1) is provided for sub clause 16.1 of TS 38.213 in section 4.1.

Clarification of the notation “uslotsSL”

For parameter sl-TDD-Config, the spec says “a5, a6, a7,a8, a9, a10, a11 are the 7th to 1st LSBs of uslotsSL, respectively”, without any further explanation of what “uslotsSL” is. Furthermore, “uslotsSL” is not referred to in any other spec text. So it is unclear what “uslotsSL” really means.

A TP (i.e. TP#2) is provided for sub clause 16.1 of TS 38.213 in section 4.2 to clarify the above issue.

QoS management

Clarification on sidelink congestion control

In sidelink congestion control for resource allocation mode 2, UE would determine the CR limit CRLimit(k) which is associated with the priority value k and the configured CBR range that includes the CBR measured in slot n-N.

### 8.1.6 Sidelink congestion control in sidelink resource allocation mode 2 If a UE is configured with higher layer parameter sl-CR-Limit and transmits PSSCH in slot n, the UE shall ensure the following limits for any priority value k; i≥kCRi≤CRLimitk where CRi is the CR evaluated in slot n-N for the PSSCH transmissions with 'Priority' field in the SCI set to i, and CRLimitk corresponds to the high layer parameter sl-CR-Limit that is associated with the priority value k and the CBR range which includes the CBR measured in slot n-N, where N is the congestion control processing time.

Note that there was no description in TS38.331 for the configured CBR range levels that the maximum CBR range level is 1, i.e. the last entry of sl-CBR-LevelsConfig is not necessarily configured as 100, thus, the measured CBR could be larger than every CBR range level and the UE is unable to determine the CR limit as mentioned above, since the CR limit is associated with the CBR range level which includes the measured CBR.

SL-CBR-CommonTxConfigList field descriptions

sl-CBR-RangeConfigList Indicates the list of CBR ranges. Each entry of the list indicates in SL-CBR-LevelsConfig the upper bound of the CBR range for the respective entry. The upper bounds of the CBR ranges are configured in ascending order for consecutive entries of sl-CBR-RangeConfigList. For the first entry of sl-CBR-RangeConfigList the lower bound of the CBR range is 0. Value 0 corresponds to 0, value 1 to 0.01, value 2 to 0.02, and so on.

As discussed above, it is straightforward if the measured CBR exceeds the last entry of sl-CBR-LevelsConfig, then the UE assumes the CBR range level corresponding to the last entry includes the measured CBR.

A TP (i.e. TP#3) is provided for sub clause 8.1.6 of TS 38.214 in section 4.3.

Conclusion

In this contribution, we discuss remaining issues on synchronization mechanism and QoS management for NR sidelink, and propose corresponding TPs.

Text proposals

TP#1 (for TS 38.213)

-------------------------------------------- Start of TP#1 -------------------------------------------

< Unchanged parts are omitted >

## 16.1	Synchronization procedures

< Unchanged parts are omitted >

For transmission of an S-SS/PSBCH block, a UE includes a bit sequence a0, a1, a2, a3, …, a11 in the PSBCH payload to indicate sl-TDD-Config and provide a slot format over a number of slots.

For paired spectrum or supplementary uplink band, or if tdd-UL-DL-ConfigurationCommon and sl-TDD-Configuration are not provided for a spectrum indicated with only PC5 interface in Table 5.2E.1-1 in [TS 38.101-1],

-	a0, a1, a2, a3,a4, a5, a6, a7,a8, a9, a10, a11 are set to '1';

else

-	a0=0 if pattern1 is provided by sl-TDD-Configuration-r16 or tdd-UL-DL-ConfigurationCommon; a0=1 if both pattern1 and pattern2 are provided by sl-TDD-Configuration-r16 or tdd-UL-DL-ConfigurationCommon as described in Clause 11.1

< Unchanged parts are omitted >

-------------------------------------------- End of TP#1 --------------------------------------------

TP#2 (for TS 38.213)

-------------------------------------------- Start of TP#2 -------------------------------------------

< Unchanged parts are omitted >

## 16.1	Synchronization procedures

< Unchanged parts are omitted >

For paired spectrum, or if tdd-UL-DL-ConfigurationCommon and sl-TDD-Configuration are not provided for a spectrum indicated with only PC5 interface in Table 5.2E.1-1 in [TS 38.101-1],

-	a0, a1, a2, a3,a4, a5, a6, a7,a8, a9, a10, a11 are set to '1';

else

-	a0=0 if pattern1 is provided by sl-TDD-Configuration-r16 or tdd-UL-DL-ConfigurationCommon; a0=1 if both pattern1 and pattern2 are provided by sl-TDD-Configuration-r16 or tdd-UL-DL-ConfigurationCommon as described in Clause 11.1

-	a1, a2, a3,a4 are determined based on

-	P in pattern1 as described in Table 16.1-1 for a0=0

-	P in pattern1 and P2 in pattern2 as described in Table 16.1-2 for a0=1

where P and P2 are as described in Clause 11.1

-	a5, a6, a7,a8, a9, a10, a11 are the 7th to 1st LSBs of a number of UL slots uslotsSL, respectively

-	for a0=0, uslotsSL=uslots*2μ-μref+usym*2μ-μrefL+I1

-	for a0=1, uslotsSL=uslots,2*2μ-μref+usym,2*2μ-μrefL+I2w*P*2μ+1w+uslots*2μ-μref+usym*2μ-μrefL+I1w

where

-	L is the number of symbols in a slot: L=12 if cyclicPrefix = "ECP"; else, L=14

-	I1 is 1 if usym*2μ-μref mod L≥ L-Y, else I1 is 0

-	I2 is 1 if usym,2*2μ-μref mod L≥L-Y, else I2 is 0

-	Y is the sidelink starting symbol index provided by sl-StartSymbol

-	w is the granularity of slots indication as described in Table 16.1-2

-	μref, uslots, usym, uslots,2, usym,2 are the parameters of TDD-UL-ConfigurationCommon as described in Clause 11.1, or the parameters of sl-TDD-Configuration-r16 as defined in [9.3, TS 38.331]

-	μ=0, 1, 2, 3 corresponds to SL SCS as defined in [4, TS 38.211]

Table 16.1-1: Slot configuration period when one pattern is indicated

| a1, a2, a3,a4 | Slot configuration period of pattern1 P (msec) |
| --- | --- |
| 0, 0, 0, 0 | 0.5 |
| 0, 0, 0, 1 | 0.625 |
| 0, 0, 1, 0 | 1 |
| 0, 0, 1, 1 | 1.25 |
| 0, 1, 0, 0 | 2 |
| 0, 1, 0, 1 | 2.5 |
| 0, 1, 1, 0 | 4 |
| 0, 1, 1, 1 | 5 |
| 1, 0, 0, 0 | 10 |
| Reserved | Reserved |

Table 16.1-2: Slot configuration period and granularity when two patterns are indicated

| a1, a2, a3,a4 | Slot configuration period of pattern1 P (msec) | Slot configuration period of pattern2 P2 (msec) | Granularity w in slots with different SCS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 15kHz | 30 kHz | 60 kHz | 120 kHz |
| 0, 0, 0, 0 | 0.5 | 0.5 | 1 |  |  |  |
| 0, 0, 0, 1 | 0.625 | 0.625 |  |  |  |  |
| 0, 0, 1, 0 | 1 | 1 |  |  |  |  |
| 0, 0, 1, 1 | 0.5 | 2 |  |  |  |  |
| 0, 1, 0, 0 | 1.25 | 1.25 |  |  |  |  |
| 0, 1, 0, 1 | 2 | 0.5 |  |  |  |  |
| 0, 1, 1, 0 | 1 | 3 | 1 |  |  | 2 |
| 0, 1, 1, 1 | 2 | 2 |  |  |  |  |
| 1, 0, 0, 0 | 3 | 1 |  |  |  |  |
| 1, 0, 0, 1 | 1 | 4 |  |  |  |  |
| 1, 0, 1, 0 | 2 | 3 |  |  |  |  |
| 1, 0, 1, 1 | 2.5 | 2.5 |  |  |  |  |
| 1, 1, 0, 0 | 3 | 2 |  |  |  |  |
| 1, 1, 0, 1 | 4 | 1 |  |  |  |  |
| 1, 1, 1, 0 | 5 | 5 | 1 |  | 2 | 4 |
| 1, 1, 1, 1 | 10 | 10 | 1 | 2 | 4 | 8 |

< Unchanged parts are omitted >

-------------------------------------------- End of TP#2 --------------------------------------------

TP#3 (for TS 38.214)

-------------------------------------------- Start of TP#3 -------------------------------------------

< Unchanged parts are omitted >

8.1.6	Sidelink congestion control in sidelink resource allocation mode 2

If a UE is configured with higher layer parameter sl-CR-Limit and transmits PSSCH in slot n, the UE shall ensure the following limits for any priority value k;

i≥kCRi≤CRLimitk

where CRi is the CR evaluated in slot n-N for the PSSCH transmissions with 'Priority' field in the SCI set to i, and CRLimitk corresponds to the high layer parameter sl-CR-Limit that is associated with the priority value k and the CBR range which includes the CBR measured in slot n-N, where N is the congestion control processing time. If the CBR measured exceeds the last entry of the associated higher layer parameter SL-CBR-LevelsConfig, the UE assumes the CBR range corresponding to the last entry includes the CBR measured.

< Unchanged parts are omitted >

-------------------------------------------- End of TP#3 --------------------------------------------