3GPP TSG-RAN WG1 Meeting #104bis-e	R1-2103545

eMeeting, April 12th – 20th, 2021

Agenda Item:	8.1.2.3

Source:	Ericsson

Title:		On beam management enhancements for simultaneous multi-TRP transmission with multi-panel reception

Document for:	Discussion

# 1	Introduction

The Rel-17 feMIMO WID [1] objective 2c states:

- Evaluate and, if needed, specify beam-management-related enhancements for simultaneous multi-TRP transmission with multi-panel reception

In this paper, we provide Ericsson’s views related to this objective taking into account the agreements from RAN1#104-e.

# 2	Discussion

## 2.1  Beam measurement/reporting

Agreement

For beam measurement in support of M-TRP simultaneous transmission

- Support a single CSI-report consisting of N beams pairs/groups and M (M>1) beams per pair/group, and different beams within a pair/group can be received simultaneously

  - Support M = 2

  - Support extending the maximum value of N > 1, exact value FFS

  - N=1 and N=2

    - FFS: Other values larger than 2

    - FFS: Whether the UE could report beams are received with different RX beams

- Further study the support of option 1 and option 3

- The above applies at least for L1-RSRP

  - FFS: L1-SINR

In RAN1#104-e, the following agreement was made with respect to beam measurement/reporting enhancements:

Given that Option 2 is already agreed, we do not prefer introducing multiple options as that would lead to higher specification impact, potential market fragmentation and more complicated UE capabilities in NR Rel-17.  Hence, we make the following proposal:

For beam measurement/reporting enhancement to facilitate inter-TRP beam pairing, Options 1 and 3 are not supported in NR Rel-17.

Regarding supporting additional values of N larger than 2, a slightly larger number of beam pairs/groups may provide the scheduler with some flexibility.  Hence, we propose to additionally support N=3.

For beam measurement/reporting enhancement to facilitate inter-TRP beam pairing, additionally support N = 3 with Option 2 in NR Rel-17.

In the above agreement, it was not clarified if the different beams within a pair/group can be received simultaneously with different receive beams.  However, it should be kept in mind that objective 2c in the WID [1] clearly states that this enhancement is for ‘simultaneous multi-TRP transmission with multi-panel reception’.  As shown in the results of our previous contribution [2], significant system-level performance gains are observed from enhancing group-based reporting for the simultaneous multi-TRP transmission with multi-panel reception use case to ensure 4 layers reception in FR2.  However, if the two beams from a pair/group are received with the same receive beam (i.e., using a single UE panel), 4 layer reception may not be possible and there may not be peak throughput improvement.  Hence, simultaneous reception of the two beams from a pair/group with different Rx beams is needed in order to see benefits of this enhancement.  In NR Rel-16, the following UE capability was introduced for simultaneous reception with different QCL Type D assumption, which allows the UE to receive with two different receive beams the signals transmitted from two different TRPs:

| simultaneousReceptionDiffTypeD-r16 Indicates whether the UE supports simultaneous reception with different QCL Type D reference signal as specified in TS38.213 [11]. | Band | No | N/A | FR2 only |
| --- | --- | --- | --- | --- |

It is quite natural to assume that a UE that supports the above capability can simultaneously receive the beams transmitted from within a pair/group with different receive beams.  Hence, a new UE capability is not needed in NR Rel-17 to indicate whether a UE could report that the beams within a pair/group are received with different RX beams.

Reuse simultaneousReceptionDiffTypeD-r16 UE capability to indicate if the beams within a beam pair/group can be received with different Rx beams.

In [3], the following draft proposal was discussed with regards to how to configure the channel measurement resources corresponding to the two TRPs to the UE:

Draft Proposal 1.2 from [3]:

- Down-select from the following alternatives in RAN1#104b-e

  - Alt-1: For option 2, support configuration of S=2 CMR resource sets corresponding to a periodic/semi-persistent CMR resource setting or an aperiodic trigger state

    - UE reports M beams (e.g. CMR resource indices) from S=2 different CMR resource sets which can be received simultaneously

    - NOTE: UE may assume that different CMR resources in different CMR sets can be received simultaneously, and CMR resources in the same CMR set cannot be received simultaneously

    - FFS: whether S = M

  - Alt-2: For option 2, support configuration of two CMR resource subsets in a CMR resource set corresponding to a CMR resource setting

    - UE reports M beams (e.g. CMR resource indices) from different CMR resource subsets, which can be received simultaneously

    - NOTE: UE may assume that different CMR resources in different CMR subsets can be received simultaneously, and CMR resources in the same CMR subset cannot be received simultaneously

    - FFS: a specific ID can be used to differentiate CMR resource subsets in a CMR resource set.

  - Alt-3: For option 2, support indication of S=2 SSB sets, where CMRs are implicitly mapped to a

a CMR set where a CMR in the set is QCLed (Type D) with a SSB in the same SSB set

    - UE reports M=2 beams (e.g. CMR resource indices) from S=2 CMR sets

    - NOTE: UE may assume that different CMR resources in different CMR sets can be received simultaneously, and CMR resources in the same CMR set cannot be received simultaneously

Agreement

For CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT, the UE can be configured with Ks ≥ 2 NZP CSI-RS resources in a CSI-RS resource set for CMR and N ≥ 1 NZP CSI-RS resource pairs whereas each pair is used for a NCJT measurement hypothesis

- Configure UE with two CMR groups with Ks=K1+K2 CMRs. CMR pairs are determined from two CMR groups by following method(s).

  - K1 and K2 are the number of CMRs in two groups respectively. FFS K1=K2 or different K1/K2.

  - Note that CMRs in each CMR group can be used for both NCJT and Single-TRP measurement hypotheses

  - N CMR pairs are higher-layer configured by selecting from all possible pairs

    - signalling mechanism can be discussed further, e.g. using a bitmap

    - FFS: Whether MAC-CE or RRC+MAC CE indication is needed

    - FFS: how to support NCJT measurement hypotheses in FR2

- Support N=1 and Ks =2, FFS other maximal values of N>1 and Ks>2

- Note: for CPU/resource/port occupation, NCJT hypothesis is considered separately from single TRP hypothesis

Note that there is an existing agreement in RAN1#104-e related to the above proposal in agenda 8.1.4.  The related agreement is copied below:

According to the above agreement, two CMR groups are configured within a CSI-RS resource set, where CMR pairs for NC-JT CSI are picked from the two CMR groups.  This is very close to Alt-2 in proposal 1.2 above when M is set to 2 (i.e., two CMR groups is equivalent to two CMR resource subsets).  Since multi-TRP CSI and enhanced group based beam reporting are to be supported in the same framework, in our view, the two CMR group framework can be reused for enhanced group based beam reporting.  Hence, we propose the following:

For beam measurement/enhancement to facilitate inter-TRP beam pairing, reuse the two CMR groups framework agreed for multi-TRP CSI where each beam in a beam group/pair is selected from one of the two CMR groups.

## 2.2  M-TRP beam failure detection and new beam identification

Agreement

- For M-TRP beam failure detection, support independent BFD-RS configuration per-TRP, where each TRP is associated with a BFD-RS set.

  - FFS: The number of BFD RSs per BFD-RS set, the number of BFD-RS sets, and number of BFD RSs across all BFD-RS sets per DL BWP

  - Support at least one of explicit and implicit BFD-RS configuration

    - With explicit BFD-RS configuration, each BFD-RS set is explicitly configured

      - FFS: Further study QCL relationship between BFD-RS and CORESET

    - FFS: How to determine implicit BFD-RS configuration, if supported

- For M-TRP new beam identification

  - Support independent configuration of new beam identification RS (NBI-RS) set per TRP if NBI-RS set per TRP is configured

    - FFS: detail on association of BFD-RS and NBI-RS

    - Support the same new beam identification and configuration criteria as Rel.16, including  L1-RSRP, threshold

In RAN1#103-e, the following agreement was reached with regards to M-TRP BFD:

Firstly, regarding explicit and implicit BFD-RS configuration, both these configurations are supported in NR Rel-15 and Rel-16. If the explicit BFD-RS configuration is extended to per-TRP operation, multiple BFD-RS sets will need to be configured via higher layer signaling. This extension seems straightforward. On the other hand, with implicit BFD-RS configuration, the BFD-RS sets can be determined implicitly via the activated TCI states of the CORESETs associated with different TRPs. Since the activated TCI states for the CORESETs can be updated via MAC CE, the implicit approach provides more flexibility with regards to updating the RSs in the BFD-RS set.  Our preference is to support both explicit and implicit BFD-RS configuration in NR Rel-17.

In NR Rel-17, support both explicit and implicit BFD-RS configuration in order to support per-TRP beam failure detection.

In RAN1#104-e, it was agreed to support 2 BFD-RS sets per BWP.  However, the number of resources per BFD-RS set is still FFS.  In NR Rel-15/16, up to 2 BFD RSs per BFD-RS set is supported.  It is natural to support this same number of BFD RSs per BFD-RS set in NR Rel-17.  Furthermore, in our view, the number of BFD RSs can be fixed in the specifications and there is no need to define a UE capability for this.

In NR Rel-17, support up to two BFD RSs per BFD RS set which is fixed in the specifications.

With regard to supporting explicit and implicit per-TRP BFD-RS configuration, a further open issue is whether to support one or both of multi-DCI based multi-TRP and single-DCI based multi-TRP.  In the case of explicitly configured BFD-RS resource sets, these can be used to support per-TRP beam failure detection for both multi-DCI based multi-TRP and single-DCI based multi-TRP.  Hence, we propose the following:

In NR Rel-17, support explicit per-TRP BFD-RS configurations for both multi-DCI based multi-TRP and single-DCI based multi-TRP.

In RAN1#103-e, companies proposed to only support implicit per-TRP BFD-RS configurations only for multi-DCI based multi-TRP.  The reasoning given is that the already specified CORESETPoolIndex (e.g., k = 0 or 1) can be leveraged when determining BFD-RS sets for multi-DCI based multi-TRP.  For instance, the BFD-RSs in BFD-RS sets #1 and #2 can be determined via the activated TCI states of the CORESETs associated with CORESETPoolIndex values 0 and 1, respectively.  On the other hand, for single-DCI based multi-TRP, supporting implicit per-TRP BFD-RS configurations would require introduction of a CORESET group identifier or something similar.  In terms of standardization effort, it appears that supporting implicit per-TRP BFD-RS configurations only for multi-DCI based multi-TRP requires less effort.  Hence, our preference is to support only multi-DCI based multi-TRP for implicit per-TRP BFD-RS configurations in NR Rel-17.

In NR Rel-17, support implicit per-TRP BFD-RS configurations only for multi-DCI based multi-TRP.

In NR Rel-16, multi-DCI based multi-TRP operation was limited to intra-cell operation (i.e., both TRPs correspond to the same PCI).  In NR Rel-17, multi-DCI based multi-TRP operation is being extended to inter-cell operation (i.e., both TRPs correspond to the different PCIs).  Hence, when introducing per-TRP BFD-RS configurations in Rel-17, it is beneficial to support both intra-cell and inter-cell multi-DCI based multi-TRP operations.

In NR Rel-17, support per-TRP BFD-RS configurations for both intra-cell and inter-cell multi-DCI based multi-TRP operation.

Agreement

For PDCCH reliability enhancements, support SFN scheme + Alt 1-1.

- FFS: TCI state activation for CORESET, impact on default beam, BFD resource for BFR

In RAN1#103-e, SFN based PDCCH reliability was agreed.

In the above scenario, a CORESET will be activated with two TCI states.  Then, the open question is which TCI state the UE should use for determining the BFD-RS.  Note that the SFN scheme + Alt 1-1 is essentially a single-DCI based multi-TRP scheme since the same DCI is transmitted from two TRPs in SFN fashion.  If implicit per-TRP BFD-RS configuration is not supported for single-DCI based multi-TRP in Rel-17, then there will be only a single BFD-RS set.  A simple solution is to let the UE assume that the reference signals used as QCL sources (if each TCI state contains two reference signal, the UE will utilize the QCL-TypeD RS) in the two activated TCI states for the CORESET can be used as BFD-RSs in the single BFD-RS set.  As shown in the example of Figure 1,  the QCL source reference signal with CSI-RS resource IDx (or SSB IDx) corresponding to the 1st activated TCI state and QCL source reference signal with CSI-RS resource IDy (or SSB IDy) corresponding to the 2nd activated TCI state shall be included by the UE in the beam failure detection resource set q0.

When two TCI states are activated for a CORESET, support inclusion of reference signals used as QCL sources in the two activated TCI states as BFD-RSs in the single BFD-RS set q0. If the activated TCI states contains two reference signals, the UE will use the reference signals configured with QCL-TypeD

Figure 1.  An example of BFD resource determination with single BFD resource set when CORESET is configured for SFN-based PDCCH diversity

## 2.3  BFRQ

Agreement

For BFRQ of M-TRP BFR

- Option 3: Up to two dedicated PUCCH-SR resources in a cell group

- FFS: Whether PUCCH-SR for SCell can be reused for M-TRP

- Support BFRQ MAC-CE that can convey information of failed CC indices, one new candidate beam for the failed TRP/CC (if found), and whether new candidate beam is found

  - Support at least indication of a single TRP failure

    - FFS: whether/what information of failed TRP(s) is conveyed in the MAC-CE

    - FFS: whether/how to support  indication of more than one TRP failure, corresponding BFR procedure, and applicable cell type (SCell vs. SpCell)

- FFS: UE behavior when TRP failure status is different across cells

- FFS: Whether PUCCH SR resource can be configured with 2 spatial relations

With regards to BFRQ framework, the following agreement was reached in RAN1#104-e:

Since up to two dedicated PUCCH-SR resources in a cell group are agreed where each dedicated PUCCH-SR resource is associated to one of two TRPs, we do not see a need to configure 2 spatial relations per PUCCH-SR resource.  Hence, a single spatial relation per PUCCH-SR resource is sufficient.  Which PUCCH-SR resource the UE should use may be up to the UE to select depending on which TRP does not have BFR.  Hence, we make the following proposals:

For BFRQ of M-TRP BFR, support a single spatial filter associated with each the PUCCH-SR resource.

Regarding what information of failed TRP(s) is conveyed in the MAC-CE, we think at least the BFD-RS set index corresponding to the TRP that failed and a candidate RS ID in the NBI-RS set corresponding the failed TRP (if at least one of the SSBs with SS-RSRP above rsrp-ThresholdBFR is found) should be included in the MAC CE.

In NR Rel-17, support the inclusion of at least BFD-RS set index corresponding to the failed TRP and a candidate RS ID in the NBI-RS set corresponding the failed TRP (if at least one of the SSBs with SS-RSRP above rsrp-ThresholdBFR is found) in the MAC CE.

# 3	Conclusion

Based on the discussion in the previous section, we make the following observation and proposal:

Proposal 1	For beam measurement/reporting enhancement to facilitate inter-TRP beam pairing, Options 1 and 3 are not supported in NR Rel-17.

Proposal 2	For beam measurement/reporting enhancement to facilitate inter-TRP beam pairing, additionally support N = 3 with Option 2 in NR Rel-17.

Proposal 3	Reuse simultaneousReceptionDiffTypeD-r16 UE capability to indicate if the beams within a beam pair/group can be received with different Rx beams.

Proposal 4	For beam measurement/enhancement to facilitate inter-TRP beam pairing, reuse the two CMR groups framework agreed multi-TRP CSI where each beam in a beam group/pair is selected from one of the two CMR groups.

Proposal 5	In NR Rel-17, support both explicit and implicit BFD-RS configuration in order to support per-TRP beam failure detection.

Proposal 6	In NR Rel-17, support up to two BFD RSs per BFD RS set which is fixed in specifications.

Proposal 7	In NR Rel-17, support explicit per-TRP BFD-RS configurations for both multi-DCI based multi-TRP and single-DCI based multi-TRP.

Proposal 8	In NR Rel-17, support implicit per-TRP BFD-RS configurations only for multi-DCI based multi-TRP.

Proposal 9	In NR Rel-17, support per-TRP BFD-RS configurations for both intra-cell and inter-cell multi-DCI based multi-TRP operation.

Proposal 10	When two TCI states are activated for a CORESET, support inclusion of reference signals used as QCL-Type D sources in the two activated TCI states as BFD-RSs in the single BFD-RS set q0.

Proposal 11	For BFRQ of M-TRP BFR, support a single spatial filter associated with each the PUCCH-SR resource.

Proposal 12	In NR Rel-17, support the inclusion of at least BFD-RS set index corresponding to the failed TRP and a candidate RS ID in the NBI-RS set corresponding the failed TRP (If at least one of the SSBs with SS-RSRP above rsrp-ThresholdBFR is found) in the MAC CE.

# References

RP-193133, New WID: Further enhancements on MIMO for NR, Samsung, RAN#86, Sitges, December 2019.

R1-2009225, ‘On beam management enhancements for simultaneous multi-TRP transmission with multi-panel reception’, Ericsson, RAN1#103-e, October 26th – November 13th, 2020.

R1-2102153, Moderator summary on M-TRP simultaneous transmission with multiple Rx panels (round 2), RAN1#104-e, January 25th – February 5th, 2021