3GPP TSG RAN WG1 #104b-e	R1-2103564

e-Meeting, Apr. 12th – 20th, 2021

Source:	NTT DOCOMO, INC

Title:	Discussion on SRS enhancement

Agenda Item:	8.1.3

Document for: 	Discussion and Decision

# Introduction

In RAN1#104-e meeting, following agreements were made regarding study aspects on SRS enhancements [1].

Agreement For Rel-17 SRS capacity and coverage enhancement, support the following - Increase the maximum number of repetition symbols in one slot and one SRS resource to S - Support at least one S value from {8, 10, 12, 14} - FFS other candidate values - Support to transmit SRS only in contiguous RBs in one OFDM symbol, where indicates the number of RBs configured by BSRS and CSRS - Support at least one PF value from {2, [3], 4, 8} - FFS other candidate values, e.g., non-integer values for PF - Note: SRS sequence shorter than the minimum length supported in the current specification is not pursued. - No new sequence including length is introduced - FFS it is applicable to frequency hopping and non-frequency hopping - FFS detailed signaling mechanism to determine PF and the location of the RBs - Support Comb 8 - Note: SRS sequence shorter than the minimum length supported in the current specification is not pursued. - FFS whether and if needed, how to use harmonized approach to define the three supported schemes - Note: other schemes for SRS capacity and coverage enhancements are not supported in Rel-17. Agreement - For aperiodic antenna switching SRS, support to configure N <=N_max resource sets, where totally K resources are distributed in the N resource sets flexibly based on RRC configuration. - For 1T6R, K=6, N_max = [4], and each resource has 1 port. - For 1T8R, K=8, N_max = [4], and each resource has 1 port. - For 2T6R, K=3, N_max = [3], and each resource has 2 ports. - For 2T8R, K=4, N_max = [4], and each resource has 2 ports. - (Working Assumption) For 4T8R, K=2, N_max = [2], and each resource has 4 ports. - FFS the number of supported candidate values of N for each xTyR. - FFS extension to increase N_max for 1T4R, 2T4R, T=R and 1T2R cases for aperiodic, periodic and semi-persistent SRS resources - FFS the number of resources and resource sets for semi-persistent and periodic antenna switching SRS - Note: SRS could be transmitted over the last 6 OFDM symbols, or over any OFDM symbols within the slot subject to UE capability. Agreement Further study whether and if needed, how to achieve further enhancements on aperiodic SRS triggering and resource management based on repurposing unused fields in DCI format 0_1/0_2 without data and without CSI. Consider the following examples - CAT A: Time-domain parameters - A-1: Indication of available slot position, i.e., the t values - A-2: Indication of slot offset - A-3: Indication of SRS symbol-level offset - A-4: Indication of time-domain behavior for SRS transmission over multiple OFDM symbols, e.g., repetition, hopping, and/or splitting - CAT B: Frequency-domain parameters - B-1: Indication of a group of CCs for SRS transmission - B-2: Indication of frequency domain resource in a BWP for SRS transmission - B-3: Indication of whether DL/UL BWP is applied for SRS transmission - CAT C: Power control parameters - C-1: Re-purpose ‘TPC command for PUSCH’ as ‘TPC command for SRS’ - FFS impact on power control, impact from triggering a group of CCs for SRS - C-2: Indication of open loop power control parameter e.g., p0. - CAT D: Spatial-domain parameters, i.e., indication of SRS port and beamforming - CAT E: Extend the number of DCI codepoints for aperiodic SRS trigger states - Other examples are not precluded Agreement A list of t values is configured in RRC for each SRS resource set. Adopt at least one of the following for DCI indication of t. - In DCI format 0_1/0_2 without data and without CSI request, - Alt 1-1: Reuse the same scheme used for DCI format 0_1/0_2/1-1/1-2 that schedules a PDSCH or PUSCH - Alt 1-2: Re-purpose unused DCI field to indicate t - Alt 1-3: t is indicated by a configurable DCI field, where the DCI field may contain bits from unused fields and additional bits configured by gNB - FFS design details with other potential field(s) - FFS: whether t can be slot offset - In DCI format 0_1/0_2/1-1/1-2 that schedules a PDSCH or PUSCH - Alt 2-1: t is indicated by adding a new configurable DCI field - Alt 2-2: t is indicated without adding DCI payload - Note: The size of DCI payload does not change dynamically - Note: RAN1 should strive for unified solution for different DCI formats. - FFS: The number of RRC configured t values per SRS resource set and DCI bit field size. Agreement Confirm the following working assumption with modifications An “available slot” is a slot satisfying there are UL or flexible symbol(s) for the time-domain location(s) for all the SRS resources in the resource set and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set. - From the first symbol carrying the SRS request DCI and the last symbol of the triggered SRS resource set, UE does not expect to receive SFI indication, UL cancellation indication or dynamic scheduling of DL channel/signal(s) on flexible symbol(s) that may change the determination of “available slot”. - Note: Collision handling between the triggered SRS and any other UL channel/signal is performed after the determination of available slot. - FFS: Rules to handle the case of multiple SRS resource sets with overlapping symbols and/or triggered by a same DCI

In this contribution, we discuss possible SRS enhancements focusing on aforementioned study aspects captured in agreements from RAN1#104-e meeting.

# Flexible aperiodic SRS triggering

In order to enhance the triggering flexibility of aperiodic SRS (A-SRS), during RAN1#103-e meeting, it was agreed that when an A-SRS resource set is triggered using DCI, that A-SRS resource set should be transmitted from (t+1)-th available slot counting from a reference slot (see Fig. 2-1). Note that, a list of values for parameter t is configured using RRC signaling and one value out of those is selected using DCI.

Figure 2-1: Relation between the slot with the triggering DCI, reference slot and available slot

During RAN1#104-e meeting, an agreement on the definition of reference slot could not be made. In particular, there were two options proposed for identifying the reference slot. They were, option 1) reference slot is the slot with the triggering DCI, and option 2) reference slot is the slot indicated by the legacy triggering offset. With the proposed option 2, since the reference slot is different from the slot with the triggering DCI, first UE has to align with the reference slot based on the configured legacy triggering offset. Afterwards, UE has to wait additional (t+1) slots from the reference slot for an available slot. On the other hand, the proposed option 1 does not have an intermediate reference slot and UE can directly look for an available slot on or after (t+1) slots from the slot with the triggering DCI.

As per our view, option 2 does not bring any additional gains over option 1. In fact, option 2 reduces A-SRS triggering flexibility since it involves two steps as discussed previously while requiring additional signaling overheads, i.e., for configuring legacy triggering offset, compared to option 1. Hence, we think option 1 is a simple and a more flexible solution. Accordingly, we make the following proposal.

Proposal 1:

- Support Opt. 1 which proposes ‘reference slot’ as the slot with the triggering DCI.

Regarding the definition of available slot, a working assumption was agreed during RAN1#104-e meeting. The main objective of introducing an available slot is to provide UE with some flexibility for identifying a slot for transmitting a triggered A-SRS resource set. In particular, as per the agreed working assumption, as part of its capability reporting, UE reports the minimum timing requirement between A-SRS triggering PDCCH and all the SRS resources in the triggered A-SRS resource set. This minimum timing requirement can then be considered for identifying the minimal time interval with different usages, i.e., for ‘codebook’, ‘antennaswitching’ and ‘non-codebook’, ‘BeamManagement’. In general, we are supportive to the proposed working assumption.

Proposal 2:

- Confirm the proposed working assumption on the definition of ‘available slot’.

## 2.1 Extending the UE specific DCI format(s) for more flexible aperiodic SRS triggering

During RAN1#103-e meeting it was agreed to support UE specific DCI formats, DCI 0_1 and 0_2 without uplink data and CSI, for A-SRS triggering. Note that, one of the main purposes of this enhancement is to effectively utilize available DCI fields within DCI formats 0_1 and 0_2 without uplink data and CSI, for more flexible A-SRS triggering.

If the higher layer signaling can only switch the UE behavior between the existing DCI with and without uplink-data and CSI, the usefulness of the extended DCI becomes very limited. Hence, we propose to consider dynamic switching. In particular, to enable dynamic switching, two options can be considered. They are, option 1) introduce a new RNTI, and option 2) use a special combination of the existing DCI field to indicate that this DCI is the extended DCI. In particular, with option 1, if the UE specific DCI is CRC scrambled using the new RNTI, this DCI is treated as the extended DCI without uplink-data and CSI. Otherwise, the DCI is the existing DCI with uplink-data and CSI. Further, with option 2, if the UE specific DCI has the special combination, this DCI is the extended DCI without uplink-data and CSI. Otherwise, the DCI is the existing DCI with uplink-data and CSI.

Aforementioned option 1 is illustrated in Fig. 2-2. As captured, the total DCI size between the existing DCI and the extended DCI is the same. However, more number of DCI bits can be allocated for the SRS request field in the extended DCI, which leads to more flexible SRS triggering. In particular, additional DCI bits within the SRS request field can be used for configuring proposed parameter t. Figure 2-3 illustrates the proposed option 2. Similar to option 1, the total DCI size between the existing DCI and the extended DCI is the same while more number of DCI bits can be allocated for the SRS request field in the extended DCI with option 2 as well.

Figure 2-2: Option 1 – Introduce a new RNTI to indicate the extended DCI

Figure 2-3: Option 2 - Special combination of the existing DCI field indicates the extended DCI

Proposal 3:

- Support dynamic switching of the UE behavior between the extended UE specific DCI format(s) without data/CSI and the existing UE specific DCI format(s) with data/CSI. Following options can be considered for the dynamic switching:

  - Option 1: Introduce new RNTI to indicate the extended DCI.

    - If the UE specific DCI is CRC scrambled by the new RNTI, this DCI is the extended DCI format 0_1/0_2 without uplink-data/CSI. Otherwise, this DCI is the existing UE specific DCI with uplink-data/CSI.

  - Option 2: Special combination of the existing DCI field to indicate the extended DCI.

    - If the UE specific DCI has the special combination in the existing DCI field, this DCI is the extended DCI format 0_1/0_2 without uplink-data/CSI. Otherwise, this DCI is the existing UE specific DCI with uplink-data/CSI.

## 2.2 Indication of parameter t using DCI formats 0_1/0_2

As per the agreements from RAN1#104-e meeting, a list of t values is configured using RRC signalling for each SRS resource set. Afterwards, using DCI, one value for t from the list, is indicated. In particular, there were different solutions proposed based on whether the DCI which indicates t value schedules data and CSI or not. One solution out of those is to add a new configurable DCI field to indicate t value. This idea is succinctly captured in Fig. 2-4.

Figure 2-4: Structure of the DCI with newly added DCI field for indicating t value

Note here that, the new DCI field can only exist when RRC configures it. A new RRC parameter can be introduced for this purpose. Also note that, with the addition of new DCI field for indicating t value, the DCI payload size increases.

Observation 1:

- Adding a new DCI field to indicate t value increases the DCI payload size

As another solution for indicating t value using DCI, it was proposed to repurpose unused DCI fields. However, this solution is mainly applicable for the DCI formats 0_1 and 0_2 without data and CSI scheduling.

As per our understanding, rather than going for a unified solution to indicate the t value using DCI with and without data and CSI scheduling, it makes more sense to consider different solutions for two cases. Otherwise, we are unnecessarily increasing the DCI payload size. In particular, as discussed previously, if the RRC configures the availability of the new DCI field, that DCI field can be used to indicate t value dynamically using DCI with and without data and CSI scheduling. On the other hand, if RRC does not configure the new DCI field, by repurposing one of the existing DCI field(s) (which is not used for data and CSI scheduling), it is possible to dynamically indicate the value of t. This is especially applicable for the case of DCI triggering A-SRS without data and CSI scheduling. Note here that, the value of t can be semi-statically configured for the case of DCI triggering A-SRS with data and CSI scheduling. Accordingly, we make the following proposal.

Proposal 4:

- Rather than stick into a unified solution for the indication of t value using DCI with and without data and CSI scheduling, consider a more flexible solution. In particular, following can be one viable approach

- If RRC configures the availability of a new DCI field, t value is indicated using that new DCI field

- Otherwise, repurpose one of the existing DCI field(s) (which is not used for data and CSI scheduling) to indicate t value for DCI without data/CSI, and t value is selected by pre-determined rule from the list configured by higher layer signaling for DCI with data/CSI.

- Note: maintain the same DCI size for both cases with and without data and CSI scheduling.

## 2.3 Repurposing DCI fields in DCI formats 0_1 and 0_2 without scheduling data and CSI

During RAN1#104-e meeting, it was discussed how to repurpose unused fields in DCI format 0_1 and 0_2 without data and CSI scheduling. In particular, an agreement was made to identify what parameters can be considered for capturing within the unused DCI fields.

Agreement

Further study whether and if needed, how to achieve further enhancements on aperiodic SRS triggering and resource management based on repurposing unused fields in DCI format 0_1/0_2 without data and without CSI. Consider the following examples

- CAT A: Time-domain parameters

  - A-1: Indication of available slot position, i.e., the t values

  - A-2: Indication of slot offset

  - A-3: Indication of SRS symbol-level offset

  - A-4: Indication of time-domain behavior for SRS transmission over multiple OFDM symbols, e.g., repetition, hopping, and/or splitting

- CAT B: Frequency-domain parameters

  - B-1: Indication of a group of CCs for SRS transmission

  - B-2: Indication of frequency domain resource in a BWP for SRS transmission

  - B-3: Indication of whether DL/UL BWP is applied for SRS transmission

- CAT C: Power control parameters

  - C-1: Re-purpose ‘TPC command for PUSCH’ as ‘TPC command for SRS’

    - FFS impact on power control, impact from triggering a group of CCs for SRS

  - C-2: Indication of open loop power control parameter e.g., p0.

- CAT D: Spatial-domain parameters, i.e., indication of SRS port and beamforming

- CAT E: Extend the number of DCI codepoints for aperiodic SRS trigger states

- Other examples are not precluded

However, there is no clear understanding on how to modify or map the existing DCI fields to capture newly proposed parameters within the aforementioned agreement.

Observation 2:

- There is no clear understanding on how to modify or map the existing DCI fields to capture newly proposed parameters

Let us now look at the available fields in DCI format 0_1 which is basically used for scheduling of one or multiple PUSCH in one cell or indicating configuration grant downlink feedback information (CG-DFI) to a UE. As per [2] section 7.3.1.1.2, some of the fields available in DCI format 0_1 is captured in Fig. 2-5.

Figure 2-5: Available fields in DCI format 0_1 [2]

One possible approach to capture new parameters within the existing DCI fields is by renaming the DCI fields.   An example of new field names and their associated new parameters can be given as in Table 1.

Table 1: New field names and their associated parameters

Another possible approach to capture new parameters within the existing DCI fields is by defining a mapping rule between the existing DCI fields and newly proposed parameters. An example of a mapping rule is captured in Table 2.

Table 2: A mapping rule associating the existing field names and new parameters

Similar approaches can be considered for DCI format 0_2 as well. Accordingly, we make the following proposal.

Proposal 5:

- Consider following approaches to associate DCI fields within DCI formats 0_1/0_2 when repurposing the existing DCI fields to capture new parameters

  - Rename the existing DCI fields

  - Define a mapping rule between the existing DCI field names and new parameters

## 2.4 Extending number of DCI codepoints for A-SRS trigger states

During RAN1#104e-meeting, there were some discussions on the requirement of extending number of DCI codepoints for A-SRS trigger states. As per our view, this is particularly important to support DL CSI acquisition with newly proposed transceiver architectures.

Further, we think that extending DCI codepoints to support 7 A-SRS trigger states (currently there are only 3 trigger states available) may be sufficient and hence adding additional 1-bit (currently max. 2-bits are used) to the SRS request field is enough. In order to control the DCI overhead, it is possible to configure the size of SRS request field within DCI using RRC signaling similar to srs-RequestDCI-0-2 parameter configuring the SRS request field size in DCI format 0_2. Accordingly, we make the following proposal.

Proposal 6:

- Support extending number of DCI codepoints to have at least 7 A-SRS trigger states

## 2.5 MAC CE for enhancing flexibility in aperiodic SRS triggering

A-SRS is useful in practical networks which enables efficient UL resource utilization with SRS transmission. DCI code point of SRS request field can correspond to one or multiple SRS resource set(s). Further, DCI-level flexible indication of SRS resources is supported in Rel-15/16. However, we have observed following issues pertaining to the configuration of A-SRS:

- Some of the parameters (e.g. slotOffset) are RRC configured and hence, using MAC CE or DCI, they cannot be updated

- The number of configurable SRS resource sets for some SRS usages are limited, especially for usage set to ‘codebook’, ‘non-codebook’, and ‘antennaSwitching’

For example, if we look at slotOffset configured in a SRS resource set with usage set to ‘codebook’, only one SRS resource set can be configured. This is because, slotOffset is higher layer configured per SRS resource set and hence, can’t be modified using different DCI codepoints.

In order to improve the flexibility of triggering A-SRS, we propose it is better to involve MAC CE as well for configuring/triggering an A-SRS. In particular, following three alternatives, Alt.1, Alt. 2 and Alt. 3 illustrated in Fig. 2-6, 2-7, and 2-8, respectively, should be considered for triggering aperiodic SRS.

Proposal 7:

- Introduce MAC CE to improve the flexibility for triggering aperiodic SRS, and following alternatives should be considered

  - Alt.1) MAC CE activates/deactivates one or multiple SRS resource sets, and only active SRS resource set(s) can be triggered by SRS request field of DCI

  - Alt.2) MAC CE updates the mapping b/w DCI codepoint of SRS request field and aperiodicSRS-ResourceTrigger

  - Alt. 3) MAC CE updates specific parameter (e.g. slotOffset) in SRS resource sets/SRS resource directly

- For Alt. 1/2, the number of configurable SRS resource sets for a given usage is increased.

Figure 2-6: Proposed Alt.1

Figure 2-7: Proposed Alt.2

Figure 2-8: Proposed Alt.3

# SRS resource(s) with multiple usages

For SRS overhead reduction, reusing same SRS resource(s) among multiple usages (especially between ‘codebook’ and ‘antennaswitching’) was identified as one viable solution. After RAN1#104-e meeting, there was an offline discussion on how to configure an SRS resource to share between different usages. It is worth remarking here that the current specification does not restrict configuring an SRS resource to be shared between different usages. However, as per the offline discussion, one major issue in such a configuration is that, the specification does not clearly define what antenna virtualization and power allocation UE should consider for a shared SRS resource. Five different alternatives were identified during the offline discussion which are captured below.

Alt 0: Offline conclusion:  When a UE is configured an x-port SRS in a set with usage “codebook”, the gNB can assume that the UE transmits these SRS resources in the same way (e.g. same virtualization) as if the set would have been configured with usage ‘antennaswitching’ with xTxR. Hence, it is RAN1 understanding that the SRS measurement for ‘codebook’ based uplink in this case can also be used for downlink reciprocity based operation.

Alt 0’: In Rel-15, UE is allowed to choose any implementation operation for CB SRS

    - It is up to gNB implementation whether to derive DL precoder based on CB SRS or not

This should be the baseline for comparison with other alternatives for SRS with dual usage

Alt 1: At least for a UE supporting maxNumberMIMO-LayersCB-PUSCH = x, and, maxNumberMIMO-LayersPDSCH = x, and, supportedSRS-TxPortSwitch = ’t2r2’ / ’t4r4’ / 't1r1-t2r2’ / 't1r1-t2r2-t4r4', when a UE is configured only one x-port SRS resource in a set with usage “antennaswitching” with xTxR for DL CSI acquisition, the gNB can assume that the UE transmits this SRS resource in the same way (e.g. same virtualization) as if the set would have been configured with usage “codebook”, and such x-port SRS resource can be used for an x-port UL MIMO precoder selection (i.e TPMI selection) by the gNB

- A RRC parameter is introduced to enable the feature, which is subjected to UE capability

- Above at least applies for single-TRP operation and when uplink full power mode 1 or mode 2 is not enabled

- FFS: other cases

Alt 2:

- An RRC parameter is introduced to enable the sharing of SRS resource for CB and AS

- The shared SRS resource is configured to be included in set 1 for CB and in set 2 for AS

- A UE capability signaling is introduced to report what SRS resource can be shared for CB and AS

- Only apply for single-TRP operations

- Note: Rel-15/16 UL transmission operation is single-TRP operation

Alt 3:

- No spec change or new conclusion.

Regarding the proposed Alt 2, it seems that this does not address the original issue of the ambiguity on antenna virtualization and power control applicable to the shared SRS resource between multiple usages. On the other hand, Alt 0 and Alt 1 address the antenna virtualization and power control issue associated with the shared SRS resource.

Observation 3:

- Both Alt 0 and Alt 1 configure the specific antenna virtualization and power control associated with the shared SRS resource among multiple usages.

Compared to proposed conclusion under Alt 0, Alt 1 requires additional UE capability reporting and separate RRC configuration to enable the sharing of an SRS resource among multiple usages. As per our understanding, Alt 1 provides more flexibility and control. Further, there can be some backward compatibility issues with the proposed Alt 0. Due to those reasons, we slightly prefer Alt 1 over Alt 0. Accordingly, we make the following proposal.

Proposal 8:

- Slightly prefer Alt 1 which proposes additional UE capability reporting and separate RRC configuration to enable the sharing of an SRS resource among multiple usages.

For the transceiver architecture xTyR where x<y, by configuring shared SRS resources between ‘codebook’ and ‘antennaswitching’, it is possible to achieve antenna selection for PUSCH transmission. This is because, the channel estimations obtained using one or more SRS resource sets with usage ‘antennaswitching’ can be used for identifying which antennas are more preferable for PUSCH transmission. Subsequently, using SRI, associated SRS resource can be indicated to the UE. This is particularly important for handling UL inter-cell interference issues and hand blocking of terminal antennas. Accordingly, we make the following proposal.

Proposal 9:

- For xTyR case (x<y) with SRS resources shared between multiple usages, channel estimations obtained using one or more SRS resource sets with usage ‘antennaswitching’ can be used for identifying which antennas are more preferable for PUSCH transmission.

# SRS switching for DL CSI acquisition with up to 8 Rx antenna ports

During RAN1 #103-e meeting following new transceiver architectures, 1T6R, 1T8R, 2T6R, 2T8R, and 4T8R were agreed to support for antenna switching while 4T6R architecture was left for further study. Further, during RAN1#104-e meeting, it was agreed to flexibly allocate SRS resources among SRS resource sets for aperiodic antenna switching SRS. That is, for each newly introduced transceiver architecture, specification defines total number of available SRS resources and SRS resource sets. How to allocate these total number of SRS resources among SRS resource sets is flexible. In this section, we discuss how SRS resources can be flexibly allocated among SRS resource sets for DL CSI acquisition with antenna switching.

First of all, we propose to update UE capability indication parameter, supportedSRS-TxPortSwitch as follows to support agreed transceiver architectures:

Proposal 10:

- Following new capability indications should be supported by the parameter, supportedSRS-TxPortSwitch to capture new transceiver architectures supported with SRS antenna switching

  - 't1r6' for 1T6R

  - ‘t2r6' for 2T6R

  - 't1r1-t1r2-t1r4-t1r6’ for 1T=1R/1T2R/1T4R/1T6R

  - 't1r1-t1r2-t2r2-t1r4-t2r4-t1r6-t2r6' for 1T=1R/1T2R/2T=2R/1T4R/2T4R/1T6R/2T6R

  - ‘t1r6-t2r6' for 1T6R/2T6R

  - ‘t1r8' for 1T8R

  - ‘t2r8' for 2T8R

  - 't1r1-t1r2-t1r4-t1r6-t1r8’ for 1T=1R/1T2R/1T4R/1T6R/1T8R

    - 't1r1-t1r2-t2r2-t1r4-t2r4-t1r6-t2r6-t1r8-t2r8' for 1T=1R/1T2R/2T=2R/1T4R/2T4R/1T6R/2T6R/1T8R/2T8R

  - ‘t1r8-t2r8' for 1T8R/2T8R

  - ‘t4r6' for 4T6R

  - ‘t4r8' for 4T8R

  - 't1r1-t1r2-t2r2-t1r4-t2r4-t1r6-t2r6-t4r6-t1r8-t2r8-t4r8' for               			1T=1R/1T2R/2T=2R/1T4R/2T4R/1T6R/2T6R/4T6R/1T8R/2T8R/4T8R

## As per the agreement from RAN1#104-e meeting, K SRS resources in total are distributed among N <=N_max resource sets. In particular, specific K values and possible N_max values for each transceiver architecture have been captured within the agreement as follows.

Agreement

- For aperiodic antenna switching SRS, support to configure N <=N_max resource sets, where totally K resources are distributed in the N resource sets flexibly based on RRC configuration.

  - For 1T6R, K=6, N_max = [4], and each resource has 1 port.

  - For 1T8R, K=8, N_max = [4], and each resource has 1 port.

  - For 2T6R, K=3, N_max = [3], and each resource has 2 ports.

  - For 2T8R, K=4, N_max = [4], and each resource has 2 ports.

  - (Working Assumption) For 4T8R, K=2, N_max = [2], and each resource has 4 ports.

  - FFS the number of supported candidate values of N for each xTyR.

- FFS extension to increase N_max for 1T4R, 2T4R, T=R and 1T2R cases for aperiodic, periodic and semi-persistent SRS resources

- FFS the number of resources and resource sets for semi-persistent and periodic antenna switching SRS

- Note: SRS could be transmitted over the last 6 OFDM symbols, or over any OFDM symbols within the slot subject to UE capability.

## As per our view, for 1T6R, N_max = 3 is sufficient. This will allow configuring number of SRS resource sets, N to be 1, 2 or 3 for SRS antenna switching with 1T6R transceiver architecture. For 1T8R, 2T6R, and 2T8R, we are fine with the proposed N_max values in the agreement. Accordingly, we make the following proposal.

Proposal 11:

- For aperiodic antenna switching SRS, support to configure N <=N_max resource sets, where totally K resources are distributed in the N resource sets flexibly based on RRC configuration.

  - For 1T6R, K=6, N_max = 3, and each resource has 1 port.

  - For 1T8R, K=8, N_max = 4, and each resource has 1 port.

  - For 2T6R, K=3, N_max = 3, and each resource has 2 ports.

  - For 2T8R, K=4, N_max = 4, and each resource has 2 ports.

Further, regarding the 4T8R transceiver architecture, a working assumption was proposed for K and N_max values. The main reason for the working assumption was, some companies suggested to consider the coherence capability of the UE for this particular transceiver architecture. However, it is not clear to us why UE coherence capability needs to be considered for SRS antenna switching. Hence, we are supportive to the proposed working assumption and accordingly, we make the following proposal.

Proposal 12:

- Confirm working assumption: For 4T8R, K=2, N_max = [2], and each resource has 4 ports.

It is worth remarking here that, in order to reduce time associated with DL CSI acquisition (especially when there are multiple slots involved) considering SRS antenna switching, it is better to support zero-symbol gap SRS resource configuration. This can be useful for those capable UEs which can handle zero-symbol gap antenna switching. UE can report whether it can support zero-symbol gap antenna switching as part of its capability reporting. NW can then configure zero-symbol gap SRS resources only to those capable UEs.

# Summary

In this contribution, we have discussed possible approaches for enhancing SRS transmission in NR Rel. 17. Based on the discussion, we have made following proposals.

Proposal 1:

- Support Opt. 1 which proposes ‘reference slot’ as the slot with the triggering DCI.

Proposal 2:

- Confirm the proposed working assumption on the definition of ‘available slot’.

Proposal 3:

- Support dynamic switching of the UE behavior between the extended UE specific DCI format(s) without data/CSI and the existing UE specific DCI format(s) with data/CSI. Following options can be considered for the dynamic switching:

  - Option 1: Introduce new RNTI to indicate the extended DCI.

    - If the UE specific DCI is CRC scrambled by the new RNTI, this DCI is the extended DCI format 0_1/0_2 without uplink-data/CSI. Otherwise, this DCI is the existing UE specific DCI with uplink-data/CSI.

  - Option 2: Special combination of the existing DCI field to indicate the extended DCI.

    - If the UE specific DCI has the special combination in the existing DCI field, this DCI is the extended DCI format 0_1/0_2 without uplink-data/CSI. Otherwise, this DCI is the existing UE specific DCI with uplink-data/CSI.

Observation 1:

- Adding a new DCI field to indicate t value increases the DCI payload size.

Proposal 4:

- Rather than stick into a unified solution for the indication of t value using DCI with and without data and CSI scheduling, consider a more flexible solution. In particular, following can be one viable approach

- If RRC configures the availability of a new DCI field, t value is indicated using that new DCI field

- Otherwise, repurpose one of the existing DCI field(s) (which is not used for data and CSI scheduling) to indicate t value for DCI without data/CSI, and t value is selected by pre-determined rule from the list configured by higher layer signaling for DCI with data/CSI.

- Note: maintain the same DCI size for both cases with and without data and CSI scheduling.

Observation 2:

- There is no clear understanding on how to modify or map the existing DCI fields to capture newly proposed parameters.

Proposal 5:

- Consider following approaches to associate DCI fields within DCI formats 0_1/0_2 when repurposing the existing DCI fields to capture new parameters

  - Rename the existing DCI fields

  - Define a mapping rule between the existing DCI field names and new parameters.

Proposal 6:

- Support extending number of DCI codepoints to have at least 7 A-SRS trigger states.

Proposal 7:

- Introduce MAC CE to improve the flexibility for triggering aperiodic SRS, and following alternatives should be considered

  - Alt.1) MAC CE activates/deactivates one or multiple SRS resource sets, and only active SRS resource set(s) can be triggered by SRS request field of DCI

  - Alt.2) MAC CE updates the mapping b/w DCI codepoint of SRS request field and aperiodicSRS-ResourceTrigger

  - Alt. 3) MAC CE updates specific parameter (e.g. slotOffset) in SRS resource sets/SRS resource directly

- For Alt. 1/2, the number of configurable SRS resource sets for a given usage is increased.

Observation 3:

- Both Alt 0 and Alt 1 configure the specific antenna virtualization and power control associated with the shared SRS resource among multiple usages.

Proposal 8:

- Slightly prefer Alt 1 which proposes additional UE capability reporting and separate RRC configuration to enable the sharing of an SRS resource among multiple usages.

Proposal 9:

- For xTyR case (x<y) with SRS resources shared between multiple usages, channel estimations obtained using one or more SRS resource sets with usage ‘antennaswitching’ can be used for identifying which antennas are more preferable for PUSCH transmission.

Proposal 10:

- Following new capability indications should be supported by the parameter, supportedSRS-TxPortSwitch to capture new transceiver architectures supported with SRS antenna switching

  - 't1r6' for 1T6R

  - ‘t2r6' for 2T6R

  - 't1r1-t1r2-t1r4-t1r6’ for 1T=1R/1T2R/1T4R/1T6R

  - 't1r1-t1r2-t2r2-t1r4-t2r4-t1r6-t2r6' for 1T=1R/1T2R/2T=2R/1T4R/2T4R/1T6R/2T6R

  - ‘t1r6-t2r6' for 1T6R/2T6R

  - ‘t1r8' for 1T8R

  - ‘t2r8' for 2T8R

  - 't1r1-t1r2-t1r4-t1r6-t1r8’ for 1T=1R/1T2R/1T4R/1T6R/1T8R

    - 't1r1-t1r2-t2r2-t1r4-t2r4-t1r6-t2r6-t1r8-t2r8' for 1T=1R/1T2R/2T=2R/1T4R/2T4R/1T6R/2T6R/1T8R/2T8R

  - ‘t1r8-t2r8' for 1T8R/2T8R

  - ‘t4r6' for 4T6R

  - ‘t4r8' for 4T8R

  - 't1r1-t1r2-t2r2-t1r4-t2r4-t1r6-t2r6-t4r6-t1r8-t2r8-t4r8' for               			1T=1R/1T2R/2T=2R/1T4R/2T4R/1T6R/2T6R/4T6R/1T8R/2T8R/4T8R

Proposal 11:

- For aperiodic antenna switching SRS, support to configure N <=N_max resource sets, where totally K resources are distributed in the N resource sets flexibly based on RRC configuration.

  - For 1T6R, K=6, N_max = 3, and each resource has 1 port.

  - For 1T8R, K=8, N_max = 4, and each resource has 1 port.

  - For 2T6R, K=3, N_max = 3, and each resource has 2 ports.

  - For 2T8R, K=4, N_max = 4, and each resource has 2 ports.

Proposal 12:

- Confirm working assumption: For 4T8R, K=2, N_max = [2], and each resource has 4 ports.

# References

- 3GPP RAN1#104-e, “RAN1 Chairman’s Notes”, Jan. 2021

- 3GPP TS 38.212, “NR; Multiplexing and channel coding (Release 16)”