3GPP TSG RAN WG1 #104-bis-e		R1-2103617

e-meeting, April 12th –  20th, 2021

Source:	Lenovo, Motorola Mobility

Title: 	Enhancements for joint channel estimation for multiple PUSCH

Agenda Item:   8.8.1.3

Document for:	Discussion

# 1	Introduction

In RAN#91-e, revised work item for NR coverage enhancements in Rel-17 has been approved with following objectives for in RP-210855 [1]:

- Specification of PUSCH enhancements [RAN1, RAN4]

  - Specify the following mechanisms for enhancements on PUSCH repetition type A [RAN1]

    - Increasing the maximum number of repetitions up to a number to be determined during the course of the work.

    - The number of repetitions counted on the basis of available UL slots.

  - Specify mechanism(s) to support TB processing over multi-slot PUSCH [RAN1]

    - TBS determined based on multiple slots and transmitted over multiple slots.

  - Specify mechanism(s) to enable joint channel estimation [RAN1, RAN4]

    - Mechanism(s) to enable joint channel estimation over multiple PUSCH transmissions, based on the conditions to keep power consistency and phase continuity to be investigated and specified if necessary by RAN4 [RAN1, RAN4]

      - Potential optimization of DMRS location/granularity in time domain is not precluded

    - Inter-slot frequency hopping with inter-slot bundling to enable joint channel estimation [RAN1]

- Specification of PUCCH enhancements [RAN1, RAN4]

  - Specify signaling mechanism to support dynamic PUCCH repetition factor indication [RAN1]

  - Specify mechanism to support DMRS bundling across PUCCH repetitions [RAN1, RAN4]

    - When applicable, based on similar mechanism(s) for enabling joint channel estimation for PUSCH

- Specify mechanism(s) to support Type A PUSCH repetitions for Msg3 [RAN1]

In this contribution, we provide our views on following aspects related to joint channel estimation across multiple PUSCHs for improved coverage:

- Use cases for joint channel estimation

- DM-RS time-domain pattern across multiple PUSCHs

- Inter-slot frequency hopping with inter-slot bundling

- Time domain window for joint channel estimation

- Configuration/indication of joint channel estimation

# 2	Use cases for joint channel estimation

In RAN1#104-e, following use cases have been discussed for joint channel estimation [2]:

Agreements:

- Following potential use cases are considered for joint channel estimation for PUSCH:

  - Use case 1: back-to-back PUSCH transmissions within one slot.

  - Use case 2: non-back-to-back PUSCH transmissions within one slot.

  - Use case 3: back-to-back PUSCH transmissions across consecutive slots.

  - Use case 4: non-back-to-back PUSCH transmissions across consecutive slots.

  - Use case 5: PUSCH transmissions across non-consecutive slots.

Note: RAN1 assumes “back-to-back PUSCH transmission” has zero gap in-between adjacent PUSCH transmissions.

- For back-to-back PUSCH transmissions across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation at least for the following case:

  - Over back-to-back PUSCH transmissions (of the same TB) for repetition type A scheduled by dynamic grant or configured grant

  - FFS details (including possible other cases)

For the use cases with back-to-back PUSCH transmissions including use case 1 and 3, joint channel estimation should be supported under the condition of phase continuity and power consistency. For the use cases with non-back-to-back PUSCH transmissions for both inter-slot and intra-slot, it should be up to RAN4’e feedback whether the phase continuity can be maintained and the maximum allowed gap between adjacent PUSCH transmissions. Therefore, the starting assumption for specifying joint channel estimation should be mainly for back-to-back PUSCH transmissions for both intra-slot and inter-slot.

Proposal 1: For specifying joint channel estimation with DM-RS bundling across multiple PUSCHs for coverage enhancements in NR Rel-17, use cases with back-to-back PUSCH transmission within one slot in addition to across multiple slots (repetition type A) are supported.

- Support of joint channel estimation for non-back-to-back PUSCH transmissions is dependent up on RAN4’s input

# 3	DM-RS time-domain pattern across multiple PUSCH for joint channel estimation

One of the key design aspects is related to the DM-RS pattern in time domain for joint channel estimation. Currently, when multiple PUSCH transmissions or repetitions are scheduled, all the transmission occasions have at least one front-loaded DM-RS. In this case, if DM-RS bundling is applied for low-mobility scenarios, then joint channel estimation across multiple front-loaded DM-RS across multiple PUSCHs should be sufficient and no enhancement to patterns would be needed. Basically, the front-loaded DM-RS symbols are equally spaced across all PUSCH.

However, if additional DM-RS symbols within each of the PUSCH transmission are needed for relatively higher mobility and DM-RS bundling is applied, then the overall DM-RS time-domain pattern across multiple PUSCHs might not always be ideal in terms of spacing between the DM-RS symbols according to current additional DM-RS patterns supported in NR Rel-15/16. For example, as illustrated in Figure 1, when one additional (1-symbol type) DM-RS is transmitted in each of the PUSCH transmission (with mapping type A) according to the current patterns specific in TS 38.211. Then the DM-RS symbols are not equally spaced. Basically, front-loaded DM-RS is transmitted on 3rd symbol of each of the slot and the additional DM-RS symbol is transmitted on 12th symbol for each of the slot. As a result, the additional symbol of the slot is very closely spaced (4 symbols gap) with the front-loaded DMRS symbol of the next slot, while the gap between the front-loaded DM-RS and the additional DM-RS within a slot is quite large i.e. 8 symbols gap. When DM-RS bundling is not applied, as it is currently the case in NR, then such DM-RS pattern is adopted to avoid too long extrapolation between channel estimates. Therefore, from the last slot point of view, the pattern is still fine as it needs to extrapolate channel estimates only for 2 symbols. Therefore, enhancements to additional DM-RS patterns should be supported when DM-RS bundling is applied across multiple PUSCH transmissions.

Figure 1: Example of additional DM-RS pattern for multiple PUSCH (currently supported by NR Rel-15/16)

Proposal 2: For supporting joint channel estimation with DM-RS bundling across multiple PUSCHs for coverage enhancements in NR Rel-17, additional DM-RS time-domain pattern should be enhanced with following consideration:

- to support equally spaced DM-RS symbols across multiple PUSCHs (new design for additional DMRS symbols)

- to avoid extrapolation for large number of symbols for the last PUSCH (similar design aspect as supported in NR)

Based on the proposed design aspects for additional DM-RS time-domain pattern in proposal 1 above, an example is illustrated in Figure 2 that is an alternative design to the one shown in Figure 1. In this design, the position of additional DM-RS on slot 1 and slot is adjusted to create equal spacing, while the slot 3 has same configuration.

Figure 2: Example of enhanced additional DM-RS pattern for multiple PUSCH

To generalize this, it could be said that the additional DM-RS time-domain pattern for all the PUSCH transmissions, except the last one should be enhanced to support equal spacing. For the last PUSCH transmission, current additional DM-RS pattern still applies to avoid extrapolation on large number of symbols. For indicating such design, one solution would be to define extended DM-RS positions beyond 14 symbols. However, this is not a feasible solution as it might require specifying a large number of additional DM-RS patterns depending upon the duration for DM-RS bundling. One possibility could be to configured UE with two configurations for additional DM-RS pattern in time domain for each of the PUSCH transmission. One configuration should be the same as currently supported in NR for applying on the last PUSCH transmission occasion. Second configuration could be newly defined configuration for applying on all, but last PUSCH transmission occasion. This would not require extending the additional DM-RS configurations more than 14 symbols.

Proposal 3: For supporting joint channel estimation with DM-RS bundling across multiple PUSCHs for coverage enhancements in NR Rel-17, UE should be configured with an additional new configuration for additional DM-RS patterns (for up to 14 symbols) to apply to all PUSCH transmissions, but last

- For the last PUSCH transmission, the current additional DM-RS configuration should be applied

# 4	Inter-slot frequency hopping with inter-slot bundling

Another aspect related to joint channel estimation using DM-RS bundling is for multiple PUSCH transmissions with frequency hopping. In our view, if frequency hopping is enabled for multiple PUSCH transmissions, then the DM-RS bundling should be contained within a single hop. Alternatively, DM-RS bundle duration should be same as frequency hop duration. Otherwise, phase continuity might be an issue for joint channel estimation across multiple frequency hops. However, if such limitation is applied, then the DM-RS bundling is not really beneficial as currently only intra-slot or inter-slot frequency hopping is supported. Therefore, multi-slot frequency hopping should be supported. This would provide better opportunity for joint channel estimation with DM-RS bundling. As illustrated in Figure 3, frequency hop duration is across 3 slots and therefore, the DM-RS bundling is also applied in multiples of 3 slots for each of the frequency hop.

Proposal 4: For supporting joint channel estimation with DM-RS bundling across multiple PUSCHs for coverage enhancements in NR Rel-17, support multi-slot frequency hopping and multi-slot DM-RS bundling for joint channel estimation for entire hop:

- Association between frequency hop duration and DM-RS bundle duration should be supported

- At least hop duration of 2 slots should be supported with DM-RS bundling

Figure 3: Example multi-slot frequency hopping with DM-RS bundling for multiple PUSCHs

# 5	Time domain window for joint channel estimation

In RAN1#104-e, following agreement related to definition of time-domain window for joint channel estimation has been made [2]:

Agreements:

- For joint channel estimation, a time domain window is introduced to facilitate further discussion, during which UE is expected to maintain power consistency and phase continuity among PUSCH transmissions subject to power consistency and phase continuity requirements.

  - FFS: whether the window should be specified

  - FFS: the length of the time domain window is defined by a set of repetitions/slots/symbols

  - FFS: single or multiple time domain windows

- FFS: relation with UE capability

- FFS: the time domain window may or may not be configured or specified.

- FFS: whether the term "time domain window" is used in the specification or replaced by other technical terms

- FFS: Whether the window is determined by the power consistency and phase continuity requirements and/or by other factors is to be decided.

In our view, time domain window for the purpose of joint channel estimation DM-RS bundling should be specified and following details should be supported:

- Maximum duration for the time-domain window should be determined based on the minimum of following two durations:

  - Maximum duration for which power consistency and phase continuity can be maintained

  - Maximum duration of PUSCH transmissions (for maximum value of repetition factor)

- Depending upon coverage requirements, the duration of the time-domain window can be additionally configured/indicated (duration value could be smaller than the maximum duration) and the UE is expected to maintain phase continuity over at least each successive non-overlapping time-domain window duration starting from the first PUSCH transmission

- For a burst of PUSCH transmissions with joint channel estimation, only a single duration of the time domain window should be configured/indicated

- For the case of joint channel estimation with frequency hopping and back-to-back PUSCH transmissions at least within a hop, explicit indication/configuration for time domain window can be avoided and implied by the duration of the hop

- DM-RS bundling duration could be possibly considered as a term to be included in specifications.

Proposal 5: For supporting joint channel estimation with DM-RS bundling across multiple PUSCHs for coverage enhancements in NR Rel-17, time domain window for the purpose of joint channel estimation DM-RS bundling should be specified with following details:

- Maximum duration for the time-domain window should be determined based on the minimum of following two durations:

  - Maximum duration for which power consistency and phase continuity can be maintained

  - Maximum duration of PUSCH transmissions (depend on maximum value of repetition factor)

- Depending upon coverage requirements, the duration of the time-domain window can be configured/indicated (duration value could be smaller than the maximum duration)

- For a burst of PUSCH transmissions with joint channel estimation, only a single duration of the time domain window should be configured/indicated

- For the case of joint channel estimation with frequency hopping, explicit indication/configuration for time domain window can be avoided and implied by the duration of the hop

- DM-RS bundling duration could be possible considered as a term to be included in specifications.

# 6	Configuration/indication of joint channel estimation

In RAN1#104-e, there has been discussion on how the joint channel estimation can be configured/indicated. Two possibilities have been considered:

- Dedicated signaling to enable or disable joint channel estimation

- Implicit/joint signaling based on other parameters

In our view, if configuration/indicated related to time domain window is introduced, then separate configuration/indication of joint channel estimation can be avoided. Further discussion could be on whether dynamic indication or semi-static indication of time domain window is needed. Considering the availability of UL symbols/slots is based on dynamic SFI and cancellation indication (in addition to semi-static TDD configuration), the duration for the time domain window (i.e. enable/disable of joint channel estimation) should be dynamically signaled such that it can be based on actual duration of PUSCH transmission.

Proposal 6: For supporting joint channel estimation with DM-RS bundling across multiple PUSCHs for coverage enhancements in NR Rel-17, enabling or disabling of joint channel estimation can be jointly indicated by the presence of signalling for time domain window duration

- Dynamic signalling of time domain window duration should be supported

# 7	Conclusion

In this section, we summarize the observation/proposals from above section:

Proposal 1: For specifying joint channel estimation with DM-RS bundling across multiple PUSCHs for coverage enhancements in NR Rel-17, use cases with back-to-back PUSCH transmission within one slot in addition to across multiple slots (repetition type A) are supported.

- Support of joint channel estimation for non-back-to-back PUSCH transmissions is dependent up on RAN4’s input

Proposal 2: For supporting joint channel estimation with DM-RS bundling across multiple PUSCHs for coverage enhancements in NR Rel-17, additional DM-RS time-domain pattern should be enhanced with following consideration:

- to support equally spaced DM-RS symbols across multiple PUSCHs (new design for additional DMRS symbols)

- to avoid extrapolation for large number of symbols for the last PUSCH (similar design aspect as supported in NR)

Proposal 3: For supporting joint channel estimation with DM-RS bundling across multiple PUSCHs for coverage enhancements in NR Rel-17, UE should be configured with an additional new configuration for additional DM-RS patterns (for up to 14 symbols) to apply to all PUSCH transmissions, but last

- For the last PUSCH transmission, the current additional DM-RS configuration should be applied

Proposal 4: For supporting joint channel estimation with DM-RS bundling across multiple PUSCHs for coverage enhancements in NR Rel-17, support multi-slot frequency hopping and multi-slot DM-RS bundling for joint channel estimation for entire hop:

- Association between frequency hop duration and DM-RS bundle duration should be supported

- At least hop duration of 2 slots should be supported with DM-RS bundling

Proposal 5: For supporting joint channel estimation with DM-RS bundling across multiple PUSCHs for coverage enhancements in NR Rel-17, time domain window for the purpose of joint channel estimation DM-RS bundling should be specified with following details:

- Maximum duration for the time-domain window should be determined based on the minimum of following two durations:

  - Maximum duration for which power consistency and phase continuity can be maintained

  - Maximum duration of PUSCH transmissions (depend on maximum value of repetition factor)

- Depending upon coverage requirements, the duration of the time-domain window can be configured/indicated (duration value could be smaller than the maximum duration)

- For a burst of PUSCH transmissions with joint channel estimation, only a single duration of the time domain window should be configured/indicated

- For the case of joint channel estimation with frequency hopping, explicit indication/configuration for time domain window can be avoided and implied by the duration of the hop

- DM-RS bundling duration could be possible considered as a term to be included in specifications.

Proposal 6: For supporting joint channel estimation with DM-RS bundling across multiple PUSCHs for coverage enhancements in NR Rel-17, enabling or disabling of joint channel estimation can be jointly indicated by the presence of signalling for time domain window duration

- Dynamic signalling of time domain window duration should be supported

# 8	References

[1]	3GPP RP-210855, “New WID on NR coverage enhancements”, China Telecom

[2] 	3GPP RAN1#103-e. “Chairman Notes”