3GPP TSG RAN WG1 #104b-e				R1-2103693

e-Meeting, April 12th – 20th, 2021

Source:	WILUS Inc.

Title:	Discussion on multi-PDSCH/PUSCH scheduling for NR from 52.6GHz to 71GHz

Agenda item:	8.2.5

Document for:	Discussion/Decision

- Introduction

New Rel-17 WI on supporting NR from 52.6 GHz to 71 GHz was approved in RAN#90-e meeting and its objectives are listed as following [1]:

- Physical layer aspects including [RAN1]: - In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported. Note: Except for timing line related aspects, a common design framework shall be adopted for 480kHz to 960kHz - Time line related aspects adapted to 480kHz and 960kHz, e.g., BWP and beam switching timeing, HARQ timing, UE processing, preparation and computation timelines for PDSCH, PUSCH/SRS and CSI, respectively. - Support of up to 64 SSB beams for licensed and unlicensed operation in this frequency range. - Supports 120kHz SCS for SSB and 120kHz SCS for initial access related signals/channels in an initial BWP. - Study and specify, if needed, additional SCS (240kHz, 480kHz, 960kHz) for SSB, and additional SCS(480kHz, 960kHz) for initial access related signals/channels in initial BWP. - Study and specify, if needed, additional SCS (480kHz, 960kHz) for SSB for cases other than initial access. - Note: coverage enhancement for SSB is not pursued. - Specify timing associated with beam-based operation to new SCS (i.e., 480kHz and/or 960kHz), study, and specify if needed, potential enhancement for shared spectrum operation - Rel-15/16 and any Rel-17 beam management enhancements can be considered for 52.6-71 GHz. Whether particular features should be excluded for 52.6-71 GHz can be further discussed - Note: As per usual procedure, duplication of work between work items in Rel-17 should be avoided - Support enhancement for PUCCH format 0/1/4 to increase the number of RBs under PSD limitation in shared spectrum operation. - Support enhancements for multi-PDSCH/PUSCH scheduling and HARQ support with a single DCI Note: coverage enhancement for multi-PDSCH/PUSCH scheduling is not pursued - Support enhancement to PDCCH monitoring, including blind detection/CCE budget, and multi-slot span monitoring, potential limitation to UE PDCCH configuration and capability related to PDCCH monitoring. - Specify support for PRACH sequence lengths (i.e. L=139, L=571 and L=1151) and study, if needed, specify support for RO configuration for non-consecutive RACH occasions (RO) in time domain for operation in shared spectrum - Evaluate, and if needed, specify the PTRS enhancement for 120kHz SCS, 480kHz SCS and/or 960kHz SCS, as well as DMRS enhancement for 480kHz SCS and/or 960kHz SCS. …

In this contribution, we discuss enhancements for multi-PDSCH/PUSCH scheduling and HARQ support with a single DCI.

- Discussion on multi-PDSCH/PUSCH scheduling and HARQ support with a single DCI

Issue 1: Type-2 HARQ-ACK codebook for multi-PDSCH scheduling with a single DCI

Agreement at RAN1 #104-e: [2] For generating type-2 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, the following alternatives can be considered to DAI counting and will be down-selected in RAN1#104bis-e. - Alt 1: C-DAI/T-DAI is counted per DCI. - Alt 2: C-DAI/T-DAI is counted per PDSCH. - Alt 3: C-DAI/T-DAI is counted per M scheduled PDSCH(s), where M is configurable (e.g., 1, 2, 4, …). - FFS: Codebook generation details - FFS: How to signal DAI values (e.g., increase of DAI bits for Alt 2 and Alt 3) - FFS: Whether to apply time domain bundling of HARQ-ACK feedback

Since a single DCI schedules more than one PDSCHs with different TBs, it is not clear how to interpretate C-DAI and T-DAI in the DCI. In the last RAN1#104-e meeting, three alternatives were identified [2]. The three alternatives are illustrated in Figure 1.

Figure 1. DAI counting rules. (a) Alt 1, (b) Alt 2, and (c) Alt 3. oi represents HARQ-ACK information of TBi.

In Alt 1, C-DAI and T-DAI are counted per DCI and each increment represents the maximum number of PDSCHs scheduled by a single DCI. In Figure 1-(a), the maximum number of PDSCHs is assumed to 4. The merit of the Alt 1 is that the definition of C-DAI and T-DAI in Rel-15/16 is not changed and the number of bits for C-DAI and T-DAI can be kept as 2 bits. However, it may be resulted in increasing some overhead. For example, even if the DCI in CC#0 with (C-DAI,T-DAI)=(2,3) schedules 3 PDSCHs and the DCI in CC#2 with (C-DAI,T-DAI)=(4,4) schedules 2 PDSCHs, but a UE assumes that each of these DCIs schedules 4 PDSCHs to avoid Type-2 CB size ambiguity between UE and gNB. Therefore, some filler bits, ‘x’ would be inserted to the Type-2 CB. As a result, 13 TBs are scheduled, but the Type-2 CB includes 16 bits (with 3 filler bits).

In Alt 2, C-DAI and T-DAI are counted per PDSCH and each increment represents single PDSCH. Contrary to Alt 1, Alt 2 has no overhead in Type-2 CB. That is, if 13 TBs are scheduled, then the Type-2 CB includes 13 bits without any filler bits. Therefore, in terms of Type-2 CB size, Alt 2 would be the best among the three alternatives. However, it requires a greater number of bits for C-DAI and T-DAI than the number of bits as in Rel-15 to keep the same DTX capability as in Rel-15/16. Note that in Rel-15/16 up to 3 consecutive DTXs can be detected by 2-bit C-DAI and 2-bit T-DAI. For example, if 3-bit C-DAI and 3-bit T-DAI are used, then (C-DAI,T-DAI)=(4,4) in the DCI of CC#1, (C-DAI,T-DAI)=(7,3) in the DCI of CC#0, (C-DAI,T-DAI)=(3,3) in the DCI of CC#3, and (C-DAI,T-DAI)=(5,5) in the DCI of CC#2. If the DCIs of CC#0/1/3 are missed (consecutive 3 DTXs), then the UE generates 5-bit Type-2 CB since (C-DAI,T-DAI)=(5,5) in the DCI of CC#2 is detected. Note that 4-bit C-DAI and 4-bit T-DAI can address this issue.

In Alt 3, C-DAI and T-DAI are counted per M scheduled PDSCH(s), where M is configurable. The Alt 3 is a generalization of Alt 1 and Alt 2 by introducing a configurable parameter M. That is, if M=1 is configured, then the Alt 3 is same as the Alt 1, and if M is configured to the maximum number of PDSCHs scheduled by a single DCI, then the Alt 3 is same as the Alt 2. Figure 2 illustrates M=2 case, where each increment represents 2 PDSCH(s). Even if the DCI in CC#0 schedules 3 PDSCHs, then 4 bits (including 1 filler bit) are inserted in Type-2 CB. Therefore, the Type-2 CB size is 14 bits, which is middle of Alt 1 and Alt 3.

In fact, similar issues were discussed in case of CBG-based transmission in Rel-15, where the number of scheduled CBGs can be varied across different DCIs so that the number of HARQ-ACK bits for the scheduled CBGs can be also varied. Also, the number of CBGs can be configured as 2/4/8 per PDSCH and it can be different in different CCs. The pros and cons of each alternatives were intensively discussed. Finally, RAN1 agreed to support Alt 1-like approach for CBG-based transmission, i.e., C-DAI and T-DAI are counted per DCI and each increment corresponds to the maximum number of CBGs. For this issue, the same approach can be reused.

- Proposal 1: We propose to support Alt 1 as DAI counting for Type-2 HARQ-ACK CB when scheduling multiple PDSCHs in single DCI.

  - Alt 1: C-DAI/T-DAI is counted per DCI.

Issue 2: Type-1 HARQ-ACK codebook for multi-PDSCH scheduling with a single DCI

In Rel-15/16, Type-1 HARQ-ACK codebook is generated based on SLIVs in a TDRA table and K1 values. The difference of multi-PDSCH scheduling by a single DCI is the followings:

- 1) the K1 value in the single DCI represents the slot offset between the slot of the last PDSCH scheduled by the singe DCI and the slot carrying the HARQ-ACK information corresponding to the scheduled PDSCHs. So, the corresponding effective K1 values, defined by the slot offset between the slot of the PDSCH and the slot carrying the HARQ-ACK information, can be derived.

- 2) More than one SLIVs for an entry in the TDRA table can be configured and the SLIVs is allocated to the same or different slots. Note that the TDRA table may not include K0 values for each SLIV, but a UE can know which slot is assigned to a SLIV in the TDRA table by pre-determined way.

According to the above two points, when a PUCCH carrying HARQ-ACK information of multi-PDSCH scheduling is transmitted at slot n, the UE can identify which SLIVs are associated to a slot n-K1eff, where K1eff is an effective K1 value. The pseudo-code for Rel-15/16 Type-1 HARQ-ACK CB construction rule can be reused by replacing with 1), 2) as follows:

- the set K1 including the configured K1 values to the set K1 including the effective K1 values and

- the set R including all SLIVs in a TDRA table to the set R including SLIVs corresponding to the effective K1 value.

The Figure 2 illustrates the proposed Type-1 HARQ-ACK CB construction rule, where a TDRA table in a single DCI is given in Table 1 and K1 values in the single DCI are configured to {1,2}. Here, the effective K1 values are {1,2,3,4,5}, and the UE identifies that

- the SLIVs (S,L) = (0,14), (0,7), (7,7) are associated to the slot n-5 (where the effective K1 value is 5),

- the SLIVs (S,L) = (0,14), (0,7), (7,7) are associated to the slot n-4 (where the effective K1 value is 4),

- the SLIVs (S,L) = (0,14), (0,7), (7,7) are associated to the slot n-3 (where the effective K1 value is 3),

- the SLIVs (S,L) = (0,14), (0,7), (7,7) are associated to the slot n-2 (where the effective K1 value is 2),

- the SLIVs (S,L) = (0,14), (0,7), (7,7) are associated to the slot n-1 (where the effective K1 value is 1).

Figure 2. Type-1 HARQ-ACK CB construction

Therefore, the pseudo-code for Type-1 HARQ-ACK CB can be performed with K1={1,2,3,4,5} and R={(0,14), (0,7), (7,7)} for all the effective K1 value. For example, if K1,k=3 is selected, two HARQ-ACK occasions, one for (0,14) and (0,7) and another for (7,7) can be derived. In this example, 10 HARQ-ACK occasions (index of 10 HARQ-ACK occasions are shown in Figure 2) can be derived.

It is worth noting that the Type-1 HARQ-ACK CB construction rule in the proposal 2 is not optimal in terms of CB size. In fact, with the TDRA table in Table 1, up to 8 HARQ-ACK occasions are enough. It is because a single DCI schedules multi-PDSCH so that SLIVs in slots are not independent. For example, if a UE detects a DCI with TDRA index of 4 and K1=2, then the UE is, additionally, able to detect a DCI with TDRA index of 4 and K1=2 or TDRA index of 4 and K1=1. In this case, the UE receives 8 PDSCHs so that 8 HARQ-ACK occasions are needed, but the proposed Type-1 HARQ-ACK CB construction rule generates 10 HARQ-ACK occasions. However, such an overhead can be controlled by gNB’s scheduler and optimization of Type-1 HARQ-ACK CB may cause an additional UE complexity. Also, Type-2 HARQ-ACK CB can be used by configuration if the size of HARQ-ACK CB is crucial.

- Proposal 2: For Type-1 HARQ-ACK codebook construction for multi-PDSCH scheduling by a single DCI, reuse Rel-15/16 pseudo-code in the Clause 9.1.2 of TS38.213 with the following change:

  - the set K1 including the configured K1 values to the set K1 including the effective K1 values and

  - the set R including all SLIVs in a TDRA table to the set R including SLIVs corresponding to the effective K1 value.

- Conclusion

In this contribution, we discussed enhancements for multi-PDSCH/PUSCH scheduling and HARQ support with a single DCI. The followings are proposed:

- Proposal 1: We propose to support Alt 1 as DAI counting for Type-2 HARQ-ACK CB when scheduling multiple PDSCHs in single DCI.

  - Alt 1: C-DAI/T-DAI is counted per DCI.

- Proposal 2: For Type-1 HARQ-ACK codebook construction for multi-PDSCH scheduling by a single DCI, reuse Rel-15/16 pseudo-code in the Clause 9.1.2 of TS38.213 with the following change:

  - the set K1 including the configured K1 values to the set K1 including the effective K1 values and

  - the set R including all SLIVs in a TDRA table to the set R including SLIVs corresponding to the effective K1 value.

References

- RP-210862, Revised WID: Extending current NR operation to 71 GHz, CMCC

- Draft Report of 3GPP TSG RAN WG1 #104e v0.3.0