3GPP TSG RAN WG1#104b-e					R1-2103765

e-Meeting, April 12th – 20th, 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	Maintenance for Resource allocation for sidelink - Mode 2

Agenda item:	7.2.4

Document for:	Discussion and Decision

# Introduction

In this contribution we discuss remaining editorial issues relevant for mode 2.

# Discussion

TS 38.331 defines a parameter in resource pool configuration for mode 2, sl-ResourceReservePeriodList:

sl-ResourceReservePeriodList-r16       SEQUENCE (SIZE (1..16)) OF SL-ResourceReservePeriod-r16           OPTIONAL,   -- Need M

SL-ResourceReservePeriod-r16 ::=       CHOICE {

sl-ResourceReservePeriod1-r16          ENUMERATED {ms0, ms100, ms200, ms300, ms400, ms500, ms600, ms700, ms800, ms900, ms1000},

sl-ResourceReservePeriod2-r16          INTEGER (1..99)

}

SL-UE-SelectedConfigRP field descriptions

sl-ResourceReservePeriodList Set of possible resource reservation period allowed in the resource pool in the unit of ms. Up to 16 values can be configured per resource pool.

As can be seen, sl-ResourceReservePeriodList is a list/set of periodicities, each of whose elements is a single periodicity which can be either sl-ResourceReservePeriod1 or sl-ResourceReservePeriod2.

TS 38.214 on the other hand references sl-ResourceReservePeriod1 where sl-ResourceReservePeriodList is intended:

## 16.4 UE procedure for transmitting PSCCH A UE can be provided a number of symbols in a resource pool, by sl-TimeResourcePSCCH, starting from a second symbol that is available for SL transmissions in a slot, and a number of PRBs in the resource pool, by sl-FreqResourcePSCCH, starting from the lowest PRB of the lowest sub-channel of the associated PSSCH, for a PSCCH transmission with a SCI format 1-A. A UE that transmits a PSCCH with SCI format 1-A using sidelink resource allocation mode 2 [6, TS 38.214] sets - "Resource reservation period" as an index in sl-ResourceReservePeriod1 corresponding to a reservation period provided by higher layers [11, TS 38.321], if the UE is provided sl-MultiReserveResource

Proposal 1: Adopt the following text proposal replacing sl-ResourceReservePeriod1 by sl-ResourceReservePeriodList

-------------------------- Start of Text Proposal for TS 38.214 --------------------------

## 16.4	UE procedure for transmitting PSCCH

------------------- < Unchanged parts are omitted > --------------------------

A UE that transmits a PSCCH with SCI format 1-A using sidelink resource allocation mode 2 [6, TS 38.214] sets

-	"Resource reservation period" as an index in sl-ResourceReservePeriodList corresponding to a reservation period provided by higher layers [11, TS 38.321], if the UE is provided sl-MultiReserveResource

------------------- < Unchanged parts are omitted > --------------------------

-------------------------- End of Text Proposal for TS 38.214 --------------------------

# Conclusions

In this contribution we discussed remaining editorial issues in sidelink resource allocation mode 2 and made the following proposal:

Proposal 1: Adopt the following text proposal replacing sl-ResourceReservePeriod1 by sl-ResourceReservePeriodList

-------------------------- Start of Text Proposal for TS 38.214 --------------------------

## 16.4	UE procedure for transmitting PSCCH

------------------- < Unchanged parts are omitted > --------------------------

A UE that transmits a PSCCH with SCI format 1-A using sidelink resource allocation mode 2 [6, TS 38.214] sets

-	"Resource reservation period" as an index in sl-ResourceReservePeriodList corresponding to a reservation period provided by higher layers [11, TS 38.321], if the UE is provided sl-MultiReserveResource

------------------- < Unchanged parts are omitted > --------------------------

-------------------------- End of Text Proposal for TS 38.214 --------------------------

# References

- RP-200129, “Revised WID on 5G V2X with NR sidelink”, 3GPP RAN #87e