3GPPTSG-RAN WG1 #104b-e                                                                                   R1-2103958

e-Meeting,April12th –20th,2021

AgendaItem:      8.10.1

Source:               Moderator(AT&T)

Title:                   Summaryof[104b-e-NR-eIAB-01]

Documentfor:    Discussion/Approval

# 1	Introduction

ThiscontributionprovidesasummaryofthediscussioninRAN1#104bis-eforthefollowingemail discussion:

/ThisoneistouseNWM–pleaseuse RAN1-104b-e-NWM-NR-eIAB-01 asthedocumentname

[104b-e-NR-eIAB-01]Emaildiscussiononenhancementstoresourcemultiplexingbetweenchildand parentlinksofanIABnode–Thomas(AT&T)

1st checkpoint: April15

2nd checkpoint: April20

# 2	Simultaneous Operation of Access and Backhaul Links

FromtheeIABWID:

SpecificationofenhancementstotheresourcemultiplexingbetweenchildandparentlinksofanIAB node,including:

Supportofsimultaneousoperation(transmissionand/orreception)ofIAB-node’schild andparentlinks(i.e.,MTTx/DUTx,MTTx/DURx,MTRx/DUTx,MTRx/DU Rx)

Supportfordual-connectivityscenariosdefinedbyRAN2/RAN3inthecontextoftopology redundancyforimprovedrobustnessandloadbalancing.

## ISSUE2.1:FREQUENCYDOMAINMULTIPLEXINGRESOURCE GRANULARITY

Agreement

Theextensionofthesemi-staticDUresourcetypeindicationtofrequency-domainresourceswithina carrier(inadditiontoexistingRel-16per-carriergranularity)forH/S/NAresourcetypesis supported

Proposal2.1.2TofurthersupportFDMwithinacarrier,thefollowingisconsidered:

Frequency-domaingranularityofNPRBswhereNisconfigurable

Aseparatesemi-staticDUresourceconfigurationisintroducedforfrequency-domainresources

DCI_Format2_5isenhancedtosupportsoftresourceavailabilityindicationsforfrequency-domain resources

FeedbackForm1: ViewsonProposal2.1.2?

| Item | Company | Comments |
| --- | --- | --- |
| 1 | Qualcomm Incorporated | Thefirstbullet/itemisOK. The second bullet / item should be expanded to include the possibility of additionaloptions. Byintroducingaseparatesemi-staticDUresourceconfigurationforfrequency-domainresources,theinteractionbetweentime-domainand frequency-domain configurations need to be defined, and existing “H/S/NA” resourcetypemayneedtobereinterpreted. Asanalternativeoption,wethink that extending the Rel-16 “gNB-DU cell resource configuration” IE defined in TS38.473 so that HSNA Slot configuration List may contain more than one HSNAslotConfigurationitems,eachassociatedwithoneRBSet,canprovide full flexibility on configuration on time-frequency grid. In this alternative option, existing definition of “Hard/Soft/NA” still holds for a resource unit in time-frequencygrid. Inregardtothethirdbullet/item,theDCI25frameworkconsistsofmultiple components: DCI25 format, RRC configured availability combinations table, spec defined availability element table. The extension to frequency-domain can be achieved by different approaches. In our opinion, extending the RRC configuredtabletofrequency-domainwhilekeepingthesameDCI25formatwill have smaller signaling overhead and less spec changes than extending DCI25 with additional bits. The suggestion is to say ”DCIFormat 25 framework is enhanced...”inlieuofjustsayingweshouldenhanceDCI_Format2_5. |
| 2 | NTT DO- COMO INC. | Supporttheproposal. |

| Item | Company | Comments |
| --- | --- | --- |
| 3 | vivo Mobile Commu- nication Co., | Fine with the first two bullet, for the third bullet, we share view with QC, the DCI format 2-5 may not change, instead we can enhance the availability indicationtable. |
| 4 | CEWiT | Supporttheproposal |
| 5 | ZTE Cor- poration | Forthesecondbullet,themotivationof‘separateconfiguration’ isnotclearto us,andwethinkitistooearlytodeterminethesignalingdetailsbeforewehave consensusontherelationship b/wtime-domainHSNAandfrequencydomain configuration. Soweproposetodiscussthefollowingfirst. Option 1: For a given symbol/slot of a IAB DU cell, IAB is provided with either time-domain H/S/NA configuration or frequency domain configuration, e.g.,TDMandFDMresourcescouldbeorthogonallyconfiguredperslotbasis. Option2:Foragivensymbol/slotofaIABDUcell,IABcanbeprovidedwith bothtime-domainandfrequencydomainconfigurations,andIABcandetermine touseoneoftheconfigurationsdynamically,inthiscase,itmaycausedifferent understanding on the used IAB-DU resource configurations among IAB, its parentnodeandCU. |
| 6 | Ericsson Limited | WethinktherearebenefitswithusingRBGsinsteadofNPRBssinceUEsare scheduledinRBGsandnotPRBs. Hence,inordertohaveanefficientscheduling and use of resources, it is preferable if the H/S/NA is partitioned similar tothat. Second,configurationinPRBsmayrequireasubstantialoverhead,to littleornogain. Hence,wedonotsupportconfigurationinNPRBs. Wesupportseparatesemi-staticFDMDUresourceconfiguration. We share QC’s view that the DCI 2_5 may actually not change but enhancementsontheRRCconfiguredsetofavailabilitycombinationsmaybepreferred. |
| 7 | LG Elec- tronics Inc. | Wearefinewiththeproposalingeneral. Regarding on frequency-domain granularity, we are not sure that the configurability of N is required where N is the minimum unit of frequency resource fortheconfiguration. Wewanttodiscussfurtherandmake‘Nisconfigurable’ asFFS. |
| 8 | Huawei Technologies France | Wearefinewiththefirstbulletalthoughfurtherdiscussionsonthedetailsare needed. Forthesecondbullet,itisabitunclearwhataseparateDUresourceconfiguration really means here. Our view is that RAN1 can decide what a intended FDMDUresourcepatternisandhowitrelatestotheTDMDUresourceconfiguration. ThenthesignalingdetailscanbelefttoRAN3todecide. For the third bullet, we think it is a bit early to agree that enhancement for DCI format 2_5 is required. We can change it to ”DCI format 2_5 is supportedtoindicatetheavailabilityofsoftresourcesinfrequencydomain,FFSif enhancementsareneeded” |
| Item | Company | Comments |
| 9 | Motorola Mobility France S.A.S | 1. WearefinewitheitherNPRBsorNRBGs,whereNisconfigurable. 2. Itisnotveryclearwhata’separate’configurationmeanshere. Doesitmean separateintermsofinterpretationandbehaviorbyL1ordoesitmeanseparate intermsofthehigher-layersignalingformat? Iftheformer,thistopicneedsto bediscussedmoreextensively. Ifthelatter,itmaynotbebesttodiscussitin RAN1. Ineithercase,wedonotsupportthesecondbulletitemyet. 3. WesupporttheconceptofusingDCI2_5withmodifications/enhancements forthispurpose. |
| 10 | Intel Deutschland GmbH | We are fine with the first bullet and need more details on the second/third bullets. Forexample, wearenotsurewhetherthethirdbulletmeansjointorseparate indicationoftime/frequency-domainsoftavailability. |
| 11 | Apple France | Wearealsosupportiveofthefirstbullet. For2ndbullet,howistheinteraction ofthenewly”separate”semi-staticDUwithtimedomainDU. |
| 12 | Nokia Solutions & Networks (I) | WesupportthedirectionproposedbytheFL. 1stsub-bullet: Okwithus. RBGmakesmoresensehere. 2nd sub-bullet: weshare a similar view as QC. From signalling perspective, it iseasiertoextendtheIAB-DUHSNAconfiguration. AlistofIAB-DUHSNA configurations can be indicated with each H/S/NA configuration in the list associatedwithoneRBset. 3rdsub-bullet: similarviewasQC. |
| 13 | ETRI | WesupporttheFLproposalinprinciple. Rethe1stbullet: OK Re the 2nd bullet: We think other options need to be listed first. E.g. the H/S/NAconfigurationinthefrequencydomaincouldbeasub-IEoftheexisting H/S/NAconfigurationinthetimedomain. Re the 3rd bullet: OK with ”DCIFormat 25 framework is enhanced ...” as suggestedbyQC. |

Proposal2.1.3

IncaseofFDMwithinacarrier,thefrequency-domaingranularityisconfigurable

FFS:resourcesizee.g. NPRBs/NPRGs

FFS:SeparateorjointTDMandFDMsemi-staticDUresourceconfigurations

FeedbackForm2: ViewsonProposal2.1.3?

| Item | Company | Comments |
| --- | --- | --- |
| 1 | Apple France | SupportFL’sproposal |
| Item | Company | Comments |
| 2 | Qualcomm Incorporated | Wesupportthemainbullet. We suggest modifying the first FFS point as: ”FFS: minimum resource size, e.g. NPRBs/NPRGs”. |
| 3 | NTT DO- COMO INC. | support. |
| 4 | ZTE Cor- poration | Agreeinprinciple,someclarificationisneededforthesecondsub-bullet,arewe talking about “Separate or joint signalling for TDM and FDM semistaticDUresourceconfigurations” ? Ifyes,itseemsaRAN3topic. |
| 5 | CEWiT | Supportthemainbullet For1stFFS,suggesttomodifyas”minimumfrequencydomaingranularity” For2ndFFS,weareoktodiscussbothoptions |
| 6 | vivo Mobile Commu- nication Co., | Supporttheproposal |
| 7 | Ericsson Limited | Wesupporttheproposal,alsowithQC’smodification. Wecouldalsoagreeto furthernarrowingdownaccordingtoZTE’sOption2abovesinceanydynamic changeinmultiplexingconditionswouldrequireaTDMfallbackconfiguration. |
| 8 | LG Elec- tronics Inc. | Wearefinewiththemainbullet. In the 2nd FFS, the sentence mentions to further study on ’separate or joint TDMandFDMsemi-staticDUresourceconfiguration.’ But,itseemsthatthis sentence is not tied with discussion of granularity of frequency domain. So, it needstoelaboratetheintentiontoputthe2ndFFSatthisproposalifwanted . |
| 9 | Motorola Mobility France S.A.S | Support |

Agreement

Forthesemi-staticDUresourceconfigurationinthefrequencydomain withinacarrier,the frequency-domaingranularityisconfigurable

FFS: minimumresourcesizee.g. NPRBs/NRBGsFFS:SeparateorjointTDMandFDM semi-staticDUresourceconfigurations

Proposal2.1.4

Softresourceavailabilityindicationsforfrequency-domainresourcesaresupported

FFSenhancementstoDCIFormat2_5

FeedbackForm3: ViewsonProposal2.1.4?

| Item | Company | Comments |
| --- | --- | --- |
| 1 | Apple France | Supporttheintention |
| 2 | Qualcomm Incorporated | Wesupporttheproposal. |
| 3 | NTT DO- COMO INC. | support. |
| 4 | ZTE Cor- poration | FortheFFS,wethinkitismoregeneraltosay ‘FFS enhancements to Rel-16 soft resource availability indication framework’ |
| 5 | vivo Mobile Commu- nication Co., | supportZTE’smodification |
| 6 | Ericsson Limited | Wesupporttheproposal. |
| 7 | CEWiT | Supporttheproposal |
| 8 | LG Elec- tronics Inc. | Wesupporttheproposal |
| 9 | Motorola Mobility France S.A.S | We support the proposal, although as mentioned in the email discussion, we suggestaslightrewordingofthemainlinetooneofthefollowingalternatives: Frequency-domain availability indication for soft resources is supported or Availabilityindicationofsoftresourcesissupportedinthefrequency domain Thesealternativeswouldnotconflictwitha‘joint’configurationandindication, because they don’t emphasize that the resource is in the frequency domain (which may be interpreted as ‘separate’ configuration only), but the notion of availability indication is in the frequency domain, which is the intent of the proposalasweunderstandit. Notethat’separatevs. joint’isFFSinProposal 2.1.3’. |
| 10 | Motorola Mobility France S.A.S | [UPDATE] Theproposalasrevisedintheemaildiscussionisfine. |

Agreement Softresourceavailabilityindicationsforfrequency-domainresourcesaresupported

FFSenhancementstoDCIFormat2_5FFS:SeparateorjointTDMandFDMindications

## ISSUE2.2:SPATIALDOMAINMULTIPLEXINGRESOURCEGRANULARITY

Proposal2.2.1H/S/NAresourcetypesareextendedtothespatialdomain,includingthefollowing:

Beam-basedand/orpanel-basedgranularityissupported

Theexistingbeam-managementframeworkistakenasthestartingpointforsignalling indications/restrictionsonsupportedbeams/panelsforagivenbackhaullinkorresourcetype betweenchildandparentnode(s).

FeedbackForm4: ViewsonProposal2.2.1?

| Item | Company | Comments |
| --- | --- | --- |
| 1 | Qualcomm Incorporated | Agreewiththeproposal. |
| 2 | NTT DO- COMO INC. | Donotsupportthemainbullet. Wethinksemi-staticconfiguredH/S/NAresourcetypeisnotsuitableforspatial domain resource. Best beam used for MT/DU on its backhaul/child link is decided by parent node/IAB-DU based on dynamic channel condition. It is hardforCUtosemi-staticallyconfigurebeamssuitableforMTandDU. Agree that beam-based/panel-based granularity can be considered for spatial domainresourcemanagement. Agree that existing beam management framework can be taken as a starting pointofspatialdomainresourcemanagement. In the last bullet, clarification is needed on the intention of ”given backhaul link”,itmeans”DU’schildlink”or”MT’sparentlink”? ”resourcetype”means ”childnode’sD/U/F”or”childnode’sH/S/NA”? |

| Item | Company | Comments |
| --- | --- | --- |
| 3 | vivo Mobile Commu- nication Co., | Donotagreetothemainbullet. Tocoordinatebeams/panelsbetweenALlink andBHlink,IABimplementationcandecideasetofbeamstobeusedforBH link transmission and another set of beam used for AL transmission. For BH beam training or beam reporting, IAB assumes the trained/reported beam is from the predetermined beam set for BH link. So we just reuse legacy beam management framework to address the spatial domain resource coordination b/wDUandMT,thefurtherenhancementcanbepermultiplexingmodebeam training/reportingasagreedinRAN104section8.10.2asfollowing 104Agreement •RAN1todecidewhethertoenhanceinterferencemitigationthroughinformationexchangetosupportbeam-managementattheparentorchildnode. •FFS: reporting of desired beams for reception in DL or desired beams for transmissioninULbytheIABnodeforagivenmultiplexingmode •FFS: indicating applicable beams in DL or beams in UL for a given multiplexingmode. Moreover, for the 1st subbullet, panel based management is not supported in MIMO session in Rel-15/16, so beam based management can be baseline for furtherdiscussion. |
| 4 | CEWiT | We have similar view as Docomo regarding main bullet. Semi-static H/S/NA configurationfromCUmaynotbesuitableforspatialdomainresources. |
| 5 | ZTE Cor- poration | Disagreewiththeproposal,asdiscussedinourcontribution,itisdifficultforCU to semi-static control all the IAB-DUs resource availability on spatial-domain duetothetime-variantchannelconditions. Forthesignallingindications/restrictionsonsupported/used beams/panelsfor agivenbackhaullink, dynamicsignalingbetweenIABnodeanditschildnodes canbeconsideredforthispurpose. |
| 6 | Ericsson Limited | WesupportextendingH/S/NAtothespatialdomain. However,space domainwillhave conditional orthogonalitybetweenIAB-MTandIAB-DUlinks. Hence,therewillbesituationswhereorthogonalityappliesandothersituations where it does not apply, independent of any CU configuration. One situation that will need further study is, e.g, the consequences of defining a certain beamorpanelasNAwillresultinlostcoverageinthatdirectionwhichisnot acceptable. We propose a fallback to no-SDM for such cases, based on, e.g., beamsorinterferencelevels. Furthermore,wethinkthereisnoneedtospecifypanel-basedgranularity sincethatitdependingonimplementation. |
| 7 | LG Electronics Inc. | WesharetheviewwithNTTDOCOMO. |
| 8 | Huawei Technologies France | Wedonotsupportthemainbullet. It is a bit blur to agree on the support of extension of H/S/NA to spatial domain without knowning how it is going to supported with the subbllet. We sharesimilarviewswithothersthatpanel-basedSDMshouldbeimplementation issue. WearestillopentofurtherdiscussthewaytoenableflexibleSDM. |
| Item | Company | Comments |
| 9 | Motorola Mobility France S.A.S | We continue to hold our position from the last meeting that, unlike extension tothefrequencydomain,itisnotclearhowtheH/S/NAframeworkcan be extended to the spatial domain. The way time-frequency ”resources” are defined and managed in NR is fundamentally different from that in the spatialdomain. Wesupportsignalingindications/restrictionsforthepurposeofreserving or using panels/beams, managing interference, and so on; and indeed, that approachmayenduphavingsimilarities,inprinciple,withsomespatial-domain availabilityindication. Butthatdoesnotautomaticallytranslatetoextending theH/S/NAframework. Wedonotsupportthemainparagraph. |
| 10 | Intel Deutschland GmbH | We think more discussion are needed for space-domain. On one hand, we understand that beam selection can be dynamic due to channel conditions. On the other hand, it should be okay to semi-statically restrict some directions between MT/DU (for example, 180 degree for MT and 180 degree for DU) to createH/S/NAspaceresourceforDU. |
| 11 | Apple France | WesharesimilarviewasIntel |
| 12 | Nokia Solutions & Networks (I) | WearesupportiveoftheFLproposalonSDM. #firstsub-bullet: beam-basedgranularitymaynotbefullyfeasiblewithsemistatic signalling. Panel-based granularity should be OK as it generally deals with angular restrictions. We suggest using the wording like ”beam/angular granularity(orrestrictions)and/orpanel-basedgranularity(orrestrictions)can beconsidered.” #secondsub-bulletseemsok. |
| 13 | Samsung Electron- ics Co., Ltd | WeshareaviewwithHuaweiandIntel. Wethinkmoreclearpictureisneeded abouthowtoworkSDMbyextendingH/S/NAtospacialdomain. |
| 14 | ETRI | WethinkthemaindiscussionpointshouldbethemaximumnumberofH/S/NA configurations per DU cell considering various aspects such as beam/panel/numberofchildlinks,etc. |

Proposal2.2.3:

Tofacilitatesimultaneousoperationcasesandinterferencemanagement,therestrictionofbeams whichmaybeusedforagivenchild/parentbackhaullinktospecifictime/frequencyresourcesis supported

FFS:WhetherIAB-specificenhancementsbeyondtheexistingbeammanagementframeworkare neededtosupportthefunctionality

FFS:Impactonextendingthesemi-staticresourceconfigurationtothespatialdomain(e.g. H/S/NAresourceattributes)

FFS:Whetherpanel-basedgranularityisadditionallysupported

FeedbackForm5: ViewsonProposal2.2.3?

| Item | Company | Comments |
| --- | --- | --- |
| 1 | Qualcomm Incorporated | Weagreewiththeassumedintentoftheproposal. However,wesuggestreplacing”foragivenchild/parentbackhaullink”,with”foragivenlink”,unlessthere is the intention to explicitly exclude access links, and if so, further discussion isneeded. |
| 2 | NTT DO- COMO INC. | Donotsupport. Inourunderstanding, restrictionofbeamsusedforMT/DUonparent/child linkmayormaynotbeneeded. Whethertherestrictionisneededisrelated totheFFSsinthesub-bullets,aboutwhichsolutionissupported. Forexample, based on enhancement of MT beam reporting, desired beam for simultaneous operationcanbereported,andwiththeenhancedbeamreport,itneedfurther discussionwhetherthereisrestrictionorthebeammanagementcanbestillup toimplementation. Thus,wesuggestfollowingrevision. To facilitate simultaneous operation cases and interference management FFS:whethertherestrictionofbeamswhichmaybeusedforagiven child/parent backhaul link to specific time/frequency resources is needed |
| 3 | ZTE Cor- poration | Supporttheproposalingeneral. And for the second FFS, the text sounds that spatial domain resource configuration has been supported, but it is not the case, we propose to change the secondFFSas: FFS: Whether to extend the semi-static resource configuration to thespatialdomain(e.g. H/S/NAresourceattributes) |
| 4 | vivo Mobile Communication Co., | Some concern to the main bullet, the beam usage between IAB DU and IAB MT can be somehow restricted by each other. In this case we do not need to mentionaspecificIABDUlink,i.e.,agivenchildbackhaullink. |
| 5 | Ericsson Limited | Wesupporttheproposal. WeagreewithDOCOMOthatsuchrestrictions maynotalwaysbeneeded. However,ourviewisthat no restrictionisincluded asaspecialcaseof specific time/frequency resources. |
| 6 | CEWiT | Supporttheproposal |
| 7 | LG Electronics Inc. | We’d like to understand the necessity of the restriction of beams, and how it workswithbeammanagementframework. |
| Item | Company | Comments |
| 8 | Motorola Mobility France S.A.S | Weagreewiththeintentoftheproposalandsuggestafewchangestoaddress remainingissues: - The indication may not necessarily be in the form of a ’restriction’. Instead,itmaybeanindicationofavailabilityofabeam/panel,orindicating thatabeam/panelistobeused,whichmayormaynotbeequivalenttoa beam/panelrestriction. Sincethisisstillahigh-levelproposalwithseveral FFS points, we suggest to change restriction to ’restriction/usage/availability’. Thisshouldaddresssomeoftheissuesraisedhere. - WepartiallyagreewithQualcomm’scomment,butsincewehavenoagreement on ’per-link’ basis, we suggest to remove the phrase altogether, at leastuntil’per-link’issolidifiedinotherdiscussions. - AsbrieflymentionedinthepreviousGTWcall,’extending’theH/S/NA frameworkisoneexample,soit’sbettertopushtherelatedphrasesinside thebrackets. Hereistheproposalbasedontheabove: To facilitate simultaneous operations and interference management, dynamicindicationforrestriction/usage/availabilityofbeamstospecifictime-frequencyresourcesissupported FFS:WhetherIAB-specificenhancementsbeyondtheexistingbeam managementframeworkareneededtothesupportthefunctionality FFS:Impactonthesemi-staticresourceconfigurations(e.g.,extendingtheH/S/NAresourceattributestothespatialdomain) FFS:Whetherpanel-basedgranularityisadditionallysupported |

Proposal2.2.3’:

Tofacilitatesimultaneousoperationsandinterferencemanagement,dynamicindication forrestriction/usage/availabilityofbeams

FFS:Applicabilitytospecificmultiplexingcasesorspecifictime-frequencyresources

FFS:WhetherIAB-specificenhancementsbeyondtheexistingbeammanagement frameworkareneededtothesupportthefunctionality

FFS:Impactonthesemi-staticresourceconfigurations(e.g.,extendingtheH/S/NA resourceattributestothespatialdomain)

FFS:Whetherpanel-basedgranularityisadditionallysupported FeedbackForm6:Viewson2.2.3’(hopefullyeditorial only-thereisnottimetoreturntothistopicinthe nextGTW)

| Item | Company | Comments |
| --- | --- | --- |
| 1 | NTT DO- COMO INC. | Wehaveaquestiontoclarifyourunderstanding, In the mainbullet, ”dynamic indication for restriction/usage/availability of beams”, by saying ”beam” here, the intention is to indicate for ”MT beam”, or”DUbeam”orisopenfornow? |
| 2 | Ericsson Limited | Wesupporttheproposalwithaslighteditorialmodification(underlined): To facilitate simultaneous operations and interference management, dynamicindicationforrestriction/usage/availabilityofbeamsissupported. |
| 3 | Nokia Solutions & Networks (I) | Supportthecurrentwording. Looksok. |
| 4 | ZTE Cor- poration | Wearefinewiththeproposal,andEricsson’smodificationisalsoOKforus. |
| 5 | Intel Deutschland GmbH | Wearenotsureabout”dynamicindication”inthemainproposalandactually moresupportiveoftheoriginalProposal2.2.3. |
| 6 | AT&T GNS Belgium SPRL | @DOCOMO - Given the starting point is the beam management framework, it at least includes the IAB-MT, however given this indication/restriction is duetoIABnodemaybeduetomultiplexingconstraintsinsomecasesitseems reasonabletoatleastconsiderIAB-DUbeamsaswell,butthisproposalisquite general. @INTEL-Givenmajorityofcompaniesprefertostartfromthebeammanagement framework and semi-static restrictions are more controversial, this was selectedasthestartingpoint. Ofcoursesemi-staticorothertypesofindications beyondbeammanagementcanbefurtherconsideredandarenotprecludedat thisstage. |
| 7 | vivo Mobile Communication Co., | We support the FL proposal, the main bullet is supported as commented by Ericsson. |
| 8 | Motorola Mobility France S.A.S | WesupporttheproposalaswellasEricsson’ssuggestedrevision. |
| 9 | NTT DO- COMO INC. | @AT&T-Thanksalotfortheexplanation. Wesupporttheproposal. |
| 10 | CEWiT | Supporttheproposal. Also,okwiththemodificationsuggestedbyEricsson |
| Item | Company | Comments |
| 11 | Samsung Electron- ics Co., Ltd | OKwiththeproposal. |
| 12 | Huawei Technologies France | Justtoconfirmourunderstanding,theproposalintendtocoverindicationboth upstreamanddownstreambetweentheIABnodeandtheitsparentnode,right? |
| 13 | Intel Deutschland GmbH | @AT&TThanksfortheresponse. Wecansupportthisproposal. |

## ISSUE2.3:MULTIPLEXINGCAPABILITYINDICATIONENHANCEMENTS

Proposal2.3.1. Dynamicindicationofachildnode’smultiplexing-capabilitytoaparentnodeis supportedincludingthefollowing:

Indicationsbasedonresourcetype

Indicationsbasedsetsoftime/frequencyresources

FFS:Linkagebetweenindicatedmultiplexing-capabilitiesandnumberofrequiredguardsymbols, supportedtimingmodes,andpowercontrolenhancements(ifsupported)

FeedbackForm7: ViewsonProposal2.3.1?

| Item | Company | Comments |
| --- | --- | --- |
| 1 | Qualcomm Incorporated | Wesupportthemainproposal. The first sub-bullet requires some clarification: does ”resource type” refer to “Hard/Soft/NA”type? Orto“DL/UL/Flexible”type? Itwouldbehelpfulto provideaclarificationandsomeexplanation/motivationfortheenvisioneduse cases. |
| 2 | NTT DO- COMO INC. | Donotsupport. Forthemainbullet,wethinkclarificationisneededthatthedynamicindication ”toparentnode”is”fromchildnodetoparentnode”, or”fromdonornodeto parentnode”? For the first sub-bullet. We do not see motivation/use case to have different multiplexingcapabitliyfordifferentresourcetype. Forthesecondsub-bullet,wethinkclarificationisneededonthegranularityof time-frequencyresources. Intimedomain,wethinkthemultiplexingcapability ofchildnodeshouldbeapplicableatleastforaperiodoftime. Wedonotsee motivation/usecasetohavedifferentmltiplexingcapbilityfromslottoslot. |

| Item | Company | Comments |
| --- | --- | --- |
| 3 | vivo Mobile Commu- nication Co., | Agreetothemainbullet. Inthefirst2subbullets,theassociatedtime/frequencyresourceisdiscussed,we think that the indication should base on resource where a given multiplexing modeisfeasible. For the FFS bullets, clarification is needed, does the information is used for parentnodetodeterminewhetherthereportedmultiplexingcapabilitycanbe finallyapplied? |
| 4 | Samsung Electron- ics Co., Ltd | [Samsung]Wearesupportiveofthemainbullet. Forthethirdbullet,wethink time/frequency resource can be associated with the indication. But, for the secondbullet,webelievetheresourcetypecanbetakenintoaccounttogenerate theindication,butwearenotsurewhyresourcetypebasedindicationisneeded. |
| 5 | CEWiT | More clarification is needed. Does the main bullet means child node deciding byitsowntoworkinamultiplexingmodeandsignallingittoparent? In our view, the decision about mode of operation of child node should come fromCU/parent Regarding sub-bullets, the mux capability and mode of operation should not changefrequently,e.g.,acrossresourcetypes. |
| 6 | ZTE Cor- poration | The main bullet is also not clear to us, does it mean a IAB node can indicate itsmultiplexingcapabilitydynamicallytoitsparentoraIABnodecanindicate itschildnodes’ multiplexingcapabilitydynamicallytoitsparent? Fromourpointview, therecanbetwoapproachesfordynamicsignallingb/w IABanditsparent. Approach1: dynamicindicationofassistantinfofromaIABnodetoitsparent node, and the parent node determines which multiplexing case is applicable basedonreceivedindication. Approach 2: dynamic indication of assistant info(beam related information) fromaIABnodetoitschildnode,andthechildnodedetermineswhich multiplexingcaseisapplicablebasedonreceivedindication. Approach 2 is simpler and has less normative work, and indication similar as DCI 2-0 and 2-5 can be a starting point for approach 2, so instead of this proposal,approach2-likesolutionisourpreference. |
| 7 | Ericsson Limited | WesupporttheproposalprovidedresourcetypemeansDL/UL/F. |
| 8 | LG Electronics Inc. | Wearenotsupportivefortheproposal. At the first step, we think the applicability of simultaneous operation can be determined by TDD configuration for MT and DU. If the Tx/Rx direction of the MT and DU by TDD configuration in a time resource is capable of simultaneousoperation,theIAB-nodecanoperatesMTandDUsimultaneously intheresource. Otherwise,itshouldoperateMTandDUinTDM-manner. Additionally, dynamic features may impact to the temporal applicability of simultaneousoperation,suchastransmissionpower,timingmodes,guardsymbols, etc. However, we need more discussion on whether and how to indicate dynamicapplicabilityofmultiplexing-capabilityexplicitly. |
| Item | Company | Comments |
| 9 | Huawei Technologies France | Partially. Wearefinewiththemainbullet. For the first bullet, we share similar view with QC that some clarification is neededonwhatresourcetypemeans. If the intention is to provide information on whether a given multiplexing capability should be restricted in DL or UL symbols. Our view is that same to an access UE, the IAB-MT will transmit in UL or flexible symbols as long as it is scheduled or configured to do so and similar for the DL reception. There isnoobviousneedfortheIABnodetoreportorindicatewhetheragivenmultiplexcapabilityisonlyapplicabletoDLorULsymbolssincethemultiplexing capability is already coupled with UL or DL, e.g. the multiplexing cases with MT-TxcanonlybedoneinULorflexiblesymbolswhichhasbeenindicatedas UL. Similarly,clarificationsareneededforthesecondbullet. whyweneedsuchan indication? Moreover,wethinkthethirdbulletisnecessaryforthedynamicindication. in somescenarios,multiplexingcapabilityisapplicableonlyifsomeconditionsare met. Withoutthelinkagebetweenthemultiplexingcapabilityandconditions, theparentnodemaynotbeabletofigureouttheproperwaytoschedulingthe childIAB-MT. |
| 10 | Motorola Mobility France S.A.S | Wearefinewiththeconcept, butthisproposalseemstobeprematureatthis pointbefore further progressin 8.10.2. The argumentsfordynamicindication of capability/applicability are related to power constraints, interference constraints, and so on. Therefore, it seems premature to agree on the signaling and content without a sufficient discussion on the conditions that may trigger thesignalingordetermineitscontent. Weproposetogetbacktothisdiscussion uponfurtherprogressin8.10.2. |
| 11 | Intel Deutschland GmbH | Wethinkfurtherclarificationisneeded. For the main proposal, we need to clarify whether this indication is regarding ”multiplexing-capability”or”activemodeofoperationatagiventime”. |
| 12 | Apple France | We like to have more discussion on capability indication, like why dynamic indication is required rather than semi-static indication, what exactly defines multiplexingcapabilityfordifferentduplexingmodes,etc. |
| 13 | Nokia Solutions & Networks (I) | Wefeelthatthisproposalisrelatedtothefirsttwoproposals. Dynamicindication of multiplexing mode capability will depend on the H/S/NA resources of the IAB DU for a given multiplexing mode. If the semi-static resource is allocatedforFDMorSDMmodes,wethinkthatthiskindofindicationalsoneeds to discuss switching of TDM/FDM configurations etc. We suggest discussing thisafterwemakesomeprogresswiththefirsttwoproposals. |
| 14 | ETRI | Wewouldliketounderstandtheexactmeaningofdynamicindication. Doesit mean that we will introduce a new PUCCH format for this, for example? Or doesitjustmeanthattherewillbeanewIEinF1AP? |

Agreement AdaptationofanIAB-node’smultiplexingoperationissupported. Theadaptationmaybebasedon multiplefactors,forexample(notnecessarytosupportallofthefollowing):

Resourcetype(D/U/F)attheIAB-DUandIAB-MT

Specificsetsoftime/frequencyresources

Certainconditionsbeingmet(e.g. supportedtimingmodes,powercontrolenhancements(if supported),etc.)

FFS: Mechanismsforinforming/coordinationthechangeinmultiplexingoperation(s)betweenchild andparentnodes(includingwhethertheadaptationisdynamicorsemi-static)

FFS:Needforexplicitlinkagebetweenindicatedmultiplexingoperationsandother features/enhancements–e.g. numberofrequiredguardsymbols,supportedtimingmodes,and powercontrolenhancements(ifsupported)

3	Resource allocation for dual-connectivity scenarios (i.e. IAB-MT with concurrent BH links with two parentnodes)

FromtheeIABWID:

SpecificationofenhancementstotheresourcemultiplexingbetweenchildandparentlinksofanIAB node,including:

Supportofsimultaneousoperation(transmissionand/orreception)ofIAB-node’schildandparent links(i.e.,MTTx/DUTx,MTTx/DURx,MTRx/DUTx,MTRx/DURx)

Supportfordual-connectivityscenariosdefinedbyRAN2/RAN3inthecontextof topologyredundancyforimprovedrobustnessandloadbalancing.

## ISSUE3.1:INTRA-BAND,INTER-CARRIERDCSUPPORT

Proposal3.1.1: Enhancementstosupportintra-bandinter-carrierdualconnectivity(inter-donor andintra-donor)scenariosareintroducedinRel-17

FFS:ExtendRel-16CATDDconflictionresolutionframeworkforatleastsynchronousintra-band NR-DCoperation

FFS:TDDconfigurations(andusageofDCIFormat2-0ifsupported)fortheparentnodesand

H/S/NAconfigurationsforthechildnodearecoordinatedincaseofinter-donoroperation

FFS:SupportforsimultaneousTx/RxatthechildIAB-MT

FeedbackForm8: ViewsonProposal3.1.1?

| Item | Company | Comments |
| --- | --- | --- |
| 1 | OPPO | [OPPO] For the 2nd FFS, would the ”coordination” allow the parent node temporarilysteppingawayfromtDDConfigCommontogiveawaytoH/S/NA for the child? So far there is no restriction on ”coordination”. We start to haveabitconcernontheimpactstonormalUEservedbytheparentnodeand sharingthesameTDDconfiguration. For the 3rd FFS, does this deal with the child MT that has simultaneous UL Tx with one parent and DL Rx with another parent? If yes, what additional coordination is needed in 2nd FFS and what collision should be concerned in 1stFFS? |
| 2 | Qualcomm Incorporated | Wesupportthemainproposal. ThesecondandthirdFFSpointscouldbenefitfromsomeclarifications: Forthesecondsub-bullet,itwouldbehelpfultoaddthefollowinginformation “Rel16IEintendedTDDDL-ULconfigurationdefinedoverF1andXninterface can be used for TDD coordination between parent nodes”, so that existing signalingcanbeusedforcoordination. For the third sub-bullet, it is not 100% clear what “simultaneous TX/RX at the child IAB-MT” refers to. Does it refer to “capability of simultaneous TX tooneparentandRXfromanotherparentattheIAB-MT”ordoesitreferto “capabilityofsimultaneousTXtobothparentsandcapabilityofsimultaneous RXfrombothparentsattheIAB-MT”? |
| 3 | NTT DO- COMO INC. | Supportinprinciple. For the third sub-bullet, we share similar view with OPPO and Qualcomm to clarify”simultaneousTx/RxatchildIAB-MT”. |
| 4 | vivo Mobile Communication Co., | Supportthefirstbullet. For the 2nd bullet, we have agreed to exchange TDD and H/S/NA between donorsforthecoordination. ifmyunderstandingiscorrect,theintentionof2nd bulletistoallowtheparentnodestoknowtheTDDandH/S/NAmutually. if my understanding is correct, the wording needs refinement and this is applied forbothinter-donorandintra-donoroperation. Forthe3rdbullet,couldyouclarifythespec. impactifsupported. |
| 5 | Samsung Electron- ics Co., Ltd | [Samsung] In general, we think it is a little bit premature to discuss intraband inter-carrier DC although we are open to discuss it. Rather, we prefer having a discussion about inter-band inter-carrier DC before discussing the intra-bandinter-carrierDCbecauseitwasagreedthatintra-bandinter-carrier DCissupportedatleasttotheextenditreusessolutionsforsupportinginterbandinter-carrierDC.Ifsolutionsforinter-bandinter-carrierDCareidentified, itcanbeclarifiedwhetherthesolutionscanbereusedforintra-bandinter-carrier DCandthenwhatotherenhancementsareneededforintra-bandinter-carrier DC. |

| Item | Company | Comments |
| --- | --- | --- |
| 6 | CEWiT | Supportthemainproposal. Regarding2ndFFS,coordinationisneededininter-donorandintra-donorscenarios Regarding3rdFFS,moreclarificationisneeded. DoesthatmeanchildIAB-MT Tx/RxlikeafullduplexUE? IfyesthentherewillnotbeanyTDDconflictas mentionedinfirsttwoFFSbullets. |
| 7 | Fujitsu Limited | Wesupporttheproposal. |
| 8 | ZTE Cor- poration | Thesecondandthethirdbulletsarenotfullyclear. Forthesecondbullet, isthe‘TDDconfiguration’ aMTconfigurationorDU configuration? Forthethirdbullet,doesitmeanhalf-duplexconstraintisnotrequiredatthe childMT? |
| 9 | Ericsson Limited | We support this proposal. We share prior companies’ views to clarify ”simultaneousTx/RxatchildIAB-MT”. |
| 10 | LG Elec- tronics Inc. | OKwiththeproposalingeneral. For the second FFS point, MT TDD configuration and DU configuration (includingD/F/UandH/S/NAconfiguration)canbecoordinatedincaseofinterdonor operation, since it is determined and configured bydonor. However, we don’tthinktheusageofDCIformat2-0canbecoordinatedbetweentwoparent nodes, since it is dynamically determined and indicated by each parent node. So, wewantto remove‘(and usage ofDCI Format2-0 if supported)’from the proposal. |
| 11 | Huawei Technologies France | Partly. Wesupportthemainbullet. Thesub-bulletsneedmoreclarifications. We share similar view with OPPO that the third bullet should be discussed firstthentheothertwosubbullets. Forthefirstbullet,wearenotsurewhetherthecurrentframeworkforCAcan beappliedforthecasewithschedulingcollisionsconsideringthattherewillbe independentschedulersfromtwoparentnodes. |
| 12 | Motorola Mobility France S.A.S | Wesupportthemainproposal. |
| 13 | Intel Deutschland GmbH | Wesupportthisproposal. |
| 14 | Apple France | WesupporttheintentionandsharesameviewasQConthesecondsub-bullet |
| 15 | Nokia Solutions & Networks (I) | Weagreewiththemainbulletand2nd/3rdsub-bullet. #1stsub-bullet: notfullsurewhatismeanttoincludebythe”ExtendRel-16 CA TDD confliction resolution framework”. Is this related to the alignment of TDD configurations between parents or handling conflicts that IAB MT experiences(mainlyduetohalfduplexing)? |
| Item | Company | Comments |
| 16 | ETRI | Wesupporttheproposal. |

Proposal3.1.2:

Thefollowingenhancementstosupportintra-bandinter-carrierdualconnectivityforboth inter-donorandintra-donorscenariosareconsidered(inadditiontoreusingsolutionsforinter-band dualconnectivity):

-ExtendingtheRel-16CATDDconflictionresolutionframeworkforatleastsynchronous intra-bandNR-DCoperation

-ExchangingTDDconfigurationsfortheparentnodes(intra-donorandinter-donor)and exchanging/coordinatingH/S/NAconfigurationsforthechildnode(atleastforinter-donor operation)

-FFS:NeedtosupporteithersimultaneousTxorsimultaneousRxatthechildIAB-MTto/from bothparentlinks

FeedbackForm9: ViewsonProposal3.1.2?

| Item | Company | Comments |
| --- | --- | --- |
| 1 | Qualcomm Incorporated | Weagreeinprinciplewiththeintentionoftheproposal. Howeversomeclarificationsareneeded. Forexample,itseemsthelastsub-byullet,whichisFFS,is arequisiteforCATDDoperationandtheneedforconflictionresolutionrules, forwhichthefirstsub-bulleteventalksaboutfurtherenhancements. Moreover, the second sub-bullet should be split into two, as exchanging TDD configurationsfortheparentnodes,andexchanging/coordinatingH/S/NAfor thechildnodeservedifferentpurposes. |
| 2 | NTT DO- COMO INC. | support. |
| 3 | ZTE Cor- poration | For the FFS, we think an IAB MT already supports to simultaneous Tx or simultaneousRxto/frombothparentlinksforinter-carrierDCscenario. |
| 4 | CEWiT | Supporttheproposalingeneral. MoreclarificationisneededfortheFFS.Inouropinion,discussionaboutTDD conflictandresolutionoptionsisbasedontheassumptionthatIAB-MTsupport simultaneousTx/simultaneousRxwithparentnodes. |
| 5 | vivo Mobile Communication Co., | For the 2nd subbullet, does it means the folloiwng, we are confused to the currentwording. - Parent nodes exchanges TDD configurations for the parent nodes (intra-donorandinter-donor) -IABdonorsofparentdonorsexchangesH/S/NAconfigurationsfor thechildnode(atleastforinter-donoroperation) |
| Item | Company | Comments |
| 6 | Ericsson Limited | Wesupporttheproposal. |
| 7 | LG Elec- tronics Inc. | Forthefirstsub-bullet,InIABdeploymentscenario,DLtransmissiontimebetweendifferentDUswithinacarrierisalignedbyusingtimingmode1. Also,for TDDoperation,DLtimingbetweendifferentcarrierswithinabandshouldbe aligned. Inourunderstanding,onlysynchronousintra-bandNR-DCoperation is required for IAB operation. We’d like to ask the intention to put ‘at least’ in the sentence. Do we need to further discuss in asynchronous intra-band NR-DCoperationforIAB? Forthesecondsub-bullet,wethinkTDDconfigurationformultipleparentnodes andH/S/NAconfigurationforchildnode,whicharedeterminedandconfigured bydonor,canbecoordinatedregardlessofintra/inter-donorscenario. InRAN1 perspective, it can be assumed that TDD configuration for multiple parent nodesandH/S/NAconfigurationsforchildnodearealignedinintra-bandDC. Forthelastsub-bullet,evenforsynchronousintra-bandNR-DCoperation,supportofsimultaneousTx/simultaneousRxcannotbeguaranteed. Forexample, in case of UL asynchronous operation due to Case-7 timing mode of parent node(s),thesupportofsimultaneousTxshouldbefurtherdiscussed. |
| 8 | Motorola Mobility France S.A.S | Support |
| 9 | NTT DO- COMO INC. | Support. |
| 10 | Nokia Solutions & Networks (I) | WearefinewiththeQCsuggestions. |
| 11 | Intel Deutschland GmbH | Wesupporttheproposal. |

Proposal3.1.3:

Thefollowingenhancementstosupportintra-bandinter-carrierdualconnectivityfor bothinter-donorandintra-donorscenariosareconsidered(inadditiontoreusing solutionsforinter-banddualconnectivity)tosupportsimultaneousTxorsimultaneous RxatthechildIAB-MTto/frombothparentlinks:

-ExtendingtheRel-16CATDDconflictionresolutionframeworkforsynchronous intra-bandNR-DCoperation

-ExchangingTDDconfigurationsfortheparentnodes(intra-donorandinter-donor) andexchanging/coordinatingH/S/NAconfigurationsforthechildnode(atleastfor inter-donoroperation)

FeedbackForm10: ViewsonProposal3.1.3?

| Item | Company | Comments |
| --- | --- | --- |
| 1 | Ericsson Limited | Wesupporttheproposal. |
| 2 | Nokia Solutions & Networks (I) | ThistheupdatedversionoftheProposal3.1.2. Maybetheaboveversion3.1.2 couldbelocked. Anyways,wearefinewiththis. |
| 3 | Intel Deutschland GmbH | Wesupporttheproposal. |
| 4 | vivo Mobile Commu- nication Co., | I copy our concern in previous round, clarify in GTW or in email is beneficial forusetosupporttheproposal For the 2nd subbullet, does it means the folloiwng, we are confused to the currentwording. - Parent nodes exchanges TDD configurations for the parent nodes (intra-donorandinter-donor) -IABdonorsofparentdonorsexchangesH/S/NAconfigurationsfor thechildnode(atleastforinter-donoroperation) |
| 5 | Motorola Mobility France S.A.S | Support |
| 6 | Huawei Technologies France | We would like to have one clarification, the main bullet talks about the enhancementtosupportsimultaneousTxorsimultaneousRxtoorfrom two parent nodes, NOT about Tx from parent node and Rx from another parentnodeatthesametime,right? Iftheaboveunderstandingiscorrect,we arenotsurewhetherthereisaneedforthefirstsubbulet,sincethecurrentrule defined for CA with half duplex constraint is to resolve collisions between Tx inonecellandRxinanothercell. |

## ISSUE3.2:MULTI-PARENTSOFTRESOURCEAVAILABILITYINDICATION

Proposal3.2.1: ExplicitconflictresolutionrulesincaseofreceptionofDCIFormat2_5from multipleparents/cellsaresupportedinRel-17.

FFS:Enhancementstosupportindicationofsoftresourceavailabilityfromchildnodetoparent node(s)

FeedbackForm11: ViewsonProposal3.2.1?

| Item | Company | Comments |
| --- | --- | --- |
| 1 | Qualcomm Incorporated | Wesupportthemainbullet. Some clarification is needed for FFS item. It is not clear on the intention of “indicationofsoftresourceavailabilityfromthechildnodetoparentnodes(s)”. Isitintendedtohelpresolvingconflictingavailabilityindicationsreceivedfrom multiple parent nodes, or intended for a child node to request available soft resources from parent node? For latter case, it may also be applicable for single-parentcase. |
| 2 | NTT DO- COMO INC. | Supportthemainbullet. For the FFS, we think report from child node to parent node is not necessary and the benefit is not clear to us. It may have benefit only if child node can reportbeforetheparentnodedecideandindicatesoftresourceavailability. And itcausessignalingoverhead. |
| 3 | vivo Mobile Commu- nication Co., | Supporttheproposal |
| 4 | Samsung Electron- ics Co., Ltd | [Samsung] OK with the main bullet. But, share with other companies’ view thatsomeclarificationsfortheFFSareneeded. |
| 5 | CEWiT | Supportthemainproposal. FortheFFS,wesharesimilarviewwithDOCOMO |
| 6 | Fujitsu Limited | We support the main bullet. Some clarifications are needed for the FFS. Is there any specific relation between this FFS and the second FFS of proposal 3.1.1? |
| 7 | ZTE Cor- poration | Do not support the proposal, as pointed in our contribution R1-2103497, the benefitandmotivationto indicatetheresourceavailabilityforsoftsymbol(s) ofthesameDUcellfromMCGandSCGisnotclear,and itwillleadtoamore complex specification procedure especially when DCI 2-5 may be extended to frequencyandspatialdomain. Soweproposethatjustoneresourceavailability indicationeitherfromMCGorfromSCGisexpectedforaIAB-DUsoftsymbol ofaIABDUcell. |
| 8 | Ericsson Limited | We support the proposal. We are not convinced about the FFS, but are willingtoacceptitforthesakeofprogress. |
| 9 | LG Electronics Inc. | Wesupportthemainbullet. FurtherdiscussionseemsnecessaryfortheFFSpoint. |
| 10 | Huawei Technologies France | We are fine with the main bullet. Similar as others, we are not convinced on theneedoftheFFSbullet. |
| Item | Company | Comments |
| 11 | Motorola Mobility France S.A.S | WesupportthemainproposalandtheFFSitem. |
| 12 | Intel Deutschland GmbH | Wesupportthemainproposal. Explicitlytworulescanbelisted: - Samecarrier(s)indicationispreferredforinter-carrierDC. - Sameindicationfrombothparentsispreferredforintra-carrierDC(ifsupported). Wedon’tthinktheFFSisneeded,sincedynamicresourcecoordinationbetween parentnodesisnotpreferredfordynamicsoftavailabilityindication. |
| 13 | Apple France | Supporttheproposal |
| 14 | Nokia Solutions & Networks (I) | Supporttheproposal. |
| 15 | ETRI | WesharesimilarviewwithZTE. WethinkthatRel-17multi-parentoperatesmainlywiththeinter-bandDCscenario. Furthermore,thereisanon-goingdiscussioninissue3.1forthepotential conflictionsinintra-bandDCscenario. Ifthisisthecorrectunderstanding,the motivationtohavefurtherenhancementsontopof Issues2.1, 2.2, 2.3and3.1 isnotthatbig(ornotclearyet). Therefore,weprefertofocusonotherissuesfirst. |

Proposal3.2.2:

ExplicitconflictresolutionrulesincaseofreceptionofDCIFormat2_5frommultipleparents/cells aresupportedinRel-17.

-FFS:Enhancementstosupportcoordinationbetweenparentnodes(e.g. indicationofsoftresource availabilityfromchildnodetoparentnode(s))

FeedbackForm12: ViewsonProposal3.2.2?

| Item | Company | Comments |
| --- | --- | --- |
| 1 | Apple France | Supporttheproposal |
| 2 | NTT DO- COMO INC. | Supporttheproposal. Although we do not see the necessity of indication from chlid node to parent node,wecanacceptasFFS. |
| Item | Company | Comments |
| 3 | Qualcomm Incorporated | We support the intent of the proposal, but we think it should be modified. Fundamentally, there are no conflicts and hence no conflict resolution rules requiredincaseofmultiplereceptionsofDCIFormat25frommultipleparents, sinceviaDCIFormat25onecanonlyindicateavailabilityandcannotindicate unavailability. Henceamoreaccuratedescriptioncouldbe: ”RulesonexplicitavailabilityindicationincaseofreceptionofDCIFormat2_5 frommultipleparents/cellsaresupportedinRel-17”. |
| 4 | ZTE Cor- poration | From our point view, the precondition of the main bullet(i.e., in case of reception of DCI Format2_ 5 from multiple parents/cell)has notbeenagreed,wecannotsupportthisproposal. |
| 5 | CEWiT | Supporttheproposal. RegardingFFS,weareopentodiscussenhancementsforcoordinationbetween parentnodes. But,sharesimilarviewwithDOCOMOregardingchildreporting IA |
| 6 | vivo Mobile Commu- nication Co., | support |
| 7 | Ericsson Limited | Wesupporttheproposal. However,wedon’tthinkasolutiontothisshould be agreed before we have agreed on how the DU of a multi-connected MT is configurablewithH/S/NA. |
| 8 | LG Elec- tronics Inc. | Inourunderstanding,theintentionofproposal3.2.2’istoclarifythedeterminationofthesoftresourceavailabilitywhentheexplicitavailabilityindication isindicatedfromonlyoneparentnodewhileDCIformat2_5foraDU-cellcan betransmittedfrommultipleparentnodes. Ifourunderstandingiscorrect,We areokforthemainbullet. Regarding on the FFS in sub-bullet, it needs to clarify the intention. We’d liketoknowthepurposeandoftheenhancementandtherelationshipwiththe mainbullet. |
| 9 | Motorola Mobility France S.A.S | Support |

Proposal3.2.2’:

RulesonexplicitavailabilityindicationincaseofreceptionofDCIFormat2_5from multipleparents/cellsaresupportedinRel-17

-FFS:Needforenhancementstosupportcoordinationbetweenparentnodes(e.g.

indicationofsoftresourceavailabilityfromchildnodetoparentnode(s)) FeedbackForm13: ViewsonProposal3.2.2’?

| Item | Company | Comments |
| --- | --- | --- |
| 1 | NTT DO- COMO INC. | support. |
| 2 | Nokia Solutions & Networks (I) | Support |
| 3 | Ericsson Limited | Wesupporttheproposal. |
| 4 | ZTE Cor- poration | Wearenotconvincedtosupporttheproposal. Firstly,howcanwejustagreeontheseruleswithoutkonwingwhattheserules are,futherdiscussionandclarificationisneeded. Secondly,aswepointedbefore,indicatingDCI2-5foraIABDUcellfromboth MCG and SCG has not been agreed, and the benefit to support this has not beenjustified. |
| 5 | vivo Mobile Commu- nication Co., | support |
| 6 | Motorola Mobility France S.A.S | Support |
| 7 | CEWiT | Supporttheproposal |
| 8 | Huawei Technologies France | Fine with the main bullet. We don’t support the subullet as it is with the example. It has not been well explained what the intention is and how it relatedtothemainbullet. Wewouldbefinewithremovingtheexample. |
| 9 | Intel Deutschland GmbH | Support |

## ISSUE3.3:RESOURCECOORDINATION

Proposal3.3.1: Per-backhaullink(e.g. perchildIAB-MTlink)resourceconfigurationsaresupported inadditiontoper-DUresourceconfigurations

FeedbackForm14: ViewsonProposal3.3.1?

| Item | Company | Comments |
| --- | --- | --- |
| 1 | Qualcomm Incorporated | Wesupporttheproposal. |
| 2 | CEWiT | Wesupporttheproposal |
| 3 | Samsung Electron- ics Co., Ltd | [Samsung]Wesupporttheproposal |
| 4 | Fujitsu Limited | Wesupporttheproposal. |
| 5 | ZTE Cor- poration | We do not see the necessity to support this, IAB DU can flexibility schedule different child links by its implementation, a common per-DU configuration is sufficient. On the other hand, it’s foreseeable that a lot of normative works couldbeincurredifper-childlinkconfigurationweresupported,e.g.,theinteractionofper-DUconfigurationandper-linkconfigurations. |
| 6 | Ericsson Limited | Wesupporttheproposalbutwouldlikeaclarificationthattheaboveapplies toaparentnodeinaDCscenarioandnotingeneral. |
| 7 | Huawei Technologies France | WewouldliketounderstandthemotivationtosupportperchildIAB-MTlink resource configurations in addition to per-DU resource configurations. On the other hand, we see some benefit to support per parent backhaul link IAB-DU resource configuration, i.e. it is not from the parent point of view but rather fromtheIAB-DUpointofview. |
| 8 | Intel Deutschland GmbH | Wesupporttheproposal. |
| 9 | Motorola Mobility France S.A.S | ItisnotclearhowthisproposalisonlyrelatedtoDC.Pleaseclarifyandstate thepurposeoftheproposalinthewording. |
| 10 | Apple France | Supporttheintention |
| 11 | Nokia Solutions & Networks (I) | Supporttheproposal. |
| 12 | ETRI | Moreclarificationisneeded. DoesitmeanthatmultipleH/S/NAresourcetype configurationsperDUcellwillbesupported? OrDoesitmeanthattherecould bemultiple IAB-DU-Resource-Configuration perDUcell? |

Proposal3.3.2: Incaseofdualconnectivityoperation,per-backhaullink(e.g. perchildIAB-MT link)resourceconfigurationsattheparentnodesaresupportedinadditiontoper-DUresource configurations

FeedbackForm15: ViewsonProposal3.3.2?

| Item | Company | Comments |
| --- | --- | --- |
| 1 | Qualcomm Incorporated | Wesupporttheproposal. |
| 2 | NTT DO- COMO INC. | wesuggestmoreclaficiationonthemotivation/benefit. |
| 3 | ZTE Cor- poration | Firstly,themotivation/benefittosupportthisneedstobeclarify,especiallyfor inter-carrierDCscenario. Secondly, we don’t see the urgency to discuss this, it is better to get a clear pictureofIABDUcellbasisconfigurationsbeforewegofurthertotheper-link basisconfiguration,e.g.,whetherthefrequency/spatialdomainconfigurationis needed, the relationships among time/frequency/spatial domain DU resource configurations. |
| 4 | CEWiT | Supporttheproposal |
| 5 | Ericsson Limited | Wesupporttheproposal. |
| 6 | Motorola Mobility France S.A.S | Thisisamajorproposalwithpossiblysignificantimpactonhigher-layerconfigurations,L1behavior,etc. Theproposalshouldincludemoredetailsincluding thepurpose. Wedonotsupportthisproposalyet. |

# 4	Summary

Agreement

The extension of the semi-static DU resource type indication to frequency-domain resources within a carrier (in addition to existing Rel-16 per-carrier granularity) for H/S/NA resource types is supported

Agreement

Adaptation of an IAB-node’s multiplexing operation is supported. The adaptation may be based on multiple factors, for example (not necessary to support all of the following):

- Resource type (D/U/F) at the IAB-DU and IAB-MT

- Specific sets of time/frequency resources

- Certain conditions being met (e.g. supported timing modes, power control enhancements (if supported), etc.)

FFS:  Mechanisms for informing/coordination the change in multiplexing operation(s) between child and parent nodes (including whether the adaptation is dynamic or semi-static)

FFS: Need for explicit linkage between indicated multiplexing operations and other features/enhancements – e.g. number of required guard symbols, supported timing modes, and power control enhancements (if supported)

Agreement

For the semi-static DU resource configuration in the frequency domain within a carrier, the frequency-domain granularity is configurable

- FFS:  minimum resource size e.g. N PRBs/N RBGs

- FFS: Separate or joint TDM and FDM semi-static DU resource configurations

Agreement

Soft resource availability indications for frequency-domain resources are supported

- FFS enhancements to DCI Format 2_5

- FFS: Separate or joint TDM and FDM indications

Proposed Agreement:

To facilitate simultaneous operations and interference management, dynamic indication for restriction/usage/availability of beams (in both upstream and downstream directions) is supported

- FFS: Applicability to specific multiplexing cases or specific time-frequency resources

- FFS: Whether IAB-specific enhancements beyond the existing beam management framework are needed to the support the functionality

- FFS: Impact on the semi-static resource configurations (e.g., extending the H/S/NA resource attributes to the spatial domain)

- FFS: Whether panel-based granularity is additionally supported

Proposed Agreement:

The following enhancements to support intra-band inter-carrier dual connectivity for both inter-donor and intra-donor scenarios are considered (in addition to reusing solutions for inter-band dual connectivity) to support simultaneous Tx and/or Rx at the child IAB-MT to/from both parent links:

-      Extending the Rel-16 CA TDD conflict resolution framework for synchronous intra-band NR-DC operation

-      Exchanging TDD configurations between the parent nodes (for both intra-donor and inter-donor operation) and exchanging/coordinating H/S/NA configurations for the child node between donors (at least for inter-donor operation)

Proposed Agreement:

Rules on explicit availability indication in case of reception of DCI Format 2_5 from multiple parents/cells are supported in Rel-17

-      FFS: Feasibility and need for enhancements to support coordination between parent nodes (e.g. indication of soft resource availability from child node to parent node(s))