3GPP TSG RAN WG1 #104b-e							    R1-2104093

e-Meeting, April 12 –20, 2021

Source:	Moderator (OPPO)

Title:	FL summary for AI 8.11.1.1 – resource allocation for power saving (final)

Agenda item:	8.11.1.1

Document for:	Discussion and Decision

Introduction

In the latest version of Rel-17 WID for NR sidelink enhancement [1], the objective for enhancing RA to reduce UE power consumption in mode 2 is captured as followed.

2. Resource allocation enhancement: - Specify resource allocation to reduce power consumption of the UEs [RAN1, RAN2] - Baseline is to introduce the principle of Rel-14 LTE sidelink random resource selection and partial sensing to Rel-16 NR sidelink resource allocation mode 2. - Note: Taking Rel-14 as the baseline does not preclude introducing a new solution to reduce power consumption for the cases where the baseline cannot work properly. - This work should consider the impact of sidelink DRX, if any.

This contribution provides a summary of the submitted contributions, email discussion topics and outcomes during RAN1#104b-e meeting.

Collection of agreements / conclusion in RAN1#104b-e

Conclusion:

- In periodic-based partial sensing,

  - It is not necessary to further discuss whether or not to introduce a threshold to re-define T1 and T2.

Agreements:

- In periodic-based partial sensing,

- For the set of Preserve values, down-select to one of the following in RAN1#105-e

    - Alt.1: Preserve corresponds to all values from the configured set sl-ResourceReservePeriodList

    - Alt.2: A set of Preserve values is (pre-)configured and includes up to the full set of values from the configured set sl-ResourceReservePeriodList

      - FFS if support multiple sets of Preserve values based on one or more metrics

      - FFS whether/how to restrict the set of values

- For the k value, down-selection to one of the following in RAN1#105-e (further refinement of each of the alternatives is possible)

        - Alt 1: Option 1 as in RAN1#104-e

        - Alt 2: A modified Option 5 as in RAN1#104-e, where the modification is such that it also includes option 1

          - FFS how to (pre-)configure (e.g. including bitmap), whether a maximum number of k values is needed, and whether it can be up to UE implementation to select a k value based on the (pre-)configuration

        - FFS details, e.g., sensing before the resource (re)selection trigger or the first slot of the set of Y candidate slots subject to processing time restriction, etc.

      - Note: companies are encouraged to provide more evaluations

Agreement:

- When periodic-based partial sensing is potentially performed by UE in a mode 2 Tx resource pool provided by higher layer, at least all of the followings are met:

  - Periodic reservation for another TB (sl-MultiReserveResource) is enabled for the resource pool

  - The resource pool is (pre-)configured to enable partial sensing

  - Partial sensing configured by higher layer in the UE

Topics for email discussion

[104b-e-NR-R17-Sidelink-01] Email discussion on resource allocation for power saving– Kevin (OPPO)

- 1st check point: 4/15

- 2nd check point: 4/19

- 3rd check point: 4/20

## Topic #1: Periodic-based partial sensing – (RSW definition [n+T1, n+T2]), (Preserve and k values) and (report timing)

Background: In RAN1#104-e meeting, a basic framework for periodic-based partial sensing based on the R14 LTE-V partial sensing scheme was agreed. Some remaining details left to be further discussed including the resource selection window definition, determining periodic sensing occasions from defining the Preserve and k values, and deciding the timing for reporting a subset of candidate resources to the higher layer.

- For the resource selection window definition [n+T1, n+T2], majority of views expressed (except one) favours keeping the existing T1 and T2 definition from R16 and apply no further restriction on the resource selection window to maintain the full flexibility, while one company thinks restriction on the resource selection window is beneficial for the UE to reduce the complexity.

- For determining the Preserve value (Option 1 and Option 2 are repeated in the sub-bullets), majority (except one) favours either Option 1 (13 companies) or Option 2 (20 companies). And out of those preferred Option 2, 17 companies think the subset should be (pre-)configured. The main reason for supporting Option 1 is to ensure sidelink transmission reliability is maintained by sensing slots that corresponds to all configured periodicities for the resource pool at a cost of higher power consumption. On the other hand, the main reasons for supporting Option 2 are for power saving and flexibility at a cost of performance degradation for not sensing all configured periodicities.

  - Option 1: Preserve corresponds to all values from the configured set sl-ResourceReservePeriodList

  - Option 2: Preserve corresponds to a subset of values from the configured set sl-ResourceReservePeriodList

    - FFS how to determine the subset (e.g., by (pre-)configuration, UE determination)

- For the selection of k value (top 3 options are repeated in the sub-bullets), based on the selected options the majority thinks that only one periodic sensing occasion is needed for a given Preserve value, regardless it is the most recent one or selected by UE implementation. In FL’s understanding, it is uncertain if the extra flexibility is needed for the UE to select the k value by implementation or for the configurability. Technically, if another UE decides not to reserve a certain resource anymore, then the most recent sensing occasion would be the most reliable instance to obtain this information for the sensing UE. And out of those supported Option 1, some expressed that the most recent sensing occasion should be before the “set of Y candidate slots” subject to processing time restriction to account for small reservation periodicities and/or the case when selected Y candidate slots are much later than the resource (re)selection trigger slot n (but still within the remaining PDB) to align with SL DRX on-period of the RX UE. Subsequently, this means the reporting of subset of resources to the higher layer will need to be postponed after the trigger slot n (e.g., just before the first Y candidate slot).

  - Option 1: Only the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger or the set of Y candidate slots subject to processing time restriction (14 companies)

  - Option 4: Only one periodic sensing occasion for one reservation period. The k value is up to UE implementation. Max value for k is (pre-)configured (6 companies)

  - Option 5: k is (pre-)configured, including multiple values (5 companies)

Tuesday GTW session: Comments raised on the GTW session includes the followings, and for which FL’s comments are also added.

- On the first main bullet (definition for T1 and T2), where SL DRX operation should be considered when defining T1 and T2 values. FL: In the existing R16 definition for T1 and T2, the range for T1 is already bounded by Tproc,1SL  which is the processing time required to prepare PSCCH/PSSCH transmission. And T2 is between a minimum selection value and the remaining PDB, and the exact value is up to UE implementation. As such the selection window [n+T1,n+T2] can be quite large and the UE has full flexibility to determine this range or the Y candidate slots that coincide with ON-period of SL DRX configuration. To ensure the packet TB is (re)transmitted within the SL DRX active or ON period is intended to be treated in Topic #7.

- On the second bullet (Preserve), the main comment raise was related to supporting a full set of values from sl-ResourceReservePeriodList. FL: technically I agree that it is best performance wise to monitor periodic sensing occasions that correspond to all the reservation periodicities supported for the resource pool in sl-ResourceReservePeriodList. But at the same time, the configurability sometimes can help to reduce power consumption from performing sensing all the time without sacrificing the performance, for which a large number of companies preferred to have. Therefore, I propose to have the configurability including up to the full set of periodicities supported in the resource pool, covering both options.

- On the third bullet (k) and related sub-bullet, main concerns / comments raised were related to

- Insufficient to monitor only one (the most recent one) periodic sensing occasion due to collisions/re-evaluation/pre-emption and half-duplex may happen in that sensing occasion. FL: As already replied during the GTW session, the most recent one is needed and hance proposed due to it has most recent periodic reservation information. To eliminate the concern of resource collision and re-selection due to other factors, this bullet is updated to have the configurability where more than one k values can be used, but including at least k = 1 (the most recent one) as it contains the most up to date and reliable reservation information.

- It will complicate the UE process if sensing occasions can be up to / just before the selected Y candidate slots. FL: there are two main reasons for this choice. 1) For short reservation periodicities, its corresponding sensing occasions may be located after the triggering slot n. It is important that these sensing occasions are still monitored by the UE before the resource selection. 2) since Y candidate slots can be selected anywhere by UE implementation within [n+T1, n+T2], it is likely that a UE will select the Y slots to be align with SL DRX ON-period, which could be must further down the resource selection window. At the same time, the UE should also perform contiguous partial sensing up to 32 slots before the Y slot. Then in this case, the report timing should be after the triggering slot n and just before the Y slots to take into account of sensing results from monitoring the contiguous 32 slots and sensing occasions for short periodicities.

- Clarification is needed on the processing time restriction. FL: this is related to Tproc,0SL and Tproc,1SL before and after the reporting slot, respectively. For short reservation periodicities, the corresponding period sensing occasions may overlap with the reporting slot and even the start of Y candidate slots. In this case, we still want to leave room for processing sensing results.

- For the fourth bullet, it is proposed to remove this bullet as it is preferred by some companies to align the triggering slot and the reporting slot as in R16. FL: this is related to the comment on the third bullet, where UE should only monitor sensing occasions before the triggering slot n. As already explained with 2 main reasons above, I hope the intention and proposal behind this bullet is now clearer.

Then based on the above summary and explanations, FL’s Proposal 1 for this topic is now updated and provided in Section 3.1.1 for further comments and suggestions.

### Proposals before 1st check point (April 15th)

Proposal 1:

- In periodic-based partial sensing,

  - The baseline for T1 and T2 defined in the same way as in R16 NR-V2X according to step 1 [TS 38.214 Sec. 8.1.4] is confirmed.

  - A set of Preserve values is (pre-)configured and includes up to the full set of values from the configured set sl-ResourceReservePeriodList.

  - For the k value, combine Option 1 and Option 5 as:

    - k is (pre-)configured, including multiple values with at least k=1 before the set of Y candidate slots subject to processing time restriction.

  - The timing for UE to formulate and report a subset of resources to higher layer is

max(n, ty0SL-Tproc,1SL), where ty0SL is the slot index of the first selected Y candidate slots.

| Company | Comments |
| --- | --- |
| NTT DOCOMO | Support 1st one. On 2nd one, we do not understand why full set leads to larger power consumption. the number of k values and/or the number of P_reserve in the resource pool can control this, so the motivation to achieve this by using subset of P_reserve is quite unclear for us. If subset is allowed, many collisions could happen. This is not the direction of NR-SL. On 3rd one, we are OK with the updated version only if the k=1 is maintained. Support 4th one. |
| Xiaomi | We are fine with the 1st and 2nd subbullet. We do not agree on the 3rd and 4th subbullet. For the 3rd bullet, our preference is to leave it to UE implementation to choose one or more sensing occasion(s) among the most recent sensing occasions before resource selection is triggered, for each reservation period. This can help UE to align the sensing occasions of different periodicities to reduce the overall sensing duration. The probability that the sensing results is outdated would be low, and it is worthwhile for power saving. In addition, we cannot agree to introduce a sensing window overlapping with resource selection window for periodic-based partial sensing. Periodic-based partial sensing is from LTE-V2x partial sensing, where the resource selection decision is made based on the sensing results before the resource selection is triggered. For the 4th bullet, we do not see the need to change the principle that resource (re)selection is performed once it is triggered, which is valid since Rel-14 V2x. Further sensing and resource reselection behaviour of UE is defined as re-evaluation and pre-emption in Rel-16 NR V2x. We do not see any reason to change this principle. Furthermore, if the scheduling decision can be delayed as proposed by the FL, there will have much additional RAN2 impact which is not our preference. Therefore, we strongly suggest to keep the principle that resource (re)selection is performed once it is triggered. And it will not harm our further discussion on UE sensing behaviour after slot n in re-evaluation and pre-emption topic. |
| OPPO | 1st bullet: yes 2nd bullet: the motivation of the proposal is to balance the power consumption and performance. While how much the effect of monitoring only subset values of set sl-ResourceReservePeriodList on the PRR performance is not evaluated. If the PRR performance lost is larger, it is not reasonable to only monitoring subset values. Then we propose: - Preserve includes full set of values from the configured set sl-ResourceReservePeriodList. - FFS subset values from the configured set sl-ResourceReservePeriodList (pre-)configured for Preserve 3rd bullet: we prefer to keep k=1, which includes the most recent reservation information. Considering insufficient sensing results due to collisions/re-evaluation/pre-emption and half-duplex, it can be addressed by following re-evaluation/pre-emption or continuous partial sensing after slot n. furthermore, based on our evaluation results (with k=1), the PRR performance of period-based partial sensing is very close to full sensing. There is no necessary to support k>1 at the cost of more power consumption. 4th bullet: we prefer to remove the proposal. How and when UE reports resource set to higher layer is up to UE implementation. Even in LTE-V2X partial sensing mechanism, there is no specification for that. |
| vivo | For the 1st bullet Agree. For the 2nd bullet We agree that the set of Preserve values can be (pre)configured. In addition, some limitations on the size of this set should be considered. If the number of Preserve values selected for determining the periodic sensing occasions is too small, e.g., a pool is configured with 10 period values, but only 1 or 2 values are selected from the 10 different period values configured for a pool, the possibility of resource collisions can be high because a UE performing sensing based on Preserve values cannot identify potential collisions between its candidate resources and resources reserved with a periodicity of the other 8 configured period values. To avoid such issue, a minimum number of Preserve values should be further considered. - A set of Preserve values is (pre-)configured and includes up to the full set of values from the configured set sl-ResourceReservePeriodList. - The size of the Preserve values set should be no smaller than Pmin, FFS on how to determine the Pmin For the 3rd bullet We are generally fine with this bullet, i.e., having one or multiple values for k. But the detailed signaling design can be discussed later or up to RAN2 (e.g., integer values, a list of integer numbers, bitmap, etc.) - For the k value, combine Option 1 and Option 5 as: - k is (pre-)configured, including multiple values with at least k=1 before the set of Y candidate slots subject to processing time restriction. - FFS the detailed signaling design for k For the 4th bullet Disagree. Timing for reporting is up to UE implementation and does not need to be specified in the spec. we suggest removing it |
| Intel | 1st bullet - Support 2nd bullet – Support 3rd bullet – Support k=1 only 4th bullet – Propose to leave up to UE implementation with Tproc,1 respected. |
| CMCC | OK with bullet 1~3. Regarding the 4th bullet, share similar views as OPPO that it should be up to UE implementation |
| Fujitsu | We are generally fine with the 1st and 2nd bullets. For the 3rd bullet, we still prefer to agree Option 1 only, because we think the main motivation is to do the enhancements for power saving on this topic. Performing sensing for a given periodicity more than once is not beneficial from power efficiency perspective. Further, we think in LTE-V and R-16 NR V2X, only the most recent sensing result is used to do resource exclusion for a given periodicity, so we think reuse this mechanism is enough to obtain the reliable sensing results. In addition, to make the subset reporting time clearer, we propose to do the modification for the last bullet as following: - The timing for UE to formulate and report a subset of resources to higher layer is not earlier than max (n, ty0SL-Tproc,1SL), where ty0SL is the slot index of the first selected Y candidate slots. Furthermore, we think the FFS in the agreement of last meeting, “FFS relationship between periodic sensing occasions and SL-DRX”, still needs to be added in this proposal or discussed in other proposals since it has not been addressed yet. |
| Panasonic | For the first bullet, as partial sensing is naturally a short term sensing. To allow a shortened [T1, T2] window would be beneficial to reduce the complexity to select the Y slots. A UE can use either [T1, T2] or a shortened window. We are ok with the second and the third bullets. For the fourth bullet, up to UE implementation would be enough. |
| LGE | We generally agree to FL proposal, but have the following comments. - Regarding T1 restriction in SL-DRX operation, we agree that Y candidate slots can be selected within RX-UE’s DRX active time by UE implementation, so no need to modify the current T1 restriction. However, it’s not true for T1 determination in aperiodic transmission case as in FL proposal 4. The candidate slots are all slots within the resource selection window in aperiodic traffic case, so T1 should not be bound by Tproc,1 for aperiodic transmission case. There are two options – separate rule for T1 for each case, or common rule for both case. We prefer the latter approach. The following modification is suggested for the 1st sub-bullet. - The baseline for T1 and T2 defined in the same way as in R16 NR-V2X according to step 1 [TS 38.214 Sec. 8.1.4] is confirmed. T1 and T2 are UE implementation as long as 0≤T1<T2≤remaining PDB. - As the partial sensing should be done only before the first candidate slot (+ UE processing time), the value of k=1 cannot always be included. The following modification is suggested for the 3rd sub-bullet. - For the k value, combine Option 1 and Option 5 as: - Multiple k is values are (pre-)configured, including multiple values with at least k=1. UE performs sensing only before the set of first Y candidate slots subject to processing time restriction. - Considering no specification text regarding the reporting timing in LTE-V2X partial sensing, the same rule is used for Rel.17 SL. - No specification on the timing of reporting a subset of resources to higher layer. |
| Samsung | Support 1st bullet. 2nd bullet: We would like update as follows: A set of Preserve values is (pre-)configured and includes up to the full set of values derived from the configured set sl-ResourceReservePeriodList. We prefer to additionally consider (pre-)configured Preserve values that is derived from (rather than directly included in) sl-ResourceReservePeriodList, e.g. LCM of sl-ResourceReservePeriodList, to reduce power consumption of sensing. For example, if sl-ResourceReservePeriodList includes {10,15} the derived value is the LCM which is 30, then power consumption is reduced by half. 3rd bullet: We consider the most recent occasion i.e. k=1 is sufficient for partial sensing. Monitoring extra values of k is less efficient considering its power consumption. In addition, the motivation of using sensing occasion before the set of Y candidate slots is unclear for us. It implies the sensing occasion may occur after slot n where resource selection is triggered, then the timing of UE determining TX resource will be correspondingly modified. We consider it as unnecessary complexity and not align with the principle of LTE partial sensing. 4th bullet: We prefer to remove the proposal. There was no similar restriction in LTE partial sensing or NR Rel-16 sensing, and the timing of UE reporting candidate resource set to higher layer can be totally up to UE implementation. |
| NEC | 1&2 Accept: We agree with 1st [T1, T2] proposal and can accept the 2nd Preserve values proposal as compromise (prefer full set). 3: Disagree, only option 1 with the most recent one is acceptable which is in align with Release 16 and Release 14 sensing procedure where only the last reservation (k =1) before sensing end window will be used to exclude resources in selection. The other occasions will not apply due to Q = 1. 4. One comment: what if the first selected Y candidate slots is already excluded during the sensing period before ty0SL-Tproc,1SL? Could we further postpone the report timing to the next available candidate slots for more accurate sensing result? Because some short period reservation may exist after ty0SL-Tproc,1SL. and we don't have to maintain the processing time for a excluded slot ty0SL . We sugget to modify as 4. The timing for UE to formulate and report a subset of resources to higher layer is max(n, ty0SL-Tproc,1SL), where ty0SL is the slot index of the first available slot among selected Y candidate slots. |
| Lenovo, Motorola Mobility | We are OK with this proposal. |
| Spreadtrum | We agree with 1-3 bullets. For 4th bullet, the timing for UE to formulate and report a subset of resources should be up to UE implementation. |
| Ericsson | We have some comments for this proposal: Regarding bullet 3, in our view at least the most recent occasion should be monitored. There are two main reasons for this approach: first, it is aligned with the current Rel-16 which reduces the potential specification/complexity issue. The second one as commented by FL, the last sensing occasion is more likely to provide a more up-to-date information of the resource pool. In case the current proposal is agreed, and several periodic sensing occasions are taking into consideration k >1, there should be studied ways to combine the information from the several windows which could potentially be contradictory. This will add more specification work and need to be captured as an FFS in the proposal. Regarding bullet 4, some clarification is need. We do not understand the purpose of having such bullet. In addition, it is not clear to us that it is even feasible. In our understanding, the procedure for resource selection would be like this: - Resource selection is triggered at time n. Consequently: - Assuming that the UE has already performed sensing in the corresponding slots (i.e., those for the Y slots in the selection window), resource selection can proceed immediately. Thus, the set of resources is reported to higher layer already at time n. - The UE selects resources, with the first selected resource in slot ty0SL. - Re-evaluation and re-selection, if applicable, are used. - Transmission in slot ty0SL takes places (assuming no reselection). The point we try to illustrate is that in this case, max(n, ty0SL-Tproc,1SL) = ty0SL-Tproc,1SL, but the resource set is reported at time n. Of course, the UE may report again the resource set to perform re-evaluation and re-selection if applicable, but it is not clear to us that this is the intention of the proposal. |
| Huawei, HiSilicon | - For the first bullet, baseline is already agreed, and therefore does not require any confirmation. This bullet should be removed. - Second bullet: We think that allowing less than all Preserve to be configured does not provide advantages. We evaluated the benefit of reducing the list of k to be very small, and expect the same behaviour for reducing the list of Preseve. At least, it should be the default case that all are assumed by the UE, and we further discuss what, if any, subsets to permit. . - Third bullet: We support this. Allowing multiple k values is beneficial for reliability, and configuration flexibility is helpful on achieving diversity and adaption to different cases/conditions. - Fourth bullet is OK. Given that slot ty0SL is the first potential slot for transmission based on partial sensing instead of slot T1, the timing on report of SA does not need to be at slot n as in full-sensing, hence it should be ty0SL-Tproc,1SL. |
| CATT, GOHIGH | We agree 1st and 2nd one Regarding the 3rd one, we have two comments, first is that only k=1 is valid resource reservation for sensing, when k>1, the sensing result is not used according current spec; second is that the reference time for resource selection, we think it should be aligned with R16 NR-V2X, i.e. resource re-selection trigger time n. Regarding the 4th one, with the 1st bullet, if we follow the R16 behaviour, the candidate could be n, the time between n and ty0SL-Tproc,1SL could be used for re-evaluation or pre-emption, there is no need to specify the exact reporting time. |
| Fraunhofer | We are in general fine with the FL’s proposal. We are supportive of the 2nd and 3rd main bullet that allows the parameters to be configurable. This enables the UE to select the required configuration based on its need to save power, as well as based on the priority and reliability required for the intended transmission. For the 3rd main bullet, we support the text changes by LG. |
| ETRI | We are OK with 1 and 2. However, we still have a concern on 3 since multiple K values would diminish power saving effect, and an issue regarding insufficient sensing results could be addressed by re-evaluation/pre-emption or additional continuous partial sensing. In case of 4, it could be up to UE implementation. |
| Apple | For the first bullet: we still think the impact from SL DRX needs to be mentioned in defining T1 and T2. If the resource selection trigger time n is within Rx UE’s SL DRX off duration, and it may not be optimal restricting T1 by Tproc,1. Hence, we suggest adding a sub-bullet of “FFS impact from SL DRX”. For the second bullet: we are fine with the direction of the proposal. One more question is whether a single set of Preserve values is configured, or multiple sets of Preserve values can be configured? The multiple sets of Preserve values could be used for different data priority levels. Generally, for high priority data, the set of Preserve values is larger to achieve a more reliable resource selection. For example, the full set of values from sl-ResourceReservePeriodList is used for high priority data, while a subset of values from sl-ResourceReservePeriodList is used for low priority data. For the third bullet: we are generally fine. One clarification question is whether the multiple k values are used conditionally? For example, we could always assume k=1 initially. Only if k=1 is unavailable for sensing due to e.g., half duplex, etc, then the k value is extended from 1. For the fourth bullet: we may define the “n” is the resource selection trigger time. Also, we think this bullet may not be needed. |
| InterDigital | 1st bullet: Support 2nd bullet: We agree that Preserve values is (pre-)configured. Moreover, in our perspective, periodic-based partial sensing is necessary for both periodic and aperiodic traffic to detect semi-persistent reservation. For periodic traffic, the UE can perform sensing before the arrival of the data and the full set of Preserve can be (pre-)configured. For aperiodic traffic, the UE cannot predict the arrival time of the data. Hence, periodic-based partial sensing should be performed after the arrival time of the data and the UE can monitor a subset of Preserve based on the PDB. Therefore, we propose the following FFS for the 2nd bullet: - A set of Preserve values is (pre-)configured and includes up to the full set of values from the configured set sl-ResourceReservePeriodList. - FFS the subset of values for periodic and aperiodic traffic if periodic partial sensing is also supported for aperiodic traffic. 3rd bullet: Support k=1 only In our view, SCI decoded in the most recent period contain the most recently updated reservation information, which is reliable enough while providing power saving benefit. 4th bullet: Similar to NR R16 and LTE, it can be up to UE implementation. |
| Qualcomm | - For (1): - “is confirmed” should be removed as there was no earlier working assumption. - We are fine with this proposal, however, it should be clarified that not all slots within [n+T1,n+T2] are available for selection. In our view, only the slots in the intersection of [n+T1,n+T2] and the periodic partial sensing occasions are in the set of candidate slots. This is shown in the figure below: - On item (2), we do not think that configuring a subset of P_reserve values is desirable. The lack of sufficient sensing information due to monitoring only a subset of P_reserve values could increase the chance of collision; thereby, increasing UE’s power consumption. If a subset of values should be specified, then a UE should also perform sensing based on Prsvp_Tx to avoid consistent collision. - On item (3), we think setting k = 1 is sufficient as the SCIs monitored during the most recent period are the only ones that convey useful information about the current period. This is due to the fact that in every period, only the reservations for the immediate next period can be signalled. This is consistent with the Rel. 16 behaviour. - On item (4), the proposal seems to ignore the UE processing timeline; otherwise, the second term inside the max argument should always be larger than the first term. The feature lead mentioned that this condition is needed for sensing in case small periods are configured. However, we do not think that there is any difference in terms of sensing between the case under consideration and the scenarios considered in Rel. 16 (even in Rel. 16, a resource pool can be configured with small periodicity values.) As a result, no additional consideration is needed. |
| Futurewei | We are ok with 1 and 2. For 3, the update for 3 is in the right direction (multiple values). However, (pre-)configured k with multiple values can be equivalently represented with a bitmap. Since the bitmap is used in LTE-V, it is unnecessary to use a different format to represent the same thing. To incorporate option 1 and other proposals, we propose to use a bitmap representing the sensing occasions with LSB (the most recent one) bit equal to 1. To achieve better power saving, we propose to specify a maximum number of sensing occasions, which translates to the maximum number of 1’s in the bitmap or maximum number of k values. For 4, since UE can formulate and report a subset of resources, or select resource, on a slot later in ty0SL , considering contiguous partial sensing, we suggest change it to “The earliest or minimum timing for UE …..” |
| Convida Wireless | We are fine with the proposal. |
| MediaTek | We are fine with bullet 1 and bullet 2. On bullet-3, we should determine k occasions prior ‘n’ instead of ‘Y’. We should keep the same Rel-16 sensing window definition. With prior to Y, sensing window and selection window may overlap, which is different from Rel-16 NR sidelink design. On bullet-4, we prefer to leave processing time up to UE implementation. |
| Nokia, NSB | We support Bullet 1, and 2. For bullet 3 on k value, we would prefer that “The k value is up to UE implementation. Max value for k may be (pre-)configured.” Bullet 4: this timing of resource report shall be up to UE implementation. SL Rx UE won’t need this information for its SL reception. |
| Bosch | Main bullet 1: - We support the FL direction; however, we are in line with the opinions that says that not all slots within [n+T1,n+T2] are available for selection. In this case, the UE may freely adapt / or being configured to restrict its selection window. We also support non- contiguous slots for selection. Main bullet 2: Fully support Main bullet 3: Fully support. However, as a compromise we may go for: - k is (pre-)configured, including multiple values including k=1. FFS size of the values of k. Main bullet 4: we don’t see a need for restriction. The UE should be able to report resources up to its implementation. |
| ZTE | We agree sub-bullet 1 and 2. But for sub-bullet 3 and 4, we would like to clarify that the report timing should be the triggering slot n subject to some processing time, i.e. same as R16, considering the cooperation between MAC and PHY. The motivation for having an extended sensing window is more biased towards the contiguous sensing scenario, but given we are discussing the periodic-based partial sensing with which contiguous sensing should not be always bundled, then we shouldn’t assume the reporting timing would be always after slot n. With the above consideration, we think in sub-bullets 3, “before the set of Y candidate slots..” should be removed at this stage. Sub-bullet 4 should be removed as well. |

### Proposals before 2nd check point (April 19th)

FL observations and comments based on inputs received in Sec. 3.1.1:

- On the 1st main technical bullet of Proposal 1,

  - Support: DCM, Xiaomi, OPPO, vivo, Intel, CMCC, Fujitsu, Samsung, NEC, Lenovo, MM, Spreadtrum, CATT, GOHIGH, ETRI, IDC, Futurewei, Convida, MediaTek, Nokia, NSB, Bosch

  - The candidate slots are all slots within the resource selection window in aperiodic traffic case, so T1 should not be bound by Tproc,1 for aperiodic transmission case.

    - FL: Although the comment is more related to aperiodic transmission case in Proposal 4, but in that case T1 should still be bounded by Tproc,1 just as in R16. There should be no difference in terms of UE processing time whether or not UE performs partial sensing or full sensing.

  - If the resource selection trigger time n is within Rx UE’s SL DRX off duration, and it may not be optimal restricting T1 by Tproc,1.

    - FL: I expect Y candidate slots will be selected to overlap with SL DRX ON duration.

  - Preference:

    - To allow a shortened [T1, T2] window would be beneficial to reduce the complexity to select the Y slots. A UE can use either [T1, T2] or a shortened window

      - FL: It will, however, cause inflexibility. In R16, T1 and T2 values are choosen by UE implementation to allow full flexibility to adapt to number of resources what the higher layer would like to select. From my perspective, it is best to keep the same.

    - Remove this bullet since it is already agreed as a baseline.

    - It should be clarified that not all slots within [n+T1,n+T2] are available for selection.

- On the 2nd main technical bullet,

  - Support: Xiaomi, Intel, CMCC, Fujitsu, Pana, NEC, Lenovo, MM, Spreadtrum, CATT, GOHIGH, ETRI, Fraunhofer, Futurewei

  - Concerns:

    - If subset is allowed or too small, many collisions could happen.

      - FL: It is (pre-)configurable and the full set is allowed. I trust they will make a right (pre-)configuration choice.

  - Preference:

    - Making full set of periodicities for Preserve as the baseline and further evaluate if a subset if feasible.

    - Additionally consider (pre-)configured Preserve values that is derived from (rather than directly included in) sl-ResourceReservePeriodList, e.g. LCM of sl-ResourceReservePeriodList, to reduce power consumption of sensing. For example, if sl-ResourceReservePeriodList includes {10,15} the derived value is the LCM which is 30, then power consumption is reduced by half.

    - The size of the Preserve values set should be no smaller than Pmin, FFS on how to determine the Pmin.

    - Allowing less than all Preserve to be configured does not provide advantages

    - Whether a single set of Preserve values is configured, or multiple sets of Preserve values can be configured? The multiple sets of Preserve values could be used for different data priority levels.

    - For aperiodic traffic, the UE cannot predict the arrival time of the data. Hence, periodic-based partial sensing should be performed after the arrival time of the data and the UE can monitor a subset of Preserve based on the PDB.

- On the 3rd main technical bullet,

  - Support: Intel (k=1 only), CMCC, Fujitsu, Pana, Lenovo, MM, Spreadtrum, Fraunhofer, HW, HiSi

  - Concerns

    - Overlapping of sensing window and resource selection window

      - FL: this should be OK because resource (re)selection is only within the Y candidate slots, which are after the reporting time slot. If the selection of Y candidate slots is at the beginning of the resource selection window, there would not be an overlap. In LTE-V, the smallest periodicity for partial sensing and PDB are both 100ms, so there won’t be any overlapping of the two windows. But in NR sidelink, we could have very periodicities and therefore these should be taken into account.

    - Need to consider signalling design for k

      - FL: it is best leave RRC signalling design to RAN2.

    - As the partial sensing should be done only before the first candidate slot (+ UE processing time), the value of k=1 cannot always be included.

      - FL: Agree, I believe the current wording is fine.

    - Only option 1 with the most recent one is aligned with R16 and R14 sensing procedure where only the last reservation (k =1) before sensing end window will be used to exclude resources. The other occasions will not apply due to Q = 1.

      - FL: we may need to adjust the Q formulate later if other k values can be used.

    - In case the current proposal is agreed, and several periodic sensing occasions are taking into consideration k >1, there should be studied ways to combine the information from the several windows which could potentially be contradictory. This will add more specification work and need to be captured as an FFS in the proposal.

  - Preference:

    - Keep k=1 [OPPO, Intel, Fujitsu, Samsung, Ericsson, CATT, GOHIGH, ETRI, IDC, QC]

      - k=1 will contain the most up-to-date

      - It is inline with R14 and R16 mechanism and specification description

      - Reservation is only up to one period

      - Shown by evaluation results the PRR performance is very clost to full sensing

    - Up to UE implementation to choose one or more most recent occasions

      - Align the sensing occasions of different periodicities to reduce the overall sensing duration to save power; likelihood of sensing results are outdated would be low

    - Whether the multiple k values are used conditionally?

      - FL: we can still discuss how to handle unavailability due to half duplex issue later.

    - (Pre-)configured k with multiple values can be equivalently represented with a bitmap

      - FL: In LTE-V, this is feasible because a common divisor for Pstep was used. For NR sidelink it is not possible to find a common divisor unless Preserve = 1 is used. Then in this case, the bitmap would be very large considering 1000ms periodicities and SCS=120kHz. If taking into account of varying PDB range, then bitmap length would be varying as well.

- On the 4th main technical bullet,

  - Support: DCM, Lenovo, MM, HW, HiSi

  - Concerns:

    - Sensing and resource (re)selection behaviour of UE is defined as re-evaluation and pre-emption in R16

      - FL: If R16 re-evaluation and pre-emption procedure is followed, the triggering slot (n) is at m-T3. In R17 for periodic-based partial sensing, this triggering slot would be just before or mostly within the selected Y candidate slot. If followed the proposed formula, the formulation and reporting of a subset of resources would be the always same as the triggering slot.

    - Additional impact to RAN2 if triggering and reporting slots are not aligned

      - FL: In my understanding of MAC description, there is no relationship between the triggering of resource (re)selection procedure and the timing when it receives the subset of resource reported from L1. The timing restrictions are only related to selecting resources that can be indicated by a prior SCI and the HARQ RTT time gap (z). We could send an LS to RAN2 to confirm if there is any concerns if the triggering and reporting slots are not aligned.

    - It is never described in R14 LTE-V and R16 NR sidelink specifications.

  - Preference:

    - Aligning the resource (re)selection trigger and reporting time in slot (n) - Xiaomi

    - Leave it up to UE implementation, remove the bullet – OPPO, vivo, Intel (Tproc,1 respected), CMCC, Pana, LGE, Samsung, Spreadtrum, Ericsson, CATT, GOHIGH, ETRI, Apple, IDC

    - “FFS relationship between periodic sensing occasions and SL-DRX” from the last meeting still not address

      - FL: please refer to Topic #7 below

Proposal 1 (round II) for Thursday GTW session:

- In periodic-based partial sensing,

- For the set Preserve values

    - Alt. 1: Preserve corresponds to all values from the configured set sl-ResourceReservePeriodList

    - Alt. 2: A set of Preserve values is (pre-)configured and includes up to the full set of values from the configured set sl-ResourceReservePeriodList.

- For the k value,

    - Alt. 1: k=1 only

    - Alt. 2: k is (pre-)configured, including multiple values with at least k=1

  - UE performs sensing only before the set of Y candidate slots subject to processing time restriction.

  - The set SA is initialized to the set of all the candidate single-slot resources within the selected Y candidate resource.

Thursday GTW session: Comments raised on the GTW session includes the followings, and for which FL’s comments are also added.

- Relating to Preserve:

  - If only a subset of periodicities is used for Preserve, it may not include its own periodicity and cause persistent collision.

  - There was one proposal to use a value that is common multiple from the configured periodicities, e.g., when 10ms and 15ms are configured, the UE uses 30ms as the Preserve value to reduce sensing effort.

  - Another proposal to have configured multiple sets of Preserve, based on transmission priority.

  - A lot of comments relating to better performance and ensuring collisions are avoided by using the full set of configured periodicities for the RP. On the opposite, many comments were about power saving benefits can be obtained when a subset of periodicities are used. It was then suggested the decision should be based on further evaluations in the next meeting and companies are encourage to submit results including PRR performance and power saving gain to justify the choice.

- Relating to k value:

  - When k=1, it may fall within Y slots

  - Similar to Preserve value discussion, where the commets are most related to performance/reliability vs. power saving gains and flexibility. And also whether or not the behavior is inline with existing R14 and R16 resource reservation mechanism.

- Relating to 3rd bullet:

  - There were quite many comments relating to re-evaluaiton and pre-emption aspects should be clarified / considered

  - It should be changed to “before the first Y candidate slots”

  - Why sensing is only performed before the Y candidate slots? Whether sensing is allowed after slot n

  - The meaning of “sensing” and “just before” sould be clarified

- Relating to 4th bullet:

  - While some companies agreed with this bullet, some think it should still be FFS for now.

Proposal 1-1 (round III):

- In periodic-based partial sensing,

- For the set of Preserve values, down-select to one of the following in RAN1#105-e

    - Alt.1: Preserve corresponds to all values from the configured set sl-ResourceReservePeriodList

    - Alt.2: A set of Preserve values is (pre-)configured and includes up to the full set of values from the configured set sl-ResourceReservePeriodList

    - Alt.3: UE can use a value that is common multiple from the configured set sl-ResourceReservePeriodList

    - Alt.4: Multiple sets of Preserve values based on transmission priority

- For the k value, down-select to one of the following in RAN1#105-e

    - Alt.1: Only the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger or the set of Y candidate slots subject to processing time restriction

    - Alt.2: k is (pre-)configured, including multiple values with at least k=1

    - Alt.3: k is from a (pre-)configured bitmap, as in LTE-V

  - Note: companies are encouraged to provide more evaluations

| Company | Comments |
| --- | --- |
| NTT DOCOMO | Generally we support this direction, i.e. down-selection at the next meeting based on evaluations. For P_reserve, we think Alt 1 and Alt 2 with many supports are sufficient. Alt 3 and Alt 4 can be removed. Adding alternatives based on one or few support is not desirable; otherwise companies need huge evaluation efforts. For k value, Alt 2 should be aligned with Alt 1 as commented in GTW. The motivation of k=1 is to monitor the most recent sensing occasion subject to processing time restriction. But monitoring slot based on k=1 with small periodicity could be included within Y slots, which is not intended behaviour. In addition, the smallest k for the most recent sensing occasiton subject to processing time restriction is different per periodicity value. So Alt 2 should be the following: - Alt.2: - k is (pre-)configured, including multiple values, and with at least k=1 - the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger or the set of Y candidate slots subject to processing time restriction, regardless of (pre-)configured k value(s) |
| Apple | Overall, we are fine with the proposal. For Preserve values, we think Alt. 4 covers both Alt. 1 and Alt. 2, if the first set of Preserve values are all values from the configured set sl-ResourceReservePeriodList, and the second set of Preserve values are a subset of values from the configured set sl-ResourceReservePeriodList. UE may determine which set of Preserve values to be used based on data priority. For high priority data, the first set is used for high reliability. For low priority data, the second set is used for power saving. This achieves the configurability and provides a compromised solution between Alt.1 and Alt. 2. For k value, we are fine with Alt. 1 or Alt. 2. For Alt. 1, we need to clarify (in proposal) that if the most recent sensing occasion is infeasible, then a replacement sensing occasion could be used. |
| Futurewei | We are ok for the proposal on Preserve values For the k values, we need some clarifications on Alt.3: k is from a (pre-)configured bitmap, as in LTE-V. If it is as what FL explained in the response before Thursday meeting, that using bitmap with the common divisor to specifie the sensing on the period slots, we do not support it. What we support is using a bitmap to represent k values where each k specified a sensing occasions. So it is alt. 2, multiple k values represented with a bitmap with a fix bitwidth, where bitmap length indicates max k value. Forward thinking, if partial sensing is supported for inter-UE coordination, a bitmap fixed bitwidth for k values is an efficient format for UE B informing to UE A if using PHY signaling. Also we propose to specify a maximum number of sensing occasions. As most companies who support k=1 or k=[1 2] have a concern on the power consumption for k having multiple values (too many sensing occasions). Specifying the maximum number of k values address this concern. Therefore, we propose revise Alt. 2 as - Alt.2: k is (pre-)configured, including multiple values with at least k=1 - FFS multiple values of k are represented in a bitmap with a fix bitwidth. - FFS specify a maximum number of k values Then Alt 3 can be removed (unless others support it). |
| LGE | - Main bullet To avoid any misunderstanding that the proposal is not for resource re-evaluation or pre-emption checking, add the following text: “In periodic-based partial sensing except for resource re-evaluation and/or pre-emption checking,” - Preserve Support FL proposal. We don’t need to spend time any more for downselection at this meeting. - k value Support FL proposal with one modification if the following is a common understanding: - If Alt.1 is selected at the next meeting, we will have FFS point on whether the resource (re)selection trigger or the set of Y candidate slots is a reference timing to determine the most recent occasion. - Rephrase text “or the set of Y candidate slots” as “the first candidate slot” Otherwise, we support “Option 7: FFS others” in the previous agreements, and want to add the following alternatives to the 2nd sub-bullet: - Alt.4: Only the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger - Alt.5: Only the most recent sensing occasion for a given reservation periodicity before the first candidate slot subject to processing time restriction |
| NEC | Support. Suggest to delete as in LTE-V, in alt.3 to k is from a (pre-)configured bitmap, as in LTE-V. Because in LTE-V, the sensing occasion is k*Pstep, and we are sensing occasion here is k*Preserve , as Pstep and Preserve don’t have exactly same meaning, the bitmap in LTE-V may neither doesn't have same meaning with k here. |
| CMCC | For the sake of making progress, we are generally fine to leave options in this meeting and let the evaluation results speak in the next meeting for down-selection. However, we still have some comments regarding the listed alternatives: - On P_resreve We prefer to down scope to Alt.1 and Alt. 2, as they were options from last meeting, adding new alternatives as Alt. 3 and Alt.4 does not help much. In addition, regarding Alt. 3 to allow using common multiple from configured periodicities, as the example taken in the GTW session (i.e., when 10ms and 15ms are configured, the UE uses 30ms as the Preserve value to reduce sensing effort), though it was claimed to have power saving gain, we think that it also brings the possibility of miss-deteciton of reservations. This similar issue can also be identified in LTE-V, where the P_reserve = 100ms is not optimized towards the reservation perioidicities of 20ms and 50ms. To be specific, if an initial transmission occurs at the latest 10ms or 15 ms to the end of the sensing occasion, then using P_reserve = 30ms has no chance to detect the reservations and will lead to further collision. In our views, Alt. 2 has already taken power saving issue into account, and Alt.3 is not really needed. - On k value We also prefer to only discuss Alt. 1 and Alt. 2. Regarding Alt. 3, the potential overhead could be quite large, considering the case that very small reservation period is enabled in a resource pool. |
| Fujitsu | Genarally we are fine with this proposal, the down-selection can be done after more evaluations are provided. Besides, for Preserve, considering evaluation efforts, , we suggest that only Alt 1 and Alt 2 should be kept for evaluation work as most companies support to select one from these two options during GTW. For k value, we agree DOCOMO’s view that Alt 2 should be aligned with Alt 1. |
| OPPO | Fine with the proposal |
| Panasonic | We are ok with FL’s proposal. |
| Huawei, HiSilicon | We agee the principle to down select the altenatives basd on the evalution results and benefits observed. However, quite divergent alternatives will increase the simulation efforts (especially for short period between April and May RAN1 meeting) and make the results hard to be compared. Therefore, higher level directions for simulation are preferred and prioritized – full set or subset for Preserv, one or multiple values for k. For the Alt.4 for Preserve determination, it can be further discussed when subsets is justified beneficial, and how to decide the multiple sets can be next step. For the Alt.3, we have similar feeling with others, the applicability of it is still unclear. So focusing on first two alternatives are enough. Similarly, Alt.3 for k value determination is caring about how to derived the multiple values which should be also incorporated in Alt.2. Therefore, the modified proposals are as following: - For the set of Preserve values, down-select to one of the following in RAN1#105-e - Alt.1: Preserve corresponds to all values from the configured set sl-ResourceReservePeriodList - Alt.2: A set of Preserve values is (pre-)configured and includes up to the full set of values from the configured set sl-ResourceReservePeriodList - Alt.3: UE can use a value that is common multiple from the configured set sl-ResourceReservePeriodList - Alt.4: Multiple sets of Preserve values based on transmission priority - For the k value, down-select to one of the following in RAN1#105-e - Alt.1: Only the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger or the set of Y candidate slots subject to processing time restriction - Alt.2: k is (pre-)configured, including multiple values with at least k=1 - Alt.3: k is from a (pre-)configured bitmap, as in LTE-V - FFS how to (pre-)configure, e.g. including bitmap |
| Samsung | We are generally fine with the proposals. In our understanding, in 1st main bullet Alt 4 can be used together with other alternatives, so we suggest the following modifications: - In periodic-based partial sensing, - For the set of Preserve values, down-select to one of the following in RAN1#105-e - Alt.1: Preserve corresponds to all values from the configured set sl-ResourceReservePeriodList - Alt.2: A set of Preserve values is (pre-)configured and includes up to the full set of values from the configured set sl-ResourceReservePeriodList - FFS if support multiple sets of Preserve values based on one or more metrics (e.g. priority, CBR, HARQ error rate, etc.) - Alt.3: UE can use a value that is common multiple from the configured set sl-ResourceReservePeriodList - Alt.3: A set of Preserve values is (pre-)configured and includes common multiple(s) of values from the configured set sl-ResourceReservePeriodList - FFS if support multiple sets of Preserve values based on one or more metrics (e.g. priority, CBR, HARQ error rate, etc.) - Alt.4: Multiple sets of Preserve values based on transmission priority The 2nd main bullet looks OK for us and we prefer Alt 1. |
| Intel | Regarding Preserve – It is OK to discuss PRESERVE further. Some comments: - We do not see how evaluations can help. There is no evaluation methodology for that case. It is unclear how group wants to compare analysis. We should consider all scenarios with possibly different intensity for different periods, etc. For us tradeoffs are clear. - The Alt.3 needs to be clarified. We cannot accept it in the current form. UE should follow what is configured. We also think that Alt.2 covers Alt.1 and Alt.3 - Alt.4 look also problematic. UEs with low priority trnamission should actually sense more to be aligned with priority/pre-emption principles. Regarding k settings - We suggest k = 1 instead of Alt.1. It seems we miss aspects here. As of now it is unclear how Y is defined. We can separately discuss the cases when sensing window and resource selection window overlap. We also should separately consider contiguous and periodic sensing even if at some points they overlap. BTW it is already possible in Rel16 since reavaluation is done until last transmission i.e. eventually UE continues sensing in resource selection window. What is the difference here? We just need to adapt clausa 8.1.4 of 38.214 to partial sensing. |
| Fraunhofer | We support the proposal from the FL with the following comments: For Preserve, we agree with Apple that Alt. 4 covers both Alt. 1 and Alt. 2, and provides the UE with the required flexibility to ensure reliability and power saving based on the priority of the transmission. In this regard, we are also supportive of the wording from Samsung to include Alt. 4 under Alt. 2. For the k value, we agree with DCM that the text for Alt. 2 needs to be modified according to the changes reflected in Alt. 1. - Alt.2: k is (pre-)configured, including multiple values with at least k=1 the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger or the set of Y candidate slots subject to processing time restriction. |
| Xiaomi | We support the direction of the FL proposal. For alternatives in the second subbullet, we agree with HW that the difference between alt.2 and alt.3 is just on how to (pre-)configure. We think the most important issue is whether multiple sensing occasions would be benefitial or not. In addition, we agree with DOCOMO and Fujtsu that the description of alt.2 should be aligned with that of alt.2. Therefore, we would like to suggest the following revision for alt.2 of the 2nd subbullet: - Alt.2: k is (pre-)configured, including multiple values with at least k=1 the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger or the set of Y candidate slots subject to processing time restriction - FFS how to determine k values for sensing occasions other than the most recent sensing occasion, e.g. by (pre-)configuration, up to UE implementation with (pre-)configured max value for k |
| Vivo | - Preseve We prefer to keep alt.1 and alt.2 only as they have support of most companies. - K It should be clarified that k=1 in alt.2 refers to the most rescent occasion in option1. We prefer to keep alt.1/2/3 and would like to remove ‘as in LTE-V’ in alt.3 as suggested by NEC. In general, the N most recent sensing occasions provide the most reliable sensing results, it is reasonable to assume that only N consecutive bits will be set to 1 to enable the corresponding sensing occasions, in which case the length of the bitmap will be limited and the overhead of alt.3 will not be an issue. |
| Ericsson | We propose to delete Alt. 3 and Alt.4 from the first bullet. In our view, it is enough if we study Alt.1 and Alt.2 since they provide a good trade-off between detection of collisions and power saving, while the other alternatives are just optimizations on top of them. For the value of k. We do not think adding a bit-map is a simple solution and moreover, does not add anything extra regarding flexibility or applicability when compared with Alt. 2. Therefore, we propose to delete Alt.3 from the second sub-bullet. |
| ZTE | We prefer to capture in the note the setting of multiple periodicities, - Note: companies are encouraged to provide more evaluations including aspects such as how to ensure the resource selection is done using the sensing results with all the configured periodicities. During the previous RAN1 meeting, the following in the agreement needs to be clarified in case more than 1 periodicity is set, how is the resource selection is supposed to be performed? the UE monitors slots of at least one a set of periodic sensing occasions, where a periodic sensing occasion is a set of slots according to if tvSL is included in the set of Y candidate slots.. |
| Nokia, NSB | For the set of Preserve values, these 4 alternatives are not much different from the four options in the agreement of last RAN1 meeting, indicating no progress. Can we remove Alt 3 and Alt 4 to limit our evaluating efforts? We support the 2nd bullet on k value. |
| CATT,GOHiGH | In general OK with this proposal . Regarding K’s value, we think alt3 is just a special signalling form of alt2 . |
| MediaTek | We support the general direction of this proposal to evaluate performance for next meeting. However, we prefer to keep the simulation efforts reasonable. With 4 options under Preserve and 3 options under k value, we are looking at 12 different simulation scenarios. Perhaps, we can try to merge some of the alternatives together. For example, Alt.2 and Alt.3 under Preserve can be merged as below: - Alt.1: Preserve corresponds to all values from the configured set sl-ResourceReservePeriodList - Alt.2: A set of Preserve values is (pre-)configured and includes up to the full set of values from the configured set sl-ResourceReservePeriodList - Alt.3: UE can use a value that is common multiple from the configured set sl-ResourceReservePeriodList - Alt.2: A set of Preserve values is (pre-)configured from {1ms, 2ms, …, 99ms} and {100ms, 200ms,..1000ms}, possibly including the full set of values from the configured set sl-ResourceReservePeriodList and/or common multiples from the configured set sl-ResourceReservePeriodList - Alt.4: Multiple sets of Preserve values based on transmission priority In addition, regarding Alt.2 under k value, could you please clarify the meaning? If the intention is to preconfigure k as a list of values, we are not sure how this is different from Alt.3 bitmap configuration. If the intention of Alt.2 is to configure sensing on the most recent k number of occasions, k should be defined as integer. For example, when k=3, UE performs sensing on the most recent 3 occasions for each Preserve. If the intention is the latter, we would like to re-phrase Alt.2 as below: - Alt.2: k is (pre-)configured as an integer, indicating a number of most recent occasions with a minimum value of k=1 |
| Qualcomm | For P_reserve, we propose to remove both Alt3 and Alt4. Alt3 leads to obtaining even less sensing information at the UE as compared to Alt2. The objective of Alt4 is not clear. Furthermore, it increases the latency for packet transmission since that a UE needs to be triggered for resource selection first (to know the priority of the packet), and then performs sensing. We propose to remove Alt3 for the determination of k. |
| InterDigital | For Preserve WE support Samsung’s formulation, in which Alt. 4 is moved under Alt. 2 and Alt. 3 as FFS. Moreover, in our view, periodic-based partial sensing is necessary for both periodic and aperiodic traffic to detect semi-persistent reservation. For periodic traffic, the UE can perform sensing before the arrival of the data and the full set of Preserve can be (pre-)configured. For aperiodic traffic, the UE cannot predict the arrival time of the data. Hence, periodic-based partial sensing should be performed after the arrival time of the data and the UE can monitor a subset of Preserve based on the PDB. Therefore, Preserve should be (pre-)configured baed on traffic type and/or priority. We propose to modify the proposal for Preserve as follow: - In periodic-based partial sensing, - For the set of Preserve values, down-select to one of the following in RAN1#105-e - Alt.1: Preserve corresponds to all values from the configured set sl-ResourceReservePeriodList - Alt.2: A set of Preserve values is (pre-)configured and includes up to the full set of values from the configured set sl-ResourceReservePeriodList - FFS whether to support multiple sets of Preserve values based on traffic type and/or transmission priority - Alt.3: UE can use a value that is common multiple from the configured set sl-ResourceReservePeriodList - FFS whether to support multiple sets of Preserve values based on traffic type and/or transmission priority - Alt.4: Multiple sets of Preserve values based on transmission priority For k We are ok with the proposal. |

Proposal 1-2 (round III):

- In periodic-based partial sensing,

  - For periodic transmissions , the set SA is initialized to the set of all the candidate single-slot resources within the selected set of Y candidate resource.

| Company | Comments |
| --- | --- |
| NTT DOCOMO | The motivation to limit to periodic transmissions or to have this proposal is unclear for us |
| Apple | We think the case can also be applied to aperiodic transmissions. |
| Futurewei | We do not support this proposal. The contiguous partial sensing is in discussion. With contiguous partial sensing, it is possible that UE continues the sensing after ty0, e.g. at ty, y>y0. Then the set SA can be initialized to the candidate signal slots after ty. Although, SA can still be initialized to the entires set in the Y candidate resource, the exclusion procedure then needs to be redefined. Therefore for now, we suggest rephrase the proposal as - In periodic-based partial sensing, - For periodic transmissions , the set SA is initialized to the entire set or a subset of all the candidate single-slot resources within the selected set of Y candidate resource. |
| LGE | - We support FL proposal with clarification for the main bullet. To avoid any misunderstanding that the proposal is not for resource re-evaluation or pre-emption checking, add the following text: “In periodic-based partial sensing except for resource re-evaluation and/or pre-emption checking,” We wonder if the proposed operation is different from that of LTE-V2X. If it’s the case, we need further clarification of the proposal. |
| NEC | We also think it's applicable to both periodic and aperiodic traffics. S_A is initialized to the set of all the candidate single-slot resources within the selected set of Y candidate resource is the pricinple of partial sensing and should be applied regardless of traffic type. |
| CMCC | The intention of this proposal is not clear to us. Besides, we do not think the periodic-based partial sensing should be limited to perioidic transmission only. |
| Fujitsu | Agree. |
| OPPO | Agree |
| Panasonic | We are ok with this proposal, but we don’t quite understand the motivation. It should be applied to both periodic and aperiodic transmission. |
| Huawei, HiSIlicon | We disagree “For periodic transmission,” part of the proposal. As we have already explained in our first round reply and also commented by others, how partial sensing works (and which kind is used) does not depend on the traffic type UE would transmit. The sensing procedure aims to find the resources resevations by others. Periodic based partial sensing, when the resource pool is enabled for periodic reservation, can be performed when UE has either aperodic or periodic transmission. So does the candidate resource set dermination, it is unrelated to thetraffic type. So suggest to delete the “for period transmission” in the subbullet. |
| Samsung | The motivation of this agreement is unclear for us since it seems quite similar to legacy behaviour. In addition, considering the determination of selection window/candidate resource of aperiodic transmissions may need to keep consistency with periodic transmission as much as possible, but there was no sufficient discussion on the total sensing procedure of aperiodic transmission, we think it’s too early to try this proposal now. |
| Intel | To us it is unclear how the Y slots are defined. If these slots are selected as for the set SA in clausa 8.1.4 of 38.214 we are fine with it. Otherwise we cannot agree on this without further defining how Y is selected. |
| Fraunhofer | As mentioned by other companies, it is unclear to us why the proposal is restricted to only periodic transmissions. |
| Xiaomi | We are generally fine with the proposal, but suggest the following wording revision as there may be more than Y slots selected: - In periodic-based partial sensing, - For periodic transmissions , the set SA is initialized to the set of all the candidate single-slot resources within the selected set of at least Y candidate slotsresource. In addition, it is still unclear on how UE would perform if both periodic-based partial sensing and contiguous partial sensing are used, therefore, we propose to add the FFS: FFS interaction with contiguous partial sensing, if any |
| vivo | The motivation to have this proposal is unclear. We are also not clear why this proposal is for periodic case only. |
| Ericsson | We are supportive of this proposal |
| Nokia, NSB | So far we have a very rough definition of Y candidate resource for periodic partial sensing. This limitation of set SA would be good if we have a better picture on the selection of Y candidate resource. Suggest to delay the agreement of Proposal 1-2. |
| CATT, GOHIGH | The motivation to have this proposal is unclear. Also not sure how Y slot is selected. |
| MediaTek | We think that this proposal should be applicable to both periodic amnd aperiodic transmissions. Also, Y should refer to “candidate slots” instead of “candidate resources”. We can at least suggest the following changes. - In periodic-based partial sensing, - For periodic transmissions , the set SA is initialized to the set of all the candidate single-slot resources within the selected set of Y candidate resource slots. |
| Qualcomm | We do agree with other companies that the initialization of set SA should not depend on the traffic pattern. We suggest to discuss this proposal once more details are available. |
| InterDigital | We support the proposal. In our view, periodic-based partial sensing shall be performed for periodic transmissions to avoid congituous collision between two reservation processes of two UEs. Therefore, the transmission resource for periodic transmissions shall be selected from Y candidate slots, which have periodic-baed partial sensing result. |

### Proposals for the Monday GTW session (April 19th)

FL observations and comments based on inputs received in Sec. 3.1.2:

- On Proposal 1-1 (round III), FL comments:

  - On Preserve value,

    - Some comments were related to removing Alt.3 and Alt.4 for Preserve to save evaluation effort. Since the proponent of Alt.4 commented that multiple sets of Preserve is only needed if Alt.2 or Alt.3 is adopted (i.e., when a full set is not used), Alt.4 is now merged into Alt.2 and Alt.3. For Alt.3, since there is more than 1 company that support it, my suggestion is to keep it for now and companies can provide further analysis and evaluation results to justify its benefit.

    - @ZTE: The possibility of UE monitoring just one periodic sensing occasion comes from the case when only one Preserve is configured and only a single value for k. The track change shown in the last meeting agreement was based on a last minute comment for the such possibility. It should be probably re-worded back to how it was before the change. Thanks for picking up this point.

    - @MediaTek: The intention of Alt.2 is restricted to be within the configured set sl-ResourceReservePeriodList, but not the entire possibility of the whole range as suggested. For example, if the sl-ResourceReservePeriodList includes only [100, 200ms], then it is not necessary for the UE to monitor say 57ms periodicity value. For the (pre-)configuration of a set of k values, it could be represented as a bitmap [1 0 1 1 0 0 0 0 1 0] as in LTE-V or simply just {1, 3, 4, 9}.

  - On k value,

    - Taken the suggestion from some companies that the way to (pre-)configure a set of multiple k values (including a bitmap) could be considered as part of Alt.2 for the next level of detail if Alt.2 is chosen.

  - Regarding Preserve and k in the case of periodic-based partial sensing is used for re-evaluation and pre-emption checking, it is encouraged that companies starting looking into this aspect.

  - There is also very divergent views in this meeting on whether a UE should perform sensing between the selection trigger slot and the first slot of the Y candidate resources. We should aim to resolve this issue in the next meeting as well, as this will also impact contiguous partial sensing operation.

Proposal 1-1 (round IV):

- In periodic-based partial sensing,

- For the set of Preserve values, down-select to one of the following in RAN1#105-e

    - Alt.1: Preserve corresponds to all values from the configured set sl-ResourceReservePeriodList

    - Alt.2: A set of Preserve values is (pre-)configured and includes up to the full set of values from the configured set sl-ResourceReservePeriodList

      - FFS if support multiple sets of Preserve values based on one or more metrics (e.g. priority, CBR, HARQ error rate, traffic type, etc.)

    - Alt.3: A set of Preserve values is (pre-)configured and includes common multiple(s) of values from the configured set sl-ResourceReservePeriodList

      - FFS if support multiple sets of Preserve values based on one or more metrics (e.g. priority, CBR, HARQ error rate, traffic type, etc.)

    - Alt.4: Multiple sets of Preserve values based on transmission priority

- For the k value, down-select to one of the following in RAN1#105-e

    - Alt.1: Only the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger or the first slot of the set of Y candidate slots subject to processing time restriction

    - Alt.2: k is (pre-)configured, including multiple values with at least k=1 the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger or the first slot of the set of Y candidate slots subject to processing time restriction

      - FFS how to (pre-)configure (e.g. including bitmap), whether a maximum number of k values is needed, and whether it can be up to UE implementation to select a k value from the (pre-)configuration

    - Alt.3: k is from a (pre-)configured bitmap, as in LTE-V

  - Note: companies are encouraged to provide more evaluations

  - FFS: whether sensing should be performed for periodic sensing occasions located between the resource (re)selection triggering and the first slot of the set of Y candidate slots subject to processing time restriction

| Company | Comments |
| --- | --- |
| xiaomi | For the last bullet, the issue is not on whether sensing is performed after slot n, but on whether sensing after slot n is for resource (re)selection triggered in slot n or for pre-evaluation/pre-emption decision after resource (re)selection. Therefore, we suggest to change the last FFS bullet to be : FFS: For perioidic-based partial sensing, whether the sensing results in slots after resource (re)selection triggering are considered for resource (re-)selection FL: It is my fault of not being clear. But in physical layer, the slot index (n) is used in both cases of resource (re)selection trigger and re-evaluation/pre-emption checking trigger. This FFS bullet is meant to cover only the resource (re)selection trigger case. In case of re-evaluation and pre-emption trigger in slot (n), which is at “m-T3”, the UE would not be able to performing further sensing between the triggering slot n and the set of resources indicated for re-evaluation and pre-emption checking, in my understanding. But surely, I will make this point clear in the next version, i.e., “this does not cover the case of re-evaluation and pre-emption checking”. |
| Qualcomm | - For Alt2, we suggest to remove the examples (e.g. priority, CBR, HARQ error rate, traffic type, etc.) - It is unclear how Alt3 works; we propose to remove this option. - The last FFS is not needed. This behavior is covered by re-evaluation similar to Rel. 16. FL: For the last FFS, please refer to my reply above to Xiaomi. The trigger for re-evaluation and pre-emption checking should be considered / treated separately, as it is triggered separately to the initial resource (re)selection. |
| Sharp | For Alt3 of Preserve, if the list sl-ResourceReservePeriodList includes more than one prime numbers, e.g. 7ms, 19ms, does it mean Preserve would be 7*19=133? If so, it does not make sense to us and we propose to remove Alt3. For the last sub-bullet, i.e. FFS part, we know FL’s intention, while still the wording seems a little confused to us, since in our understanding, sensing is performed periodically all the time. We propose to update as e.g. FFS: whether sensing should be performed for periodic sensing occasions located between the resource (re)selection triggering and the first slot of the set of Y candidate slots subject to processing time restriction are considered for identification of candidate resources within the set of Y candidate slots. FL: On the last FFS bullet, I am not sure the suggested modification is clearer than before, since in the spec the action related to sensing is referred as “monitoring slots”. Maybe I should change it to “whether sensing should be performed for periodic sensing occasions located between the resource (re)selection triggering and the first slot of the set of Y candidate slots subject to processing time restriction are monitored by the UE considered for identification of candidate resources within the set of Y candidate slots.” |
| NTT DOCOMO | Alt 3 for P_reserve (e.g. 30 ms is monitored for periodicities of 10 ms and 15 ms) should be removed; this option is not aligned with Alt 1/2 of k value, where most recent sensing occasion is monitored. Alt 2 for the k value should be modified as suggested in the last round. ‘The most recent sensing occasion’ should not be included in (pre-)configured values. For example, a resource pool is pre-configured with 1 ms, 5 ms and 20 ms of periodicities. In this case, k value for ‘the most recent sensing occasion’ could be k=10 for 1 ms, k=2 for 5 ms, but k=1 for 20 ms. That is, k value for ‘the most recent sensing occasion’ is not common among periodicities. If ‘the most recent sensing occasion’ is included in (pre-)configuration, quite so many values will be (pre-)configured and power saving performance is degraded significantly. I think this is not the intention. FL: It is true. How about for Alt.2: “k is (pre-)configured, including multiple values with at least k=1. If the corresponding the most recent sensing occasion for a given reservation periodicity and a k value is located after before the resource (re)selection trigger or the first slot of the set of Y candidate slots subject to processing time restriction, the most recent sensing occasion before the resource (re)selection trigger or the first slot of the set of Y candidate slots subject to processing time restriction shall be monitored by the UE.” |
| vivo | - Preseve Similar to the FFS of alt.2, we think some limitations on the size of Preseve set should also be considered. If the number of Preserve values selected for determining the periodic sensing occasions is too small, the possibility of resource collisions can be high. - For the set of Preserve values, down-select to one of the following in RAN1#105-e - Alt.1: Preserve corresponds to all values from the configured set sl-ResourceReservePeriodList - Alt.2: A set of Preserve values is (pre-)configured and includes up to the full set of values from the configured set sl-ResourceReservePeriodList - FFS if support multiple sets of Preserve values based on one or more metrics (e.g. priority, CBR, HARQ error rate, traffic type, etc.) - FFS if the size of set of the Preserve values should be no smaller than Pmin, and how to determine Pmin - Alt.3: A set of Preserve values is (pre-)configured and includes common multiple(s) of values from the configured set sl-ResourceReservePeriodList - FFS if support multiple sets of Preserve values based on one or more metrics (e.g. priority, CBR, HARQ error rate, traffic type, etc.) - FFS if the size of set of the Preserve values should be no smaller than Pmin, and how to determine Pmin - K Support FL: Based on the comments so far, most likely Alt.3 for Preserve will be removed. For Alt.2, it is already up to (pre-)configuration, I am not sure setting a minimum value adds any value. |
| NEC | We think Alt.1 and 2 covered the issue touched in the FFS bullet. In Alt.1 and 2, it mentioned "… before the first slot of the set of Y candidate slots subject to processing time restriction," this implied that sensing is preformed in "periodic sensing occasions located between the resource (re)selection triggering and the first slot of the set of Y candidate slots subject to processing time restriction " in the FFS bullet. In other words, we think the FFS bullet will be agreed if we agree " … or the first slot of the set of Y candidate slots subject to processing time restriction " in Alt.1 and 2. FL: I see your point, but I really want to emphasize this FFS (instead of just leave it within Alt.1 and Alt.2 descriptions) for companies to consider. |
| Samsung | We’re fine with the proposal. |
| Huawei, HiSilicon | We are generally fine with the proposal, except the Alt.3 for Preserve. We think the effect of Alt.3 is same as Alt.2 espectially just a subset is used.A more focused proposal on Alt.1 and Alt.2 for Preserve is preferred. For Alt. 1 and 2 of k value, the “or” should include an FFS to choose one or the other (and not an “and/or”). FL: The last FFS bullet should address your point, if I understand your meaning correctly. For Alt. 2, if there is a pre-configured list, how can it be other than up to UE implementation? For the purpose of having two clearly distinct alternatives, this “…and whether…” part should be deleted. |
| Spreadtrum | For P_reserve, Alt.3 should be removed. For k, we support the proposal. |
| LGE | Support FL proposal. |
| Ericsson | We propose the following modifications to the proposal: For the sub-bullets regarding the value of k. We propose to delete the last part of the sub-bullets “or the first slot of the set of Y candidate slots” until a better understanding of the concept of Y candidate slots within the RAN1 group. Therefore, we propose to remove the last part and the sub-bullet should look like: - Alt.1: Only the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger or the first slot of the set of Y candidate slots subject to processing time restriction - Alt.2: k is (pre-)configured, including multiple values with at least k=1 the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger or the first slot of the set of Y candidate slots subject to processing time restriction |
| Convida Wireless | We are ok with the proposal. |
| Fraunhofer | We support the FL’s proposal. |
| ZTE | OK with the understanding that other alternatives could be considered with the evaluations. |

- On Proposal 1-2 (round III) , FL summary and comments:

  - Support: Futurewek, LGE, Fujitsu, OPPO, [Intel], Xiaomi, Ericsson, IDC

  - Not support/postpone: DCM, Apple, NEC, CMCC, Panasonic, Samsung, Fraunhofer, vivo, Nokia, NSB, CATT, GOHIGH, MediaTek, Qualcomm

    - It should be applied to both periodic and aperiodic transmissions (not traffic type dependent)

    - The sensing procedure aims to detect the resources resevations by others.

    - Determination of selection window/candidate resource of aperiodic transmissions may need to keep consistency with periodic transmission as much as possible.

    - It is unclear how Y slots are fefined, too early to discuss this.

  - @supporting aperiodic transmissions, a question from FL: when a UE has only aperiodic traffic, why the set of candidate resources for SA should be limited to a selected Y candidate slots? In my understanding,

    - it provides more flexibility and more candidates for selection if the candidate slots cover the entire resource selection window [n+T1, n+T2], and thus less chance of collision probability, and

    - common understanding of packet arrival / resource selection trigger for aperiodic transmissions is not predictable, as such it is not always possible to select a set of Y candidate slots and perform corresponding pre-sensing before the triggering slot n that matches with the aperiodic packet arrival time, especially ones with short PDB.

It would be good the proponents of this type of operation could clarify these aspects until the next meeting. For now, I think at least for periodic tranmissions, there is no problem of initializing set SA within the selected Y candidate slots.

  - @Futurewei: For the case that contiguous partial sensing is performed after ty0, this would apply to re-evaluation and pre-emption checking, which we should further consider. An FFS bullet is added.

  - @LGE: The proposed operation is not intended to be different from LTE-V, but different from R16. That’s why this is proposed.

  - @Intel/Nokia/CATT: According to last meeting’s agreement, “… it is up to UE implementation to determine a set of Y candidate slots within a resource selection window, …”. Hope this clarifies. The purpose for this proposal is to start looking into the next bit of details in the overall Step 1 procedure for resource (re)selection. I think this is independent of the Preserve and k values issues.

Proposal 1-2 (round IV):

- In periodic-based partial sensing,

  - At least for periodic transmissions, the set SA is initialized to the set of all the candidate single-slot resources within the selected set of Y candidate resource slots.

  - FFS the definition of set SA in the case of re-evaluation and pre-emption checking.

- FFS if the set of SA should be restricted by the a set of Y candidate slots within the resource selection window for aperiodic transmissions in a resource pool with reservation for another TB enabled.

  - Note: companies are encouraged to provide evaluation results

| Company | Comments |
| --- | --- |
| Qualcomm | As commented in the previous round, we do not think this proposal needs to be discussed for now. It can be considered at a later stage if needed. |
| Sharp | Not support. The FL’s question seems to already imply the periodic based partial sensing does not apply for the aperiodic transmissions. In our understanding, the set SA is not related to the traffic type of the UE. |
| NTT DOCOMO | Still motivation of this proposal is unclear for us. Based on FL’s comment, I assume that the question is whether only Y candidate slots are resources in S_A for aperiodic transmissions or not. In other words, for periodic transmissions, it is straightforward. Is it correct understanding? If YES, then we think aperiodic transmissions should be covered by this proposal as well. FL mentioned that more slots should be included in S_A, but if UE wants, the UE can select more Y slots. Why does the UE select only small number as Y slots but want more slots in S_A? What is the motivation to do so? |
| NEC | Support alought we think Y candidates also applied to aperiodic traffic. SA is set as Y candidate slot resources but not [n+T1, n+T2], in our view, is for power saving purpose which is the baseline of partial sening. |
| Samsung | We think from PHY layer perspective, there is no common understanding on the definition of periodic/aperiodic transmissions. As we commented in previous rounds (sorry for repeating our view but it wasn’t reflected in updated proposals), periodic transmission is not RAN1 spec wording. Traffic type are invisible by PHY layer, and how to indicate the traffic characteristics, which may reflect traffic type, to PHY layer should be determined by RAN2. Therefore, we prefer using “UE is able to know in advance / expect a resource (re)selection will be triggered in slot n” instead of “periodic transmission” (and vice versa for aperiodic transmissions) in RAN1 agreement. |
| Huawei, HiSilicon | We disagree the proposal. We think the proposal misunderstands the logic of performing periodic based partial sensing and causes of sensing and transmission. When a UE is configured to perform periodic based sensing by MAC layer (like LTE-V procedure), it will determine a set of Y candidate resoruces up to its implementation, note this is already agreed in RAN1#104. For the transmission, not matter periodic or aperiodic, resources are selected based on the Y candidate slots. UE cannot predict the next arriving packet traffic type, nor select resources beyond Y slots based on the sensing results of periodic partial sensing. FL: In RAN1#104, the determination of Y candidate slots is after the resource (re)selection trigger at slot n and between [n+T1, n+T2]. As you said, the next arriving packet traffic type (and timing in my view) cannot be predicted in advanced, how can a UE determine a set of Y candidate slots? I think this should be clarified by the proponents of periodic-based partial sensing can be used for aperiodic transmissions. So far, only DOCOMO’s reply / questions is OK for selection of Y slots for aperiodic transmissions. But still does not explain how periodic based partial sensing could work for aperiodic transmission. Note that, this is never a problem in LTE-V, where PUE traffic is always assumed to be periodic. For the second subbullet (FFS the definition of set S_A in the case of…), we do not think the S_A is related to re-evaluation and pre-emption. S_A is determined as candidate resources set for initial selection, but re-evaluation and pre-emption uses (r0,r1,r2,…) and (r0',r1',r2',…) which indicated by MAC layer. FL: When re-evaluation and pre-emption checking is triggered by higher layer, the same set of L1 procedure is applied as in the case for resource (re)selection. The set SA is still to be determined in L1. The set of resources provided by higher layer , L1 is to determine if anyone of these resources are still within the set SA after performing step 1 ~ 7. So we suggest to have a simple and uniform proposal: - In periodic-based partial sensing, - At least for periodic transmissions, the set SA is initialized to the set of all the candidate single-slot resources within the selected set of Y candidate resource slots. - FFS the definition of set SA in the case of re-evaluation and pre-emption checking. - FFS if the set of SA should be restricted by the a set of Y candidate slots within the resource selection window for aperiodic transmissions in a resource pool with reservation for another TB enabled. - Note: companies are encouraged to provide evaluation results |
| LGE | - First bullet Support. - Second bullet Y candidate slots are only related to periodic-based partial sensing. It has no relation with contiguous partial sensing or random resource selection. Therefore, unless periodic-based partial sensing is also used for aperiodic transmission, we don’t need this FFS point. Remove the second bullet at this stage, and discuss after decision on the use of periodic-based partial sensing for aperiodic transmission. - FFS if the set of SA should be restricted by the a set of Y candidate slots within the resource selection window for aperiodic transmissions in a resource pool with reservation for another TB enabled. - Note: companies are encouraged to provide evaluation results FL: I think we have the same understanding of exactly how periodic-based partial sensing can be applied in aperiodic transmissions. Certainly if a periodic-based partial sensing is already performed by the UE for resource (re)selection of another TB, then the same set of sensing results can be reused for any other TB transmission in the same resource pool. |
| Ericsson | We do not agree with this proposal. For the first sub-bullet, as periodic-based partial sensing is performed when the UE transmits periodically or other UEs transmit periodically, we propose to delete the “at least” part. For the second FFS, we think this should not be discussed since we do not have the notion of Y candidate slots for aperiodic transmissions, and therefore, limiting the set of resources SA to the set Y makes no sense for aperiodic transmisisons. Therefore, we suggest the following proposal: - In periodic-based partial sensing, - At least for When periodic transmissions is enabled, the set SA is initialized to the set of all the candidate single-slot resources within the selected set of Y candidate resource slots. - FFS the definition of set SA in the case of re-evaluation and pre-emption checking. - FFS if the set of SA should be restricted by the a set of Y candidate slots within the resource selection window for aperiodic transmissions in a resource pool with reservation for another TB enabled. Note: companies are encouraged to provide evaluation results |
| Fraunhofer | We are not supportive of the FL’s proposal. We share concerns with other companies as to why partial sensing has to be restricted to only periodic transmissions. We agree with Huawei’s wording for this proposal. |
| ZTE | The intention of this proposal seems to leave the door open for the S_A initialization for the ap Tx cases, this could be discussed when evaluation results are available for justification. Otherwise, a unified solution to periodic and aperiodic transmission including S-A initialization is preferred. In that sense, this proposal is not needed. |

### Proposals for the Tuesday GTW session (April 20th)

FL observations and comments based on inputs received in Sec. 3.1.3:

- On Proposal 1-2 (round IV), FL comments:

  - It is clear at this stage that quite a few companies have an opinion that UE performing periodic-based partial sensing is not limited by the traffic type that it is transmitting or potentially to transmit (periodic or aperiodic). As such, the set SA is always initialized according to the selected Y candidate slots (and the resources within) after the resource (re)selection trigger.

  - On the other hand, there is also opinion that periodic-based partial sensing may not always be applicable for aperiodic type of traffic transmission since the resource (re)selection trigger timing is always unpredictable, as such the UE cannot determine the right periodic sensing occasions that is aligned with triggering timing and Y candidate slots.

  - It is observed that many proposals in this meeting could not make any progress due to this difference in opinion. It is, therefore, we should as a group further discuss on this aspect and determine whether it is technically beneficial for a UE to perform periodic-based partial sensing for aperiodic transmission and how it would be performed, before we can decide the definition for set SA.

  - FL’s strong recommendation is to go ahead with the following FFS proposal until the next meeting, otherwise I don’t see how this can be resolved.

Proposal 1-2 (round V):

- FFS how periodic-based partial sensing process can be applied when resource (re)selection is triggered for PSCCH/PSSCH transmission without enabled periodic reservation for another TB by higher layer, when the reservation for another TB (when carried in SCI) is enabled in a resource pool.

| Company | Comments |
| --- | --- |
| Apple | We are fine with the proposal. Our thinking is that periodic-based partial sensing can be applied for aperiodic traffic. The candidate resource slots resulting from periodic-based partial sensing are more reliable than other resource slots. Maybe, we could change “FFS” to “Study” in the proposal. FL: Thanks for accepting the proposal. It is greatly appreciated if the details of “periodic-based partial sensing can be applied for aperiodic traffic” is provided to the next meeting to help with discussion in RAN1 😊 |
| Nokia, NSB | Periodic-based partial sensing can be applied for aperiodic traffic. We are fine with the proposal at this stage. FL: Thanks for accepting the proposal. It is greatly appreciated if the details of “periodic-based partial sensing can be applied for aperiodic traffic” is provided to the next meeting to help with discussion in RAN1 😊 |
| NTT DOCOMO | Generally fine with the proposal. For clarification, not ‘shall’ but ‘can’ is used here, it means that UE does not mandate periodic-based partial sensing for aperiodic transmission. Is this correct understanding? Further study is how perioidic-based partial sensing ‘can’ be used or not, this is my understanding. FL: The FFS bullet and the wording is not mandating any UE behaviour (and it is not intended to), but merely describing what should be studied. So your last sentence is the right understanding. |
| CATT,GOHIGH1 | Out understanding of this proposal is this is for further study that how the aperiodic traffic can also utilize the periodic partial sensing result. We are open for this FFS. |
| InterDigital | In general, we are ok with the proposal but we haven’t agreed to support periodic-based partial sensing for aperiodic transmission yet. Thus, we can add “whether” to clarify that it is also part of study whether periodic-based partial sensing for aperiodic transmission is supported or not. FFS whether/how periodic-based partial sensing process can be applied when resource (re)selection is triggered for PSCCH/PSSCH transmission without enabled reservation for another TB. FL: Yes, we have not agreed anything regarding the traffic type for which the periodic-based partial sensing procedure should be applied to, and hence the study. At the end of this study, I expect we will determine whether its feasibility and benefits, and decide accordingly, including the suggested “whether”. |
| Futurewei | We are ok with the proposal. |
| Qualcomm | It does not seem possible to perform periodic-based partial sensing if a resource pool is not configured with reservation for another TB as there wouldn't be any reservations periodicities to monitor. In our view, however, partial sensing in a periodic set of resources in a resource pool where periodic reservations are disabled is still possible and it is covered by the following FFS from the last meeting: - Option 1: For the purpose of resource (re-)selection, the UE monitors slots between [n+TA, n+TB] and performs identification of candidate resources, in or after slot n+TB, based on all available sensing results, including periodic-based partial sensing results (if applicable). FFS TA, TB (including the possibility of equal to zero, positive or negative) and remaining details (in particular, whether there should be exclusion of slots, changes in TA/TB values for different purposes, etc.) FL: I can understand QC’s thinking is coming from a set of periodically occurring resources even for a resource pool without enabled reservation for another TB. However, the sensing operation you’ve described is not the same as the periodic-based partial sensing we agreed last time, where “the reservation for another TB (when carried in SCI) is enabled for the resource pool”. In your operation, I can understand how such sensing in periodic occurring resources would be appliable for aperiodic transmission, due to the UE is performing a “full sensing” within the periodically occurring resources and the UE is only allowed to perform transmission within those resources. But for the periodic-based partial sensing operation we agreed in the last meeting with “the reservation for another TB (when carried in SCI) is enabled for the resource pool”, my proposal is to study further how such sensing operation is able to support aperiodic transmissions without confining to a set of periodically occurring resources. |
| NEC | One concern is "aperiodic traffic" may not exactly equals "without enabled reservation for another TB", even for a resource pool with enable reservation for another TB, transmission for aperiodic traffic is also allowed, right? May be we explicitly say " FFS whether/how periodic-based partial sensing process can be applied when resource (re)selection is triggered for PSCCH/PSSCH transmission for aperiodic traffic" FL: The wording “PSCCH/PSSCH transmission without enabled reservation for another TB” is referring to aperiodic transmission, which has been commented a lot from Samsung that it should not be used. |
| Fujitsu | Generally we are fine with this proposal. Our view is that for a UE which is about to transmit aperiodic traffic, it can perform periodic-based partial sensing to avoid the interference of other UE’s periodic transmissions after resource (re)selection is triggered for PSCCH/PSSCH transmission, but it is not necessary to do this all the time. For example, it may depend on whether the remaining PDB of current TB is long enough. FL: Thanks for the further details and agreeing to the proposal. |
| Xiaomi | We are generally fine with the proposal. Below is a small suggestion: There is at least one case that periodic-based partial sensing cannot be applied for aperiodic data transmission, i.e. when perioidic reservation is not enabled in a resource pool. Therefore, we also suggest to add “whether” into the FFS: - FFS whether/how periodic-based partial sensing process can be applied when resource (re)selection is triggered for PSCCH/PSSCH transmission without enabled reservation for another TB. FL: Please refer to my reply to IDC above. I think the case you are referring to is already a common understanding from the discussion. |
| Huawei, HiSilicon | Basically fine with the proposal. Our understanding is that perioidic based partial sensing is applicable for both periodic and aperiodic traffic, because the sensing procdure is used for detecting revervation by others, which is not relaed to traffic type. FL: Thanks for accepting this FFS. |
| Ericsson | We are in general fine with this proposal. However, we would like to reiterate that the use of periodic partial sensing has to be linked with the use of periodic reservations (by other UEs). FL: Sure, I see this is the next level of details of how it would operate. |
| Panasonic | Support |
| Samsung | Try to agree on a total FFS proposal seems not like substantial progress, but since it can be considered as guideline for next meeting’s discussion, we can accept the proposal. Some modifications are suggested as follows for further clarification: FFS how periodic-based partial sensing process can be applied when resource (re)selection is triggered for PSCCH/PSSCH transmission without enabled periodic reservation for another TB by higher layer. FL: Implemented |
| LGE | Periodic-based partial sensing is only needed in a resource pool where periodic transmission is enabled. FL proposal is too generic in that it could mislead as periodic-based partial sensing is possible even for a resource pool where periodic transmission is not enabled. We suggest to add the following clarification. - FFS how periodic-based partial sensing process can be applied when resource (re)selection is triggered for PSCCH/PSSCH transmission without enabled reservation for another TB, when the reservation for another TB (when carried in SCI) is enabled in a resource pool. FL: Implemented. I think this would also address the comment from QC. |
| Convida Wireless | We are generally fine with the proposal. |

### Final proposals for Tuesday GTW session (April 20th)

Proposal 1-2 (Topic #1):

- FFS how periodic-based partial sensing process can be applied when resource (re)selection is triggered for PSCCH/PSSCH transmission without enabled periodic reservation for another TB by higher layer, when the reservation for another TB (when carried in SCI) is enabled in a resource pool.

## Topic #2: Periodic-based partial sensing – conditions/cases, sensing trigger timing

Background: In LTE-V, periodic transmission was always assumed and as such the LTE-V partial sensing mechanism was based on a TX UE monitoring PSCCH transmissions in subframes that correspond to a set of Y selected candidate subframes by a (pre-)configured reservation periodicity (Pstep) and occurrences (k). To mimic this behaviour so to avoid collisions with period transmissions, periodic-based partial sensing (PBPS) that based on LTE-V’s partial sensing mechanism was agreed in the last meeting. However, from reviewing the submitted contributions, it is unclear whether this PBPS can work well for aperiodic transmission which has an unpredictable triggering / packet arrival time. Without knowing some of transmission parameters such as the triggering slot n and remaining PDB of an aperiodic transmission, the UE would not be able to obtain per-sensing results that matches / useable for the resource selection of the aperiodic transmission. From contributions, the similar can be said for the initial selection or re-selection of a periodic transmission when the triggering slot n, transmission/reservation periodicity and remaining PDB are not available in advance. If UE performs PBPS assuming certain transmission parameters (without an actual data TB), it is raised the power consumed by constant sensing is wasteful when UE does not a data to transmit, a long gap between transmissions, or the sensing results do not match / useable for the actual transmission. On the other hand, it is also raised in some contributions that PBPS should be always performed in a resource pool that supports periodic reservation and partial sensing. Please refer to Section 4.1 for the collected summary.

Based on the above summary and usage of partial sensing in LTE-V, a Proposal 2 is provided in Section 3.2.1 to collect initial company views.

### Proposals before 1st check point (April 15th)

Proposal 2:

- Periodic-based partial sensing is performed by UE in a mode 2 Tx resource pool provided by higher layer when:

  - Periodic reservation for another TB (sl-MultiReserveResource) is enabled for the resource pool

  - The resource pool is (pre-)configured to enable partial sensing

  - UE is triggered to perform resource (re)selection procedure for periodic transmission (i.e., resource reservation interval Prsvp_TX≠0)

    - FFS the case when UE is triggered to perform resource (re)selection procedure for aperiodic transmission

  - UE is configured to perform partial sensing

- Periodic-based partial sensing is also performed by UE for the purpose of re-evaluation and pre-emption checking (if pre-emption is enabled for the resource pool).

- FFS whether UE performing periodic-based partial sensing is further conditioned by one or more of the followings

  - When the priority value of a packet is above a threshold

  - When the congestion/interference level in a resource pool is above a threshold

  - When the required reliability level of a packet transmission is above a threshold

  - When the number of received HARQ NACKs of a TB is above a threshold

  - When the number of retransmissions of a packet is below a threshold

- FFS whether a TX UE is able to know in advance / expect a resource (re)selection will be triggered in slot n

- FFS whether need to resolve the issue of periodic-based partial sensing without sufficient sensing results

| Company | Comments |
| --- | --- |
| NTT DOCOMO | Support 1st bullet. On 2nd bullet, as pointed out in our contribution [29/DCM], just reusing periodic-based partial sensing for re-evaluation/pre-emption check is quite problematic from power saving perspective. One FFS should be added as ‘FFS: details of periodic-based partial sensing for the purpose of re-evaluation and pre-emption checking’. On 4th bullet, the intention is unclear for us. For selection, n is known in advance for periodic transmission while not for aperiodic transmission. For reselection, n is known after re-evaluation/pre-emption check is triggered. I guess this is common understanding. Or different view is raised? |
| Xiaomi | We are generally fine with the FL proposal. |
| OPPO | - 1st main bullet: - 1st sub-bullet: OK - 2nd sub-bullet: OK - 3rd sub-bullet: OK with modification. In our view, if UE performs periodic transmission, it should do periodic-based partial sensing so that it can do resource selection triggered at slot n based on partial sensing results. There are several criterions defined in RAN2 spec which can trigger PHY layer to report resource set at slot n. There is an FFS (second last bullet) to discuss whether the slot n is predicable or not. In some cases, it is predicable, such as “SL RESOURCE RESELECTION COUTNER” reached zero, in some cases, it is not predicable, such as the resource cannot afford the data packet to be transmitted. While considering that the slot n is predicable for periodic traffic in former case, UE should do periodic based partial sensing so as to do resource selection based on partial sensing results at slot n. Therefore, we prefer the support the 3rd bullet with following modification: - UE is triggered to performs resource (re)selection procedure for periodic transmission (i.e., resource reservation interval Prsvp_TX≠0) - FFS the case when UE is triggered to perform resource (re)selection procedure for aperiodic transmission - FFS whether a TX UE is able to know in advance / expect a resource (re)selection will be triggered in slot n - 4th sub-bullet: OK with modification According to description of partial sensing procedure in TS36.213, “If partial sensing is configured by higher layers then the following steps are used:” we generally OK with the proposal with following modification: - UE is configured to perform partial sensing by higher layers - 2nd main bullet: No It is a little early to discuss periodic based partial sensing combined with re-evaluation/pre-emption. We can delay the discussion when the mechanism of periodic-based partial sensing is clear enough. - 3rd main bullet: same comment as above. Low priority for the FFS points. - 4th main bullet: moved into 1st main bullet. - 5th main bullet: No necessary to address this. It is corner case. We also did not discuss it during R16 full sensing. |
| vivo | Support this proposal in general. Regarding the conditions, whether PDB and/or latency requirement is above a threshold can also be considered. For example, if the service is urgent or the corresponding PDB/remaining PDB is small, UE can use random selection, while partial sensing should be used if the service is not urgent or the PDB/remaining PDB is large. - When the priority value of a packet is above a threshold - When the congestion/interference level in a resource pool is above a threshold - When the required reliability level of a packet transmission is above a threshold - When the number of received HARQ NACKs of a TB is above a threshold - When the number of retransmissions of a packet is below a threshold - When the PDB/remaining PDB/latency requirement is above a threshold |
| Intel | 1st main bullet: - 1st sub-bullet: Support - 2nd sub-bullet: Support - 3rd sub-bullet: Support except the 1st period of periodic transmission - 4th sub-bullet: Support 2nd main bullet: Needs to be discussed whether it is for all periods or only when reselection is expected 3rd main bullet: OK for FFS 4th main bullet: Propose to remove FFS 5th main bullet: OK for FFS |
| CMCC | For the 1st main bullet, does it intend to say that periodic-based partial sensing is applied when all 4 listed conditions are met? If so, we have concerns on the 3rd sub-bullet, to our understanding, the resource allocation mechanism has nothing to do with the traffic type to be transmitted by the Tx UE. For the 2nd main bullet, we think at this stage, this should be FFS. For the 3rd main bullet, no objections to be discussed, however, we believe it makes more sense to introduce these conditions for random resource selection. |
| Fujitsu | We are OK for the 1st and the 2nd bullet. We are open for the FFSs in the 3rd and 4th bullet. For the 5th bullet, we are not quite clear about the case and the intention of this FFS point, we think more clarifications are needed. |
| Panasonic | We are ok with FL’s proposal. |
| LGE | A ‘burst-type’ resource selection should be supported for power saving if UE performs resource re-evaluation/pre-emption checking. The burst-type resource selection shortens the required overall sensing duration thereby the power consumption for the required partial sensing. The burst-type resource selection can be achieved e.g. by limiting the max. distance between any two selected resources signalled by a SCI, to a (pre-)configured value less than 31 logical slots. We propose to add the following. - The max. distance or interval between the resources indicated by a SCI is (pre-)configured as a value less than 31 logical slots - FFS details on the (pre-)configured value |
| Samsung | 1st main bullet: Support 1st, 2nd and 4th sub-bullets. For 3rd sub-bullet, we consider the condition restricts feasible scenarios of periodic-based partial sensing, thus we feel negative to simply agree on the current wording. 2nd main bullet: generally fine, but we prefer to make consensus on how re-evaluation/pre-emption can be enabled and disabled for power saving mode at first, then go back to this issue. 3rd main bullet: fine to study it, but we consider the conditions seems more related to how to determine/switch between resource allocation schemes, and we prefer using a separate issue involving all RA schemes as high level structure. In addition, we prefer the following changes: - Partial sensing can be used if above or below threshold for each metric. For example, one side UE can use full sensing on the other side UE can random selection. - We prefer to reword item four as: “When the number of received HARQ NACKs error rate of a TB is above a threshold” - Item 5 (number of retransmissions) can be removed. If HARQ-ACK feedback is enabled, number of retransmission is linked to HARQ error rate. 4th and 5th main bullets: Fine to study the issues. In addition, we suggest to change “whether” to “Whether/how” in 4th main bullet. |
| NEC | - 1st bullet: Agree - 2nd bullet: We're not sure if re-evaluation and pre-emption are enable in the pool, what if one of the condition are not met in the first bullet? Could the PBPS be performed? - 3rd bullet: either fine to keep it or delete it - 4th and 5th bullet: What's the background of these two FFS, clarification are appreciated |
| Lenovo, Motorola Mobility | Agree with following considerations. For ‘FFS whether a TX UE is able to know in advance / expect a resource (re)selection will be triggered in slot n’, we need to assume that the TX UE is able to know in advance / expect a resource (re)selection will be triggered in slot n. If not, how to select the resource for initial transmission will be discussed again. It is necessary to discuss that ’FFS whether need to resolve the issue of periodic-based partial sensing without sufficient sensing results’. This issue may be caused by the power sensing UE determines an unsuitable Y value within resource selection window, and detected available resource based on the corresponding sensing occasion is not enough. |
| Spreadtrum | We are fine with this proposal. |
| Ericsson | We think that further discussion around this proposal and some clarifications are still necessary. - We see several issues regarding bullet number 3 (“UE is triggered to perform …”): - In our view, a UE should perform periodic-based partial sensing whenever there is a possibility that other UEs are performing periodic transmissions. Independently of whether its own traffic is periodic or not. - Note also that if a UE does not have a packet to transmit, it would not be able to perform periodic-based partial sensing. So partial sensing cannot start until a packet arrives. As a general rule, that should not be acceptable. - Regarding the further conditions (third main bullet) to perform periodic-based sensing, we would like to know how these conditions are applied, are these conditions included as an extra condition to the ones defined in the first bullet? - We would like to ask for clarification regarding the intention of this FFS: FFS whether a TX UE is able to know in advance / expect a resource (re)selection will be triggered in slot n - For the last FFS, it is not clear for us what “without sufficient sensing results” means and which related issue needs to be solved. |
| Huawei, HiSilicon | - For the first main bullet, it is not clear whether all 4 subbullets should be supported then UE would perform periodic based partial sensing. In our understanding, once condition 1, 2 and 4 are satisfied, UE would perform the periodic based partial sensing. For each condition, we have analysis as following: - 1 and 2: these are agreed already last time as cases for the applicability of the agreement defining periodic sensing occasions. - 3rd Disagree to have this subbullet. This either implies non-causal operation, or embeds an assumption that UE will be expected to predict the future traffic based on side information. There is no agreement to have or allow the use of such side information, and hence this operation cannot be supported at this time. - 4th bullet - based on our understanding, no RRC signalling is needed, so the subbullet should be clarified accordingly. - For the second bullet: is not needed currently. We prefer to finish the resource selection procedure for initial selection, then come back to the revaluation and pre-emption checking. Based on Rel-16 procedure, a short sensing window is applied just before the selected/pre-empted resource. Therefore, no need to repeat the procedure of periodic based sensing. - For the third bullet: We see none of them is necessary and this extensive list implies too much standards workload. If a resource pool enables periodic reservation, each of these conditions would degrade the partial-sensing performance and cause collisions, meaning there is no certainty of power saving overall. - For the fourth bullet: This is a duplication of the Prsvp_TX ≠ 0 condition attempting to require the UE to predict the future, and should be removed. If it is limited to periodic traffic only, then it is the baseline LTE operation and does not need re-agreeing. - For the fifth bullet: it is not clear what does “without sufficient sensing results” mean? We think the case for “no sufficient sensing results” occur the SL DRX is configured and sensing occasions locate within the inactive time. That why we think it should be supported to perform sensing on those occasion within inactive time. We see it is under discussing in proposal 7, so no FFS is needed here. |
| CATT, GOHIGH | We are generally fine with the first three proposals. Regarding the 4th bullet, from our understanding, in case of periodic-based partial sensing for periodic traffic transmission, the packet arrival time should be known in advance. Regarding the 5th bullet, we think this issue has been existed in LTE-V2X, in case of insufficient sensing result, it can be left for UE implementation by either one-shot transmission or drop the transmission. |
| Fraunhofer | We are supportive of the FL’s proposal. |
| Apple | For the first bullet: do all 4 conditions (sub-bullets) satisfied or either of 4 conditions satisfied to allow UE to perform periodic-based partial sensing? For the FFS point, we think if a resource pool configuration supports both periodic and aperiodic transmissions, a UE may still perform periodic-based partial sensing to detect other UE’s periodic resource reservation, even if the UE has aperiodic traffic. Hence, we think the third sub-bullet may not be needed. For the second bullet: we are supportive in general. Maybe the triggering conditions of periodic-based partial sensing for re-evaluation and pre-emption should be discussed as well, or we could leave it FFS. For the third bullet: we do not see the difference between the first sub-bullet and the third sub-bullet. |
| InterDigital | 1st bullet: - Support the 1st, 2nd, and 4th sub-bullets. - For the 3rd sub-bullet: We are generally ok with the intention. However, for periodic transmission, when the UE triggered to perform resource (re)selection in slot n, periodic-based partial sensing should be performed up to 1000ms before that. Therefore, higher layer should trigger periodic-based partial sensing for periodic transmission before n. We propose to modify as follow: - UE is triggered to perform periodic-based partial sensing resource (re)selection procedure for periodic transmission (i.e., resource reservation interval Prsvp_TX≠0) - FFS the case when UE is triggered to perform resource (re)selection procedure for aperiodic transmission 2nd bullet: We need to further discuss about sensing for re-evaluation and pre-emption. Sensing for re-evaluation happens after slot n, it cannot be the same as sensing for resource selection. For pre-emption checking, the UE may not need to perform sensing in the whole resource pool instead it should monitor in the reserved subchannels since semi-persistent reservation allows the UE to reserve the resource in the same frequency at future time. In addition, requiring the UE to perform pre-emption in every period may result in more power consumption, instead pre-emption can be used in some condition or a subset of reservation intervals. We propose to modify the bullet as follow: - Periodic-based partial sensing is also performed by UE for the purpose of re-evaluation and pre-emption checking (if pre-emption is enabled for the resource pool). - FFS when to perform resource re-evaluation and pre-emption (e.g., triggering conditions) 3rd bullet: Support 4th bullet: Support 5th bullet: Support |
| Qualcomm | - Partial sensing with repeated occasions can be used even if periodic reservations for another TB is not enabled in a given resource pool. - If periodic sensing is dependent on whether a UE is triggered to perform periodic reservations, then, sensing could only be performed after the resource selection for periodic reservations is triggered. By the time the UE has sufficient sensing information, it is highly likely that the PDB of the first packet has been expired. This increases latency and packet dropping specially for small PDB values. In general, we do not think that a dynamic triggering event for performing periodic partial sensing is desirable. - On “UE is configured to perform partial sensing”, it is not clear which entity configures the UE. Could you elaborate? - On “Periodic-based partial sensing is also performed by UE for the purpose of re-evaluation and pre-emption checking”, we think some clarification is needed. In our view, partial sensing is performed by a UE in a periodic set of resources. The transmission and reception for the same UE also take place over the same set of periodic resources. Hence, the sensing information acquired during such occasions are used for the purpose of pre-emption/re-evaluation checking. The purpose of the last two FFS points is unclear. Could you please elaborate? |
| Futurewei | 1st main bullet: For the UE with periodic traffic, when the traffic first arrives, the UE cannot schedule and perform the periodic based partial sensing. For the initial transmission of periodic traffic, contiguous partial sensing can be performed instead. Also some conditions as in the 3rd main bullet can be applied. Current wording seems preventing UE from not performing sensing. More discussions and clarifications are needed. 2nd main bullet: We do not support this proposal. The periodic-based partial sensing is for selecting the resource from a resource pool of Y slots. Mixing the periodic based sensing,or partial sensing in general, and re-evaluation/pre-emption will cause a lot of issues on determining the timing/procedures for periodic based partial sensing which are in discussions in other proposals. 3rd main bullet: Since the first main bullet is still in discussion, we suggest remove “further” in this main bullet. For 2, congestion/interference level can be indicated by channel occupancy rate (CR). Also, the sensing can be conditioned whether the interference level is below a threshold. Therefore, we suggest rephase it to make it - When the congestion/interference level such as CR in a resource pool is above a threshold and/or below another threshold Since the available resource ratio after a sensing process can also be a metric to set a condition for periodic sensing, we propose add - When the available resource ratio in a resource selection window from a sensing process is above a threshold and/or below another threshold 4th main bullet: We are not clear on the purpose of this proposal. If it is for the initial transmission of a periodic traffic, it is then better to discuss it with 1st main bullet. 5th main bullet: Details on “without sufficient sensing results” are needed before discussing the item |
| Convida Wireless | We are Ok with the proposal. |
| MediaTek | We are generally OK with this proposal. We support bullets 1, 3, 4. For bullet-2, we are not clear. In Rel-16, sensing for resource (re)-selection happens before ‘n’ whereas re-evaluation/pre-emption measurements happen after ‘n’. For bullet-5, we are not clear what ‘without sufficient sensing results’ means |
| Nokia, NSB | The first main bullet is agreeable with some minor modifications. Of the 4 conditions (sub-bullets of the 1st main bullet), if periodic reservation for another TB is enabled, the resource pool is configured for partial sensing, AND the UE is configured to perform partial sensing, the periodic-based partial sensing shall be performed. The 3rd sub-bullet (UE is triggered to perform resource re-selection) is another condition, parrell to the 1st sub-bullet. There is no need to agree other main bullets at this stage. |
| Bosch | For the 1st main bullet, we are fine with the 1st, 3rd, and 4th sub-bullets. However, for the 2nd bullet it may be FFS or at least we add FFS backward compatibility to Rel-16. We support 2nd main bullet. We don’t support 3rd bullet. We have a major concern if periodic resources is conditioned by priority (where we see all priorities can be used for periodic resources, at least for automotive use cases). Also other conditions are not supported. We support 4th main bullet; however without “FFS whether”. We support 5th main bullet is being rephrased to: “FFS how to resolve the issue of periodic-based partial sensing without sufficient sensing results”. |
| ZTE | The first FFS part should be removed. Following LTE and Rel-16 logic, when to do sensing is up to MAC layer triggering, so the other conditions can be discussed only if there is an obvious benefit justified by performance evaluations For the second FFS part, we are confused about the scenario for a TX UE to know in advance/expect a resource selection for initial transmission. With the belief that a unified sensing solution should be defined for both initial and non-initial resource selection, we propose to remove this FFS. |

### Proposals before 2nd check point (April 19th)

FL observations and comments based on inputs received in Sec. 3.2.1:

- 1st bullet:

  - 3rd sub-bullet, to our understanding, the resource allocation mechanism has nothing to do with the traffic type to be transmitted by the Tx UE [CMCC]

  - UE should perform periodic-based partial sensing whenever there is a possibility that other UEs are performing periodic transmissions. Independently of whether its own traffic is periodic or not. [Ericsson]

  - Note also that if a UE does not have a packet to transmit, it would not be able to perform periodic-based partial sensing. So partial sensing cannot start until a packet arrives. As a general rule, that should not be acceptable. [Ericsson]

  - This either implies non-causal operation, or embeds an assumption that UE will be expected to predict the future traffic based on side information. There is no agreement to have or allow the use of such side information, and hence this operation cannot be supported at this time. [HW, HiSi]

  - If a resource pool configuration supports both periodic and aperiodic transmissions, a UE may still perform periodic-based partial sensing to detect other UE’s periodic resource reservation, even if the UE has aperiodic traffic.  Hence, we think the third sub-bullet may not be needed. [Apple]

  - Partial sensing with repeated occasions can be used even if periodic reservations for another TB is not enabled in a given resource pool. [QC]

  - If periodic sensing is dependent on whether a UE is triggered to perform periodic reservations, then, sensing could only be performed after the resource selection for periodic reservations is triggered. By the time the UE has sufficient sensing information, it is highly likely that the PDB of the first packet has been expired. This increases latency and packet dropping specially for small PDB values. In general, we do not think that a dynamic triggering event for performing periodic partial sensing is desirable. [QC]

  - FL: Based on reviewing contributions and comments received on this topics, it is clear there is no comment understanding of whether periodic-based partial sensing should be performed

    - only when there is packet to transmit or all the time (even without any packet to transmit) as this can have significant power saving impact, and

    - only for periodic transmission or both periodic/aperiodic transmissions as the triggering of aperiodic transmission is unpredictable.

Furthermore, if periodic-based partial sensing is performed only when there is packet to transmit, then how to deal with the case when UE has insufficient sensing results. It is mentioned when the reselection counter reaches zero, the UE is able to do per-sensing before the re-selection trigger. But how about other cases such as the initial selection trigger re-selection triggered by other conditions (e.g., re-evaluation, pre-emption).

At this stage, I think at least we can agree that periodic-based partial sensing is performed for periodic transmissions. It is suggested to further study on other aspects until the next meeting.

- 2nd bullet (re-evaluation and pre-emption checking):

  - Too early to discuss, should wait until the mechanism of periodic-based partial sensing is clear x 3

  - Prefer to make consensus on how re-evaluation/pre-emption can be enabled and disabled for power saving mode at first, then come back to this issue.

  - We're not sure if re-evaluation and pre-emption are enable in the pool, what if one of the condition are not met in the first bullet? Could the PBPS be performed?

    - FL: the answer is NO

  - The transmission and reception for the same UE also take place over the same set of periodic resources. Hence, the sensing information acquired during such occasions are used for the purpose of pre-emption/re-evaluation checking. [QC]

  - In Rel-16, sensing for resource (re)-selection happens before ‘n’ whereas re-evaluation/pre-emption measurements happen after ‘n’. [MTK]

  - FL: I think the current general framework for periodic-based partial sensing is sufficient enough to make a decision that it should also be used for re-evaluation and pre-emption checking purposes. Other conditions and timings of when to perform re-evaluation and pre-emption checking can be decided later.

- 3rd bullet:

  - Low priority for now

  - How these conditions are applied, are these conditions included as an extra conditions to the ones defined in the first bullet?

    - FL: Yes, that’s the intention

  - We see none of them is necessary and this extensive list implies too much standards workload. If a resource pool enables periodic reservation, each of these conditions would degrade the partial-sensing performance and cause collisions, meaning there is no certainty of power saving overall. [HW, HiSi]

  - FL: This is now separately listed in Proposal 2-2 for further study.

- 4th bullet:

  - This is a duplication of the Prsvp_TX ≠ 0 condition attempting to require the UE to predict the future, and should be removed. If it is limited to periodic traffic only, then it is the baseline LTE operation and does not need re-agreeing. [HW, HiSi]

  - In case of periodic-based partial sensing for periodic traffic transmission, the packet arrival time should be known in advance. [CATT]

  - FL: This has now merged into the first main bullet.

- 5th bullet:

  - It is corner case. We also did not discuss it during R16 full sensing.

  - This issue has been existed in LTE-V2X, in case of insufficient sensing result, it can be left for UE implementation by either one-shot transmission or drop the transmission.

  - FL: This has now merged into the first main bullet.

- General comments:

  - The burst-type resource selection can be achieved e.g. by limiting the max. distance between any two selected resources signalled by a SCI, to a (pre-)configured value less than 31 logical slots.

    - FL: this topic only deals with conditions and triggering in which periodic-based partial sensing should be performed and related issues

Proposal 2-1 (round II):

- Periodic-based partial sensing is performed by UE in a mode 2 Tx resource pool provided by higher layer when the followings are met:

  - Periodic reservation for another TB (sl-MultiReserveResource) is enabled for the resource pool

  - The resource pool is (pre-)configured to enable partial sensing

  - UE is triggered to performs resource (re)selection procedure for periodic transmission (i.e., resource reservation interval Prsvp_TX≠0)

    - FFS the case when UE is triggered to perform resource (re)selection procedure for aperiodic transmission

    - FFS how to deal with the initial period(s) in periodic/aperiodic transmission where the UE have insufficient sensing results, e.g.,

      - whether a TX UE is able to know in advance / expect a resource (re)selection will be triggered in slot n for early sensing,

      - performing random resource selection or contiguous partial sensing,

      - dropping transmission(s),

      - up to UE implementation to handle this case, or

      - etc

  - UE is configured to perform partial sensing by higher layer

| Company | Comments |
| --- | --- |
| NTT DOCOMO | Generally fine with this proposal but comment on the 2nd FFS. We do not think RAN1 needs to consider initial period of periodic traffic. UE waits to complete periodic-based partial sensing, and then starts periodic transmission. This is LTE-SL as well and no specific mechanism for initial period is introduced in LTE-SL, this is my understanding. It is unclear for us why suddenly NR-SL needs to consider this aspect. In addition, the 2nd FFS is discussing not the condition but detailed behaviour in some sitautions. This is not related to main bullet. Therefore, the 2nd FFS (all parts) should be removed. |
| Apple | When a UE is configured to perform partial sensing by higher layer, UE may only perform contiguous partial sensing, and not periodic-based partial sensing (even if UE has periodic transmission). This could save UE’s power by avoiding periodic-based partial sensing. Could we take this use case into account? If so, could we modify the last sub-bullet to UE is configured to perform periodic-based partial sensing by higher layer |
| Futurewei | As inidicated in proposal 2-3, other conditions that enable/disable periodic-based partial sensing will be discussed. Then the conditions listed in this proposal are necessary conditions, not the sufficient conditions. Also, when periodic traffic is arrive, the periodic-based partial sensing cannot be applied for the initial transmission. Therefore, we propose to rephrase “when the followings are met” in the main bullet as - Periodic-based partial sensing is performed by UE in a mode 2 Tx resource pool provided by higher layer when at least the followings are met: |
| LGE | - Main bullet Two conditions are needed for resource pool in the main bullet. The original 2nd sub-bullet should be the condition for UE to perform periodic-based partial sensing in the main bullet. The 1st sub-bullet should be the condition for UE to perform the original 3rd sub-bullet. Based on the texts from the existing agreements to avoid any misunderstanding, we suggest the following modification. “Periodic-based partial sensing is performed by UE in a mode 2 Tx resource pool provided by higher layer In a resource pool (pre-)configured with at least partial sensing, at least when the reservation for another TB (when carried in SCI) is enabled for the resource pool and resource selection/reselection is triggered at slot n, UE performs periodic-based partia sensing at least when the followings are met:” - First sub-bullet We prefer the original text so as to be aligned with the sub-bullet for the aperiodic case. “UE is triggered to performs resource (re)selection procedure for periodic transmission (i.e., resource reservation interval Prsvp_TX≠0)” - Second sub-bullet of the first sub-bullet According to Rel.16 NR-V2X, random selection on sl-TxPoolExceptional is used when the sensing results are not available for sl-TxPoolSelectedNormal. We don’t see any motivation not to follow the existing Rel.16 NR-V2X rule. We prefer to remove the sub-bullet including all FFS. - FFS how to deal with the initial period(s) in periodic/aperiodic transmission where the UE have insufficient sensing results, e.g., - whether a TX UE is able to know in advance / expect a resource (re)selection will be triggered in slot n for early sensing, - performing random resource selection or contiguous partial sensing, - dropping transmission(s), - up to UE implementation to handle this case, or - etc |
| NEC | Agree if the following understanding is the intention. "UE is triggered to performs resource (re)selection procedure for periodic transmission (i.e., resource reservation interval Prsvp_TX≠0)" means UE has periodic traffic, is this understand correct? We assume one UE may both have periodic traffic and aperiodic traffic, and as long as UE has periodic traffic to send, this condition is met. |
| CMCC | The 1st sub-bullet makes no sense to us. As commented in the previous round, to our understanding, the periodic-based partial sensing is to exclude periodic resource reservations by other UEs, but has nothing to do with the traffic type of the Tx UE. |
| Fujitsu | Agree. For more clarifications, the original 1st sub-bullet should be brought back, since UE can only do periodic-based partial sensing when the periodic reservation is enabled by the resource pool configuration. |
| OPPO | Fine with the proposal |
| Panasonic | We are ok with FL’s proposal. |
| Huawei, HiSilicon | We disagree this proposal. For the first sub-bullet (“UE performs periodic transmission…”), we can understand FL’s intention that periodic based partial sensing is performed when UE has periodic traffic to transmit, however, it is wrongly captured periodic transmission as a condition to perform periodic based partial sensing. Just as we explained in the first round, it either implies non-causal operation, or embeds an assumption that UE will be expected to predict the future traffic based on side information. There is no agreement to have or allow the use of such side information, and hence this operation cannot be supported at this time. For the second square sub bullet (“FFS how to deal with…”), it is not clear the exact meaning of “insufficient sensing results” , if it is due to SL configuration which all or part of sensing occasions locate within the SL inative time, it can be discussed in proposal 7 although it is deferred to future meeting. If it is similar definition as Rel-16, i.e. specified in TS 38.331 (clause 5.8.8), the situation of sensing results is not avaliable, we think Rel-16 procedure can be reused and no principles need to be introduced. For the last sub-bullet (“UE is configured to…”), we think FL mentions same thing in LTE-V which RA scheme is configured by MAC layer, if so, it is better to make the proposal clearly. Therefore, we suggest changes for the proposal: - Periodic-based partial sensing is performed by UE in a mode 2 Tx resource pool provided by higher layer when the followings are met: - Periodic reservation for another TB (sl-MultiReserveResource) is enabled for the resource pool - The resource pool is (pre-)configured to enable partial sensing - UE is triggered to performs resource (re)selection procedure for periodic transmission (i.e., resource reservation interval Prsvp_TX≠0) - FFS the case when UE is triggered to perform resource (re)selection procedure for aperiodic transmission - FFS how to deal with the initial period(s) in periodic/aperiodic transmission where the UE have insufficient sensing results, e.g., - whether a TX UE is able to know in advance / expect a resource (re)selection will be triggered in slot n for early sensing, - performing random resource selection or contiguous partial sensing, - dropping transmission(s), - up to UE implementation to handle this case, or - etc - UE is configured to perform partial sensing by MAC layer. |
| Samsung | Not support. Original 1st sub-bullet about resource pool configuration: the motivation of removing it from proposal 2-1 is unclear to us. In our understanding, it is straightforward that only when inter-period reservation via SCI is supported in a resource pool, periodic-based resource reservation is efficient. Therefore, we think original 1st sub-bullet should be a mandatory condition. Original 3rd sub-bullet about periodic transmission: As we commented in 1st round, periodic transmission is not RAN1 spec wording, and it’s inaccurate to define periodic/aperiodic transmissions only according to the value of Prsvp_TX. The periodic/aperiodic transmissions are invisible by PHY layer, and how to provide the traffic characteristics or traffic type should be determined by RAN2, but current proposal 2-1 seems out of RAN1 scope and may restrict subsequent RAN2 discussions. In addition, even for periodic traffic, periodic-based transmission may not be feasible for initial period and the period of resource reselection. Therefore, it’s a risky and inappropriate way to determine sensing type depending on traffic type. In our understanding, the condition of periodic transmission mainly target to describe the feasibility of periodic-based sensing since the sensing window happends before trigger slot n. Therefore, we can modify the wording as: - Periodic-based partial sensing is performed by UE in a mode 2 Tx resource pool provided by higher layer when the followings are met: - … - UE is triggered to performs resource (re)selection procedure for periodic transmission (i.e., resource reservation interval Prsvp_TX≠0) - UE is able to know in advance / expect a resource (re)selection will be triggered in slot n. FFS details including how UE is able to know in advance/expect it. |
| Intel | As the first and second subbullet are deleted we think the condition that the resource pool has periodic reservation and partial sensing enabled need to be included into the main bullet as otherwise this applies in general. We are fine with the other bullets. |
| Fraunhofer | We agree in general with the FL’s proposal with the following comments: In the main bullet, a simple correction – “when the following conditions are met”. In the second FFS, another way for a UE to carry out transmissions with insufficient sensing results is by using inter-UE coordination. An assistance message can be used by the UE for its resource selection process. We would also prefer to bring back the deleted first and second sub-bullets since it brings clarity on the resource pool configurations that would allow a UE to carry out periodic-based partial sensing. |
| Xiaomi | We support the proposal with the original wording of the following sub-bullet: - UE is triggered to performs resource (re)selection procedure for periodic transmission (i.e., resource reservation interval Prsvp_TX≠0) In addition, we also do not think RAN1 needs to consider initial period of periodic traffic. But we can accept the FFS. |
| vivo | The motivation of the second FFS on how to handle the very initial TX is not clear, even in R16 and LTE V, there is no specific description on UE behavior for such case. We suggest removing the FFS together with its subbullets. |
| Ericsson | We are not supportive of this proposal for several reasons: - First, we think that the sub-bullet “The resource pool is (pre-)configured to enable partial sensing” should be included in the proposal. - For the bullet “UE is triggered to performs resource (re)selection procedure for periodic transmission (i.e., resource reservation interval Prsvp_TX≠0) - FFS the case when UE is triggered to perform resource (re)selection procedure for aperiodic transmission” - In our view, a UE should perform periodic-based partial sensing whenever there is a possibility that other UEs are performing periodic transmissions. Independently of whether its own traffic is periodic or not. - Note also that if a UE does not have a packet to transmit, it would not be able to perform periodic-based partial sensing. So partial sensing would not start until a packet arrives. As a general rule, that should not be acceptable - Moreover, the issues regarding “insufficient sensing results” are not sufficiently clear in our view and we would prefer something more general such as: - FFS whether and how to deal with the initial period(s) in periodic/aperiodic transmission where the UE have insufficient sensing results, e.g., - whether a TX UE is able to know in advance / expect a resource (re)selection will be triggered in slot n for early sensing, - performing random resource selection or contiguous partial sensing, - dropping transmission(s), - up to UE implementation to handle this case, or - etc |
| ZTE | Following LTE and Rel-16 logic, when to do sensing is up to MAC layer triggering, so for the first sub-bullet, we prefer the original text that “UE is triggered to perform resource (re)selection procedure for periodic transmission (i.e., resource reservation interval Prsvp_TX≠0)” And the first sub-sub-bullet(first FFS part) should be the same level as first sub-bullet. For the second FFS part, we think it should be removed. A common design for initial period and other periods is expected and can be studied till next meeting. - FFS how to deal with the initial period(s) in periodic/aperiodic transmission where the UE have insufficient sensing results, e.g., - whether a TX UE is able to know in advance / expect a resource (re)selection will be triggered in slot n for early sensing, - performing random resource selection or contiguous partial sensing, - dropping transmission(s), - up to UE implementation to handle this case, or - etc |
| CATT,GoHIGH | We think the proposal still need some change. There should be at least three condition for the UE to perform periodic partial sensing - The resource pool is (pre-)configured to enable partial sensing - UE is configured to perform partial sensing by MAC layer. - Further conditioned by one or more of configured thereshold |
| MediaTek | We agree with FL’s proposal from technical perspective. We tend to think that we should make the 2nd FFS a separate proposal, instead of a sub-bullet here. The main bullet point here defines the triggering conditions for periodic partial sensing. However, 2nd FFS is describing another important issue that can be addressed by different resource selection methods potentially, e.g., contiguous sensing, random selection, etc. |
| Qualcomm | We think the first two bullets that are removed should be brought back: Instead, we suggest to remove the third bullet since it requires the UE to perform sensing only after the resource selection is triggered. The last bullet is unclear. We therefore propose the following changes: - Periodic-based partial sensing is performed by UE in a mode 2 Tx resource pool provided by higher layer when the followings are met: - Periodic reservation for another TB (sl-MultiReserveResource) is enabled for the resource pool - The resource pool is (pre-)configured to enable partial sensing - UE is triggered to performs resource (re)selection procedure for periodic transmission (i.e., resource reservation interval Prsvp_TX≠0) - FFS the case when UE is triggered to perform resource (re)selection procedure for aperiodic transmission - FFS how to deal with the initial period(s) in periodic/aperiodic transmission where the UE have insufficient sensing results, e.g., - whether a TX UE is able to know in advance / expect a resource (re)selection will be triggered in slot n for early sensing, - performing random resource selection or contiguous partial sensing, - dropping transmission(s), - up to UE implementation to handle this case, or - etc - UE is configured to perform partial sensing by higher layer |
| InterDigital | We support the proposal. In our view, periodic-based partial sensing shall be performed for periodic transmission; otherwise, persistent collision may happen. We can further study whether/how to support periodic-based partial sensing for aperiodic traffic and the initial period(s) of the periodic traffic. |

Proposal 2-2 (round II):

- Periodic-based partial sensing is also performed by UE for the purpose of re-evaluation and pre-emption checking (if pre-emption is enabled for the resource pool).

  - FFS re-evaluation and pre-emption checking is for all periods or only when reselection is expected

  - FFS any triggering conditions necessary

| Company | Comments |
| --- | --- |
| NTT DOCOMO | How to set Y candidate slots for re-evaluation/pre-emption check is unclear for us. If the slots are set independent to Y candidate slots corresponding to resource selection, so many additional slots could become monitoring slots. Therefore, one FFS should be added: - FFS how to determine Y candidate slots for re-evaluation and pre-emption checking |
| Apple | Support the proposal. |
| Futurewei | We do not support this proposal. As we commented before, the periodic-based partial sensing is for selecting the resource from a resource pool of Y slots, not to serve as a purpose of re-evaluation and pre-emption. Re-evaluation and pre-emption are more dynamic which happen after the resource selection. Mixing the periodic based sensing,or partial sensing in general, and re-evaluation/pre-emption will cause a lot of issues on determining the timing/procedures for periodic based partial sensing which are still in discussions. |
| LGE | - Main bullet For clarification as in Proposal 2-1, we suggest the following modification: “In a resource pool (pre-)configured with at least partial sensing, at least when the reservation for another TB (when carried in SCI) is enabled for the resource pool, (and if pre-emption checking is (pre-)configured in the resource pool,) Pperiodic-based partial sensing is also performed by UE for the purpose of re-evaluation (and pre-emption checking) (if pre-emption is enabled for the resource pool). - Sub-bullets The meanings of FFS points are not clear. We suggest the following modification. - FFS re-evaluation and pre-emption checking is for all periods or only when reselection is expected - FFS any triggering conditions necessary - FFS details |
| NEC | Support |
| CMCC | Not sure if we understand it correctly. In R16, the re-evaluation and pre-emption can be applied to further alleviate the impact of aperiodic reservations on selected or reserved resources. Here, however, I do not see much benefit brought by using periodic-based paritial sensing to do re-evluation/pre-emption. |
| Fujitsu | Support |
| OPPO | Some clarification is needed. This proposal apply to periodic transmission or aperiodic transmission, or both? |
| Panasonic | Support |
| Huawei, HiSilicon | We do not think the proposal is needed. In Rel-16, candidate resources set, S_A, is provided by PHY layer based on full sensing and reported to MAC layer. Re-evaluation and pre-emption checking are performed (if enabled) to for the (r0,r1,r2,…) and(r0',r1',r2',…), respectively is part of the reported S_A, and a short term sensing before m-T3 is introduced, which aim to exclude the reservation by aperiodic traffic and higher priority transmission. In Rel-17, the initial resource selection for S_A can be determined by periodic based and contiguous partial sensing, the reevaluation and pre-emption checking procedure should be same as Rel-16. If the periodic based partial sensing results are used for re-evaluation or pre-emption checking, the evaluation/checking results for periodic based partial sensing would not make any differences because the sensing results have already applied to determine S_A which cover all the (r0,r1,r2,…) and(r0',r1',r2',…). Therefore, we do not see the necessity to perform periodic based partial sensing for re-evaluation and pre-emption checking. |
| Samsung | The main bullet seems fine but FFS sub-bullets looks ambiguous. We suggest to remove the FFS sub-bullets or modify it as, similar to LGE, “FFS details”. |
| Intel | We support proposal |
| Fraunhofer | We support the FL’s proposal. |
| Xiaomi | We do not fully understand the intention of the proposal. Whether the re-evaluation and pre-emption check is for a resource selected by perioidic-based partial sensing RA scheme? If not, why suddenly periodic-based partial sensing is perfomed? Is the following the intention of FL? If yes, we agree. - Periodic-based partial sensing is also performed by UE for the purpose of re-evaluation and pre-emption checking (if pre-emption is enabled for the resource pool) for a resource selected by periodic-based partial sensing. |
| vivo | Not support. It is hard for UE to predict the location of slot triggering re-evaluation and pre-emption, it is also not clear how could UE determine the periodic sensing occasions corresponding to the triggring slot and perform SCI reception in advance. On the other hand, re-evaluation and pre-emption checking are introduced to handle the collision brought by aperiodic reservation from other UE, we don’t see how periodic sensing can help in this case. |
| Ericsson | We are in general OK with the proposal, however, we would like to get more clarification for the last FFS. Is the intention to trigger periodic sensing just for re-evaluation and pre-emption checking? In our view, it is not needed to trigger a periodic-based partial sensing just for the purpose of re-evaluation and pre-emption checking, so it should not have an independent triggering and just be performed whenever the periodic-based partial sensing is done. |
| CATT,GOHIGH | The meaning of the FFS is not clear to us, we prefer to remove the FFS. |
| MediaTek | We do not support. It is not clear to us how UE can determine sensing occasions beforehand based on some candidate slots for re-evaluation/pre-emption. |
| Qualcomm | We support the intention of the proposal, but prefer a simpler wording as follows: “The information obtained from partial sensing is used for pre-emption and reevaluation checking.” In addition, both FFS points should be removed. |
| InterDigital | In general, we are ok with the proposal. However, in our view, to reduce sensing power, pre-emption can be performed in a subset of periods (e.g., the UE can perform pre-emption one every N reservation periods). We propose to modify the proposal as follow: - Periodic-based partial sensing is also performed by UE for the purpose of re-evaluation and pre-emption checking (if pre-emption is enabled for the resource pool). - FFS re-evaluation and pre-emption checking is for all periods, a subset of periods, or only when reselection is expected - FFS any triggering conditions necessary |

Proposal 2-3 (round II):

- FFS whether UE performing periodic-based partial sensing is further conditioned by one or more of the followings

  - When the priority value of a packet is above a threshold

  - When the congestion/interference level in a resource pool is above a threshold and/or below another threshold

  - When the required reliability level of a packet transmission is above a threshold

  - When the number of received HARQ NACKs error rate of a TB is above a threshold

  - When the number of retransmissions of a packet is below a threshold

  - When the PDB/remaining PDB/latency requirement is above a threshold

  - When the available resource ratio in a resource selection window from a sensing process is above a threshold and/or below another threshold

| Company | Comments |
| --- | --- |
| NTT DOCOMO | We do not see motivation to have additional condition. If in some cases periodic-based partial sensing is not performed while it is capable/possible, resource collisions will increase. To drop this proposal is preferred. If RAN1 notice deemed necessary in future, then we can open the door after that. |
| Apple | We are open to discuss this. |
| Futurewei | We are ok with this proposal. |
| LGE | - Sub-bullets We prefer to keep the 5th sub-bullet for FFS. If the number of retransmissions is small, UE needs to ensure TB transmission successfully as much as possible with the limited transmission opportunity. I don’t understand why this sub-bullet is removed from the FFS point stage. With this direction, we suggest the following modification. - When the number of received HARQ NACKs or HARQ error rate of a TB is above a threshold - When the number of retransmissions of a packet is below a threshold - Other conditions are not precluded |
| CMCC | We are open to discuss it |
| Fujitsu | We think that such many FFSs may bring too much standards workload, as pointed out by some other companies. And we are not clear about the FFS in last sub-bullet, since sensing should be done before and used to obtain the available resource ratio, then how to use the available resource ratio to decide whether to do periodic-based partial sensing on the contrary? More clarifications are needed. |
| OPPO | No necessary for the proposal. If no agreement, everything is open for discussion. |
| Panasonic | We are ok with the FFS. |
| Huawei, HiSilicon | This FFS is not needed. As what we commented before. We see none of them is necessary. If a resource pool enables periodic reservation, each of these conditions would degrade the partial-sensing performance and cause collisions which results in retransmissions, meaning there is no certainty benefits for power saving overall. |
| Samsung | We are open to study further conditons. As the main bullet says “periodic-based partial sensing is further conditioned” which implies adaptation of the some of the parameters of periodic partial sensing, it would be better to rephrase all the sub-bullets by just listing the metric without mentioning above or below a threshold. Furthermore, other metrics should not be precluded in this meeting. Therefore, we suggest the following modifications: - FFS whether UE performing periodic-based partial sensing is further conditioned by one or more of the followings - When the priority value of a packet is above a threshold - When the congestion/interference level in a resource pool is above a threshold and/or below another threshold - When the required reliability level of a packet transmission is above a threshold - When the number of received HARQ NACKs error rate of a TB is above a threshold - When the number of retransmissions of a packet is below a threshold - When the PDB/remaining PDB/latency requirement is above a threshold - When the available resource ratio in a resource selection window from a sensing process is above a threshold and/or below another threshold - Other metrics In addition, as commented in 1st round, we prefer to consider all possible resource allocation schemes (random selection, periodic-based partial sensing, contiguous partial sensing) under a high level structure. Therefore, we prefer to combine proposal 5 and proposal 2-3, e.g. to discuss under what conditions a given resource allocation scheme can be used, or how UE switches between different resource allocation schemes. |
| Intel | We agree to discuss this topic. However, we think at this stage of the discussion the list of potential conditions is not complete. Thus, adding FFS for other conditions is desirable - FFS other conditions |
| Fraunhofer | We support the FL’s proposal. |
| Xiaomi | We do not think this proposal is necessary but we are OK to leave these FFS. |
| Vivo | We are ok with this proposal. |
| Ericsson | We are supportive of this proposal |
| ZTE | We think in physical layer, the triggering for sensing can be obtain from MAC layer, the discussion of conditions can be left to RAN2. |
| CATT,GOHIGH | We support this proposal. |
| MediaTek | We are open to discuss more. But, we should be cautious not to reach any restrictive agreement at the moment before having detailed discussion/study on this. All companies may need to perform simulation results to evaluate potential performance gain. For now, we can agree on such proposal if we also include “FFS other conditions”, as suggested by Intel. Also, we should also not preclude the possibility to define such conditions to trigger random selection instead of partial sensing. |
| Qualcomm | We are open to discuss this topic, but the discussion should cover both the periodic partial sensing as well as contiguous partial sensing. We do not see a reason to include the options at this point. While some options might be applicable to one sensing type, they may not be usable for the other. Hence, it is preferable to discuss the solutions separately for each of the two schemes. |
| InterDigital | We support the FL’s proposal. |

### Proposals before Monday GTW session (April 19th)

FL observations and comments based on inputs received in Sec. 3.2.2:

- On Proposal 2-1 (round II), FL comments:

  - DCM/LGE/Xiaomi/vivo:

    - No need to take care of the initial period/selection in sensing for periodic transmissions

    - When sensing results are not available for sl-TxPoolSelectedNormal resource pool, the UE performs random selection in sl-TxPoolExceptional in R16 NR-V2X.

    - FL recommendation: Although in LTE-V the problem of insufficient sensing results during the initial period(s) of periodic transmissions was not solved by specification, some others agreed to study further. Therefore, the suggested modification from Ericsson is taken onboard.

  - @Apple: As agreed in the last meeting that the terminology “periodic-based partial sensing” is based on the “partial sensing” used in LTE-V and it is intended to be used for the design and discussion of partial sensing in Rel-17, we may not use or need to use this wording later on in the spec. E.g. once a resource selection/re-selection procedure is triggered, the UE performs partial sensing according to both periodic-based and contiguous partial sensings for periodic transmission, instead of describing these sensing behaviours in separate sections. As long as we have clearly defined conditions in which the type of sensing should be performed by L1, we don’t need the MAC layer to do the selection.

  - @All:

    - The first two sub-bullets were removed based on a comment from 2 rounds ago that these two conditions were already agreed in the last meeting, but now it seems majority still want these to be included in this proposal. Reading again the agreement from the last meeting, I think the majority is right that it does not explicit stating the first two removed bullets were agreed to be conditions in which the UE should perform periodic-based partial sensing. So now I have added them back.

      - @LGE: Thanks for the suggestion, I hope you also read it in the same way.

    - For the discussion on whether a UE should perform periodic-based partial sensing that is dependent on the traffic type, while some companies agreed with the last round’s proposal to consider them separately (periodic and aperiodic), others have the opinion that periodic-based partial sensing should be performed whenever there is a possibility that other UEs are performing periodic transmissions. FL believes within this meeting, we wouldn’t be able to progress further on this, so let’s reconsider this aspect in the future if necessary. This is added as a new FFS point in Proposal 2-3 (round III). Having said this and based on the comments raised during the last round, I wonder how the group think for the following scenarios/questions. These can be considered until the next meeting.

- When UE is performing periodic-based partial sensing based on an on-going periodic transmission and a new resource (re)selection procedure is triggered for a different periodic transmission (e.g. with different triggering timing, periodicity, remaining PDB, etc), should the UE start a new/separate periodic-based partial sensing process?

- When UE is performing periodic-based partial sensing based on an on-going periodic transmission and a new resource (re)selection procedure is triggered for an aperiodic transmission (e.g. with different triggering timing, remaining PDB, etc), should the UE start a new/separate periodic-based partial sensing process or the sensing data from the existing periodic-based partial sensing process is reused whenever applicable (e.g. with overlapping sensing occasions/window)?

- When UE does not have any on-going periodic transmission and a new resource (re)selection procedure is triggered for an aperiodic transmission, should the UE start a new periodic-based partial sensing process?

  - @HW, HiSi: On the sub-bullet “UE is configured to…”, based on existing spec description we know it is the MAC layer that configures which RA scheme that is to be used at L1. But the term commonly used in the LTE-V and NR-V spec is “… by higher layer”. I think it is OK here resuing this common term.

Proposal 2-1 (round III):

- Periodic-based partial sensing is performed by UE in a mode 2 Tx resource pool provided by higher layer when at least the followings are met:

  - Periodic reservation for another TB (sl-MultiReserveResource) is enabled for the resource pool

  - The resource pool is (pre-)configured to enable partial sensing

  - UE is configured to perform partial sensing by higher layer

  - UE is triggered to performs resource (re)selection procedure at least for periodic transmission (i.e., resource reservation interval Prsvp_TX≠0)

    - FFS the case(s) when UE is triggered to perform resource (re)selection procedure for aperiodic transmission

  - FFS whether and how to deal with the initial period(s) in periodic/aperiodic transmission where the UE have insufficient sensing results, e.g.,

    - whether a TX UE is able to know in advance / expect a resource (re)selection will be triggered in slot n for early sensing,

    - performing random resource selection or contiguous partial sensing,

    - dropping transmission(s),

    - up to UE implementation to handle this case, or

    - etc

| Company | Comments |
| --- | --- |
| Xiaomi | With “UE is configured to perform partial sensing by higher layer” , whether RAN1 will continue to discuss when partial sensing should be configured by higher layer, or leave it to RAN2 decision? Still do not think the last subbullet is needed but can accept it. FL: Thanks for being flexible. This is how it is written in the LTE-V spec. |
| Qualcomm | - We propose to have “UE is configured to perform partial sensing by higher layer” as an FFS for now. More discussion on the details of this condition is needed. - The last FFS, i.e., “FFS whether and how to deal with the initial period(s) in periodic/aperiodic transmission where the UE have insufficient sensing results” is only needed to be discussed if sensing is performed after resource selection is triggered. Since there is no agreement on the relation between sensing triggering and resource selection triggering, discussion for this topic should be deferred until later. FL: For the first bullet, in LTE-V, when a PUE supports random resource selection and partial sensing, it is configured by the higher layer to perform one of them. Here, I believe the same intention / mechanism can be reused. For the second bullet, so far we don’t have a sensing trigger but we do have relation between sensing occasion/window to a resource (re)selection trigger. As I explained in the background before the proposal, it is good for companies to consider these aspects. Deferring is not a good idea in my view. |
| Sharp | We are in generally fine with the proposal except the last FFS part and propose to remove it. “Insufficient sensing results” is not clear enough. To us, if the results are not enough, that would make the corresponding slot(s) not among the Y candidate slots. FL: The question is how do you ensure a UE always have sufficient sensing results for the initial resource selection of both periodic and aperiodic transmission (e.g., when not all corresponding sensing occasions are monitored by the UE before the initial resource selection). This concern was brought up by some companies, and I think it is OK to further study this aspect. |
| NTT DOCOMO | Now there are two alternatives are here - Periodic-based partial sensing should be performed for any transmission type - Periodic-based partial sensing should be performed only for periodic transmissions Then which is the intention of this proposal? Main part seems first one based on removing ‘UE is triggered to performs resource (re)selection procedure at least for periodic transmission (i.e., resource reservation interval P_"rsvp_TX" ≠ 0)’ while FFS seems to intend the second alternative. If the second one is the intention, then the removed ‘UE is triggered to performs resource (re)selection procedure at least for periodic transmission (i.e., resource reservation interval P_"rsvp_TX" ≠ 0)’ should be back with ‘at least’. FL: I think some companies feel strongly that the triggering should not be dependent on the traffic type. Therefore, they preferred not to have the previous formulation. Now the FFS is more focus on the case when there is insufficient sensing results. |
| NEC | We can accept the proposal. |
| Samsung | Support first 3 bullets but not the last one. At first, we think the FL’s view “once a resource selection/re-selection procedure is triggered, the UE performs partial sensing according to both periodic-based and contiguous partial sensings for periodic transmission” is not common understanding so far, and we need to clarify it before discussing proposal 2-1. Secondly, the pre-condition of FL’s view above is that UE performs periodic transmission, but there is no clue showing it in current version of proposal 2-1. As we commented in previous rounds (sorry for repeating our view but it wasn’t reflected in updated proposals), periodic transmission is not RAN1 spec wording. Traffic type are invisible by PHY layer, and how to indicate the traffic characteristics, which may reflect traffic type, to PHY layer should be determined by RAN2. Therefore, we prefer using “UE is able to know in advance / expect a resource (re)selection will be triggered in slot n” instead of “periodic transmission” (and vice versa for aperiodic transmissions) in RAN1 agreement. |
| Huawei, HiSilicon | We are basically fine with the proposal, except the FFS. The UE behaviour for initial period is clear in Rel-16 NRV, and we do not see the necessity to change it in Rel-17. FL: Since we don’t have partial sensing in R16, then we didn’t have this problem. |
| LGE | - Main bullet and first two sub-bullets Our proposal seems slightly misunderstood. If we see the last agreement below, it’s already allowed for UE to perform periodic-based partial sensing on the conditions yellow. Therefore, as we commented in our response, the first two sub-bullets should be included as conditions in the main bullet, as same as the last agreement. Agreements: In a resource pool (pre-)configured with at least partial sensing, if UE performs periodic-based partial sensing, at least when the reservation for another TB (when carried in SCI) is enabled for the resource pool and resource selection/reselection is triggered at slot n, it is up to UE implementation to determine a set of Y candidate slots within a resource selection window, where The resultant suggestion is as follows. - In a resource pool (pre-)configured with at least partial sensing, at least when the reservation for another TB (when carried in SCI) is enabled for the resource pool and resource selection/reselection is triggered at slot n, Pperiodic-based partial sensing is performed by UE in a mode 2 Tx resource pool provided by higher layer when at least the followings are met: - Periodic reservation for another TB (sl-MultiReserveResource) is enabled for the resource pool - The resource pool is (pre-)configured to enable partial sensing FL: As I also replied earlier, the agreement (yellow highlighted part) may not be read as a condition for UE to perform periodic-based partial sensing, because the wording “… if UE performs periodic-based partial sensing, …”. This point has been discussed back and forth. I think it is Okay to include them here. |
| Ericsson | We are in general supportive of this proposal, however, the issue of the last FFS regarding insufficient sensing results is still not clear to us. |
| Convida Wireless | We are generally ok with the proposal. |
| Fraunhofer | We are supportive of the FL’s proposal. |
| ZTE | OK |

- On Proposal 2-2 (round II), FL comments:

  - @Futurewei, CMCC, HW, HiSi, Xiaomi, vivo: In R16, re-evaluation and pre-emption checking is already supported and it is also agreed to be supported by UEs that perform sensing. Technically, without performing periodic-based partial sensing, a UE would not be able to monitor and detect resources that are periodically reserved by others in the subsequent period(s) until the next resource (re)selection trigger. In R16, re-evaluation and pre-emption checking were not only to monitor aperiodic reservations. Since the UE is performing full sensing all the time, it detects everything.

  - @LGE: Thanks for the suggested modifications, which make it clear the scenario in which the periodic-based partial sensing should be performed.

  - @OPPO: The question can be part of the sub-bullet “FFS deails”.

  - @MediaTek: As DOCOMO mentioned, how to determine the Y candidate slots can be further discussed. But technically, based on R16 procedure for re-evaluaiton and pre-emption checking, the higher layer provides (r0,r1,r2,…) andr0',r1',r2',… as HW pointed out.

  - @Qualcomm: Thanks for the suggestion. I think it could be one way to describe the intention, but I expect others will likely question which “partial sensing(s)“ it is referring to. Then we are back to the same discussion as expressed here. Let’s try this route first, if others prefer your simpler wording, I am find as well.

  - @InterDigital: I think your proposed enhancements can be part of the sub-bullet “FFS details”.

Proposal 2-2 (round III):

- In a resource pool (pre-)configured with at least partial sensing, at least when the reservation for another TB (when carried in SCI) is enabled for the resource pool, (and if pre-emption checking is (pre-)configured in the resource pool), at least periodic-based partial sensing is also performed by UE for the purpose of re-evaluation (and pre-emption checking) (if pre-emption is enabled for the resource pool).

  - FFS details

| Company | Comments |
| --- | --- |
| Qualcomm | It is not clear to us what “at least periodic-based partial sensing is also performed by UE for the purpose of re-evaluation (and pre-emption checking)” intends to imply. In our view, the sensing information that a UE accumulates during its periodic partial sensing can be re-used by the UE for the purpose of re-evaluation and pre-emption checking. If this is the intention of the proposal, the following wording is suggested: “The information obtained from partial sensing is additionally used for pre-emption and reevaluation checking.” FL: I think you are referring to the period in which the resources are selected initially. But for the subsequent periods, re-evaluation and pre-emption checking should be also performed, and as such additional periodic sensing occasions should be monitored by the UE. |
| Sharp | We don’t think the proposal is needed. The resources for re-evaluation and pre-emption check are provided by MAC layer, together with some parameters for partial sensing, which means when determining subset of resources for transmission, i.e. sensing, re-evaluation and pre-emption check are performed. FL: I agree this is R16 behaviour for UE performing full sensing. But since we are discussing partial sensing in R17, sensing behaviour should be rediscussed again not only for resource (re)selection, but also for re-evaluation and pre-emption checking. |
| NTT DOCOMO | For us, whether or not UE needs to do additionally/newly sensing for re-evaluation/pre-emption check is unclear. Current wording seems UE shall do but this aspect should be discussed further. Therefore we suggest the following, where monitored slots for this periodic-based partial sensing might be new one, might not. - In a resource pool (pre-)configured with at least partial sensing, at least when the reservation for another TB (when carried in SCI) is enabled for the resource pool, (and if pre-emption checking is (pre-)configured in the resource pool), resource exclusion based on at least periodic-based partial sensing is also performed by UE for the purpose of re-evaluation (and pre-emption checking) (if pre-emption is enabled for the resource pool). - FFS details FL: Thanks, I am fine with the addition. |
| vivo | We are not sure whether the proposal is to intended to allow UE to trigger a sepearate periodic sensing process for re-evaluation and pre-emption checking, or just to allow UE to reuse the already available periodic sensing for re-evaluation and pre-emption checking as commented by Qualcomm. If it is the former case, we are still not clear how could UE predict the timing of checking and perform the corresponding pre-periodic-sensing especially when the enabled Preserve value is small. FL: It is both the former and latter cases. When resources are selected during the initial selection, their locations are known to the UE and pre-periodic-sensing can be performed. But I am not very certain that when Preserve value is small, some periodic sensing occasions could be missed by the UE. Currently, I think all periodic sensing occasions can be predicted in advanced for the purpose of re-evaluatoin and pre-emption checking. If there is any missing cases, it should be covered by the sub-bullet “FFS details”. |
| NEC | For clarification, the difference between proposal 2-2 and 2-1 is the condition "UE is configured to perform partial sensing by higher layer " in 2-1 is not captured in 2-2, is this proposal's intention that even if UE is NOT configured to perform partial sensing by higher layer while pre-emption checking is (pre-)configured in the resource pool, UE also performs periodic-based partial sensing? If so, maybe we can combine the two proposals to make it clearly. FL: Good point, I should add “subject to other conditions, if agreed, in which the UE performs periodic-based partial sensing” |
| Huawei, HiSilicon | We disagree the proposal. We can understand FL explaination that the re-evaluation and pre-emption checking should also take the periodic reservation into account, but we concern whether it is useful to have those periodic based partial sensing results. Periodic-based partial sensing is performed for determination of reported S_A from the set of Y candidate slots, and re-evaluation/pre-emption checking are performed after reporting S_A, which aims to see if S_A will be updated. The periodic-based partial sensing result will not change, because it was already obtained before S_A. So we do not think if periodic partial sensing results are considered, the re-evaluation/pre-emption checking consequence would be different. FL: Please refer to earlier comments to other companies above. |
| LGE | Support FL proposal |
| Ericsson | We are supportive of the direction of this proposal, but some rewording is needed in our view since the intention is not completely clear. We propose the following revision: - In a resource pool (pre-)configured with at least partial sensing, at least when the reservation for another TB (when carried in SCI) is enabled for the resource pool, (and if pre-emption checking is (pre-)configured in the resource pool), at least periodic-based partial sensing is also performed by UE including re-evalution and pre-emption checking for the purpose of re-evaluation (and pre-emption checking) (if pre-emption is enabled for the resource pool). - FFS details FL: Thanks and OK. |
| Fraunhofer | We are supportive of the FL’s proposal. |
| ZTE | Similar confusion as others, in case this refers to a novel triggering mechanism in addition to the MAC, the timing is unpredictable. |

- On Proposal 2-3 (round II), FL comments:

  - Support or open to have the FFS proposal: Apple, Futurewei, LGE, CMCC, Pana, Samsung, Intel, Fraunhofer, vivo, Ericsson, ZTE (up to RAN2), CATT, GOHIGH, MTK, Qualcomm, IDC (16)

  - Not support/prefer to have the FFS proposal: DCM, OPPO, HW, HiSi, Xiaomi (5)

  - @Fujitsu: further details can be studied

  - @All: Due to overwhelming preference to study further on the condition(s) in which the UE should perform partial sensing and it is also part of the agreements from the last meeting, let’s keep the FFS.

Proposal 2-3 (round III):

- FFS whether UE performing periodic-based partial sensing and/or contiguous partial sensing is further conditioned by one or more of the followings

  - When the priority value of a packet is above a threshold

  - When the congestion/interference level in a resource pool is above a threshold and/or below another threshold

  - When the required reliability level of a packet transmission is above a threshold

  - When the number of received HARQ NACKs or HARQ error rate of a TB is above a threshold

  - When the number of retransmissions of a packet is below a threshold

  - When the PDB/remaining PDB/latency requirement is above a threshold

  - When the available resource ratio in a resource selection window from a sensing process is above a threshold and/or below another threshold

  - When the traffic is periodic and/or aperiodic

  - When the UE has packet to transmit

  - Other conditions are not precluded

| Company | Comments |
| --- | --- |
| Qualcomm | Since the entire proposal is an FFS, we suggest to only leave it in the feature lead’s notes as a reminder for the companies to consider for the next meeting. No further action at this point seems to be needed. FL: I am affair not everyone read FL’s notes after the meeting, perhaps RAN1 Chairman’s notes is a better place 😊, if everyone is OK with this approach. |
| Sharp | We share similar view as Qualcomm. |
| NTT DOCOMO | Agree with QC. No need to agree this FFS. If these features are really necessary, then we can discuss and agree. |
| Samsung | We’re fine with the proposal. |
| Huawei, HiSilicon | We are still not convinced why the FFS is needed. As we have explained in previous rounds, stop sensing will result in highe level of collision and degrading the reliability for all UEs in the resource pool. We suggest that rather than exploding the workload with this huge list of FFS, that companies need to have another meeting of raising key issues in their papers, to see which few of them gather more interest and evaluation in the May meeting. |
| LGE | Support in general. If the third to the last sub-bullet is listed, the following sub-bullet also needs to be added. - When the traffic is periodic and/or aperiodic - When the reservation for another TB (when carried in SCI) is enabled for the resource pool FL: The blue bullet is already listed and the red bullet is already covered by Proposal 2-1. |
| Ericsson | We are supportive of this proposal. |
| Convida Wireless | We are fine with the proposal. |
| Fraunhofer | We are supportive of the FL’s proposal. |
| ZTE | RAN2 triggering is preferred. |

### Proposals before Tuesday GTW session (April 20th)

FL observations and comments based on inputs received in Sec. 3.2.3:

- On Proposal 2-1 (round III), FL comments:

  - Some expressed that the meaning of insufficient sensing results was unclear. In my observation, it refers to the following scenarios:

    - During the initial resource selection for a periodic transmission, similar to an aperiodic transmissions, the TX UE does not know the trigger timing. As such, the UE would not abel to monitor slots according to periodic sensing occasions in advance for the initial resource selection trigger. In this case, and also for aperiodic transmissions in a resource pool with reservation for another TB enabled, the UE does not have sensing information for all periodic sensing occasions. If the UE perfoms resource selection in this situation, it will have higher chance of Tx collision with other UEs.

Proposal 2-1 (round IV):

- Periodic-based partial sensing is performed by UE in a mode 2 Tx resource pool provided by higher layer when at least all of the followings are met:

  - Periodic reservation for another TB (sl-MultiReserveResource) is enabled for the resource pool

  - The resource pool is (pre-)configured to enable partial sensing

  - UE is configured to perform pPartial sensing is configured by higher layer

  - FFS whether and how to deal with cases the initial period(s) in periodic/aperiodic PSCCH/PSSCH transmission with and without enabled reservation for another TB where the when UE have insufficient sensing results

    - At least for the initial period(s) of PSCCH/PSSCH transmission with and without enabled reservation for another TB

| Company | Comments |
| --- | --- |
| Apple | We support the proposal. |
| Nokia, NSB | The first three sub-bullets define the condition for periodic-based partial sensing. All these conditions shall be met. Suggest to have a minor modification: “Periodic-based partial sensing is performed by UE in a mode 2 Tx resource pool provided by higher layer when at least all of the followings are met: …” FL: that’s fine. |
| NTT DOCOMO | We are fine this proposal itself. But what is the difference from proposal 1-2? This FFS seems similar to proposal 1-2. FL: In this proposal, it also deals with the case of periodic transmission. Please refer to the description just above the proposal and past discussions and comments in Section 3.2.3. |
| CATT,GOHIGH | More clarification is needed for ‘initial period(s) in PSCCH/PSSCH transmission”. For example , for aperiodic traffic, how this is initial is defined is not clear FL: In aperiodic traffic, there is only one transmission period bounded by the remaining PDB. In periodic traffic, the initial period(s) starts from the intial resource selection for a number of periods until all periodic sensing occasions are monitored by the UE to obtain all sufficient sensing results. |
| InterDigital | We support the proposal |
| Futurewei | For the last FFS, we would like to remove “where the UE have insufficient sensing results”, as UE may apply contiguous partial sensing for the initial traffic. Also the wording “insufficient sensing results” is ambiguous as the criterion or the definition on “insufficient” is not clear. FL: I can understand your preference. Please refer to my explanation to CATT/GOHIGH. In the FFS bullet, “whether” is also included. So in the end, we may not do anything to solve this issue. I hope you can accept this way forward to resolve concerns from others. |
| Qualcomm | We are not fine with the addition of the last FFS as it implies that the UE starts sensing only after resource selection is triggered. On “The resource pool is (pre-)configured to enable partial sensing”, we would like to point out that a resource pool could be configured to allow for partial sensing, but the switching across different sensing schemes could be based on dynamic mechanisms. Since the details are not discussed in this meeting, we cannot agree to this bullet. FL: The FFS bullet does not necessary link UE starts sensing only after resource selection is triggered. Even in the case when UE is performing periodic-based partial sensing all the time without any packet to transmit, it does not guarantee the sensing information is always applicable when an aperiodic traffic arrived, unless the UE is performing “full sensing” within a set of periodically occurring resources where the UE must select its transmission resources from. Since we are discussing the periodic-based partial sensing operation that we agreed in the last meeting (and not yet agree to confine whether the resource selection window [n+T1, n+T2] within a set of periodic set of resources), I think it is fair to study the case when the periodic-based partial sensing does not always provide sensing information that is useful / match to the aperiodic transmission. |
| Sharp | We are fine with the proposal except for the FFS part. In our understanding, when MAC layer triggers sensing, the UE would determine the Y candidate slots based on the already monitored periodic sensing occasions, which means if a slot in the resource (re-)selection window does not have “insufficient sensing results”, the slot cannot be a candidate slot. On this point, we observed that it aligns with Qualcomm’s comment for Proposal 2-2 as in the following table. Meanwhile, in RAN1#88 for LTE partial sensing, we excerpt “Note for minutes” as following, Note for minutes: RAN1 assumes that if the number of possible candidate subframes in the selection window [m+T1, m+T2] for which sensing measurements are available, is less than the minimum allowed value of Y, then the UE does not transmit using partial sensing. In our view, the slots with “insufficient results” seem similar description as “NOT possible candidate subframes”. In LTE V2X, we once discussed the case when the number of the candidate subframes cannot reach Y, due to lack of monitoring some periodic sensing occasions and for partial sensing in NR sidelink, we don’t see the difference. Thus, we don’t think the FFS part is needed here. FL: Thanks for providing information on the discussion happened in LTE-V for partial sensing. Unless we agree to the same note for R17 NR-sidelink, we can still consider enhancement on top of the LTE-V partial sensing scheme. BTW, the operation you described is very restrictive in term of when a resource (re)selection should be triggered (selection of Y candidate slots according to already sensed occasions). It may be fine for LTE-V as the smallest periodicity is 100ms. In NR sidelink, the periodicity can be much smaller and aperiodic transmissions are supported additionally. In the end, we don’t have to do any enhancement as said in the FFS bullet. |
| NEC | Support |
| Fujitsu | We support this proposal. |
| Xiaomi | We agree with QC that there is still no consensus on whether there exists such a “initial perioid” or not. From our understanding, what the FFS wants to discuss is just an exceptional case. Therefore, a more general FFS is suggested: - FFS whether there exist exceptional cases. FL: I worry that such general FFS bullet does not bring out the issue that was intended. Please also refer to the comment reply to QC above. |
| Huawei, HiSilicon | For the first three sub-bullets as the conditions to perform periodic based partial sensing, we are fine with them. As the FFS sub-bullet, we still have some converns on the necessity. We agree that there it no partial sensing in Rel-16, but for the issue that initial period has no sufficient sensing results, we do not see the different handlings are needed. UE can always select resources from exceptional pool for the initial period without sufficient sensing results. FL: I assume transmission in exceptional pool is not always intended for aperiodic transmissions, right? |
| Ericsson | We are in general supportive of this proposal, however, the issue of the last FFS regarding insufficient sensing results is still not completely clear to us. FL: Please refer to earlier comments and replies for the explanation of “insufficient sensing results”. |
| Panasonic | Support |
| Samsung | Since the main bullet use the wording “when at least all of the followings are met”, we think it’s still open to further discuss additional conditions that needed to be satisfied. If our understanding is correct, then we’re generally fine with the proposal. For 3rd sub-bullet, we prefer to remove “by higher layer”, or change “perform” to “enbale”. Our motivation is that, when UE may be configured with multiple RA schemes by higher layer, its PHY layer may dynamically determine to perform a RA scheme e.g. partial sensing, for a given transmission. We would like to not exclude this possibility at this stage. FL: To align with spec description, the 3rd bullet is revised to “Partial sensing is configured by higher layer”. |
| LGE | We are fine with FL proposal except the last sub-bullet. We don’t have to restrict the case where there are insufficient sensing results into the initial period. It can be more generalized and the details can be left as FFS. We suggest the following modification. - FFS whether and how to deal with the cases initial period(s) in periodic/aperiodic PSCCH/PSSCH transmission with and without enabled reservation for another TB where the UE have insufficient sensing results, e.g., FL: The deleted case is added as a sub-bullet. |

- On Proposal 2-2 (round III), FL comments:

  - Some re-wording suggestions are taken onboard.

Proposal 2-2 (round IV):

- In a resource pool (pre-)configured with at least partial sensing, at least when the reservation for another TB (when carried in SCI) is enabled for the resource pool, (and if pre-emption checking is (pre-)configured in the resource pool), resource exclusion based on at least periodic-based partial sensing is performed by UE including for re-evaluation (and pre-emption checking).

  - This is subject to other conditions in which the UE performs periodic-based partial sensing, if agreed

  - FFS details

| Company | Comments |
| --- | --- |
| Apple | We are supportive to the proposal in principle. Just some rewording on the first sentence, which follows the similar wording as in Proposal 2-1. In a resource pool (pre-)configured to enable partial sensing and to enable periodic reservation for another TB (sl-MultiReserveResource), (and if pre-emption checking is (pre-)configured in the resource pool), resource exclusion based on at least periodic-based partial sensing is performed by UE including re-evaluation (and pre-emption checking). - This is subject to other conditions in which the UE performs periodic-based partial sensing, if agreed - FFS details FL: Thanks for the suggestion to optimise the wording. During the last GTW session, we have seen it was difficult to find optimal wording and trying to go back and reuse last meeting’s description. The current wording formulation is taken from the last meeting. It may not be optimum, but it is something that is acceptable to everyone in the last meeting. From my perspective, it is safer to reuse it unless there is a technical concern. |
| Futurewei | We do not support this proposal. Unlike full sensing in R16, periodic-based partial sensing is for selecting the resource from a resource pool of Y slots, with specific sensing occasions that does not to serve as a purpose of re-evaluation and pre-emption. On the other hand, we do not think that the periodic partial sensing for pre-evaluation and pre-emption will have any benefit with most recent sensing occasions for resource selection, particularly if only the most recent sensing occasion for resource selection is agreed later. If something must be agreed now, we suggest add "Study further whether..." in the main bullet so that we can study more and consider if it is needed next meeting jointly with the resolution of k. - Study further whether In a resource pool (pre-)configured with at least partial sensing, at least when the reservation for another TB (when carried in SCI) is enabled for the resource pool, (and if pre-emption checking is (pre-)configured in the resource pool), resource exclusion based on at least periodic-based partial sensing is performed by UE including re-evaluation (and pre-emption checking) FL: Without repeating my past explanations, I just want to say that periodic-based partial sensing (i.e., UE monitoring periodic sensing occasions) is also for resource re-evaluation and pre-emption checking in subsequent transmission periods, other than just the initial periodic where the initial set of resources are selected. Pleaes refer to discussions and my comments in previous sections. |
| Qualcomm | Our previous comment was not only for the first period. A UE performs monitoring periodically and when it has data to transmit, it relies on the past sensing results to choose the candidate resources. In a similar manner, when a UE is performing re-evaluation or pre-emption checking, it can reuse the past periodic sensing results in addition to any contiguous partial sensing done after resource selection/reservation. We are still unsure what this proposal implies. FL: Since we have only agree to support re-evaluation and pre-emption checking for UE performs partial sensing, but we have not agree how to do it. So this is the next step to agree that at least periodic-based partial sensing is performed for re-evaluation and pre-emption checking. If this intention is not clear from the wording, then I will roll-back to previous wording. |
| Sharp | We share similar view as Qualcomm. |
| NEC | Fine with comment: If this proposal is intend to define another condition to perfrom PBPS, we still think it's better to combine it with proposal 2-1 or keep it and list "other conditions" under the first subbullet to make it more clear. FL: No, that’s not the intention. The sub-bullet was added based on a comment that other conditions may be further defined for performing periodic-based partial sensing. |
| Fujitsu | We are generally fine with this proposal. And we also propose to add a sentence in the main bullet to emphasize that the UE should be configured to perform partial sensing in this case, which is aligned with proposal 2-1. - In a resource pool (pre-)configured with at least partial sensing, at least when the reservation for another TB (when carried in SCI) is enabled for the resource pool, (and if pre-emption checking is (pre-)configured in the resource pool), for a UE which is configured to perform partial sensing by higher layer, resource exclusion based on at least periodic-based partial sensing is performed by UE including re-evaluation (and pre-emption checking). - This is subject to other conditions in which the UE performs periodic-based partial sensing, if agreed - FFS details FL: For the newly added condition in red text, I tend to agree technically. But currently since it is covered by Proposal 2-1 and not yet agreed, I have added the sub-bullet in this proposal. |
| Ericsson | We are fine with this proposal. However, we would like to ask why the part of “pre-emption checking” is in parenthesis? Maybe it is better to have it as a separate bullet from re-evaluation. FL: This is like an optimised expression suggested from LGE. If it is sufficient clear, which I believe it is, can we keep the current format? BTW, please check the updated description based on comments about unclear intention of this proposal. Please refer to QC’s comment above and my reply. |
| Panasonic | We are generally ok with the proposal. However, the conditional wording in the bracket of “(and if pre-emption …)” is not very clear to us whether it’s optional or mandaotory. FL: it means pre-emption checking is only performed when it is configured for the resource pool. |
| Samsung | We’re fine with the proposal in principle, but we think another issue need to be clarified at first: whether re-evaluation can be enabled/disabled for power saving mode. Current proposal implies that re-evaluation is always enabled, thus we suggest the following update: - In a resource pool (pre-)configured with at least partial sensing, at least when the reservation for another TB (when carried in SCI) is enabled for the resource pool, (and if re-evaluation and/or pre-emption checking is (pre-)configured in the resource pool), resource exclusion based on at least periodic-based partial sensing is also performed by UE including for the purpose of re-evaluation (and/or pre-emption checking) (if pre-emption is enabled for the resource pool). FL: Following R16 principle, re-evaluation is supported witout (pre-)configuration. |
| Huawei, HiSilicon 2 | We see the explanation from the FL that this proposal is supposed to say that periodic based partial sensing is an input to re-evaluation and pre-emption checking in periods subsequent to the initial. But the proposal does not say such words, and we don’t agree that this proposed understanding can be inferred from it. We don’t think such a proposal can be agreed while it remains near to the current formulation. FL: Please check the above updated and hopefully clearer version. Some old removed texts are deleted for easier reading. |
| LGE | After some more thought, we share the view with several companies that the proposal is not clear. In addition, it seems to early to determine whether periodic-based partial sensing in addition to contiguous partial sensing is effiecient way of sensing for power saving. We suggest the whole proposal as FFS. FL: I was looking to do step-by-step, agreeing periodic-based partial sensing first. But if it is preferred to include both periodic-based and contiguous partial sensing in the same proposal, I can suggest adding it to the Chairman. |

- On Proposal 2-3 (round III), FL comments:

  - It seems more supporter than objectors from the last round. FFS does not imply one or more of the following conditions will be adopted in the end.

Proposal 2-3 (round IV):

- FFS whether UE performing periodic-based partial sensing and/or contiguous partial sensing is further conditioned by one or more of the followings

  - When the priority value of a packet is above a threshold

  - When the congestion/interference level in a resource pool is above a threshold and/or below another threshold

  - When the required reliability level of a packet transmission is above a threshold

  - When the number of received HARQ NACKs or HARQ error rate of a TB is above a threshold

  - When the number of retransmissions of a packet is below a threshold

  - When the PDB/remaining PDB/latency requirement is above a threshold

  - When the available resource ratio in a resource selection window from a sensing process is above a threshold and/or below another threshold

  - When the traffic is with and/or without enabled reservation for another TBperiodic and/or aperiodic

  - When the UE has packet to transmit

  - Other conditions are not precluded

| Company | Comments |
| --- | --- |
| Apple | Maybe, we could change “FFS” to “Study” in the proposal. |
| Futurewei | We are ok with the proposal. |
| Qualcomm | We still do not think that this proposal, with a long list of features, is needed. In addition, none of the features is discussed during this meeting. They can be considered in the future meetings when detailes discussed and further studied if needed. |
| NEC | We are open on it. |
| Fujitsu | We are open on it. |
| Huawei, HiSilicon | Such a long list for FFS is always not preferred and we think the whole FFS is not needed. If companies deem the additional restrictions are needed, they can rely on the submitted contributions to decide what they want to study further, and clearly one more meeting is needed to see which gather enough attention to make them something that all must agree to study. For this meeting, we do not need to have this FFS. FL: it is hard to remove as a moderator when so far majority is supportive or at least open. I will leave it to the Chairman. |
| Ericsson | We are supportive of this proposal |
| Panasonic | Support |
| Samsung | We’re fine with the proposal in principle. Considering it’s an open issue for future study, we prefer only list potential factors which may be used as condition, and remove the threshold in sub-bullets to avoid introducing too much restrictions before companies spending sufficient time to study the issue. In addition, we prefer to remove the last two blue bullets: - When the traffic is periodic and/or aperiodic - When the UE has packet to transmit For the former one, periodic/aperiodic traffic is invisible at PHY layer and can hardly be used as condition. For the latter one, its intention seems unclear for us. FL: In my view, all these are valid conditions for further study. If start removing any based on company preference, then in my opinion in the end there will be none left. I can rephrase periodic / aperiodic traffic, although I still don’t get your point. |
| LGE | Fine with FL proposal. |

### Final proposals for Tuesday GTW session (April 20th)

Proposal 2-1’ (Topic #2):

- Periodic-based partial sensing is performed by UE in a mode 2 Tx resource pool provided by higher layer when at least all of the followings are met:

  - Periodic reservation for another TB (sl-MultiReserveResource) is enabled for the resource pool

  - The resource pool is (pre-)configured to enable partial sensing

  - Partial sensing is configured by higher layer

- FFS whether and how to deal with cases when UE have insufficient sensing results

  - E.g., the initial period(s) of PSCCH/PSSCH transmission with and without enabled reservation for another TB

Proposal 2-3 (Topic #2):

- FFS whether UE performing periodic-based partial sensing and/or contiguous partial sensing is further conditioned by one or more of the followings

  - When the priority value of a packet is above a threshold

  - When the congestion/interference level in a resource pool is above a threshold and/or below another threshold

  - When the required reliability level of a packet transmission is above a threshold

  - When the number of received HARQ NACKs or HARQ error rate of a TB is above a threshold

  - When the number of retransmissions of a packet is below a threshold

  - When the PDB/remaining PDB/latency requirement is above a threshold

  - When the available resource ratio in a resource selection window from a sensing process is above a threshold and/or below another threshold

  - When the traffic is with and/or without enabled reservation for another TB

  - When the UE has packet to transmit

  - Other conditions are not precluded

## Topic #3: Contiguous partial sensing – TA and TB value range (include reporting slot)

Background: As well explained by various contributions, the intention for a TX UE performing contiguous partial sensing is to obtain information on aperiodic reservation made by other UEs. And since the max indication window (W) is 32 slots (or a gap of 31 slots) in the existing SCI format 1-A design, most company proposed the sensing window for CPS to cover up to 31 slots before a selection window / first candidate. Furthermore, in most contributions (please refer to Section 4.1), companies categorized their proposed range for the sensing window [n+TA, n+TB] in terms of periodic and aperiodic transmission. This is simply due to that a set of Y candidate slots (a restricted selection window) will likely be used in the resource (re)selection for periodic transmission, whereas a full random selection window [n+T1, n+T2] for full flexibility of selection is expected for aperiodic transmission.

Furthermore, it is also stated in various contributions that it should be allowed for TA and TB values to be zero, positive and negative to provide full flexibility and use cases when a TX UE has periodic or aperiodic transmission, and whether the UE has sufficient remaining PDB such that a contiguous partial sensing can be performed before the subset of resources is reported to the higher layer.

In terms of report timing for the subset of resources to higher layer, when the UE is triggered for periodic transmission, it is expected both periodic-based and contiguous partial sensing would be jointly performed by the UE for the formulation of the resource subset. And as such, there would be just one report timing that follows the periodic-based partial sensing. On the other hand, when the UE is triggered for aperiodic transmission, the formulation and reporting of the resource subset to higher layer would be just after the contiguous partial sensing and taking into account of necessary UE processing time.

### Proposals before 1st check point (April 15th)

Proposal 3:

- UE performs contiguous partial sensing when a resource (re)selection procedure is triggered for either periodic or aperiodic transmission and UE is configured to perform partial sensing by higher layer.

  - For periodic transmission, n+TA = ty0SL-31 and n+TB = ty0SL- Tproc,0SL-Tproc,1SL, where ty0SL is the slot index of the first selected Y candidate slots.

  - For aperiodic transmission, it is up to UE implementation to select an integer value for TA within [0, 1], and an integer value for TB within [0, 32] not smaller than TA.

- For contiguous partial sensing, the timing for formulating and reporting the subset of resource to higher layer is

  - the same as periodic-based partial sensing when resource (re)selection procedure is triggered for periodic transmission, and

  - n+TB+Tproc,0SL when resource (re)selection procedure is triggered for aperiodic transmission.

| Company | Comments |
| --- | --- |
| NTT DOCOMO | On the 1st part, - for periodic TX, we are supportive of this direction, but we wonder n+TB can includes the slot of ty0SL- Tproc,0SL-Tproc,1SL. I think ty0SL-(Tproc,0SL+1)-Tproc,1SL is correct one since in Rel-16 the sensing window [n-T0,n-Tproc,0SL) does not include slot n-Tproc,0SL. This means that actual sensing window is [n-T0,n-Tproc,0SL+1]. Current proposal seems UE is required to support severer processing time. - for aperiodic TX, T_A is OK but T_B is not OK. T_B should be large sufficiently, otherwise, UE can select 0 as T_B and hence no slot is monitored as contiguous partial sensing. It is OK as random selection but certain rule to decide T_B should be introduced. We do not support completely up to UE. On the send part, as commented above n+TB+(Tproc,0SL+1) would be correct. |
| Xiaomi | We are supportive to set n+TA = ty0SL-31. However, similar as comments in 3.1, we do not see the need to delay the timing to report the candidate resource set, and suggest to keep the principle that resource (re)selection is done when it is triggered. In addition, for periodic transmission we do not think seting n+TB = ty0SL- Tproc,0SL-Tproc,1SL would make it necessary to report candidate set at n+TB+Tproc,0SL. UE can still report candidate resource set as usual, i.e. at slot n, and keep sensing until n+TB for re-evaluation and preemption. Finally, for aperiodic transmission, the same principle as for periodic transmission should be applied, i.e. n+TA = n+T1-31 and n+TB = T1- Tproc,0SL-Tproc,1SL. We can further discuss how to treat the situation if PDB cannot be satisfied. |
| OPPO | - 1st main bullet: - 1st sub-bullet: OK - 2nd sub-bullet: No. For aperiodic traffic, when to start continuous partial sensing should be determined by PDB of the data packet. If the PDB is very small, UE can do very short sensing, or even extremely no sensing (TA=TB=0), i.e., does resource selection randomly and avoid potential collision based on re-evaluation/pre-emption checking. If the PDB is large, there is no necessary to limit the value of TB. For example, if there is periodic based sensing results for Y slot within selection window, TB can be determined by n+TB = ty0SL- Tproc,0SL-Tproc,1SL, instead of limited by 32. - 2nd main bullet: no necessary to specify that. As commented in proposal 1, when to report the sensing results is left to UE implementation. |
| vivo | For the first bullet, We agree that the value of n+TA should be ty0SL-31. However, n+TB can be further extended to the end of Y slots subjecting to the processing time - For periodic transmission, n+TA = ty0SL-31 and n+TB = n+ty1SL- Tproc,0SL-Tproc,1SL, where ty0SL is the slot index of the first selected Y candidate slots, ty1SL is the slot index of the last slot in the selected Y candidate slots. Regarding the 2nd sub-bullet for aperiodic transmission, it seems the proposal allows the combination that TA=0 and TB>0. However, since the UE cannot predict slot n triggering resource (re)selection for an aperiodic packet, we understand that TB must also be 0 when TA=0, i.e., the UE performs random resource selection. There is no need to set TA to 0 if sensing is expected. Second, if sensing is expected for such aperiodic transmission, there is no need to restrict n+TB to <32 slots and it can be determined in the same way as periodic TX case. Moreover, to ensure the reliability of the contiguous sensing-based RA, the number of sensing samples in the sensing window should not be too small, otherwise, it may result in a higher possibility of collision, which not only degrades the PRR performance but also increases unnecessary energy consumption due to retransmission, so we propose to introduce a minimum window size, i.e. min(TB-TA) - For aperiodic transmission, if TB-TA≠0, (TB-TA) should be no smaller than a (pre-)configured value M. it is up to UE implementation to select an integer value for TA within [0, 1], and an integer value for TB within [0, 32] not smaller than TA. For the second bullet, we are not still convinced that the timing for reporting should be specified. |
| Intel | 1st main bullet - 1st sub-bullet: Do not support. - Definition of sensing window for periodic transmission should not depend on the first candidate resource rather than defined with respect to ‘n’ only. Duration of sensing window should also depend on resource selection window duration. We also would like to discuss where wake up time from sleep state is absorbed. - 2nd sub-bullet: Do not support - TB should be dependent on time instance of last retransmission or HARQ feedback 2nd main bullet - Do not support. Propose to leave up to UE implementation |
| CMCC | Regarding the 1st sub-bullet of the 1st main bullet, we think that the slot index should not be limited to the first select Y candidate slots, we propose to replace ty0SL to all indices of the selected Y slots tySL. The reason is that the Y candidate slots are chosen by UE implementation, and it may be scattered, i.e., the interval between two adjacent candidate resources is longer than 32 slots. In such a case, to avoid the impact of aperiodic reservations, the contiguous partial sensing should be able to perform before every tySL. Regarding the condition of the 1st and 2nd sub-bullets of the 1st main bullet (i.e, “for periodic transmission” and “for aperiodic transmission”), similar as our comment to Proposal 2, we don’t think the configuration of the contiguous partial sensing occasion should be related to the traffic type to be transmitted by the Tx UE. |
| Fujitsu | We are fine with the 1st bullet and the 1st corresponding sub-bullet, but for the 2nd sub-bullet we propose to do the modifications below to distinguish the cases for UE performs random selection and contiguous partial sensing: - For aperiodic transmission, it is up to UE implementation to select an integer value for TA within [0, 1], and when TA is selected as “0”, TB should also be “0” which means UE will perform random selection; When TA is selected as “1”, an integer value for TB should be determined within [1, 32] not smaller than TA. We are fine with the 2nd bullet and the 1st corresponding sub-bullet; For the 2nd sub-bullet, we also propose to do the modifications as following to make the subset reporting time clearer: - not earlier than n+TB+Tproc,0SL when resource (re)selection procedure is triggered for aperiodic transmission. |
| Panasonic | We are ok with the first main bullet. We think there’s no need for the second main bullet that restricts the reporting time. |
| LGE | In general, sensing for resource selection consists of two steps – step 1) determination of candidate resources within a resource selection window, and step 2) determination of a sensing window required to identify idle resources among the candidate resources. With this in mind, the following modification is suggested for the 1st bullet. - For aperiodic transmission, n+TA = max⁡(n,t0SL-31) and n+TB = t0SL- Tproc,0SL-Tproc,1SL, where t0SL is the first slot index within a selection window, and TB is not smaller than TA. For 2nd bullet, considering no specification text regarding the reporting timing in LTE-V2X partial sensing, the same rule is used for Rel.17 SL, as in response to Proposal 1. - No specification on the timing of reporting a subset of resources to higher layer. |
| Samsung | 1st main bullet: At first, we consider the wording of “periodic” and “aperiodic” transmissions are inaccurate and invisible by PHY layer. We prefer the wording in Proposal 2, i.e. “whether a TX UE is able to know in advance / expect a resource (re)selection will be triggered in slot n”. Secondly, compared with associating sensing window with traffic type at the early stage, we prefer only to define the position of two types of sensing window (before and after triggering slot), and then further study the condition of using each type. Sub-bullet 1.1: Since the determination of ty0SL is up to UE’s implementation, we prefer to determine the sensing window based on the start of the resource selection window, i.e. n+TA = n+T1-31 and n+TB = n+T1- Tproc,0SL-Tproc,1SL. Wherein, n+T1 is the first logical slot in the resource selection window. In addition, n+TB can be further restricted as no later than trigger slot n and satisfies the restriction of processing delay. Sub-bullet 1.2: Not support. We consider the sensing window could follow similar rule of sub-bullet 1.1, i.e. up to 31 slots before the resource selection window, but no earlier than trigger slot n i.e. zero/positive TA, and TB should satisfy PDB requirement. 2nd main bullet: Similarly as commented in proposal 1, we consider the restriction on timing is unnecessary and should be up to UE implementation. |
| NEC | - 1st bullet: - 1st sub bullet: We agree with n+TA = ty0SL-31 and n+TB = ty0SL- Tproc,0SL-Tproc,1SL without periodic transmission condition. In our view, no matter the traffic is periodic or aperiodic, the Y candidates slot will be selected for partial sensing and a CPS sensing window of [ty0SL-31, ty0SL- Tproc,0SL-Tproc,1SL] is better to have. - 2nd sub bullet: Another CPS window derived based on trigger slot n could be based on transmission type - If it's periodic transmission, n is predicable, due to some small P and later Y, the possible reservation slots may locates after slot n, in this case , TA = negative T_proc,0 to extend the sensing window end time n - T_proc,0. - If it's aperiodic transmission, n could be a positive time determined by UE to start CPS after slot n - As for TB in both periodic transmission and aperiodic transmission, it could be n+TB = ty0SL- P,, P is the smallest reservation period supported in the pool in slots. And also can restricted by n+TB = ty0SL- Tproc,0SL-Tproc,1SL - 2nd bullet: only one report timing should be supported and only one sensing result should be reported to MAC layer - the same as periodic-based partial sensing when resource (re)selection procedure is triggered for periodic transmission only periodic-based partial sensing is performed, and - n+TB+Tproc,0SL when both periodic-based partial sensing and contiguous partial sensing are performed resource (re)selection procedure is triggered for aperiodic transmission. |
| Lenovo, Motorola Mobility | Agree. BTW, we also want to clarify the slot index in this discussion is logic slot index. |
| Spreadtrum | 1st bullet - 1st sub-bullet: agree. - 2nd sub-bullet: Do not agree - First, it is up to UE implementation to select an integer value for TA within [0, 1]. Second, when“0” is selected, it means random selection will performed, so TB should also be “0”. When“1” is selected, TB should be satisfied with n+TB =ty0SL- Tproc,0SL-Tproc,1SL. 2nd bullet: Do not agree. - The timing for formulating and reporting the subset of resource should be up to UE implementation. |
| Ericsson | We do not agree with this proposal and we propose some modifications. First bullet, we would like to ask why the contiguous sensing window is always active during 32 slots. This may be necessary in some cases, but in others (e.g., low load) it is detrimental to power saving. For the second bullet, this behaviour cannot be left up to UE implementation since the sensing operation of one UE may also affect the peer UEs, i.e., its capability of detecting a potential collision, and should be adapted based on channel conditions. We have shown in our contribution the advantages of this adaptation in terms of PRR and power saving. Therefore, for this second bullet we propose to modify the proposal as: - For aperiodic transmission, it is up to a UE implementation to selects an integer value for TA within [0, 1], and an integer value for TB within [0, 32] not smaller than TA based on channel conditions. - FFS the conditions/parameters to adapt the values TA and TB, e.g., HARQ feedback, CR/CBR parameters. |
| Huawei, HiSilicon | As we have explained in Q2, which partial sensing is performed, periodic based or contiguous, is not related to what traffic type UE will transmit, aperiodic or periodic. The purpose of performing sensing is detecting the reservation from others and selecting resource with limited interference. For period based partial sensing, it is used to find periodic reservation and for contiguous partial sensing, it is going to find aperiodic reservation, but they are not associated with the traffic type. Even for aperiodic transmission, periodic based partial sensing should be also performed if it is enabled for the resource pool. - For the first main bullet, - 1st sub-bullet and 2nd sub-bullet, sensing is to detect other UE’s reservation, which can be either periodic or aperiodic, i.e. detection of incoming SCIs is not related to the UE’s upcoming transmission type. Hence regardless of type of transmission, the sensing window should be same. In this case, we agree that n+TA = ty0SL-31 and n+TB = ty0SL- Tproc,0SL-Tproc,1SL, because ty0SL is the first possible selected resource, hence only most relevant 31 slots prior to ty0SL is needed to detect aperiodic reservations. For the second bullet and its associated sub-bullets, same as in first bullet, type of transmission is not relevant to detection of reservation from other UEs which can be either periodic or aperiodic. Hence the timing on report of SA is same for periodic and aperiodic transmission, which should be ty0SL-Tproc,1SL, because ty0SL is the first potential slot for transmission instead of T1, the SA reporting time does not need to be at slot n as in full-sensing, hence it should be ty0SL-Tproc,1SL. Proposal: - In a resource pool (pre-)configured with at least partial sensing, if UE performs contiguous partial sensing and resource (re-)selection is triggered in slot n: - When periodic reservation is enabled in a resource pool, n+TA = ty0SL-31 - When periodic reservation is disabled in a resource pool, n+TA = max(n, ty0SL-31) - n+TB = ty0SL- Tproc,0SL-Tproc,1SL - where ty0SL is the slot index of the first selected Y candidate slots. |
| CATT, GOHIGH | Regarding the 1st bullet, we have a concern on the 2nd sub-bullet of 1st bullet. As provided in our contribution, the sensing results could be from the periodic-based partial sensing results, sensing results in DRX_on duration, and contiguous partial sensing results for another packet transmission, therefore both TA and TB could be negative values. Even there is no available sensing results before n, the Ta can not be 0, since there is a processing time for trigger sensing operation and RF tuning time, Ta should be larger than 0. Additionally, from our understanding, regardless of values of Ta and Tb, there should be a minimum sensing duration to ensure the sufficient sensing results. Regarding 2nd bullet, from main bullet, we think it should be resource re-selection trigger time, since current spec has no definition of the timing for resource set formulating and reporting. |
| Fraunhofer | We are in general supportive of the FL’s proposal for the 1st main bullet, but agree with Ericsson’s question as to why this sensing window has to be contiguous. We feel that the 2nd main bullet can be up to UE implementation. |
| ETRI | Regarding the first part, - for periodic transmission, it should be considered the relationship with periodic-based partial sensing. For example, for periodic-based partial sensing, if a subset of periodicities is configured for Preserve and the subset of periodicities are above a certain threshold, the remaining periodicities which are below the certain threshold should be also considered. Therefore, n+TA = ty0SL-X where X can be the threshold. In case of n+TB, it could be beyond the proposed value considering pre-emption and re-evaluation. - for aperiodic transmission, TA is OK. However, TB could larger than 32 also considering pre-emption and re-evaluation. Regarding the second part, it is up to UE implementation. |
| Apple | For the first bullet: First sub-bullet: Even if UE has periodic transmission (traffic), it is possible that UE does not perform periodic-based partial sensing (e.g., for power saving purpose). For example, UE may perform random resource selection or only contiguous partial sensing (without periodic-based partial sensing) even if it has periodic traffic. In this case, we do not have t_{y0}. Hence, the first sub-bullet needs to be extended to cover the case where periodic-based partial sensing is not performed. In our view, TA=-31 and TB=-Tproc,0 in this case. Second sub-bullet: Even if UE has aperiodic transmission (traffic), it is possible that UE performs periodic-based partial sensing (if the conditions in Proposal 2 are satisfied). This periodic-based partial sensing allows UE to detect other UE’s periodic reservations. This case also needs to be considered. In this case, TA and TB could be depending on t_{y0}, where t_{y0} is the slot index of the first candidate slots resulting from the periodic-based partial sensing. For example, n+TA=max{1, t_{y0}-31}. Also, we prefer to at least specify the value of TA, rather than left for UE implementation. For the second bullet: Depending on the discussions of last bullet in Proposal 1, we may not need the first sub-bullet here. |
| Qualcomm | - “UE is configured to perform” in the main bullet: It is not clear how the UE is configured or which entity configures the UE. - “n+TA = ty0SL-31”: Instead of fixing the offset to 31, 31 is set as the maximum value. The mechanism for the selecting the exact value can be FFS for now. - n+TA = ty0SL-31 and n+TB = ty0SL- Tproc,0SL-Tproc,1SL: - Sensing for the purpose of re-evaluation and pre-emption checking is still done after n+TB - Tproc,1SL is the upper bound on the UE processing time; this should be replaced by T1 - For aperiodic transmission: - It should be clarified that this is addition to reusing all available sensing results based on the previous agreement. - For TA, it is not clear if the value of 0 is feasible. It is also too soon to define a maximum value at this stage. - Setting the values for TA and TB can be left as an FFS for now. - “the same as periodic-based partial sensing when resource (re)selection procedure is triggered for periodic transmission”: We are not sure what this means. Please clarify. We would like to add that not all the slots within [n+TA,n+TB] are available for selection. In our view, only the slots in the intersection of [n+TA,n+TB] and the periodic partial sensing occasions are in the set of candidate slots. |
| Futurewei | For item 1 of the first bullet, we suggest n+TA >= ty0SL-31, as sensing window can be smaller than 31, which is consistent to item 2 of this bullet. For both sub-bullets, we suggest specifying a minimum window size, i.e., TAmin for item 1 and TBmin for item 2, which can be fixed or configurable, to ensure reliable sensing results. TAmin or TBmin value is FFS. With current item 2, TBmin is 1 and sensing window is 1 slot, which is not a good choice. Furthermore, for item 1, Y can be larger than 31 and T2 can be larger than n+TB+31. UE can continue sensing as needed. We can actually specify TBmin too, possibly with a TBmax, for both item 1 and 2. From the discussions in the last meeting, TA=TB=0 is for indicating the random resource selection. This case shall be discussed with a separate bullet as the main bullet 1 clearly states “UE performs contiguous partial sensing” For item 2 of the 2nd bullet, since UE can continue the sensing and perform resource selection on a slot later than n+TB+Tproc,0SL , in conjunction with the comments above, we suggest change it to “n+TB,min+Tproc,0SL is the earliest slot for resource (re)selection procedure for aperiodic transmission.” |
| Nokia, NSB | For the 1st main bullet, the first sub-bullet for periodic transmission is okay, although the main purpose of contiguous partial sensing is for aperiodic traffic. The 2nd sub-bullet for aperiodic transmission, which is the major purpose of contiguous partial sensing, shall be left for implementation. At least n+T_A shall be specified as n+TA=max(n, t0SL-31). For the 2nd main bullet, this resource report timing is not needed. |
| Bosch | In general, for periodic reservation, we don’t support that the UE performs contiguous partial sensing only. Additionally, we have the same concern as Ericsson, why should the why the contiguous sensing window is always active during 32 slots? In our previous contribution (R1-2101400) we proposed that the sensing window is function of many factors including channel load, etc.. Additionally, we do not support the second main bullet. The timing for formulating and reporting the subset of resource to higher layer should be left to UE implementation. |
| ZTE | We wonder if the first sub-bullet in bullet one is applied, how is the coordination between MAC and PHY layer timing supposed to be done? As we know the Y candidate resources are up to UE implementation in PHY layer. We suggest to follow the similar logic as LTE V2X or Rel-16, that when to do contiguous partial sensing, when to report the sensing result is triggered by MAC layer. For bullet 2, we think sub-bellet 1 is reasonable, but for sub-bullet 2, a MAC layer trigger can be defined for the reporting purpose, similar as periodic transmission. |

### Proposals before 2nd check point (April 19th)

FL observations and comments based on inputs received in Sec. 3.3.1:

- 1st Bullet:

  - Consider the wording of “periodic” and “aperiodic” transmissions are inaccurate and invisible by PHY layer. We prefer the wording in Proposal 2, i.e. “whether a TX UE is able to know in advance / expect a resource (re)selection will be triggered in slot n”. [SS]

  - 1st sub-bullet (periodic sensing window):

    - Do not see the need to delay the timing to report the candidate resource set, and suggest to keep the principle that resource (re)selection is done when it is triggered. [Xiaomi]

    - n+TB can be further extended to the end of Y slots subjecting to the processing time. [vivo]

    - Definition of sensing window for periodic transmission should not depend on the first candidate resource rather than defined with respect to ‘n’ only. Duration of sensing window should also depend on resource selection window duration. We also would like to discuss where wake up time from sleep state is absorbed. [Intel]

    - the slot index should not be limited to the first select Y candidate slots, we propose to replace ty0SL to all indices of the selected Y slots tySL. The reason is that the Y candidate slots are chosen by UE implementation, and it may be scattered, i.e., the interval between two adjacent candidate resources is longer than 32 slots. In such a case, to avoid the impact of aperiodic reservations, the contiguous partial sensing should be able to perform before every tySL. [CMCC]

    - Since the determination of ty0SL is up to UE’s implementation, we prefer to determine the sensing window based on the start of the resource selection window, i.e. n+TA = n+T1-31 and n+TB = n+T1- Tproc,0SL-Tproc,1SL. n+TB can be further restricted as no later than trigger slot n and satisfies the restriction of processing delay. [SS]

    - Not dependent on traffic type (periodic or aperiodic) [NEC]

    - n+TA = ty0SL-X where X can be the threshold. In case of n+TB, it could be beyond the proposed value considering pre-emption and re-evaluation. [ETRI]

    - Instead of fixing the offset to 31, 31 is set as the maximum value. The mechanism for the selecting the exact value can be FFS for now. [QC]

    - Sensing for the purpose of re-evaluation and pre-emption checking is still done after n+TB [QC]

    - Tproc,1SL is the upper bound on the UE processing time; this should be replaced by T1 [QC]

    - Suggest n+TA >= ty0SL-31, as sensing window can be smaller than 31. Suggest specifying a minimum window size, i.e., TAmin for item 1 and TBmin for item 2, which can be fixed or configurable, to ensure reliable sensing results. TAmin or TBmin value is FFS. [FW]

    - FL: Similar to periodic-based partial sensing discussion, some prefers the end of partial sensing is at the triggering slot (n) and other thinks it can extend beyond slot n+TB to the end of Y candidate slots for re-evaluation and pre-emption checking purposes. Note, it is probably better to discuss the sensing window for re-evaluation and pre-emption purposes later as the checking is associated to the individual selected resources which is different from sensing for the Y candidate slots. And their triggering time slots are different as well.

At the same time, there are also proposals for enhancement to allow a sensing window of maximum 32 slots or define a minimum length.

Also, similar to periodic-based partial sensing discussion in Topic #2, contiguous partial sensing should not be dependent on the traffic type to define the sensing window.

  - 2nd sub-bullet (aperiodic sensing window):

    - It is OK as random selection but certain rule to decide T_B should be introduced. We do not support completely up to UE. [DCM]

    - if there is periodic based sensing results for Y slot within selection window, TB can be determined by n+TB = ty0SL- Tproc,0SL-Tproc,1SL, instead of limited by 32. [OPPO]

    - For aperiodic transmission, if TB-TA≠0, (TB-TA) should be no smaller than a (pre-)configured value M. it is up to UE implementation to select an integer value for TA within [0, 1] [vivo]

    - TB should be dependent on time instance of last retransmission or HARQ feedback [Intel]

    - Configuration of the contiguous partial sensing occasion should not be related to the traffic type to be transmitted by the Tx UE. [CMCC]

    - For aperiodic transmission, it is up to UE implementation to select an integer value for TA within [0, 1], and when TA is selected as “0”, TB should also be “0” which means UE will perform random selection; When TA is selected as “1”, an integer value for TB should be determined within [1, 32] not smaller than TA. [Fujitsu]

    - For aperiodic transmission, n+TA = max⁡(n,t0SL-31) and n+TB = t0SL- Tproc,0SL-Tproc,1SL, where t0SL is the first slot index within a selection window, and TB is not smaller than TA. [LGE]

    - We consider the sensing window could follow similar rule of periodic Tx [SS]

    - when“0” is selected, it means random selection will performed, so TB should also be “0”. When“1” is selected, TB should be satisfied with n+TB =ty0SL- Tproc,0SL-Tproc,1SL. [Spreadtrum]

    - For aperiodic transmission, it is up to a UE implementation to selects an integer value for TA within [0, 1], and an integer value for TB within [0, 32] not smaller than TA based on channel conditions. [Ericsson]

      - FFS the conditions/parameters to adapt the values TA and TB, e.g., HARQ feedback, CR/CBR parameters.

    - n+TA = ty0SL-31 and n+TB = ty0SL- Tproc,0SL-Tproc,1SL [HW, HiSi]

    - Ta can not be 0, since there is a processing time for trigger sensing operation and RF tuning time, Ta should be larger than 0. Additionally, from our understanding, regardless of values of Ta and Tb, there should be a minimum sensing duration to ensure the sufficient sensing results. [CATT]

    - TB could larger than 32 also considering pre-emption and re-evaluation. [ETRI]

    - It should be clarified that this is addition to reusing all available sensing results based on the previous agreement. For TA, it is not clear if the value of 0 is feasible. It is also too soon to define a maximum value at this stage. Setting the values for TA and TB can be left as an FFS for now. [QC]

    - Suggest specifying a minimum window size, i.e., TAmin for item 1 and TBmin for item 2, which can be fixed or configurable, to ensure reliable sensing results. TAmin or TBmin value is FFS. [FW]

    - n+T_A shall be specified as n+TA=max(n, t0SL-31). [Nokia, NSB]

    - FL: Similar to the periodic transmission case, the same fundamental questions related to whether contiguous partial sensing can be performed beyond the triggering slot n and whether the sensing window will be dependent on the transmission type. Without considering other proposals for enhancements of introducing a min winow or adaptive window, these fundamental questions should be resolved first. I think the answers will be clear once Proposal 1-1 and 2-1 are resolved.

Therefore, this topic should not pursuit anylonger in this meeting.

- 2nd Bullet:

  - Leave it up to UE implementation. [OPPO, vivo, Intel, Pana, LGE, SS, Spreadtrum, CATT, GOHIGH, Fraunhofer, ETRI, Nokia, NSB, Bosch]

  - FL: similar to Proposal 1, there is overwhelming view that the report timing of a subset of resources to higher layer should not be specified and leave it up to UE implementation. It is now removed. It should be sufficient to specify the sensing and selection windows in order for the UE to determine on its own the reporting time.

## Topic #4: Contiguous partial sensing – Resource selection window

Background: In the last RAN1#104-e meeting, only the overall framework of monitoring slots for contiguous partial sensing was defined. However, the resource selection window (RSW) for the resource (re)selection procedure when the contiguous partial sensing is used has not yet been defined.

When resource (re)selection is triggered for periodic transmission, it is expected that both periodic-based partial sensing and contiguous partial sensing are both performed by the UE and only one subset of resources needs to be reported to the UE. And therefore, the candidate resource set (SA) should be initialized for the selected Y candidate slots for both periodic-based partial sensing and contiguous partial sensing operation.

On the other hand, when resource (re)selection is triggered for aperiodic transmission, although the maximum sensing window will only need to be 32 slots to account for aperiodic reservation, but it seems no reason why the resource selection window should be different from the full sensing operation specified in R16 NR-V2X.

### Proposals before 1st check point (April 15th)

Proposal 4:

- Resource selection window to be used as part of contiguous partial sensing is defined as:

  - For periodic transmissions: the same Y candidate slots selected during periodic-based partial sensing operation

  - For aperiodic transmissions: [n+T1, n+T2]

    - T1 and T2 are defined in the same way as in R16 NR-V2X according to step 1 [TS 38.214 Sec. 8.1.4]

    - FFS any further restriction on the time interval to limit the sparse resource selection by MAC layer or confined within a selected/configured resource set.

| Company | Comments |
| --- | --- |
| NTT DOCOMO | We are supportive of same T1 and T2 as Rel-16. Meanwhile we think enhancement to limit sparse resource selection should be supported and it is not only for aperiodic transmission and contiguous partial sensing but also for periodic transmission and periodic-based partial sensing. In addition, a possible solution is to restrict how to select Y candidate slots. So the last FFS should be out of the main bullet and be updated as follows: - FFS: any further restriction on the time interval to limit the sparse resource selection by MAC layer or confined within a selected/configured resource set or restriction on a set of Y candidate slots. |
| Xiaomi | For aperiodic transmission, the value of T1 should be based on the sensing behaviour of UE. As we discussed in 3.3, T1 should be set such that UE can satisfy the contiguous partial sensing requirements. |
| OPPO | Support the proposal. |
| vivo | Impact of SL DRX on the determination of Y slots should be specified, i.e., the Y slots should be overlapped or partially overlapped with the DRX on duration. - Resource selection window to be used as part of contiguous partial sensing is defined as: - For periodic transmissions: the same Y candidate slots selected during periodic-based partial sensing operation - For aperiodic transmissions: [n+T1, n+T2] - T1 and T2 are defined in the same way as in R16 NR-V2X according to step 1 [TS 38.214 Sec. 8.1.4] - FFS any further restriction on the time interval to limit the sparse resource selection by MAC layer or confined within a selected/configured resource set. - Y candidate slots in the selection window should be overlapped or partially overlapped with the SL DRX on duration when SL DRX configuration of target destination (i.e., broadcast, groupcast, unicast UE) is known |
| Intel | 1st sub-bullet – Propose FFS 2nd sub-bullet - Support |
| CMCC | First, we have concerns on the condition to the configuration of the resource selection window of the contiguous partial sensing. Please refer to our comments to Proposal 3. In addition, we believe that the resource selection window should consider the impact of SL DRX of the Rx UE. |
| Fujitsu | We are OK with this proposal. And we agree Vivo’s view that “the selection window should be overlapped or partially overlapped with the SL DRX on duration” needs to be added. |
| Panasonic | Similar to our comments in 3.1, we think to allow a shortened window is beneficial (as an alternative option). |
| LGE | In conjunction with our response to Proposal 1, the following modification is suggested for the 2nd sub-bullet e.g. to adapt to SL-DRX operation. - For aperiodic transmissions: [n+T1, n+T2] T1 and T2 are defined in the same way as in R16 NR-V2X according to step 1 [TS 38.214 Sec. 8.1.4] T1 and T2 are UE implementation as long as 0≤T1<T2≤remaining PDB. |
| Samsung | Similarly as commented in proposal 3, we consider the wording of “periodic” and “aperiodic” transmissions are inaccurate and should be modified, and it is unnecessary to link sensing window to traffic type at the early stage. We prefer to divide contiguous partial sensing as sensing before and after slot n and further discuss corresponding conditions. For contiguous partial sensing before slot n, we think the relationship between periodic-based and contiguous partial sensing, e.g. whether always coexist, needs to be clarified at first. For contiguous partial sensing after slot n, T1 may need to be further modified based on conclusion of TA and TB in proposal 3. |
| NEC | In our understanding, the principle to select Y candidate slots should be respected regardless the transmission is periodic or aperiodic. Then we conduct CPS after n to refine sensing result of the Y slots; if aperiodic transmission exist, UE also conduct PBPS to help to refine the sensing result of Y slots. |
| Lenovo, Motorola Mobility | We don’t agree with the first bullet. We support, for periodic transmissions, different Y candidate slots can be selected during periodic-based partial sensing operation. This is because that various period traffics have different payload ratios in a resource pool. If different Y candidate slots can be selected/determined for sensing occasions for various types of period traffic, the short sensing occasion for some period traffic with low payload ratio can reduce the power consumption of UE and the long sensing occasion for some period traffic with high payload ratio will reduce the collision probability. Network can help to configure multiple minimum Y candidate slots in responding to various period traffics respectively per resource pool. |
| Spreadtrum | We are fine with this proposal. |
| Ericsson | We would like to ask for clarification for the first point “for periodic transmissions” and whether the distinction between periodic and aperiodic transmissions is based on the type of transmission performed by the UE or based on whether periodic reservation is enabled in the resource pool. We suggest removing the FFS in the second point since no other restrictions are needed for the resource selection window apart from the procedure specified in NR Rel-16. |
| Huawei, HiSilicon | The current categorization is incorrect. The purpose of sensing is to detect other UE’s periodic or aperiodic reservation, so the Tx UE’s transmission is periodic or aperiodic is irrelevant. In RAN1#104-e, we already defined Y and selection window [n+T1, n+T2] for UEs performing periodic-based partial sensing. We can use similar wording for the contiguous partial sensing case as the following proposal shows. Agreements from RAN1#104e: In a resource pool (pre-)configured with at least partial sensing, if UE performs periodic-based partial sensing, at least when the reservation for another TB (when carried in SCI) is enabled for the resource pool and resource selection/reselection is triggered at slot n, it is up to UE implementation to determine a set of Y candidate slots within a resource selection window, where - FFS condition(s) and timing(s) for which periodic-based partial sensing is performed by UE - The resource selection window is [n+T1, n+T2] - As a baseline, T1 and T2 are defined in the same way as in R16 NR-V2X according to step 1 [TS 38.214 Sec. 8.1.4] - … Proposal: - In a resource pool (pre-)configured with at least partial sensing, if UE performs contiguous partial sensing and resource (re-)selection is triggered in slot n, it is up to UE implementation to determine a set of Y candidate slots within a resource selection window, where - The resource selection window is [n+T1, n+T2] - T1 and T2 are defined in the same way as in R16 NR-V2X according to step 1 [TS 38.214 Sec. 8.1.4] |
| CATT, GOHIGH | We have a concern on the 2nd sub-bullet. As comment in topic#3, we think the resource re-selection trigger time is changed in contiguous partial sensing. So we think the resource reselection time for contiguous partial sensing should be replaced by n’, i.e. [n’+T1, n’+T2] . and n’ definition could be depended on the output of topic#3. |
| Fraunhofer | We are supportive of the FL’s proposal. |
| ETRI | We support the proposal. |
| Apple | For aperiodic transmissions, UE needs to perform contiguous partial sensing after resource selection trigger at n. Hence, the values of T1 and T2 needs to be larger than TB. Also, we agree with vivo’s view that impact of SL DRX on the determination of Y slots should be specified. |
| InterDigital | - 1st bullet - 1st sub-bullet: Support - 2nd sub-bullet: It can be up to UE implementation to select the window, but the minimum TB (i.e., TBmin) should be required to gurantee the reliability of the sensing result. Therefore, we propose to modify the 2nd sub-bullet as follow: - For aperiodic transmission, it is up to UE implementation to select an integer value for TA within [0, 1], and an integer value for TB within [0 TBmin, 32] not smaller than TA. - FFS how to determine TBmin - 2nd bullet - Similar to our comment in proposal 1, it can be up to UE implementation when to report the sensing result before resource selection window. |
| Qualcomm | - The last FFS is applicable to both periodic and aperiodic transmissions. |
| Futurewei | For first item, as commented for the proposal 3, Y can be larger than 31. So similar to the FFS for 2, we suggest also add FFS any further restriction on the time interval to limit resource selection in Y for some conditions. One scenario is: Y>31, the contiguous sensing detect a lot of reservations in the effective region in Y, i.e, the first 31-Tproc,0,-Tproc,1 slots, while periodic partial sensing only identify a small number of resource. The sensing results based on the periodic partial sensing on the slots after the first 31-Tproc,0,-Tproc,1 slots in Y are not reliable. Both FFS items should be discussed jointly sensing procedures/timings in the proposal 3. |
| Convida Wireless | - We are fine with the FL’s proposal. |
| MediaTek | - We support this proposal |
| Nokia, NSB | - This proposal is okay. |
| Bosch | - For the first main bullet, we support adding “FFS whether”. - We support the second main bullet. |
| ZTE | - We prefer same mechanism for both periodic and aperiodic transmission. The FFS part for the aperiodic transmission is not needed. |

### Proposals before 2nd check point (April 19th)

FL observations and comments based on inputs received in Sec. 3.4.1:

- FL: Since this topic is strongly tied with topic #3 (especially with the TB value), which is not pursuit anylonger, this topic should be also postponed to future meeting once those fundamental questions are answered. Furthermore, some companies have a view that the selection window should jointly considered with SL DRX configuration, for which the operation is still yet unclear to us in RAN1.

## Topic #5: Random resource selection –low priority randomly selected resource

Background: One of the identified issues among the contributions submitted in this meeting on random resource selection was about the mix of different RA schemes operating in a same resource pool. In particular when random resource selection is allowed and coexist in a same resource pool with RA schemes that use full or partial sensing. One specific problem is when low priority randomly selected transmission (with no reception capability and no re-evaluation and pre-emption checking) colliding with high priority transmitted from full/partial sensing UE due to mixed configuration of full/partial/no sensing in a same pool. Since the random selected resource has lower priority than a partially/full sensing UE and the random selection UE has no SL reception/PSCCH monitoring capability (i.e., Type A and B UEs) to perform re-evaluation and pre-emption checking in this case, transmissions from random selection and full/partial sensing will collide since the ransom selection UE is not able to detect collision in advanced and perform resource (re)selection.

To resolve this issue, several options are provided in Proposal 5 based on reviewing contributions in this meeting. Note that, a combination of more than one option is possible.

### Proposals before 1st check point (April 15th)

Proposal 5:

- In a mode 2 resource pool (pre-)configured to enable random resource selection and at least one of sensing-based RA schemes:

  - Option 1: A priority threshold is configured for the resource pool, at which reduced sensing UEs can select resources in a pool configured for mixed types of RA.

  - Option 2: Increase the priority for UE with random selection and use the corresponding priority value in the priority field in SCI format 1-A.

  - Option 3: Different RSRP thresholds or increased RSRP threshold value is (pre-)configured for different resource selection scheme.

  - Option 4: The pre-emption priority for power saving UE is separately (pre-)configured from that for full sensing UE.

  - Option 5: SCI indicates at least power-saving UE or full sensing UE.

  - Option 6: Higher priority is assigned to the resources reserved by a UE performing random selection, to preserve these selected resources from being pre-empted by other UEs.

  - Option 7: Exclude resources reserved by UE performing random selection without re-evaluation / pre-emption checking, regardless of their priorities

  - Option 8: others

| Company | Option(s) | Comments |
| --- | --- | --- |
| NTT DOCOMO | Option 5+7 or Option 6 | Option 7 seems easy and beneficial to avoid collisions. To achieve this, option 5 needs to be used together. Option 6 would also be easy and beneficial. |
| Xiaomi | 1,3,5,7,8 | The priority of data transmission is from UE data traffic characteristic, and thus option 2, 4 and 6 is not preferred. We have another proposal which is not included here: A CR threshold is configured for the resource pool; a UE can select resource using random RA only if the CR measurement for its data transmission using random RA is below the threshold. |
| OPPO |  | Prefer to take it as FFS and not discussed in this meeting. More companies can think further and come back with more detailed analysis in next meeting. |
| vivo | option6 | option1 limits the candidate resources that can be used for UEs using random selection or using sensing-based RA schemes in the same resource pool, based on the priority of the transmissions, which may lead to PRR performance degradation (e.g., for UE using sensing-based RA schemes). option2/3/4/5 work only if the PSCCH/PSSCH of the UEs using random selection carries some additional information, e.g., UE priority/RA type/UE type, so that the sensing UE receiving the PSCCH or PSSCH can identify UE priority/RA type/UE type of the TX UE and perform the corresponding process to avoid the collision, but this information cannot be identified by the R16 UE and therefore still cannot reduce the conflicts between the power-saving UE and R16 UE when there is also an R16 UE in the same resource pool. Option 7 also cannot solve the collision problem when there are R16 UEs in the same pool. option 6 reuses the current SCI which can be recognized by R16 UE. Thus, it can reduce the probability of collision by setting a higher priority in SCI, even if there are R16 UEs in the pool, so option 6 is preferred. |
| Intel |  | Propose to further study this aspect. In general, we do not think that scenario with pre-emption and random resource selection is a practical one |
| CMCC | Support options 3, 5, 8 (see comments) | Option 8: UE reports whether one candidate resource overlaps with resources reserved by random resource selection UE to higher layer for further resource selection. Regarding option 2, 6, we believe that the priority of a packet should be defined by its QoS requirement, not by the RA mechanism. |
| Fujitsu | Except Option 5 and 7 | In our opinion, we think for option 5, additional indication in SCI is needed, from the perspective of backward compatibility, legacy UEs cannot identify such new introduced information and they also cannot treat the resources reserved by R-17 UE with this kind of new indication in SCI separately. Option 7 should be clarified about how a UE can identify whether the resource is reserved by a UE performing random selection. |
| Panasonic | 1 or 3 | To segregate the different RA by using different threshold seems easier to be implemented. We also support to list all options and put it as FFS for next meeting. |
| LGE | 3, 4, 5, 6 | We support option 5 combined with option 6 rather than option 2, so that the logical channel priority can be interpreted as same as in full sensing UE case. We don’t support option 1 as a packet of all priorities can be able to be transmitted through random selection. We don’t support option 7 as the pre-emption should still be based on the priority even for the random selection UE case. |
| Samsung | 1, 2, 3, 6, 7 | We suggest to modify the main bullet as: Study the following options when a SL Mode 2 Tx resource pool is (pre-)configured more than one resource allocation schemes. (Pre-)configured with random selection can be further condition of some random selection-related sub-bullets. |
| NEC | Neutral | We can keep them for FFS or postpone them to next meeting |
| Lenovo, Motorola Mobility |  | Above listed options may be depended on the other issue’s conclusion. So, we don’t think we must determine one of above options at this stage. It is better to delay this discussion in next meeting. |
| Spreadtrum |  | Suggest to take it as a lower priority in this meeting and leave it for the next meeting to further study. |
| Ericsson | Option 1, 3 and 6 | We think that these options provide the simplest implementation without any extra signalling or indication within SCI. Moreover, it is also not a good approach to assign a higher priority to random resource selection UEs since these UEs may/do not have the capability of performing re-selection, and therefore, a higher number of collisions may occur among the peer UEs. |
| Huawei, HiSilicon | Option 1 | The motivation to have a priority threshold is to control the population of reduced sensing UE in a mixed RA resource pool, which to protect both full sensing UE and reduced sensing UE with important traffic. In Rel-16, pre-emption is enabled with a configuration, and a priority threshold is used to control the pre-emptions, e.g. to be admitted to important traffics, such as URLLC-like. This principle can be re-used for random selection, only the random selection transmission with low priority value which is below the configured threshold can select the RP configured with mixed types of RA to share with sensing-based UE. Our evaluations show that, if this is not done, the mixing of random selection UEs into a pool with full sensing UEs results in a large PRR loss for the full-sensing UEs, even when the random-selection packets are low priority (high value of Priority field in SCI). |
| CATT, GOHIGH | See comment | We think it is not necessary to touch this topic until we have clear idea on the partial sensing and random selection based RA schemes. |
| Fraunhofer |  | We prefer to take this proposal as an FFS and can be discussed in the next meeting. |
| ETRI | 5 and/or 6 | It is not clear how to achieve option 7. |
| Apple | Options 3, 5 | A random resource selection UE does not perform sensing and hence, is unexpected to detect any potential resource collision. On the other hand, a full sensing UE is able to detect the potential resource collision from a random resource selection UE. Hence, it is desirable that the full sensing UE prioritizes the resource reservation from a reduced sensing UE. Specifically, in the full sensing UE’s resource allocation procedure, the RSRP threshold used to exclude a candidate resource may not only depend on data priority, but also depend on the sensing scheme used by another resource reservation UE. If another resource reservation UE preforms full sensing, then the legacy RSRP threshold is re-used. If another resource reservation UE performs reduced sensing, then a separate RSRP threshold is used by the full sensing UE in its resource allocation procedure. For example, a maximum RSRP threshold is applied for a resource reserved by a UE performing reduced sensing, or a RSRP threshold corresponding to the highest data priority of the resource reservation UE is applied if the resource reservation UE performs reduced sensing. Here, SCI could be used to indicate whether a UE is a full sensing UE or a reduced sensing UE. |
| InterDigital | All | We support to further study all options. In addition, we need to further study another option (e.g., Option 8) in which a minimum time gap between two transmissions of random resource selection is configured. This will help the UE perform sensing having enough time to perform resource re-evaluation and pre-emption if collision is detected. |
| Qualcomm | Option 9: None of the above | At this point, it is unclear if any specific enhancement is needed. |
| Futurewei | 8 | Since the issue only happens when the sensing UE detects a reserved resource by a UE with lower-priority packet (which happens to be random selection) with pre-emption after the resource selection based on sensing results, it is not clear how much performance loss if sensing UE keeps the resource due to high priority. Before any further discussions, we suggest add “no enhancement is needed” as an option. |
| Convida Wireless |  | We prefer to postpone the discussion or treat this proposal at low priority for this meeting. |
| MediaTek |  | We can agree this as FFS including all of the listed options for futher discussion in the next meetings. |
| Nokia, NSB | Option 2 | When we address the priority-based random selection, we need to consider backward compatibility with Rel-16 UEs. The resource reservation by a Rel-17 UE with random resource selection can be pre-empted by a Rel-16 UE with full sensing. But this Rel-17 UE can’t identify the situation leading to collision. We may consider increasing the priority at PHY for a Rel-17 UE with random selection and use the corresponding priority value in the priority field in the 1st-stage SCI. This can increase the capability of this Rel-17 UE to protect itself from being pre-empted by Rel-16 UEs. Accordingly, to retain proper operations between Rel-17 UEs with different resource selection schemes, an extra priority field containing the original priority value associated with QoS requirements can be added to the SCI. |
| Bosch |  | We need to study how mixing random resource selection (at least for aperiodic reservation) and sensing-based (at least for periodic) can be optimize. We agree with the opinions that we should take this proposal for the next meeting. |
| ZTE | We prefer to capture a new option 9 | We think the collision will get worse if a UE randomly selects a resource without sensing. To mitigate the effects, a reserve signal (similar as SCI) can be used before transmission on the resource which is selected randomly. So we suggest to introduce a reserve signal for random resource selection prior to transmission Option 9: A reserve signal for random resource selection prior to transmission |

### Proposals before 2nd check point (April 19th)

FL observations and comments based on inputs received in Sec. 3.5.1:

- Preferred list:

  - Option 1: Xiaomi, Panasonic, Samsung, Ericsson, HW, HiSi,

  - Option 2: Samsung, Nokia, NSB

  - Option 3: Xiaomi, CMCC, Panasonic, LGE, Samsung, Ericsson, Apple

  - Option 4: LGE

  - Option 5: DCM, Xiaomi, CMCC, LGE, ETRI, Apple

  - Option 6: DCM, vivo, LGE, Samsung, Ericsson, ETRI

  - Option 7: DCM, Samsung

- Not Preferred list:

  - Option 1: vivo, LGE

  - Option 2: Xiaomi, vivo, CMCC

  - Option 3: vivo

  - Option 4: Xiaomi, vivo

  - Option 5: vivo, Fujitsu

  - Option 6: Xiaomi, CMCC

  - Option 7: vivo, Fujitsu, LGE, ETRI

- Option 8: Others

  - Further study (more analysis is needed): OPPO, Intel, Panasonic, Samsung, NEC, Spreadtrum, CATT, GOHIGH, Fraunhofer, IDC, Convida, MTK, Bosch

  - A CR threshold is configured for the resource pool; a UE can select resource using random RA only if the CR measurement for its data transmission using random RA is below the threshold.

  - UE reports whether one candidate resource overlaps with resources reserved by random resource selection UE to higher layer for further resource selection.

  - A minimum time gap between two transmissions of random resource selection is configured.

- Option 9: None (Qualcomm, Futurewei)

FL: It is observed that there is a wide range of preferences and no preferences, and at the same time there are also many companies have not looked into this issue in detailed. Since this is the first meeting that the issue is raised and discussed, it is suggested more time for further study and evaluate all possible solutions. Companies are encourage to consider all options listed and raised in this topic and we can come back to this in a later meeting.

Proposed conclusion:

- Further study the issue of random resource selection for low priority transmission in a resource pool (pre-)configured with mixed RA schemes.

| Company | Comments |
| --- | --- |
| NTT DOCOMO | We are supportive of this conclusion. |
| Apple | Fine with the proposed conclusion. |
| Futurewei | Although we think the study is not necessary, we are ok with this proposal. |
| LGE | FL proposal suggest FFS for this issue, so it is fine for us. |
| NEC | Support |
| CMCC | Support. |
| Fujitsu | Support |
| OPPO | Support |
| Panasonic | Support |
| Huawei, HiSilicon | We are fine with the proposal itself to further study the the issue of random resource selection for low priority transmission in a resource pool (pre-)configured with mixed RA schemes However, it may be a workload concern to have so many options listed in previous round proposal making the future discussion too divergent. Therefore, we suggest we can focus on some big granularity consideration for further study, for example, control the low priority transmission in the mixed RA resource pool by priority, control the low priority transmission in the mixed RA resource pool by RSRP threshold, etc. it is better to have a scope, rather than simply to say further study. |
| Samsung | Fine to study the issue. As commented in Proposal 2-3, we prefer to combine proposal 2-3 and 5 within same high level structure to keep consistency and avoid missing possible scenarios. |
| Intel | We are OK to keep it FFS |
| Fraunhofer | We support the FL’s conclusion. |
| Xiaomi | support |
| vivo | Support in general. However, the 'low priority transmission' in the proposal is not clear, what type of transmission should be considered a low priority transmission? will a transmission with a priority value greater than a threshold be treated as a low priority transmission? This aspect should be clarified. |
| Ericsson | We are supportive of this proposal but with some modifications. We do not think that this issue should be tied only to low priority transmissions and should be studied in the general case of random resource selection in a resource pool with mixed RA schemes. Therefore, we propose to change the conclusion to: - Further study the issue of random resource selection for low priority transmission in a resource pool (pre-)configured with mixed RA schemes. Moreover, we think that we should strive to do some down-selection of the options during this meeting. In our view, Option 1, 3 and 6 should be prioritized to be studied since they provide the simplest implementation without any extra signalling or indication within SCI. |
| ZTE | Support |
| Nokia, NSB | Support |
| MediaTek | Support |
| Qualcomm | Even though we think no enhancement is required, we would be fine with RAN1 to study further. However, we suggest to treat this topic with a low priority as there is more work needed for specifycing the baseline design of the other features. In addition: - we have not identified whether an issue exists yet; - the low priority transmission is by the UEs performing random selection? If so, it should be clarified; - As we understand from the companies’ contribution, the case in question is only for the random selection UEs that do not receive on SL. Hence, we propose the following changes: - “Further study the transmission of low priority packets by UEs performing random selection without SL reception.” |
| InterDigital | Support |

### Proposals before Monday GTW session (April 19th)

FL observations and comments based on inputs received in Sec. 3.5.2:

- On Proposed conclusion

  - @HW, HiSi, Ericsson: Thanks for the suggestion as always. Since it is for further study, any narrowing down of solutions or directions before the study will always spark another round of debate of which one is or is not feasible.

  - @Samsung: the topics between P2-3 and P5 are quite different though.

  - @vivo, your question can be part of the study too and I am sure others will consider the same question as well if a threshold is applied.

  - @Qualcomm: suggestions are taken onboard.

Proposed conclusion (round II):

- Further study the issue of random resource selection (e.g., random selection for low priority transmission) by UE without PSSCH reception in a resource pool (pre-)configured with mixed RA schemes.

| Company | Comments |
| --- | --- |
| Xiaomi | Why “without PSSCH repection” is added? Should it be “without PSCCH repection” as you are considering a random RA UE cannot be preempted? FL: It’s a typo, I meant PSCCH 😊, thanks for picking this up. |
| Qualcomm | Thanks for the update. We still suggest to remove the word “issue”. It is still unclear whether there is any issue to address under the scenario considered in the proposed conclusion. Also, as pointed out by Xiaomi, PSSCH should be replaced by PSCCH. FL: I can replace it with “issue” “case” |
| Sharp | We share similar view as Qualcomm. |
| NTT DOCOMO | We do not agree the update (i.e. blue text). Now we are discussing UE that is not capable of performing re-evaluation/pre-emption check in my understading. If UE can receive PSCCH, the UE can be aware of reservation by other UE and thus re-evaluation/pre-emption check is possible. Of course random selection with re-evaluation/pre-emption has not been agreed yet, but at least the following update seems necessary: ‘by UE without PSSCCH reception’ |
| NEC | Maybe ..by UE doesn't perform sensing…is better. FL: OK for me, it seems to align with the wording used in the first meeting. |
| Samsung | The motivation of blue text “by UE without PSSCH reception” (or as commented by other companies, change PSSCH to PSCCH) is unclear to us. For UE with PSCCH/PSSCH reception capability, if multiple RA schemes are configured, it’s also possible that UE performs random selection under some conditions. Therefore we prefer to remove the wording to keep consistency between different UE capabilities. FL: I hope NEC’s wording is OK for you. |
| Huawei, HiSilicon | We are fine with the proposal. |
| Spreadtrum | “without PSSCH repection” should be replaced by “without PSCCH repection”. |
| LGE | Random resource selection can also be performed by e.g. type-C UE. Therefore we see no reason for any restriction on the UE type, so suggest the following modification. - Further study the issue of random resource selection (e.g., random selection for low priority transmission) by UE without PSSCH reception in a resource pool (pre-)configured with mixed RA schemes. FL: I hope NEC’s wording is OK for you. |
| Ericsson | We have some comments on this proposal: We propose to avoid any narrowing-down of the solutions/issues which will trigger more discussions, and therefore, propose to delete the example. Moreover, in our view, we should not limit UEs that perform random resource selection to those that do not receive PSSCH. Therefore, we propose to modify the proposal as: - Further study the issue of random resource selection (e.g., random selection for low priority transmission) by UE without PSSCH reception in a resource pool (pre-)configured with mixed RA schemes. |
| Convida Wireless | We are fine with proposal with correction on typo. |
| Fraunhofer | We are supportive of the FL’s proposal, with the following change: Further study the issue of random resource selection (e.g., random selection for low priority transmission) by Type A and B UEs without PSSCH reception in a resource pool (pre-)configured with mixed RA schemes.” |
| ZTE | The example part seems unnecessary given there is no consensus for high priority random selection either. |

### Proposals before Tuesday GTW session (April 20th)

FL observations and comments based on inputs received in Sec. 3.5.3:

- Trying to make it more agreeable for everyone.

Proposed conclusion (round III):

- Further study the issue case of random resource selection (e.g., random selection for low priority transmission) by UE without PSSCH reception does not perform sensing in a resource pool (pre-)configured with mixed RA schemes.

  - For at least the case when random resource selection for low priority transmission by UE does not perform sensing

| Company | Comments |
| --- | --- |
| Nokia, NSB | Support |
| NTT DOCOMO | We think the removed part should be back. Otherwise, this conclusion seems meaningless. The issue companies raised is that UE performing random selection transmits anytime TB with low priority without collision avoidance (re-evaluation/pre-emption check) and hence more collisions with full/partial sensing UE would happen. FL: Let’s hear more views on this. |
| InterDigital | Support |
| Futurewei | We are ok with the proposal |
| Qualcomm | By removing the text above, the conclusion is too generic; it is now unclear what aspects of the random resource selection should be further studied. We do think that this conclusion should only consider the UEs that do not perform sensing. Otherwise, they can perform re-evaluation and no further study is needed. Hence, we propose the following: - Further study the issue case of random resource selection (e.g., random selection for low priority transmission) by UE without PSSCH reception does not perform sensing in a resource pool (pre-)configured with mixed RA schemes. If there is any concern, we are also fine to not have this conclusion in this meeting. Companies can and have been bringing their views on this topic. FL: Let’s hear more views on this. |
| NEC | We prefer to add back " … does not perform sensing …" while also can accept the current version for further refined discussion. |
| Fujitsu | Support |
| Xiaomi | We are fine with FL proposal. |
| Huawei, HiSIlicon | Without the example in the parentheses, the whole proposal means nothing, RAN1 WID has clearly stated the study of random selection, it is unnecessary to repeat it again. We think the intention of this propose conclusion is going to resolve the issue that random selection for lowr priorty would possible pre-empt the reservations by high priority tranmissions. It should be clear stated. FL: Seems like more and more companies prefer to add back the deleted example. A new sub-bullet is added. |
| Ericsson | Support |
| Panasonic | We also prefer to add back the “does no perform sensing”, otherwise it will be too generic. |
| Samsung | We prefer NOT adding back the removed part “by UE without PSSCH reception does not perform sensing” since it may restrict use scenarios of this proposal. Even for a UE that has PSSCH/PSCCH reception capability and be configured with sensing, it still may determine to perform random selection e.g. due to insufficient sensing result of initial period. Therefore, a more general description is beneficial to keep consistency between different scenarios. FL: Please check the above update. |
| LGE | Fine with FL proposal. |
| Convida Wireless | We are ok with the proposal. |

### Final proposals for Tuesday GTW session (April 20th)

Proposed conclusion’ (Topic #5):

- Further study the case of random resource selection in a resource pool (pre-)configured with mixed RA schemes.

  - E.g., the case when random resource selection for low priority transmission by UE does not perform sensing

## Topic #6: Random resource selection – R16 principles of resource (re)selection

Background: Although random resource selection would be one of the easiest power saving RA schemes to be performed by a UE, some basic resource (re)selection principles from R16 should still be respected to ensure early aperiodic reservation and sufficient SL HARQ processing and PSCCH/PSSCH preparation time for both TX and RX UEs.

Tuesday GTW session: Many comments were raised during the GTW session on Tuesday for Proposal 6. Based on the received comments, the proposal was updated on the screen during the GTW session. The latest version on the screen is now captured below. However, there was still one more comment relating to “making this proposal applicable to UE performing partial sensing-based RA in general” that has not been addressed. While this comment is perfectly valid on its own that a mode 2 UE shall always follow these existing resource (re)selection R16 principles regardless of the RA scheme used, this has led me thinking “is it necessary to remake all these existing R16 resource (re)selection principles again for R17”. Therefore, the FL would like to ask the two questions in Section 3.6.1.

Proposal 6:

- R16 principles of resource (re)selection are followed when random resource selection is used by a power saving UE, particularly,

  - Maximum distance separation in logical slots is 31 for any two resources indicated by a prior SCI

    - FFS whether or not there are exceptional cases (e.g., due to re-evaluation and pre-emption), including the possibility of min distance for blind transmissions

  - HARQ feedback time gap (Z) between PSSCH-to-PSFCH-to-PSSCH is respected when HARQ feedback indicator is enabled by the higher layer for the 2nd stage SCI

    - FFS whether or not there are exceptional cases (e.g., due to blind retransmissions)

  - Re-evaluation and pre-emption checking are supported for UE receiving PSCCH

### Proposals before 1st check point (April 15th)

Questions for Proposal 6:

- Do you agree the following two R16 timing restrictions for resource (re)selection should be reused in R17 for both partial sensing and random resource selection based RA?

  - Maximum distance separation in logical slots is 31 for any two resources indicated by a prior SCI, including all exceptional cases agreed in R16.

  - HARQ feedback time gap (Z) between PSSCH-to-PSFCH-to-PSSCH when HARQ feedback indicator is enabled by the higher layer for the 2nd stage SCI.

- Is there any further enhancement needed in R17 on these two timing restrictions for power saving based RA schemes? If yes, what should be further studied?

| Company | Comments |
| --- | --- |
| NTT DOCOMO | Support a. Not support b. In our understanding, HARQ-based retransmission is not good for better power saving since UE cannot sleep until transmissions are completed. Transmission, HARQ feedback, Retransmission, HARQ feedback, ... means long active. To avoid this, mixed mechanism of HARQ-based and blind retransmissions should be supported. b precludes this, so the following update should be applied. - HARQ feedback time gap (Z) between PSSCH-to-PSFCH-to-PSSCH for HARQ-based retransmissions when HARQ feedback indicator is enabled by the higher layer for the 2nd stage SCI. |
| Xiaomi | We do not see the necessity to discuss and agree the proposal in this meeting. We are generally fine to agree the principle. However, these two principles may have some impact on the Y subframes selection for partial sensing. If possible, we'd rather suggest to discuss it after the basic partial sensing procedures are clear. |
| OPPO | 1.: yes 2.: no |
| Vivo | Yes, and these timing restrictions should also be used for both partial sensing and random resource selection based RA. No other further enhancement is needed But we are wondering any explicit agreement to confirm this aspect is needed. Without new agreement to confirm that every principle adopted in R16 is supported for R17, R17 PUE would by default support the R16 principle unless additional modifications are introduced. |
| Intel | Support original proposal. We also suggest supporting mix blind and feedback based retransmissions. |
| Fujitsu | We agree to support both of the two R16 timing restrictions to be reused in R17. We think further enhancements on them are not needed. |
| Panasonic | 1a. Yes. 1b. As pointed out by docomo, we need to consider when power saving UE goes to sleep. The time gap for power saving needs to be separately defined from a normal UE. 2. Nil |
| LGE | If we agree on the 3rd sub-bullet, the 1st sub-bullet needs to be clarified as below. - Maximum distance separation in logical slots is 31 for any two resources indicated by a prior SCI if UE does not perform re-evaluation and pre-emption checking - FFS whether or not there are exceptional cases (e.g., due to re-evaluation and pre-emption), including the possibility of min distance for blind transmissions We’d like to rephrase the 2nd sub-bullet to better capture the comments made in the call. - HARQ feedback time gap (Z) between PSSCH-to-PSFCH-to-PSSCH is respected when HARQ feedback indicator is enabled by the higher layer for the 2nd stage SCI - FFS whether or not there are exceptional cases (e.g., due to mixture of blind and HARQ feedback-based retransmissions) The responses to two FL questions are answered with the above comments. |
| Samsung | Fine with the proposal with the following updates: - HARQ feedback time gap (Z) between PSSCH-to-PSFCH-to-PSSCH is respected when HARQ feedback indicator is enabled by the higher layer for the 2nd stage SCI FFS whether or not there are exceptional cases (e.g., due to blind retransmissions) |
| NEC | - Yes, while HARQ feedback time gap (Z) should be referred to TS 38.321 to make the definition more accuracy - No |
| Lenovo, Motorola Mobility | Support 1, not support 2. |
| Ericsson | For the first point: We agree that the bullets a and b should be reused for partial sensing and random resource selection. For the second point, we propose to further study the possibility of adding a restriction/rule for the minimum distance between consecutive blind (re-)transmission for random resource UEs. Therefore, to be added into the proposal: FFS: restriction/rule for the minimum distance between consecutive blind (re-)transmission for UEs performing random resource selection. The motivation for this point is: - Since a random resource selection UE may co-exist with other UEs which can perform sensing, i.e., partial sensing and full sensing UEs, it will be beneficial for the sensing UEs to have enough time to detect any potential collision created by the reservations from the random selection UE. Therefore, adding a minimum distance between reserved resources for a random resource UE, the sensing UEs have enough time to re-select their resources and avoid a collision. |
| Huawei, HiSilicon | We do not need to re-confirm aspects of Rel-16, since they are by default the case by virtue of being currently specified. Only if something has to be changed should discussion and agreement be needed. - Proposal 6 is therefore generally OK, but we do not suggest wording an agreement on them. We note that companies maybe not fully aligned on the understanding the following part: - “Maximum distance separation in logical slots is 31 for any two resources indicated by a prior SCI” We assume this parts means the “Time resource assignment” field in 1st SCI can indicate resources that is up to 31 logical slots away. If so, there is no exceptional cases, and “, including all exceptional cases agreed in R16” needs to be removed. We feel some companies may misunderstand this part as the following agreement in RAN1#101-e. Anyway, these are two totally different things. Agreements: - In Step 2, a UE shall select resources so that HARQ retransmission resources can be reserved by a prior SCI, except that - In case no resource can be found for reservation (e.g., based on the identified candidate set after Step 1) for a retransmission of a TB, the re-transmission can be transmitted on a resource that is not reserved - After the resource selection is performed, HARQ retransmission on a resource not reserved by a prior SCI is allowed due to transmission dropping caused by prioritization, pre-emption and congestion control |
| CATT, GOHIGH | From our understanding, this timing restrictions in R16 should be reused in R17. If there has different views, we think we need a conclusion on that. As comment by other companies in GTW session, in case of blind retransmission, FFS on the minimum slot distance due to re-evaluation or pre-emption. |
| Fraunhofer | We are fine with the FL’s proposal to retain the timing restrictions as per Rel-16. |
| ETRI | We support both timing restrictions, and no further enhancements is necessary. |
| Apple | We are fine that R16 timing restrictions apply to UEs with partial sensing and random resource selection. |
| InterDigital | - Yes We also propose to support mixture of blind and feedback-based retransmissions. |
| Qualcomm | In addition to (a) and (b), as mentioned earlier, we think re-evaluation should also be performed by the UEs that can receive PSCCH. If RAN1 is listing the procedures that should be followed by a UE, the list should be reasonably comprehensive; otherwise, including a subset of procedures can be interpreted as that following other procedures is not required. |
| Futurewei | - Yes - No |
| Convida Wireless | - Yes - Open for discussion |
| Nokia, NSB | Agree to the 1st question. For the 2nd question: no. |
| Bosch | We support the 1st main bullet only. |
| ZTE | We support b in bullet 1, but for a, we don’t think the limitation is necessary, we wonder whether we could remove a given it can be left to resource selection in MAC layer. 5 bits are available in SCI, thus at maximum 31 distance is a common understanding. |

### Proposals before 2nd check point (April 19th)

FL observations and comments based on inputs received in Sec. 3.6.1:

- The two R16 timing restrictions for resource (re)selection should be reused

  - Maximum distance separation in logical slots is 31: DCM, OPPO, Intel, Fujitsu, Pana, SS, NEC, Lenovo, MM, Ericsson, Fraunhofer, ETRI, Apple, IDC, CATT, GOHIGH, QC, Futurewei, Convida, Nokia, NSB, Bosch

  - HARQ feedback time gap (Z) between PSSCH-to-PSFCH-to-PSSCH: OPPO, Intel, Fujitsu, SS, NEC, Lenovo, MM, Ericsson, Fraunhofer, ETRI, Apple, IDC, CATT, GOHIGH, QC, Futurewei, Convida, Nokia, NSB, Bosch

- Further enhancement / modification

  - No: OPPO, vivo, Fujitsu, Pana, NEC, Lenovo, MM, ETRI, Futurewei, Nokia, NSB, Bosch

  - Support or further study mixed blind and HARQ-feedback based retransmissions

    - DCM, Intel, LGE, IDC

  - FFS: restriction/rule for the minimum distance between consecutive blind (re-)transmission for UEs performing random resource selection

    - Ericsson, CATT, GOHIGH

  - Re-evaluation should also be performed by the UEs that can receive PSCCH

    - Qualcomm

- Postponed this discussion until basic partial sensing procedures are clear

  - Xiaomi

- No explicit agreement is needed since R16 principle will be reused by default

  - vivo, HW, HiSi,

FL: It is evident that the two existing R16 timing restrictions should be kept (or at least no deletion or modification) while some companies would like to study further on some potential enhancements. At the same time, it is true that unless we agree to remove or modify the existing resource selection procedure, the two timing restrictions would be applied by default in the specification. Therefore, the following is proposed.

Proposal 6 (round II):

- Further study the followings:

  - Support or further study mixed blind and HARQ-feedback based retransmissions

  - Restriction/rule for the minimum distance between consecutive blind (re-)transmission for UEs performing random resource selection

| Company | Comments |
| --- | --- |
| NTT DOCOMO | We are supportive of this proposal. To clarify these are studied for power saving purpose, which is different from Rel-16 discussions, small update can be added as ‘Further study the followings for power saving’ |
| Apple | We prefer to deprioritize the study of these two topics. |
| Futurewei | We are ok with the proposal. |
| LGE | - First sub-bullet Support FL proposal. - Add new sub-bullet As we commented at the GTW session, we want to add the restriction/rule for the maximum distance between resources for power saving. We suggest to add the following sub-bullet. - Restriction/rule for the maximum distance between any two resources indicated by a single SCI for UE performing partial sensing |
| NEC | Support |
| CMCC | OK to further study the potential enhancement, and if no concensus is reached, the R16 rules apply anyways. |
| Fujitsu | We agree Apple’s view that the study of these two topic should be deprioritized. |
| OPPO | Not support. 1st bullet: it has been discussed in R16. There is no necessary to reopen the discussion unless evaluation results justify the issue. 2nd bullet: there is no critical issue identified and no necessary to optimize it. |
| Panasonic | We share similar with docomo to specify the purpose is for power saving. |
| Huawei, HiSilicon | We disagree this proposal. Based on the agreement in RAN2 #110e, the mixed blind and HARQ feedback based retransmissions is not supported in Rel-16. The agreement is copied as below Agreements on sidelink HARQ operation: 1: RAN2 does not support the mixing of blind and feedback-based HARQ retransmissions of a TB in Rel-16. On the other hand, the benefits on saving power to support mixed retransmission have not justified yet. HARQ feedback serves to save power in the Tx UE, and adding blind retransmission would consume more power, especially when the receiver already receive the packet successfully. So in Rel-17, we have not seen the necessity to reopen this discussion. For the second sub-bullet, enlarge the gaps between adjacent transmissions does not solve collisions, but increase the latency of transmission, which was one of the key benefits for using blind (re-)transmissions. With or without the min gap, there is no power saving gain for blind retransmissions. |
| Samsung | Not support. 1st sub-bullet was already discussed in Rel-16 without justified gain, and in our understanding it’s out of Rel-17 scope. The motivation of 2nd sub-bullet is unclear to us, since for blind retx, no PSFCH related gaps are needed. Unless any critical issue is observed, otherwise we think 2nd sub-bullet is unnecessary. |
| Intel | We agree to study further these aspects. We also think it is important to study further at least the following aspects: - Dependency of minimum number of candidate slots Y for partial sensing on priority level - Techniques to reduce TB transmission time/UE partial sensing time - Communication b/w UEs in partial sensing mode - Adaptation b/w partial sensing/ random resource selection and full sensing |
| Xiaomi | We do not think the proposal is a critical issue, but we are open. |
| vivo | We prefer to focus on fundamental design of partial sensing and continuous sensing, study of these two topics can be deprioritized. |
| Ericsson | We are supportive of the second bullet of this proposal. We would like to comment on the motivation for this bullet and why it is important to add a mechanism for the consecutive blind (re-)transmissions for random resource selection UEs: - Random resource selection UEs may co-exist with other UEs which can perform sensing, i.e., partial sensing and full sensing UEs, as already agreed, and therefore, it will be beneficial for the sensing UEs to have enough time to detect any potential collision created by the reservations from the random selection UE. - Therefore, by adding a minimum distance between consecutive reserved resources for a random resource UE, the sensing UEs have enough time to re-select their resources and avoid a collision. |
| ZTE | Similar view as OPPO, Not support. |
| Nokia, NSB | This proposal (Proposal 6 round II) is more like the enhancement of HARQ with mixed blind retransmission. At this stage this study should be deprioritized. |
| CATT,GOHIGH | We are open to further study these. |
| Qualcomm | This proposal can be revisited at a later stage when the design of the baseline schemes is progressed reasonably. |
| InterDigital | Support |

### Proposals before Monday GTW session (April 19th)

FL observations and comments based on inputs received in Sec. 3.6.2:

- On Proposed conclusion

  - Support or open to further study: DCM, Futurewei, LGE, NEC, CMCC, Pana, Intel, Xiaomi, Ericsson, IDC (10)

  - Not support proposed conclusion: OPPO, HW, HiSi, Samsung, ZTE (5)

  - Deprioritize proposed topics: Apple, Fujitsu, vivo, Nokia, NSB, Qualcomm (6)

  - @LGE: for the proposed new topic (3rd sub-bullet below), in R16 this max distance is 32 logical slots. Just to clarify, are you proposing to change this for partial sensing UEs to a number smaller than 32?

  - @Intel: for items “techniques to reduce TB transmission time / UE partial sensing time” and “communication b/w UEs in partial sensing mode”, I think these are the goals that we always aim towards in this agenda item. I think it is not necessary to list it here again.

  - @All: It is almost 50:50 between support for further study (10) and not support + deprioritize (11). FL proposal is to treat these areas after we have achieved reasonable progress with the baseline schemes that we have already agreed to support (e.g. partial sensing, re-evaluation/pre-emption checking, congestion control, random resource selection, etc).

Proposed conclusion (round II):

- Further study the followings for power saving [with low priority]:

  - Support or further study mixed blind and HARQ-feedback based retransmissions

  - Restriction/rule for the minimum distance between consecutive blind (re-)transmission for UEs performing random resource selection

  - [Restriction/rule for the maximum distance between any two resources indicated by a single SCI for UE performing partial sensing]

  - [Dependency of minimum number of candidate slots Y for partial sensing on priority level]

  - [Adaptation b/w partial sensing/ random resource selection and full sensing]

| Company | Comments |
| --- | --- |
| Qualcomm | As mentioned before, we are open to discuss other schemes; however, we prefer to focus first on the design of the baseline schemes. Therefore, we do not think an agreement or a conclusion at this point is needed. If time permits, and if there is any need, other enhancements can be discussed in RAN1. FL: I full understand each company’s preference. This is meant to be a compromised way forward. |
| Sharp | We share similar view as Qualcomm. |
| NTT DOCOMO | We are supportive of this proposal, with removing the text of [with low priority]. This conclusion is necessary since it is unclear whether these are on the table or not. FL: I full understand each company’s preference. This is meant to be a compromised way forward. |
| Samsung | We share similar view as QC. Since the the main bullet is for for further study and is with low priority, we prefer not spend online time to discuss it and leave it in FL summary as a guidance for following meetings. |
| Huawei, HiSilicon | Disagree. We start to worry about the scope and progress of power saving RA agenda. If we (RAN1) deems lots of item can be further studied, we are not sure whether RAN1 can finalize Rel-17 design on this part on time. In our understanding, the issues highly related to the power saving RA and critical to RA design should be prioritized. After dealing with those, futher judgement can be triggered to study whether other enhancements are needed. So we think the items proposed in this conclusion neither have a strong connection with power saving RA schmes, nor have an essential impact on RA designs, and they can be treated same as the items in topic #9, coming back after basic power saving RA deisgn is more stable (if needed). |
| Spreadtrum | We should be open for other enhancements. And we don’t need any conclusion and agreement in this meeting. |
| LGE | - Main bullet We don’t see the rationale why some or all of the FFS points should be marked as [low priority]. At this moment, there are no sufficient analysis or justification on whether any of the FFS points are beneficial in terms of any criteria, such as power saving or performance. We remove the bracket, [with low priority]. - Third sub-bullet @FL: you’re understanding is correct. We think that limiting the max distance smaller than 32 is beneficial for power saving. For example, it enables UE to save power for additional sensing required for resource re-evaluation and pre-emption checking. - As a result, we suggest the following modifications. Proposed conclusion (round II): - Further study the followings for power saving [with low priority]: - Support or further study mixed blind and HARQ-feedback based retransmissions - Restriction/rule for the minimum distance between consecutive blind (re-)transmission for UEs performing random resource selection - [Restriction/rule for the maximum distance between any two resources indicated by a single SCI for UE performing partial sensing] - [Dependency of minimum number of candidate slots Y for partial sensing on priority level] - [Adaptation b/w partial sensing/ random resource selection and full sensing] |
| Ericsson | We are supportive of this conclusion |
| Convida Wireless | We are open for discussions of any further enhancements but would prefer to focus more on baseline enhancement at this stage. |
| ZTE | This proposal seems to be the next step optimization after the functional part is done. |

### Proposals before Tuesday GTW session (April 20th)

FL observations and comments based on inputs received in Sec. 3.6.3:

- I full understand each company’s preference. This is meant to be a compromised way forward.

Proposed conclusion (round III):

- Further study the followings for power saving [with low priority]:

  - Support or further study mixed blind and HARQ-feedback based retransmissions

  - Restriction/rule for the minimum distance between consecutive blind (re-)transmission for UEs performing random resource selection

  - [Restriction/rule for the maximum distance between any two resources indicated by a single SCI for UE performing partial sensing]

  - [Dependency of minimum number of candidate slots Y for partial sensing on priority level]

  - [Adaptation b/w partial sensing/ random resource selection and full sensing]

| Company | Comments |
| --- | --- |
| Apple | Fine with the proposal. |
| Nokia, NSB | There is no need to have this agreement at this stage. We are open for other power saving schemes to support random resource selection. But this list of schemes won’t help much. FL: Let’s hear more views on this. |
| NTT DOCOMO | We still think ‘with low priority’ should be removed. This proposal is for ‘further study’, so the labelling is unnecessary. But if majority supports this proposal, we are OK with current version as compromise. |
| InterDigital | Support |
| FUTUREWEI | We do not see this as a compromise as it has greatly increased the number of items listed from earlier in the meeting. Low priority will not decrease the contributions and analysis on the items. For the 3r and 4th bullet, they are not related to the random resource selection, particularly, for 4th point, Ymin will be discussed under periodic partial sensing. It is not appropriate to include these two items here. FL: The discussion has evolved a bit. It is no longer just about random reosurce selection as it was originally intended for this topic at the beginning. Let’s hear more more on this proposed conclusion. |
| Qualcomm | We are not fine with this proposed conclusion for the same reasons mentioned before. |
| NEC | We are open on it. |
| Fujits | We do not prefer to deprioritize the study of the 4th sub-bullet. Since in NR V2X, some new features were introduced compared to LTE-V. For example, HARQ-ACK feedback can be enabled, and the maximum numer of transmission for a TB can be extended to 32. So reusing single minimum number of candidate slots Y configuration in LTE-V may not fullfill the QoS requirement in Rel-17. Then the configuration with multiple values can be considered based on different priority, CBR or PDB values, etc. This is mentioned/ proposed in many companies’ contributions and we propose to discusse this sub-bullet with a higher priority. |
| Huawei, HiSilicon | Disagree. As we have explained in our previous reply, the proposed items for further studied have not been justified beneficial on power saving, neither related to core design about the RA schemes. Companies can propose these optimizations rely on the submitted contribution and decide whether to have further study. There is no need to have such a agreement in this meeting. FL: Like before, I will leave it to the Chairman to decide. |
| Ericsson | We would like to get some clarification in this proposal: - Only the blue points are considered as low priority or all the bullets of this conclusion? One possible way-forward if only the blues ones are low priority, it is to focus only on the first two sub-bullets and once these have been agreed/studied continue with the rest on later meetings FL: No, the use of different text colors is just for the purpose of what has been changed in each round. There is no particular relation for texts with same color. Low priority applies to all sub-bullets. If we go with the suggested approach, I think it will be even harder to agree, unfortunately. |
| Panasonic | We are generally ok with the proposal and prefer to remove the wording of “with low priority”. FL: I will leave it to the Chairman to decide, but not likely deleting “with low priority”. |
| Samsung | We are not fine with this proposal. It seems not like compromise in our view. As commented in last rounds, we feel negative to study the issues. According to discussion result in last round, almost a half of companies don’t see the need of the proposal and it’s not a good way by making conclusion to request companies further studying it. FL: One half said yes, the other half said no. I will leave it to the Chairman to decide. |
| LGE | Support FL proposal except that we share the view with NTT DOCOMO that “with low priority” should be removed. - Further study the followings for power saving [with low priority]: FL: I will leave it to the Chairman to decide, but not likely deleting “with low priority”. |
| ZTE | This seems to be next step optimization, we could discuss them at due time. |
| Convida Wireless | We are open for further discussions. |

### Final proposals for Tuesday GTW session (April 20th)

Proposed conclusion’ (Topic #6):

- Further study the followings for power saving [with low priority]:

  - Support or further study mixed blind and HARQ-feedback based retransmissions

  - Restriction/rule for the minimum distance between consecutive blind (re-)transmission for UEs performing random resource selection

  - [Restriction/rule for the maximum distance between any two resources indicated by a single SCI for UE performing partial sensing]

  - [Dependency of minimum number of candidate slots Y for partial sensing on priority level]

  - [Adaptation of sensing parameters including adaption b/w partial sensing/ random resource selection and full sensing]

## Topic #7: Impact of sidelink DRX on sensing for resource (re)selection

Background: According to the latest version of the WID objective for power saving RA, the work on power saving RA should consider any impact from sidelink DRX operation. So far, identified impacts are mostly related to the sensing operation. That is, whether SL DRX should take into account of PSCCH monitoring for sensing based on a working assumption made in RAN2.

Based on submitted contributions in this meeting, the following two points in Proposal 7 (Section 3.7.1 below) are the most discussed points and proposals.

Tuesday GTW session:

For the first main bullet and related sub-bullet, there were many comments related to how would the SL DRX configuration of the target destination/UE(s) is known to the TX UE, how would a TX UE able to ensure the transmitted data is “delivered” or successfully received, it should be the ON-period of the target destination and not the active-timer. In FL’s understanding of RAN2’s agreements so far on sidelink DRX, the SL DRX configuration is delivered via SIB or pre-configuration for broadcast and groupcast and exchanged between two UEs via PC5-RRC in unicast. Based on this, the TX and RX UEs are always aligned / has knowledge about each other’s SL DRX configuration. Having said this, it is also now FL’s understanding that at the time of resource (re)selection, the TX UE may not know who is the target destination / RX UE or multiple RX UEs and hence the related SL DRX configuration may not be available during the resource (re)selection procedure step 1 and step 2. Or at least, this part is still unclear in RAN2. As such, RAN1 should not make any assumptions and should wait for further progress is made in RAN2 before we can assess the impact and make any conclusions or agreements on the first main bullet and the related sub-bullet.

For the second main bullet, the main comment during the GTW session was to make this a UE implementation based.

Original Proposal 7 discussed during Tuesday GTW session:

- When SL DRX configuration of target destination (i.e., broadcast, groupcast, unicast UE) is known, the TX UE should ensure that the transmitted data is delivered during SL active time of the target destination.

  - At least resource for the initial transmission of a TB is selected within the SL-DRX active time of the target destination and within its own PDB.

- A UE can perform SL reception of PSCCH and RSRP measurement for sensing during its SL DRX inactive time.

### Proposals before 1st check point (April 15th)

Proposal 7:

- It is up to UE implementation to perform SL reception of PSCCH and RSRP measurement for sensing during its SL DRX inactive time.

| Company | Comments |
| --- | --- |
| NTT DOCOMO | Not support. In shared resource pool with full sensing UE, power saving UE shall receive all PSCCHs corresponding to P_reserve (details are decided in proposal 1). Otherwise, many collisions could happen. |
| Xiaomi | We are fine with FL proposal. |
| OPPO | Support the proposal. The DRX pattern is determined by the traffic pattern of peer UE, while the resource selection window of the sensing UE is determined by the PDB of its own traffic pattern. These two traffic patterns are independent each other. If sensing is limit to active time, it is possible that the UE cannot do re-evaluation/pre-emption for the selected resources if these resources are in inactive time. That will affect PRR performance. |
| vivo | Not support we have performed simulations to evaluate the PRR of performing sensing in active duration only and the PRR of allowing sensing in both active and inactive duration, and results showed that the PRR performance of the two methods are almost the same in most cases, while the former method provides 7-11% power saving gain[R1-2102539]. Give that sensing in active time only can achieve much lower power consumption and avoids PRR degradation, we think it should be specified as follows: It is up to UE implementation to perform SL reception of PSCCH and RSRP measurement for sensing during its SL DRX inactive time.UE performs SL reception of PSCCH and RSRP measurement for sensing during its SL DRX active time. |
| Intel | We would like to make sure that above proposal respects T2min for transmission(s) in active time (i.e. T2min is within active time). Otherwise it seems channel access may be broken. |
| CMCC | We basically think that SL reception for sensing should not be limited by SL DRX active time, but have no strong objection to this Proposal. To our understanding, partial sensing and random selection are power saving RA mechanisms for resource selection based on full sensing; meanwhile, SL DRX is another power saving mechanism. For the Tx UE, it should be enough to adopt one power saving mechanism for resource selection, and SL DRX is more like a power saving solution for data reception. On the other hand, SL DRX is semi-static configured while the sensing pattern is highly related to the arriving of the data packet, which is quite dynamic, it could be difficult to match these two patterns. Hence, to ensure enough sensing samples to obtain reliable sensing results, the SL reception for sensing should be independent of the SL DRX active time. |
| Fujitsu | Not support. We prefer the original proposal and share the same view as DOCOMO that the current proposal may bring more collisions. Regarding to the original 1st bullet, we think after RAN2 has confirmed that the SL DRX configuration of target destination (i.e., broadcast, groupcast, unicast UE) can be noticed to physical layer, it can be agreed to make sure the target destination / RX UE or multiple RX UEs can receive the transmitted packet correctly. |
| Panasonic | Our understanding of the definition is DRX active time is UE shall receive certain channels/signals and DRX inactive time is UE is not required to receive certain channels/signals. The specification does not prevent any reception in DRX inactive time. Based on certain activity (for example, after the reception of initial transmission to receive retransmission), if UE needs to receive PSCCH, such period should be defined as DRX active time, which is determined dynamically. So DRX active time is the sum of (semi-static configuration part) and (dynamically extended by the activity). Therefore, proposal 7 is just equal to the definition of inactive time in our view. More aligned wording with the definition of inactive would be "UE is not required to perform SL reception of PSCCH and RSRP measurement for sensing during its SL DRX inactive time". |
| LGE | In our understanding, the following agreements were made in RAN2 this meeting. Proposal 5 – [18/21] For unicast, the TX UE behaviors should be specified to keep aligned with the RX UE regarding the DRX Active time. FFS the specific Spec impacts needed at the TX side. Proposal 12 – [19/21] For unicast, the TX UE maintains a timer corresponding to the SL Inactivity timer in the RX UE for each pair of src/dest L2 ID, and uses the timer as part of criterion for determining the allowable transmission time for the RX UE According to the above agreements, TX-UE is supposed to know RX-UE’s active time. Therefore we suggest to recover the sub-bullet of the original 1st bullet. We think partial sensing in SL-DRX inactive time is needed depending on the conditions. Proposal 7: - At least resource for the initial transmission of a TB is selected within the SL-DRX active time of the target destination and within its own PDB. - SL reception of PSCCH and RSRP measurement for sensing during its SL DRX inactive time is supported. - FFS detailed conditions on sensing in SL-DRX inactive time. |
| Samsung | Not support. We prefer to allow sensing only in active time of SL DRX to better control power consumption during sensing procedure. |
| NEC | More evaluation/discussion is needed on whether to specify this issue. |
| Lenovo, Motorola Mobility | Not support. We are OK if ‘It is up to UE implementation to’ is deleted. Sensing in inactive time can be performed by UE so as to avoid potential resource collision. |
| Spreadtrum | Not support. We support UE perform sensing only in active time of SL DRX. |
| Ericsson | Agree with the proposal. |
| Huawei, HiSilicon | As written this allows the UE to abandon any resource allocation configurations that may exist (e.g. partial, full, random, etc.), and do whatever it likes during SL DRX inactive time. The proposal needs qualifying e.g. “… according to the configurations of the resource pool and its resource allocation method(s)”. We think a UE can perform SL reception of PSCCH and RSRP measurement for sensing during its SL DRX inactive time, particularly if the monitored slots within DRX active time are insufficient to select reliable resources for transmissions. This should not be left fully up to UE implementation, to ensure that the baseline system-level performance can be ensured and predicted. Thus, we suggest writing: “A UE can perform SL reception of PSCCH and RSRP measurement for sensing during its SL DRX inactive time according to the configurations of the resource pool and its resource allocation method(s). FFS details.” |
| CATT, GOHIGH | Not support, we think the sensing performance should be ensured. |
| Fraunhofer | We do not support this proposal. Sensing is one of the UE’s most power hungry process, and it makes sense to restrict this to only the active time. |
| Apple | Not support, the power saving gain from SL DRX will be largely sacrificed if a UE performs sensing during SL RDX inactive time. Hence, we prefer UE does not perform sensing during its SL DRX inactive time, just like in the Uu interface that UE does not monitor PDCCH in its Uu DRX inactive time. Also, based on the RAN2 agreements cited by LG, we think the original 1st bullet could be discussed as well. |
| InterDigital | Not support We should not leave for UE implementation to determine whether to perform sensing in inactive time since if UE determines not to perform sensing in inactive time, it will severely affect the performance of the system and collision may happen frequently. Instead, the UE should follow the partial sensing configuration regardless of DRX status. |
| Qualcomm | Agree. |
| Futurewei | We do not agree with the revised proposal 7. Since the sensing during DRX inactive time is conflict with DRX definition, the standard shall first explicitly specify whether a UE can or cannot preforms sensing during its SL DRX inactive time. Our view is that it should be up to the configuration that UE performs or not the sensing during its SL DRX inactive time. We suggest modify the 2nd bullet in the original proposal as the proposal 7 - It is up to a configuration that UE perform SL reception of PSCCH and RSRP measurement for sensing during its SL DRX inactive time. - FFS the configuration for UE’s sensing behaviour during its SL DRX inactive time. |
| Convida Wireless | We are open for the proposal. More discussions are needed. |
| MediaTek | We agree with the direction of this proposal. However, we should also consider that in some cases, Tx-UE may not be able to take any sensing measurements, e.g., in case of long SL-DRX inactive time. Such cases could cause high collision risks. We suggest the following alternative: - It is up to UE implementation is not expected to perform SL reception of PSCCH and RSRP measurement for sensing during its SL DRX inactive time. FFS exceptional cases where this rule does not apply. |
| Nokia, NSB | We agree with the revised Proposal 7. UE shall have the freedom for sensing during its RX inactive time. |
| Bosch | In our contribution in RAN#91, we proposed two important features that needs to be considered for Rel-17 power saving for sidelink: wake-up signal structure (for important automotive use cases, e.g.. automated valet parking) and coexistence between Rel-16 and 17 UEs in the same resource pool. If DRX is enabled and the UE needs to monitor PSCCH/RSRP outside the active time, this needs to be designed for monitoring WUS PSCCH/ RSRP and/or for coexistence between Rel-16/17. Therefore, we support configuration for UEs to perform SL reception of PSCCH and RSRP measurement for sensing during its SL DRX inactive time. |
| ZTE | Agree. |

### Proposals before 2nd check point (April 19th)

FL observations and comments based on inputs received in Sec. 3.7.1:

- Support:

  - Xiaomi, OPPO, LGE, Ericsson, QC, Nokia, NSB

  - “Shall” monitor in DRX inactive time: DCM, CMCC, Fujitsu, Lenovo, MM, HW, HiSi, CATT, GOHIGH, IDC

- No support: vivo, SS, Spreadtrum, Fraunhofer, Apple

- Others

  - T2min for transmission(s) in active time (i.e. T2min is within active time) should be respected

  - UE sensing during inactive time based on configuration

FL: There are 3 camps of thoughts. On one extreme, quite significant number of companies want to mandate that a UE should always perform sensing/partial sensing even in inactive time. On the other extreme, some companies think partial sensing is not needed during the inactive time. Then there are some companies are OK with up to UE implementation to perform sensing or not during the inactive time (like a compromise).

It was also brought up by some companies that SL DRX configuration applies to UE that performs full sensing as well. As such, there would not be a concern on performing sensing during its SL DRX inactive time. For power saving UEs, we can further study any conditions on sensing during the SL DRX inactive time as suggested by LGE. Therefore, FL suggest to still consider a compromise way forward as followed.

Proposal 7 (round II) for Thursday GTW session:

- It is up to UE implementation to perform SL reception of PSCCH and RSRP measurement for sensing during its SL DRX inactive time.

  - FFS any conditions on sensing in SL-DRX inactive time.

Thursday GTW session:

FL: It is very clear from online comments (during GTW today) that there are still many not willing to compromise and wanted to go with either sensing or no sensing at all during the inactive time. I think this is also due to SL DRX operation and design are still not clear enough in RAN2 for RAN1 to make a decision on this. Therefore, this topic will not pursuit any further during this meeting.

## Topic #8: SL-HARQ feedback enabling and PSFCH monitoring

Background: In R16, it is already required for a UE to monitor PSFCH resources when SL HARQ feedback is enabled in its transmission (i.e., in SCI format 2A and 2B). Although it is straight forward, but it is worthwhile to confirm this principle also for UEs operating power saving RA.

Tuesday GTW session:

During the GTW session, main comments raised were related to whether the PSFCH reception is intended only during the SL DRX active time, whether this PSFCH reception is also for inter-UE coordination, the main bullet is only applicable for Type B and D UEs, and there is no need to mention the UE type in the proposal as this may have impact on the UE capability discussion later on. More importantly, there was a comment that monitoring of associated PSFCH resources for transmissions with enabled SL HARQ feedback is already an existing behaviour from R16, and therefore, this main bullet is not needed. For this, the FL tend to agree as well. Furthermore, it is expected that only UE that is able to receive PSFCH will be assigned / given a TB in higher layer that requires SL HARQ feedback to be enabled in SCI. As such, we should not remake the same agreement / existing R16 behaviour again for R17.

On the contrary, there may be some value in making the sub-bullet an agreement in RAN1 and inform RAN2 about it. Therefore, a question is formulated instead in Section 3.8.1.

Original Proposal 8 discussed during Tuesday GTW session:

- UEs using partial sensing or random resource selection for transmissions with enabled SL HARQ feedback are required to monitor the associated PSFCH resources.

  - For Type A UE, it is not expected to receive higher layer indication to enable HARQ feedback for SCI format 2A and 2B.

### Proposals before 1st check point (April 15th)

Questions for Proposal 8:

- Is it necessary / worthwhile making the following sub-bullet an agreement in RAN1 and subsequently informing RAN2 about it?

  - For a UE that does not receive PSFCH, it is not expected to receive higher layer indication to enable HARQ feedback for SCI format 2A and 2B.

| Company | Yes/No | Comments |
| --- | --- | --- |
| NTT DOCOMO | No | UE that does not receive PSFCH does not request HARQ feedback. This is straightforward and there will be no possibility of different behaviour, in our understanding. |
| xiaomi |  | We understand FL intention but think the wording is confusing. We are not sure what is the exact meaning of “a UE does not receive PSFCH”. Does it means that a UE does not have PSFCH reception capability, or a UE can freely decide whether to receiver PSFCH, or there are some configurations on whether a UE receives PSFCH? |
| OPPO | No | Type-A UE was agreed and RAN2 can consider this kind of UE (w/o PSFCH reception capability) during their design. There is no necessary to inform them again. |
| vivo | NO | First of all, this seems to be a RAN2 issue. Moreover, it can be achieved by different (RAN2) solutions, e.g., introducing LCP restriction for type A UE, while the cons and pros should be analyzed in RAN2. We suggest asking RAN2 to handle this. |
| Fujitsu | No | We think this proposal is not necessary. |
| Panasonic | See our comments | In our interpretation of FL’s proposal, it seems a UE in the state of “not receive PSFCH” cannot be enabled for HARQ feedback by higher layer signalling. We think for a UE capable of receiving PSFCH but configured not to receive, higher layer signalling could enable it. |
| LGE |  | FL proposal is supported, but no need to send LS to RAN2. |
| Samsung | No | We understand the intention of “a UE that does not receive PSFCH” as UE with no capability on PSFCH reception. In our understanding this issue will be handled by RAN2 and there is no need to informing it. |
| NEC | No |  |
| Lenovo, Motorola Mobility | No | Agree with NTT DoCoMo, and no need to send LS to RAN2. |
| Spreadtrum | No | It is not necessary to send LS to RAN2. |
| Ericsson | No | There is no need to create an agreement for this behaviour. |
| Huawei | No | This is not necessary. The HARQ feedback enable/disable indication as indicated in SCI is instructed by higher layers, hence the higher layer of a UE which does not support PSFCH reception will not indicate PHY to enable HARQ-based (re-)transmissions. It is redundant to make new RAN1 agreement in rel-17 as well as for generating LS. |
| CATT, GOHIGH | No | We don’t have strong view on whether an agreement is necessary or not. But from our understanding, it a UE does not intent to receive PSFCH, the HARQ feedback shall not enable in SCI. |
| Fraunhofer | No |  |
| ETRI | No |  |
| Apple |  | We are fine with the proposal in principle. The wording may be modified, e.g., “For a UE not capable of performing PSFCH reception, it is not expected to receive higher layer indication to enable HARQ feedback for SCI format 2A and 2B.” |
| InterDigital | No | It is straightforward and no need to have an agreement. |
| Qualcomm | No | The consequence of not receiving PSFCH is clear. In our view, informing RAN2 is not necessary. |
| Futurewei | No | First, we have a comment on the statement 1. "does not receive" seems ambiguous as it could include a cast that a UE missed one of them. It is better rephrased as "For a UE that is not capable of receiving"... If the statement is as what we revised, it is not necessary whether ran2 needs to know this now. |
| Convida Wireless | No |  |
| MediaTek | No | Not necessary |
| Nokia, NSB | No | If the UE is a type-A UE, it won’t be able to have HARQ feedback. |
| Bosch | No | Does not need agreement |
| ZTE | No | The capability of PSFCH can be same as rel-16, based on that, UE should follow the instruction from high layer. |

### Proposals before 2nd check point (April 19th)

FL observations and comments based on inputs received in Sec. 3.8.1:

- FL: Based on the answers, it is overwhelmingly clear that this aspect does not need to pursuit any further. Any further decision or action can be taken directly by RAN2.

## Topic #9: Other potential power saving enhancements

Tuesday GTW session:

During the GTW session, it was raised that RAN1 could start look into some of other power saving enhancements listed in Section 4.10 of this document. And it is instructed by the Chairman to list some of potential ones for further discussion in this section. For this, power saving enhancements / topics proposed by more than one company are listed in the following section 3.9.1.

As a FL, it should be reminded the WID objective for this agenda (copied just below), as this should be taken into consideration when selecting a potential power saving enhancement / technique.

2. Resource allocation enhancement: - Specify resource allocation to reduce power consumption of the UEs [RAN1, RAN2] - Baseline is to introduce the principle of Rel-14 LTE sidelink random resource selection and partial sensing to Rel-16 NR sidelink resource allocation mode 2. - Note: Taking Rel-14 as the baseline does not preclude introducing a new solution to reduce power consumption for the cases where the baseline cannot work properly. - This work should consider the impact of sidelink DRX, if any.

### Proposals before 1st check point (April 15th)

Question for Proposal 9:

- Which one(s) of the following potential power saving enhancement topics / techniques should be discussed and further considered for this WI?

  - The mechanisms of power saving enhancement for mode-2 should focus on reducing power consumption of Sensing and PSFCH transmission.

    - Longer PSFCH period or enhancement of conducting resource selection should be studied.

  - In the case of inadequate sensing results and for power saving, UE uses assistance information messages in order to obtain the required sensing information for carrying out resource selection.

  - Allowing a UE can enter a non-sensing mode for power reduction whilst selecting resources based on indication from inter-UE coordination.

  - Sidelink bandwidth adaptation for transmission / reception is supported as a power saving feature.

| Company | 1, 2, 3, and/or 4 | Comments |
| --- | --- | --- |
| NTT DOCOMO | 1 | Inter-UE coordination is topic to improve reliability and latency. Not for power saving. Of course power saving UEs can use inter-UE coordination if want, but no optimization for these UEs is necessary. |
| Xiaomi |  | None of the above topics should be considered at the current stage |
| OPPO |  | Prefer to prioritize discussion of partial sensing mechanism. Other power saving mechanisms should be down-prioritized. |
| vivo | Option1 | According to our simulation results in R1-2102539, the PSFCH transmission also occupy significant parts of total power consumption in both groupcast option 1 and option 2 scenarios in addition to sensing. Therefore, mechanisms of power saving of PSFCH transmission should be studied. One way is to introduce a longer PSFCH period. In our simulation, PSFCH with a period of 8 slots brings 5%-20% power-saving gain while similar PRR performances compared with PSFCH with a period of 1/2/4 slots. Moreover, the Rel-16 PSFCH power control mechanism leads to very high but unnecessary TX power of PSFCH, due to the problem that SL pathloss between TX and RX UEs are not utilized. To handle this issue, SL pathloss based OLPC for PSFCH can be considered. Our simulation results show that SL pathloss based OLPC for PSFCH also brings around 10-30% power saving gain. Thus, we support to study the following aspects - The mechanisms of power saving enhancement for mode-2 should focus on reducing power consumption of Sensing and PSFCH transmission. - Longer PSFCH period or enhancement on PSFCH power control(e.g., SL pathloss based power control for PSFCH) or enhancement of conducting resource selection should be studied. |
| Intel |  | We propose to include into the discussion the following important topics: - Dependency of minimum number of candidate slots Y for partial sensing on priority level - Resource selection enhancements to reduce TB transmission time/UE partial sensing time - Communication b/w UEs in power saving modes - Communication with reduced sidelink bandwidth |
| Fujitsu | 2 | The “non-monitored” slots issue may cause the happening of excessive exclusion, this issue is also discussed in R16 NR V2X maintenance session. In our opinion, assistance information from a coordinating UE can be used to provide the sensing results in the “non-monitored” slots for the coordinated UE and this can make the resource selection more reliable. |
| Panasonic | 3 | With or without inter-UE coordination information, we think it would be more power saving that a UE can switch between different mode to have a better balance of performance and power saving. |
| LGE |  | It seems the above topics can be further discussed after the basic partial sensing mechanisms were agreed in RAN1. |
| Samsung | 2 and 3 | Although power saving is not the intention of inter-UE coordination, power saving will benefit from inter-UE coordination by reducing or skipping sensing slots. In addition, inter-UE coordination could also be used to improve SL reliability under power saving schemes. Therefore, the combination of these two features can be further studied. |
| NEC | 1 |  |
| Lenovo, Motorola Mobility |  | All of them are not needed |
| Spreadtrum | 3 | For UEs having sidelink Rx capability, using random selection may help to further reduce power reduction compared with partial sensing. In order to benefit from random selection and further reduce resource collision, additional information can be provided for UEs performing random selection. |
| Ericsson |  | In our view, we do not need to study any of these topics at this point in time. Topic 3 can be revisited once the inter-UE coordination scheme(s) are agreed in order to check whether this technique can be/needs to be implemented. |
| Huawei, HiSilicon | 3 | The inter-UE coordination can be reused for power saving. This is beneficial in terms of power reduction for sensing, due to the coordination information provided by a non-power-saving UE (e.g. a RSU) to power-saving UE to assist resource selection. It is also beneficial for PRR improvement, because this coordination can be the resources to be used, which are shown with gain in RAN1#104e conclusions. Most importantly, this cross-agenda combination design does not require physical layer structure changes, like alternatives 1 and 4, which may not coexist with Rel-16 UEs. |
| CATT,GOHIGH | 1 | Inter-UE coordination for power saving should be down prioritized. |
| Fraunhofer | 2, 3 and 4 | Inter-UE coordination can allow a UE to avoid sensing entirely, which results in power saving gains. Restrictions on the bandwidth used by power saving UEs can allow UEs to save power due to smaller frequency regions to carry out sense. |
| ETRI | 2, 3, 4 (as second priority) | If time is allowed, additional enhancements can be studied further. |
| Apple | 2 and 3 | We think UE does not perform full sensing could use assistant information to enhance the resource selection reliability. The assistant information could come from a helper UE or could come from network (in case of in-coverage). |
| InterDigital | Option 1 |  |
| Qualcomm |  | In our view, RAN1 should for now focus on the details of the baseline power saving schemes, i.e., random selection, partial sensing and DRX. Other topics can be considered later. |
| Futurewei | Modified 3 | We see no need to discuss further on 1, 2, 4. For 3, random resource selection is an important power saving scheme that is in discussions. A UE can enter a non-sensing mode with random resource selection. Therefore, we suggest a modified 3 as 3A. Allowing a UE can enter a non-sensing mode for power reduction and perform random resource selection on certain conditions. FFS the conditions that lead UE to a non-sensing mode with random selection. |
| Convida Wireless | 2, 3 |  |
| MediaTek | Option-2 and Option-3 | Also, Option-4 can also be considered later if time allows. |
| Nokia, NSB | 2nd bullet (low priority) | We’d like to prioritize the partial sensing mechanmism. Then we can discuss other schemes for power saving, for example, new power saving schemes for SL reception for public safety and commercial use cases, and further power saving schemes of resource allocation enhancements for type-A/B UEs. |
| Bosch | 4, 2 and/or 3 + 5/6 (other) | 4- We mainly support adaptive bandwidth for SL reception/transmission. 5- we support specifying SL WUS. 6- We support designing DRX allowing coexistence between R16/17 at least if sharing the same Resource pool. |

### Proposals before 2nd check point (April 19th)

FL observations and comments based on inputs received in Sec. 3.9.1:

- 1: DCM, vivo, NEC, CATT,GOHIGH, IDC

- 2: Fujitsu, Fraunhofer, ETRI, Convida

- 3: Panasonic, Spreadtrum, HW, HiSi, Fraunhofer, ETRI, Convida

  - Apple: Allowing a UE can enter a non-sensing mode for power reduction and perform random resource selection on certain conditions. FFS the conditions that lead UE to a non-sensing mode with random selection.

- 4: Intel, Fraunhofer, ETRI (2nd priority)

- None: Lenovo, MM

- Others

  - Dependency of minimum number of candidate slots Y for partial sensing on priority level

  - Resource selection enhancements to reduce TB transmission time/UE partial sensing time

  - Communication b/w UEs in power saving modes

- To focus on details of the baseline power saving schemes first, then other topics later: Xiaomi, OPPO, Ericsson, QC

FL: It is observed that there is no overwhelming support for one particular enhancement scheme to be considered right away. On the other hand, there are also views that none of them should be considered at all or other enhancements should be considered at a later stage. It is also observed by FL that none of these identified topics have direct impact to the design of partial sensing RA that we are currently discussing.

- It is encourage that companies continuing to study other enhancement areas and the FL suggests to start discussing some of them once the basic power saving RA deisgn is more stabled.

Contribution summary for power saving RA

## Periodic-based partial sensing

- Definition of resource selection window [n+T1, n+T2]

  - Keeping T1 and T2 as defined in R16 – baseline agreement in #104-e

    - No further restriction is needed for the selection window bound

    - Supported by: [2/HW, HiSi], [4/OPPO], [8/CATT, GH], [10/Fujitsu], [14/CMCC], [19/Samsung], [22/ETRI], [26/NEC], [28/ Lenovo, MM], [29/DCM], [33/E///]

  - The resource selection window is confined within a set of periodic set of resources

    - Yes: [18/QC]

    - No: [33/E///]

  - The determination of the Y slots can be from a fraction of the selection window, i.e., from [n+T1, n+j*T2] where 0<j≤1 [20/Pana]

- Identification of Y candidate slots (within resource selection window)

  - The minimum number of candidate slots Y for partial sensing is (pre)-configured per priority level [16/Intel], [27/IDC]

  - [18/QC]: For a UE performing partial sensing, if the resource selection procedure is triggered in slot n, the resource selection window consists of the slots associated with a given partial sensing window that lie within [n+T1, n+T2].

  - With the consideration of congestion control of UE with battery constraint, the minimum candidate slots for partial sensing could be configured based on CBR value. [8/CATT, GH]

  - Specify a new list of X% for partial sensing or set new rules for partial sensing on X% with the existing list sl-TxPercentageList. [11/Futurewei]

  - To reduce the amount of partial sensing (including re-evaluation/pre-emption), the length of Y should be limited as tyY-ty1≤Tthreshold, where Tthreshold is configured. [29/DCM]

  - The min. number of candidate slots Y is determined based on the transmission packet priority, QoS requirement, congestion/interference level, HARQ feedback enabled/disabled, remaining PDB, or resource re-evaluation or pre-emption checking enabled/disabled. [23/LGE]

  - UE should determine the minimum value of “Y” based on the priority and/or CBR value, from a range of values (pre-)configured in a resource pool. [10/Fujitsu]

  - Y shall be greater than or equal to a higher layer parameter minMumCandidateSF, where minMumCandidateSF is (pre-)configured for each priority value and per resource pool. [14/CMCC]

  - When PSFCH is configured, the impact of the HARQ RTT related timing restriction should also be considered when UE determines the “Y” candidate slots. [10/Fujitsu], [21/Sony]

    - RAN1 needs to discuss whether UE should decide partial sensing/candidate slots for all possible HARQ (re)transmissions of the same TB.

  - Minimum number of Y candidate slots is (pre)configured per the number of PSCCH/PSSCH resources to be selected. [17/Apple]

  - The determination of the Y slots should be pre-defined patterns instead of up to UE implementation.  The pattern timing could be with reference to t=n or absolute slot number. [20/Pana]

  - Support multiple range sets of Y values in high layer. E.g., each set per priority/SCS and a minimum value for Y is (pre-)configured from a proper set. [26/NEC]

  - Selected Y candidate slots are not overlapped with off-durations of the RX UE(s). [26/NEC]

  - Configure multiple minimum Y candidate slots corresponding to various periodic traffics respectively per resource pool. [28/ Lenovo, MM]

- Set of reservation periods, Preserve, to determine periodic sensing occasions

  - Option 1:  Preserve corresponds to all values from the configured set sl-ResourceReservePeriodList

    - [2/HW, HiSi], [3/Nokia, NSB], [4/OPPO], [11/Futurewei], [26/NEC], [28/ Lenovo, MM], [29/DCM], [31/ ASUSTeK], [34/ZTE, Sanechips]

  - Option 2:  Preserve Preserve corresponds to a subset of values from the configured set sl-ResourceReservePeriodList

    - By (pre-)configuration: [5/ Spreadtrum], [6/vivo], [9/MTK], [10/Fujitsu], [12/ Zhejiang Lab] – first N values, [13/ Fraunhofer], [14/CMCC], [15/Xiaomi], [16/Intel], [17/Apple], [19/Samsung], [22/ETRI] – larger than a threshold value e.g. 100ms, [23/LGE], [27/IDC], [30/ Hyundai], [31/ ASUSTeK], [33/E///]

    - By UE determination: [6/vivo] config a min number, [8/CATT, GH]

      - a subset from  Preserve ∈sl_ResourceReservePeriodList, if tySL-Preserve belongs to the same partial sensing set [18/QC]

  - Option 3: Preserve Preserve is a common divisor among values in the configured set sl-ResourceReservePeriodList

    - [32/ITL]

- Selection of k

  - Option 1: Only the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger or the set of Y candidate slots subject to processing time restriction

    - [4/OPPO], [8/CATT, GH], [10/Fujitsu], [12/ Zhejiang Lab], [16/Intel], [19/Samsung], [22/ETRI], [23/LGE], [26/NEC], [27/IDC], [28/ Lenovo, MM], [33/E///]

  - Option 3: All possible sensing occasions after n –T0

    - [5/ Spreadtrum]

  - Option 4: Only one periodic sensing occasion for one reservation period. The k value is up to UE implementation. Max value for k is (pre-)configured

    - [3/Nokia, NSB] with at least one PSO per reservation period, [9/MTK], [12/ Zhejiang Lab], [15/Xiaomi], [31/ ASUSTeK]

  - Option 5: k is (pre-)configured, including multiple values

    - [2/HW, HiSi], [13/ Fraunhofer] per priority, [14/CMCC], [17/Apple]

  - Option 6: (pre-)configuration of a bitmap, same as in LTE-V

    - [6/vivo] one bitmap per Preserve, [11/Futurewei] with kmax, [30/ Hyundai]

  - Option 7: Others

    - A (pre-)configured k most recent sensing occasions for a given reservation periodicity before the resource (re)selection trigger or the set of Y candidate slots subject to processing time restriction. [29/DCM]

- Conditions and timing to perform periodic-based partial sensing

  - If a resource pool enables periodic reservation, periodic-based partial sensing is performed regardless of Tx traffic type (both periodic and aperiodic transmissions). [2/HW, HiSi], [26/NEC], [27/IDC]

    - A ‘default’ periodic-based partial sensing, which is always performed over the limited periodic region regardless of the resource (re)selection triggering from the higher layer. [23/LGE]

  - Only when UE has periodic data to transmit and initiated by higher layer to perform periodic-based partial sensing in a resource pool that allows periodic reservation (i.e., sl-MultiReserveResource is enabled). [4/OPPO]

  - Regarding the issues for periodic-based partial sensing without sufficient sensing results, it can be left up to implementation for performing random resource selection or contiguous partial sensing-based resource selection. [8/CATT, GH]

  - Periodic-based partial sensing can be performed by TX UE if UE expects resource selection will be triggered in slot n. [19/Samsung]

  - UE does not perform periodic-based partial sensing when the reservation for another TB is disabled for the resource pool. [15/Xiaomi]

  - Conditions / cases in which a UE performs periodic-based partial sensing: [23/LGE]

    - When UE performs periodic transmission in a resource pool where periodic transmission and partial sensing are enabled

    - When the priority value of a packet is above a threshold (e.g. pre-emption priority value)

    - When the congestion/interference level in a resource pool is above a threshold

    - When the required reliability level of a packet transmission is above a threshold

    - When the number of received HARQ NACKs of a packet is above a threshold

    - When the number of retransmissions of a packet is below a threshold

  - Periodic-based partial sensing is applied for the following cases: [29/DCM]

    - A UE transmits a traffic with a resource reservation interval Prsvp_TX≠0, or

    - A UE transmits a traffic with a resource reservation interval Prsvp_TX=0 and all sensing results corresponding to a set of Y candidate slots are available

    - When a UE transmits a periodic traffic, the set of Y candidate slots are commonly applied to both periodic-based partial sensing and contiguous partial sensing

  - To align the sensing occasions of all reservation periods, and to meet the requirement of traffic PDB, the periodic partial sensing should be performed periodically in a short sensing duration (e.g., every 100ms). [34/ZTE, Sanechips]

- Others

  - Slots that are hypothetically reserved by non-monitored slots due to SL transmissions are excluded from Y candidate slots. [26/NEC]

## Contiguous partial sensing

- Conditions to perform contiguous partial sensing

  - All traffic types: periodic and aperiodic (without periodic reservation) transmissions

    - [2/HW, HiSi], [3/Nokia, NSB], [4/OPPO], [9/MTK], [16/Intel], [19/Samsung], [22/ETRI], [23/LGE], [27/IDC]

  - Conditions/cases in which a UE performs contiguous partial sensing: [32/LGE]

    - When partial sensing is enabled in a resource pool

    - When the priority value of a packet is above a threshold (e.g. pre-emption priority value)

    - When the congestion/interference level in a resource pool is above a threshold

    - When the required reliability level of a packet transmission is above a threshold

    - When the number of received HARQ NACKs of a packet is above a threshold

    - When the number of retransmissions of a packet is below a threshold

    - When aperiodic transmission is triggered

    - When periodic transmission is triggered

      - For sensing before the first candidate slot, combined with periodic-based partial sensing

      - When the time that can be used for periodic-based partial sensing is below a threshold

  - After receiving a NACK that the previous transmission was not successful. [33/E///]

- Sensing window [n+TA, n+TB] (values for TA and TB)

  - Replace n with index of the Y candidate slots: [25/Sharp], [26/NEC]

  - TA and TB values can be zero, positive and negative: [3/Nokia, NSB], [4/OPPO], [14/CMCC], [19 Samsung], [23/LGE], [29/DCM]

    - Depending on periodic or aperiodic traffic: [3/Nokia, NSB], [29/DCM]

    - Up to UE implementation: [4/OPPO]

    - Positive values if priority < threshold and PDB > threshold: [19/Samsung]

  - For periodic transmissions,

    - [2/HW, HiSi]: n+TA = ty0SL-31 and n+TB = ty0SL-Tproc,0SL-T1

    - [4/OPPO]: n+TA ≥ ty0SL-31 and n+TB ≤  ty0SL- Tproc,0SL-Tproc,1SL

    - [9/MTK]: n+TA ≥ tySL-32 and n+TB ≤ n+T2 -T3

    - [10/Fujitsu]: n+TA=ty0SL-31 and n+TB=n-Tproc,0SL

    - [11/Futurewei]: n+TA ≥ ty0SL-31 and n+TB = n+[TB,min, TB,max]

    - [15/Xiaomi]:  n+TA=m-31 and n+TB=n-Tproc,0SL

    - [16/Intel]:

      - TA within a range: -max(tn-32, resource selection window size) ≤ TA ≤ 1 slot, or

      - TA within a range: –max((∆A + tn-32), resource selection window size) ≤ TA ≤ 1 slot

    - [17/Apple]: TA=ty-31 and TB=max⁡{TA,ty-Tproc,0-Tproc,1}

    - [19/Samsung]: n+TA=ty0SL-31 and n+TB=n-Tproc,0SL

    - [23/LGE]: n+TA ≤ ty0-31 and n+TB ≤ ty0- Tproc

    - [25/Sharp]: TA=-Preserve (subject to processing time) and TB=-Preserve+31

    - [26/NEC]: [y_k -31, y_k – T_1 – T_proc,0]

    - [29/DCM]: n+TA=ty1-31 and n+TB=n+TC-Tproc,0SL+1, where n+TC≥ty1-Tproc,1SL is the resource selection timing

    - [34/ZTE, Sanechips]: [n1+Tproc, n2-Tproc], where n1 and n2 are respectively the triggering times for the start and end of the contiguous partial sensing window

  - For aperiodic transmissions,

    - [2/HW, HiSi]: n+TA = maxn,ty0SL-31 and n+TB = ty0SL-Tproc,0SL-T1

    - [9/MTK]: n+TA ≥ n+1 and n+TB ≤ n+T2 -T3

    - [10/Fujitsu]: TA and TB are positive integers and TB-TA<31

    - [11/Futurewei]: TA = 1 and TB < 31-Tproc,1SL

    - [17/Apple]: TA=1 and TB= 32-Tproc,0

    - [19/Samsung]: n+TA=n'-31 and n+TB=n-Tproc,0SL, where n’ is n or n+T1

    - [29/DCM]: n+TA=n+Tproc,2SL and n+TB=n+TC-Tproc,0SL+1, where Tproc,2SL=[1] and n+TC≥ty1-Tproc,1SL is the resource selection timing

    - [16/Intel]:

      - ∆A is the max time for UE to switch from a sleeping state to monitoring state needs to be considered. ∆A = 1 meaning that the monitoring window starts at slot ‘n+1’

      - TB = ∆B – T3 ≤ PDB, where the value ∆B is determined by slot corresponding to the last retransmission of a given TB or HARQ feedback, T3 is processing delay in slots

    - [33/E///]:

      - The values TA and TB are adaptive and based on the received HARQ feedback by the UE performing the sensing operation. The minimum duration of partial sensing window is (pre-)configured (which can be zero slot) and is used initially. If NACK is received, the sensing window is increased up to a maximum predetermined value (i.e., min (32, PDB)).

    - Continuous sensing window should be [max(n, n+T1-32), n+T2-T3] [6/vivo]

  - Define n + TB as the resource selection time for contiguous partial sensing based resource selection. [8/CATT, GH]

    - If there are sufficient available sensing results when packet arrives, resource selection time is n where TB is equal to 0.

    - If there are no sufficient available sensing results when packet arrives, resource selection time is n + L– M, where L is the (pre-)configured contiguous partial sensing (minimum) duration and M = |TA| + 1.

      - TA depends on the actual sensing starting time, and |TA |+1 means that the UE has performed a contiguous sensing until slot n.

      - If there are no available sensing results, contiguous partial starts at the next logical slot, i.e., TA = 1.

    - Introduce a higher layer parameter to indicate the minimum contiguous partial sensing duration before resource selection.

  - Sensing from the past 32 slots that are within the intersection of the sensing window and the same selected/configured resource set [18/QC]

  - Adopt predefined windowing with sensing occasions to detect periodic reservations in the pool when UE is transmitting aperiodic traffic [9/MTK]

  - The maximum contiguous partial sensing window is 32 slots. The minimum contiguous partial sensing window is (pre-)configured per priority and can be zero. [27/IDC]

  - Include alignment to the contiguous partial sensing when coexisting with the periodic-based partial sensing, e.g., window size of the contiguous partial sensing or triggering time (or slot) of the contiguous sensing. [33/E///]

- Definition of resource selection window (RSW)

  - UE selected Y candidate slots after resource (re)selection trigger slot n: [2/HW, HiSi], [29/DCM]

    - Y candidate slots is selected with a constraint of ty1≥n+Xy1, where Xy1 is (pre-)configured. [29/DCM]

  - RSW for aperiodic transmissions is between [n+T1, n+T2]

    - [4/OPPO], [19/Samsung]

  - In order to avoid selecting no sensed resources, T2 should not exceed 32 logical slots. [8/CATT, GH]

  - RSW window or a set of slots for selection is confined within a selected/configured resource set. [18/QC]

  - A (pre-)configured resource range / time interval for the final selection of resources among the reported idle resources, to limit the sparse resource selection by MAC layer. [23/LGE]

## Random resource selection (including mixed full/partial sensing with random selection in a same pool)

- Identified issue 1: Persistent collision between a random resource selecting UE with other UEs due to same reservation period [2/HW, HiSi], [3/Nokia, NSB]

  - Due to contiguous NACK for multiple TBs across consecutive periods, when using random selection, reception of NACK across multiple periods of a periodic reservation is a condition for (re-)selecting resources by using exclusion (to turn on sensing). FFS how many periods are required to trigger (re-)selection. [2/HW, HiSi], [8/CATT, GH]

  - Random selecting UE makes resource reservation for a new TB in the next period with a pseudo-random frequency hopping, i.e. reserve the resource R_(x+A,y+P) where P denotes the reservation period in logical SL slots in the pool converted from the reservation period in milliseconds and A denotes the frequency hopping offset determined pseudo-randomly e.g. by the CRC bits of the associated PSCCH. 1 bit in SCI format 1-A to indicate (ON/OFF) the enhanced resource reservation. [3/Nokia, NSB]

    - Conditions in which the UEs may use random resource selection per a TB or more than one consecutive TBs, CBR conditions, priorities of SL transmissions, uses of SL HARQ Option-2 so that the UEs using random resource selection may reselect resources based on HARQ NACKs when persistent collisions happen.

- Identified issue 2: Low priority randomly selected transmission (with no reception capability and no re-evaluation and pre-emption checking) colliding with high priority transmitted from full/partial sensing UE due to mixed configuration of full/partial/no sensing in a same pool [2/HW, HiSi], [4/OPPO], [23/LGE], [29/DCM]

  - A priority threshold is configured for a resource pool, at which reduced sensing UEs can select resources in a pool configured for mixed types of RA [2/HW, HiSi], [27/IDC] – results provided

  - Increase the priority for UE with random selection and use the corresponding priority value in the priority field in the 1st-stage SCI. An extra field is added in SCI for indicating the original priority value associated with QoS requirement. [3/Nokia, NSB]

  - Further study the impact of collision due to sensing UE with higher Tx priority than random selecting UE’s priority [4/OPPO]

  - Different RSRP thresholds or increased RSRP threshold value is (pre-)configured for different resource selection scheme; [14/CMCC], [17/Apple], [27/IDC] Or UE reports whether one candidate resource overlaps with resources reserved by random resource selection UE to higher layer for further resource selection. [14/CMCC]

  - For NR SL random resource selection, consider partitioning of candidate SL resources to reduce collision probability and improve PRR of high priority traffic. [19/Samsung]

  - The pre-emption priority for power saving UE is separately (pre-)configured from that for vehicle UE. [23/LGE]

  - SCI indicates at least one of the following information using the reserved bits. [23/LGE]

    - Type of UE: power-saving UE or vehicle UE [22/ETRI], [27/IDC]

    - Type of RA scheme: partial sensing based or the random resource selection

  - Higher priority is assigned to the resources reserved by a UE performing random selection, to preserve these selected resources from being pre-empted by other UEs. [6/vivo], [9/MTK], [22/ETRI]

  - Adjusting the priority level to lower values or introducing new signalling so that a UE may skip re-evaluation and pre-emption checking when the amount of battery consumed / the number of sensed slots is higher than a (pre)-configured threshold. [10/Fujitsu]

  - Excludes resources reserved by UE performing random selection without re-evaluation / pre-emption checking, regardless of their priorities [29/DCM], [21/Sony]

- Conditions on dynamic switching random selection or partial sensing for resource selection should be specified, and it may include measurement results and QoS requirement (e.g., priority). [6/vivo], [9/MTK], [11/Futurewei]

- The UE capability, requirement on power saving, resource pool configuration, congestion condition (as indicated by CBR etc.) and (pre-)configured minimum contiguous partial sensing duration can be the criteria for random resource selection. [8/CATT, GH]

- R16 principles of resource (re)selection should be followed for random resource selection is used (e.g., Type A and/or B UEs): [16/Intel]

  - Maximum distance separation in logical slots should be 32 for any two resources indicated in a SCI

  - HARQ feedback time gap (Z) between PSSCH-to-PSFCH-to-PSSCH is respected (i.e., Type B UE with PSFCH reception)

- The frequency that a UE performs random resource selection should be restricted (e.g., a minimum duration can be defined between two consecutive triggering of random selections) [15/Xiaomi]

- Conditions / criteria to use random resource selection [16/Intel]

  - UE does not have sidelink RX chain to perform sidelink sensing (i.e., Type A UE)

  - UE is configured to operate in power saving resource allocation mode

- Introduce random selection with subsequent re-evaluation

  - [18/QC]

- For random selection, to re-use the steps specified for LTE as baseline [20/Pana]

- Conditions / cases in which the UE perform random resource selection when it is enabled in a resource pool: [23/LGE]

  - When resource (re)selection is triggered within a threshold from the start of SL DRX ON duration

  - When congestion/interference level in a resource pool is below a threshold

  - When the priority value of a packet is below a threshold (e.g. pre-emption priority value)

  - When PDB is smaller than a (pre-)configured threshold if periodic transmission is not allowed in a resource pool

- When UE randomly selected a resource for periodic transmission, the resource is reselected based on the NR-V2X SPS resource reservation procedure for the following periodic transmissions, similar to LTE-V2X operation. [23/LGE]

- PSFCH resources associated with the randomly selected resources are separately configured from those with the resources based on partial sensing. [23/LGE]

- Support priority based resource set report and resource selection. UE should reserve resources for multiple TBs if partial sensing is allowed in the pool and sl-MultiReserveResource is configured with {enable}. [26/NEC]

- Conditions / cases in which the UE uses random resource selection: [26/NEC]

  - Resource pool configuration

  - Power limitation

  - Sensing accuracy

- A non-sensing UE sharing a resource pool with sensing UEs shall select/reserve resources for consecutive transmissions with a separation/gap large enough so that the sensing UE can react accordingly if a collision happens, i.e., trigger resource re-evaluation/re-selection or pre-emption. [33/E///], [27/IDC]

## Re-evaluation and pre-emption checks

- No re-evaluation and pre-emption checks for UE performing random resource selection

  - [2/HW, HiSi]

- When HARQ-feedback is enabled, detection of a number of NACKs on PSFCH occasions corresponding to a UE’s own PSSCH transmissions can be used to trigger re-evaluation and pre-emption for partial sensing RA.

  - [2/HW, HiSi]

- In order to achieve power saving gain, when performing re-evaluation/pre-emption after contiguous partial sensing based resource selection, the end of checking window should be fixed to n + TB + T2. [8/CATT, GH]

- The ending time of contiguous partial sensing should be the same as ending time of re-evaluation/pre-emption checking (i.e., m-T3) which follows the rules of re-evaluation/pre-emption in R16. [8/CATT, GH]

- UE is only required to sense in the slots in which the SL transmission may reserve a resource overlapping with the resource to be pre-empted or re-evaluated. [15/Xiaomi]

- The partial sensing window of size max {resource selection window size, SCI signalling window size (N = 32 logical slots)} can be used for pre-emption check [16/Intel]

- Support re-evaluation and pre-emption checking based on partial sensing (including both periodic-based partial sensing and contiguous partial sensing) after the resource selection. [17/Apple]

- Both re-evaluation and pre-emption checking with power saving mode(s) can be enabled/disabled by resource pool (pre-)configuration. [19/Samsung]

- The procedure of pre-emption check and re-evaluation check in Rel-16 NR V2X is reused for Rel-17 power saving mode with a fixed sensing window size of W=31 slots  [m-W, m-T3-Tproc,0). [19/Samsung]

- RAN1 should discuss whether a partial sensing UE can select resources in noncandidate slots defined by periodic-based partial sensing slots. [21/Sony]

- Conditions and cases in which the UE should perform re-evaluation and pre-emption checking: [23/LGE]

  - When random resource selection is performed before resource (re)selection by UE that is capable of sensing

    - if additional sensing is possible within remaining PDB

    - if there are any sensing results available for transmission of other packets

  - When the number of the periodic-based partial sensing slots before resource (re)selection is below a threshold

  - When only the contiguous partial sensing is performed before resource (re)selection in a resource pool where the periodic transmission is enabled

  - When the priority value of a packet is above a threshold (e.g., pre-emption priority value) [27/IDC]

  - When the congestion/interference level in a resource pool is above a threshold

  - When the required reliability level of a packet transmission is above a threshold

  - When the number of retransmissions of a packet is below a threshold

  - For selected resources for which sensing results more than a threshold in a contiguous partial sensing window are not available (e.g., the resources selected in the latter part of a selection window)

- Periodic-based partial sensing is kept performed after resource (re)selection for resource re-evaluation or pre-emption checking purpose. [23/LGE]

- UE performs contiguous partial sensing on the window [tr+TC, tr+TD] (TC<0, TD<0) before the timing tr of every selected resource for re-evaluation or pre-emption checking. [23/LGE]

- When periodic transmission is enabled in a resource pool and UE performs aperiodic transmission, resource reselection due to re-evaluation/pre-emption checking can be based on the resource (re)selection procedure defined for periodic transmission, depending on the remaining PDB. [23/LGE]

- For re-evaluation/pre-emption check of a resource at UE performing periodic-based partial sensing and contiguous partial sensing, [29/DCM]

  - The UE uses the same set of Y candidate slots as that determined in the corresponding resource selection.

    - Sensing slots for periodic-based partial sensing are the same.

    - Sensing slots for contiguous partial sensing includes additionally slots within n+TB, m-Tproc,0SL

- For re-evaluation/pre-emption check of a resource at UE performing random resource selection [29/DCM]

  - When a UE selects at slot n resource(s) randomly from a window of [n+T1, n+T2], the UE monitors slots of [n+Tproc,2SL, m−Tproc,0SL] and performs re-evaluation/pre-emption check at slot m, where

    - Tproc,2SL = [1] and m+Tproc,1SL is the slot index of the selected/reserved resource

  - A set of Y candidate slots within [m+T1, m+T2] is determined in the same way as partial sensing.

- For semi-persistent reservation, the UE can skip pre-emption for certain reservation periods. The number of skip periods is (pre-)configured per priority. [27/IDC]

## Congestion control for power saving RA

- CBR measurement needs adaptation for Rel-17 UE with partial sensing or sidelink DRX configuration, to take into account power consumption reduction [2/HW, HiSi]

- UE is not mandated to perform measurement for CBR/CR outside the DRX active time. [6/vivo]

  - Enhancements to CBR/CR calculations are needed due to reduced measurements

- If UE performs periodic-based partial sensing, CBR in slot n can be measured by UE in M periodic partial sensing occasions before slot n, M periodic partial sensing occasions could be a subset of the configured partial sensing occasions. [8/CATT, GH]

- Support enhancements to reduce the number of slots to be sensed for CBR measurements [15/Xiaomi]

- Decisions related to congestion control for partial sensing UEs are discussed when design of partial sensing mechanism is completed (at least enough details are defined). FFS how to support congestion control in case of random resource selection if no SL reception is supported by UE. [16/Intel]

- The evaluation of CR and the definition of CRlimit for power saving resource allocation schemes reuse the design for full sensing resource allocation schemes. [17/Apple]

- If P-UE has no PSCCH/PSSCH reception capability, a (pre-)configured CBR value is used for PHY parameter selection, as in LTE-V2X operation. [23/LGE]

- If P-UE has PSCCH/PSSCH reception capability, the following CBR value is used for PHY parameter selection: [23/LGE]

  - CBR measured in the partial sensing slots if the number of decoded PSCCH/PSSCH slots is above a threshold

  - a (pre-)configured CBR value, otherwise

- Measured CBR in slot n is the ratio of sub-channels whose SL RSSI exceed a (pre-)configured threshold to all the sub-channels in the partial sensing slots within a window [n-a, n-1], where a is (pre-)configured. [23/LGE]

- RSSI measurement should be adjusted based on PSCCH/PSSCH reception types. CBR measure occasion should be adjusted based on monitoring occasions. CBR/CR window should be adjusted considering DRX configuration. [26/NEC]

- Restriction of transmission parameter based on the CBR measurement is performed per active period of a DRX cycle. [28/Lenovo, MM]

## SL HARQ, CSI, RSRP feedback reporting for power saving UE

- UEs using partial sensing or random resource selection for transmissions with enabled SL HARQ feedback are required to monitor the associated PSFCH resources [16/Intel]

  - For Type B and D UE

  - For a Type A UE, it is not expected to receive higher layer indication to enable HARQ feedback for SCI format 2A and 2B.

- When HARQ feedback is enabled for a TB, the resource selection for mixture of blind and HARQ feedback-based retransmissions of the TB is supported. [23/LGE]

- Sidelink unicast, groupcast and broadcast communication should be supported for power saving UE in Rel-17. Legacy HARQ feedback can be reused for power saving UE in Rel-17. [34/ZTE, Sanechips]

## Consideration of SL-DRX in power saving RA

- At this stage the design of the SL DRX in RAN2 is not sufficiently complete to discuss all details in RAN1

  - [16/Intel]

- When DRX is configured for a given UE, its communicating TX UE should ensure that the transmitted data is delivered during SL active time of the UE according to the DRX configuration [2/HW, HiSi], [12/ Zhejiang Lab], [15/Xiaomi], [17/Apple]

- A UE can perform SL reception of PSCCH for sensing during its SL DRX inactive time [2/HW,HiSi], [4/OPPO], [8/CATT, GH], [10/Fujitsu], [18/QC] up to UE implementation, [23/LGE], [27/IDC], [34/ZTE, Sanechips]

- UE is not required to perform sensing out of the DRX active time [6/vivo], [13/ Fraunhofer], [17/Apple], [19/Samsung]

  - UE performs a sensing in a different slot (e.g., with a different value of k for the same Preserve). [17/Apple]

  - Sensing window is adjusted into DRX active time with configured sensing window [19/SS], [13/ Fraunhofer]

  - Sensing is not performed (e.g., using random selection instead) if no sensing slot is within active time [19/SS]

- Contiguous sensing in the DRX ON duration is beneficial to reduce the collision probability of resource selection caused by muted sensing activity during DRX inactive duration. For P2P communication, the candidate resources selected by Tx UE need to be within the DRX active time of Rx UE. The DRX parameters of the Rx UE need to be considered by Tx UE when the Tx UE performs resource selection. [8/CATT, GH]

- In the V2P/D2D broadcast scenario, a UE-common DRX configuration can be pre-configured or configured by the network. In the V2P/D2D groupcast or unicast scenario, intra-group UE-common or UE-specific DRX configurations can be indicated by the Tx UE. [8/CATT, GH]

- Study wake-up signal in sidelink to enhance power saving from Rx-UE’s perspective. [9/MTK]

- For periodic-based partial sensing, it may be beneficial to refine the determination rule of “k” for a given periodicity when SL DRX is configured, from power saving perspective. [10/Fujitsu]

- Align partial sensing with SL DRX on period as much as possible which is facilitated by sensing occasions configured with a bitmap for partial sensing. [11/Futurewei]

- Introduce PSCCH monitoring intervals as a function of L2 or L1 destination IDs, and inter-UE signalling to negotiate sidelink resources (e.g., PSCCH monitoring intervals) where UE(s) are expected to monitor PSCCH resources and perform sensing for sidelink communication [16/Intel]

- The design of SL DRX cycle needs to ensure that UE partial sensing behaviour is respected (i.e., UE wake up time intervals for the purpose of partial sensing need to be aligned with ON duration intervals, as well as traffic characteristics) [16/Intel]

- For unicast and groupcast, the Tx UE transmits on the resources outside of the Rx UE's ON duration only if it receives a NAK in response to the transmission inside the ON duration indicating reservations [18/QC]

- The sidelink UE can take sidelink information (including the sensing/resource allocation timing) into account for the UE assistance information for network to inform the gNB for a better coordination with Uu at the network. [20/Pana]

- UE should exclude resources which it has not monitored during the OFF state of sidelink DRX operation in a resource sensing window. [21/Sony]

- Sensing results obtained by SL DRX operation in active time are used for resource (re)selection, resource re-evaluation/pre-emption checking, in addition to the partial sensing results. [23/LGE]

- Inactivity timer for SL DRX is not less than the maximum interval between the resources that can be indicated by a single SCI (e.g., 31 SL logical slots). [23/LGE]

- The source and destination ID carried by the 2nd SCI can be used for detecting the PSCCH of interest. Whether or not these partial ID information are sufficient for PSCCH detection needs to be further investigated. [23/LGE]

- Interference to Uu link communication due to SL RF ON and OFF operation needs to be considered in configuring or managing the active and the inactive time of SL DRX cycle. [23/LGE]

- If RX-UE is performing SL-DRX operation, resources for the initial and a (pre-)configured minimum number of retransmissions are selected to be included in SL-DRX ON duration within PDB. [23/LGE], [6/vivo], [8/CATT, GH], [10/Fujitsu], [27/IDC]

  - A range of beginning part of a resource selection window overlaps with RX-UE’s SL DRX ON duration, so that a (pre-)configured number of candidate resources are selected in the SL DRX ON duration.

- Study the following potential solutions to reduce impact of SL DRX: [26/NEC]

  - Alignment between sensing occasion and on-duration

  - Request assistance information from other UEs during off-duration

  - Keep awake to monitoring potential PSCCH occasions during off-duration

  - Hypothetical reservations during off-duration

- The UE in SL DRX can perform either sensing-based resource selection or random resource selection. [27/IDC]

- UEs performs candidate resource selection considering common active period [28/Lenovo, MM]

- RAN1 study on the transmission of assistance indication like go-to-sleep to aid Rx UE(s) enter early DRX sleep state. [28/Lenovo, MM]

- When TX-UE has a TB to be transmitted to RX-UE, and if a SL resource scheduled by gNB is not included in active time in the RX-UE, TX-UE skips transmission at the resource and reports the misalignment by HARQ-ACK report to the gNB. [29/DCM]

- No separate TX/RX alignment procedure is specified in RAN1 for partial sensing. [33/E///]

- The (partial) sensing operation and the resource selection performed by a UE takes into account the active time defined by SL DRX configuration, and a UE is mandated to perform the sensing operation in the Active Time determined by the SL-DRX configuration. It is up to UE implementation to perform the sensing outside the Active Time. [33/E///]

- Partial sensing parameter should be taken into sidelink DRX (pre-)configuration, i.e., in RAN1’s perspective, it is assumed that sidelink DRX (pre-)configuration would be aligned with the partial sensing parameter. Power saving UE should perform sensing according to configured partial sensing window irrespective of sidelink DRX configuration. [34/ZTE, Sanechips]

- The length of timer drx-HARQ-RTT-Timer in SL DRX is related to the duration of contiguous partial sensing when SL TX adopts contiguous partial sensing to select resource for the TB retransmission. [7/CAICT]

## SL power control

- SL pathloss based OLPC for PSFCH [6/vivo]

## Other resource allocation for power saving techniques

- gNB (pre-)configures a monitoring interval and a retransmission interval in a period, which can be determined based on latency requirement and data traffic characteristic. A receiving UE only receives/decodes PSCCHs/PSSCHs in the monitoring interval and then determines whether to turn on in the retransmission interval. [3/Nokia,NSB]

- Utilize the geographical location of group UEs and destination-L2 ID, as the reference parameters for partial sensing, in the application layer connection-less group. Utilize the destination-L2 ID, as the reference parameter for partial sensing, in the application layer managed group. [10/Fujitsu]

- Considering public safety and commercial use cases where both transmitters and receivers have the requirement of power saving, enhancements for sensing pattern alignment should be studied. [14/CMCC]

- Partial sensing mechanism in NR sidelink needs to take into account the unlimited increasing of RSRP threshold. [14/CMCC]

  - an upper limit of the number of RSRP threshold increments or the maximum value of increased RSRP threshold can be configured. When the upper limit or the maximum value is reached, UE increases the number of determined set of slots.

- Define a set of periodic partial sensing resource sets partitioning a resource pool. A UE can perform partial sensing over a single or multiple resource sets. The reservation of a resource in a given set can only be signalled from another slot associated with the same resource set. [18/QC]

- A partial sensing UE performs reception over the set of periodic resources assigned for sensing/transmission. Other UEs should know when a partial sensing UE can receive in order to communicate with it. [18/QC]

- For a resource pool enables combination of full sensing, partial sensing and random selection, it could be pre-segregated / partitioned into corresponding portions for each sensing/selection scheme or based on Tx priority to achieve more efficient resource utilization. [20/Pana], [21, Sony], [28/Lenovo, MM]

  - A resource pool partition is configured by a set of disjoint resource patterns.

  - For a resource pool selected for use, a UE can further (re-)select resource pattern(s) based on sensing results.

  - Resource alignment can be performed by indicating identity of resource pattern among UEs.

- A dedicated resource pool should be allowed for partial sensing UEs that the Tx pool not overlap with full sensing UEs’ Tx pool and within full sensing UEs’ Rx pool; while the Rx pool is known to full-sensing UEs for partial-sensing UE targeted SL messages. [20/Pana]

- Selection of early in time resources [29/DCM]

- Mixed blind retransmissions and HARQ-based retransmissions [29/DCM]

- Since Type A UEs cannot perform reception on the PSFCH, we propose that resource pools with random resource selection enabled are defined with PSFCH disabled. [13/ Fraunhofer]

- Restrict the maximum number of retransmissions to be carried out for transmissions by Type A UEs based on the priority of the transmission. [13/ Fraunhofer]

## Other enhancements for power saving

- The mechanisms of power saving enhancement for mode-2 should focus on reducing power consumption of Sensing and PSFCH transmission. [6/vivo]

- In order to ensure reliability, backward indication should be supported to provide as more resource reservation information as possible by limited sensing results. [8/CATT, GH]

- Periodic partial sensing, contiguous partial sensing and random selection are configured per resource pool. [8/CATT, GH]

- In the case of inadequate sensing results and for power saving, UE uses assistance information messages in order to obtain the required sensing information for carrying out resource selection. [13/ Fraunhofer], [5/ Spreadtrum]

- Since power saving UEs are required to be active based on their location, we propose to enable these UEs to wake up and carry out transmissions only when they are in a pre-configured region. [13/ Fraunhofer]

- For the sake of UE sidelink power saving, NR supports adaptation of sidelink power saving resource allocation schemes (i.e., b/w random, partial, or full sensing-based resource selection) [16/Intel]

- Use the reserved bits in SCI format 1-A to indicate the destination ID, so that power saving UEs can avoid decoding transmissions that are not meant for the UE. [13/ Fraunhofer]

- Sidelink bandwidth adaptation for transmission / reception is supported as a power saving feature [16/Intel], [13/ Fraunhofer]

- Reduce UE transaction time for transmission of a TB by randomly picking one out of N first in time candidate resources, where the value N is pre-configured [16/Intel]

- Conditions / criteria by which how a UE determines the RA scheme for a given transmission when more than one scheme is configured for a resource pool (random selection, partial and/or full sensing) [19/Samsung]

  - Higher layer configuration

  - Priority of traffic

  - CBR

  - Remaining PDB

  - HARQ-ACK error rate

  - Status of inter-UE coordination information

  - Existing sensing result

  - Battery level

  - DRX configuration

  - UE capability

  - UE implementation

- To allow 1st SCI only reception in rel.17, and its power modelling of 1st SCI only reception is [0.6]* power consumption level of “PSCCH/PSSCH RX” [20/Pana]

- Allowing a UE can enter a non-sensing mode for power reduction whilst selecting resources based on indication from inter-UE coordination [2/HW, HiSi], [21/Sony], [24/ Convida]

- Supporting narrow BWP (and/or reduced resource pool) operation with BWP switching. [22/ETRI]

- Candidate resource ratio for P-UE in resource (re)selection procedure is separately (pre-)configured from that for vehicle UE. [23/LGE]

- Longer PSFCH period or enhancement of conducting resource selection should be studied. [26/NEC], [7/CAICT]

- Support partial sensing and resource selection based on subset(s) of frequency resources. [27/IDC]

- Mechanism of sensing result sharing by RSU or other UE can be considered for VRU to achieve power saving. [28/Lenovo, MM]

- RAN1 study the cross-slot scheduling enhancement for power saving purposes. [28/Lenovo, MM]

- To reduce power consumption, dedicated BWP can be configured for power saving UE and the transmission bandwidth of S-SSB should be (pre-)configured within the dedicated BWP in Rel-17. [34/ZTE, Sanechips]

- References

- RP-202846	WID revision: NR sidelink enhancement	LG Electronics

- R1-2102323	Sidelink resource allocation to reduce power consumption	Huawei, HiSilicon

- R1-2102361	Sidelink resource allocation for power saving	Nokia, Nokia Shanghai Bell

- R1-2102411	Power saving mechanism in NR sidelink	OPPO

- R1-2102467	Discussion on sidelink resource allocation for power saving	Spreadtrum Communications

- R1-2102539	Resource allocation for sidelink power saving	vivo

- R1-2102575	Considerations on partial sensing in NR V2X	CAICT

- R1-2102606	Discussion on resource allocation for power saving	CATT, GOHIGH

- R1-2102708	 Discussion on sidelink power saving	MediaTek Inc.

- R1-2102719	Considerations on partial sensing and DRX in NR sidelink	Fujitsu

- R1-2102780	Power consumption reduction for sidelink resource allocation	FUTUREWEI

- R1-2102797	Discussion on resource allocation for power saving	Zhejiang Lab

- R1-2102811	NR Sidelink Resource Allocation for UE Power Saving	Fraunhofer HHI, Fraunhofer IIS

- R1-2102897	Discussion on resource allocation for power saving	CMCC

- R1-2102965	Discussion on sidelink resource allocation enhancement for power saving	Xiaomi

- R1-2103048	Sidelink power saving solutions	Intel Corporation

- R1-2103121	Discussion on Sidelink Resource Allocation for Power Saving	Apple

- R1-2103184	Power Savings for Sidelink	Qualcomm Incorporated

- R1-2103257	On Resource Allocation for Power Saving	Samsung

- R1-2103272	Discussion on Sidelink Resource Allocation for Power Saving 	Panasonic Corporation

- R1-2103314	Discussion on sidelink resource allocation for power saving	Sony

- R1-2103331	Discussion on resource allocation for power saving	ETRI

- R1-2103378	Discussion on resource allocation for power saving	LG Electronics

- R1-2103416	On Resource Allocation for Power Saving in NR Sidelink	Convida Wireless

- R1-2103483	Discussion on resource allocation for power saving	Sharp

- R1-2103517	Discussion on resource allocation for power saving	NEC

- R1-2103537	Sidelink resource allocation for power saving	InterDigital, Inc.

- R1-2103548	Sidelink resource allocation for power saving	Lenovo, Motorola Mobility

- R1-2103592	Discussion on sidelink resource allocation for power saving	NTT DOCOMO, INC.

- R1-2103635	Discussion on resource allocation for power saving	Hyundai Motors

- R1-2103640	Discussion on partial sensing and SL DRX impact	ASUSTeK

- R1-2103663	Resource allocation for power saving with partial sensing in NR sidelink enhancement	ITL

- R1-2103704	Resource allocation procedures for power saving	Ericsson

- R1-2103710	Discussion on resource allocation for power saving	ZTE, Sanechips

- R1-2102577	Discussion on LS from RAN2 on SL DRX design	CATT, GOHIGH

- R1-2102928	Draft Reply LS on SL DRX design	vivo

- R1-2103283	[draft]Reply LS on sidelink DRX	ZTE, Sanechips

- R1-2103717	Consideration of SL DRX	ZTE, Sanechips

- R1-2103752	Discussion on RAN2 LS on DRX impact	Huawei, HiSilicon

Appendix (past meeting outcomes)

## RAN1#103-e (26/Oct – 13/Nov 2020)

Conclusion

- SL reception Type A and Type D should be used as the reference for evaluation and designing of SL power saving features in R17.

  - Type A: UE is not capable of performing reception of any SL signals and channels, FFS with exception of performing PSFCH and S-SSB reception (aim to conclude in RAN1#104-e)

  - Type D: UE is capable of performing reception of all SL signals and channels defined in R16. It does not preclude UE to perform reception of a subset of SL signals/channels

  - If there are evaluations with assumptions other than the above reference, the detailed assumptions need to be reported

  - Note: the types and the associated capability defined here are not intended to be defined as Rel-17 UE features as is.

Agreements:

- Partial sensing based RA is supported as a power saving RA scheme

  - FFS details

- Random resource selection is supported as a power saving RA scheme

  - FFS any changes or enhancement

  - FFS on conditions to apply random resource selection

Agreements:

- In R17, a SL Mode 2 Tx resource pool can be (pre-)configured to enable full sensing only, partial sensing only, random resource selection only, or any combination(s) thereof

  - FFS details, including usage, potential restrictions, whether/how any enhancement or condition is needed for the coexistence of full sensing and power saving RA scheme(s) in a same resource pool, etc.

Agreements:

- Re-evaluation and pre-emption checking are not supported by UEs that do not perform any sensing (i.e. PSCCH reception)

- Re-evaluation and pre-emption checking are supported by UEs that perform sensing

  - FFS details and any conditions(s) in which re-evaluation and pre-emption can be performed

- FFS whether/how re-evaluation and pre-emption can be supported by UEs performing random resource selection that do perform sensing

- Note: details about sensing in this context, including when it is performed, are not decided yet.

Agreements:

- Further study congestion control based on CBR and CR for power saving RA schemes

  - Identify necessary changes from R16 CBR/CR (if any), including transmission resource selection and transmission parameters that can be adjusted and applicable to power savings RA schemes

  - Note: this is not intended to require all UEs to perform sensing for the purpose of CBR measurement

## RAN1#104-e (25/Jan – 05/Feb 2021)

Agreements:

- Random resource selection is applicable to both periodic and aperiodic transmissions

  - FFS conditions for random resource selection

Conclusion:

- PSFCH reception is not included for Type A UE

- S-SSB reception is not included for Type A UE

- SL reception Type B is additionally added

  - Type B: Same as Type A with an exception of performing PSFCH and S-SSB reception

- Note: the same conditions as in RAN1#103-e regarding the context of the discussion of Type A and Type D still apply (also applicable to type B)

Agreements: In a resource pool (pre-)configured with at least partial sensing, if UE performs periodic-based partial sensing, at least when the reservation for another TB (when carried in SCI) is enabled for the resource pool and resource selection/reselection is triggered at slot n, it is up to UE implementation to determine a set of Y candidate slots within a resource selection window, where

- FFS condition(s) and timing(s) for which periodic-based partial sensing is performed by UE

- The resource selection window is [n+T1, n+T2]

  - As a baseline, T1 and T2 are defined in the same way as in R16 NR-V2X according to step 1 [TS 38.214 Sec. 8.1.4]

  - Further discuss whether or not to introduce a threshold to re-define T1 and T2 such that

    - T1 ≥ 0 (subject to processing time constraint Tproc, 1), and T2 ≤ remaining PDB

    - T2-T1 ≤ (pre-)configured threshold

- A minimum value for Y is (pre-)configured from a range of values, FFS details

- FFS any restriction to determine Y candidate slots (including its relationship with SL-DRX)

- FFS whether the resource selection window [n+T1, n+T2] should be confined within a set of periodic set of resources and its relationship with SL-DRX

- Note: The terminology “periodic-based partial sensing” is based on the “partial sensing” used in LTE-V and it is intended to be used for the design and discussion of partial sensing in Rel-17.

Agreements: In a resource pool (pre-)configured with at least partial sensing, if UE performs periodic-based partial sensing, at least when the reservation for another TB (when carried in SCI) is enabled for the resource pool and resource selection/reselection is triggered at slot n, the UE monitors slots of at least one a set of periodic sensing occasions, where a periodic sensing occasion is a set of slots according to

if tvSL is included in the set of Y candidate slots.

- Preserve is a periodicity value from the configured set of possible resource reservation periods allowed in the resource pool (sl-ResourceReservePeriodList). Down select to one:

  - Option 1:  Preserve corresponds to all values from the configured set sl-ResourceReservePeriodList

  - Option 2:  Preserve Preserve corresponds to a subset of values from the configured set sl-ResourceReservePeriodList

    - FFS how to determine the subset (e.g., by (pre-)configuration, UE determination)

  - Option 3: Preserve Preserve is a common divisor among values in the configured set sl-ResourceReservePeriodList

  - Option 4: FFS others

- k equals tois selected according to (down select to one)

  - Option 1: Only the most recent sensing occasion within sensing window for a given reservation periodicity before the resource (re)selection trigger or the set of Y candidate slots subject to processing time restriction

  - Option 2: The two most recent sensing occasions within sensing window for a given reservation periodicity before the resource (re)selection trigger or the set of Y candidate slots subject to processing time restriction

  - Option 3: All possible sensing occasions after n –T0

  - Option 4: Only one periodic sensing occasion for one reservation period. The k value is up to UE implementation. Max value for k is (pre-)configured.

  - Option 5: k is (pre-)configured, including multiple values

  - Option 6: (pre-)configuration of a bitmap, same as in LTE-V

  - Option 7: FFS others

- FFS relationship between periodic sensing occasions and SL-DRX

- FFS condition(s) and timing(s) for which periodic-based partial sensing is performed by UE

- Note: companies are encouraged to show performance data for the down selections

Agreements:

- In a resource pool (pre-)configured with at least partial sensing, if UE performs contiguous partial sensing and resource (re-)selection is triggered in slot n, support the following option:

  - Option 1: For the purpose of resource (re-)selection, the UE monitors slots between [n+TA, n+TB] and performs identification of candidate resources, in or after slot n+TB, based on all available sensing results, including periodic-based partial sensing results (if applicable).

    - FFS TA, TB (including the possibility of equal to zero, positive or negative) and remaining details (in particular, whether there should be exclusion of slots, changes in TA/TB values for different purposes, etc.)

    - FFS whether n can be replaced by e.g., index of some of Y candidate slots

  - FFS condition(s) in which contiguous partial sensing is performed by UE

  - FFS interaction with SL-DRX, if any

  - FFS interaction with periodic-based partial sensing, if any

  - Other options are not precluded

  - Note: This option is not to replace random resource selection only without sensing or re-evaluation and pre-emption checking