3GPP TSG RAN WG1 Meeting #105-e  	          	     		  R1-2105205

e-Meeting, May 19th – 27th, 2021

Agenda Item:	8.11.1.2

Source: 	LG Electronics

Title: 	Discussion on inter-UE coordination for Mode 2 enhancements

Document for:	Discussion and decision

- Introduction

In RAN1#104bis-e meeting [1], followings are concluded for the inter-UE coordination for resource allocation enhancement:

Agreement: - Support the following schemes of inter-UE coordination in Mode 2: - Inter-UE Coordination Scheme 1: - The coordination information sent from UE-A to UE-B is the set of resources preferred and/or non-preferred for UE-B’s transmission - FFS details including a possibility of down-selection between the preferred resource set and the non-preferred resource set, whether or not to include any additional information other than indicating time/frequency of the resources within the set in the coordination information - FFS condition(s) in which Scheme 1 is used - Inter-UE Coordination Scheme 2: - The coordination information sent from UE-A to UE-B is the presence of expected/potential and/or detected resource conflict on the resources indicated by UE-B’s SCI - FFS details including a possibility of down-selection between the expected/potential conflict and the detected resource conflict - FFS condition(s) in which Scheme 2 is used Agreements: - Study further to determine the conditions for UEs to be UE-A(s)/UE-B(s) for inter-UE coordination: - Details include applicable scenario(s)/inter-UE coordination scheme(s) - E.g., only UE(s) among the intended receiver(s) of UE-B can be a UE-A, any UE can be a UE-A, high-layer configured, etc. - Including the possibility of being subject to certain conditions and/or capability Agreement: - When UE-B receives the inter-UE coordination information from UE-A, consider at least one of the following options (with details FFS including possibly down-selecting/merging one or more of the options below, applicable scenario(s)/condition(s) for each option, UE behavior) for UE-B’s to take it into account in the resource (re)-selection for its own transmission - For scheme 1: - Option 1-1: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on both UE-B’s sensing result (if available) and the received coordination information - Option 1-2: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based only on the received coordination information - Option 1-3: UE-B’s resource(s) to be re-selected based on the received coordination information - Option 1-4: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on the received coordination information - For scheme 2: - Option 2-1: UE-B can determine resource(s) to be re-selected based on the received coordination information - Option 2-2: UE-B can determine a necessity of retransmission based on the received coordination information

In FL’s summary made in RAN1#104bis-e meeting [2], it is discussed that the information that can be used for generating inter-UE coordination information as follows:

- For inter-UE coordination in Mode 2, consider at least one of the following information (with details FFS) - For Scheme 1 to determine by UE-A the set of resources preferred and/or non-preferred for UE-B’s transmission: - Other UEs’ reserved resources based on UE-A’s sensing result and/or coordination information (e.g., non-preferred resource set) received from other UEs - Coordination information (e.g., preferred resource set) received from other UEs - Information on UE-B’s traffic requirements (e.g., conveyed via triggering information from UE-B, if any) - Location information on UE-B and other UEs - Subset or all of UE-A’s NR SL resources selected for its transmission(s) of TB(s) - UE-A’s scheduled/configured resources for UL - LTE SL transmission and/or reception of UE-A - Resource set selected by UE-A for other UE-Bs’ transmissions - PSFCH transmission and/or reception - UE-A’s candidate resource set based on UE-A’s sensing - UE-B’s ability to use coordination information - Etc. - For Scheme 2 to determine by UE-A the presence of expected/potential and/or detected resource conflict on the resources indicated by UE-B’s SCI: - Other UEs’ reserved resources and/or existing transmission (i.e. used resources) based on UE-A’s sensing result (e.g., measurement, information extracted from SCI) - Information on UE-B’s traffic requirements - Location information on UE-B and other UEs - Subset or all of UE-A’s NR SL resources selected for its transmission(s) of TB(s) - UE-A’s scheduled/configured resources for UL - LTE SL transmission and/or reception of UE-A - PSFCH transmission and/or reception - UE-B’s ability to use coordination information - Etc.

In this contribution, we continue to discuss the inter-UE coordination mechanism in mode 2 resource allocation for enhanced reliability and reduced latency.

- Discussion

In RAN1#104bis-e meeting, two schemes are agreed for inter-UE coordination mechanism. In scheme 1, UE-A can provide UE-B with the set of resources that can be used for UE-B’s resource (re)selection procedure. On the other hand, in scheme 2, UE-A can provide UE-B with the indication of the existence of resource conflict for the resources indicated by UE-B’s SCI, and UE-B can reselect some of the resources indicated by UE-B’s SCI to avoid the resource conflict.

In scheme 1, considering that the form of “A set of resources” can be the set of non-contiguous time-and-frequency resources, depending on the traffic condition or congestion level, the required payload size to express the preferred or non-preferred resources for UE-B’s transmission will be different. To be specific, when the congestion level is low, the number of resources with high interference would be small as well. In this case, it would be beneficial in terms of signaling overhead to consider the non-preferred resources as inter-UE coordination information. On the other hand, if the congestion level is high, the required payload size to express resources with high interference could be huge. Moreover, since UE-A would not indicated all the resources preferred or not preferred for UE-B’s transmission due to the payload size limit, the preferred resource set and the non-preferred resource set could be complementary each other. To be specific, UE-A can provide the preferred resource set in the form of contiguous time-and-frequency resources, and the UE-A can further provide the non-contiguous non-preferred resource set within the preferred resource set. It would be beneficial to efficiently reduce the payload size of the coordination information in scheme 1. In those points of views, it can be considered to support both types of “A set of resources” for inter-UE coordination information.

Proposal 1: For scheme 1, support both the preferred resource set and the non-preferred resource set for UE-B’s transmission.

In scheme 2, according to the FL summary in the last meeting [2], there are divergent views on the gain/loss of the indication of the detected resource conflict compared to Rel-16 Mode 2 resource allocation. In case of resource collision on UE-B’s transmission in the past, the intended RX UE of the UE-B may store contaminated coded bits due to the resource collision in the RX buffer. In this case, even though the UE-B transmits a number of retransmissions for the same TB upon receiving the post-collision indication, the coded bits in the RX buffer may not be recovered. In other words, the post-collision indication may not work properly to enhance the PRR performance. Alternatively, it can be considered that UE-B transmits buffer flush indication to the intended RX UE as in Rel-15 URLLC mechanism (DL pre-emption). Upon receiving the buffer flush indication, the RX UE will abandon the contaminated coded bits stored in the RX buffer and try to decode the TB by using only the recently received coded bits. In this case, the post-collision indication may be helpful to enhance the PRR performance.

Regarding the half-duplex problem on UE-B’s transmission in the past, when the UE-B’s transmission is unicast or groupcast with SL HARQ-ACK feedback Option 2, this problem will be mitigated by the conventional SL HARQ-ACK process. To be specific, for the resources with half-duplex problem, the UE-B will determine the absence of PSFCH of the RX UE, and then the UE-B will determine to perform retransmissions for the same TB. Even for the groupcast with SL HARQ-ACK feedback Option 1, the half-duplex indication can cause unnecessary retransmissions. For instance, the RX UE of the UE-B could be actually located outside the communication range requirement from UE-B’s location. Or, it is possible that the RX UE succeed to decode the TB in the previous transmission(s) for the same TB. In other words, UE-B can transmit a number of PSSCHs for the same TB, and a part of the PSSCH transmissions can suffer from half-duplex problem. Depending on the number of (re)transmissions of the TB, the half-duplex problem may not be critical problem. To support the indication of the detected resource conflict on the resources indicated by UE-B’s SCI, it is necessary to further investigate when this feature will have clear benefit.

Proposal 2: For scheme 2, support at least indication of the expected/potential resource conflict on the resources indicated by UE-B’s SCI.

  - Conditions for UEs to be UE-A(s)/UE-B(s) for inter-UE coordination

For both coordination schemes, it is necessary to determine the relationship between UE(s) providing inter-UE coordination information and UE(s) using the inter-UE coordination information for its resource (re)selection procedure.

One approach is that any peer UEs can provide inter-UE coordination information each other, and the UE receiving the information can use it to determine its own PSCCH/PSSCH transmission resources. For instance, the PSCCH/PSSCH TX UE can determine transmission resources based on inter-UE coordination information provided by the target PSCCH/PSSCH RX UE. Another example is that the RX UE(s) can transmit inter-UE coordination information to the PSCCH/PSSCH TX UE to improve the detection performance of another RX UE. In this approach, UE-A and UE-B would be connected via PC5-RRC at least when UE-B’s PSCCH/PSSCH transmission is unicast. For groupcast and/or broadcast PSSCH transmission, the UE-A and UE-B would try to detect the same set of destination ID(s). Moreover, in case of distance-based operation, UE-A would need to be located within the communication range requirement from the center of the zone indicated by the UE-B. Since the coordination mechanism would require additional computational complexity and battery consumption, all the RX does not need to be UE-A and/or UE-B. In this case, UE’s higher layer can decide whether or not to perform coordination mechanism.

Another approach is that only a certain UE provides inter-UE coordination information to another UE, but the coordination UE does not need to receive inter-UE coordination information from other UE. UE(s) receiving the information can use it to transmit PSCCH/PSSCH to other UE in the group or outside the group. For instance, RSU located in the center of the intersection can provide inter-UE coordination information to each UE group, and the UE(s) along the streets of the intersection can use the information to determine PSCCH/PSSCH transmission resources as shown in Figure 1. Another example is that a leader UE of a group can transmit inter-UE coordination information to other UEs in the group as shown in Figure 2. In this scenario, the leader UE provide a set of resources to a certain UE group, and it would be expected that UEs in the UE group uses these resources for their own transmissions while UEs not belonging to the UE group does not use these resources. Another example is that UE-A is not intended receiver of UE-B’s transmission, and the UE-A detects resource conflict on the resources indicated by UE-B’s SCI. In this case, to accurately determine half-duplex problem between UE-B and UE-B’s RX UE other than UE-A, the UE-A may need to decode UE-B’s PSSCH(s) transmission even though the UE-A is not interested in the destination ID of the PSSCH. To do this, the UE-A could be provided with the set of L2-destination ID(s) to receive for inter-UE coordination.

Figure 1: Example of inter-UE coordination.

Figure 2: Example of inter-UE coordination.

Proposal 3: For the conditions for UEs to be UE-A(s)/UE-B(s) for inter-UE coordination, support one or more of followings:

- Option 1: UE-B uses coordination information from UE-A for resource (re)selection procedure for its own PSCCH/PSSCH transmission to UE(s) including the UE-A.

  - When UE-B’s PSCCH/PSSCH is unicast and UE-A and UE-B are PC5-RRC connected, or

  - When UE-B’s PSCCH/PSSCH is groupcast and UE-A and UE-B share the same groupcast destination ID(s) and UE-A’s location is within the communication range requirement from UE-B’s location if applicable, or

  - When UE-B’s PSCCH/PSSCH is broadcast and UE-A and UE-B share the same broadcast destination ID(s), or

  - When UE(s) are provided by higher layer to be UE-A and/or UE-B

- Option 2: UE-B uses coordination information from UE-A for resource (re)selection procedure for its own PSCCH/PSSCH transmission to UE(s) other than the UE-A.

  - When the radio-distance between UE-A and UE-B is smaller than a certain threshold, or

  - When UE-A is provided with L2-destination ID(s) that can be used for UE-B's transmission(s) via PC5-RRC between UE-A and UE-B if applicable, or

  - When UE(s) are provided by higher layer to be UE-A and/or UE-B

  - Factors for generating inter-UE coordination information

Considering that UE-A is the intended RX UE of UE-B, the UE-A will decide when the UE can perform SL reception and which resources are preferable or not preferable in terms of detection performance to receive UE-B’s PSCCH/PSSCH transmission. First of all, the UE-A needs to check when the UE will transmit UL or SL channels to determine when the UE-A can perform SL reception or when the UE-A cannot perform SL reception. Next, the UE-A can use its own sensing results to determine the resources preferred or not preferred for UE-A’s reception. UE-A’s sensing results can be used for the case when UE-B’s transmission targets UEs other than UE-A. To be specific, UE-A can provide information about high interference resource based on its own sensing results that may or may not be detected by UE-B.

Meanwhile, all the factors to be used for generating the coordination information needs to be known to UE-A before the UE-A prepares and transmits the coordination information as shown in Figure 3. For instance, in Figure 3, UE-A cannot use UL grant received after the coordination information signaling to generate the coordination information.

Figure 3: Example of timeline of inter-UE coordination.

The processing time to decode PSCCH and/or PSSCH or UL grant to be used for inter-UE coordination information needs to be considered. For simplicity, T_proc,0 or T_0 can be inherited to represent this processing time. In scheme 1, the inter-UE coordination information would be transmitted on SCI and/or PSSCH, so the processing time for PSCCH and/or PSSCH preparation needs to be further considered. In this case, T_proc,1 or T_1 can be reused to express this processing time.

Proposal 4: For scheme 1, the set of resources preferred or not preferred for UE-B’s transmission will be determined based on one or more of followings with subject to processing time budget:

  - UE-A’s NR SL resources selected for its transmission(s) of TB(s)

  - UE-A’s configured resources for UL

  - LTE SL transmission and/or reception of UE-A

  - Other UEs’ reserved resources based on UE-A’s sensing result

  - Coordination information (e.g., non-preferred resource set) received from other UEs

- Note: The coordination information transmitted in slot n is generated based on the above factor(s) known to UE-A before slot n-(T_proc,0+T_proc,1).

The minimum PUSCH preparation time is smaller than 1 msec, and then the typical UL grant-to-PUSCH timing would be also relatively small. If the time gap between the end of the coordination information signaling and the beginning of UE-B’s resource selection window is large, in most cases, the detected UL grant will schedules a PUSCH transmission before the UE-B’s selection window, and the PUSCH transmission within the selection window would be transmitted after the coordination information signaling. In this point of view, we prefer to focus on configured UL resources rather than scheduled UL resources in scheme 1.

In case of scheme 2, the inter-UE coordination information would be transmitted on PSFCH format. In this case, the total processing time to prepare the coordination information signaling would be T_proc,0 or T_0 can be reused to express this processing time.

Proposal 5: For scheme 2, the existence of resource conflict on the resources indicated by UE-B’s SCI will be determined based on one or more of followings with subject to processing time budget:

  - UE-A’s NR SL resources selected for its transmission(s) of TB(s)

  - UE-A’s scheduled/configured resources for UL

  - LTE SL transmission and/or reception of UE-A

  - Other UEs’ reserved resources based on UE-A’s sensing result

  - Coordination information (e.g., non-preferred resource set) received from other UEs

- Note: The coordination information transmitted in slot n is generated based on the above factor(s) known to UE-A before slot n-T_proc,0.

Depending on the design of the coordination information signaling in scheme 2, the coordination information signaling can be located near the resources with conflict rather than the SCI providing the information about the reserved resources. For instance, if the PSFCH will be transmitted T_3 slots before the resources with conflict, it is possible to use UE-A’s scheduled UL resources for inter-UE coordination information generation as well. Moreover, in scheme 2, the UE-A can generate the coordination information based on the coordination information received by the UE-A, and the received coordination information could be associated with scheme 1. In other words, the received coordination information in scheme 1 can be used to determine the contents of the coordination information in scheme 2.

When a UE receives more than one PSCCH/PSSCH with SL HARQ-ACK feedback enabled in slot(s) associated with the same PSFCH occasions, then PSFCH TX/TX collision could occur. Depending on the UE capability, for the PSFCH TX/TX collision, the UE may or may not transmit all the PSFCH(s). Meanwhile, the UE-A would not know how many PSFCH UE-B can transmit simultaneously. Moreover, among the periodic reserved resources, it is up to UE’s decision how to use it. To be specific, resources in each period can be used for different RX UEs, and it is also possible to enable or disable the SL HARQ-ACK feedback differently. In this case, even though UE-A detects two PSSCH transmissions of which PSFCH occasions are the same based on the SCIs received in a certain reserved period, it does not always mean that the PSFCH TX/TX or PSFCH TX/RX collision happens in other reserved period(s). For instance, as shown in Figure 4, a UE can use the PSCCH/PSSCH resources for a TB with SL HARQ-ACK feedback enabled, but the UE can use the PSCCH/PSSCH resources in the next reservation period for another TB with SL HARQ-ACK feedback disabled. In this case, PSFCH TX/TX or TX/RX collision does not actually happen.

Figure 4: Example of the usage of periodic reserved resources.

In summary, the UE-A may not know whether PSFCH TX/TX or PSFCH TX/RX collision really happens at another UE side and how many PSFCH TX(s) will be dropped or not. In this case, it is unclear whether PSFCH TX/TX or TX/RX collision really happens or how PSFCH TX/TX or TX/RX collision will be problematic.

Observation 1: UE-A may not know PSFCH TX/TX or PSFCH TX/RX collision happens at UE-B side since the target UE and/or SL HARQ-ACK feedback enabled/disabled can be changed across different periods of UE-B’s periodic reserved resources.

Observation 2: For PSFCH TX/TX collision, UE-A may not know whether a number of PSFCH TXs are dropped or not at UE-B side since the maximum number of simultaneous PSFCH transmissions is determined by UE capability.

Observation 3: For PSFCH TX/RX collision, UE-A may not know whether UE-B prioritizes PSFCH TX or PSFCH RX especially when their SL priorities are the same.

Proposal 6: Since the third UE does not know whether or how PSFCH TX(s) or RX(s) are dropped, it is not desirable to consider PSFCH TX/TX or TX/RX collision for inter-UE coordination generation.

  - Inter-UE coordination information generation

In scheme 1, UE-A will generate the set preferred for UE-B’s transmission to avoid UE-A’s SL or UL transmission occasion(s) and/or high interference resources derived by the UE-A’s sensing results. On the other hand, UE-A will generate the set not preferred for UE-B’s transmission to include UE-A’s SL or UL transmission occasion(s) and/or high interference resources derived by the UE-A’s sensing results. Since UE-A may not know whether UE-B’s PSCCH/PSSCH transmission will target UE-A or not in the future, it can be considered that the UE-A further indicates when the UE-B can use the coordination information for its resource (re)selection procedure. For instance, if the resource set is preferred for UE-A’s reception, the UE-A can provide the indication of that this coordination information is for the case when the UE-A is the intended RX UE of the PSCCH/PSSCH transmission. Alternatively, it can be considered that the UE-A can provide information about how the coordination information is generated (which factor is used to generate it).

In scheme 1, the UE-A may not need to retransmit the coordination information unless the factors used for the coordination information are changed. Considering processing time budget and signaling flooding, it can be considered to focus on long-term scaling variations. For instance, in case of SL transmissions, when the resource reselection counter expires or when the UE-A triggers resource reselection procedure, the UE-A can trigger or transmit the inter-UE coordination information to UE-B in scheme 1. In case of UL transmissions, if periodic or semi-persistent CSI reporting configuration is reconfigured, or if configured grant for PUSCH is reconfigured, or if PUCCH resource for SPS PDSCH is reconfigured, the UE-A can trigger or transmit the inter-UE coordination information to UE-B in scheme 1. Meanwhile, if active UL BWP of the UE-A is changed, these periodic UL transmission information could be changed. If active DL BWP of the UE-A is changed, periodic or semi-persistent CSI reporting and PUCCH resource for SPS PDSCH could be changed as well.

In case of RSU-based inter-UE coordination, the coordination information could be triggered periodically with (pre)configured period. After the inter-UE coordination information is triggered, UE-A would transmit the information in the earliest selected resources of the UE-A, or UE-A would trigger resource (re)selection procedure for transmitting the information. Meanwhile, considering signaling overhead, it is necessary consider that UE-A skips transmitting the inter-UE coordination information for a certain condition. For instance, if the set of resources indicated by the information is not changed before, the UE-A can cancel triggering of the inter-UE coordination information.

Another approach is that UE-B sends request, and the UE-A transmits the inter-UE coordination information upon the received request. In case of resource reselection, UE-A can generate and send the coordination information when the UE-A recognizes that UE-B’s resource(s) are overlapping with another SL transmission or are not suitable for UE-A’s reception.

Proposal 7: For scheme 1, UE-A (re)sends the coordination information with one or more of following conditions:

- if UE-A triggers resource (re)selection procedure due to expiration of resource reselection counter, or

- if UE-A’s configured resources for UL are changed, or

  - e.g. UL CG or periodic/SP CSI reporting or PUCCH for SPS PDSCH is (re)configured, or

  - e.g. active DL or UL BWP is changed

- if a certain period time is passed after the UE-A previously transmits the semi-static inter-UE coordination information, or

- if higher layer at the UE-A side determines to transmit the inter-UE coordination information, or

- if the UE-A receives request to update the coordination information from UE-B, or

- if UE-A recognizes that the resources indicated by UE-B’s SCI are overlapping with another SL transmission

In scheme 1, UE-A may not know which TX resource pool will be used for UE-B’s transmission. Moreover, it is not always guaranteed that the same resource pool ID implies the same set of SL slots belonging to the resource pool. In other words, it would not be sufficient to share resource pool ID only between UE-A and UE-B. Alternatively, when the UE-B provides whole information of the TX resource pool to be used, the overhead for the request signaling could be too large. Moreover, depending on the factors for inter-UE coordination information, UE-A could transmit the coordination information redundantly to each UE-B with different TX resource pools. Alternatively, it can be considered that the resource set in the coordination information is independent on the resource pool. Instead, the resource set could be expressed in a form of resources that can be used for sidelink transmission. In this case, when UE-B receives this resource set, the UE-B can ignore resources outside its TX resource pool.

Observation 4: It is necessary to investigate the domain of the resource set for preferred or non-preferred for UE-B’s transmission

- e.g. TX resource pool to be used for UE-B’s resource (re)selection based on the coordination information, or

- e.g. physical resource domain, or

- e.g. the set of resources that can be used for sidelink

In scheme 2, UE-A will generate the existence of the resource conflict on the resources indicated by UE-B’s SCI based on UE-A’s SL or UL transmission occasion(s) and/or the location of other UEs’ reserved resources derived by UE-A’s sensing result. To be specific, to determine half-duplex problem between UE-B and UE-A, the UE-A can check whether the resources indicated by UE-B’s SCI are overlapping with UE-A’s SL or UL transmission occasion(s) or LTE SL transmission/reception occasion. To determine half-duplex problem between UE-B and UE-B’s RX UE other than UE-A, the UE-A will check whether there are another SL transmission targeting UE-B on the resource occasions indicated by UE-B’s SCI. For instance, when UE-B’s transmission is groupcast, UE-A can check whether the resources indicated by UE-B’s SCI are overlapping with another groupcast PSSCH with the same destination ID or unicast PSSCH of which destination ID is the same as the source ID of the UE-B’s transmission if available.

Meanwhile, after UE-A receives SCI(s) in a certain period, it is not always guaranteed that the next resources reserved by these SCI(s) will use the same setting of source ID, destination ID, and cast type. In other words, unlike resource collision problem, half-duplex problem in the next reservation period could be not actual problem especially when UE-B uses the resources in the next reservation period for a PSCCH/PSSCH with different set of source ID and/or destination ID.

In summary, for a given periodic reserved resources indicated by UE-B, following cases needs to be considered for determining the existence of the half-duplex problem:

- Case 1: Resources reserved by a SCI targeting UE-A are used for PSCCH/PSSCH targeting UE-A.

- Case 2: Resources reserved by a SCI targeting UE-A are used for PSCCH/PSSCH targeting UE other than UE-A.

- Case 3: Resources reserved by a SCI are used for PSCCH/PSSCH with the source ID and/or destination ID used in the SCI.

- Case 4: Resources reserved by a SCI are used for PSCCH/PSSCH with the source ID and/or destination ID not used in the SCI.

Since UE-A may not know how UE-B will use its next reserved resources and UE-B may not know which UE transmits the coordination information, it can be considered that UE-A indicates UE-A’s assumption to determine the existence of the potential/expected resource conflict on the resources indicated by UE-B’s SCI. In this case, UE-B can decide whether or how to use the received coordination information depending on how the UE-B will use its next reserved resources.

Proposal 8: For scheme 2, UE-A indicates following status separately:

- Resource collision is expected on the resource(s) indicated by UE-B’s SCI

- Half-duplex problem is expected on the resource(s) indicated by UE-B’s SCI

  - The intended RX UE is expected to be UE-A

  - The intended RX UE is expected to be a UE other than UE-A

To determine half-duplex problem, the UE-A needs to successfully decode all the 1st and 2nd SCI and TBs associated with the resource conflict. In this case, the UE-A can decide whether the UE-A sends the resource conflict indication based on the L2-source IDs and L2-destination IDs.

For the resource collision, the UE-A needs to transmit the resource conflict indication only if the overlapping PSSCH transmissions are really problematic at the RX UE side. In this case, it can be considered that the UE-A measures RSRP based on the DMRS of the overlapping PSSCHs and compares the RSRP values to decide whether the potential resource collision needs to be informed to the UE-B or not. For instance, if PSSCH#1 and PSSCH#2 are overlapping each other, and if their RSRP measurements at the UE-A side are comparable, it can be expected that the overlapping PSSCHs suffer from high interference each other. Meanwhile, if the portion of the overlapping resources is smaller, the RX UE may successfully decode the TB. In this case, the UE-A may not need to transmit the resource conflict indication to the UE-B.

When we consider distance-based operation (i.e. groupcast PSSCH scheduled by a SCI format 2-B), distance between UE-A and UE-B or distance between UEs associated with the resource conflict could be used for the condition of sending coordination information. For instance, the UE-A is selected among UEs of which location is within the communication range requirement from each UE associated with the resource conflict. It will reduce the number of UEs transmitting the coordination information for the same resource conflict. For the distance between UEs associated with the resource conflict, when one of the UEs is located outside the communication range requirement from another UE associated with the resource conflict, the UE-A does not need to transmit resource conflict indication. The UE-A could estimate the distance between UEs based on the Zone ID in the received SCI format 2-B from these UEs. According to evaluation results in [3], resource conflict indication outperforms over Rel-16 mode 2 RA especially for the short distance between TX UE and RX UE. In this point of view, in scheme 2, it can be considered that the UE-A can send the coordination information if the indicated communication range requirement(s) associated with the resource conflict are small enough.

Proposal 9: For scheme 2, when UE-A observes resource conflict, the UE-A can send coordination information with further checking following conditions:

- When UE-A successfully decodes TBs in PSSCHs associated with resource conflict due to half-duplex problem (i.e., after obtaining L2-Source IDs and L2-Destination IDs), or

- When RSRP measurement(s) associated with resource collision are larger than a (pre)configured threshold at UE-A side, or

- When the difference of RSRP measurements associated with resource collision is smaller than a (pre)configured threshold at UE-A side, or

- When the portion of RBs/sub-channels shared by different PSSCHs is larger than a (pre)configured threshold at UE-A side, or

- For the case when “TX-RX distance-based HARQ feedback” is enabled,

  - When each distance between UE-A and UE-B associated with the resource conflict is smaller than each targeted communication range requirement, or

  - When at least one of UE-Bs associated with resource conflict is located within a targeted communication range requirement of another UE-B, or

  - When the communication range requirement(s) indicated by the received SCI(s) are smaller than a (pre)configured threshold

  - Inter-UE coordination information transmission

For scheme 1, the candidates for the container are new 2nd SCI format, and PSSCH (MAC CE or PC5-RRC signaling). In this case, the UE-A may need to perform resource (re)selection procedure at least for the transmission of the inter-UE coordination information. If the UE-A already has selected resources for its own transmission, the UE may use them for transmitting the coordination information.

If UE-A can transmit the inter-UE coordination information for each UE (group), UE-B may need to know source ID and destination ID corresponding to the inter-UE coordination information to check the validity of the received inter-UE coordination information. In this case, when the 2nd SCI format is used as container for the inter-UE coordination information, this 2nd SCI format needs to include at least L1-source ID and L1-desitmation ID, and then the 2nd SCI format size could be excessively large. Moreover, if the PSSCH associated with the 2nd SCI transmits a TB, additional SCI fields (such as HPN, NDI, RV, HARQ feedback enabled/disabled, and/or cast type indicator) may also need to be included as well.

Meanwhile, considering interleaving pattern used in polar coding, the maximum number of input bits including 24-bit CRC will be 164 bits. Furthermore, since the maximum number of encoded bits of the mother code used for SCI encoding is 512 bits, the maximum payload size for the 2nd SCI format would need to be further reduced considering SCI detection performance. In those point of views, it is not preferable that the payload size for 2nd SCI for the inter-UE coordination information is too large. On the other hand, if the 2nd SCI only on PSSCH without SL-SCH is supported to achieve repetition gain further, it is necessary to investigate whether or how to convey the remaining L2-source ID and L2-desitation ID bits which are supposed to be known after decoding MAC PDU. If these information is conveyed on the 2nd SCI, the payload size of the 2nd SCI format needs to be increased further or the payload size of the inter-UE coordination information would be limited to keep the total payload size of the 2nd SCI format.

Observation 5: When the inter-UE coordination information in scheme 1 is conveyed on the 2nd SCI format, the mother code rate of the polar coding used for the 2nd SCI encoding could be extremely larger than that of the existing 2nd SCI format in Rel-16.

Another approach is the inter-UE coordination information in scheme 1 is conveyed by MAC CE on PSSCH. In this case, the RX UE can derive L2-source ID and L2-destination ID from the received 2nd SCI and the received TB. The conventional 2nd SCI format could be reused to schedule the inter-UE coordination information. When the MAC CE is used as container of the inter-UE coordination information, the payload size could be relatively large. Moreover, when the MAC CE is used, the relevant processing time would be few msec. For instance, the MAC CE is known to UE-B 3*Number of slots in a subframe+1 slots after the end of PSFCH associated with PSSCH containing the inter-UE coordination information. On the other hand, considering HARQ operation, if a TB consists of the inter-UE coordination information and other MAC PDU, the UE would need to transmit the inter-UE coordination information for every retransmissions of the TB. It might cause large signaling overhead.

Observation 6: When the inter-UE coordination information in scheme 1 is conveyed on MAC CE, its payload size could be larger than that of the case of using 2nd SCI format at the expense of the processing time (e.g. roughly 6 mesc).

It can be considered that the inter-UE coordination information is conveyed by PC5-RRC signaling. In this case, the relevant processing time for achieving the coordination information could be few tens or hundreds msec. Moreover, to use PC5-RRC signaling, the coordination information can be transmitted on one or more unicast PSSCH(s). Since the inter-UE coordination information is transmitted on TB(s), the UE would need to transmit the inter-UE coordination information for every retransmissions of the TB(s).

Observation 7: When the inter-UE coordination information in scheme 1 is conveyed on PC5-RRC signaling, the processing time could be excessively larger (e.g. few tens or hundreds msec) compared to the case of using 2nd SCI format or MAC CE. Also, it requires PC5-RRC connection between UE-A and UE-B.

Meanwhile, it is necessary to investigate how to set the parameters of PSCCH/PSSCH transmission for inter-UE coordination information. For instance, destination ID of the coordination information signaling could be (pre)configured for predefined so that only UE-B tries to decode PSCCH/PSSCH to get the inter-UE coordination information. Next, source ID of the coordination information can be determined by UE-A based on the set of IDs that UE-A want to receive. To be specific, in case of unicast, UE-A’s source ID determined by PC5-RRC signaling will be reused for the source ID of the coordination information. In case of groupcast or broadcast, destination IDs that UE-A tries to detect can be used for the source ID of the coordination information. In this case, after UE-B receives the coordination information from UE-A, UE-B will use it for its resource (re)selection only if the destination ID used for UE-B’s transmission is matched with that of the coordination information. In this case, it is possible that more than one UE-B having the same groupcast destination ID can use the coordination information from UE-A commonly. Alternatively, all the parameters of PSCCH/PSSCH for the coordination information signaling could be (pre)configured, and each coordination information could include target destination ID(s) to use it for a resource (re)selection.

Proposal 10: For the container of semi-static coordination information, support MAC CE on PSSCH

- It supports at least the case where the inter-UE coordination information is not multiplexed with other data of UE-A in the same TB.

- FFS: Source ID, destination ID, cast type, SL HARQ-ACK enabled/disabled, priority value of the coordination information.

For scheme 2, it can be considered to reuse a PSFCH format for indicating the existence of resource conflict on the resources indicated by UE-B’s SCI. In the perspective of UE-B, it would be beneficial to distinguish SL HARQ-ACK feedback and the coordination information in scheme 2. To be specific, when the UE-B receives ACK from the RX UE, the UE-B will not perform retransmission or resource (re)selection for the same TB even though the UE-B receives the coordination information in scheme 2.

When the coordination information is transmitted on slot n, UE-A can only considers factors that can cause resource conflict before the slot n-T_proc,0. In this case, if the time gap between the coordination information signaling occasion and the resources with resource conflict is large, the benefit of using scheme 2 would be limited. To be specific, if the PSFCH determination rule for SL HARQ-ACK feedback is directly reused for scheme 2, UE-A may not use scheduled UL resources of UE-A and/or SCI or another coordination information received after the coordination information transmission occasion as shown in Figure 5. Even for SCI reception, it is not always guaranteed that the UE transmitting later SCI (green color) is capable of receiving the coordination information. Considering that the resource reservation period could be few hundreds of msec, this approach will not fully cover resource conflicts caused by aperiodic UL or SL transmission(s).

Figure 5: Example of timeline of the inter-UE coordination in scheme 2.

To mitigate this inefficiency, it can be considered that the coordination information for a resource indicated by UE-B’s SCI is located near the resource itself rather than the location of the SCI providing the information about the resource. Considering specification work load, it can be considered to reuse the PSFCH resource determination rule for SL HARQ-ACK feedback except that the applying order in time domain is reverse, and the minimum time gap between PSFCH-to-PSSCH can be large. To be specific, for a given resource with potential/expected resource conflict, its associated PSFCH resource for the coordination information will be located in the most recent PSFCH occasion K slots before the resource with potential/expected resource conflict as shown in Figure 6. Considering processing time for re-evaluation/pre-emption checking, the minimum time gap between the coordination information transmission occasion and the resource with potential/expected resource conflict will be T_3 or T_proc,1. In this approach, the UE-A can use all the resource conflict factor such as SCI or UL grant or another coordination information known to the UE-A T_proc,0+T_proc,1 slots before the resource with potential/expected resource conflict as shown in Figure 6.

Figure 6: Example of timeline of the inter-UE coordination in scheme 2.

As mentioned in previous section, since UE-A may not know how UE-B will use its next reserved resources, UE-A needs to indicates UE-A’s assumption to determine the existence of the potential/expected resource conflict on the resources indicated by UE-B’s SCI. In this case, depending on UE-A’s assumption, different PSFCH resource and/or different PSFCH state could be used for the inter-UE coordination information in scheme 2.

Proposal 11: For the container of coordination information in scheme 2, support PSFCH format.

- PSFCH resource set is separately configured for PSFCH resource set for SL HARQ-ACK feedback

- PSFCH timing is derived from the resources with potential/expected resource conflict

  - Apply implicit PSFCH resource determination rule with time-reverse order

  - Minimum PSFCH-to-PSSCH timing K is replaced with the processing time budget T_3

- Different PSFCH resource and/or PSFCH state (i.e. m_CS) are used to indicate the type of resource conflict (e.g. half-duplex or resource collision problem)

- Different PSFCH resource and/or PSFCH state (i.e. m_CS) are used to indicate whether the UE-A is intended RX of UE-B or not

  - Usage of Inter-UE coordination information for resource (re)selection

In scheme 1, UE-B will use the received inter-UE coordination information after the UE-B triggers resource (re)selection procedure for its own transmission. Moreover, the coordination information needs to be valid for UE-B’s resource (re)selection procedure. In other words, the resource set provided by the inter-UE coordination information needs to be related to the resources within UE-B’s resource selection window.

If the destination ID to be used for UE-B’s transmission is not the same as the target destination ID of the coordination information, the information would be invalid. For instance, if the inter-UE coordination information is resource set which is preferred or not preferred for UE-A’s reception, and if the UE-B will not transmit PSCCH/PSSCH to the UE-A, the UE-B does not need to use the information for UE-B’s resource (re)selection procedure.

Considering the accuracy of the inter-UE coordination information in scheme 1, UE-B may need to consider radio distance or geographical distance between UE-A and UE-B to decide whether or not to use the information transmitted from UE-A. Depending on the intended receiver UEs of the inter-UE coordination information, the UE-B could use the information based on the cast type of its own transmission. Even for the broadcast, the inter-UE coordination information sent by UE-A could be used for UE-B’s resource (re)selection procedure to ensure UE-A can receive the broadcast message sent by the UE-B.

Proposal 12: In scheme 1, for the condition of using the inter-UE coordination information at UE-B side, one or more of followings is supported:

- When UE-B triggers resource (re)selection for its own transmission (i.e. UE-B has data to transmit), or

- When receiving the valid inter-UE coordination information with satisfying one or more of following conditions

  - E.g. the information is related to a PSCCH/PSSCH to be transmitted by UE-B, or

  - E.g. The intended receiver of a PSCCH/PSSCH to be transmitted by UE-B is UE-A, or

  - E.g. At least one preferred resource provided by the information overlaps with/included in UE-B’s candidate resources during the resource (re)selection procedure, or

  - E.g. RSRP measurement based on the container of the information at UE-B side is higher than the (pre)configured RSRP threshold.

Observation 8: Even though the received inter-UE coordination information targets a certain cast type and/or link, it could be beneficial for UE-B to take it into account in a resource (re)selection procedure for packet transmission targeting other cast type and/or link.

Depending on the coordination information types (e.g. Type A or Type B), when the UE-B receives the inter-UE coordination information in scheme 1, the UE-B may or may not use resources inside the indicated “A set of resources” from UE-A to transmit its own PSCCH/PSSCH to UE-A. It will improve the detection performance of PSCCH/PSSCH at UE-A side. On the other hand, the UE-B still have a choice not to use some portion of the resources provided by UE-A. For instance, even for the preferred resources for UE-B’s transmission provided by UE-A, the UE-B can determine some of them as high interference resource via its own sensing operation. In this case, if the UE-B uses these resources with high interference at UE-B side for its own PSCCH/PSSCH transmission, this PSCCH/PSSCH transmission will make another interference to other UE even though these resources could suffer from low interference in the perspective of the UE-A.

When the UE-A determines the contents of “A set of resources” to include these problematic resources for UE-B’s transmission, the UE-B may not use resources indicated by the inter-UE coordination information from UE-A to transmit its own PSCCH/PSSCH to UE-A. It will improve the detection performance of PSCCH/PSSCH at UE-A side.

UE-B will perform resource (re)selection procedure based on the received SCI at UE-B side. When the UE-B decide to use the inter-UE coordination information for its resource (re)selection procedure, UE-B will use the information to determine the candidate resources or selected resources for its transmission in addition to the received SCI at UE-B side. In our perspective, it is separate issue whether or how to handle the excessive number of RSRP threshold boosting when the UE-B use both its sensing results and the received coordination information for its resource selection procedure.

In RAN1#104bis-e meeting, it was discussed that the possibility that UE-B’s sensing results is not available and the possibility that the UE-B use the coordination information for its resource (re)selection procedure in scheme 1. For further discussion, it would be necessary to clarify first the exact meaning of the case when UE-B’s sensing results is unavailable. It can simply imply that the UE-B has no RX capability. However, in this case, the UE-B cannot receive the coordination information from another UE as well. Alternatively, it can be considered that the UE with RX capability is configured with random resource selection procedure. However, it is not yet agreed to support random resource selection for the UE with RX capability in the normal resource pool. In our understanding, it is necessary to decide first whether or not to support this feature. Another approach would be the case when the number of non-monitored slots within a sensing window is larger than a certain threshold due to half-duplex restriction or SL-DRX.

Considering that the inter-UE coordination information is the set of slots where UE-A cannot perform SL reception, UE-B first determine candidate resources based on the received SCI as in Rel-16 resource (re)selection procedure. If the intended receiver UE of UE-B is UE-A, then the UE-B determine selected resources from the candidate resources to avoid the resource set provided by the information.

Proposal 13: For scheme 1, when UE-B receives the inter-UE coordination information from UE-A, UE-B performs following procedure

- Option 1-1: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on both UE-B’s sensing result and the received coordination information

  - Alt 1: UE-B finally selects the resources within the candidate resource set identified based on UE-B’s sensing results as in Rel-16 Mode-2 after excluding the non-preferred resource set provided by the information.

  - Alt 2: UE-B finally selects the resources among the intersection of candidate resource set identified based on UE-B’s sensing results as in Rel-16 Mode-2 and the preferred resource set provided by the information.

  - Alt 3: When the selected resources of UE-B based on its own sensing results is overlapping with the non-preferred resources provided by the information, the UE-B can report this overlapping resource to reselect the corresponding resource.

  - Alt 4: When the selected resources of UE-B based on its own sensing results is overlapping with the resources outside the preferred resources provided by the information, the UE-B can report this overlapping resource to reselect the corresponding resource.

In Rel-16 resource (re)selection procedure, when the ratio of the candidate resources over a resource selection window is smaller than (pre)configured threshold X%, the UE finds candidate resources again with the boosted RSRP threshold. If the inter-UE coordination information can cause the case where the RSRP threshold is boosted, UE-B may use reserved resources of other UEs for PSCCH/PSSCH transmission. It would have impact on Rel-16 UE which determine candidate resources and selected resource based on the received SCI only. In this case, it can be considered that at least RSRP threshold for RX pattern associated with the received SCI will be boosted only if the ratio of the candidate resources determined based on the received SCI only is smaller than X%. In other words, after applying the inter-UE coordination information, the final ratio of the candidate resources over a resource selection window can be smaller than X%.

On the other hand, if the final ratio of the candidate resource is too small, it would not be useful to avoid resource collision among mode 2 UEs. In this case, it can be considered that if the final ratio of the candidate resource is smaller than another (pre)configured threshold Y%, UE-B can determine candidate resource with boosted RSRP threshold for the inter-UE coordination information. To avoid that UE-B uses resources with high interference for PSCCH/PSSCH transmission, it can be considered to employ maximum value of the RSRP threshold for the inter-UE coordination information. Alternatively, if the final ratio of the candidate resource is too small, UE-B can cancel to use the inter-UE coordination information for its resource (re)selection procedure.

Proposal 14: For UE-B’s resource (re)selection procedure, the minimum ratio of candidate resources over a resource selection window is guaranteed with one or more of following ways:

- The minimum ratio is guaranteed only based on UE-B’s sensing results as in Rel-16 Mode-2 (i.e. No RSRP boosting due to other condition), or

- The minimum ratio is guaranteed for the outcome resources derived by both UE-B’s sensing results as in Rel-16 Mode-2 and the resource set provided by the inter-UE coordination information.

Depending on the condition of triggering the inter-UE coordination information, the UE-B could receive the updated information from the same UE-A during the resource (re)selection procedure. For simplicity, UE-B can use the updated information in the next resource (re)selection procedure, but the efficiency of using the inter-UE coordination information could be degraded due to the old-fashioned information. Alternatively, the UE-B performs the resource (re)selection procedure again by using the updated information for a certain condition. For instance, the UE-B can trigger the resource (re)selection again when the remaining PDB of UE-B is large enough or when the resource set provided by the inter-UE coordination information is quite different compared to the previous assistance information.

Proposal 15: For scheme 1, when UE-B receives the updated coordination information from the same UE-A, UE-B can retrigger resource (re)selection procedure to use the updated information when one or more of following conditions is met:

- When the remaining PDB of UE-B’s transmission is higher than a (pre)configured threshold, or

- When the portion of difference between the previous information and the updated information is higher than a (pre)configured threshold.

For scheme 2, the UE-B can receives the information about the existence of resource conflict on the resources indicated by UE-B’s SCI. The UE-B could reserve retransmission resources of a TB by using a SCI of the initial transmission of the same TB, and the UE-B could also reserve initial transmission resource and/or retransmission resources for the different TB(s). In case of retransmission resources, the UE-B may or may not use them depending on the received SL HARQ-ACK feedback. To be specific, when the UE-B receives ACK for a TB, the UE-B will not use remaining reserved resources for the same TB. In this case, the UE-B does not need to perform resource reselection for these retransmission resources.

As mentioned in earlier section, UE-B can decide how to use the periodically reserved resources. To be specific, for each period, UE-B can change the setting of source ID, destination ID, cast type, and whether SL HARQ-ACK feedback is enabled or disabled. In this case, when UE-A generates the inter-UE coordination information with an assumption that the UE-B will transmit PSCCH/PSSCH targeting UE-A, the UE-B can use this coordination information only if the PSCCH/PSSCH to be transmitted by the UE-B will target the UE-A. Another example is that when UE-A determines half-duplex problem based on the SCIs received by UE-B and another UE, the UE-B can ignore the coordination information from the UE-A if the UE-B changes the setting of the source ID and/or destination ID for the PSCCH/PSSCH on the resource associated with the coordination information.

Proposal 16: In scheme 2, for the condition of using the inter-UE coordination information at UE-B side, one or more of followings is supported:

- When receiving the valid inter-UE coordination information with satisfying one or more of following conditions

  - E.g. the information is related to a PSCCH/PSSCH to be transmitted by UE-B, or

  - E.g. The intended receiver of a PSCCH/PSSCH to be transmitted by UE-B is UE-A, or

  - E.g. The parameters (e.g. source ID, destination ID) of a PSCCH/PSSCH to be transmitted by UE-B is not changed

Proposal 17: For scheme 2, when UE-B receives the inter-UE coordination information from UE-A, UE-B performs following procedure

- Option 2-1: UE-B can determine resource(s) to be re-selected based on the received coordination information

  - When the coordination information indicates the existence of resource conflict on UE-B’s reserved resources, or

  - When the indicated resource is associated with the initial transmission for a TB, or

  - When the indicated resource(s) are associated with the retransmission for a TB and when UE-B have not received ACK for the TB

When UE-B receives the coordination information in scheme 2, the UE-B can report its own reserved resources associated with the resource conflict to higher layer for resource reselection. In case of half-duplex problem, the UE-B can assume that all the frequency resources in a slot associated with the resource conflict is non-preferred resources for UE-B’s transmission. In case of resource collision, the UE-B can assume that the reserved time-and-frequency PSSCH resources with resource conflict is non-preferred resources for UE-B’s transmission. Alternatively, it can be considered that the resource conflict indication can indicate which part of reserved resources is problematic, and the UE-B can consider it for its resource resection.

  - Evaluation results

In this contribution, we provide evaluation results for scheme 1. To be specific, in urban scenario, RSU for inter-UE coordination is deployed in the center of each intersection as shown in Figure 1. Therefore, there is no initial setup between UE-A and UE-B. Moreover, in this case, the inter-UE coordination scheme does not need to be transmitted frequently, the latency and signaling overhead for transmitting and processing inter-UE coordination information are not major factors to determine Mode 2 RA performance. It is assumed that there are two UE groups depending on the direction of the intersection. Note that due to the building blockage, UEs in different UE group would not recognize each other. In other words, hidden-node problem will occur between different UE groups if there is no inter-UE coordination. For simplicity, it is considered that the RSU of each intersection provide different sets of preferred resources across different UE groups. Furthermore, it is assumed that adjacent RSUs will provide different sets of preferred resources for the same street. In this case, UEs along with the same street could use different resources for its own transmission, and it will alleviate the hidden-node problem due to vehicle blockage or large pathloss.

Observation 9: For scheme 1, when RSU provides the set of resources to UE-B in advance of UE-B’s transmission, latency and signaling overhead for transmitting and processing inter-UE coordination information are negligible.

Next, the UE-B will perform sensing and resource (re)selection procedure for the resource set provided for the UE group of the UE-B. Remaining detailed evaluation assumption can be found in the appendix. In reference system, each UE will perform resource (re)selection procedure for the all the SL resources without help of the RSU.

As shown in Figure 7, the RSU-based inter-UE coordination could achieve 4.96% gain of PRR compared to the reference system in urban scenario for periodic broadcast traffic at 150m. The gain in terms of coverage extension at PRR=0.95 is 10m. This gain could be achievable by mitigating the hidden-node problem due to building blockage, vehicle blockage, or other SCI reception failure.

Figure 7: Average PRR gain vs distance for the semi-static inter-UE coordination.

Observation 10: Where RSU provides the set of resources to UE-B in advance of UE-B’s transmission, 4.96% PRR gain at 150m and coverage extension of 10m at PRR=0.95 are observed in urban scenario for periodic broadcast traffic.

- Conclusions

In this contribution, we discussed inter-UE coordination for mode 2 resource allocation enhancement. Based on the above discussion, our observations and proposals are given as follows:

Observation 1: UE-A may not know PSFCH TX/TX or PSFCH TX/RX collision happens at UE-B side since the target UE and/or SL HARQ-ACK feedback enabled/disabled can be changed across different periods of UE-B’s periodic reserved resources.

Observation 2: For PSFCH TX/TX collision, UE-A may not know whether a number of PSFCH TXs are dropped or not at UE-B side since the maximum number of simultaneous PSFCH transmissions is determined by UE capability.

Observation 3: For PSFCH TX/RX collision, UE-A may not know whether UE-B prioritizes PSFCH TX or PSFCH RX especially when their SL priorities are the same.

Observation 4: It is necessary to investigate the domain of the resource set for preferred or non-preferred for UE-B’s transmission

- e.g. TX resource pool to be used for UE-B’s resource (re)selection based on the coordination information, or

- e.g. physical resource domain, or

- e.g. the set of resources that can be used for sidelink

Observation 5: When the inter-UE coordination information in scheme 1 is conveyed on the 2nd SCI format, the mother code rate of the polar coding used for the 2nd SCI encoding could be extremely larger than that of the existing 2nd SCI format in Rel-16.

Observation 6: When the inter-UE coordination information in scheme 1 is conveyed on MAC CE, its payload size could be larger than that of the case of using 2nd SCI format at the expense of the processing time (e.g. roughly 6 mesc).

Observation 7: When the inter-UE coordination information in scheme 1 is conveyed on PC5-RRC signaling, the processing time could be excessively larger (e.g. few tens or hundreds msec) compared to the case of using 2nd SCI format or MAC CE. Also, it requires PC5-RRC connection between UE-A and UE-B.

Observation 8: Even though the received inter-UE coordination information targets a certain cast type and/or link, it could be beneficial for UE-B to take it into account in a resource (re)selection procedure for packet transmission targeting other cast type and/or link.

Observation 9: For scheme 1, when RSU provides the set of resources to UE-B in advance of UE-B’s transmission, latency and signaling overhead for transmitting and processing inter-UE coordination information are negligible.

Observation 10: Where RSU provides the set of resources to UE-B in advance of UE-B’s transmission, 4.96% PRR gain at 150m and coverage extension of 10m at PRR=0.95 are observed in urban scenario for periodic broadcast traffic.

Proposal 1: For scheme 1, support both the preferred resource set and the non-preferred resource set for UE-B’s transmission.

Proposal 2: For scheme 2, support at least indication of the expected/potential resource conflict on the resources indicated by UE-B’s SCI.

Proposal 3: For the conditions for UEs to be UE-A(s)/UE-B(s) for inter-UE coordination, support one or more of followings:

- Option 1: UE-B uses coordination information from UE-A for resource (re)selection procedure for its own PSCCH/PSSCH transmission to UE(s) including the UE-A.

  - When UE-B’s PSCCH/PSSCH is unicast and UE-A and UE-B are PC5-RRC connected, or

  - When UE-B’s PSCCH/PSSCH is groupcast and UE-A and UE-B share the same groupcast destination ID(s) and UE-A’s location is within the communication range requirement from UE-B’s location if applicable, or

  - When UE-B’s PSCCH/PSSCH is broadcast and UE-A and UE-B share the same broadcast destination ID(s), or

  - When UE(s) are provided by higher layer to be UE-A and/or UE-B

- Option 2: UE-B uses coordination information from UE-A for resource (re)selection procedure for its own PSCCH/PSSCH transmission to UE(s) other than the UE-A.

  - When the radio-distance between UE-A and UE-B is smaller than a certain threshold, or

  - When UE-A is provided with L2-destination ID(s) that can be used for UE-B's transmission(s) via PC5-RRC between UE-A and UE-B if applicable, or

  - When UE(s) are provided by higher layer to be UE-A and/or UE-B

Proposal 4: For scheme 1, the set of resources preferred or not preferred for UE-B’s transmission will be determined based on one or more of followings with subject to processing time budget:

  - UE-A’s NR SL resources selected for its transmission(s) of TB(s)

  - UE-A’s configured resources for UL

  - LTE SL transmission and/or reception of UE-A

  - Other UEs’ reserved resources based on UE-A’s sensing result

  - Coordination information (e.g., non-preferred resource set) received from other UEs

- Note: The coordination information transmitted in slot n is generated based on the above factor(s) known to UE-A before slot n-(T_proc,0+T_proc,1).

Proposal 5: For scheme 2, the existence of resource conflict on the resources indicated by UE-B’s SCI will be determined based on one or more of followings with subject to processing time budget:

  - UE-A’s NR SL resources selected for its transmission(s) of TB(s)

  - UE-A’s scheduled/configured resources for UL

  - LTE SL transmission and/or reception of UE-A

  - Other UEs’ reserved resources based on UE-A’s sensing result

  - Coordination information (e.g., non-preferred resource set) received from other UEs

- Note: The coordination information transmitted in slot n is generated based on the above factor(s) known to UE-A before slot n-T_proc,0.

Proposal 6: Since the third UE does not know whether or how PSFCH TX(s) or RX(s) are dropped, it is not desirable to consider PSFCH TX/TX or TX/RX collision for inter-UE coordination generation.

Proposal 7: For scheme 1, UE-A (re)sends the coordination information with one or more of following conditions:

- if UE-A triggers resource (re)selection procedure due to expiration of resource reselection counter, or

- if UE-A’s configured resources for UL are changed, or

  - e.g. UL CG or periodic/SP CSI reporting or PUCCH for SPS PDSCH is (re)configured, or

  - e.g. active DL or UL BWP is changed

- if a certain period time is passed after the UE-A previously transmits the semi-static inter-UE coordination information, or

- if higher layer at the UE-A side determines to transmit the inter-UE coordination information, or

- if the UE-A receives request to update the coordination information from UE-B, or

- if UE-A recognizes that the resources indicated by UE-B’s SCI are overlapping with another SL transmission

Proposal 8: For scheme 2, UE-A indicates following status separately:

- Resource collision is expected on the resource(s) indicated by UE-B’s SCI

- Half-duplex problem is expected on the resource(s) indicated by UE-B’s SCI

  - The intended RX UE is expected to be UE-A

  - The intended RX UE is expected to be a UE other than UE-A

Proposal 9: For scheme 2, when UE-A observes resource conflict, the UE-A can send coordination information with further checking following conditions:

- When UE-A successfully decodes TBs in PSSCHs associated with resource conflict due to half-duplex problem (i.e., after obtaining L2-Source IDs and L2-Destination IDs), or

- When RSRP measurement(s) associated with resource collision are larger than a (pre)configured threshold at UE-A side, or

- When the difference of RSRP measurements associated with resource collision is smaller than a (pre)configured threshold at UE-A side, or

- When the portion of RBs/sub-channels shared by different PSSCHs is larger than a (pre)configured threshold at UE-A side, or

- For the case when “TX-RX distance-based HARQ feedback” is enabled,

  - When each distance between UE-A and UE-B associated with the resource conflict is smaller than each targeted communication range requirement, or

  - When at least one of UE-Bs associated with resource conflict is located within a targeted communication range requirement of another UE-B, or

  - When the communication range requirement(s) indicated by the received SCI(s) are smaller than a (pre)configured threshold

Proposal 10: For the container of semi-static coordination information, support MAC CE on PSSCH

- It supports at least the case where the inter-UE coordination information is not multiplexed with other data of UE-A in the same TB.

- FFS: Source ID, destination ID, cast type, SL HARQ-ACK enabled/disabled, priority value of the coordination information.

Proposal 11: For the container of coordination information in scheme 2, support PSFCH format.

- PSFCH resource set is separately configured for PSFCH resource set for SL HARQ-ACK feedback

- PSFCH timing is derived from the resources with potential/expected resource conflict

  - Apply implicit PSFCH resource determination rule with time-reverse order

  - Minimum PSFCH-to-PSSCH timing K is replaced with the processing time budget T_3

- Different PSFCH resource and/or PSFCH state (i.e. m_CS) are used to indicate the type of resource conflict (e.g. half-duplex or resource collision problem)

- Different PSFCH resource and/or PSFCH state (i.e. m_CS) are used to indicate whether the UE-A is intended RX of UE-B or not

procedure to ensure UE-A can receive the broadcast message sent by the UE-B.

Proposal 12: In scheme 1, for the condition of using the inter-UE coordination information at UE-B side, one or more of followings is supported:

- When UE-B triggers resource (re)selection for its own transmission (i.e. UE-B has data to transmit), or

- When receiving the valid inter-UE coordination information with satisfying one or more of following conditions

  - E.g. the information is related to a PSCCH/PSSCH to be transmitted by UE-B, or

  - E.g. The intended receiver of a PSCCH/PSSCH to be transmitted by UE-B is UE-A, or

  - E.g. At least one preferred resource provided by the information overlaps with/included in UE-B’s candidate resources during the resource (re)selection procedure, or

  - E.g. RSRP measurement based on the container of the information at UE-B side is higher than the (pre)configured RSRP threshold.

Proposal 13: For scheme 1, when UE-B receives the inter-UE coordination information from UE-A, UE-B performs following procedure

- Option 1-1: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on both UE-B’s sensing result and the received coordination information

  - Alt 1: UE-B finally selects the resources within the candidate resource set identified based on UE-B’s sensing results as in Rel-16 Mode-2 after excluding the non-preferred resource set provided by the information.

  - Alt 2: UE-B finally selects the resources among the intersection of candidate resource set identified based on UE-B’s sensing results as in Rel-16 Mode-2 and the preferred resource set provided by the information.

  - Alt 3: When the selected resources of UE-B based on its own sensing results is overlapping with the non-preferred resources provided by the information, the UE-B can report this overlapping resource to reselect the corresponding resource.

  - Alt 4: When the selected resources of UE-B based on its own sensing results is overlapping with the resources outside the preferred resources provided by the information, the UE-B can report this overlapping resource to reselect the corresponding resource.

Proposal 14: For UE-B’s resource (re)selection procedure, the minimum ratio of candidate resources over a resource selection window is guaranteed with one or more of following ways:

- The minimum ratio is guaranteed only based on UE-B’s sensing results as in Rel-16 Mode-2 (i.e. No RSRP boosting due to other condition), or

- The minimum ratio is guaranteed for the outcome resources derived by both UE-B’s sensing results as in Rel-16 Mode-2 and the resource set provided by the inter-UE coordination information.

Proposal 15: For scheme 1, when UE-B receives the updated coordination information from the same UE-A, UE-B can retrigger resource (re)selection procedure to use the updated information when one or more of following conditions is met:

- When the remaining PDB of UE-B’s transmission is higher than a (pre)configured threshold, or

- When the portion of difference between the previous information and the updated information is higher than a (pre)configured threshold.

Proposal 16: In scheme 2, for the condition of using the inter-UE coordination information at UE-B side, one or more of followings is supported:

- When receiving the valid inter-UE coordination information with satisfying one or more of following conditions

  - E.g. the information is related to a PSCCH/PSSCH to be transmitted by UE-B, or

  - E.g. The intended receiver of a PSCCH/PSSCH to be transmitted by UE-B is UE-A, or

  - E.g. The parameters (e.g. source ID, destination ID) of a PSCCH/PSSCH to be transmitted by UE-B is not changed

Proposal 17: For scheme 2, when UE-B receives the inter-UE coordination information from UE-A, UE-B performs following procedure

- Option 2-1: UE-B can determine resource(s) to be re-selected based on the received coordination information

  - When the coordination information indicates the existence of resource conflict on UE-B’s reserved resources, or

  - When the indicated resource is associated with the initial transmission for a TB, or

  - When the indicated resource(s) are associated with the retransmission for a TB and when UE-B have not received ACK for the TB

Reference

[1] 3GPP RAN1#104bis-e meeting chairman’s note.

[2] R1-2104103, “Feature lead summary for AI 8.11.1.2 Inter-UE coordination for Mode 2 enhancements,” Moderator (LG Electronics).

[3] R1-2102166, “Detailed observations from evaluation results,” Moderator (LG Electronics).

Appendix

Details on system-level simulation assumptions are summarized in Table 1.

Table 1: System-level simulation assumption.

| Parameters | Values |
| --- | --- |
| Cast type | Broadcast |
| Sidelink Frequency | 6GHz |
| Traffic models | Periodic traffic: Medium Intensity |
| Simulation Environment | Urban Option A (60kmph) |
| Number of Tx/Rx Antenna elements | 1Tx/2Rx |
| Antenna Models | Option 1 |
| SL Simulation BW | 40MHz |
| SCS | 30 kHz |
| Pathloss, shadowing, blocking and dual mobility models | Enabled (as per TR 37.885) |