3GPP TSG RAN WG1 #105			R1-2105666

e-meeting, May 10th- 27th, 2021

## Agenda item:  8.1.2.3

Source: 	AT&T

Title: 	Beam Management Enhancements for Multi-TRP

Document for:	Discussion/Decision

# Introduction

A WI [1] was agreed in RAN#86 for further enhancements on MIMO for NR. In particular Rel. 17 WI will specify enhancements on the support of multi-TRP deployments, targeting both FR1 and FR2. The objective on enhancements on multi beam operation for multi-TRP is:

- Evaluate and, if needed, specify beam-management-related enhancements for simultaneous multi-TRP transmission with multi-panel reception

Beam management related enhancements for simultaneous multi-TRP transmission with multi-panel reception are divided into two categories

- Beam measurement/reporting enhancements

- Beam failure recovery for multi-TRP.

In this contribution, we present our views on both issues related to beam management enhancements for multi-TRP.

# Enhancements on Beam Measurement/Reporting

Multi-TRP transmission is a key enabler for robustness and high data rates for NR. Implementing multi-TRP transmission, however, especially in FR2, remains a challenge in the presence of blockage. In Rel. 16, the TCI framework was updated to account for new default QCL assumptions for multi-DCI and single DCI based multi-TRP transmission. Much remains to be specified, however, for multi-TRP operation in FR2, especially for UEs with multiple panel reception.

Group-based beam reporting was introduced in Rel. 15 for L1-RSRP. Rel. 16 extended the framework to include beam reporting based on L1-SINR measurements. For UE beam reporting, UE group-based reporting can be used for multi-TRP enhancements. For non-group-based beam reporting, UE is not assumed to simultaneously receive reported CRIs associated with the L1-measurements. When group-based beam reporting is used, the UE is assumed to simultaneously receive up to a given number of reported beams. The network can configure one or more beam reporting groups, with each group including a given number of beams.

In RAN1#104e [3], the following agreement was made on support of group-based beam reporting.

Agreement

For beam measurement in support of M-TRP simultaneous transmission

- Support a single CSI-report consisting of N beams pairs/groups and M (M>1) beams per pair/group, and different beams within a pair/group can be received simultaneously

  - Support M = 2

  - Support extending the maximum value of N > 1, exact value FFS

  - N=1 and N=2

    - FFS: Other values larger than 2

    - FFS: Whether the UE could report beams are received with different RX beams

- Further study the support of option 1 and option 3

- The above applies at least for L1-RSRP

  - FFS: L1-SINR

An important aspect to enable multi-TRP joint transmission in the beam management framework is the consideration of inter-beam interference and inter-TRP interference. One proposal is to leverage the L1-SINR reporting framework specified in Rel. 16. L1-SINR can be used for multi-TRP to find the beam that achieves the highest signal strength and the lowest interference. Evaluations are needed to check on the gains of using L1-SINR measurement framework in addition to L1-RSRP for multi-TRP joint transmission, and enhancements to the L1-SINR measurements and resources are needed.

Proposal 1: L1-SINR measurement framework is used for multi-TRP joint transmission with multi-panel reception, in addition to L1-RSRP

Specifying enhancements on group-based beam reporting and non-group based beam reporting should take into account applicability to single DCI and multi-DCI multi-TRP operation, with ideal and non-ideal backhaul, and consider multi-beam interference with the ability to use both L1-SINR and L1-RSRP measurement frameworks.

Option 3 advocates enhancements on non-group based beam reporting with multiple CSI-reports reporting on multiple beams that can be received simultaneously. This can be specified in the absence of option 2 for multi-DCI based multi-TRP with multiple CSI reports, corresponding for example to non-ideal backhaul operation.

Proposal 2: Non-group based beam reporting enhancements based on option 3 can be considered

For beam reporting option 2, the following agreements were made in RAN1#104e-bis [4].

Agreement

For beam reporting option 2

- On the maximum number of beam pairs/groups (N) that can be reported in a single CSI-report, discuss and down-select from the following two alternatives in RAN1#105-e:

  - Alt1: Support maximum value N = {1, 2}

  - Alt2: Support maximum value N = {1, 2, 3, 4}

- FFS: Introduce a UE capability Ncap on the maximum value of N in Rel.17

- On the number of beam pairs/groups (N) reported in a single CSI-report, discuss and down select between the following two alternatives in RAN1#105-e

  - Alt1: The value of N is fixed by RRC configuration

  - Alt2: The value of N is upper bounded by a maximum value Nmax configured by RRC, and dynamically selected/indicated by UE

Agreement

On CMR resource configuration for beam reporting option 2, adopt the following alternative:

- Two CMR resource sets or subsets, per periodic/semi-persistent CMR resource setting

  - FFS: extension to aperiodic CMR resource setting

- Each reported beam pair in a single CSI-report consists of M = 2 SSB-RI / CRI values, where each SSB-RI / CRI points to a CMR resource in a different CMR resource set or subset.

- Decide in RAN1#104b-e whether to adopt “set” or “subset” in the above.

Agreement

On CMR resource configuration for beam reporting option 2, decide in RAN1#105-e whether to adopt “set” or “subset”:

- NOTE: the following has been agreed

  - Two CMR resource sets or subsets, per periodic/semi-persistent CMR resource setting

    - FFS : extension to aperiodic CMR resource setting if two CMR resource sets are supported

  - Each reported beam pair in a single CSI -report consists of M = 2 SSBRI/CRI values, where each SSBRI /CRI points to a CMR resource in a different CMR resource set or subset.

- FFS : bitwidth of each SSBRI/CRI determined based on the number of SSB/CSI-RS resources from the associated set/subset, or across two sets/subsets

It was agreed that the reported beam pair in a CSI report consists of 2 SSBRI/CRI values, where each SSBRI/CRI corresponds to a CMR resource. It remains to be agreed, however, whether the CMR resources correspond to 2 different CMR resource sets, or 2 subsets of the same CMR resource set. Both options provide the same functionality, but for the purpose of alignment across different MIMO agenda items for the same CSI framework in Rel. 17, two CMR resource subsets can be adopted.

Proposal 3: On CMR resource configuration for beam reporting option 2, adopt two CMR resource subsets, per periodic/semi-persistent CMR resource setting

Another topic that remains to be agreed for beam reporting option 2 is the maximum number of beam pairs/groups (N) that can be reported in a single CSI-report. This number can correspond to the hypothesis at the UE for receiving the signals from the different TRPs. For a UE with multiple panels, N>=2 would be beneficial to account for the multiple possible receiving hypothesis. We thus support Alt. 2. Note that the UE does not always have to report the maximum number of beam pairs/groups, and the number of groups can be indicated to the network with the beam report.

Proposal 4:  For beam reporting option 2, on the maximum number of beam pairs/groups (N) that can be reported in a single CSI-report, support maximum value N = {1,2,3,4}.

# Beam Failure Recovery for Multi-TRP

A topic of importance for multi-TRP operation is how to handle beam failure from one or multiple TRPs. In fact, blockage - be it self-blocking or environment blocking - is one of the main problems for operation in FR2, and it is detrimental to multi-TRP operation. In Rel. 15, beam failure recovery procedure is triggered only when all the beams in the beam failure detection set have failed. For the case of multi-TRP, that implies that beam failure recovery is triggered only when all the transmissions to the UE from all TRPs have failed. Such a case should be prevented. In Rel. 17, enhancements to enable per-TRP based beam failure recovery were agreed to be evaluated. These enhancements are needed in all aspects of the beam failure recovery procedure.

To enable per-TRP beam failure recovery, we first need to configure TRP-specific BFD RSs. In RAN1#103e [2], the following agreement was made on TRP-specific beam failure detection.

Agreement

- For M-TRP beam failure detection, support independent BFD-RS configuration per-TRP, where each TRP is associated with a BFD-RS set.

  - FFS: The number of BFD RSs per BFD-RS set, the number of BFD-RS sets, and number of BFD RSs across all BFD-RS sets per DL BWP

  - Support at least one of explicit and implicit BFD-RS configuration

    - With explicit BFD-RS configuration, each BFD-RS set is explicitly configured

      - FFS: Further study QCL relationship between BFD-RS and CORESET

    - FFS: How to determine implicit BFD-RS configuration, if supported

- For M-TRP new beam identification

  - Support independent configuration of new beam identification RS (NBI-RS) set per TRP if NBI-RS set per TRP is configured

    - FFS: detail on association of BFD-RS and NBI-RS

    - Support the same new beam identification and configuration criteria as Rel.16, including  L1-RSRP, threshold

Independent BFD-RS configurations were agreed to be supported per TRP, where each TRP is associated with a BFD-RS set.

Agreement

For M-TRP BFR

- Support 2 BFD-RS sets per BWP, and up to N resources per BFD-RS set

  - FFS: value of N (e.g. fixed in specification, or UE capability)

- FFS: number of BFD RSs across all BFD-RS sets per DL BWP (e.g. fixed maximum value or UE capability)

Whether the configuration is explicit or implicit is, however, yet to be determined. In Rel. 15 BFR procedure and Rel. 16 BFR procedure for SCell, both explicit and implicit BFD-RS configurations are supported. Explicit configuration via periodic SSB and CSI-RS or implicit configuration via QCL information with the corresponding CORESETs. For Rel. 17 per TRP beam failure detection, both explicit and implicit configurations should also be supported.

Proposal 5: Support both implicit and explicit BFD-RS configuration

For both configurations, we need to specify how to associate the configured BFD-RSs with the TRPs.

The following agreement was further made in RAN1#104e-bis [4]

Agreement

- Support S-DCI and M-DCI in TRP-specific BFR in Rel.17

  - S-DCI is low priority, M-DCI is high priority

  - Unified design for S-DCI and M-DCI should not be precluded due to the prioritization

Both S-DCI and M-DCI multi-TRP were agreed to be supported in TRP-specific BFR in Rel. 17. It would make sense to agree on a unified design for S-DCI and M-DCI for BFD-RS configuration. For implicit configuration, per the discussion in RAN1#103e, one possible option that can be applicable to both M-DCI and S-DCI was discussed:

- Option 1: all CORESETs in the BWP/CC are divided into multiple groups (e.g. each corresponding to a TRP/panel), and BFD-RS set k is implicitly derived from the corresponding CORESET group k (e.g. k = 0,1,2,…).

CORESETs are divided into multiple groups and the BFD-RSs are divided into sets, where each set is QCLed with a corresponding CORESET group. For example, if the CORESETs are divided into two groups, group 1 and group 2, the BFD-RSs are divided into two sets, with the first set corresponds to CORESET group 1, and the second set corresponds to CORESET group 2. These CORESET groups can, for example, correspond to different TRPs or different panels.  Option 1 is a general framework that is easily applicable to M-DCI based multi-TRP, and at the same time applicable to S-DCI based multi-TRP, as well as single TRP use cases that require partial beam failure recovery to decrease the latency and improve the efficiency of beam failure recovery procedure. It is a more flexible and forward compatible solution. Note that in RAN1#101e, the group has agreed to include partial beam failure recovery in a common framework with multi-TRP beam failure recovery.

Proposal 6: Support implicit BFD-RS configuration where CORESETs in the BWP are divided into multiple groups (e.g., each corresponding to a TRP/panel) and the BFD-RS set k is implicitly derived from the corresponding CORESET group.

# Conclusions

In this contribution we described our views on enhancements for Release 17 feMIMO WI related to multi-beam for multi-TRP transmission. We made the following proposals:

Proposal 1: L1-SINR measurement framework is used for multi-TRP joint transmission with multi-panel reception, in addition to L1-RSRP

Proposal 2: Non-group based beam reporting enhancements based on option 3 can be considered

Proposal 3: On CMR resource configuration for beam reporting option 2, adopt two CMR resource subsets, per periodic/semi-persistent CMR resource setting

Proposal 4:  For beam reporting option 2, on the maximum number of beam pairs/groups (N) that can be reported in a single CSI-report, support maximum value N = {1,2,3,4}.

Proposal 5: Support both implicit and explicit BFD-RS configuration

Proposal 6: Support implicit BFD-RS configuration where CORESETs in the BWP are divided into multiple groups (e.g., each corresponding to a TRP/panel) and the BFD-RS set k is implicitly derived from the corresponding CORESET group.

# References

[1] RP-193133 New WID: Further Enhancements on MIMO for NR, 3GPP TSG RAN Meeting #86, Sitges, Spain, Dec. 2019

[2] RAN1#103e chairman’s notes, e-meeting, October  26th – November 13th , 2020.

[3] RAN1#104e chairman’s notes, e-meeting, January 25th – February 5th, 2021

[4] RAN1#104e-bis chairman’s notes, e-meeting, April 12th – April 25th, 2021