3GPP TSG-RAN WG1 Meeting #105-e 	R1-210xxxx

# e-Meeting, May 10th – May 27th, 2021

Title:	Draft LS on the physical layer aspects for small data transmission

Response to:

Release:	Rel-17

Work Item:	NR_SmallData_INACTIVE-Core

Source:	Moderator (ZTE) [RAN1]

To:	RAN2

Contact Person:

Name:	Li Tian

E-mail Address:	tian.li150@zte.com.cn

Attachments:	None

1. Overall Description:

TA validation

[TBD]

Association between SSBs and CG resources for CG-SDT

RAN1 has further discussed the association between SSBs and CG resources for CG-SDT, and the following agreements have been achieved. RAN1 will continue working on the details of the association.

In RAN1#104b-e

- It is RAN1’s common understanding that the CG configuration mechanism in licensed band can be reused for CG-SDT in principle.

- CG resources per CG configuration are associated with a set of SSB(s) configured by explicit signalling.

  - FFS how to define an SSB-to-PUSCH resource mapping within the CG configuration.

  - FFS specific changes to the CG configuration to support the additional SSB-to-PUSCH mapping, if any.

In RAN1#105-e

  - The SSB-to-PUSCH resource mapping within the CG configuration is implicitly defined.

    - The ordering of the SSB and CG PUSCH resources are to be captured in RAN1 spec.

      - A PUSCH resource refers to a transmission occasion and a DMRS resource used for PUSCH transmission

      - The ordering of the SSB can reuse from the SSB-to-RO mapping

      - The ordering of CG PUSCH resources can reuse from that of MsgA PUSCH as much as possible

    - FFS determination of mapping ratio and association period, e.g., explicitly signaled or implicitly derived

    - FFS any limitation on the combination of the parameters for CG resources

2. Actions:

To RAN2:

ACTION: 	RAN1 respectfully requests that RAN2 takes the above into account.

4. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #106-e 	16 Aug. – 27 Aug. 2021	Electronic Meeting

TSG-RAN WG1 Meeting #106-bis-e 	11 Oct. – 19 Oct.  2021		Electronic Meeting