3GPP TSG RAN WG1 Meeting #105-e	                                                R1-21XXXXX

# e-Meeting, May 10th – 27th, 2021

Agenda item:	7.1

Source:	Moderator (Nokia)

Title:	Summary of [105-e-NR-7.1CRs-03] Issue#7 DRX interaction with CPU occupancy

Document for:	Discussion and Decision

# Introduction

This document is created to facilitate the email discussion [105-e-NR-7.1CRs-03] on DRX interaction with CPU occupancy. This thread is triggered by Issue #7 of [1] and originates from a draft CR to TS38.214 in [2] and accompanying discussion paper in [3].

| Issue# | Tdoc# | Source | Issue description |
| --- | --- | --- | --- |
| 7 | R1-2105278 R1-2105349 | Nokia, NSB | Clarification on how to determine the CPU occupation for periodic or semi-persistent CSI reporting, when a UE is configured with DRX. |

# Background

Sec. 5.2.1.6 of TS 38.214 defines the CPU occupancy rules; however, it does not provide any indication on the applicability of the rules in relation to DRX. This creates ambiguity of interpretation, as elaborated in [3], because a UE configured with DRX may assume that the CPU count is not affected by a DRX configuration, whereas another UE may assume that, for CPU(s) to be occupied “until the last symbol of the PUSCH/PUCCH carrying the report”, this reporting occasion has to occur in DRX active time.

Consequently, when a UE is configured with DRX, a gNB cannot be sure if CPU overbooking occurs, which would result in one or more reports not being updated.

# Company views

To facilitate the discussion, we can consider two separate issues:

Issue 1. When DRX is configured, what is the interpretation of the end point of a CPU occupation, according to the following description?

“For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity not set to 'none', the CPU(s) are occupied for a number of OFDM symbols as follows:

- A periodic or semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report) occupies CPU(s) from the first symbol of the earliest one of each CSI-RS/CSIIM/ SSB resource for channel or interference measurement, respective latest CSI-RS/CSI-IM/SSB occasion no later than the corresponding CSI reference resource, until the last symbol of the PUSCH/PUCCH carrying the report.

- ...

- An initial semi-persistent CSI report on PUSCH after the PDCCH trigger occupies CPU(s) from the first symbol after the PDCCH until the last symbol of the PUSCH carrying the report.”

Alt 1. A CPU is occupied in each reporting period until the configured reporting occasion, regardless of whether the reporting occasion is inside or outside DRX active time.

Alt 2. A CPU is occupied until the reporting occasion that carries the CSI report, i.e., until the earliest reporting occasion in DRX active time.

Issue 2. When DRX is configured and a UE is outside DRX active time, a UE may “skip” a CSI calculation if it can predict the report will not be sent in the current reporting period (i.e., postpone calculation to a later reporting period) or there is no new CSI-RS/SSB measurement to process (i.e., an old CSI may be sent). Should CPU occupancy rules consider these special cases in which a CPU is not occupied?

Please provide company’s view in the table below:

| Company | View |
| --- | --- |

# Conclusion

To be added after the discussion.

# Reference

- RAN1#105-e Session notes for 7.1 (Maintenance of Release 15 NR), Ad-hoc chair (Samsung)

- R1-2105278, “DRX interaction with CPU occupancy,” Nokia, Nokia Shanghai Bell

- R1-2105349, “Discussion on DRX interaction with CPU occupancy,” Nokia, Nokia Shanghai Bell