3GPP TSG-RAN WG1 Meeting #105-e	R1-210xxxx

e-Meeting, May 10th – 27th, 2021

Agenda item:		7.1

Source:		Moderator (Apple Inc.)

Title:	Summary of email discussion [105-e-NR-7.1CRs-09] on the correction for UL cancellation due to DCI format 2_0

Document for:		Discussion and Decision

# 1	Introduction

This contribution provides the summary for the following email discussion in RAN1#105-e:

Issue#23

R1-2105077	Correction on UL cancellation due to DCI format 2_0	Apple

R1-2105078	Correction on UL cancellation due to DCI format 2_0	Apple

[105-e-NR-7.1CRs-09] Issue#23: Correction on UL cancellation due to DCI format 2_0 – Sigen (Apple) by May 25

Section 2 provides the background information for the issues raised in R1-2105077 and R1-2105078. Section 3 captures the detailed email discussions. Section 4 summarizes the outcome of the email discussion.

# 2	Background

In RAN1 #103e, the following agreement was made:

Agreements:

- Clarify that partial cancelation of PUCCH/PUSCH/PRACH triggered by dynamic SFI or dynamically assigned PDSCH/CSI-RS is not supported in Rel-15

  - Prepare CR for above clarification in next meeting

- Introduce a new Rel-16 FG for partial cancelation of PUCCH/PUSCH/PRACH as below

| FG 22-x | Cancellation of PUCCH, PUSCH or PRACH with a DCI scheduling a PDSCH or CSI-RS or a DCI format 2_0 for SFI | A UE supports the partial cancellation of the SRS or PUCCH or PUSCH or PRACH configured transmission: - The UE cancels the configured PUCCH or PUSCH or PRACH in a set of symbols of a slot due to detection of a DCI format 2_0 with a slot format value other than 255 that indicates a slot format with a subset of symbols from the set of symbols as downlink or flexible - The UE cancels the configured PUCCH or PUSCH or PRACH in a set of symbols of a slot due to the detection of a DCI format 1_0, DCI format 1_1, DCI format 1_2 or DCI format 0_1 and DCI format 0_2 indicating to the UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols. |  | Yes | N/A |  | Per FS | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

The corresponding CRs were agreed in RAN1#104-e in R1-2101990 (Rel. 15) and R1-2101991 (Rel.16).

However, the case when DCI format 2_0 is not detected by the UE was not changed accordingly in the agreed CRs. Draft CRs were proposed in:

- R1-2105077 (Rel-15) to clearly capture that only full cancellation is supported for configured PUCCH/PUSCH/PRACH if DCI format 2_0 is not detected by the UE.

- R1-2105078 (Rel-16) to clearly capture that the behavior of partial or full cancellation for configured PUCCH/PUSCH/PRACH depending on UE capability if DCI format 2_0 is not detected by the UE.

# 3	Email Discussions

## 3.1	First Round of Email Discussion

- Companies are invited to provided comments on the draft CRs in R1-2105077 (Rel-15) and R1-2105078 (Rel-16).

For the draft CR in R1-2105077 (Rel-15), please indicate if you support it in principle.

| Yes |  |
| --- | --- |
| No |  |

Companies please provide detailed comments on the draft CR in R1-2105077 (Rel-15), if any.

| Company | Comments |
| --- | --- |

For the draft CR in R1-2105078 (Rel-16), please indicate if you support it in principle.

| Yes |  |
| --- | --- |
| No |  |

Companies please provide detailed comments on the draft CR in R1-2105078 (Rel-16), if any.

| Company | Comments |
| --- | --- |

# 4	Outcome of the Email Discussion

# References