3GPP TSG RAN WG1 Meeting #105-e	R1-21xxxxx

E-meeting, May 10th – May 27th, 2021

Agenda Item:	7.1

Source:	Moderator (vivo)

Title:	Summary of [105-e-NR-7.1CRs-10] Draft 38.212 CR on spreading factor for PUCCH format 4

Document for:	Discussion and Decision

Introduction

This document is created to collect company views on the proposed changes in [1].

In addition, according to the collected comments in preparation phase, some companies think the discussion for clarifying the spreading factor of PUCCH format 2/3 in TS38.211 is also needed in Rel-16 NR. Some changes for TS38.211 are also provided in the document. Companies can also provide views on the changes.

Background

In NR Rel-15, PUCCH format 4 supports multiplexing of different UEs in the same PUCCH resource via spreading, and the spreading factor can be either 2 or 4.

In NR Rel-16, for PUCCH format 2 and PUCCH format 3 with one interlace, multiplexing of different UEs in the same PUCCH resource via spreading can also be supported, and its spreading factor can be 1/2/4.

However, the description/name of spreading factor for PUCCH format 2/3/4 is not consistent among different specifications of TS38.211, TS38.212, TS38.213 and TS38.331.

Problem description

For NR Rel-16, in TS38.331, occ-Length can be configured for PUCCH format 2/3/4, and the followings are captured TS38.331.

PUCCH-ResourceExt-r16 ::= SEQUENCE { interlaceAllocation-r16 SEQUENCE { rb-SetIndex INTEGER (0..4), interlace0 CHOICE { scs15 INTEGER (0..9), scs30 INTEGER (0..4) } } OPTIONAL, --Need R formatExt-v1610 CHOICE { interlace1-v1610 INTEGER (0..9), occ-v1610 SEQUENCE { occ-Length-v1610 ENUMERATED {n2,n4} OPTIONAL, -- Need M occ-Index-v1610 ENUMERATED {n0,n1,n2,n3} OPTIONAL -- Need M } } OPTIONAL, -- Need R ... } PUCCH-format4 ::= SEQUENCE { nrofSymbols INTEGER (4..14), occ-Length ENUMERATED {n2,n4}, occ-Index ENUMERATED {n0,n1,n2,n3}, startingSymbolIndex INTEGER(0..10) }

The followings are captured in TS38.213.

- 9.2.1 PUCCH Resource Sets …. If the format indicates PUCCH-format2 or PUCCH-format3, the PUCCH format configured for a PUCCH resource is PUCCH format 2 or PUCCH format 3, respectively, where the PUCCH resource also includes a number of PRBs provided by nrofPRBs, a number of symbols for a PUCCH transmission provided by nrofSymbols, and a first symbol for the PUCCH transmission provided by startingSymbolIndex. If a UE is provided by useInterlacePUCCH-PUSCH in BWP-UplinkDedicated, and the format indicates PUCCH-format2 or PUCCH-format3 and PUCCH-ResourceExt is provided, the PUCCH resource also includes an index of a second interlace by interlace1, if provided; otherwise, if interlace1 is not provided, the PUCCH resource also includes, if provided, an orthogonal cover code length by occ-Length and an orthogonal cover code index by occ-Index. If the format indicates PUCCH-format3 and PUCCH-ResourceExt is provided, the UE assumes that the MRBPUCCH,3 [4, TS38.211] PRBs with the lowest indexes within the first, and if configured, second interlace are used for PUCCH transmission. If the format indicates PUCCH-format4, the PUCCH format configured for a PUCCH resource is PUCCH format 4, where the PUCCH resource also includes a number of symbols for a PUCCH transmission provided by nrofSymbols, an orthogonal cover code length by occ-Length, an orthogonal cover code index by occ-Index, and a first symbol for the PUCCH transmission provided by startingSymbolIndex. … - 9.2.5.2 UE procedure for multiplexing HARQ-ACK/SR/CSI in a PUCCH … In the following - is a code rate given by maxCodeRate as in Table 9.2.5.2-1. - is a number of PRBs for PUCCH format 2, or PUCCH format 3, or PUCCH format 4, respectively, where is provided by nrofPRBs in PUCCH-format2 for PUCCH format 2 or by nrofPRBs in PUCCH-format3 for PUCCH format 3, and for PUCCH format 4 - for PUCCH format 2 or, if the PUCCH resource with PUCCH format 2 includes an orthogonal cover code with length provided by occ-Length, , for PUCCH format 3 or, if the PUCCH resource with PUCCH format 3 includes an orthogonal cover code with length provided by occ-Length, , and for PUCCH format 4, where is a number of subcarriers per resource block [4, TS 38.211]

The followings are captured in TS38.212.

- 6.3.1.4 Rate matching For PUCCH formats 2/3/4, the total rate matching output sequence length is given by Table 6.3.1.4-1, where , , and are the number of symbols carrying UCI for PUCCH formats 2/3/4 respectively; and are the number of PRBs that are determined by the UE for PUCCH formats 2/3 transmission respectively according to Clause 9.2 of [5, TS38.213]; and NSFPUCCH,2, NSFPUCCH,3, and are the spreading factors for PUCCH format 2, PUCCH format 3, and PUCCH format 4, respectively. - 6.3.1.6 Multiplexing of coded UCI bits to PUCCH … For PUCCH format 3, set NUCIsymbol=12⋅NPRBPUCCH,3/NSFPUCCH,3 , where is the number of PRBs that is determined by the UE for PUCCH format 3 transmission according to Clause 9.2 of [5, TS 38.213], and NSFPUCCH,3 is the spreading factor for PUCCH format 3 [4, TS 38.211]. For PUCCH format 4, set , where is the spreading factor for PUCCH format 4. Find the smallest such that .

The followings are captured in TS38.211.

- 6.3.2.5 PUCCH format 2 ##### 6.3.2.5.2A Spreading Spreading shall be applied according to zmNSFPUCCH,2+i=wnidm i=0,1,…,NSFPUCCH,2-1 m=0,1,…,Msymb-1 resulting in a block of complex-valued symbols z0,…,z(NSFPUCCH,2Msymb-1). If the higher layer parameter interlace1 is not configured, and the higher-layer parameter OCC-Length is configured, - NSFPUCCH,2∈2,4 is given by the higher-layer parameter OCC-Length; - wni is given by Tables 6.3.2.5A-1 and 6.3.2.5A-2 where n=n0+nIRB mod NSFPUCCH,2, the quantity n0 is the index of the orthogonal sequence to use given by the higher-layer parameter OCC-Index, and nIRB is the interlaced resource block number as defined in clause 4.4.4.6 within the interlace given by the higher-layer parameter Interlace0. otherwise NSFPUCCH,2=1 and wni=1. - 6.3.2.6 PUCCH formats 3 and 4 ##### 6.3.2.6.3 Block-wise spreading For both PUCCH format 3 and 4, MscPUCCH,s=MRBPUCCH,sNscRB with MRBPUCCH,s representing the bandwidth of the PUCCH in terms of resource blocks according to clauses 9.2.3, 9.2.5.1 and 9.2.5.2 of [5, TS 38.213] and shall for non-interlaced mapping fulfil where is a set of non-negative integers and . For interlaced mapping, MRBPUCCH,3=10 if a single interlace is configured and MRBPUCCH,3=20 if two interlaces are configured. For PUCCH format 3, if interlaced mapping is not configured, no block-wise spreading is applied and where MRBPUCCH,3≥1 is given by clauses 9.2.3, 9.2.5.1 and 9.2.5.2 of [5, TS 38.213] and NSFPUCCH,3=1. For PUCCH format 3 with interlaced mapping and PUCCH format 4, block-wise spreading shall be applied according to ylMscPUCCH,s+k=wnkNSFPUCCH,sMscPUCCH,sdlMscPUCCH,sNSFPUCCH,s+k mod MscPUCCH,sNSFPUCCH,s k=0,1,…, MscPUCCH,s-1 l=0,1,…,NSFPUCCH,sMsymbMscPUCCH,s-1 where - for PUCCH format 3 with interlaced mapping, NSFPUCCH,3∈1,2,4 if a single interlace is configured and NSFPUCCH,3=1, wn=1 if two interlaces are configured; - for PUCCH format 4,MRBPUCCH,4=1, NSFPUCCH,4∈2,4; and is given by Tables 6.3.2.6.3-1 and 6.3.2.6.3-2 for NSFPUCCH,s>1 where n is the index of the orthogonal sequence to use according to clause 9.2.1 of [5, TS 38.213].

Based on the above quoted current specifications, it can be found that there is some insistency regarding the description of spreading factor for PUCCH format 2/3/4. For TS 38.212, current specification does not have detail on how the spreading factor for PUCCH format 2/3/4 is determined.

In [1], some changes are proposed for TS38.212 to clarify that the value of PUCCH spreading factor is determined by a higher parameter (occ-Length). And the exact changes are quoted as following for convenience.

- Proposed CR in R1-2105456

- For Rel-15:

#### 6.3.1.4	Rate matching

For PUCCH formats 2/3/4, the total rate matching output sequence length  is given by Table 6.3.1.4-1, where  , , and  are the number of symbols carrying UCI for PUCCH formats 2/3/4 respectively;  and  are the number of PRBs that are determined by the UE for PUCCH formats 2/3 transmission respectively according to Subclause 9.2 of [5, TS38.213]; and  is the spreading factor provided by OCC-Length for PUCCH format 4.

Table 6.3.1.4-1: Total rate matching output sequence length

| PUCCH format | Modulation order |  |
| --- | --- | --- |
|  | QPSK | π/2-BPSK |
| PUCCH format 2 |  | N/A |
| PUCCH format 3 |  |  |
| PUCCH format 4 |  |  |

#### 6.3.1.6	Multiplexing of coded UCI bits to PUCCH

If CSI of two parts are transmitted on a PUCCH, the coded bits corresponding to UCI bit sequence  is denoted by and the coded bits corresponding to UCI bit sequence  is denoted by . The coded bit sequence , where , is generated according to the following.

Table 6.3.1.6-1: PUCCH DMRS and UCI symbols

| PUCCH duration (symbols) | PUCCH DMRS symbol indices | Number of UCI symbol indices sets | 1st UCI symbol indices set | 2nd UCI symbol indices set | 3rd UCI symbol indices set |
| --- | --- | --- | --- | --- | --- |
| 4 | {1} | 2 | {0,2} | {3} | - |
| 4 | {0,2} | 1 | {1,3} | - | - |
| 5 | {0, 3} | 1 | {1, 2, 4} | - | - |
| 6 | {1, 4} | 1 | {0, 2, 3, 5} | - | - |
| 7 | {1, 4} | 2 | {0, 2, 3, 5} | {6} | - |
| 8 | {1, 5} | 2 | {0, 2, 4, 6} | {3, 7} | - |
| 9 | {1, 6} | 2 | {0, 2, 5, 7} | {3, 4, 8} | - |
| 10 | {2, 7} | 2 | {1, 3, 6, 8} | {0, 4, 5, 9} | - |
| 10 | {1, 3, 6, 8} | 1 | {0,2,4,5,7,9} | - | - |
| 11 | {2, 7} | 3 | {1,3,6,8} | {0,4,5,9} | {10} |
| 11 | {1,3,6,9} | 1 | {0,2,4,5,7,8,10} | - | - |
| 12 | {2, 8} | 3 | {1,3,7,9} | {0,4,6,10} | {5, 11} |
| 12 | {1,4,7,10} | 1 | {0,2,3,5,6,8,9,11} | - | - |
| 13 | {2, 9} | 3 | {1,3,8,10} | {0,4,7,11} | {5,6,12} |
| 13 | {1,4,7,11} | 2 | {0,2,3,5,6,8,10,12} | {9} | - |
| 14 | {3, 10} | 3 | {2,4,9,11} | {1,5,8,12} | {0,6,7,13} |
| 14 | {1,5,8,12} | 2 | {0,2,4,6,7,9,11,13} | {3, 10} | - |

Denote  as UCI OFDM symbol index. Denote  as the number of elements in UCI symbol indices set  for , where  and  are given by Table 6.3.1.6-1 according to the PUCCH duration and the PUCCH DMRS configuration. Denote  as the number of OFDM symbols carrying UCI in the PUCCH. Denote  as the modulation order of the PUCCH.

For PUCCH format 3, set  , where  is the number of PRBs that is determined by the UE for PUCCH format 3 transmission according to Subclause 9.2 of [5, TS 38.213].

For PUCCH format 4, set , where  is the spreading factor provided by OCC-Lengthfor PUCCH format 4.

Find the smallest such that .

<unchanged part omitted>

- For Rel-16:

#### 6.3.1.4	Rate matching

For PUCCH formats 2/3/4, the total rate matching output sequence length  is given by Table 6.3.1.4-1, where  , , and  are the number of symbols carrying UCI for PUCCH formats 2/3/4 respectively;  and  are the number of PRBs that are determined by the UE for PUCCH formats 2/3 transmission respectively according to Clause 9.2 of [5, TS38.213]; and NSFPUCCH,2, NSFPUCCH,3, and  are the spreading factors for PUCCH format 2, PUCCH format 3, provided by occ-Length-v1610 or equalled to 1 if occ-Length-v1610 is not provided, and for PUCCH format 4, provided by occ-Length, respectively.

Table 6.3.1.4-1: Total rate matching output sequence length

| PUCCH format | Modulation order |  |
| --- | --- | --- |
|  | QPSK | π/2-BPSK |
| PUCCH format 2 | 16∙Nsymb,UCIPUCCH,2∙NPRBPUCCH,2/NSFPUCCH,2 | N/A |
| PUCCH format 3 | 24∙Nsymb,UCIPUCCH,3∙NPRBPUCCH,3/NSFPUCCH,3 | 12∙Nsymb,UCIPUCCH,3∙NPRBPUCCH,3/NSFPUCCH,3 |
| PUCCH format 4 |  |  |

#### 6.3.1.6	Multiplexing of coded UCI bits to PUCCH

If CSI of two parts are transmitted on a PUCCH, the coded bits corresponding to UCI bit sequence  is denoted by and the coded bits corresponding to UCI bit sequence  is denoted by . The coded bit sequence , where , is generated according to the following.

Table 6.3.1.6-1: PUCCH DMRS and UCI symbols

| PUCCH duration (symbols) | PUCCH DMRS symbol indices | Number of UCI symbol indices sets | 1st UCI symbol indices set | 2nd UCI symbol indices set | 3rd UCI symbol indices set |
| --- | --- | --- | --- | --- | --- |
| 4 | {1} | 2 | {0,2} | {3} | - |
| 4 | {0,2} | 1 | {1,3} | - | - |
| 5 | {0, 3} | 1 | {1, 2, 4} | - | - |
| 6 | {1, 4} | 1 | {0, 2, 3, 5} | - | - |
| 7 | {1, 4} | 2 | {0, 2, 3, 5} | {6} | - |
| 8 | {1, 5} | 2 | {0, 2, 4, 6} | {3, 7} | - |
| 9 | {1, 6} | 2 | {0, 2, 5, 7} | {3, 4, 8} | - |
| 10 | {2, 7} | 2 | {1, 3, 6, 8} | {0, 4, 5, 9} | - |
| 10 | {1, 3, 6, 8} | 1 | {0,2,4,5,7,9} | - | - |
| 11 | {2, 7} | 3 | {1,3,6,8} | {0,4,5,9} | {10} |
| 11 | {1,3,6,9} | 1 | {0,2,4,5,7,8,10} | - | - |
| 12 | {2, 8} | 3 | {1,3,7,9} | {0,4,6,10} | {5, 11} |
| 12 | {1,4,7,10} | 1 | {0,2,3,5,6,8,9,11} | - | - |
| 13 | {2, 9} | 3 | {1,3,8,10} | {0,4,7,11} | {5,6,12} |
| 13 | {1,4,7,11} | 2 | {0,2,3,5,6,8,10,12} | {9} | - |
| 14 | {3, 10} | 3 | {2,4,9,11} | {1,5,8,12} | {0,6,7,13} |
| 14 | {1,5,8,12} | 2 | {0,2,4,6,7,9,11,13} | {3, 10} | - |

Denote  as UCI OFDM symbol index. Denote  as the number of elements in UCI symbol indices set  for , where  and  are given by Table 6.3.1.6-1 according to the PUCCH duration and the PUCCH DMRS configuration. Denote  as the number of OFDM symbols carrying UCI in the PUCCH. Denote  as the modulation order of the PUCCH.

For PUCCH format 3, set NUCIsymbol=12⋅NPRBPUCCH,3/NSFPUCCH,3 , where  is the number of PRBs that is determined by the UE for PUCCH format 3 transmission according to Clause 9.2 of [5, TS 38.213], and NSFPUCCH,3 is the spreading factor for PUCCH format 3 [4, TS 38.211] provided by occ-Length-v1610 or equalled to 1 if occ-Length-v1610 is not provided.

For PUCCH format 4, set , where  is the spreading factor for PUCCH format 4 provided by occ-Length.

<unchanged part omitted>

In addition, some changes are also provided for TS38.211 based on the comments in preparation phase.

- Proposed changes for TS 28.211

- For Rel-15:

#### 6.3.2.6	PUCCH formats 3 and 4

##### 6.3.2.6.3	Block-wise spreading

For both PUCCH format 3 and 4, MscPUCCH,s=MRBPUCCH,sNscRB  with MRBPUCCH,s representing the bandwidth of the PUCCH in terms of resource blocks according to subclauses 9.2.3, 9.2.5.1 and 9.2.5.2 of [5, TS 38.213] and shall fulfil

where  is a set of non-negative integers and .

For PUCCH format 3, no block-wise spreading is applied and

where MRBPUCCH,3≥1 is given by subclauses 9.2.3, 9.2.5.1 and 9.2.5.2 of [5, TS 38.213] and NSFPUCCH,3=1.

For PUCCH format 4, block-wise spreading shall be applied according to

where MRBPUCCH,4=1, NSFPUCCH,4∈2,4 is given by the higher-layer parameter occ-Length and  are given by Tables 6.3.2.6.3-1 and 6.3.2.6.3-2 where n is the index of the orthogonal sequence to use according to subclause 9.2.1 of [5, TS 38.213].

Table 6.3.2.6.3-1: Orthogonal sequences  for PUCCH format 4 when NSFPUCCH,4=2.

| 0 |  |
| --- | --- |
| 1 |  |

Table 6.3.2.6.3-2: Orthogonal sequences  for PUCCH format 4 when NSFPUCCH,4=4.

| 0 |  |
| --- | --- |
| 1 |  |
| 2 |  |
| 3 |  |

- For Rel-16:

- 6.3.2.5	PUCCH format 2

##### 6.3.2.5.2A	Spreading

Spreading shall be applied according to

zmNSFPUCCH,2+i=wnidm
i=0,1,…,NSFPUCCH,2-1
m=0,1,…,Msymb-1

resulting in a block of complex-valued symbols z0,…,z(NSFPUCCH,2Msymb-1).

If the higher layer parameter interlace1 is not configured, and the higher-layer parameter occ-Length-v1610 is configured,

-	NSFPUCCH,2∈2,4 is given by the higher-layer parameter occ-Length-v1610;

-	wni is given by Tables 6.3.2.5A-1 and 6.3.2.5A-2 where n=n0+nIRB mod NSFPUCCH,2, the quantity n0 is the index of the orthogonal sequence to use given by the higher-layer parameter OCC-Index, and nIRB is the interlaced resource block number as defined in clause 4.4.4.6 within the interlace given by the higher-layer parameter Interlace0.

otherwise NSFPUCCH,2=1 and wni=1.

Table 6.3.2.5A-1: Orthogonal sequences wni for PUCCH format 2 when NSFPUCCH,2=2.

| n | wni |
| --- | --- |
| 0 | +1+1 |
| 1 | +1-1 |

Table 6.3.2.5A-2: Orthogonal sequences wni for PUCCH format 2 when NSFPUCCH,2=4.

| n | wni |
| --- | --- |
| 0 | +1+1+1+1 |
| 1 | +1-1+1-1 |
| 2 | +1+1-1-1 |
| 3 | +1-1-1+1 |

- 6.3.2.6	PUCCH formats 3 and 4

- 6.3.2.6.3	Block-wise spreading

For both PUCCH format 3 and 4, MscPUCCH,s=MRBPUCCH,sNscRB  with MRBPUCCH,s representing the bandwidth of the PUCCH in terms of resource blocks according to clauses 9.2.3, 9.2.5.1 and 9.2.5.2 of [5, TS 38.213] and shall for non-interlaced mapping fulfil

where  is a set of non-negative integers and . For interlaced mapping, MRBPUCCH,3=10 if a single interlace is configured and MRBPUCCH,3=20 if two interlaces are configured.

For PUCCH format 3, if interlaced mapping is not configured, no block-wise spreading is applied and

where MRBPUCCH,3≥1 is given by clauses 9.2.3, 9.2.5.1 and 9.2.5.2 of [5, TS 38.213] and NSFPUCCH,3=1.

For PUCCH format 3 with interlaced mapping and PUCCH format 4, block-wise spreading shall be applied according to

ylMscPUCCH,s+k=wnkNSFPUCCH,sMscPUCCH,sdlMscPUCCH,sNSFPUCCH,s+k mod MscPUCCH,sNSFPUCCH,s
k=0,1,…, MscPUCCH,s-1
l=0,1,…,NSFPUCCH,sMsymbMscPUCCH,s-1

where

-	for PUCCH format 3 with interlaced mapping, NSFPUCCH,3∈1,2,4 if a single interlace is configured and NSFPUCCH,3=1, wn=1 if two interlaces are configured; NSFPUCCH,3∈2,4 is given by the higher-layer parameter occ-Length-v1610, otherwise, NSFPUCCH,3=1.

-	for PUCCH format 4,MRBPUCCH,4=1, NSFPUCCH,4∈2,4 is given by the higher-layer parameter occ-Length;

and  is given by Tables 6.3.2.6.3-1 and 6.3.2.6.3-2 for NSFPUCCH,s>1 where n is the index of the orthogonal sequence to use according to clause 9.2.1 of [5, TS 38.213].

Table 6.3.2.6.3-1: Orthogonal sequences  for PUCCH format 3 with interlaced mapping and PUCCH format 4 when NSFPUCCH,s=2.

| 0 | +1+1 |
| --- | --- |
| 1 | +1-1 |

Table 6.3.2.6.3-2: Orthogonal sequences  for PUCCH format 3 with interlaced mapping and PUCCH format 4 when NSFPUCCH,s=4.

| 0 | +1+1+1+1 |
| --- | --- |
| 1 | +1-j-1+j |
| 2 | +1-1+1-1 |
| 3 | +1+j-1-j |

Company views

Q1: Do you agree with proposed changes for TS28.212 for Rel-15? If not, why?

| Company | Agree or not | Comment |
| --- | --- | --- |
| OPPO | More on NO side. | Firstly, we think the issue (if any) would be editorial, given the current spec text does not give a second technical interpretation. Secondly, if certain editorial improvement (e.g. to further clarify where the parameter should come from) is indeed desired for the long-time stable Rel-15 spec, besides the proposed change, the current terminology in 38.212, “spreading factor”, is better to be changed to “orthogonal cover code length” in order to align with 38.213 and 38.331, or to be simply removed to avoid giving two different PHY names for the same higher layer parameter. |
| QC | leaning towards NO | Similar view as OPPO, this is editorial change. We admit the CR beautifies the spec and improves the readability of the spec. But current spec is good enough. I don’t think it can be interpreted in a different/wrong way. |
| Apple |  | Same view as OPPO and QC |
| Intel |  | We share similar view as other companies that this is mainly editorial change. Even without such change, there would not have different interpretations. Some of the changes in the proposed CR are not correct, i.e., OCC-Length should be occ-Length, occ-Length-v1610 should be occ-Length. We can be okay to leave to editor to make the change. |
| CATT |  | We share the similar view that the proposed changes are not essential. The spec cannot be interpreted in a different way. |
| Samsung | Agree in principle | We agree that the clarification for the spreading factor of each PUCCH format is necessary because the relationship between the spreading factor (or symbol NSFPUCCH,4) and occ-Length is not explained anywhere in Rel-15 (TS38.211, 38.212, 38.213, and 38.331). However, we think it is better to clarify the higher layer parameter of the spreading factor (NSFPUCCH,4) in detail in 38.211, which directly describes the spreading step (6.3.2.6.3 in 38.211), and it seems that at least a reference to the described part can be referred in 38.212 if necessary. |
| ZTE |  | Similar view as other companies that the changes are editorial and no ambiguity would be caused without any change. But, we would be also ok to leave this to editor. |

Q2: Do you agree with proposed changes for TS28.212 for Rel-16? If not, why?

| Company | Agree or not | Comment |
| --- | --- | --- |
| OPPO | More on NO side. | Our comments under Q1 apply to Q2 as well. In addition, occ-Length and occ-Length-v1610 are defined in 38.331 per single PUCCH resource, and each PUCCH resource would not be configured with both occ-Length and occ-Length-v1610. Therefore it does not seem necessary to distinguish in RAN1 spec between occ-Length and occ-Length-v1610 --- “if occ-Length [of occ-Length-v1610] is provided” means the same as “if occ-Length is provided for the concerned PUCCH resource”. The existing RAN1 spec does not seem to propagate the parameter suffix of “-v16xx” quite much. This discussed CR should follow the same way. On the other hand, we agree it is good to add some text for the case where occ-Length is not configured for PUCCH formats 2 and 3. |
| QC | Leaning toward No | Similar view as OPPO, this is editorial change. We admit the CR beautifies the spec and improves the readability of the spec. But current spec is good enough. I don’t think it can be interpreted in a different/wrong way. |
| Apple |  | Similar view as OPPO and QC |
| Intel |  | Same comments as above. |
| CATT |  | Same comments as above. |
| Samsung | Agree in principle | Our comments under Q1 apply to Q2 as well, even in the case of Rel-16. Although the relationship between the symbol NSFPUCCH,2/3 and occ-Length is explained in Rel-16 (TS38.213), it seems that at least a reference to the described part can be referred in 38.212 if necessary. |
| ZTE |  | Similar view as commented in Q1. |

Q3: Do you agree with the proposed changes for TS28.211 for Rel-15? If not, why?

| Company | Agree or not | Comment |
| --- | --- | --- |
| OPPO | No. | The change does not seem to be essential because the current spec text does not generate any technical issues, and the same math notation is explained in 38.212. |
| QC | NO | Similar view as OPPO, this is editorial change. We admit the CR beautifies the spec and improves the readability of the spec. But current spec is good enough. I don’t think it can be interpreted in a different/wrong way. |
| Apple | No | Similar view as OPPO and QC |
| Intel |  | Same comment as above |
| CATT |  | Same comments as above. |
| Samsung | Agree | As mentioned in Q1, we agree that the clarification for the spreading factor of each PUCCH format is necessary because the relationship between the spreading factor (or symbol NSFPUCCH,4) and occ-Length is not explained anywhere in Rel-15 (TS38.211, 38.212, 38.213, and 38.331). |
| ZTE |  | Similar view as commented in Q1. |

Q4: Do you agree with the proposed changes for TS28.211 for Rel-16? If not, why?

| Company | Agree or not | Comment |
| --- | --- | --- |
| OPPO | No. | As explained in our comments for Q1, we do not see a reason to change current wording of occ-Length to occ-Length-v1610. The other changes are not essential because the current spec text does not generate any technical issues. |
| QC | NO | Same comment as for previous question |
| Apple | No |  |
| Intel |  | Same comment as above |
| CATT |  | Same comments as above. |
| Samsung | Agree | Our comments under Q3 apply to Q4 as well. Although, in Rel-16, the higher layer parameter of NSFPUCCH,2 is already explained in 38.211 and 38.213 and NSFPUCCH,3 case is explained in 38.213, NSFPUCCH,4 case is not explained anywhere in Rel-16 (TS38.211, 38.212, 38.213, and 38.331). So we think that the higher layer parameter of NSFPUCCH,4 should be described like other PUCCH formats. Moreover, as well as adding the description, it is better to correct some typos related to the spreading in 38.211(e.g. OCC-Length to occ-Length or occ-Lenth-v1610 and Interlace0 to interlace0), although we agree with other companies opinion that there is no need to distinguish between occ-Length and occ-Length-v1610. |
| ZTE |  | Similar view as commented in Q1. |

Conclusions

To be updated based on the discussion

- References

R1-2105456, “Draft 38.212 CR on spreading factor for PUCCH format 4”, vivo