3GPP TSG RAN WG1 Meeting #105-e	R1-210xxxx

e-Meeting, May 10th – 27th, 2021

Source:	Moderator (ZTE)

Title:	Summary of [105-e-NR-7.1CRs-12] Issue#26 SRS carrier switching

Agenda Item:	7.1

Document for:	Discussion and Decision

- Introduction

The document provides a summary for the email discussion thread [105-e-NR-7.1CRs-12] Issue#26 SRS carrier switching for Rel-16 only.

[105-e-NR-7.1CRs-12] Issue#26: SRS carrier switching – Chuangxin (ZTE) by May 25

For Rel-16 only

In order to make use of the email thread for discussion efficiently, two check points are planned as follows.

- 1st check point: 5/20 (UTC 23:59 pm).

  - Try to get some consensus for proposals

- 2nd check point: 5/24 (UTC 23:59 pm).

  - Try to get some consensus for specification change

- Discussion

Based on the contribution [1-5], the issues can be summarized as the following questions.

- Question 1: Whether priority rules are used for source CC

Contribution [1][4][5] raised the issue that whether priority rules are used between SRS in target CC and the source CC configured by higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier. There are two interpretations on whether SRS carrier switching priority rules should be used for source CC:

- Alt 1: Yes, the prioritization rules apply to the source CC

  - For companies supporting Alt 1, please also share your comments if the following highlighted sentence should be removed from the current 38.214. That’s because, based on the prioritization rules, the UL signals in source CC may not be suspended when the UL signals have higher priority than SRS in target CC.

A UE can be configured with SRS resource(s) on a carrier c1 with slot formats comprised of DL and UL symbols and not configured for PUSCH/PUCCH transmission. For carrier c1, the UE is configured with higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier the switching from carrier c2 which is configured for PUSCH/PUCCH transmission. During SRS transmission on carrier c1 (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR), the UE temporarily suspends the uplink transmission on carrier c2.

- Alt 2: No, the source CC is always suspended as described in the above highlighted part.

Please companies share your comments and preference on the two interpretations

| Company | Comment |
| --- | --- |

- Question 2: Whether/which UL CCs other than source CC should be used in priority rules

Contribution [1][3][4][5] suggested the priority rule should be used between SRS in target CC and the UL CC set with the same band as the source CC configured by higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier. The suggestion is similar as LTE, i.e. define the UL CC set S(d)= {s0(d)… sN-1(d)} as the set of carriers of serving cells that meet all the following conditions, where d is the target CC, and s0(d) is the source CC.

-	{s0(d)… sN-1(d)} are in the same band as s0(d).

-	{s0(d)… sN-1(d)} are in the same TAG as s0(d).

Contribution [2] seemed to prefer that the priority rules should be used between SRS in target CC and all potential UL CCs including CCs in different bands from the target CC as long as the UL CCs make UE transmission beyond its UL CA capability.

Contribution [1] pointed out that 38.133 (the relevant description is listed in section 5.1 in Appendix for convenience) defines an interruption period during which the UE MAY skip transmission for CCs other than source and target. So RAN4 specification handles the interruption operation between the SRS in target CC and UL signals in other CCs which are in different bands from the source CC.

In summary, there are following interpretations to define whether/which UL CCs other than source CC should be used for SRS carrier switching priority rules

- Option 1: The UL CCs in the same band as the source CC

  - Please proponents also share whether LTE-similar description as shown in the above yellow part is OK or not. If not, please share your solution.

- Option 2: The UL CCs can be any carriers which result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability

- Option 3: Not support any other UL CCs  for priority rules

  - Please proponents if any share the reason why other UL CCs for priority rules are not needed

Please companies share your comments and preference on the two interpretations

| Company | Comment |
| --- | --- |

- Question 3: Whether/how extend timelines for multiple UL CCs

In RAN1#104bis-e meeting, a CR R1-2104043 was agreed to determine timelines for SRS in target carrier c1 and the source carrier c2 as shown in section 5.2 in Appendix. If UL CCs other than source CC should also be used in priority rule, the agreed SRS carrier switching timelines should be naturally extended from the configured source CC to multiple UL CCs (which UL CCs will depend on the outcome of Question 2).

Contribution [2] has the following proposals:

Proposal 1 from [2]: For the case that aperiodic SRS transmission on the target cell has higher priority than overlapping UL transmissions on other carriers, and the simultaneous transmission is beyond UE’s capability:

- UE does not expect that the gap between the last symbol of DCI indicating A-SRS on target CC and the first symbol of the earliest low priority UL transmission, among a group of overlapping UL transmissions with a priority lower than A-SRS, to be less than Tprocmax, with Tprocmax=max{Tproc1, …,Tproci, …}

where Tproci is based on N2, which itself is determined based on the UE processing capability on the i-th low priority carrier, and the minimum of (µDL, µith-UL), where the µDL corresponds to the SCS of the PDCCH scheduling A-SRS, and µith-UL corresponds to the SCS of the uplink channel on the i-th low priority carrier.

Proposal 2 from [2]: For the case that UE is scheduled by a DCI, or a set of DCIs, to transmit a high priority UL transmission on a serving cell overlapping with a low priority SRS transmission on a carrier without configured PUSCH/PUCCH, and simultaneous transmission is beyond UE’s capability:

- UE does not expect the gap between the first symbol of the earliest low priority SRS transmission on the target cell and a last symbol of the last DCI among all DCIs indicating high priority transmissions on another carriers, to be less than Tprocmax, with Tprocmax=max{Tproc1, …,Tproci, …}

where Tproci is based on SRS-SwitchingTime + N2, and N2 is determined based on the UE processing capability on the target carrier, and the minimum of (µith-DL, µUL), where the µith-DL corresponds to the SCS of the PDCCH scheduling a high priority UL transmission on the i-th high priority carrier, and µUL corresponds to the SCS of the SRS on the target cell. SRS-SwitchingTime represents the UL or DL RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR,

Contribution [4] [5] propose keeping the wording of the agreed CR R1-2104043 to determine SRS carrier switching timelines, and redefine c2 to make that c2 can be an UL CC other than the source CC.

In short, there are two ways to extend the timelines for SRS carrier switching.

- Solution 1: Based on the proposal 1 and 2 from contribution [2]

- Solution 2: Keeping the wording of the agreed CR R1-2104043 to determine SRS carrier switching timeline , and redefine c2 to make that c2 can be an UL CC other than the source CC

Please companies share your comments and preference on the two interpretations

| Company | Comment |
| --- | --- |

- Outcome of email discussion

- List of contributions

- R1-2104647 Discussion on SRS carrier switching Qualcomm Incorporated

- R1-2105074 Timeline Considerations for SRS Carrier Switching and Dropping Procedure Apple Inc.

- R1-2105936 Correction on prioritization rules of SRS carrier switching Huawei, HiSilicon

- R1-2104580 Discussion on SRS carrier switching ZTE

- R1-2104581 Draft CR on SRS carrier switching ZTE

- Appendix

  - SRS carrier switching in 38.133

- 8.2.1.2.12	 Interruptions at NR SRS carrier based switching

SRS transmission can be configured on a carrier not configured for PUCCH/PUSCH transmission. When a UE needs to transmit periodic, semi-persistent or aperiodic SRS on a carrier of a serving cell not configured for PUCCH/PUSCH transmission, the UE can perform carrier based switching to one or more carriers not configured for PUCCH/PUSCH transmission from a carrier with PUCCH/PUSCH transmission or from a carrier not configured for PUCCH/PUSCH transmission prior to transmitting SRS, provided that:

-	switching is from a configured carrier to an active UL BWP of another activated carrier;

-	the carrier of SCells not configured for PUCCH/PUSCH transmission to which SRS carrier based switching is performed is indicated by DCI SRS request field for aperiodic SRS transmission, or indicated by MAC-CE for semi-persistent SRS transmission, or configured via RRC for periodic SRS transmission;

-	the serving cell, from which SRS carrier based switching is performed and whose UL transmission may therefore be interrupted, is indicated by srs-SwitchFromServCellIndex and srs-SwitchFromCarrier in TS38.331 [2];

-	 the SRS switching is not colliding with any other transmission with higher priority defined in TS 38.214 [26].

-	 the SRS switching is not colliding with any measurements in SCG.

-	for UE, which does not support simultaneous reception and transmission for inter-band TDD CA specified in TS 38.331 [2], and is compliant to the requirements for inter-band CA with uplink in one NR band and without simultaneous Rx/Tx specified in TS 38.101-3 [20], the SRS transmission are not simultaneously scheduled with DL SSB/CSI-RS for L3 or L1 measurements transmission on other carriers.

The UE shall not perform SRS carrier based switching if the above conditions cannot be met.

When SRS carrier based switching is performed between carriers, the UE is allowed interruptions on any active serving cell in SCG if UE is not capable of Per-FR gap, or on active serving cell(s) in SCG in FR1 if UE is capable of Per-FR gap, during the switching to the carrier of a serving cell in FR1 not configured for PUCCH/PUSCH transmission,

-	with up to X1 slot as specified in Table 8.2.1.2.12-1.

When SRS carrier based switching is performed between carriers, the UE is allowed interruptions on any active serving cell in SCG if UE is not capable of Per-FR gap, or on active serving cell(s) in SCG in FR2 if UE is capable of Per-FR gap, during the switching to the carrier of a serving cell in FR2 not configured for PUCCH/PUSCH transmission,

-	with up to X2 slot as specified in Table 8.2.1.2.12-2.

When SRS carrier based switching is performed between carriers, the UE is allowed interruptions on any active serving cell in SCG if UE is not capable of Per-FR gap, or on active serving cell(s) in SCG in FR1 if UE is capable of Per-FR gap, during the switching from the carrier of a serving cell in FR1 not configured for PUCCH/PUSCH transmission,

-	with up to X1 slot as specified in Table 8.2.1.2.12-1.

When SRS carrier based switching is performed between carriers, the UE is allowed interruptions on any active serving cell in SCG if UE is not capable of Per-FR gap, or on active serving cell(s) in SCG in FR2 if UE is capable of Per-FR gap, during the switching from the carrier of a serving cell in FR2 not configured for PUCCH/PUSCH transmission,

-	with up to X2 slot as specified in Table 8.2.1.2.12-2.

Table 8.2.1.2.12-1: Interruption length X1 (slot)

|  | NR Slot length | SRS carrier | Interruption length X1 (slots) |  |
| --- | --- | --- | --- | --- |
|  | (ms) of victim cell | switching time (us)Note 1 | Sub carrier spacing for agressor cell (kHz) |  |
|  |  |  | 15 | 30 |
| 0 | 1 | ≤ 200 | 2 | 2 |
|  |  | 300, 500 | 2 | 2 |
|  |  | 900 | 3 | 3 |
| 1 | 0.5 | ≤ 200 | 3 | 2 |
|  |  | 300, 500 | 3 | 3 |
|  |  | 900 | 4 | 4 |
| 2 | 0.25 | ≤ 200 | 4 | 3 |
|  |  | 300, 500 | 5 | 4 |
|  |  | 900 | 7 | 6 |
| 3 | 0.125 | ≤ 200 | 7 | 5 |
|  |  | 300, 500 | 9 | 7 |
|  |  | 900 | 12 | 10 |
| Note1: NR SRS carrier switching time is UE capability indicated by higher layer parameter SRS-SwitchingTimeNR. |  |  |  |  |

Table 8.2.1.2.12-2: Interruption length X2 (slot)

|  | NR Slot | SRS carrier | Interruption length X2 (slots) |  |
| --- | --- | --- | --- | --- |
|  | length (ms) of victim cell | switching time (us) Note | Sub carrier spacing for agressor cell (kHz) |  |
|  |  |  | 60 | 120 |
| 0 | 1 | ≤ 200 | 2 | 2 |
| 1 | 0.5 | ≤ 200 | 2 | 2 |
| 2 | 0.25 | ≤ 200 | 3 | 3 |
| 3 | 0.125 | ≤ 200 | 4 | 4 |
| Note1: NR SRS carrier switching time is UE capability indicated by higher layer parameter SRS-SwitchingTimeNR. |  |  |  |  |

For intra-band SRS carrier switching in FR1 or FR2, interruptions in Table 8.2.1.2.12-1 and in Table 8.2.1.2.12-2 based on SRS carrier switching time ≤ 200us shall apply. For inter-band SRS carrier switching in FR1 or between FR1 and FR2, interruptions in Table 8.2.1.2.12-1 and in Table 8.2.1.2.12-2 shall apply.

  - Endorsed CR R1-2104043 (TS38.214, Rel-16) in RAN1#104bis-e

<Unchanged parts are omitted>

- 6.2.1.3	UE sounding procedure between component carriers

A UE can be configured with SRS resource(s) on a carrier c1 with slot formats comprised of DL and UL symbols and not configured for PUSCH/PUCCH transmission. For carrier c1, the UE is configured with higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier the switching from carrier c2 which is configured for PUSCH/PUCCH transmission. During SRS transmission on carrier c1 (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR), the UE temporarily suspends the uplink transmission on carrier c2.

For an SRS transmission starting in symbol Nc1 of carrier c1 and a conflicting transmission in carrier c2 starting in symbol Nc2,  the UE shall apply the prioritization / dropping rules in the remainder of this subclause taking into account:

- DCI(s) for which the time interval between the last symbol of PDCCH and Nc1 is at least N2 symbols and an additional time duration TSRSCS,  and the time interval between the last symbol of PDCCH and Nc2 is at least   N2 symbols; and

- semi-persistent CSI reports or SRS considered active at least N2 symbols and an additional time duration TSRSCS before Nc1, and considered active at least N2 symbols before Nc2.

where TSRSCS=max⁡{switchingTimeUL,switchingTimeDL}, and the time interval unit of OFDM symbol is counted based on the smaller subcarrier spacing across c1, c2 and their corresponding scheduling cells.

For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall not transmit SRS whenever SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell and PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI/SSBRI and/or PRACH happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306].

For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall not transmit a periodic/semi-persistent SRS whenever periodic/semi-persistent SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell and PUSCH transmission carrying aperiodic CSI happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306].

For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall drop PUCCH/PUSCH transmission carrying periodic/semi-persistent CSI comprising only CQI/PMI/L1-RSRP/L1-SINR, and/or SRS transmission on another serving cell configured for PUSCH/PUCCH transmission whenever the transmission and SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the serving cell happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306].

For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall drop PUSCH transmission carrying aperiodic CSI comprising only CQI/PMI/L1-RSRP/L1-SINR whenever the transmission and aperiodic SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133]) as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306].

For an aperiodic SRS triggered in DCI format 2_3 and if the UE is configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeA', and given by SRS-CarrierSwitching, without PUSCH/PUCCH transmission, the order of the triggered SRS transmission on the serving cells follow the order of the serving cells in the indicated set of serving cells configured by higher layers, where the UE in each serving cell transmits the configured one or two SRS resource set(s) with higher layer parameter usage set to 'antennaSwitching' and higher layer parameter resourceType in SRS-ResourceSet set to 'aperiodic'.

For an aperiodic SRS triggered in DCI format 2_3 and if the UE is configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeB' without PUSCH/PUCCH transmission, the order of the triggered SRS transmission on the serving cells follow the order of the serving cells with aperiodic SRS triggered in the DCI, and the UE in each serving cell transmits the configured one or two SRS resource set(s) with higher layer parameter usage set to 'antennaSwitching' and higher layer parameter resourceType in SRS-ResourceSet set to 'aperiodic'.

A UE can be configured with SRS resource(s) on a carrier c1 with slot formats comprised of DL and UL symbols and not configured for PUSCH/PUCCH transmission. For carrier c1, the UE is configured with higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier the switching from carrier c2 which is configured for PUSCH/PUCCH transmission. During SRS transmission on carrier c1 (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR), the UE temporarily suspends the uplink transmission on carrier c2.

If the UE is not configured for PUSCH/PUCCH transmission on carrier c1 with slot formats comprised of DL and UL symbols, and if the UE is not capable of simultaneous reception and transmission on carrier c1 and serving cell c2, the UE is not expected to be configured or indicated with SRS resource(s) such that SRS transmission on carrier c1 (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) would collide with the REs corresponding to the SS/PBCH blocks configured for the UE or the slots belonging to a control resource set indicated by MIB or SIB1 on serving cell c2.

For n-th (n ≥ 1) aperiodic SRS transmission on a cell c, upon detection of a positive SRS request on a grant, the UE shall commence this SRS transmission on the configured symbol and slot provided

-	it is no earlier than the summation of

-	the maximum time duration between the two durations spanned by N OFDM symbols of the numerology of cell c and the cell carrying the grant respectively, and

-	the UL or DL RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR,

-	it does not collide with any previous SRS transmissions, or interruption due to UL or DL RF retuning time.

otherwise, n-th SRS transmission is dropped, where N is the reported capability as the minimum time interval in unit of symbols, between the DCI triggering and aperiodic SRS transmission.

In case of inter-band carrier aggregation, a UE can simultaneously transmit SRS and PUCCH/PUSCH across component carriers in different bands subject to the UE's capability.

In case of inter-band carrier aggregation, a UE can simultaneously transmit PRACH and SRS across component carriers in different bands subject to UE's capability.

<Unchanged parts are omitted>