3GPP TSG RAN WG1 #105-e							    R1-210xxxx

e-Meeting, May 10th – 27th, 2021

Source:	Moderator (Intel Corporation)

Title:	Discussion [105-e-NR-5G_V2X-05]

Agenda item:	7.2.4

Document for:	Discussion and Decision

Introduction

This document provides discussion on the following approved email thread as part of RAN1#105-e Release 16 NR V2X maintenance discussion:

[105-e-NR-5G_V2X-05] Email discussion/approval regarding

- Issue M2-1: TP to implement the agreement from [104b-e-NR-5G_V2X-03]

till 5/24 – Sergey (Intel)

Outcome

Discussion on Draft CR

## Round 1

In RAN1#104bis-e the following agreement was made:

Agreement

- Update the specification of identification of candidate resources for Mode-2 resource allocation in section 8.1.4 of TS 38.214 to handle the case when X·M_total number of identified resources could not be reached after any number of loop iterations

  - If the number of the excluded resources in step 5 is larger than (1-X)·M_total, a UE skips step 5

There was no sufficient time to implement the above agreement as a CR. In this meeting, the discussion on CR is initiated separately.

From the review of contributions [1]-[7], the following two different interpretations have been identified:

Interpretation 1 [1][2][3][5][6][7]

- The step 5 is performed, and after its completion the condition on (1-X)·M_total is checked. If the excluded number of resources is greater, then the result of step 5 is ignored, i.e. S_A is re-initialized as in step 4.

Interpretation 2 [4]

- The step 5 is interpreted as a loop over periodicities and/or non-monitored slots. In this assumption, once a UE excludes a slot / period leading to the number of excluded resources greater than (1-X)·M_total, it breaks the loop of step 5 and continues with step 6; S_A is not re-initialized.

Note, in [1] it is proposed to further introduce additional functionality beyond the agreement proposed in P2 and P3, therefore these considerations are not taken into account – only P1 is assumed relevant.

It should be noted that Interpretation 1 was also FL understanding in RAN1#104bis-e, and this could be tracked from the beginning of the discussion in [104b-e-NR-5G_V2X-03]. Nevertheless, the first question would be to confirm this interpretation (or interpretation 2), and then go to the detailed implementation discussion.

Q1: Which of the interpretations 1 and 2 above is correct in your view and should be further pursued as a CR?

| Source | Answer | Comment |
| --- | --- | --- |
| OPPO | Interpretation 1 | In our view, interpretation 2 was never intended and discussed as a solution from the beginning. It has several technical issues and these are discussed in [2]. |

- References

- R1-2104194	TP to address infinite loop due to excessive resource exclusion for Rel. 16 V2X	FUTUREWEI

-------------------------------- Start of Text Proposal for 38.214 (Based on Proposal 1) ----------------------------------- <Unchanged parts omitted> 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 <Unchanged parts omitted> 5) The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions: - the UE has not monitored slot t'mSL in Step 2. - for any periodicity value allowed by the higher layer parameter sl-ResourceReservePeriodList and a hypothetical SCI format 1-A received in slot t'mSL with 'Resource reservation period' field set to that periodicity value and indicating all subchannels of the resource pool in this slot, condition c in step 6 would be met. 5-1) If the number of candidate single-slot resources Rx,y excluded from the set SAin step 5 is greater than (1- X)⋅ Mtotal, the set SA is initialized to the set of all the candidate single-slot resources as in step 4). <Unchanged parts omitted> -------------------------------- End of Text Proposal for 38.214 (Based on Proposal 1) -----------------------------------

- R1-2104751	Discussion on TPs for skipping step 5 in mode 2 RA	OPPO

4) The set SA is initialized to the set of all the candidate single-slot resources. 5) The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions: - the UE has not monitored slot t'mSL in Step 2. - for any periodicity value allowed by the higher layer parameter sl-ResourceReservePeriodList and a hypothetical SCI format 1-A received in slot t'mSL with 'Resource reservation period' field set to that periodicity value and indicating all subchannels of the resource pool in this slot, condition c in step 6 would be met. 5-1) If the number of candidate single-slot resources excluded from the set SA in step 5 is greater than (1-X)⋅Mtotal, the set SA is initialized to the set of all the candidate single-slot resources. 6) The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

- R1-2104887	Correction to sidelink resource identification procedure to prevent infinite loop issue – implementation of the agreement from [104b-e-NR-5G_V2X-03]	Intel Corporation

<< UNCHANGED PARTS OMITTED >> 4) The set SA is initialized to the set of all the candidate single-slot resources. 5) The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions: - the UE has not monitored slot t'mSL in Step 2. - for any periodicity value allowed by the higher layer parameter sl-ResourceReservePeriodList and a hypothetical SCI format 1-A received in slot t'mSL with 'Resource reservation period' field set to that periodicity value and indicating all subchannels of the resource pool in this slot, condition c in step 6 would be met. If the number of candidate single-slot resources Rx,y excluded from the set SA in step 5 is greater than (1-X)⋅Mtotal, the set SA is initialized to the set of all the candidate single-slot resources. 6) The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions: << UNCHANGED PARTS OMITTED >>

- R1-2105463	Maintenance on NR sidelink mode-2 resource allocation mechanism	vivo

------------------------------------------------ Start of Draft TP of TS 38.214 -------------------------------------------- 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 <Unchanged parts omitted> 4) The set SA is initialized to the set of all the candidate single-slot resources. 5) The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions: - the UE has not monitored slot t'mSL in Step 2. - for any periodicity value allowed by the higher layer parameter sl-ResourceReservePeriodList and a hypothetical SCI format 1-A received in slot t'mSL with 'Resource reservation period' field set to that periodicity value and indicating all subchannels of the resource pool in this slot, condition c in step 6 would be met. - the number of candidate single-slot resources excluded from the set SA is not greater than (1-X)⋅Mtotal ---------------------------------------------------------- End of Draft TP ----------------------------------------------------

- R1-2105612	Remaining issues on mode 2	ZTE, Sanechips

### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 <Unchanged parts omitted> 4) The set SA is initialized to the set of all the candidate single-slot resources. 5) The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions: - the UE has not monitored slot t'mSL in Step 2. - for any periodicity value allowed by the higher layer parameter sl-ResourceReservePeriodList and a hypothetical SCI format 1-A received in slot t'mSL with 'Resource reservation period' field set to that periodicity value and indicating all subchannels of the resource pool in this slot, condition c in step 6 would be met. 5-1) If the number of candidate single-slot resources remaining in the set SA is smaller than X⋅Mtotal, the set SA is initialized to the set of all the candidate single-slot resources. 6) The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions: a) the UE receives an SCI format 1-A in slot t'mSL, and 'Resource reservation period' field, if present, and 'Priority' field in the received SCI format 1-A indicate the values Prsvp_RX and prioRX, respectively according to Clause 16.4 in [6, TS 38.213]; b) the RSRP measurement performed, according to clause 8.4.2.1 for the received SCI format 1-A, is higher than ThprioRX,prioTX; c) the SCI format received in slot t'mSL or the same SCI format which, if and only if the 'Resource reservation period' field is present in the received SCI format 1-A, is assumed to be received in slot(s) t'm+q×Prsvp_RX'SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots according to clause 8.1.7, Q=TscalPrsvp_RX if Prsvp_RX< Tscal and n'-m≤Prsvp_RX', where t'n'SL = n if slot n belongs to the set t'0SL,t'1SL,...,t'T'max-1SL, otherwise slot t'n'SL is the first slot after slot n belonging to the set t'0SL,t'1SL,...,t'T'max-1SL; otherwise Q=1. Tscal is set to selection window size T2 converted to units of msec. 7) If the number of candidate single-slot resources remaining in the set SA is smaller than X⋅Mtotal, then Th(pi,pj) is increased by 3 dB for each priority value Th(pi,pj) and the procedure continues with step 4. <Unchanged parts omitted>

- R1-2105897	Condition to stop the infinite loop for Mode 2 RA	Ericsson

### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 <Unchanged parts omitted> 4) The set SA is initialized to the set of all the candidate single-slot resources. 5) The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions: - the UE has not monitored slot t'mSL in Step 2. - for any periodicity value allowed by the higher layer parameter sl-ResourceReservePeriodList and a hypothetical SCI format 1-A received in slot t'mSL with 'Resource reservation period' field set to that periodicity value and indicating all subchannels of the resource pool in this slot, condition c in step 6 would be met. 5-1) If the number of candidate single-slot resources Rx,y excluded from the set SA in step 5 is greater than (1-X)⋅Mtotal, the set SA is initialized to the set of all the candidate single-slot resources as indicated in Step 4.

- R1-2105944	Maintenance for Resource allocation for sidelink - Mode 2	Nokia, Nokia Shanghai Bell

-------------------------- Start of Text Proposal for TS 38.214 -------------------------- ### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 ------------------- < Unchanged parts are omitted > -------------------------- 5a) If the number of candidate single-slot resources remaining in the set SA is smaller than X⋅Mtotal, then the set SA is initialized to the set of all the candidate single-slot resources. -------------------------- End of Text Proposal for TS 38.214 --------------------------