3GPP TSG- RAN WG1 Meeting #105-e                                                 R1-21xxxxx

# e-Meeting,  May 10th - 27th, 2021

Agenda Item:	7.2.6

Source:	Moderator (Apple Inc.)

Title:	Summary of [105-e-NR-eMIMO-02] Email Discussion

Document for:	Discussion/Decision

Introduction

In this contribution, we provide a summary on 105-e-NR-eMIMO-02.

MB.5 (E)

In R1-2105537, Huawei/HiSilicon propose a TP to avoid inconsistency between 38.331 and 38.214 with regard to time domain measurement restriction.

| Reason for change: | The current specification has defined UE behavior for measurement restriction for L1-SINR. There is a conflict between the descriptions in TS 38.214 and 38.331. Specifically, - In 38.214, the higher layer parameters timeRestrictionForChannelMeasurements and timeRestrictionForInterferenceMeasurements are considered as optional. Whether to apply measurement restriction for L1-SINR is determined based on whether the associated higher layer parameter is configured or not. - In 38.331, the higher layer parameters timeRestrictionForChannelMeasurements and timeRestrictionForInterferenceMeasurements are mandatory to be present. The candidate values for the two parameters are ‘configured’ and ‘notConfigured’. When timeRestrictionForChannelMeasurements or timeRestrictionForInterferenceMeasurements is configured as “notConfigured”, if the UE determines whether to apply measurement restriction according to the description in 38.214, it may still be mist-interpreted as that the UE should apply measurement restriction, although the true intention of gNB is to disable measurement restriction. |
| --- | --- |
| Summary of change: | Update the operating conditions for applying measurement restriction for L1-SINR in 38.214 to be aligned with signalling design in 38.331, i.e., from “is not configured with” to “the value of … is configured as ‘notConfigured’”, and from “is configured with” to “the value of … is configured as ‘configured’”. |
| Consequences if not approved: | Inconsistency between 38.214 and 38.331. |

Text Proposal for 38.214

5.2.1.4.4 L1-SINR Reporting < Unchanged parts are omitted > When one or two resource settings are configured for L1-SINR measurement - If the value of higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is configured as ‘notConfigured’, the UE shall derive the channel measurements for computing L1-SINR reported in uplink slot n based on only the SSB or NZP CSI-RS, no later than the CSI reference resource, (defined in TS 38.211[4]) associated with the CSI resource setting. - If the value of higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is configured as ‘configured’, the UE shall derive the channel measurements for computing L1-SINR reported in uplink slot n based on only the most recent, no later than the CSI reference resource, occasion of SSB or NZP CSI-RS (defined in [4, TS 38.211]) associated with the CSI resource setting. - If the value of higher layer parameter timeRestrictionForInterferenceMeasurements in CSI-ReportConfig is configured as ‘notConfigured’, the UE shall derive the interference measurements for computing L1-SINR reported in uplink slot n based on only the CSI-IM or NZP CSI-RS for interference measurement (defined in [4, TS 38.211]) or NZP CSI-RS for channel and interference measurement no later than the CSI reference resource associated with the CSI resource setting. - If the value of higher layer parameter timeRestrictionForInterferenceMeasurements in CSI-ReportConfig is configured as ‘configured’, the UE shall derive the interference measurements for computing the L1-SINR reported in uplink slot n based on the most recent, no later than the CSI reference resource, occasion of CSI-IM or NZP CSI-RS for interference measurement (defined in [4, TS 38.211]) or NZP CSI-RS for channel and interference measurement associated with the CSI resource setting. < Unchanged parts are omitted >

Companies’ view and comments

| Company | Comments |
| --- | --- |
| vivo | Fine with the alignment. |

MB.4 (ND)

In R1-2104582, ZTE proposes to update CORESETPoolIndex after BFR.

Justification

The QCL assumption of all CORESETs on a beam failure SCell is determined according to qnew reported by the UE. Consequently, in such case, all CORESETs on the beam failure SCell(s) can only be transmitted by a single TRP, and it is straightforward that M-TRP transmission with two CORESETPoolIndex values does NOT work after BFR procedure is completed. Therefore, a fall back mechanism from M-TRP to S-TRP for SCell is required in Rel-16 based on the following analysis.

- After SCell-BFR is completed, there is no need for UE to keep tracking two CORESETPoolIndex values, and the UE power consumption can be saved significantly.

- Otherwise, if going with current spec, both features of SCell BFR and mDCI-mTRP can NOT be performed well in a given UE. In other words, it may reduce the motivation/possibility of both gNB and UE vendors to deploy these two useful features together.

To achieve this, a straightforward solution is to set CORESETPoolIndex of all CORESETs of the failed SCell(s) as 0 by default in the spec.

Text Proposal for 38.213

<Unchanged part is omitted> A UE can be provided, by schedulingRequestID-BFR-SCell-r16, a configuration for PUCCH transmission with a link recovery request (LRR) as described in Clause 9.2.4. The UE can transmit in a first PUSCH MAC CE providing index(es) for at least corresponding SCell(s) with radio link quality worse than Qout,LR, indication(s) of presence of qnew for corresponding SCell(s), and index(es) qnew for a periodic CSI-RS configuration or for a SS/PBCH block provided by higher layers, as described in [11, TS 38.321], if any, for corresponding SCell(s). After 28 symbols from a last symbol of a PDCCH reception with a DCI format scheduling a PUSCH transmission with a same HARQ process number as for the transmission of the first PUSCH and having a toggled NDI field value, the UE - monitors PDCCH in all CORESETs on the SCell(s) indicated by the MAC CE using the same antenna port quasi co-location parameters as the ones associated with the corresponding index(es) qnew, if any, and assumes the CORESETPoolIndex, if configured, of all CORESETs on the SCell(s) as 0. <Unchanged part is omitted>

FL proposal

As the discussion is to make a conclusion (ND issue), a conclusion is expected. From companies’ comments from the preparation phase, the following alternatives are provided. There are some comments that this is an optimization, and other comments that BFR+mTRP should be a Rel-17 feature. In addition, I added original proposal as Alt1, but I am not sure whether Alt1 is still valid since this is a ND issue.

Possible conclusion

- Alt1: For mDCI based mTRP, UE automatically update CORESETPoolIndex to be 0 after 28 symbols after receiving beam failure recovery response

  - Endorse the TP in R1-2104582

- Alt2: For mDCI based mTRP, UE does not automatically update CORESETPoolIndex after receiving beam failure recovery response

  - No spec change is needed

- Alt3: Rel-16 does not support concurrent configuration of BFR and mDCI based mTRP in the same BWP

- Alt4: Other (Please provide details)

Companies’ view and comments

| Company | Comments |
| --- | --- |
| vivo | Alt3 seems the cleanest solution. Alt1 might be too late to introduce. We are also ok with Alt2 with network implementation to handle the issue. |

MT.8

In R1-2104651, Qualcomm proposes to update the default PDSCH beam.

Reason for change: In current spec, there is no way for UE and gNB to communicate with default beam after BFR in the case of single-DCI based mTRP.

Summary of change: Resetting the default beam by resetting all TCI codepoints to the new identified beam.

Consequences if not approved: gNB and UE cannot communicate with default beam after BFR in the case of single-DCI based mTRP.

Text Proposal for 38.213

--Unchanged part omitted------------------------ For the PCell or the PSCell, after 28 symbols from a last symbol of a first PDCCH reception in a search space set provided by recoverySearchSpaceId where a UE detects a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI, the UE assumes same antenna port quasi-collocation parameters as the ones associated with index for PDCCH monitoring in a CORESET with index 0, and if at least one TCI codepoint indicates two TCI states and UE is configured with enableTwoDefaultTCIStates, the UE assumes all TCI codepoints indicate same antenna port quasi-collocation parameters as the ones associated with index . A UE can be provided, by schedulingRequestID-BFR-SCell-r16, a configuration for PUCCH transmission with a link recovery request (LRR) as described in Clause 9.2.4. The UE can transmit in a first PUSCH MAC CE providing index(es) for at least corresponding SCell(s) with radio link quality worse than Qout,LR, indication(s) of presence of qnew for corresponding SCell(s), and index(es) qnew for a periodic CSI-RS configuration or for a SS/PBCH block provided by higher layers, as described in [11, TS 38.321], if any, for corresponding SCell(s). After 28 symbols from a last symbol of a PDCCH reception with a DCI format scheduling a PUSCH transmission with a same HARQ process number as for the transmission of the first PUSCH and having a toggled NDI field value, the UE - monitors PDCCH in all CORESETs on the SCell(s) indicated by the MAC CE using the same antenna port quasi co-location parameters as the ones associated with the corresponding index(es) qnew, if any - If at least one TCI codepoint indicates two TCI states and UE is configured with enableTwoDefaultTCIStates on the SCell(s) indicated by the MAC CE, the UE assumes all TCI codepoints indicate same antenna port quasi-collocation parameters as the ones associated with the corresponding index(es) , if any.

FL proposal

As the discussion is to make a conclusion (ND issue), the following possible conclusion is proposed based on the comments from preparation phase. Similar to MB.4, I am not sure whether Alt1 is still valid since it is an ND issue.

Possible conclusion

- Alt1: For single-DCI based multi-TRP mode, UE resets the default PDSCH beam based on newly identified beam after 28 symbols after receiving beam failure recovery response

  - Endorse TP in R1-2104651

- Alt 2: For single-DCI based multi-TRP mode, UE does not automatically update default PDSCH beam after receiving beam failure recovery response

  - No spec change is needed

- Alt3: Other (Please provide details)

Companies’ view and comments

| Company | Comments |
| --- | --- |