3GPP TSG RAN WG1 Meeting #105-e	R1-2106071

e-Meeting, May 10th – 27th, 2021

Agenda Item:	8.1.4

Source:	Huawei, HiSilicon (Moderator)

Title:	Summary of CSI enhancements for MTRP and FDD (Round 1)

Document for:	Discussion and Decision

# Introduction

# Summary of CSI enhancement for FDD

Proposal 1: For Rel-17 port selection codebook, the maximal value of CSI-RS port number P as Pmax is 32.

- Note: further reduction of the maximal value of CSI-RS port number for each possible parameter combination among codebook parameters of Rel-17 port selection codebook, e.g. {K1, Mv, Beta}, will be discussed jointly

- Note that total number of parameter combinations (for all # of CSI-RS ports) shall not exceed 8

Yes (18): vivo, OPPO, ZTE, MTK, DOCOMO, LG, Nokia/NSB, Lenovo/Mot, [Samsung] (Suggest to discuss after conclusion 2), Intel, Fraunhofer IIS/Fraunhofer HHI, Apple, Spreadtrum, CATT, Sony

Proposal 2: At least for rank 1, values of K1 for port selection matrix W1 in N^(P*K1) are {2, 4, 8, 12, 16, 24, 32}.

- Note that further reduction for possible parameter combinations among codebook parameters of Rel-17 port selection codebook, e.g. {K1, Mv, Beta}, will be discussed jointly once candidate values are determined.

- Note that total number of parameter combinations (for all # of CSI-RS ports) shall not exceed 8

Yes (17): vivo, ZTE, QC (Need further restrictions), Ericsson (2 K1 values per port), MTK (small values of K1 not needed), Nokia/NSB, Lenovo/Mot, Samsung, Fraunhofer IIS/Fraunhofer HHI, Apple, OPPO, CATT, Intel, Sony

Conclusion 1:  At least for rank 1, no further restriction or condition is applied for polarization-common based free-selection and combinatorial coefficient based port selection for W1.

Yes(19): vivo, ZTE, Ericsson, MTK, CATT, LG, Nokia/NSB, Lenovo/Mot, Samsung, Intel, Fraunhofer IIS/Fraunhofer HHI, Apple, OPPO, DOCOMO, Spreadtrum, Sony

Proposal 4: At least for rank 1, FD bases used for Wf quantization are limited within a single window with size N configured to the UE whereas FD bases in the window must be consecutive from an orthogonal DFT matrix, i.e. Alt 1.

- FFS: whether further dependence/restriction, e.g. conditioned on N3 or the number of CSI-RS ports, can be applied to above design. If does, how to support a non-consecutive FD bases used for Wf quantization

Yes (18): OPPO, ZTE, Ericsson, DOCOMO, CATT, Spreadtrum, Nokia/NSB, Lenovo/Mot, [Samsung] (Larger value of N3), Intel, Fraunhofer IIS/Fraunhofer HHI, Apple, MTK, vivo, Sony

No (1): LG (Alt 2). [Samsung](Smaller value of N3)

Proposal 10: For the compression coefficient Beta for non-zero coefficients of W2, values of Beta are {1/4, 1/2, 3/4, 1}

- Note that further reduction for possible parameter combinations among codebook parameters of Rel-17 port selection codebook, e.g. {K1, Mv, Beta}, will be discussed jointly once candidate values are determined.

- Note that total number of parameter combinations (for all # of CSI-RS ports) shall not exceed 8

Yes(17):  vivo, ZTE, Ericsson (No small values), MTK (No 1/8), DOCOMO, CATT(without notes), Spreadtrum [Support larger values first and FFS smaller values], Nokia/NSB, Lenovo/Mot, Intel, Fraunhofer IIS/Fraunhofer HHI, Samsung, Apple, Sony

Proposal 12: A polarization-specific bitmap for indication non-zero coefficients should be supported for W2.

Yes (18): vivo, OPPO, ZTE, Ericsson, MTK, LG, Spreadtrum, Nokia/NSB, Lenovo/Mot, Samsung, Intel, Fraunhofer IIS/Fraunhofer HHI, Apple, Spreadtrum, CATT

Proposal 14: For the quantization of W2 coefficient, reusing following Rel-16 quantization mechanism for Rank1 at least:

- Two polarization-specific reference amplitudes:

  - for the polarization associated with the strongest coefficient, the reference amplitude is not reported

  - for the other polarization, reference amplitude is quantized to 4 bits

    - The alphabet is{1, 1/2)^(1/4), (1/4)^(1/4), (1/8)^(1/4), …, (1/2^14)^(1/4), [Reserved]} (-1.5dB step size)

- For coefficients other than the strongest coefficient

  - differential amplitude is calculated relative to the associated polarization-specific reference amplitude and quantized to 3 bits

    - The alphabet is {1, 1/sqrt(2), 1/2, 1/(2*sqrt(2)), 1/4, 1/(4*sqrt(2)), 1/8, 1/(8*sqrt(2))} (-3dB step size)

  - phase is quantized to 16PSK

- For the reserved state for reference amplitude, down-select one Alt

  - Alt 1: it is kept to be reserved

  - Alt 2: it is replaced as (1/2)^(15/4),

  - Alt 3: it is replaced as (1/2)^(3/8)

Note: whether/how SCI is supported for R17 codebook will be discussed separately

Alt1 (15): vivo, OPPO, Ericsson, MTK, CATT, LG, Spreadtrum, Nokia/NSB, Intel, Fraunhofer IIS/Fraunhofer HHI, Apple, QC

Alt 2 or Alt 3 (2): ZTE, Samsung

Lenovo/Mot (FFS differential amplitude values)

Conclusion 2: For PS codebook enhancements utilizing DL/UL reciprocity of angle and/or delay, there is no consensus of further enhancement for CSI-RS configurations associated with Rel-17 PS codebook.

OK to make a conclusion as it is: QC, vivo, OPPO, MTK, DOCOMO, LG, Spreadtrum, Lenovo/Mot, Fraunhofer IIS/Fraunhofer HHI, Apple, Lenovo/Mot, Spreadtrum

Option 3: ZTE, Ericsson, CATT, Nokia/NSB, Samsung, Intel

Option 1: Nokia/NSB, Intel

# Summary of CSI enhancement for MTRP

Proposal 18: Whether a NZP CSI-RS resource can be referred by both a CMR pair configured for NCJT measurement hypothesis and a CMR configured for Single-TRP measurement hypothesis:

- It is feasible in both FR1 and FR2 but subject to UE capability for FR2. If a UE supports and the sharing is also enabled by gNB, two CMRs from a CMR pair configured for a NCJT measurement hypothesis can be used for Single-TRP measurement hypotheses, otherwise they cannot.

Proposal 20: Whether to support interference measurement based on NZP CSI-RS outside the CMR pair configured for NCJT measurement hypothesis, in addition to CSI-IM

Alt 1: Yes, it is supported, subject to limitations, e.g. N=1 CMR pair and Ks=2 CMR resources

Alt 2: No, it is not supported

Alt 1 (6): ZTE, Docomo, CMCC, InterDigital, Fraunhofer IIS, Fraunhofer HHI

Alt 2 (15): Vivo, QC, OPPO, MediaTek, Ericsson, Spreadtrum, LG, Nokia/NSB, Lenovo/Mot, NEC, Intel, Apple, Futurewei