3GPP TSG RAN WG1 Meeting #105-e		R1- 21XXXXX

e-Meeting, May 19th – 27th, 2021

Agenda item:       8.1.4

Source: 	Nokia, Nokia Shanghai Bell

Title: 	On the alternative SCI solutions

Document for:     Discussion and Decision

# 1	Introduction

In this document we provide some simple examples to illustrate the differences between the alternatives 0, 1 and 2 proposed for the SCI reporting.

Proposal 13: Study following alternatives for reporting the strongest coefficient indication (SCI) for Rel-17 port selection codebook in W2

- Alt 0: Reporting of the position, [il*, fl*], of the strongest coefficient of layer l using ceil(log2(K0)) bits, where K0=Beta*K1*Mv

- Alt 1: Reporting of the position, [il*, fl*], of the strongest coefficient of layer l,  using ceil(log2(K1*Mv)) or ceil(log2(K1))+ceil(log2(Mv)) bits

  - FFS whether phase shifting the strongest coefficient to fl* = 0 is needed

- Alt 2: phase shifting the strongest coefficient to fl* = 0, and using ceil(log2(N)) bits to indicate the phase shift quantity for l-th layer. The strongest coefficient is indicated by il*, using ceil (log2 (K1,l)) for l-th layer.

- Alt 3: SCI is not needed so that the SCI in R16 codebook is replaced with a strongest polarization indicator (1 bit)

We consider a simple toy example with K1=2, Mν=2, β=1 and two cases for the window size: N=3 and N=2. We assume that, for N=3, Wf is reported with log2N-1Mν-1=1 bit, whereas for N=2, Wf is not reported.

In the example we assume KNZ=K0=4 and the NZC are: W2=acbd. The strongest coefficient is c in position il*,fl*=[0,1].

# 2	Example 1: N=3,Mν=2

Let’s assume Wf=n3,l0,n3,l1=[1, 2].

- Alt 0.

  - Shift on Wf (different from Rel16). The FD components are remapped with respect to n3,l0=1, as n3,lf=n3,lf-n3,l0, such that n3,l0=0 after remapping. In the example, only the FD component n3,l1=1 is reported.

  - Shift on W2 (different from Rel16). None

  - SCI. The SCI is calculated from the bitmap B=bi,f, bi,f∈{0,1} and il*,fl* for example as follows:

SCI=f=0fl*-1i=0K1bi,f+i=0il*bi,fl*-1

In the example, SCI=2, which is represented with log2(βK1Mν)=2 bits.

  - UCI encoding (different from Rel16).

    - The SCI and bitmap need to be grouped together in G0 because the SCI depends on the bitmap.

    - If the FD component of the strongest coefficient is encoded first as in Rel16, because statistically more significant, the index f needs to be remapped with respect to fl* as f=f-fl* mod Mν , such that W2 is reported as W2=cadb, i.e., the columns of W2 are swapped

- Alt 1.

  - Shift on Wf(different from Rel16). The FD components are remapped with respect to n3,l0=1, as n3,lf=n3,lf-n3,l0, such that n3,l0=0 after remapping. In the example, only the FD component n3,l1=1 is reported.

  - Shift on W2 (same as Rel16). The index f is remapped with respect to fl* as f=f-fl* mod Mν , such that W2 is reported as W2=cadb, i.e., the columns of W2 are swapped, and the FD component of the strongest coefficient is encoded first in the UCI as in Rel16

  - SCI. The SCI is reported as il*,fl* with log2(K1)+log2(Mν)=2 bits or calculated, for example, as follows:

SCI=K1fl*+il*

which is represented with log2(K1Mν)=2 bits.

  - UCI encoding (same as Rel16).

- Alt 2.

  - Shift on Wf(same as Rel16). The FD components are remapped with respect to n3,lfl*=2, as n3,lf=n3,lf-n3,lfl*mod N3, such that n3,lfl*=0 after remapping. In the example, only the FD component n3,l0=N3-1 is reported (in the same way as for Rel16 IntS mechanism, where Minitial=-n3,lfl* and the window size is N instead of 2Mν).

n3,lfl*∈{0,…,N-1} also needs reporting with log2(N) bits for each layer.

  - Shift on W2(same as Rel16). The index f is remapped with respect to fl* as f=f-fl* mod Mν , such that W2 is reported as W2=cadb and the FD component of the strongest coefficient is encoded first in the UCI as in Rel16

  - SCI. The SCI is reported as il* with log2(K1) bits

  - UCI encoding (same as Rel16).

# 3	Example 2: N=Mν=2

In this case Wf=n3,l0,n3,l1=[0, 1] and Wf is not reported. Alt 1 and Alt 2 become equivalent (with separate reporting of il* and fl*)

- Alt 0.

  - Shift on W2. None

  - SCI. Same as example 1

  - UCI encoding.

    - The SCI and bitmap need to be grouped together in G0.

    - If the FD component of the strongest coefficient is encoded first as in Rel16 because statistically more significant, the index f needs to be remapped with respect to fl* as f=f-fl* mod Mν , such that W2 is reported as W2=cadb

- Alt 1.

  - Shift on W2. Same as example 1

  - SCI. Same as example

  - UCI encoding (same as Rel16).

- Alt 2.

  - Shift on W2. Same as example 1

  - SCI (almost same as Rel16). The SCI is reported as il* with log2(K1) bits and the shift n3,lfl*=fl*∈{0,…,Mν-1} is also reported with log2(Mν) bits per layer. In this case the SCI is the same as Alt 1 when using log2(K1)+log2(Mν) bits.

  - UCI encoding (same as Rel16).