3GPP TSG RAN meeting #92e										RP-21xxxx

Electronic Meeting, June 14-18, 2021

Status Report to TSG

Agenda item:			9.7.1.3

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_SL_enh |  |  |  |  |
| Unique ID | 860042 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-202846 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: mm/yyyy | Core part: 03/2022 | Performance part: 09/2022 | Testing part: mm/yyyy |  |
| Overall Completion level | Study Item: xx % | Core part: 45% | Performance Part: 0% | Testing part: xx% |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | Seungmin Lee |
|  | Company | LG Electronics |
|  | Email | edison.lee@lge.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

RAN1#104bis-e

Regarding resource allocation for power saving, the following agreements and conclusion were made:

- Conclusion on resource selection window of periodic-based partial sensing operation

  - In periodic-based partial sensing,

    - It is not necessary to further discuss whether or not to introduce a threshold to re-define T1 and T2.

- Agreement on sensing occasions of periodic-based partial sensing operation

  - In periodic-based partial sensing,

    - For the set of Preserve values, down-select to one of the following in RAN1#105-e

      - Alt.1: Preserve corresponds to all values from the configured set sl-ResourceReservePeriodList

      - Alt.2: A set of Preserve values is (pre-)configured and includes up to the full set of values from the configured set sl-ResourceReservePeriodList

        - FFS if support multiple sets of Preserve values based on one or more metrics

        - FFS whether/how to restrict the set of values

    - For the k value, down-selection to one of the following in RAN1#105-e (further refinement of each of the alternatives is possible)

      - Alt 1: Option 1 as in RAN1#104-e

      - Alt 2: A modified Option 5 as in RAN1#104-e, where the modification is such that it also includes option 1

        - FFS how to (pre-)configure (e.g. including bitmap), whether a maximum number of k values is needed, and whether it can be up to UE implementation to select a k value based on the (pre-)configuration

      - FFS details, e.g., sensing before the resource (re)selection trigger or the first slot of the set of Y candidate slots subject to processing time restriction, etc.

    - Note: companies are encouraged to provide more evaluations

- Agreement on conditions for UE to perform periodic-based partial sensing operation

  - When periodic-based partial sensing is potentially performed by UE in a mode 2 Tx resource pool provided by higher layer, at least all of the followings are met:

    - Periodic reservation for another TB (sl-MultiReserveResource) is enabled for the resource pool

    - The resource pool is (pre-)configured to enable partial sensing

    - Partial sensing configured by higher layer in the UE

Regarding inter-UE coordination for mode 2 enhancements, the following agreements were made:

- Agreement on inter-UE coordination schemes

  - Support the following schemes of inter-UE coordination in Mode 2:

    - Inter-UE Coordination Scheme 1:

      - The coordination information sent from UE-A to UE-B is the set of resources preferred and/or non-preferred for UE-B’s transmission

        - FFS details including a possibility of down-selection between the preferred resource set and the non-preferred resource set, whether or not to include any additional information other than indicating time/frequency of the resources within the set in the coordination information

      - FFS condition(s) in which Scheme 1 is used

    - Inter-UE Coordination Scheme 2:

      - The coordination information sent from UE-A to UE-B is the presence of expected/potential and/or detected resource conflict on the resources indicated by UE-B’s SCI

        - FFS details including a possibility of down-selection between the expected/potential conflict and the detected resource conflict

      - FFS condition(s) in which Scheme 2 is used

- Agreement on conditions for UEs to be UE-A(s)/UE-B(s) for inter-UE coordination

  - Study further to determine the conditions for UEs to be UE-A(s)/UE-B(s) for inter-UE coordination:

    - Details include applicable scenario(s)/inter-UE coordination scheme(s)

    - E.g., only UE(s) among the intended receiver(s) of UE-B can be a UE-A, any UE can be a UE-A, high-layer configured, etc.

      - Including the possibility of being subject to certain conditions and/or capability

- Agreement on UE-B’s behaviour of using inter-UE coordination information

  - When UE-B receives the inter-UE coordination information from UE-A, consider at least one of the following options (with details FFS including possibly down-selecting/merging one or more of the options below, applicable scenario(s)/condition(s) for each option, UE behavior) for UE-B’s to take it into account in the resource (re)-selection for its own transmission

    - For scheme 1:

      - Option 1-1: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on both UE-B’s sensing result (if available) and the received coordination information

      - Option 1-2: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based only on the received coordination information

      - Option 1-3: UE-B’s resource(s) to be re-selected based on the received coordination information

      - Option 1-4: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on the received coordination information

    - For scheme 2:

      - Option 2-1: UE-B can determine resource(s) to be re-selected based on the received coordination information

      - Option 2-2: UE-B can determine a necessity of retransmission based on the received coordination information

RAN1#105-e

Regarding resource allocation for power saving, the following agreements were made:

- Agreement on sensing occasions of periodic-based partial sensing operation

  - For the set of Preserve values in periodic-based partial sensing,

    - If no (pre-)configuration (i.e., by default), Preserve corresponds to all values from the (pre-)configured set sl-ResourceReservePeriodList.

    - Otherwise, a single set of Preserve values can be (pre-)configured, where the set of Preserve values are restricted to a subset of the (pre-)configured set sl-ResourceReservePeriodList

      - This is per mode 2 Tx resource pool (pre-)configuration

      - A UE by implementation may also monitor other sl-ResourceReservePeriodList values not part of the restricted subset

        - In particular, the UE may additionally monitor occasions corresponding to P_RSVP_Tx

          - FFS whether the monitoring can be mandatory

  - For the k value in periodic-based partial sensing for resource (re)selection,

    - By default, the UE monitors the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger slot n or the first slot of the set of Y candidate slots subject to processing time restriction.

    - If (pre-)configured, UE additionally monitors periodic sensing occasions that correspond to a set of values which can be (pre-)configured with at least one value

      - (Working assumption) Possible values correspond to the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger slot n or the first slot of the set of Y candidate slots, and the last periodic sensing occasion prior to the most recent one for the given reservation periodicity are included.

      - FFS: whether/which other values and details of the (pre-)configuration (e.g. max number of values or sensing occasions)

      - FFS: whether a value denotes a specific occasion to monitor or the earliest occasion to start the monitoring.

    - FFS relationship between periodic-based partial sensing occasions and SL-DRX

    - Note:

      - This is for the case when the resource (re)selection triggering slot n is expected by UE

  - In periodic-based partial sensing for resource (re)selection, the UE at least monitors in periodic sensing occasion(s) for a given reservation periodicity before the first slot of the selected Y candidate slots subject to processing time restriction for the identification of candidate resources.

    - The processing time restriction includes Tproc,0SL  and Tproc,1SL.

    - Aspects relating to sensing during SL DRX are to be discussed separately

  - Relationship to re-evaluation and pre-emption operation for periodic-based partial sensing to be discussed separately

    - FFS details including whether monitoring of periodic sensing occasions between triggering slot n and the first slot of the selected Y candidate slots subject to processing time restriction is performed as part of resource (re)selection or re-evaluation and pre-emption checking

- Agreement on random resource selection

  - For random resource selection,

    - Reuse the maximum distance separation of 32 logical slots for a HARQ retransmission resource reserved by a prior SCI for the same TB, which was defined in R16 for full sensing operation.

    - SL HARQ feedback enabled transmission is supported (FFS applicable conditions if any)

      - The minimum HARQ feedback time gap (Z) shall be respected between any two selected resources of a TB where a HARQ feedback for the first of these resources is expected.

  - FFS the impact of resource collision when random resource selection is performed by a UE which does not perform sensing / re-evaluation and pre-emption checking in a resource pool with mixed RA schemes (e.g. for low priority or any priority transmissions).

    - Including study potential solution(s) if the impact is not negligible (e.g. threshold based, raising priority, minimum time gap, pattern based, a priori SCI reserving initial transmissions, resource pool partitioning, and etc.).

- Agreement on sensing occasions of contiguous partial sensing operation

  - In contiguous partial sensing for resource (re)selection, TA and TB values can be zero, positive or negative

    - TA and TB values or range depend on different operating scenarios or conditions (e.g., periodic/aperiodic traffic, predictability of triggering slot n, remaining PDB, re-evaluation/pre-emption checking, HARQ feedback, CBR/CR parameter, power saving, etc)

      - FFS details

    - FFS: details of how periodic-based partial sensing and contiguous partial sensing are used for re-evaluation and pre-emption checking. Including how to reduce UE’s power consumption (caused by additional sensing operation of re-evaluation/pre-emption) after its resource selection, with the considerations of different operating scenarios or conditions (e.g., pre-emption enabled/disabled, HARQ-ACK enabled/disabled, etc).

2.1.2	Remaining Open issues

The followings are the remaining open issues:

- Physical layer aspects on resource allocation to reduce UE’s power consumption including;

  - Details of partial sensing based resource selection and random resource selection

  - Details and condition(s) in which re-evaluation and pre-emption can be performed by UEs performing sensing

  - Whether/how to support congestion control for power saving resource allocation schemes

  - Impacts of sidelink DRX on physical layer, if any

- Physical layer aspects on solution(s) on the enhancement(s) in mode 2 for enhanced reliability and reduced latency

2.2	RAN2

2.2.1	Agreements

RAN2#113bis-e

Regarding sidelink DRX, the following agreements and working assumptions were made:

- Agreement on details of SL DRX timer

  - The following parameters are supported as part of the SL DRX configuration for all cast types: sl-drx-StartOffset, sl-drx-Cycle, sl-drx-onDurationTimer, and sl-drx-SlotOffset.

  - The RX UE determines the symbol/slot/subframe associated with the start of the DRX cycle using the configured sl-drx-Cycle, sl-drx-StartOffset.  FFS on details.

  - The RX UE starts the sl-drx-onDurationTimer after sl-drx-slotOffset from the beginning of the subframe.

  - The RX UE’s active time includes the time in which sl-drx-on-DurationTimer is running.

  - For unicast, the TX UE behaviors should be specified to keep aligned with the RX UE regarding the DRX Active time. FFS the specific Spec impacts needed at the TX side.

  - For unicast, the RX UE maintains a separate SL inactivity timer for each pair of src/dest L2 ID.

  - For unicast, the SL inactivity timer value may take into consideration the QoS.  Whether any specification impacts are needed is FFS.

  - For unicast, RX UE starts/restarts the inactivity timer with the value configured for that pair of src/dest L2 ID.

  - For unicast, the RX UE (re)starts the inactivity timer upon reception of a new SL data transmission from the RX UE perspective for that pair of src/dest L2 ID.

  - For unicast, the RX UE (re)starts the inactivity timer based on information in SCI (SCI1+SCI2).  FFS if the MAC layer can stop the inactivity timer.

  - For unicast, the RX UE (re)starts the inactivity timer in the first slot after SCI (SCI1+SCI2) reception.

  - For unicast, the TX UE maintains a timer corresponding to the SL Inactivity timer in the RX UE for each pair of src/dest L2 ID, and uses the timer as part of criterion for determining the allowable transmission time for the RX UE.

  - For unicast, the TX UE (re)starts its timer corresponding to the SL inactivity timer at the RX UE at the slot following an SCI transmission indicating a new data transmission. FFS the specific spec impacts needed at the TX side.

  - SL Inactivity timer is supported for groupcast. FFS on the scenarios where it is supported.

  - SL Inactivity timer is not supported for broadcast transmissions.

  - The RX UE is active on sidelink (monitors SCI1+SCI2) as long as at least one of the SL inactivity timers associated with unicast or groupcast (if supported) is running.

  - As a baseline, agreements 7-13 inclusive are applied to SL inactivity timer for groupcast, with the difference that “src/dest L2 ID pair” is replaced with “groupcast L2 destination ID or src/dest L2 id pair” (dependent on the conclusion of proposal 17).  Any specific handling which may be needed for synchronization of inactivity timers for the groupcast case is FFS.

  - SL HARQ RTT timer and SL HARQ retransmission timer are maintained per SL HARQ process at the RX UE.

  - Working assumption: SL HARQ RTT timer can be derived from the retransmission resource timing when the SCI indicates a retransmission resource. FFS whether explicitly configured SL HARQ RTT timer may be still required. If big problem is identified next meeting, we can revisit it.

  - The value(s) of the SL HARQ RTT Timer, when explicitly configured and not determined via SCI (if agreed to do so), is determined by UE or NW implementation.

  - For unicast, sidelink retransmission timer can be supported for at least some cases of HARQ disabled transmissions. FFS whether HARQ RTT is supported or not.

  - For transmissions with HARQ feedback, the RX UE starts the SL HARQ RTT timer in the symbol/slot following the end of PSFCH transmission.

  - If the RX UE does not transmit PSFCH for a HARQ enabled transmission (e.g. due to UL/SL prioritization) the RX UE still starts the HARQ RTT timer in the symbol/slot following the end of PSFCH resource.

  - For cases where there is some uncertainty in the timing of a retransmission for a HARQ process (e.g. due to no retransmission resource indicated in the SCI, or possible reselection by the TX UE) the RX UE uses a configured retransmission timer.

  - Retransmission timer can be started upon expiry of the HARQ RTT timer.

  - The value(s) of the SL retransmission timer can be determined by UE or NW implementation.

  - The SL active time of the RX UE includes the time in which any of its applicable sl-drx-OnDuration(s), sl-DRXInactivityTimer(s), or sl-drx-RetransmissionTimer(s) are running.

  - Working assumption: The slots when the UE is expected CSI report following a CSI request is considered as SL active time.

  - RAN2 assumes LCP enhancements for ensuring a TX UE transmits data in the active time of an RX UE are needed. FFS on the resource (re)selection enhancements (e.g. limiting the resources to the active time for peer UE).

- Agreement on alignment between Uu DRX and SL DRX

  - Alignment of Uu DRX and SL DRX for unicast is supported. FFS on how alignment is achieved.

  - Alignment of Uu DRX and SL DRX for groupcast and broadcast is supported. FFS on whether new mechanisms are needed.

  - Alignment of Uu DRX and SL DRX for UE in RRC CONNECTED shall be a baseline.

  - The alignment of Uu DRX and SL DRX of the same UE shall be considered.

RAN2#114-e

Regarding sidelink DRX, the following agreements were made:

- Agreement on details of SL DRX configuration for SL unicast

  - In SL unicast, for DRX configuration of each direction where one UE as Tx-UE and the other UE as Rx-UE, support signalling exchange including both 1) Signaling-1: signalling from RX-UE to TX-UE, and 2) Signaling-2: signalling from TX-UE to RX-UE.

  - For SL unicast, Tx-UE centric DRX configuration based on the assistance information from Rx-UE is agreed as baseline.

  - In SL unicast, for DRX configuration of each direction where one UE as Tx-UE and the other as Rx-UE, signaling-1 (Rx->Tx) is carried via a new PC5-RRC message, from Rx-UE to Tx-UE.

  - In SL unicast, for DRX configuration of the direction where one UE as Tx-UE and the other as Rx-UE, signaling-2 (Tx->Rx) is carried via RRCReconfigurationSidelink, to deliver DRX configuration from Tx-UE to Rx-UE.

  - In SL unicast, for DRX configuration of each direction where one UE as Tx-UE and the other UE as Rx-UE, when Tx-UE is in-coverage and in RRC_CONNECTED state, Tx-UE may report the information received in signaling-1 (Rx->Tx) to the serving network.

  - In SL unicast, for DRX configuration of each direction where one UE as Tx-UE and the other as Rx-UE, when Tx-UE is in-coverage and in RRC_CONNECTED state, Tx-UE may obtain DRX configuration from dedicated RRC to generate signalling-2 (Tx->Rx).

  - In SL unicast, for DRX configuration of each direction where one UE as Tx-UE and the other as Rx-UE, when Rx-UE is in-coverage and in RRC_CONNECTED state, Rx-UE report the DRX configuration received in signalling-2 (Tx->Rx) to the serving network.

- Agreement on details of SL-specific DRX timer in Uu

  - SL-specific drx-onDurationTimer is not introduced in Uu.

  - SL-specific drx-InactivityTimer is not introduced in Uu.

  - For Tx UE configured with sidelink resource allocation mode 1, it should start or restart the Uu drx-InactivityTimer if the UE receives a PDCCH indicating a new SL transmission.

  - SL-specific drx-HARQ-RTT-Timer and SL-specific drx-RetransmissionTimer should be introduced in Uu, which are maintained based on sidelink process.

  - When sl-PUCCH-Config is configured, SL-specific drx-HARQ-RTT-Timer and SL-specific drx-RetransmissionTimer should be maintained for UE configured with sidelink resource allocation mode 1.

  - Adopt the following definitions of SL-specific drx-HARQ-RTT-Timer and drx-RetransmissionTimer (the detailed name of the timers can be further discussed):

    - drx-RetransmissionTimerSL (per Sidelink process): the maximum duration until a grant for SL retransmission is received;

    - drx-HARQ-RTT-TimerSL (per Sidelink process): the minimum duration before a SL retransmission grant is expected by the MAC entity.

  - When sl-PUCCH-Config is configured (and the PUCCH is transmitted), the UE should start the SL-specific drx-HARQ-RTT-Timer in Uu for the corresponding SL HARQ process in the first slot after the end of the corresponding transmission carrying the SL HARQ feedback via the PUCCH.

- Agreement on details of SL DRX configuration for SL groupcast/broadcast

  - WA: RAN2 assumes that the V2X layer of Rx UE passes the PC5 QoS parameters together with the corresponding destination layer-2 ID(s) for reception to the AS layer, as per TR 23.776 conclusion, and will further discuss SL DRX design based on this working assumption. RAN2 does not need to send LS to SA2 to clarify this issue.

  - For GC/BC, DRX cycle should take at least QoS requirement into consideration.

  - For GC/BC, DRX cycle(s) is configured per QoS profile. FFS on the need of down-select one DRX cycle from available DRX cycles for a specific L2 DST ID if UE has multiple QoS profiles for same DST L2 ID.

  - For GC/BC, DRX cycle is configured per QoS profile.

  - For GC/BC, RAN2 understands that sl-drx-startoffset does not take QoS requirement into consideration.

  - For GC/BC, For GC/BC, sl-drx-startoffset is set based on DST L2 ID.

- Agreement on alignment between Uu DRX and SL DRX

  - Alignment of Uu DRX and SL DRX for UE may comprise the full overlapping between Uu DRX and SL DRX in time.

  - Alignment of Uu DRX and SL DRX for UE may comprise the partial overlapping between Uu DRX and SL DRX in time.

  - For at least SL RX-UEs in RRC CONNECTED, the alignment of Uu DRX and SL DRX is up to gNB. FFS for SL TX-UE.

  - RAN2 to down-scope alignment of Uu DRX and SL DRX for UEs in RRC IDLE and RRC INACTIVE from Rel-17.

  - In case of Mode 1 scheduling, the alignment of Uu DRX of Tx UE and SL DRX of Rx UE shall be considered. FFS on how alignment is achieved.

- Agreement on geolocation based SL DRX operation

  - Geolocation based SL DRX is not supported in Rel-17.

2.2.2	Remaining Open issues

The followings are the remaining open issues:

- Protocol layer aspects on resource allocation to reduce UE’s power consumption

- Protocol layer aspects on sidelink DRX for broadcast, groupcast, and unicast including;

  - Details of timer and configuration of sidelink DRX

  - Details of mechanism aiming to align sidelink DRX wake-up time among the UEs communicating with each other

  - Details of mechanism aiming to align sidelink DRX wake-up time with Uu DRX wake-up time in an in-coverage UE

- Protocol layer aspects on solution(s) on the enhancement(s) in mode 2 for enhanced reliability and reduced latency

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

2.4.2	Remaining Open issues

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SA2

3.1.0	SA2 eV2XARC_Ph2 status – general

SA2 has progressed normative work on eV2XARC_Ph2 (Architecture enhancements for 3GPP support of advanced V2X services – Phase 2) to specify support of QoS aware NR PC5 power efficiency for pedestrian UEs in TS 23.287 based on the conclusions defined in clause 7.2 of TR 23.776.

The completion level of eV2XARC_Ph2 is 70%.

3.1.1	Agreements with cross-TSG impacts

The three CRs to TS 23.287 were approved at SA2#145E (17 – 28 May, 2021): S2-2105136, S2-2105135, S2-2103932

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

Among the approved CRs to TS 23.287, S2-2105135 includes the following Editor's notes that have RAN2 dependencies.

Editor's note:	The PC5 DRX configuration will be determined by RAN WGs. Its detail (e.g. mapping information for PC5 DRX parameters) and the reference to RAN specification will be added when defined in RAN WGs.

Editor's note:	For unicast when the UE is "not served by E-UTRA" and "not served by NR", whether the provisioned PC5 DRX configuration is needed is pending on RAN WG2 decision.

Editor's note:	The reference to RAN specification and information taken into account in the AS layer will be finalized when the PC5 DRX operations are defined in RAN WGs.

The exception sheet to shift completion date to SA#93 (September 2021) was also approved at SA2#145E: S2-2103934, including the following tasks within work which are not complete:

- Details of PC5 DRX configuration (e.g. mapping information for PC5 DRX parameters) by coordinating with RAN2.

- For unicast when the UE is "not served by E-UTRA" and "not served by NR", whether the provisioned PC5 DRX configuration is needed is pending on RAN2 decision.

- Whether the V2X layer exposes transmission schedule information to the V2X application layer and what is the content for the transmission schedule information based on the applied PC5 DRX information/parameters provided by the AS layer to the V2X layer to be defined in RAN2.

Further normative work to resolve the Editor's notes and the uncompleted task is expected in SA2#146E (16 – 27 August 2021) e.g., based on the RAN2 agreements on sidelink DRX made at RAN2#114-e (19 – 27 May 2021).

Related to the open issues with RAN2 dependencies described above, the issue(s) not yet agreed/finalized in RAN2 (e.g. the second bullet in the exception sheet) requires early decisions during RAN2#115-e for the successful completion of SA2 normative work on eV2XARC_Ph2 in Q3 because SA2#146E and RAN2#115-e have fully overlapping dates.

3.2	CT WGs

3.2.0	CT WGs eV2XARC_Ph2 status – general

To progress stage-3 normative work on PC5 DRX operation as per stage-2 requirements, new WID on CT aspects of Architecture enhancements for 3GPP support of advanced V2X services – Phase 2 (eV2XARC_Ph2) was agreed in CT1#130-e (20 – 28 May, 2021) - C1-213662.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template

RAN1#104bis-e

- R1-2102323	Sidelink resource allocation to reduce power consumption	Huawei, HiSilicon

- R1-2102324	Inter-UE coordination in sidelink resource allocation	Huawei, HiSilicon

- R1-2102361	Sidelink resource allocation for power saving	Nokia, Nokia Shanghai Bell

- R1-2102362	Inter-UE coordination in mode 2 sidelink resource allocation	Nokia, Nokia Shanghai Bell

- R1-2102411	Power saving mechanism in NR sidelink	OPPO

- R1-2102412	Inter-UE coordination in mode 2 of NR sidelink	OPPO

- R1-2102413	Wake up signal  for NR sidelink	OPPO

- R1-2102467	Discussion on sidelink resource allocation for power saving	Spreadtrum Communications

- R1-2102468	Discussion on inter-UE coordination in sidelink resource allocation	Spreadtrum Communications

- R1-2102539	Resource allocation for sidelink power saving	vivo

- R1-2102540	Discussion on mode-2 enhancements	vivo

- R1-2102541	Other aspects on SL enhancements	vivo

- R1-2102575	Considerations on partial sensing in NR V2X	CAICT

- R1-2102576	Considerations on mode 2 enhancements	CAICT

- R1-2102606	Discussion on resource allocation for power saving	CATT, GOHIGH

- R1-2102607	Discussion on inter-UE coordination in mode 2 enhancement	CATT, GOHIGH

- R1-2102608	Considerations on other aspects of NR mode2 enhancements	CATT, GOHIGH

- R1-2102690	Discussion on Mode 2 enhancements	MediaTek Inc.

- R1-2102708	Discussion on sidelink power saving	MediaTek Inc.

- R1-2102719	Considerations on partial sensing and DRX in NR sidelink	Fujitsu

- R1-2102720	Considerations on inter-UE coordination for mode 2 enhancements	Fujitsu

- R1-2102780	Power consumption reduction for sidelink resource allocation	FUTUREWEI

- R1-2102781	Discussion on techniques for inter-UE coordination	FUTUREWEI

- R1-2102797	Discussion on resource allocation for power saving	Zhejiang Lab

- R1-2102798	Inter-UE coordination for mode 2 enhancements	Zhejiang Lab

- R1-2102811	NR Sidelink Resource Allocation for UE Power Saving	Fraunhofer HHI, Fraunhofer IIS

- R1-2102812	Resource Allocation Enhancements for Mode 2	Fraunhofer HHI, Fraunhofer IIS

- R1-2102826	Inter-UE coordination for enhanced resource allocation	Mitsubishi Electric RCE

- R1-2102897	Discussion on resource allocation for power saving	CMCC

- R1-2102898	Discussion on enhancements for mode-2 resource allocation	CMCC

- R1-2102921	Discussion on the inter-UE coordination	ZTE

- R1-2102965	Discussion on sidelink resource allocation enhancement for power saving	Xiaomi

- R1-2102966	Discussion on inter-UE coordination	Xiaomi

- R1-2102967	Discssion on other design aspects for sidelink enhancement	Xiaomi

- R1-2103048	Sidelink power saving solutions	Intel Corporation

- R1-2103049	Inter-UE coordination solutions for sidelink resource allocation mode-2	Intel Corporation

- R1-2103121	Discussion on Sidelink Resource Allocation for Power Saving	Apple

- R1-2103122	Discussion on Inter-UE Coordination	Apple

- R1-2103123	Network Assisted Resource Selection	Apple

- R1-2103184	Power Savings for Sidelink	Qualcomm Incorporated

- R1-2103185	Reliability and Latency Enhancements for Mode 2	Qualcomm Incorporated

- R1-2103256	On Resource Allocation Enhacements	Samsung

- R1-2103257	On Resource Allocation for Power Saving	Samsung

- R1-2103258	On Inter-UE Coordination for Mode2 Enhancements	Samsung

- R1-2103259	On Sidelink Issues and RAN1 Impacts	Samsung

- R1-2103271	Inter-UE coordination for mode 2 enhancement	ITL

- R1-2103272	Discussion on Sidelink Resource Allocation for Power Saving	Panasonic Corporation

- R1-2103314	Discussion on sidelink resource allocation for power saving	Sony

- R1-2103315	Discussion on reliability and latency enhancements for mode 2	Sony

- R1-2103331	Discussion on resource allocation for power saving	ETRI

- R1-2103332	Discussion on mode 2 enhancements	ETRI

- R1-2103378	Discussion on resource allocation for power saving	LG Electronics

- R1-2103379	Discussion on inter-UE coordination for Mode 2 enhancements	LG Electronics

- R1-2103392	Physical layer impacts of sidelink DRX	Huawei, HiSilicon

- R1-2103416	On Resource Allocation for Power Saving in NR Sidelink	Convida Wireless

- R1-2103417	On Inter-UE Coordination for Mode 2 Enhancements	Convida Wireless

- R1-2103483	Discussion on resource allocation for power saving	Sharp

- R1-2103484	Discussion on inter-UE coordination for Mode 2 enhancements	Sharp

- R1-2103517	Discussion on resource allocation for power saving	NEC

- R1-2103518	Discussion on mode 2 enhancements	NEC

- R1-2103537	Sidelink resource allocation for power saving	InterDigital, Inc.

- R1-2103538	On Inter-UE coordination for Mode 2 enhancement	InterDigital, Inc.

- R1-2103539	On gNB-designated resources for inter-UE coordination	InterDigital, Inc.

- R1-2103548	Sidelink resource allocation for power saving	Lenovo, Motorola Mobility

- R1-2103549	Discussion on inter-UE coordination for Mode 2 enhancements	Lenovo, Motorola Mobility

- R1-2103592	Discussion on sidelink resource allocation for power saving	NTT DOCOMO, INC.

- R1-2103593	Resource allocation for reliability and latency enhancements	NTT DOCOMO, INC.

- R1-2103605	Inter-UE coordination for Mode 2 enhancements	Panasonic Corporation

- R1-2103635	Discussion on resource allocation for power saving	Hyundai Motors

- R1-2103636	Discussion on mode 2 enhancements	Hyundai Motors

- R1-2103640	Discussion on partial sensing and SL DRX impact	ASUSTeK

- R1-2103648	Discussion on V2X mode 2 enhancements	ASUSTeK

- R1-2103663	Resource allocation for power saving with partial sensing in NR sidelink enhancement	ITL

- R1-2103704	Resource allocation procedures for power saving	Ericsson

- R1-2103705	Mode 2 enhancements using Inter-UE coordination	Ericsson

- R1-2103706	Additional considerations for resource allocation procedures	Ericsson

- R1-2103710	Discussion on resource allocation for power saving	ZTE, Sanechips

- R1-2103711	Discussion on remaining issues for sidelink evaluation methodology	ZTE, Sanechips

- R1-2104090	FL summary for AI 8.11.1.1 – resource allocation for power saving (1st check point)	Moderator (OPPO)

- R1-2104091	FL summary for AI 8.11.1.1 – resource allocation for power saving (2nd check point)	Moderator (OPPO)

- R1-2104092	FL summary for AI 8.11.1.1 – resource allocation for power saving (3rd check point)	Moderator (OPPO)

- R1-2104093	FL summary for AI 8.11.1.1 – resource allocation for power saving (final)	Moderator (OPPO)

- R1-2104103	Feature lead summary for AI 8.11.1.2 Inter-UE coordination for Mode 2 enhancements	Moderator (LG Electronics)

- R1-2104148	Moderator summary of Email discussion/approval to reply LS in R1-2100021	Moderator (ZTE)

RAN1#105-e

- R1-2104176	Sidelink resource allocation for power saving	Nokia, Nokia Shanghai Bell

- R1-2104177	Inter-UE coordination in mode 2 sidelink resource allocation	Nokia, Nokia Shanghai Bell

- R1-2104192	Power consumption reduction for sidelink resource allocation	FUTUREWEI

- R1-2104193	Discussion on techniques for inter-UE coordination	FUTUREWEI

- R1-2104236	Sidelink resource allocation to reduce power consumption	Huawei, HiSilicon

- R1-2104237	Inter-UE coordination in sidelink resource allocation	Huawei, HiSilicon

- R1-2104385	Resource allocation for sidelink power saving	vivo

- R1-2104386	Discussion on mode-2 enhancements	vivo

- R1-2104440	Discussion on sidelink resource allocation for power saving	Spreadtrum Communications

- R1-2104441	Discussion on inter-UE coordination in sidelink resource allocation	Spreadtrum Communications

- R1-2104457	Inter-UE Coordination for Mode 2 Enhancements	Kyocera Corporation

- R1-2104489	Discussion on resource allocation for power saving	CATT, GOHIGH

- R1-2104490	Discussion on inter-UE coordination in mode 2 enhancement	CATT, GOHIGH

- R1-2104560	NR Sidelink Resource Allocation for UE Power Saving	Fraunhofer HHI, Fraunhofer IIS

- R1-2104561	Resource Allocation Enhancements for Mode 2	Fraunhofer HHI, Fraunhofer IIS

- R1-2104630	Discussion on resource allocation for power saving	CMCC

- R1-2104631	Discussoin on reliability and latency enhancements for mode-2 resource allocation	CMCC

- R1-2104693	Power Savings for Sidelink	Qualcomm Incorporated

- R1-2104694	Reliability and Latency Enhancements for Mode 2	Qualcomm Incorporated

- R1-2104706	Discussion on resource allocation for power saving	Zhejiang Lab

- R1-2104707	Inter-UE coordination schemes in mode 2	Zhejiang Lab

- R1-2104724	Considerations on partial sensing in NR V2X	CAICT

- R1-2104725	Considerations on mode 2 enhancements	CAICT

- R1-2104755	Power saving mechanisms in NR sidelink	OPPO

- R1-2104756	Inter-UE coordination in mode 2 of NR sidelink	OPPO

- R1-2104869	Sidelink resource allocation for power saving	Lenovo, Motorola Mobility

- R1-2104870	Discussion on inter-UE coordination for Mode 2 enhancements	Lenovo, Motorola Mobility

- R1-2104926	Sidelink Power Saving Schemes	Intel Corporation

- R1-2104927	Inter-UE Coordination Schemes for Sidelink Communication	Intel Corporation

- R1-2105066	Considerations on partial sensing and DRX in NR Sidelink	Fujitsu

- R1-2105067	Considerations on inter-UE coordination for mode 2 enhancements	Fujitsu

- R1-2105070	Discussion on Sidelink Resource Allocation for Power Saving	Panasonic Corporation

- R1-2105126	On Sidelink Resource Allocation for Power Saving	Apple

- R1-2105127	On Inter-UE Coordination	Apple

- R1-2105177	Discussion on sidelink resource allocation for power saving	Sony

- R1-2105178	Discussion on inter-UE coordination for Mode 2 enhancements	Sony

- R1-2105200	Discussion on the inter-UE coordination	ZTE

- R1-2105203	Consolidation of agreements on sidelink evaluation methodology update for power saving	LG Electronics

- R1-2105204	Discussion on resource allocation for power saving	LG Electronics

- R1-2105205	Discussion on inter-UE coordination for Mode 2 enhancements	LG Electronics

- R1-2105228	Discussion on resource allocation for power saving	ETRI

- R1-2105229	Discussion on inter-UE coordination for Mode 2 enhancements	ETRI

- R1-2105253	Discussion on resource allocation for power saving	NEC

- R1-2105254	Discussion on mode 2 enhancements	NEC

- R1-2105270	Inter-UE coordination for enhanced resource allocation	Mitsubishi Electric RCE

- R1-2105333	On Resource Allocation Enhacements	Samsung

- R1-2105334	On Resource Allocation for Power Saving	Samsung

- R1-2105335	On Inter-UE Coordination for Mode2 Enhancements	Samsung

- R1-2105380	Discussion on sidelink power saving	MediaTek Inc.

- R1-2105393	Discussion on Mode 2 enhancements	MediaTek Inc.

- R1-2105544	Discussion on sidelink resource allocation enhancement for power saving	Xiaomi

- R1-2105545	Discussion on inter-UE coordination	Xiaomi

- R1-2105598	NR SL Resource Allocation for Power Saving	Convida Wireless

- R1-2105599	NR SL Inter-UE Coordination for Mode 2 Enhancements	Convida Wireless

- R1-2105614	Discussion on resource allocation for power saving	ZTE, Sanechips

- R1-2105615	Discussion on resource allocation for power saving	Hyundai Motors

- R1-2105616	Discussion on inter-UE coordination for Mode 2 enhancements	Hyundai Motors

- R1-2105645	Discussion on resource allocation for power saving	Sharp

- R1-2105646	Discussion on inter-UE coordination for Mode 2 enhancements	Sharp

- R1-2105650	Inter-UE coordination for Mode 2 enhancements	Panasonic Corporation

- R1-2105651	Resource allocation for power saving with partial sensing in NR sidelink enhancement	ITL

- R1-2105659	Inter-UE coordination for mode 2 enhancements	ITL

- R1-2105674	Sidelink resource allocation for power saving	InterDigital, Inc.

- R1-2105675	On inter-UE coordination for Mode 2 enhancement	InterDigital, Inc.

- R1-2105718	Discussion on sidelink resource allocation for power saving	NTT DOCOMO, INC.

- R1-2105719	Resource allocation for reliability and latency enhancements	NTT DOCOMO, INC.

- R1-2105845	Discussion on partial sensing and SL DRX impact	ASUSTeK

- R1-2105848	Discussion on V2X mode 2 enhancements	ASUSTeK

- R1-2105866	Further discussion on power saving for sidelink	ROBERT BOSCH GmbH

- R1-2105881	Discussion on inter-UE coordination for sidelink mode-2	ROBERT BOSCH GmbH

- R1-2105893	Resource allocation procedures for power saving	Ericsson

- R1-2105894	Feasibility and benefits of mode 2 enhancements for inter-UE coordination	Ericsson

- R1-2105982	Reliability and Latency Enhancements for Mode 2	Qualcomm Incorporated

- R1-2106030	FL summary for AI 8.11.1.1 – resource allocation for power saving (1st check point)	Moderator (OPPO)

- R1-2106031	FL summary for AI 8.11.1.1 – resource allocation for power saving (2nd check point)	Moderator (OPPO)

- R1-2106032	FL summary for AI 8.11.1.1 – resource allocation for power saving (final check point)	Moderator (OPPO)

- R1-2106033	FL summary for AI 8.11.1.1 – resource allocation for power saving (final EOM)	Moderator (OPPO)

- R1-2106062	Feature lead summary for AI 8.11.1.2 Inter-UE coordination for Mode 2 enhancements	Moderator (LG Electronics)

- R1-2106067	Resource allocation for sidelink power saving	vivo

- R1-2106098	Discussion on resource allocation for power saving	LG Electronics

- R1-2106122	Discussion on resource allocation for power saving	ZTE, Sanechips

- R1-2106135	Moderator Summary #1 of email discussion or approval to reply LS in R1-2100021	Moderator (ZTE)

- R1-2106188	Feature lead summary#2 for AI 8.11.1.2 Inter-UE coordination for Mode 2 enhancements	Moderator (LG Electronics)

- R1-2106200	Discussion on mode-2 enhancements	vivo

- R1-2106284	Feature lead summary#3 for AI 8.11.1.2 Inter-UE coordination for Mode 2 enhancements	Moderator (LG Electronics)

- R1-2106338	Feature lead summary#4 for AI 8.11.1.2 Inter-UE coordination for Mode 2 enhancements	Moderator (LG Electronics)

RAN2#113bis-e

- R2-2102660	Reply LS on geo-area confinement (S2-2101319; contact: LGE)	SA2

- R2-2102688	DRX Design for Sidelink Unicast	CATT

- R2-2102689	Further Study on DRX for Sidelink Groupcast/Broadcast	CATT

- R2-2102690	DRX Active Time Alignment between Uu and SL	CATT

- R2-2102691	Consideration on Resource Allocation Enhancements	CATT

- R2-2102746	Discussion on inter-UE coordination	OPPO

- R2-2102771	Further discussion on Sidelink DRX	LG Electronics France

- R2-2102772	Power efficient resource allocation	LG Electronics France

- R2-2102801	Summary of [POST113-e][703][V2X/SL] Details of Timer (InterDigital)	InterDigital

- R2-2102802	Further details on SL DRX Timers	InterDigital

- R2-2102803	On TX Centric vs RX Centric DRX Configuration Determination	InterDigital

- R2-2102804	Resource Allocation for eSL	InterDigital

- R2-2102805	Discussion on Uu DRX for SL UE	InterDigital

- R2-2102815	SL DRX Configuration Impact on RAN1 and RAN2	vivo

- R2-2102816	SL DRX for Unicast	vivo

- R2-2102817	SL DRX for Groupcast and Broadcast	vivo

- R2-2102818	Discussion on inter-UE coordination for sidelink mode2	vivo

- R2-2102848	Discussion on SL DRX impact on SL resource allocation mode 1	Sharp

- R2-2102886	Discussion on DRX configuration	OPPO

- R2-2102887	Discussion on network involvement for SL related DRX	OPPO

- R2-2102888	Left issues on DRX mechanisms and granularity	OPPO

- R2-2102889	Summary of [POST113-e][704] TX UE centric or RX UE centric DRX configuration determination (OPPO)	OPPO

- R2-2102970	Resource allocation enhancement impact in RAN2	Xiaomi communications

- R2-2102971	Discussion on sidelink DRX timer handling	Xiaomi communications

- R2-2102972	DRX coordination between Uu and Sidelink	Xiaomi communications

- R2-2102973	DRX coordination between TX and RX UE	Xiaomi communications

- R2-2102979	Discussion on  Coordination between Uu DRX and SL DRX	ZTE Corporation, Sanechips

- R2-2102980	Discussion on sidelink DRX configuration for unicast	ZTE Corporation, Sanechips

- R2-2102981	Discussion on sidelink DRX configuration for groupcast and broadcast	ZTE Corporation, Sanechips

- R2-2102982	Discussion on inter-UE coordination	ZTE Corporation, Sanechips

- R2-2103003	General aspects of SL DRX	Ericsson,Qualcomm Incorporated

- R2-2103004	Alignment between SL DRX and Uu DRX	Ericsson,Qualcomm Incorporated

- R2-2103005	Interaction between partial sensing and DRX	Ericsson

- R2-2103011	NR SL DRX	Fraunhofer IIS, Fraunhofer HHI

- R2-2103040	Power Reduction for Sidelink Mode 2 Resource Allocation	Fraunhofer IIS, Fraunhofer HHI

- R2-2103041	Inter-UE Coordination for Sidelink Mode 2 Resource Allocation	Fraunhofer IIS, Fraunhofer HHI

- R2-2103068	On general SL DRX design	Intel Corporation

- R2-2103069	Discussion on SL DRX timers	Intel Corporation

- R2-2103070	On DRX wake-up time alignment	Intel Corporation

- R2-2103173	On resource allocation enhancement in Rel-17 NR eSL	Huawei, HiSilicon

- R2-2103174	Consideration on sidelink DRX for broadcast and groupcast	Huawei, HiSilicon

- R2-2103234	Discussion on HARQ RTT and Retransmission Timers for SL Unicast	Spreadtrum Communications

- R2-2103238	Discussion on resource allocation enhancement for NR sidelink	Spreadtrum Communications

- R2-2103287	Discussion on HARQ RTT and Retransmission Timer for SL DRX	Fujitsu

- R2-2103288	Alignment of sidelink DRX active time	Fujitsu

- R2-2103289	Dual-mode Configuration and Selection for NR Sidelink	Fujitsu

- R2-2103305	On the deciding entity of SL DRX configuration	Nokia, Nokia Shanghai Bell

- R2-2103306	Backward Compatibility Issue of SL DRX with Rel.16 Sidelink	Nokia, Nokia Shanghai Bell

- R2-2103400	Discussion on sidelink resource allocation enhancements	Lenovo, Motorola Mobility

- R2-2103401	SL DRX configuration for unicast	Lenovo, Motorola Mobility

- R2-2103462	Discussion on SL DRX active time for groupcast and broadcast	ASUSTeK

- R2-2103463	Discussion on MAC impact regarding Sidelink DRX	ASUSTeK

- R2-2103468	Geolocation for Sidelink DRX	Nokia, Nokia Shanghai Bell, Fujitsu, Fraunhofer IIS, Fraunhofer HHI

- R2-2103470	Coordination between Uu DRX and SL DRX	Lenovo, Motorola Mobility

- R2-2103478	SL DRX Timers	Nokia, Nokia Shanghai Bell

- R2-2103576	On detailed SL DRX model	MediaTek Inc.

- R2-2103577	On coordination between Uu DRX and SL DRX	MediaTek Inc.

- R2-2103578	Transmission of assistance information for Mode 2 enhancement	MediaTek Inc.

- R2-2103579	On SL sync search optimization	MediaTek Inc.

- R2-2103615	Discussion on Sidelink DRX	Sony Europe B.V.

- R2-2103617	Discusison on Sidelink sensing	Sony Europe B.V.

- R2-2103664	General principles for resource allocation enhancements for SL mode 2	Ericsson

- R2-2103736	Resource Allocation Enhancements	Intel Corporation

- R2-2103741	DRX Configuration for Broadcast and Groupcast SL communication	Lenovo, Motorola Mobility

- R2-2103778	Discussion on Directional SL DRX for Unicast	Qualcomm Finland RFFE Oy

- R2-2103779	Discussion on SL DRX configuration for Groupcast & Broadcast	Qualcomm Finland RFFE Oy

- R2-2103780	Discussion on SL DRX Timers and Others	Qualcomm Finland RFFE Oy

- R2-2103852	Discussion on remaining issues on SL DRX	Apple

- R2-2103853	Discussion on RX-centric and Tx-centric in SL unicast DRX	Apple, InterDigital Inc.

- R2-2103854	Discussion on Inter-UE Coordination	Apple

- R2-2103855	Discussion on resource allocation for Pedestrian UE	Apple

- R2-2103889	Coordination between DL DRX and SL DRX	Samsung

- R2-2103891	SL DRX operation for groupcast/broadcast	Samsung

- R2-2103892	Transmission UE behaviours for SL DRX	Samsung

- R2-2103894	Rel-16 SCI information related to active time in SL DRX	Samsung

- R2-2103948	On Resource Allocation Mode 2 Enhancement for NR Sidelink	Convida Wireless

- R2-2103952	SL DRX Granularity Considerations	Convida Wireless

- R2-2103988	Resource allocation enhancements	Samsung

- R2-2104083	Remaining issues in which UE decides sidelink DRX configurations	LGE, InterDigital, Huawei, ASUSTeK, Apple

- R2-2104085	Inter-UE coordination for NR V2X	LG Electronics Inc.

- R2-2104113	Discussion on SL communication impact on Uu DRX	Huawei, HiSilicon

- R2-2104114	Consideration on the sidelink DRX for unicast	Huawei, HiSilicon

- R2-2104256	Consideration on sidelink DRX determination	LG Electronics Inc.

- R2-2104266	SL DRX enabled UE Mode 2 operation	ITL

- R2-2104285	Discussion on SL DRX configuration for Groupcast & Broadcast	Qualcomm Finland RFFE Oy, Ericsson

- R2-2104472	Summary of [706]	Ericsson

- R2-2104473	[AT113bis-e][707][V2X/SL] Uu DRX impact to support SL	CATT

- R2-2104474	[AT113bis-e][708][V2X/SL] DRX configuration for SL groupcast and broadcast	ZTE

RAN2#114-e

- R2-2104750	Leftover Issues on DRX for Sidelink Unicast	CATT

- R2-2104751	DRX Design for Sidelink Groupcast and Broadcast	CATT

- R2-2104752	[AT113bis-e][707][V2X/SL] Uu DRX Impact to Support SL	CATT

- R2-2104753	Impacts of SL DRX on Other Procedures	CATT

- R2-2104769	Discussion on network involvement for SL related DRX	OPPO

- R2-2104835	Discussion on DRX configuration and DRX timers	OPPO

- R2-2104836	Left issues on SL DRX RTT timer	OPPO, Intel, Xiaomi communications

- R2-2104841	Summary of [POST113-e][704] TX UE centric or RX UE centric DRX configuration determination (OPPO)	OPPO

- R2-2104865	Reviised Summary of [POST113-e][703][V2X/SL] Details of Timer (InterDigital)	InterDigital

- R2-2104866	Open Issues on SL DRX	InterDigital

- R2-2104867	On TX Centric vs RX Centric Approaches for DRX Configuration Determination	InterDigital, Apple, Huawei

- R2-2104868	Resource Allocation for eSL	InterDigital

- R2-2105023	Further discussion on SL DRX operation	Intel Corporation

- R2-2105024	On DRX wake-up time alignment	Intel Corporation

- R2-2105073	DRX Configuration for UC BC GC and its interaction with Sensing	Lenovo, Motorola Mobility

- R2-2105077	Discussion on  SL DRX configuration	ZTE Corporation, Sanechips

- R2-2105078	Discussion on  SL DRX  timer	ZTE Corporation, Sanechips

- R2-2105079	Discussion on inter-UE coordination	ZTE Corporation, Sanechips

- R2-2105083	Consideration on the sidelink DRX for unicast	Huawei, HiSilicon

- R2-2105131	Discussion on RX-centric and Tx-centric in SL unicast DRX	Apple, InterDigtal Inc.

- R2-2105132	Discussion on remaining issues of SL DRX	Apple

- R2-2105133	Discussion on resource allocation enhacenmens	Apple

- R2-2105248	NR SL DRX	Fraunhofer IIS, Fraunhofer HHI

- R2-2105277	Discussion on co-existence with UEs not supporting SL DRX	SHARP Corporation

- R2-2105278	Discussion on SL DRX inactivity timer	SHARP Corporation

- R2-2105297	Further discussion on Sidelink DRX	LG Electronics France

- R2-2105351	SL DRX Configuration Impact on RAN1 and RAN2	vivo

- R2-2105352	Left issues on SL DRX	vivo

- R2-2105353	Discussion on inter-UE coordination for sidelink mode2	vivo

- R2-2105385	Discussion on active time regarding Sidelink DRX	ASUSTeK

- R2-2105400	Discussion on HARQ RTT and Retransmission Timer for SL DRX	Fujitsu

- R2-2105401	Alignment of sidelink DRX active time	Fujitsu

- R2-2105402	Dual-mode Configuration and Selection for NR Sidelink	Fujitsu

- R2-2105458	Coordination between Uu DRX and SL DRX	Lenovo, Motorola Mobility

- R2-2105467	Power efficient resource allocation and Inter-UE coordination	LG Electronics France

- R2-2105480	Discussion on sidelink DRX configuration	Xiaomi communications

- R2-2105484	DRX alignment between TX and RX UE	Xiaomi communications

- R2-2105485	Resource allocation enhancement impact in RAN2	Xiaomi communications

- R2-2105493	Remaining aspects of SL DRX	Ericsson

- R2-2105494	Interaction between partial sensing and DRX	Ericsson

- R2-2105495	summary offline 706	Ericsson

- R2-2105499	Inter-UE Coordination for Sidelink Mode 2 Resource Allocation	Fraunhofer IIS, Fraunhofer HHI

- R2-2105508	Power Reduction for Sidelink Mode 2 Resource Allocation	Fraunhofer IIS, Fraunhofer HHI

- R2-2105532	Remaining issues on DRX Timers for SL	Spreadtrum Communications

- R2-2105538	Discussion on resource allocation enhancement for NR sidelink	Spreadtrum Communications

- R2-2105553	Consideration on sidelink DRX for broadcast and groupcast	Huawei, HiSilicon

- R2-2105593	Discussion on SL communication impact on Uu DRX	Huawei, HiSilicon

- R2-2105707	Proposals for Sidelink DRX	Sony

- R2-2105708	Discusison on Sidelink sensing	Sony

- R2-2105733	Geolocation for Sidelink DRX	Nokia, Nokia Shanghai Bell, Fujitsu, Fraunhofer IIS, Fraunhofer HHI

- R2-2105775	General principles for resource allocation enhacements for SL mode 2	Ericsson

- R2-2105824	Discussion on sidelink resource allocation enhancements	Lenovo, Motorola Mobility

- R2-2105902	Discussion on Directional SL DRX for Unicast	Qualcomm Finland RFFE Oy

- R2-2105904	Discussion on SL DRX configuration for Groupcast & Broadcast	Qualcomm Finland RFFE Oy

- R2-2105906	Discussion on SL DRX Timers and Others	Qualcomm Finland RFFE Oy

- R2-2105912	[AT113bis-e][708][V2X/SL] DRX configuration for SL CG and BG	ZTE

- R2-2105958	Further Issues on Sidelink Traffic Pattern for SL DRX Configuration	Nokia, Nokia Shanghai Bell

- R2-2106056	On the deciding entity of SL DRX configuration	Nokia, Nokia Shanghai Bell

- R2-2106067	Resource Allocation Enhancements for Reduced Power Consumption and Enhanced Reliability	Intel Corporation

- R2-2106073	Coordination between DL DRX and SL DRX	Samsung Research America

- R2-2106074	SL DRX operation for groupcast/broadcast	Samsung Research America

- R2-2106075	Resource allocation enhancements	Samsung Research America

- R2-2106172	SL DRX enabled UE Mode 2 operation	ITL

- R2-2106202	Remaining issues in which UE decides sidelink DRX configurations	LGE, InterDigital, Huawei, ASUSTeK, Apple

- R2-2106204	Consideration on SL DRX operation	LG Electronics Inc.

- R2-2106358	On Resource Allocation Mode 2 Enhancement for NR Sidelink	Convida Wireless

- R2-2106363	SL DRX Granularity Considerations	Convida Wireless

- R2-2106364	SL DRX Configuration: TX Centric or RX Centric	Convida Wireless

- R2-2106438	On detailed SL DRX model	MediaTek Inc.

- R2-2106439	On SL DRX timer operation	MediaTek Inc.

- R2-2106440	Transmission of assistance information for Mode 2 enhancement	MediaTek Inc.

- R2-2106441	On SL sync search optimization	MediaTek Inc.

- R2-2106623	[LS to RAN1 on TX-UE’s timing information]	MCC