3GPP TSG RAN WG1 #105-e	                                              																																		R1-21xxxxx

e-Meeting, May 10th – 27th, 2021

Source:	Moderator (Qualcomm)

Title:	[104b-e-NR-XR-03] Email discussion/approval on initial performance evaluation results

Agenda Item:	8.14.3

Document for:	Discussion and Decision

# Introduction

GTW2 on 5/19 discussed the next FL proposal that was made based on the observations from initial evaluation results [1-14] summarized in Section 2 and 3.

FL proposal (presented in GTW2 on 5/19): Based on the above observation, FL proposes the following. - For capacity and power evaluations, RAN1#106-e will start to collect formal results from companies that are to be captured in the TR, and are to be used to draw formal observations/conclusions to be captured in the TR. - When companies submit results for advanced features/enhancements, they also submit results for baseline feature so as to facilitate the performance comparison between baseline and advanced feature/enhancements, e.g., delay aware scheduler, HARQ/CG enhancement, power saving schemes, etc. - RAN#105-e focuses on discussion for clarification/calibration of results from companies and clarification of definition/intention of columns of the data collection format (excel sheet) that will help have more stable/converged results from RAN1#106-e. This discussion is to be done via email threads [104b-e-NR-XR-02] and [104b-e-NR-XR-03]. To this end, I am going to present a summary of initial evaluation results from companies in the kick-off email for AI 3, where companies are encouraged to ask about other companies' results, clarification on data collection excel sheet. Again, the main goal is to do calibration of results among companies.

Accordingly, FL is proposing to have “free” discussion directly in the email thread, [104b-e-NR-XR-03], where companies are invited to make any questions, clarifications, suggestions, responses, to the group or a certain company regarding initial evaluation results.  FL will present a summary of the discussion, based on which possibly make proposals.

# Observations from Initial Evaluation Results

Key observations from initial evaluation results from companies

- Limited number of companies have submitted results, especially for UL AR capacity, FR2 capacity, and UE power consumption.  This may be mainly due to limited time for simulations since traffic models and evaluation methodology had been agreed in April RAN1#104bis-e.

  - Capacity evaluation

    - For FR1

      - 4-6 companies have submitted results for DL and UL VR/CG.

      - Limited results for UL AR.

    - For FR2

      - 2-5 companies have submitted results for DL and 2-3 for UL.

  - Power evaluation

    - Limited number of companies have submitted results.

- It seems that companies have different understanding on some columns of data collection format (excel sheets), in particular for power evaluation.  It is critical to have common understanding on the format because the results in the format are to be used for comparison/calibration of results among companies and draw formal observations that are to be captured in the TR.

- Some companies have submitted results only for a certain advanced feature.  For instance, a company’s capacity results are based on a delay aware scheduling algorithm, while PF scheduler is the baseline assumption agreed by RAN1.  In this case, it is difficult to draw an observation on how much the performance can be improved by the advanced feature. So, it is recommended that when companies submit results of advanced features, they also submit results for the baseline feature so as to facilitate the performance comparison between baseline and advanced feature.

# Initial Evaluation Results

## Capacity Results: FR1

DU, VR, 30Mbps, 10ms PDB

Observation: SU: 5-6 (E/// outlier); MU: 12-13 (CATT outlier)

DU, VR, 30Mbps, 10ms PDB

Observation: SU: 2-6, MU: 5.5 – 8.4

DU, CG, 30Mbps, 15ms PDB

Observation: SU: 7 – 13  (E/// outlier);  MU: 14 – 26

InH, VR, 30Mbps, 10ms PDB

Observation: MU 9.75-15.89

InH, VR, 45Mbps, 10ms PDB

Observation: 6.54 – 8.2

InH, CG, 30Mbps, 15ms PDB

Observation: SU: 4 - 15, MU:12.8 - 13.3

Urban Macro, VR, 30Mbps, 10ms PDB

Observation: SU: 4.3, MU:5.2-9.5

Urban Macro, VR, 45Mbps, 10ms PDB

Observation: SU: 1.6-4.2, MU: 2.9-4

Urban Macro, CG, 30Mbps, 15ms PDB

Observation: SU: 6.5-9.5, MU: 8-18.7

## Capacity Results: FR2

DL, InH, 45Mbps, 10ms PDB

DU_45Mbps_10ms

InH_30Mbps_10ms

DU_30Mbps_10ms

InH_30Mbps_15ms

DU_30Mbps_15ms

InH_8Mbps_15ms

DU_8Mbps_15ms

## UE Power Consumption Results: FR1

DU, VR30

DU, VR45

DU, CG30

DU, CG8

DU, AR45

InH, VR30

InH, VR45

InH, CG30InH, AR45

Uma, VR30

## UE Power Consumption Results: FR2

DL - InH_45Mbps_10ms  - Power

# List of contributions in RAN1 #105-e

- R1-2104209	XR initial evaluations	FUTUREWEI

- R1-2104397	Initial performance evaluation results on XR	vivo

- R1-2104500	Evaluation results of XR performance	CATT

- R1-2104557	Performance results in indoor hotspot and dense urban deployments of CG and VR applications			Nokia, Nokia Shanghai Bell

- R1-2104703	Initial Evaluation Results for XR Capacity and UE Power Consumption	Qualcomm Incorporated

- R1-2104747	Evaluation results for XR evaluation	OPPO

- R1-2105963	Initial results for XR	Intel Corporation

- R1-2105136	Performance evaluation on XR	Apple

- R1-2105392	Initial Performance and Evaluation Results for XR and CG	MediaTek Inc.

- R1-2105501	Initial Performance Evaluation Results on XR	InterDigital, Inc.

- R1-2105521	Initial evaluation results for XR and Cloud Gaming	Huawei, HiSilicon

- R1-2105605	Performance Evaluation Results for XR	ZTE, Sanechips

- R1-2105664	XR Initial Performance Results	AT&T

- R1-2105831	Initial XR performance evaluation results	Ericsson