3GPP TSG-RAN WG1 Meeting #105-e	R1-2104183

e-Meeting, 19th – 27th May 2021

Agenda Item:	8.6.2

Source:	Ericsson

Title:	RAN1 aspects for RAN2-led features for RedCap

Document for:	Discussion, Decision

# 1	Introduction

In this contribution, we discuss the RAN1 aspects related to the following RAN2-led features for RedCap [1]:

- Specify definition of one RedCap UE type including capabilities for RedCap UE identification and for constraining the use of those RedCap capabilities only for RedCap UEs, and preventing RedCap UEs from using capabilities not intended for RedCap UEs including at least carrier aggregation, dual connectivity and wider bandwidths. [RAN2, RAN1] - The existing UE capability framework is used; changes to capability signalling are specified only if necessary. - Specify functionality that will enable RedCap UEs to be explicitly identifiable to networks through an early indication in Msg1 and/or Msg3, and MsgA if supported, including the ability for the early indication to be configurable by the network. [RAN2, RAN1] - Specify a system information indication to indicate whether a RedCap UE can camp on the cell/frequency or not; it shall be possible for the indication to be specific to the number of Rx branches of the UE. [RAN2, RAN1]

# 2	Definition and constraining of reduced capabilities

## 2.1	Definition of RedCap UE type

According to the WID [1], only one RedCap UE type should be specified. The purpose of introducing the RedCap UE type is threefold: to identify RedCap UEs, i.e., to differentiate them from non-RedCap UEs, to constrain the use of RedCap UE capabilities only to RedCap UEs, and to prevent RedCap UEs from using capabilities not intended for RedCap UEs. The definition of the RedCap UE type, according to the text captured in clause 10.1 of TR 38.875 [2], can be based on one of:

- Option 1: All the reduced capabilities recommended at the end of the RedCap study.

- Option 2: Only include the reduced capabilities that the network needs to know during initial access, if any.

- Option 3: All the recommended reduced capabilities as well as recommended power saving features.

- Option 4: The corresponding minimum set of the reduced capabilities that one RedCap UE type shall mandatorily support.

Option 1 can help to give clarity on the performance, for example, in terms of peak data rate, that can be expected from a certain RedCap UE. However, Option 1 will result in multiple RedCap UE types if number of DL MIMO layers/Rx branches (1 or 2) is included in the set of capabilities that defines the RedCap UE type.

For Option 2, the capabilities that define the UE type will be the UE bandwidth and the number of Rx branches. Again, this will entail multiple UE types based on the number of Rx branches supported by the UE. Furthermore, Option 2 might result in many UE capabilities and feature combinations in the market as different vendors may have different choices regarding those reduced capabilities not included in the RedCap UE type definition.

Option 3, compared to Option 1, provides further the clarity regarding the energy efficiency performance that can be expected from a RedCap UE. The recommended power saving features could include RAN2/RAN3/RAN4 features, such as extended DRX for RRC Inactive/Idle and RRM measurement relaxations for neighbouring cells, and possibly also the power saving features specified in the UE Power Saving Enhancements WI [4]. However, the requirements on energy efficiency vary very much among the different RedCap use cases. Furthermore, these power saving features are very likely to be optional for the RedCap UEs to support, and most of these will have little to no impact on UE hardware/cost. Moreover, Option 3, like Option 1 and Option 2, would result in multiple UE types.

Among the four options listed in the TR, Option 4 is the most attractive in our opinion. This option has several advantages, such as resulting in a single RedCap UE type, minimizing market fragmentation, promoting economy of scale, and reducing specification effort, e.g., on early identification and access control. If a RedCap UE has additional or advanced capabilities beyond the set of minimum capabilities, the UE can use the existing capability signalling mechanism to convey such information.

It should be noted that even with Option 4, the exact definition of RedCap UE type in terms of reduced UE capabilities will vary between different frequency bands and ranges. For example, in FDD bands, HD-FDD type A (and not FD-FDD) will be the minimum RedCap UE capability, and in TDD bands, TDD is the mandatory capability. As another example, the maximum bandwidth supported by a RedCap UE is 20 MHz in FR1 but 100 MHz in FR2. However, when the RedCap type is indicated by a UE, the network can unambiguously infer the set of mandatory capabilities of the UE in the frequency band/range it operates.

Based on the above considerations, Option 4 should be used to define a RedCap UE type. The exact definition can be according to Table 1.

Table 1: Definition of RedCap UE

| Capability | RedCap UE | Comments |
| --- | --- | --- |
| Maximum UE bandwidth | 20 MHz for FR1 100 MHz for FR2 | No optional support of wider UE BW than 20 / 100 MHz. |
| Minimum number of Rx branches | 1 | Support of more than 1 Rx branch is optional. |
| Supported number of DL MIMO layers | 1 for UEs with 1 Rx 2 for UEs with >1 Rx |  |
| Maximum modulation order | 64QAM | Applicable for UL and DL, in both FR1 and FR2. Support of 256QAM in DL in FR1 is optional. |
| Duplex operation | HD-FDD TDD | Support of FD-FDD is optional. |

Among the list of options captured in TR 38.875 (clause 10.1) for specifying the definition of RedCap UE type, Option 1, Option 2 and Option 3 will lead to multiple UE types being specified, which is not in line with the objective in the WID.

Option 4 is the most attractive for specifying the definition of the RedCap UE type. Option 4 will result in a single RedCap UE type, minimize market fragmentation, promote economy of scale, and reduce specification effort.

The exact definition of the RedCap UE type in terms of reduced UE capabilities will vary between different frequency bands and ranges. However, the network can unambiguously infer the set of mandatory capabilities of the UE based on the used frequency band/range.

Specify the definition of the RedCap UE type based on Option 4, i.e., the corresponding minimum set of the reduced capabilities that one RedCap UE type shall mandatorily support. The definition can be according to Table 1.

## 2.2	Constraining of reduced capabilities

The specifications (including how the RedCap UE type is defined) should prevent non-RedCap UEs from using RedCap UE capabilities not intended for non-RedCap UEs (e.g., reduced number of Rx branches), and should prevent RedCap UEs from using non-RedCap UE capabilities not intended for RedCap UEs (e.g., CA, DC, and wider bandwidths). In Table 2, we summarize capabilities that are allowed (either mandatorily or optionally) and those that are not allowed for RedCap and non-RedCap UEs.

Table 2: Capabilities allowed and not allowed for RedCap and non-RedCap UEs.

| Capability | RedCap UE | Non-RedCap UE |
| --- | --- | --- |
| 20 MHz maximum UE BW for FR1 100 MHz maximum UE BW for FR2 | Allowed | Not allowed (cf. 38.306) |
| Wider than 20 MHz UE BW for FR1 Wider than 100 MHz UE BW for FR2 | Not allowed | Allowed[NOTE 1] |
| 1 Rx branch | Allowed | Not allowed |
| 2 Rx branches | Allowed | Not allowed in bands n7, n38, n41, n77, n78 and n79 (except for vehicular UEs). Allowed, otherwise. |
| 1 DL MIMO layer | Allowed, for a RedCap UE with 1 Rx branch. Not allowed, otherwise. | Not allowed for single CC standalone NR (cf. TS 38.306) |
| 2 DL MIMO layers | Allowed | Allowed, for a non-RedCap UE with 2 Rx branches. Not allowed[NOTE 2], otherwise. |
| HD-FDD | Allowed | Not allowed |
| CA | Not allowed | Allowed |
| DC | Not allowed | Allowed |
| Note 1: Support of at least 100 MHz in FR1 and 200 MHz in FR2 is mandatory (cf. TS 38.306). Note 2: For single CC standalone NR, it is mandatory with capability signaling to support at least 4 MIMO layers in the bands where 4 Rx is specified as mandatory (cf. TS 38.306). |  |  |

It should be noted that RedCap and non-RedCap UEs will support additional capabilities that are not explicitly mentioned in Table 2. However, if certain capabilities are “Not allowed”, then the specifications should mandate that those capabilities cannot be used by RedCap/non-RedCap UEs. When mapping RAN1 agreements to specifications, Table 2 can serve as a guidance on what RedCap UE capabilities are not allowed for non-RedCap UEs, and vice versa. Other aspects related to constraining of capabilities is up to RAN2.

The desired constraining of capabilities to RedCap and non-RedCap UEs can be achieved by disallowing some UE capabilities for RedCap and non-RedCap UEs, respectively, when capturing the RedCap UE type definition and the associated UE capabilities in the specifications. The detailed signaling solution is up to RAN2.

# 3	Identification of RedCap UEs

Another RAN2-led objective with relevance to RAN1 is to introduce early indication of RedCap UEs in Msg1 and/or Msg3, and MsgA (if supported). Such an indication enables the network to handle RedCap UEs differently than non-RedCap UEs during initial access, i.e., before the UE capabilities are fully known.

## 3.1	4-step RACH

Considering the 4-step RACH procedure, possible reasons to have indication in Msg3 include the following:

- It enables the network to properly schedule RedCap UEs after Msg3 and until specific UE capabilities are known, by considering baseline capabilities of the RedCap UE type. It also helps to avoid negative impact on non-RedCap UEs and system performance (e.g., due to too conservative scheduling). Note that an explicit indication of RedCap UE type in Msg3 may be needed only for UEs coming from RRC_IDLE. For UEs coming from RRC_INACTIVE, gNB can determine the full UE capabilities from the UE context retrieved using the I-RNTI in Msg3.

- It enables the network to disable frequency hopping for PUCCH in response to Msg4 for RedCap UEs, if such a mechanism is agreed to be supported. If supported, this would help to minimize the PUSCH resource fragmentation for non-RedCap UEs as highlighted in our companion paper [5].

- It enables RRC connection rejection of RedCap UEs for access restriction (for UEs coming from RRC_IDLE and RRC_INACTIVE, if the UE context is not found) [2].

- It makes it possible to differentiate or enable prioritization of non-RedCap UEs vs. RedCap UEs during contention resolution if RedCap UE type is visible to the MAC layer [2].

Possible reasons to have indication in Msg1, in addition to those described for Msg3 above, include the following:

- It enables coverage recovery of Msg2 PDSCH for a 1 Rx RedCap UE operating in legacy 4 Rx FR1 bands with 24 dBm/MHz DL PSD, without impact on non-RedCap UEs. Note that 3 dB antenna efficiency loss is not currently in the WID scope. Therefore, Msg2 requires only ~3 dB coverage recovery, and this can be recovered using existing TBS scaling technique (e.g., with scaling factor of 0.5). Due to the same reason, coverage recovery of other physical channels (in the same or different deployments) are not needed.

- It enables the RedCap UE to operate in an initial UL BWP which is wider than the RedCap UE bandwidth (currently under discussion in RAN1), as the gNB can take into account RF retuning time while transmitting RAR if the RACH occasion (RO) associated with the best SSB falls outside the RedCap UE bandwidth (without impact on non-RedCap UEs that is, since RF retuning time could be applied for all UEs).

- It enables the possibility to schedule Msg3 transmission for RedCap UEs without frequency hopping, and for non-RedCap UEs with frequency hopping, if deemed necessary e.g., if initial UL BWP is configured to be wider than the RedCap UE bandwidth.

Based on the discussion above, it can be concluded that the early indication of RedCap UE type in Msg1 is needed only for certain deployments/configurations. Msg3 indication is sufficient in most cases. Furthermore, to carry out Msg1 indication, the network needs to configure separate initial UL BWP or partition the PRACH resources for RedCap UEs. These solutions can lead to increase in UL overhead and reduction in PRACH capacity [2], impacting both RedCap and non-RedCap UEs. In addition, there are several other Rel-17 features (RAN slicing, Small Data Transmission, Coverage Enhancement, etc.) that will potentially use Msg1 indication (see [6]). Indication of a RedCap UE that supports all these features simultaneously would indeed be complicated. Therefore, early indication of RedCap UE type in Msg1 should be configurable (as the WI objective states), and the resources for Msg1 indication should be reserved only when it is necessary. Due to the mentioned reasons, it is also imperative that specific RedCap UE capabilities (e.g., 1 Rx vs. 2 Rx) do not have to be reported via Msg1.

If the initial UL BWP of RedCap UEs is the same as the initial UL BWP for non-RedCap UEs (already agreed to be supported for the scenario when initial UL BWP is no wider than the RedCap UE bandwidth), PRACH partitioning can be used for Msg1 indication. The detailed signalling solution would be up to RAN2. For other scenarios, such as when a separate initial UL BWP is configured for RedCap UEs, or when the initial UL BWP is wider than the RedCap UE bandwidth, further decisions are expected in future RAN1 meetings, as whether to allow such scenarios is currently FFS in AI 8.6.1.1 [3].

Early indication of RedCap UE type in Msg3 (for UEs coming from RRC_INACTIVE) can be made either configurable, i.e., Msg3 indication can be present/absent if Msg1 indication is absent/present, or non-configurable, i.e., Msg3 indication is always present irrespective of whether Msg1 indication is present. Which option to support will depend on the solution(s) that RAN2 will introduce to enable Msg3 indication. For instance, if the Msg3 indication is carried out using the spare bit in existing Msg3 definition or using a new LCID, then we do not see a need to make the Msg3 indication configurable, and the indication can be always present in Msg3 regardless of whether an indication is also configured to be present in Msg1 or not. However, if extended Msg3 size is introduced to carry additional bit(s) for Msg3 indication, then it is preferrable to have the indication configurable, as it may degrade coverage of Msg3 [2].

Early indication of RedCap UE type in Msg1 is needed only for certain deployments/configurations. Msg3 indication is sufficient in most cases.

In addition to RedCap, there are several other Rel-17 features, such as RAN slicing, Small Data Transmission, Coverge Enhancement, etc., that will potentially use Msg1 indication.

Explicit indication of RedCap UE type in Msg3 is needed only for UEs coming from RRC_IDLE. For UEs coming from RRC_INACTIVE, gNB can determine the full UE capabilities from the UE context retrieved using the I-RNTI in Msg3.

Early indication of RedCap UE type in Msg1 is configurable.

Early indication of RedCap UE type is binary, i.e., the UE either indicates to be a RedCap UE or not.

If the initial UL BWP of RedCap UEs is the same as the initial UL BWP for non-RedCap UEs, partitioning of PRACH resources is used for Msg1 indication.

    - FFS how Msg1 indication is carried out in other scenarios.

    - The detailed signalling solution is up to RAN2.

For UEs coming from RRC_IDLE, whether to have Msg3 indication of RedCap UE type configurable (i.e., Msg3 indication is present/absent depending on whether Msg1 indication is absent/present) or non-configurable (i.e., Msg3 indication is always present irrespective of whether Msg1 indication is present) is up to RAN2.

## 3.2	2-step RACH

The discussion in Section 3.1 considers 4-step RACH procedure for initial access. RedCap UEs may potentially additionally support 2-step RACH procedure. The 2-step RACH procedure consists of MsgA and MsgB, where MsgA is a combination of UL transmissions, Msg1 (preamble) and Msg3 (PUSCH), and MsgB is a combination of DL transmissions, Msg2 and Msg4.

Possible reasons to have early RedCap indication in the PUSCH part of MsgA are as follows:

- It enables coverage recovery of MsgB PUSCH (if needed) when MsgA PUSCH can be successfully decoded (i.e., successRAR), without impact on non-RedCap UEs.

- It enables RRC connection rejection of RedCap UEs, prioritization of non-RedCap UEs over RedCap UEs, disabling of PUCCH frequency hopping, and RedCap UE to operate in an initial UL BWP which is wider than the RedCap UE bandwidth, in the same way as for the 4-step RACH procedure.

For the 2-step RACH procedure, there is possible added benefit in having early indication in the preamble part of MsgA in the following scenario:

- It enables coverage recovery of MsgB PUSCH (if needed) when MsgA preamble is detected, but MsgA PUSCH is not decoded correctly (i.e., fallbackRAR), without impact on non-RedCap UEs.

Therefore, whether to have the early indication in the preamble part or the PUSCH part of MsgA would depend on the need for MsgB coverage recovery. If coverage recovery (e.g., of fallbackRAR) is not needed, then the indication in the PUSCH part would be sufficient. However, coverage evaluations for 2-step RACH were not carried out during the RedCap SI phase. Therefore, RAN1 should first discuss the coverage recovery needs for MsgB, considering its different variants, such as successRAR without RRC, successRAR with RRC, and fallbackRAR. Nevertheless, in our view, early indication of RedCap UEs during 4-step RACH procedure should be prioritized.

In addition to RedCap, there are several other Rel-17 features, such as RAN slicing, Small Data Transmission, Coverage Enhancement, etc., that will potentially use MsgA indication. Indication of a RedCap UE that supports all these features simultaneously would be complicated.

Whether to have the early indication in the preamble part or the PUSCH part of MsgA would depend on the need for MsgB coverage recovery.

Coverage evaluations for 2-step RACH were not carried out during the RedCap SI phase.

Discuss in RAN1 the coverage recovery needs for MsgB of 2-step RACH procedure. However, early indication of RedCap UEs during 4-step RACH procedure will be prioritized.

# 4	Access restriction

According to the WID [1], access restriction via barring a RedCap UE from camping on the cell/frequency should be supported. The cell/frequency barring indication must be provided in the system information (e.g., SIB1), either explicitly or implicitly. Also, the indication must be specific to the number of Rx branches of the UE. In our view, this is purely a RAN2 topic. RAN1 does not need to consider this objective further, unless RAN2 asks RAN1’s input on some specific aspects.

Some RAN1#104bis-e contributions have indicated their preference for having the barring indication in the DCI associated with SIB1 (instead of SIB1), claiming it to be more beneficial for UE power saving. However, we do not think that this solution would bring any meaningful power saving benefits over the indication in SIB1. One reason is that the barring information in SIB1 is quite static. Note that the UE typically would only check once if it can camp on the cell or not. Furthermore, if the PDSCH carrying SIB1 is scheduled in the same slot as the PDCCH carrying DCI with the information on barring, the UE would anyway have to buffer PDSCH. This would further limit the power saving benefits. Moreover, RAN2 is the leading working group on this objective. Therefore, RAN1 should avoid spending time on unnecessary over-optimization without consultation with RAN2.

Cell/frequency barring indication in the DCI associated with SIB1 would not bring any meaningful power saving benefits over the indication in SIB1.

The RedCap objective on system information indication to indicate whether a RedCap UE can camp on the cell/frequency is not considered further in RAN1.

# 5	Conclusion

In the previous sections we made the following observations:

Observation 1	Among the list of options captured in TR 38.875 (clause 10.1) for specifying the definition of RedCap UE type, Option 1, Option 2 and Option 3 will lead to multiple UE types being specified, which is not in line with the objective in the WID.

Observation 2	Option 4 is the most attractive for specifying the definition of the RedCap UE type. Option 4 will result in a single RedCap UE type, minimize market fragmentation, promote economy of scale, and reduce specification effort.

Observation 3	The exact definition of the RedCap UE type in terms of reduced UE capabilities will vary between different frequency bands and ranges. However, the network can unambiguously infer the set of mandatory capabilities of the UE based on the used frequency band/range.

Observation 4	Early indication of RedCap UE type in Msg1 is needed only for certain deployments/configurations. Msg3 indication is sufficient in most cases.

Observation 5	In addition to RedCap, there are several other Rel-17 features, such as RAN slicing, Small Data Transmission, Coverge Enhancement, etc., that will potentially use Msg1 indication.

Observation 6	Explicit indication of RedCap UE type in Msg3 is needed only for UEs coming from RRC_IDLE. For UEs coming from RRC_INACTIVE, gNB can determine the full UE capabilities from the UE context retrieved using the I-RNTI in Msg3.

Observation 7	In addition to RedCap, there are several other Rel-17 features, such as RAN slicing, Small Data Transmission, Coverage Enhancement, etc., that will potentially use MsgA indication. Indication of a RedCap UE that supports all these features simultaneously would be complicated.

Observation 8	Whether to have the early indication in the preamble part or the PUSCH part of MsgA would depend on the need for MsgB coverage recovery.

Observation 9	Coverage evaluations for 2-step RACH were not carried out during the RedCap SI phase.

Observation 10	Cell/frequency barring indication in the DCI associated with SIB1 would not bring any meaningful power saving benefits over the indication in SIB1.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Specify the definition of the RedCap UE type based on Option 4, i.e., the corresponding minimum set of the reduced capabilities that one RedCap UE type shall mandatorily support. The definition can be according to Table 1.

Proposal 2	The desired constraining of capabilities to RedCap and non-RedCap UEs can be achieved by disallowing some UE capabilities for RedCap and non-RedCap UEs, respectively, when capturing the RedCap UE type definition and the associated UE capabilities in the specifications. The detailed signaling solution is up to RAN2.

Proposal 3	Early indication of RedCap UE type in Msg1 is configurable.

Proposal 4	Early indication of RedCap UE type is binary, i.e., the UE either indicates to be a RedCap UE or not.

Proposal 5	If the initial UL BWP of RedCap UEs is the same as the initial UL BWP for non-RedCap UEs, partitioning of PRACH resources is used for Msg1 indication.

	FFS how Msg1 indication is carried out in other scenarios.

	The detailed signalling solution is up to RAN2.

Proposal 6	For UEs coming from RRC_IDLE, whether to have Msg3 indication of RedCap UE type configurable (i.e., Msg3 indication is present/absent depending on whether Msg1 indication is absent/present) or non-configurable (i.e., Msg3 indication is always present irrespective of whether Msg1 indication is present) is up to RAN2.

Proposal 7	Discuss in RAN1 the coverage recovery needs for MsgB of 2-step RACH procedure. However, early indication of RedCap UEs during 4-step RACH procedure will be prioritized.

Proposal 8	The RedCap objective on system information indication to indicate whether a RedCap UE can camp on the cell/frequency is not considered further in RAN1.

# References

RP-210918, “Revised WID on support of reduced capability NR devices”, Nokia and Ericsson, 3GPP TSG RAN #91e, March 2021.

TR 38.875 V17.0.0, “Study on support of reduced capability NR devices (Release 17)”, Dec. 2020.

R1-2104027, “RAN1 agreements for Rel-17 NR RedCap”, Rapporteur (Ericsson), 3GPP TSG RAN1 #104bis-e, April 2021.

RP-200938, “Revised WID UE Power Saving Enhancements for NR”, MediaTek, 3GPP TSG RAN #88e, June 2020.

R1-2104184, “Ensuring coexistence between RedCap and non-RedCap UEs”, Ericsson, Deutsche Telekom, NTT DOCOMO, Softbank, Telecom Italia, Telstra, Verizon Wireless, Vodafone, 3GPP TSG RAN1 #105-e, May 2021.

R2-2104933, "RACH partitioning for Rel-17 features", Ericsson, 3GPP TSG RAN2 #114-e, May 2021.