3GPP TSG RAN WG1 Meeting #105	R1-2104327

e-Meeting, May 10th – 27th, 2021

Source: 	ZTE

Title: 	Discussion on CSI feedback enhancements for eURLLC

Agenda item:	8.3.1.2

Document for:	Discussion and Decision

Introduction

In the RAN1 #104bis-e meeting, several schemes for the CSI feedback enhancements were discussed and the following conclusion and agreements were reached [1].

Conclusion: For new reporting Case 1, do not consider further the following schemes: - Case 1-2: CSI prediction - Case 1-4: Interference covariance matrix - Case 1-9: Reference wideband CQI excludes worst sub-bands - Case 1-10: CSI expiration time Agreements: For new reporting Case 2, focus study on reporting of delta-CQI/MCS (Case 2-3): - Note: this delta-CQI/MCS is determined based on UE implementation (for example, using SINR, LLR, raw BER, flipped bits, LDPC iterations, BLEP, # fail parity checks, etc.) - Companies are encouraged to provide more details in their analysis - FFS: Granularity of new report type (e.g. units of CQI or MCS, how many bits) - FFS: Whether quantity reported is relative to the scheduled MCS Agreement: Focus study on the following for new reporting Case 1: - Reporting of new metric, where new metric shall be determined based on network configured channel and interference measurement interval (multiple CMR and/or IMR instances) to enable accurate MCS selection. - Downselect by RAN1#105 to at most a single method from the following options: - Mean-CQI/SINR and stdev-CQI/SINR (FFS details) - CSI based on worst IMR occasion (FFS details) - Interference standard deviation (FFS details) - Worst-M CQI (FFS details) - FFS: Whether network configured channel and interference measurement interval can also be applied to existing CSI type - Increasing granularity of subband CQI (e.g. 3-bits differential subband CQI or 4-bits full subband CQI). - Updating only CQI in a report, where CQI is conditioned on a previous instance in which RI/PMI/(CRI) is updated. - Applicable for same reporting quantity as R16 for CQI. - FFS: Whether network configured channel and interference measurement interval can also be applied - FFS: Whether RI/PMI/(CRI) is transmitted in a report where only CQI is updated - FFS: how to report the updated CQI - FFS: whether the CQI processing time can be is reduced compared to Rel-16 CSI processing delay

In this contribution, these schemes are discussed and corresponding simulation results are also provided. In addition, A-CSI on PUCCH is also discussed.

Evaluation of CSI feedback enhancement

Evaluation results for A-CSI on PUCCH and CSI feedback in Rel-16

To move forward the standardization progress [2], it is suggested that A-CSI on PUCCH triggered by DL DCI can be compared with P-CSI on PUCCH, SP-CSI on PUCCH and A-CSI on PUSCH, where the latter three CSI feedback modes have already been supported in the Rel-16. Therefore, the performance of these four types of CSI feedback are evaluated below.

- Case 0-1: P-CSI on PUCCH (supported in Rel-16)

- Case 0-2: SP-CSI on PUCCH (supported in Rel-16)

- Case 0-3: A-CSI on PUSCH (supported in Rel-16)

- Case 0-4: A-CSI on PUCCH triggered by DL grant

The detailed simulation results are shown in Table 1.

Table 1 The simulation results in R15 AR/VR scenario

| Cases Metrics | Percentage (%) | BLER of 1st | RU (%) | RU for retransmission (%) | MCS Predict too large ratio (%) | Report quantity | CSI-RS overhead |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 0-1 | 29.52 | 0.4750 | 3.52 | 1.59 | 73.24 | WB {PMI, CQI}+RI | P CSI-RS with period of 10 slots |
| Case 0-2 | 49.52 | 0.1692 | 1.88 | 0.34 | 29.73 | SB {PMI, CQI}+RI | P CSI-RS with period of 10 slots |
| Case 0-3 WB | 30.00 | 0.4574 | 4.74 | 1.97 | 68.07 | WB {PMI, CQI}+RI | A CSI-RS at each PDSCH scheduling |
| Case 0-3 SB | 52.86 | 0.1514 | 3.09 | 0.44 | 28.91 | SB {PMI, CQI}+RI | A CSI-RS at each PDSCH scheduling |
| Case 0-4 WB | 37.14 | 0.3216 | 3.88 | 1.32 | 65.14 | WB {PMI, CQI}+RI | A CSI-RS at each PDSCH scheduling |
| Case 0-4 SB | 66.67 | 0.0943 | 2.89 | 0.27 | 26.50 | SB {PMI, CQI}+RI | A CSI-RS at each PDSCH scheduling |

Case 0-1 is P-CSI feedback with Wideband CSI report, Case 0-2 is SP-CSI feedback with Subband CSI report. These two cases are used as the baseline cases for WB performance comparison and SB performance comparison separately.

Compared with Case 0-1, A-CSI feedback have better performance than P-CSI feedback case in both Case 0-3 WB and Case 0-4 WB. The performance of Case 0-4 WB is increased by a maximum of 26% compared with Case 0-1 in terms of percentage of UEs satisfying the requirements, at the meanwhile, the BLER of 1st, RU, RU for retransmission and MCS Predict too large ratio are all decreased.

Similarly, compared with Case 0-2, A-CSI feedback have better performance than SP-CSI feedback case in both Case 0-3 SB and Case 0-4 SB. The performance of Case 0-4 SB is increased by a maximum of 35% compared with Case 0-2 in terms of percentage of UEs satisfying the requirements, at the meanwhile, the BLER of 1st, RU, RU for retransmission and MCS Predict too large ratio are all decreased.

A-CSI on PUSCH is triggered by UL Grant, while A-CSI on PUCCH is triggered by DL Grant. Therefor A-CSI on PUSCH needs additional PDCCH overhead, which will increase the RU and at the same time bring more interference. So it is observed that A-CSI on PUCCH has better performance than A-CSI on PUSCH. Due to more flexible CSI feedback, both A-CSI on PUSCH and A-CSI on PUCCH have better performance than P/SP-CSI on PUCCH.

Observation 1: A-CSI on PUCCH has better performance than A-CSI on PUSCH and P/SP-CSI.

Evaluation results for Case-1 report

For Case-1 new reporting, two feedback modes are evaluated. The specific simulation cases are as follows:

Case 1-1: Traditional feedback quantities such as CQI and PMI are reported. The feedback CQI is controlled only by interference filtering operation. For gNB to obtain conservative CQI dynamically, here the interference filtering operation is configured to get the maximum value of interference samples measured in multiple IMR occasions.

Case 1-2: In addition to the PMI feedback, some statistical features of the CQI, such as the average CQI and the CQI standard deviation are reported. Then gNB estimates the predict CQI as: Predict CQI = mean(CQI)-std(CQI).

To compare the performance of the two Case-1 new reporting, we use Case 0-1 and Case 0-2 as the baseline for comparison.

Table 2 The simulation results for Case-1 reports in R15 AR/VR scenario

| Cases Metrics | Percentage (%) | BLER of 1st | RU (%) | RU for retransmission (%) | MCS Predict too large ratio (%) | Report quantity | CSI-RS overhead |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 0-1 | 29.52 | 0.4750 | 3.52 | 1.59 | 73.24 | WB {PMI, CQI}+RI | P CSI-RS with period of 10 slots |
| Case 0-2 | 49.52 | 0.1692 | 1.88 | 0.34 | 29.73 | SB {PMI, CQI}+RI | P CSI-RS with period of 10 slots |
| Case 1-1 WB | 48.58 | 0.2369 | 3.80 | 0.81 | 41.97 | WB {PMI, min(CQI)}+RI | P CSI-RS with period of 10 slots |
| Case 1-1 SB | 57.62 | 0.1239 | 2.25 | 0.30 | 22.97 | SB {PMI, min(CQI)}+RI | P CSI-RS with period of 10 slots |
| Case 1-2 WB | 30.48 | 0.3277 | 3.63 | 1.13 | 55.12 | WB { PMI, mean(CQI), std(CQI)}+RI | P CSI-RS with period of 10 slots |
| Case 1-2 SB | 30.95 | 0.2650 | 2.87 | 0.85 | 47.77 | SB { PMI, mean(CQI), std(CQI)}+RI | P CSI-RS with period of 10 slots |

According to the simulation results, Case 1-1 has a slight performance gain on percentage of satisfied UEs compared with baseline case. Because the CQI reported in the Case 1-1 reflects the maximum interference sample within the filtering window such that the gNB can obtain the adaptive conservative CQI that matches the dynamic interference jitter. In this case, it has a slight performance gain. However Case 1-2 has no performance gain compared with baseline case. It seems that predicting the MCS based on the statistical features need further study to achieve the performance gain.

Therefore, for Case-1 new reporting, we suggest that the gNB can configure the interference measurement filter in order to obtain the required CQI feedback. For instance, when the gNB needs to obtain the conservative CQI, gNB can configure the UE to feed back the CQI calculated based on the maximum interference sample within the filter window but the feedback structure itself needn’t change.

Observation 2: For Case-1 new reporting, configuring interference measurement filtering is a better candidate to achieve performance gain on percentage of satisfied UEs, no additional report quantity is needed.

Evaluation results for Case-2 report

For Case-2 new reporting, we have conducted simulations focusing on specific feedback modes based on the decoding margin report and (delta) CQI/MCS/SINR report. The simulation cases are listed as follows:

Case 2-1: For PDSCH NACK case, UE feedbacks multi-NACK as delta SINR, which is quantized as 3-bit feedback overhead. Additional OLLA enhancement is introduced.

Case 2-2: For PDSCH NACK case, UE feedbacks multi-NACK as delta MCS, which is quantized as 3-bit feedback overhead. Additional OLLA enhancement is introduced.

Case 2-3: For PDSCH ACK case, UE feedbacks multi-ACK as delta SINR, which is quantized as 3-bit feedback overhead. Additional OLLA enhancement is introduced.

To compare the performance of the three Case-2 reports, we also use Case 0-1 and Case 0-2 as the baseline for comparison. The detailed simulation results are shown as Table 3.

Table 3 The simulation results for Case-2 reports in R15 AR/VR scenario

| Metrics Cases | Percentage (%) | BLER of 1st | RU (%) | RU for retransmission (%) | MCS Predict too large ratio (%) | OLLA Enhancement |
| --- | --- | --- | --- | --- | --- | --- |
| Case 0-1 | 29.52 | 0.4750 | 3.52 | 1.59 | 73.24 | Without |
| Case 0-2 | 49.52 | 0.1692 | 1.88 | 0.34 | 29.73 | Without |
| Case 2-1 | 93.81 | 0.0054 | 33.04 | 0.06 | 1.08 | With |
| Case 2-2 | 60.48 | 0.1721 | 1.92 | 0.37 | 21.48 | With |
| Case 2-3 | 61.43 | 0.1111 | 2.32 | 0.27 | 20.65 | With |

According to the simulation results, it is observed that Case 2-1, Case 2-2 and Case 2-3 can obtain performance gain on percentage of satisfied UEs compared with the baseline Case. Especially for Case 2-1, it can significantly improve percentage of satisfied UEs and greatly reduce the BLER of 1st, as well as the usage of RU for retransmission. Case 2-1 greatly improves the performance, while the cost is the much usage of RU, mainly because MCS prediction is conservative and more precise for enhanced OLLA.

In conclusion, to improve the reliability of URLLC PDSCH transmission, Case 2-1, Case 2-2 and Case 2-3 from scheme 2a-1 are good candidates, among which Case 2-1(multi-NACK as delta SINR) is the most effective one.

Observation 3: Scheme 2a-1 of Case-2 new reporting can obtain obvious reliability gain compared with Rel-16 CSI report.

Analysis on CSI feedback enhancement

Discussion on case 1 report

For case 1 report for CSI feedback enhancements, the following methods were agreed for further study.

- Mean-CQI/SINR and stdev-CQI/SINR (FFS details)

- CSI based on worst IMR occasion (FFS details)

- Interference standard deviation (FFS details)

- Worst-M CQI (FFS details)

- Increasing granularity of subband CQI (e.g. 3-bits differential subband CQI or 4-bits full subband CQI).

- Updating only CQI in a report, where CQI is conditioned on a previous instance in which RI/PMI/(CRI) is updated.

Different report quantities correspond to different CSI feedback overhead and lead to different CSI processing latency. To introduce new feedback quantity, such as directly feedback the interference value, the interference statistical information and the channel statistical information, it is necessary to define new CSI feedback overhead and new CSI processing latency according to the feedback quantization, which needs more standardization effort and requires very fine and abundant simulation comparison. Furthermore, from the simulation results in section 2.2, the performance gain obtained from the statistical information feedback is not clear.

Observation 4: Statistical CSI and interference standard deviation report are relative complex methods, which need more standardization efforts and a long time for discussion.

In the URLLC scenario, interference jitter is the main reason for the reliability degradation. If interference jitter can be reflected better by advanced CSI feedback or if the CSI feedback by the UE has greater tolerance for interference jitter, the negative impact of interference jitter will be greatly reduced. Therefore, we should focus on the enhancements in the time domain to reflect the interference fluctuation. However, for worst-M CQI report, the main benefit is the CSI report overhead reduction since CSI report for sub-band has already been supported.

Observation 5: The benefit of the worst-M sub-band CQI is the overhead reduction since CSI report for sub-band has already been supported.

For CQI-only report, it is assumed that the received signal strength variation is slower than the interference. Thus, the RI/PMI report is less frequent than CQI. The benefit is the CSI reporting overhead reduction. Additionally, since there is no need to calculate the PMI and RI in the CQI-only reporting occasion, the CSI processing time can be reduced in theory. With the reduced CSI processing time for CQI-only report, the network can get the updated CQI a bit earlier based on the timely report compared with current CSI report. Then the scheduling can be more accurate based on the updated CQI. It can be predicted that there is no improvement on the more accurate MCS selection if the CSI processing time is not reduced. Therefore, the benefit on the more accurate MCS selection is derived from the reduced processing time for CQI-only report.

Observation 6: The benefit on the more accurate MCS selection is derived from the reduced processing time for CQI-only report.

How many symbols can be reduced for the CSI processing time should be carefully discussed. However, Even though we assume the CSI processing time can be reduced by a slot, it means there is only one additional slot with updated CQI per CSI reporting period compared with the current CSI report. The performance gain based on only one additional slot with updated CQI may be limited and should be further evaluated. Therefore, there are many issues to be discussed for the CQI only report, i.e., how many symbols can be reduced for CSI processing time and the corresponding performance gain.

Observation 7: There are many issues to be discussed for the CQI only report, i.e., how many symbols can be reduced for CSI processing time and the corresponding performance gain.

A relatively simple way is to enhance the filtering mechanism for interference or channel measurement. Currently, the network can configure filtering on/off for both interference measurement and channel measurement in Rel-16. In the case of filtering off, the UE measures the channel/interference according to the last available resource occasion. However, in the case of filtering on, L1 filtering is up to UE implementation. If network can explicitly configure the channel/interference filtering to the UE, interference jitter can be reflected in the CSI feedback very well. For example, the network configure the number of the occasion participating in the L1 filtering or the conditions for the UE to choose the resource occasions participating in the L1 filtering. Alternatively, the network configures the UE to use the worst interference (e.g., the highest interference measurement result) within the L1 filtering window to obtain the CQI. The simulation results in section 2.2 proves that using the maximum interference occasion within the period for filtering can achieve additional performance gain.

Observation 8: CSI report based on the worst interference can reflect the interference jitter very well so that the network scheduling has a greater tolerance for the interference jitter to ensure the requirements of URLLC, which has been justified by the provided simulation results.

It may be argued that network can perform conservative scheduling without the proposed solution if required. However, it should be noted that the objective of the CSI feedback enhancement is to get the more accurate MCS selection. It means both too high MCS and too low MCS should be avoided. Without the proposed information report, the conservative scheduling may lead to too low MCS since the network has to schedule the PDSCH as conservative as possible to satisfy the requirements of URLLC. In this case, the resource efficiency may be reduced.

Observation 9: Current conservative scheduling by the network may lead to too low MCS.

Proposal 1: Enhancement on interference filtering configuration should be supported, e.g. CSI report based on the worst interference.

Discussion on case 2 report

For Case-2 report, the measurement is based on the PDSCH decoding. More information are feedback together with the HARQ-ACK for OLLA performed at the network side. With such information, the network can adjust the MCS more efficiently than the traditional OLLA based on HARQ-ACK only. If the NACK happens, the corresponding retransmission must be decoded successfully due to the high requirement of URLLC. It means that the network should scheduling the retransmission more carefully. In this case, the additional information feedback can help the network perform more accurate scheduling. If the ACK happens, the traditional OLLA based on ACK feedback is sufficient. The additional report in case of NACK is more important. Therefore, the multi-level NACK feedback is slightly preferred.

How to determine the multi-level NACK feedback based on the PDSCH decoding is up to UE implementation. For example, it is based on Delta SINR obtained through PDSCH decoding, and Delta SINR = Target SINR - Effective SINR. For the report quantity, if the delta SINR is reported, it is directly used for the network to adjust the backoff for the subsequent scheduling. For example, backoffnew=backoffold+ΔSINR, where ΔSINR is reported by the UE. If the delta CQI/MCS is reported, it indicates the MCS recommended by the UE according to the current channel state. Considering the report overhead, a differential CQI/MCS report relative to the scheduled MCS for the previous PDSCH can be used.

For the granularity, 2 bits or 3 bits for the multi-NACK report are good balance between the report overhead and the performance gain. In the simulation above, the Delta SINR is quantized as 3 bits, so the feedback overhead is only 3 bits (including ACK as one state). According to our simulation results in section 2.3, when the OLLA operation is further optimized based on Delta SINR, the system performance is greatly improved compared with other cases. In addition, the feedback overhead is very small. Compared with Case-1 solutions, the feedback resource can be greatly saved. Therefore, we believe that this feedback method with measurement based on PDSCH can be supported.

Proposal 2: Support new report of Case-2 in Rel-17, such as multi-level NACK feedback obtained through PDSCH decoding.

Discussion on the A-CSI on PUCCH

Support of A-CSI on PUCCH

Aperiodic CSI report on PUSCH has been supported in Rel-16. According to the simulation results in section 2.1, supporting A-CSI report on PUCCH can bring obvious gain and avoid the UL Grant overhead caused by triggering A-CSI report on PUSCH. Therefore, Rel-17 should support A-CSI report on PUCCH.

Proposal 3: Support A-CSI on PUCCH in Rel-17.

PUCCH trigger mechanism

The following alternatives can be considered as A-CSI triggering mechanism:

- Alt 1：Triggering by a new ‘CSI Request’ field in DL grant

- Alt 2：Triggering by some existing field in DL grant

- Alt 3：Triggering by group-common DCI

- Alt 4：Triggering by NACK

We evaluated the traditional UL Grant triggering, DL Grant triggering in section 2.1, and the NACK triggering in section 2.3. UL Grant triggering needs additional UL Grant overhead, which is prone to PDCCH blocking under high load case. Therefore, DL Grant triggering should be supported as a basic enhancement mode in Rel-17.

Alt 1 is the simplest way by copying the 'CSI Request' field in UL Grant into DL grant. 'CSI Request' field can indicate a specific trigger state value. gNB configures the associated CSI report configuration(s) and CSI-RS/CSI-IM resources for each trigger state value. In this way, the gNB can trigger A-CSI feedback dynamically and flexibly on demand.

It is convenient to use existing field in DL grant to indicate whether or not to trigger A-CSI feedback for Alt 2. However, it is difficult to find an existing field in DL grant to further instruct CSI report configuration and CSI-RS/CSI-IM resource. Therefore, for Alt 2, CSI report configuration and CSI-RS/CSI-IM resource should be simplified. For example, the gNB can semi-statically configure a specific CSI report configuration and CSI-RS/CSI-IM resource for the enhanced A-CSI feedback, and only use the existing field in DL grant to indicate whether or not to trigger the configured A-CSI feedback. In a word, this way is more like the A-CSI feedback triggering mode that has been abridged. If there is no need to consider too many options for CSI report configuration and CSI-RS/CSI-IM Resource, this method is also feasible.

Alt 3 seems more resource efficient since the group-common DCI could trigger more than one UEs to report A-CSI. However, it is most likely that the packet arrival for different UEs is not synchronous, and hence it is unnecessary to trigger multiple UEs to feedback A-CSI at the same time. For each triggering, only a few bits are actually used to trigger A-CSI for each UE, which makes the resource of group-common DCI inefficient in case that few UEs can be grouped together.

For Alt 4, the performance in section 2.3 is pretty good. For the case of Delta SINR feedback and with OLLA enhancement, Alt 4 has the best performance in terms of percentage of satisfied UEs.

Furthermore, Alt 4 can be enabled or disabled by gNB dynamically or semi-statically through DL grant or RRC configuration, so that the gNB can control PUCCH feedback resource overhead better.

Proposal 4: Support both DL grant based A-CSI triggering and NACK based A-CSI triggering in Rel-17.

PUCCH resource

There are two alternatives to be considered for PUCCH resource determination:

- Alt 1：Multiplexing A-CSI and HARQ-ACK on the same PUCCH resource

- Alt 2：Feedback A-CSI on a PUCCH resource independent of HARQ-ACK

According to the CSI computation delay requirement Z/Z’ and HARQ-ACK processing delay requirement Tproc,1 defined in [3]. In most cases, Z/Z’ is obviously larger than Tproc,1. Therefore, for CSI feedback based on CSI-RS/CSI-IM resource measurement, to avoid the extra delay of HARQ-ACK feedback, only Alt 2 can be used.

For the CSI feedback based on PDSCH measurement, both Alt 1 and Alt 2 can support, Alt 1 is preferred. No matter Alt 1 or Alt 2, the gNB needs to firstly demodulate the HARQ-ACK feedback, and then decide whether to receive the A-CSI feedback according to the HARQ-ACK feedback content. Therefore, if Alt 1 is used, HARQ-ACK and A-CSI carried on the same PUCCH resource should be coded separately.

The advantage of Alt 1 is that it is not necessary to separately indicate the A-CSI PUCCH resource. It is possible to multiplex the HARQ-ACK and A-CSI report in the same PUCCH resource based on the existing PRI and K1. The PUCCH resource may carry HARQ-ACK feedback only or both HARQ-ACK feedback and A-CSI feedback. Therefore, the PUCCH format and PUCCH resource size should be carefully configured to meet the requirements for the two load cases.

The advantage of Alt 2 is that HARQ-ACK PUCCH resource and A-CSI PUCCH resource can be configured independently. But the disadvantage is that additional overhead to indicate the UE the A-CSI PUCCH resource.

Based on the above analysis, we believe that both of Alt 1 and Alt 2 should be further studied.

Proposal 5: It needs to further study whether the A-CSI should be transmitted in the same PUCCH carrying HARQ-ACK.

PUCCH priority

For P/SP-CSI on PUCCH, following the principle in Rel-16, which is only supporting priority index 0 is enough.

Considering the potential conflict between multiple PUCCHs or PUCCH and PUSCH, it is necessary to determine the priority for A-CSI on PUCCH. For A-CSI on PUCCH, the A-CSI feedback priority can be associated with the HARQ-ACK feedback priority. When scheduling the PDSCH by a DL grant, the PI field in the DL grant can also be used to indicate the priority of the A-CSI feedback triggered by this DL grant or the corresponding NACK.

Proposal 6: Only priority index 0 is applicable to P/SP-CSI on PUCCH.

Proposal 7: The A-CSI on PUCCH priority can be indicated by the PI field in the DL grant.

Conclusion

According to the analysis above, we have the following observations and proposals:

Observation 1: A-CSI on PUCCH has better performance than A-CSI on PUSCH and P/SP-CSI.

Observation 2: For Case-1 new reporting, configuring interference measurement filtering is a better candidate to achieve performance gain on percentage of satisfied UEs, no additional report quantity is needed.

Observation 3: Scheme 2a-1 of Case-2 new reporting can obtain obvious reliability gain compared with Rel-16 CSI report.

Observation 4: Statistical CSI and interference standard deviation report are relative complex methods, which need more standardization efforts and a long time for discussion.

Observation 5: The benefit of the worst-M sub-band CQI is the overhead reduction since CSI report for sub-band has already been supported.

Observation 6: The benefit on the MCS selection is derived from the reduced processing time for CQI-only report.

Observation 7: There are many issues to be discussed for the CQI only report, i.e., how many symbols can be reduced for CSI processing time and the corresponding performance gain.

Observation 8: CSI report based on the worst interference can reflect the interference jitter very well so that the network scheduling has a greater tolerance for the interference jitter to ensure the requirements of URLLC, which has been justified by the provided simulation results.

Observation 9: Current conservative scheduling by the network may lead to too low MCS.

Proposal 1: Enhancement on interference filtering configuration should be supported, e.g. CSI report based on the worst interference.

Proposal 2: Support new report of Case-2 in Rel-17, such as multi-level NACK feedback obtained through PDSCH decoding.

Proposal 3: Support A-CSI on PUCCH in Rel-17.

Proposal 4: Support both DL grant based A-CSI triggering and NACK based A-CSI triggering in Rel-17.

Proposal 5: It needs to further study whether the A-CSI should be transmitted in the same PUCCH carrying HARQ-ACK.

Proposal 6: Only priority index 0 is applicable to P/SP-CSI on PUCCH.

Proposal 7: The A-CSI on PUCCH priority can be indicated by the PI field in the DL grant.

Reference

3GPP RAN1#104bis-e, Chairman's Notes final.

3GPP RAN1#103-e, R1-2009775, Feature lead summary #4 on CSI feedback enhancements for enhanced URLLC/IIoT

3GPP TS 38.214-g40, Physical layer procedures for data, Release 16.

Appendix System Level evaluation assumptions

Table 1: System Level evaluation assumptions for R15 AR/VR scenario

| Parameters | Value |
| --- | --- |
| Inter-BS distance | 500 m |
| Carrier frequency | 4 GHz |
| Simulation bandwidth | 40 MHz |
| SCS | 30 kHz |
| BS antenna element gain + connector loss | 8 dBi |
| BS antenna configurations | 4 Tx antenna ports (M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1; 1, 2) |
| UE antenna configuration | 4 Rx antenna ports Panel model 1: Mg=1, Ng=1, P=2, dH=0.5 (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 2) for 4 Rx; |
| BS antenna height | 25 m |
| UE antenna height | 1.5 m |
| UE antenna gain | 0dBi as starting point |
| BS Tx power | 49 dBm |
| UE receiver noise figure | 9 dB |
| Layout | Single layer - Macro layer: Hex. Grid |
| Number of UEs per cell | 40 |
| Channel model | UMa in TR 38.901 |
| UE distribution | 100% of users are outdoors Use 3 km/h for modeling fading channel |
| Channel estimation | Realistic |
| Periodicity of P-CSI reporting | 10 slots |
| Delay of P-CSI reporting | 4 slots |
| Physical layer configuration | 4 OS mini-slot |
| Traffic mode | FTP model 3 (100p/s) |
| Reliability | 99.999 |
| Latency | 4ms(200bytes) |