3GPP TSG RAN WG1#104-e		R1-2104375

e-Meeting, May 10th – 27th, 2021

Source:	vivo

Title:	Discussion on RLM/BFD/RRM relaxation

Agenda Item:	8.7.3

Document for:	Discussion and Decision

- Introduction

RAN 4 is leading RLM/BFD relaxation in power saving WI and RAN2 is leading RRM relaxation in RedCap WI for the purpose of power saving. Considering RLM/BFD relax and RRM relax has large impact to the power saving gain, it is proposed that RAN 1 power saving evaluation should take RLM/BFD relaxation for CONNECT mode and RRM relaxation for all modes into account. On the other hand, RAN1 is also designing connected mode PDCCH adaptation and idle/inactive mode paging enhancement. With that, the portion of  power consumption for RLM/BFD/RRM will be higher than without PDCCH adaptation.

This contribution provides background of these works in RAN2 and RAN4 and provide justifications for supporting RRM relaxation and RLM/BFD relaxation.

- Discussion

For connected mode, RAN4 is discussing RLM/BFD relaxation. Interaction between PDCCH relaxation and RLM/BM relaxation from power consumption is useful.

In RAN1#102-E, the following assumptions are agreed regarding periodic measurement activities including RLM/BFD when considering connected PDCCH relaxation schemes.

Agreements:

- Company should report assumptions used for periodic measurement activities for the Rel-17 DCI-based power saving adaptation evaluation.

  - The periodic activities defined in TR38.840 can be reused.

  - Measurement for RLM/BFD every C-DRX cycle can be optionally modelled

However, considering newRAN4 progress and agreements made, it can be further updated according to RAN4 progress on the assumptions for RLM/BFD relaxation.

Proposal 1: RAN1 can further discuss and update the evaluation assumptions for periodic measurement activities, i.e., RLM/BFD assumptions according to the RAN4 progress when evaluating PDCCH skipping/switching for connected mode.

In RAN4, the evaluation for RLM/BFD relaxation is more relevant to DRX cycle with short periodicity. In such case, the RLM/BFD measurement/evaluation is such frequent so that relaxation is necessary to save more UE power consumptions. Typical use case for configuring such short DRX cycle and its useful traffic model is needed also in RAN1 evalutions.

In [3], the traffic models are listed in Table 4. Mean inter-arrival time for FTP traffic	and Instant messaging (IM) is relatively large, e.g., >= 200ms. 160ms CDRX long cycle is suitable for FTP and IM. For VoIP, the Mean inter-arrival time for voice packets is 20ms and 40ms CDRX long cycle is suitable for VoIP assuming 2-packet bundling. However, the packet arrival for VoIP is periodic.

Table 1. Traffic models for R16 UE power saving

|  | FTP traffic | Instant messaging | VoIP |
| --- | --- | --- | --- |
| Model | FTP model 3 | FTP model 3 | As defined in R1-070674. Assume max two packets bundled. |
| Packet size | 0.5 Mbytes | 0.1 Mbytes |  |
| Mean inter-arrival time | 200 ms | 2 sec |  |
| DRX setting | Period = 160 ms Inactivity timer = 100 ms | Period = 320 ms Inactivity timer = 80 ms | Period = 40 ms Inactivity timer = 10 ms |

It is more challenging in power saving for services with smaller mean inter-arrival time (e.g., <100ms) and random packet arriving. Gaming is such kind of service and is studied in Rel-16 power saving SI phase.

In order to simplify the gaming traffic model with existing FTP model 3, the following reusing FTP Model 3 with modified mean inter-arrival time (e.g., online gaming) can be considered,

|  | Modified FTP traffic 3 |
| --- | --- |
| Model | FTP model 3 |
| Packet size | 0.1 Mbytes |
| Mean inter-arrival time | 50 ms |
| DRX setting | Period = 40 ms |

Proposal 2: A modified traffic model inter-arrival time can be considered in for power saving evaluation.

- reusing FTP Model 3 with modified mean inter-arrival time(e.g., online gaming)

|  | Modified FTP traffic 3 |
| --- | --- |
| Model | FTP model 3 |
| Packet size | 0.1 Mbytes |
| Mean inter-arrival time | 50 ms |
| DRX setting | Period = 40 ms |

- Conclusion

In this contribution,  the following proposals are presented,

Proposal 1: RAN1 can further discuss and update the evaluation assumptions for periodic measurement activities, i.e., RLM/BFD assumptions according to the RAN4 progress when evaluating PDCCH skipping/switching for connected mode.

Proposal 2: A modified traffic model inter-arrival time can be considered in for power saving evaluation.

- reusing FTP Model 3 with modified mean inter-arrival time(e.g., online gaming)

|  | Modified FTP traffic 3 |
| --- | --- |
| Model | FTP model 3 |
| Packet size | 0.1 Mbytes |
| Mean inter-arrival time | 50 ms |
| DRX setting | Period = 40 ms |

References

- RP-200938, Revised WID UE Power Saving Enhancements for NR, MediaTek

- 3GPP TS 38.213 v16.1.0.

- 3GPP TR 38.840 Study on UE power saving